| INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- |
| TELECOMMUNICATION DEVELOPMENT BUREAU ITU-D STUDY GROUPS | Document 2/169-E 20 July 2000 Original: English |
| THIRD MEETING OF STUDY GROUP 1: GENEVA, 11 – 15 SEPTEMBER 2000 THIRD MEETING OF STUDY GROUP 2: GENEVA, 18 – 22 SEPTEMBER 2000 |  |

FOR ACTION

Question 10a/2	Communications for rural and remote areas

Question 16/2: 	Preparation of Handbooks for developing countries

STUDY GROUP 2

SOURCE:	ERICSSON (SWEDEN) / NOKIA (FINLAND)

TITLE:	IMPLEMENTATION OF 3G CAPABILITIES IN DEVELOPING COUNTRIES
A STRAIGHTFORWARD PATH TO IMT-2000

________

Action required:

Participants are invited to consider the attached information.

Abstract:

The document exposes the advantages of a recent evolution of a digital cellular mobile system operating in the 450 MHz range, offering extended coverage and services comparable to the evolving GSM/EDGE system and future IMT-2000.

A more detailed technical description has been submitted to the ITU-R Working Party 8F.

Implementation OF 3G capabilities in developping Countries
a StrAightforward path to IMT-2000

## Introduction

This document presents the development of a GSM-like platform that offers an evolution path to new third generation multimedia services via GPRS and EDGE. The system operates in the 450 MHz range, so operators can provide economic wide-area coverage, and the same cell structure can be used for EDGE on 400 MHz as for 900 MHz.

The 450 MHz band continues to be valuable asset because operation in the 450 MHz band delivers an advantage in coverage over other systems operating in higher frequencies (800/900 and 1800/1900 MHz). This is particular benefit when providing coverage in coastal areas, extensive rural areas, and highways, requiring fewer cell sites than networks in higher bands. Down banded frequency will also ease the foreseen coverage shortcomings with high-speed data transmission utilising enhanced digital techniques such as GSM/EDGE.

GSM is the largest digital standard offering 3G capabilities and thus a natural choice for new and emerging markets particularly in developing countries. In light of these facts the availability of down banded GSM/EDGE is an urgent need for administrations, regulators, operators and vendors.

The European Standardisation Institute, ETSI, addressed the relevant issues and studied the possibility of deploying the new technology within frequency bands currently used for analogue cellular systems. The justification for this is that operators of these analogue systems may wish to offer services that are supported by digital systems, but not by the analogue standards. GSM400 is now included in the ETSI/SMG GSM/UMTS Release 99´. This standard allows for seamless roaming between 400 and higher frequencies.

## Extended high-speed data coverage

Use of lower frequencies is a cost-effective method of providing very wide area coverage quickly to medium to low densely populated areas.  Depending upon the market requirements these very wide area cells at 450 MHz can then be used to complement the relatively smaller cells at 900/1800/1900 MHz in higher density population areas.

GSM400 provides an extended data coverage solution for high-speed data in the evolved GSM/EDGE. The coverage reduction generated by higher data rates in GSM900/1800/1900 can be compensated for by EDGE at 400 MHz. Building complete coverage with 3G technologies requires heavy initial investments and GSM400 would be an optimal additional and complimentary solution to 3G licenses, providing an economical way of building the GSM footprint, enabling operators to provide nation-wide services though a joint GSM400/UMTS/IMT-2000 network.

## Proposed frequency allocation

- Examples of the present national frequency allocations are shown in Annex 1, Table 1. Most countries in Table 1 are supported by GSM/EDGE covering 450-467 MHz range. To enable the deployment of GSM400 in some other countries there was a need also for frequency allocations around 490 MHz.  Hence, ETSI has standardised 450-467 MHz as a primary band and 479-496 MHz as the extension of GSM400 frequency allocations. The ETSI GSM400 standard covers about 6.8 MHz uplink and 6.8 MHz downlink with 10 MHz duplex spacing in both bands.

The duplex spacing of 10 MHz is commonly utilised in analogue cellular systems around 450 MHz today. Due to national regulations it may be the case that the whole band for which the GSM400 system has been standardised can not be operated in each country. Current analogue allocations fit in most cases to standardised GSM400 bands. Because GSM400 and analogue systems have the same duplex spacing this allows GSM systems to operate even on fragments of the standardised GSM400 bands. Channel numbering is shown in Figure 1. Band specific channel numbering allows multiband 450/490 MHz MS implementations for to get more than one downbanded GSM/EDGE network on air within a region.

Current assumption is that most often a multiband 450 MHz MS is implemented so that it supports only one down banded frequency and roaming is achieved trough 900 MHz and 1800 MHz bands. Harmonised band would be beneficial but the standardised 6.8 MHz GSM400 operating band alleviates the definite need for it.

| ## GSM 400 AT A GLANCE |  |
| --- | --- |
| Frequency allocation | GSM 450 band: 450.4 - 457.6 MHz uplink 460.4 - 467.6 MHz downlink GSM 480 band: 478.8 – 486 MHz uplink 488.8 – 496 MHz downlink |
| Frequency spectrum | 6,8 MHz |
| Duplex separation | 10 MHz |
| Carrier spacing | 200 KHz |
| Coverage | Up to 120 kilometres |

Figure 1. MS frequency allocations and channel numbers.

It is therefore important to make provisions for the future allocation of the above frequency bands to the Mobile Service, with a view to provide for the introduction of this new and highly spectrum efficient and economical technique.

GSM 400 also offers an evolution path to third generation services via GPRS and EDGE  (see Figure 2). Utilising EDGE, operators can provide economic wide-area coverage for high-speed GSM data transmission. Operators can use the same cell structure for EDGE on 400 MHz as for 900 MHz, thus saving the cost of new complementary sites.

(Figure 2 not available electronically)

Figure 2. The GSM evolution path.

Extended Cell Features Link budget benefit due to larger antenna effective area for 450 MHz is about 6 dB compared to 900 MHz and about 12 dB compared to 1800 MHz. Additional 3 dB link budget gain compared to GSM1800 is available through higher transmit power in GSM400. Link budget gain can be converted to larger cell radius or to better quality in equal sized cells. From the link budget calculations, we may notice that 6 dB in link budget converts to about 70 % range benefit. Link budget gain will be in use also with high-speed GSM data transmission utilising EDGE.

The purpose of the extended-cell feature is to provide an additional coverage beyond the maximum cell radius of 35 km defined in the specifications. The extended-cell feature is best suited for applications in coastal areas (waterways, off-shores), deserts, rural areas, and in low-traffic areas where the terrain is flat.

- ETSI GSM400 standard has enabled spare bits in timing advance information element to allow up to 219 bit time advanced transmissions in MS. This corresponds to distance of 120 km. Uplink RACH will occupy 2 slots in cells which support extended ranges beyond 35 km. Thus a BTS capable to handle RACH receiving during two slots is required.

## ANNEX 1

## Pre-study on the frequency allocations

- Most NMT systems are operating in the region of 450MHz. These bands are allocated on a national basis, and these allocations differ (both within the CEPT region, and between regions). One important aspect of the ETSI feasibility study was to identify the candidate analogue cellular frequency bands, and the radio systems that are deployed in the same countries in the same and neighbouring frequency bands.

- For the spectrum harmonisation process, it is essential to find a strategy maximising EDGE technology potential on 400 MHz band. The 400 MHz band has been nationally regulated and there exist national variations of spectrum allocations. Table 1 illustrates the current frequency allocations. Some Latin-American countries have band available for cellular usage but its use is not as detailed specified as in operating countries.

Table 1: Frequency allocations on 450 MHz cellular band

| - NMT-450/ C-Netz Countries | - Mobile TX MHz | - Base TX MHz |
| --- | --- | --- |
| - ESTONIA | - 453.000 - 457.475 | - 463.000- 467.475 |
| - LATVIA | - " | - " |
| - LITHUANIA | - " | - " |
| - BELARUS | - " | - " |
| - CHINA DAGING | - " | - " |
| - MOLDOVA | - " | - " |
| - ROMANIA | - " | - " |
| - RUSSIA | - " | - " |
| - FINLAND | - " | - " |
| - DANMARK | - " | - " |
| - NORWAY | - " | - " |
| - ICELAND | - " | - " |
| - SWEDEN | - " | - " |
| - TUNISIA | - " | - " |
| - UKRAINE | - " | - " |
| - BULGARIA | - 452.500 - 457.475 | - 462.500 - 467.475 |
| - POLAND | - " | - " |
| - CZECH | - 451.310 - 455.730 | - 461.310 - 465.730 |
| - SLOVAKIA | - " | - " |
| - AUSTRIA | - 451.300 - 455.740 | - 461.310 - 465.740 |
| - BELGIUM | - " | - " |
| - NETHERLANDS | - " | - " |
| - GERMANY (C-Netz) | - 450.0 - 455.74 | - 460.0 - 465.74 |

| - NMT-450/ C-Netz Countries | - Mobile TX MHz | - Base TX MHz |
| --- | --- | --- |
| - PORTUGAL | - 453.050 – 457.525 | - 463.050 – 467.525 |
| - MALAYSIA | - 452.000- 456.475 | - 462.000 - 466.475 |
| - ITALY (RTMS-450) | - 450.000 - 454.800 | - 460.000 - 464.800 |
| - CHINA LIAO HE | - 452.000 - 454.975 | - 462.000 - 469.975 |
| - SPAIN | - 454.325 - 456.800 | - 464.535 - 468.800 |
| - HUNGARY | - 455.230 - 457.370 | - 465.230 - 467.370 |
| - FRANCE | - 450.000 - 454.475 | - 440.000 - 444.475 |
| - SOUTH AFRICA (C-Netz) | - 465.500 - 469.500 | - 455.500- 459.500 |
| - Other NMT-450 frequencies |  |  |
| - CROATIA | - 411.675 - 415.850 | - 421.675 - 425.850 |
| - SLOVENIA | - " | - " |
| - TURKEY | - 415.500 - 419.975 | - 425.500 - 429.975 |
| - INDONESIA | - 479.000 - 483.480 | - 489.000 - 493.480 |
| - THAILAND | - " | - " |
| - Available frequencies for GSM-450 |  |  |
| - BRAZIL | - 406-413 MHz | - 423-430 MHz |
| - COLOMBIA | - 403-500 MHz |  |
| - MEXICO | - 440-450 & 485-495 MHz |  |
| - PERU | - 440-450 & 485-495 MHz - 411-415 & 456-460 MHz |  |

## ANNEX 2

## List of Acronyms and Abbreviations

3G	Third Generation

BTS	Base Transceiver Station

CCCH	Common Control Channel

CEPT		Conference of European Postal and Telecommunications
	Administration

EDGE	Enhanced Data rates for Global Evolution

EIRP	Effective Isotropic Radiated Power

ERP	Equivalent Radiated Power or Effective

GPRS	General Packet Radio Services

GSM	Global System for Mobile Communication

ISDN	Integrated Services Digital Network

ME	Mobile Equipment

MS	Mobile Station

RACH	Random Access Channel

RF	Radio Frequency

RX	Receiver

SIM	Subscriber Identity Module

SMG	Special Mobile Group

TA	Traffic Area

TRX	Transceiver

TX	Transmitter

UMTS	Universal Mobile Telecommunications System

WLL	Wireless Local Loop

_________