State Commission on Radio Frequencies

of the Republic of Moldova

# NATIONAL TABLE

# OF FREQUENCY ALLOCATIONS

Adopted by Decision

of the State Commission  on Radio Frequencies

of the Republic of Moldova

nr. 11 of 27.04.2000

with modifications adopted by Decision

nr. 12 of 6.10.2000

Chisinau – 2000

This volume was prepared and published by

the State Communication Inspection

of the Ministry of Transport and Communications.

# TABLE OF CONTENTS

Page

| ## List of abbreviations | 4 |
| --- | --- |
| Introduction | 5 |
| National Table of Frequency Allocations General presentation | 6 |
| § 1.1 Allocation of frequency bands | 6 |
| § 1.2 Terms and definitions | 8 |
| § 1.3 Structure of National Table of Frequency Allocations | 12 |
| Table of Frequency Allocations | 14 |
| Annex 1 Relevant footnotes (Extract from Article S5 of the Radio Regulations) | 88 |
| Annex 2 National footnotes | 122 |
| Annex 3 Table of frequency bands, designated for industrial, scientific, medicinal and household applications | 128 |

# List of abbreviations

WARC-92		- World Administrative Conference 1992

WRC-(97)		- World Radiocommunication Conference 1997 (or other year)

CEPT			- European Conference of Postal and Telecommunications Administrations

CSFR	- State Commission on Radio Frequencies at the Government of the Republic

of Moldova

DECT			- Digital European Cordless Telecommunication System

GMDSS		- Global Maritime Distress and Safety system

GSM			- Global System for Mobile Communications

IMT-2000		- International Mobile Telecommunications

ISM			- Industrial, Scientific and Medical applications

MMDS, MVDS 	- Multimedia distribution system, Multipoint video distribution system

MRE			- Radioelectronic applications

MTC			- Ministry of Transport and Communications of the Republic of Moldova

NMT-450		- Nordic Mobile Telephone – analog cellular system, operating in 450 MHz

range

S-PCS			- Satellite Personal Communication System

SECAM		- Color television system

TFTS			- Terrestrial Flight Telecommunications System

ITU			- International Telecommunication Union

UMTS			- Universal Mobile Telecommunication System

# INTRODUCTION

Publication for the first time in Moldova of National Table of Frequency Allocations (NTFA) is of great importance and responsibility.

NTFA corresponds to national priorities and it is in conformity with allocation of frequency bands established by competent entities of the International Telecommunication Union, which contains in Article S5 of the Radio Regulations of the ITU.

NTFA has such structure, which permits its public consultation and understanding. In that sense NTFA consists of two parts: general presentation and corresponding table followed by relevant annexes.

General presentation comprises the following paragraphs:

## § 1.1   Allocation of frequency bands.

That paragraph contains summary on problem of allocation of frequency bands on international and national levels as well as conditions under which many radiocommunication services could use the same frequency band.

## § 1.2   Terms and definitions.

That paragraph was introduced with the purpose that specific terms for all users of the NTFA have the same meaning. It contains all terms used in the Table and in international and national footnotes.

Terms are followed by corresponding definitions as they appear in Article S1 of the ITU Radio Regulations.

## § 1.3   Structure of NTFA.

In that paragraph it is described structure of NTFA as well as meaning of codes used in columns of NTFA for indicating footnotes and national usage mode.

Part II - NTFA contains corresponding Table, Annexes 1 and 2 include footnotes within which frequencies could be used by services which a specific frequency band is allocated to, Annex 3 contains frequency bands for ISM applications and Annex 4 contains list of abbreviations.

NTFA is subject to periodical revision in conformity with requirements which are imposed by new national and international regulations which refer to allocation of frequency bands.

#### NATIONAL TABLE OF FREQUENCY ALLOCATIONS

# GENERAL PRESENTATION

## § 1.1 Allocation of frequency bands.

In conformity with Radio Regulations, an allocation of frequency band means an entry in Table of Frequency Allocations of that band with the purpose to use it by a terrestrial or space service or services or radioastronomy service under specified conditions.

On the international level allocation of frequency bands is done by World Radiocommunications Conferencies (WRCs), which gather all Member States of the International Telecommunication Union (The Republic of Moldova is a Member State of the International Telecommunication Union since 1992).

For the allocation of frequencies the world has been divided into three Regions as shown on the following map:

Region 1:  Region 1 includes the area limited on the east by line A (lines A, B and C are defined below) and on the west by line B, excluding any of the territory of the Islamic Republic of Iran which lies between these limits. It also includes the whole of the territory of Armenia, Azerbaijan, Russian Federation, Georgia, Kazakstan, Mongolia, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan, Turkey and Ukraine and the area to the north of Russian Federation which lies between lines A and C.

Moldova is in Region 1.

Region 2:  Region 2 includes the area limited on the east by line B and on the west by line C.

Region 3:  Region 3 includes the area limited on the east by line C and on the west by line A, except any of the territory of Armenia, Azerbaijan, Russian Federation, Georgia, Kazakstan, Mongolia, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan, Turkey and Ukraine and the area to the north of Russian Federation. It also includes that part of the territory of the Islamic Republic of Iran lying outside of those limits.

The lines A, B and C are defined as follows:

Line A:  Line A extends from the North Pole along meridian 40° East of Greenwich to parallel 40° North; thence by great circle arc to the intersection of meridian 60° East and the Tropic of Cancer; thence along the meridian 60° East to the South Pole.

Line B:  Line B extends from the North Pole along meridian 10° West of Greenwich to its intersection with parallel 72° North; thence by great circle arc to the intersection of meridian 50° West and parallel 40° North; thence by great circle arc to the intersection of meridian 20° West and parallel 10° South; thence along meridian 20° West to the South Pole.

Line C:  Line C extends from the North Pole by great circle arc to the intersection of parallel 65° 30 North with the international boundary in Bering Strait; thence by great circle arc to the intersection of meridian 165° East of Greenwich and parallel 50° North; thence by great circle arc to the intersection of meridian 170° West and parallel 10° North; thence along parallel 10° North to its intersection with meridian 120° West; thence along meridian 120° West to the South Pole.

On the national level allocation of frequency bands is done by the State Commission on Radio Frequencies of the Republic of Moldova.

## § 1.2 Terms and definitions.

Telecommunication:   Any transmission, emission or reception of signs, signals, writings, images and sounds or intelligence of any nature by wire, radio, optical or other electromagnetic systems.

Radio waves or hertzian waves:  Electromagnetic waves of frequencies arbitrarily lower than 3 000 GHz, propagated in space without artificial guide.

Radiocommunication:  Telecommunication by means of radio waves

Allocation (of a frequency band):  Entry in the Table of Frequency Allocations of a given frequency band for the purpose of its use by one or more terrestrial or space radiocommunication services or the radio astronomy service under specified conditions. This term shall also be applied to the frequency band concerned.

Allotment (of a radio frequency or radio frequency channel):  Entry of a designated frequency channel in an agreed plan, adopted by a competent conference, for use by one or more administrations for a terrestrial or space radiocommunication service in one or more identified countries or geographical areas and under specified conditions.

Asssignment (of a radio frequency or radio frequency channel):  Authorization given by an administration for a radio station to use a radio frequency or radio frequency channel under specified conditions.

Public correspondence:  Any telecommunication which the offices and stations must, by reason of their being at the disposal of the public, accept for transmission

Terrestrial radiocommunication:  Any radiocommunication other than space radiocommunication or radio astronomy.

Space radiocommunication:  Any radiocommunication involving the use of one or more space stations or the use of one or more reflecting satellites or other objects in space.

Radiodetermination:  The determination of the position, velocity and/or other characteristics of an object, or the obtaining of information relating to these parameters, by means of the propagation properties of radio waves.

Radionavigation:  Radiodetermination used for the purposes of navigation, including obstruction warning.

Radiolocation:  Radiodetermination used for purposes other than those of radionavigation.

Radio astronomy:  Astronomy based on the reception of radio waves of cosmic origin.

Industrial, scientific and medical (ISM) applications (of radio frequency energy):  Operation of equipment or appliances designed to generate and use locally radio frequency energy for industrial, scientific, medical, domestic or similar purposes, excluding applications in the field of telecommunications.

Emission:  Radiation produced, or the production of radiation, by a radio transmitting station. For example, the energy radiated by the local oscillator of a radio receiver would not be an emission but a radiation.

Harmful interference:  Interference which endangers the functioning of a radionavigation service or of other safety services or seriously degrades, obstructs, or repeatedly interrupts a radiocommunication service operating in accordance with Radio Regulations.

Station:  One or more transmitters or receivers or a combination of transmitters and receivers, including the accessory equipment, necessary at one location for carrying on a radiocommunication service, or the radio astronomy service.

Each station shall be classified by the service in which it operates permanently or temporarily.

Feeder link:  A radio link from an earth station at a given location to a space station, or vice versa, conveying information for a space radiocommunication service other than for the fixed-satellite service. The given location may be at a specified fixed point, or at any fixed point within specified areas.

Radiocommunication service:  A service as defined in this Section involving the transmission, emission and/or reception of radio waves for specific telecommunication purposes.

In this document, unless otherwise stated, any radiocommunication service relates to terrestrial radiocommunication.

Fixed service:  A radiocommunication service between specified fixed points.

Fixed-satellite service:  A radiocommunication service between earth stations at given positions, when one or more satellites are used; the given position may be a specified fixed point or any fixed point within specified areas; in some cases this service includes satellite-to-satellite links, which may also be operated in the inter-satellite service; the fixed-satellite service may also include feeder links for other space radiocommunication services.

Inter-satellite service:  A radiocommunication service providing links between artificial satellites.

Space operation service:  A radiocommunication service concerned exclusively with the operation of spacecraft, in particular space tracking, space telemetry and space telecommand.

These functions will normally be provided within the service in which the space station is operating.

Mobile service:  A radiocommunication service between mobile and land stations, or between mobile stations

Mobile-satellite service:  A radiocommunication service:

- between mobile earth stations and one or more space stations, or between space stations used by this service; or

- between mobile earth stations by means of one or more space stations.

This service may also include feeder links necessary for its operation.

Land mobile service:  A mobile service between base stations and land mobile stations, or between land mobile stations.

Maritime mobile service:  A mobile service between coast stations and ship stations, or between ship stations, or between associated on-board communication stations; survival craft stations and emergency position-indicating radiobeacon stations may also participate in this service.

Port operations service:  A maritime mobile service in or near a port, between coast stations and ship stations, or between ship stations, in which messages are restricted to those relating to the operational handling, the movement and the safety of ships and, in emergency, to the safety of persons.

Messages which are of a public correspondence nature shall be excluded from this service.

Ship movement service:  A safety service in the maritime mobile service other than a port operations service, between coast stations and ship stations, or between ship stations, in which messages are restricted to those relating to the movement of ships.

Messages which are of a public correspondence nature shall be excluded from this service.

Aeronautical mobile service:  A mobile service between aeronautical stations and aircraft stations, or between aircraft stations, in which survival craft stations may participate; emergency position-indicating radiobeacon stations may also participate in this service on designated distress and emergency frequencies.

Aeronautical mobile (R)* service:  An aeronautical mobile service reserved for communications relating to safety and regularity of flight, primarily along national or international civil air routes.

Aeronautical mobile (OR)** service:  An aeronautical mobile service intended for communications, including those relating to flight coordination, primarily outside national or international civil air routes.

Aeronautical mobile-satellite service:  A mobile-satellite service in which mobile earth stations are located on board aircraft; survival craft stations and emergency position-indicating radiobeacon stations may also participate in this service.

Aeronautical mobile-satellite (R)* service:  An aeronautical mobile-satellite service reserved for communications relating to safety and regularity of flights, primarily along national or international civil air routes.

Aeronautical mobile-satellite (OR)** service:  An aeronautical mobile-satellite service intended for communications, including those relating to flight coordination, primarily outside national and international civil air routes.

Broadcasting service:  A radiocommunication service in which the transmissions are intended for direct reception by the general public. This service may include sound transmissions, television transmissions or other types of transmission.

Broadcasting-satellite service:  A radiocommunication service in which signals transmitted or retransmitted by space stations are intended for direct reception by the general public.

In the broadcasting-satellite service, the term “direct reception” shall encompass both individual reception and community reception.

Radiodetermination service:  A radiocommunication service for the purpose of radiodetermination.

Radiodetermination-satellite service:  A radiocommunication service for the purpose of radiodetermination involving the use of one or more space stations.

This service may also include feeder links necessary for its own operation.

Radionavigation service: A radiodetermination service for the purpose of radionavigation.

Radionavigation-satellite service:  A radiodetermination-satellite service used for the purpose of radionavigation.

This service may also include feeder links necessary for its operation.

Maritime radionavigation service:  A radionavigation service intended for the benefit and for the safe operation of ships.

Aeronautical radionavigation service:  A radionavigation service intended for the benefit and for the safe operation of aircraft.

Aeronautical radionavigation-satellite service:  A radionavigation-satellite service in which earth stations are located on board aircraft.

Radiolocation service:  A radiodetermination service for the purpose of radiolocation.

Radiolocation-satellite service:  A radiodetermination-satellite service used for the purpose of radiolocation.

This service may also include the feeder links necessary for its operation.

Meteorological aids service:  A radiocommunication service used for meteorological, including hydrological, observations and exploration.

Earth exploration-satellite service:  A radiocommunication service between earth stations and one or more space stations, which may include links between space stations, in which:

- information relating to the characteristics of the Earth and its natural phenomena, including data relating to the state of the environment, is obtained from active sensors or passive sensors on Earth satellites;

- similar information is collected from airborne or Earth-based platforms;

- such information may be distributed to earth stations within the system concerned;

- platform interrogation may be included.

This service may also include feeder links necessary for its operation.

Meteorological-satellite service:  An earth exploration-satellite service for meteorological purposes.

Standard frequency and time signal service:  A radiocommunication service for scientific, technical and other purposes, providing the transmission of specified frequencies, time signals, or both, of stated high precision, intended for general reception.

Standard frequency and time signal-satellite service:  A radiocommunication service using space stations on earth satellites for the same purposes as those of the standard frequency and time signal service.

This service may also include feeder links necessary for its operation.

Space research service:  A radiocommunication service in which spacecraft or other objects in space are used for scientific or technological research purposes.

Amateur service:  A radiocommunication service for the purpose of self-training, intercommunication and technical investigations carried out by amateurs, that is, by duly authorized persons interested in radio technique solely with a personal aim and without pecuniary interest.

Amateur-satellite service:  A radiocommunication service using space stations on earth satellites for the same purposes as those of the amateur service.

Radio astronomy service:  A service involving the use of radio astronomy.

Safety service:  Any radiocommunication service used permanently or temporarily for the safeguarding of human life and property.

Special service:  A radiocommunication service, not otherwise defined in this Section, carried on exclusively for specific needs of general utility, and not open to public correspondence.

## § 1.3 Structure of the National Table of Frequency Allocations.

National Table of Frequency Allocations consists of four columns:

Column 1 – International allocation for Region 1. Frequency band – Services – Footnotes. It contains allocation of frequency bands for different radiocommunication services for Region 1 countries. Contents of this column is identical to the column 1 of the Table of Frequency Allocations of Article S5 of the Radio Regulations.

Columns, which correspond to national allocation, have the following contents:

Column 2 – Frequency Band – Services. It contains allocation of frequency bands for different radiocommunications services in Moldova. This allocation corresponds to provisions of Article S5 of the Radio Regulations.

Column 3 – Footnotes. This column contains reference numbers of footnotes under which corresponding service is permitted to be used in Moldova. Reference numbers have the following meaning:

- numbers of type S5.317A correspond to numbers under which corresponding footnotes could be found in Article S5 of the Radio Regulations. Texts of those footnotes are shown in Annex 1 to the National Table of Frequency Allocations

- three digit numbers followed by letters RN, correspond to national footnotes which describe national usage of specific frequency band. Texts of those footnotes are shown in Annex 2 to the National Table of Frequency Allocations

Footnotes are integral part of the NTFA.

Column 4 – Usage. It contains mode of use of frequency bands in Moldova. The meaning of remarks in this column is as follows:

- G – means that corresponding band is allocated exclusively for the governmental purposes (defense, national security, governmental communications, civil protection, police). Frequencies in those bands are assigned by interested entities;

- NG – means that corresponding band is allocated exclusively for the nongovernmental purposes. Frequencies in those bands are assigned by the State Communication Inspection.

- P – means that corresponding band is shared by governmental and nongovernmental users. Frequencies in those bands are assigned by interested parties and the State Communication Inspection in conformity with Procedure on mode of allocation of frequency bands and frequency assignments.

Status under which radiocommunication services could use allocated band can be:

- primary

- secondary

Services of the same status have equal rights.

Stations of a secondary service:

- shall not cause harmful interference to stations of primary services to which frequencies are already assigned or to which frequencies may be assigned at a later date;

- cannot claim protection from harmful interference from stations of a primary service to which frequencies are already assigned or may be assigned at a later date;

- can claim protection, however, from harmful interference from stations of the same or other secondary service(s) to which frequencies may be assigned at a later date.

Where it is indicated in the Table that a service may operate in a specific frequency band subject to not causing harmful interference, this means also that this service cannot claim protection from harmful interference caused by other services to which the band is allocated in conformity with the Table.

Status of services is indicated in the Table in the following manner:

- services the names of which are printed in “capitals” (example: FIXED); these are called primary services;

- services the names of which are printed in “normal characters” (example: Mobile); these are called secondary services

While delimiting frequency bands it is adopted conventionally that if frequency assignment corresponds to the lower limit of a band it refers to this band and if frequency assignment corresponds to the upper limit of a band it refers to the next upper band.

Note: Usage of radioelectronic applications in conformity with NTFA, is permitted only if standards, regulations in radiocommunications and procedures for authorisation in force are respected.

# TABLE OF FREQUENCY ALLOCATIONS

| Region 1 | National allocation |  |  |
| --- | --- | --- | --- |
| Frequency band – services - footnotes | Frequency band - services | Footnotes | Usage |
| Below 9 kHz (Not allocated) S5.53, S5.54 | Below 9 kHz (Not allocated) | S5.53, S5.54 RN035 |  |
| 9 - 14 kHz RADIONAVIGATION | 9 - 14 kHz RADIONAVIGATION | RN035 | P |
| 14 - 19.95 kHz FIXED MARITIME MOBILE S5.57 S5.55, S5.56 | 14 - 19.95 kHz FIXED MARITIME MOBILE | S5.56, S5.57 RN035 | P |
| 19.95 – 20.05 kHz STANDARD FREQUENCY AND TIME SIGNAL (20 kHz) | 19.95 – 20.05 kHz STANDARD FREQUENCY AND TIME SIGNAL (20 kHz) | RN035 | P |
| 20.05 - 70 kHz FIXED MARITIME MOBILE S5.57 S5.56, S5.58 | 20.05 - 70 kHz FIXED MARITIME MOBILE | S5.56, S5.57 RN035 | P |
| 70 - 72 kHz RADIONAVIGATION S5.60 | 70 - 72 kHz RADIONAVIGATION | S5.60 RN035 | P |
| 72 - 84 kHz FIXED MARITIME MOBILE S5.57 RADIONAVIGATION S5.60 S5.56 | 72 - 84 kHz FIXED MARITIME MOBILE RADIONAVIGATION | S5.56, S5.57, S5.60 RN001, RN035 | P |
| 84 - 86 kHz RADIONAVIGATION S5.60 | 84 - 86 kHz RADIONAVIGATION | S5.60 RN035 | P |
| 86 - 90 kHz FIXED MARITIME MOBILE S5.57 RADIONAVIGATION S5.56 | 86 - 90 kHz FIXED MARITIME MOBILE RADIONAVIGATION | S5.56, S5.57 RN035 | P |
| 90 - 110 kHz RADIONAVIGATION S5.62 Fixed S5.64 | 90 - 110 kHz RADIONAVIGATION Fixed | S5.62, S5.64 RN035 | P |
| 110 - 112 kHz FIXED MARITIME MOBILE RADIONAVIGATION S5.64 | 110 - 112 kHz FIXED MARITIME MOBILE RADIONAVIGATION | S5.64 RN035 | P |
| 112 -115 kHz RADIONAVIGATION S5.60 | 112 -115 kHz RADIONAVIGATION | S5.60 RN035 | P |
| 115 – 117.6 kHz RADIONAVIGATION S5.60 Fixed Maritime Mobile S5.64, S5.66 | 115 – 117.6 kHz RADIONAVIGATION Fixed Maritime Mobile | S5.60, S5.64 RN035 | P |
| 117.6 - 126 kHz FIXED MARITIME MOBILE RADIONAVIGATION S5.60 S5.64 | 117.6 - 126 kHz FIXED MARITIME MOBILE RADIONAVIGATION | S5.60, S5.64 RN001, RN035 | P |
| 126 - 129 kHz RADIONAVIGATION S5.60 | 126 - 129 kHz RADIONAVIGATION | S5.60 RN035 | P |
| 129 - 130 kHz FIXED MARITIME MOBILE RADIONAVIGATION S5.60 S5.64 | 129 - 130 kHz FIXED MARITIME MOBILE RADIONAVIGATION | S5.60, S5.64 RN035 | P |
| 130 – 148.5 kHz FIXED MARITIME MOBILE S5.64, S5.67 | 130 – 148.5 kHz FIXED MARITIME MOBILE | S5.64 RN035 | P |
| 148.5 - 255.0 kHz BROADCASTING S5.68, S5.69, S5.70 | 148.5 - 255.0 kHz BROADCASTING | RN002 | NG |
| 255.0 – 283.5 kHz BROADCASTING AERONAUTICAL RADIONAVIGATION S5.70, S5.71 | 255.0 – 283.5 kHz BROADCASTING AERONAUTICAL RADIONAVIGATION | RN002 | P |
| 283.5 - 315 kHz MARITIME RADIONAVIGATION (radiobeacons) S5.73 AERONAUTICAL RADIONAVIGATION S5.72, S5.74 | 283.5 - 315 kHz MARITIME RADIONAVIGATION (radiobeacons) AERONAUTICAL RADIONAVIGATION | S5.73, S5.74 | P |
| 315 - 325 kHz AERONAUTICAL RADIONAVIGATION Maritime Radionavigation (radiobeacons) S5.73 S5.72, S5.75 | 315 - 325 kHz AERONAUTICAL RADIONAVIGATION Maritime Radionavigation (radiobeacons) | S5.73, S5.75 | P |
| 325 - 405 kHz AERONAUTICAL RADIONAVIGATION S5.72 | 325 - 405 kHz AERONAUTICAL RADIONAVIGATION |  | P |
| 405 - 415 kHz RADIONAVIGATION S5.72, S5.76 | 405 - 415 kHz RADIONAVIGATION | S5.76 RN003 | P |
| 415 - 435 kHz AERONAUTICAL RADIONAVIGATION MARITIME MOBILE S5.79 S5.72 | 415 - 435 kHz AERONAUTICAL RADIONAVIGATION MARITIME MOBILE | S5.79 | P |
| 435 - 495 kHz MARITIME MOBILE S5.79, S5.79A Aeronautical Radionavigation S5.72, S5.82 | 435 - 495 kHz MARITIME MOBILE Aeronautical Radionavigation | S5.79, S5.79A, S5.82 RN035 | P |
| 495 - 505 kHz MOBILE (distress and calling) S5.83 | 495 - 505 kHz MOBILE (distress and calling) | S5.83 | P |
| 505 – 526.5 kHz MARITIME MOBILE S5.79 S5.79A, S5.84 AERONAUTICAL RADIONAVIGATION S5.72 | 505 – 526.5 kHz MARITIME MOBILE AERONAUTICAL RADIONAVIGATION | S5.79, S5.79A, S5.84 RN004 | P |
| 526.5 – 1606.5 kHz BROADCASTING S5.87, S5.87A | 526.5 – 1606.5 kHz BROADCASTING | RN005 | P |
| 1606.5 - 1625 kHz FIXED LAND MOBILE MARITIME MOBILE S5.90 S5.92 | 1606.5 - 1625 kHz FIXED LAND MOBILE MARITIME MOBILE | S5.90, S5.92 | P |
| 1625 - 1635 kHz RADIOLOCATION S5.93 | 1625 - 1635 kHz RADIOLOCATION FIXED LAND MOBILE | S5.93 | P |
| 1635 - 1800 kHz MARITIME MOBILE S5.90 FIXED LAND MOBILE S5.92, S5.96 | 1635 - 1800 kHz MARITIME MOBILE FIXED LAND MOBILE | S5.90, S5.92, S5.96 | P |
| 1800 - 1810 kHz RADIOLOCATION S5.93 | 1800 - 1810 kHz RADIOLOCATION FIXED LAND MOBILE | S5.93 | P |
| 1810 - 1850 kHz AMATEUR S5.98, S5.99, S5.100, S5.101 | 1810 - 1830 kHz FIXED MOBILE except aeronautical mobile 1830 - 1850 kHz AMATEUR | S5.98, S5.100 | P NG |
| 1850 - 2000 kHz FIXED MOBILE except aeronautical mobile S5.92, S5.96, S5.103 | 1850 - 2000 kHz FIXED MOBILE except aeronautical mobile Amateur | S5.92, S5.96, S5.103 | P |
| 2000 - 2025 kHz FIXED MOBILE except aeronautical mobile (R) S5.92, S5.103 | 2000 - 2025 kHz FIXED MOBILE except aeronautical mobile (R) | S5.92, S5.103 | P |
| 2025 - 2045 kHz FIXED MOBILE except aeronautical mobile (R) METEOROLOGICAL Aids S5.104 S5.92, S5.103 | 2025 - 2045 kHz FIXED MOBILE except aeronautical mobile (R) METEOROLOGICAL Aids | S5.92, S5.103, S5.104 | P |
| 2045 - 2160 kHz MARITIME MOBILE FIXED LAND MOBILE S5.92 | 2045 - 2160 kHz MARITIME MOBILE FIXED LAND MOBILE | S5.92 | P |
| 2160 - 2170 kHz RADIOLOCATION S5.93, S5.107 | 2160 - 2170 kHz RADIOLOCATION | S5.93 | P |
| 2170 – 2173.5 kHz MARITIME MOBILE | 2170 – 2173.5 kHz MARITIME MOBILE |  | P |
| 2173.5 – 2190.5 kHz MOBILE (distress and calling) S5.108, S5.109, S5.110, S5.111 | 2173.5 – 2190.5 kHz MOBILE (distress and calling) | S5.108, S5.109, S5.110, S5.111 | P |
| 2190.5 - 2194 kHz MARITIME MOBILE | 2190.5 - 2194 kHz MARITIME MOBILE |  | P |
| 2194 - 2300 kHz FIXED MOBILE except aeronautical mobile (R) S5.92, S5.103, S5.112 | 2194 - 2300 kHz FIXED MOBILE except aeronautical mobile (R) | S5.103 | P |
| 2300 - 2498 kHz FIXED MOBILE except aeronautical mobile (R) BROADCASTING S5.113 S5.103 | 2300 - 2498 kHz FIXED MOBILE except aeronautical mobile (R) BROADCASTING | S5.103, S5.113 | P |
| 2498 - 2501 kHz STANDARD FREQUENCY AND TIME SIGNAL (2500 kHz) | 2498 - 2501 kHz STANDARD FREQUENCY AND TIME SIGNAL (2500 kHz) |  | P |
| 2501 - 2502 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research | 2501 - 2502 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 2502 - 2625 kHz FIXED MOBILE except aeronautical mobile (R) S5.92, S5.103, S5.114 | 2502 - 2625 kHz FIXED MOBILE except aeronautical mobile (R) | S5.92, S5.103 | P |
| 2625 - 2650 kHz MARITIME MOBILE MARITIME RADIONAVIGATION S5.92 | 2625 - 2650 kHz MARITIME MOBILE MARITIME RADIONAVIGATION | S5.92 | ###### P |
| 2650 - 2850 kHz FIXED MOBILE except aeronautical mobile (R) S5.92, S5.103 | 2650 - 2850 kHz FIXED MOBILE except aeronautical mobile (R) | S5.103 | P |
| 2850 - 3025 kHz AERONAUTICAL MOBILE (R) S5.111, S5.115 | 2850 - 3025 kHz AERONAUTICAL MOBILE (R) | S5.115 | P |
| 3025 - 3155 kHz AERONAUTICAL MOBILE (OR) | 3025 - 3155 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 3155 - 3200 kHz FIXED MOBILE except aeronautical mobile (R) S5.116, S5.117 | 3155 - 3200 kHz FIXED MOBILE except aeronautical mobile (R) | S5.116 | P |
| 3200 - 3230 kHz FIXED MOBILE except aeronautical mobile (R) BROADCASTING S5.113 S5.116 | 3200 - 3230 kHz FIXED MOBILE except aeronautical mobile (R) BROADCASTING | S5.113, S5.116 | P |
| 3230 - 3400 kHz FIXED MOBILE except aeronautical mobile BROADCASTING S5.113 S5.116, S5.118 | 3230 - 3400 kHz FIXED MOBILE except aeronautical mobile BROADCASTING | S5.113, S5.116 | P |
| 3400 - 3500 kHz AERONAUTICAL MOBILE (R) | 3400 - 3500 kHz AERONAUTICAL MOBILE (R) |  | P |
| 3500 - 3800 kHz AMATEUR FIXED MOBILE except aeronautical mobile S5.92 | 3500 - 3800 kHz AMATEUR FIXED MOBILE except aeronautical mobile | S5.92 | P |
| 3800 - 3900 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE | 3800 - 3900 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE |  | P |
| 3900 - 3950 kHz AERONAUTICAL MOBILE (OR) S5.123 | 3900 - 3950 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 3950 - 4000 kHz FIXED BROADCASTING | 3950 - 4000 kHz FIXED BROADCASTING |  | P |
| 4000 - 4063 kHz FIXED MARITIME MOBILE S5.127 S5.126 | 4000 - 4063 kHz FIXED MARITIME MOBILE | S5.127 | P |
| 4063 - 4438 kHz MARITIME MOBILE S5.109, S5.110, S5.130, S5.131, S5.132 S5.79A, S5.128, S5.129 | 4063 - 4438 kHz MARITIME MOBILE | S5.109, S5.110, S5.129, S5.130, S5.131, S5.132, S5.79A | P |
| 4438 - 4650 kHz FIXED MOBILE except aeronautical mobile (R) | 4438 - 4650 kHz FIXED MOBILE except aeronautical mobile (R) | RN035 | P |
| 4650 - 4700 kHz AERONAUTICAL MOBILE (R) | 4650 - 4700 kHz AERONAUTICAL MOBILE (R) |  | P |
| 4700 - 4750 kHz AERONAUTICAL MOBILE (OR) | 4700 - 4750 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 4750 - 4850 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE BROADCASTING S5.113 | 4750 - 4850 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE BROADCASTING | S5.113 | P |
| 4850 - 4995 kHz FIXED LAND MOBILE BROADCASTING S5.113 | 4850 - 4995 kHz FIXED LAND MOBILE BROADCASTING |  | P |
| 4995 - 5003 kHz STANDARD FREQUENCY AND TIME SIGNAL (5000 kHz) | 4995 - 5003 kHz STANDARD FREQUENCY AND TIME SIGNAL (5000 kHz) |  | P |
| 5003 - 5005 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research | 5003 - 5005 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 5005 - 5060 kHz FIXED BROADCASTING S5.113 | 5005 - 5060 kHz FIXED BROADCASTING | S5.113 | P |
| 5060 - 5250 kHz FIXED Mobile except aeronautical mobile S5.133 | 5060 - 5250 kHz FIXED Mobile except aeronautical mobile | S5.133 | P |
| 5250 - 5450 kHz FIXED MOBILE except aeronautical mobile | 5250 - 5450 kHz FIXED MOBILE except aeronautical mobile |  | P |
| 5450 - 5480 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE | 5450 - 5480 kHz FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE |  | P |
| 5480 - 5680 kHz AERONAUTICAL MOBILE (R) S5.111, S5.115 | 5480 - 5680 kHz AERONAUTICAL MOBILE (R) | S5.115 | P |
| 5680 - 5730 kHz AERONAUTICAL MOBILE (OR) S5.111, S5.115 | 5680 - 5730 kHz AERONAUTICAL MOBILE (OR) | S5.115 | P |
| 5730 - 5900 kHz FIXED LAND MOBILE | 5730 - 5900 kHz FIXED LAND MOBILE |  | P |
| 5900 - 5950 kHz BROADCASTING S5.134, S5.136 | 5900 - 5950 kHz FIXED LAND MOBILE BROADCASTING | S5.134, S5.136 RN006 | P |
| 5950 - 6200 kHz BROADCASTING | 5950 - 6200 kHz BROADCASTING |  | NG |
| 6200 - 6525 kHz MARITIME MOBILE S5.109 S5.110, S5.130, S5.132, S5.137 | 6200 - 6525 kHz MARITIME MOBILE | S5.109, S5.110, S5.130, S5.132, S5.137 | P |
| 6525 - 6685 kHz AERONAUTICAL MOBILE (R) | 6525 - 6685 kHz AERONAUTICAL MOBILE (R) |  | P |
| 6685 - 6765 kHz AERONAUTICAL MOBILE (OR) | 6685 - 6765 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 6765 - 7000 kHz FIXED Land Mobile S5.139 S5.138 | 6765 - 7000 kHz FIXED LAND MOBILE | S5.138, S5.139 RN035 | P |
| 7000 - 7100 kHz AMATEUR AMATEUR-SATELLITE S5.140, S5.141 | 7000 - 7100 kHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 7100 - 7300 kHz BROADCASTING | 7100 - 7300 kHz BROADCASTING |  | NG |
| 7300 - 7350 kHz BROADCASTING S5.134, S5.143 | 7300 - 7350 kHz FIXED BROADCASTING Land Mobile | S5.134, S5.143 RN006 | P |
| 7350 - 8100 kHz FIXED Land Mobile | 7350 - 8100 kHz FIXED Land Mobile | RN035 | P |
| 8100 - 8195 kHz FIXED MARITIME MOBILE | 8100 - 8195 kHz FIXED MARITIME MOBILE | RN035 | P |
| 8195 - 8815 kHz MARITIME MOBILE S5.109, S5.110, S5.111 S5.132, S5.145 | 8195 - 8815 kHz MARITIME MOBILE | S5.109, S5.110, S5.132, S5.145 RN035 | P |
| 8815 - 8965 kHz AERONAUTICAL MOBILE (R) | 8815 - 8965 kHz AERONAUTICAL MOBILE (R) |  | P |
| 8965 - 9040 kHz AERONAUTICAL MOBILE (OR) | 8965 - 9040 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 9040 - 9400 kHz FIXED | 9040 - 9400 kHz FIXED |  | P |
| 9400 - 9500 kHz BROADCASTING S5.134, S5.146 | 9400 – 9500 kHz BROADCASTING FIXED | S5.134, S5.146 RN006 | P |
| 9500 - 9900 kHz BROADCASTING S5.147 | 9500 – 9900 kHz BROADCASTING | S5.147 | P |
| 9900 - 9995 kHz FIXED | 9900 - 9995 kHz FIXED |  | P |
| 9995 - 10003 kHz STANDARD FREQUENCY AND TIME SIGNAL (10 000 kHz) S5.111 | 9995 - 10003 kHz STANDARD FREQUENCY AND TIME SIGNAL (10 000 kHz) |  | P |
| 10003 - 10005 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research S5.111 | 10003 - 10005 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 10005 - 10100 kHz AERONAUTICAL MOBILE (R) S5.111 | 10005 - 10100 kHz AERONAUTICAL MOBILE (R) |  | P |
| 10100 - 10150 kHz FIXED Amateur | 10100 - 10150 kHz FIXED Amateur |  | P |
| 10150 - 11175 kHz FIXED Mobile except aeronautical mobile (R) | 10150 - 11175 kHz FIXED Mobile except aeronautical mobile (R) |  | P |
| 11175 - 11275 kHz AERONAUTICAL MOBILE (OR) | 11175 - 11275 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 11275 - 11400 kHz AERONAUTICAL MOBILE (R) | 11275 - 11400 kHz AERONAUTICAL MOBILE (R) |  | P |
| 11400 - 11600 kHz FIXED | 11400 - 11600 kHz FIXED |  | P |
| 11600 - 11650 kHz BROADCASTING S5.134, S5.135, S5.146 | 11600 - 11650 kHz FIXED BROADCASTING | S5.134, S5.146 RN006 | P |
| 11650 - 12050 kHz BROADCASTING S5.147 | 11650 - 12050 kHz BROADCASTING | S5.147 | NG |
| 12050 - 12100 kHz BROADCASTING S5.134, S5.146 | 12050 - 12100 kHz FIXED BROADCASTING | S5.134, S5.146 RN006 | P |
| 12100 - 12230 kHz FIXED | 12100 - 12230 kHz FIXED |  | P |
| 12230 - 13200 kHz MARITIME MOBILE S5.109, S5.110, S5.132, S5.145 | 12230 - 13200 kHz MARITIME MOBILE | S5.109, S5.110, S5.132, S5.145 RN009 | P |
| 13200 - 13260 kHz AERONAUTICAL MOBILE (OR) | 13200 - 13260 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 13260 - 13360 kHz AERONAUTICAL MOBILE (R) | 13260 - 13360 kHz AERONAUTICAL MOBILE (R) |  | P |
| 13360 - 13410 kHz FIXED RADIO ASTRONOMY S5.149 | 13360 - 13410 kHz FIXED RADIO ASTRONOMY | S5.149 | P |
| 13410 - 13570 kHz FIXED Mobile except aeronautical mobile (R) S5.150 | 13410 - 13570 kHz FIXED Mobile except aeronautical mobile (R) | S5.150 RN035 | P |
| 13570 - 13600 kHz BROADCASTING S5.134, S5.151 | 13570 - 13600 kHz FIXED Mobile except aeronautical mobile (R) BROADCASTING | S5.134, S5.151 RN006 | P |
| 13600 - 13800 kHz BROADCASTING | 13600 - 13800 kHz BROADCASTING |  | NG |
| 13800 - 13870 kHz BROADCASTING S5.134, S5.151 | 13800 - 13870 kHz FIXED Mobile except aeronautical mobile (R) BROADCASTING | S5.134, S5.151 RN006 | P |
| 13870 - 14000 kHz FIXED Mobile except aeronautical mobile (R) | 13870 - 14000 kHz FIXED Mobile except aeronautical mobile (R) |  | P |
| 14000 - 14250 kHz AMATEUR AMATEUR-SATELLITE | 14000 - 14250 kHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 14250 - 14350 kHz AMATEUR S5.152 | 14250 - 14350 kHz AMATEUR FIXED | S5.152 | P |
| 14350 - 14990 kHz FIXED Mobile except aeronautical mobile (R) | 14350 - 14990 kHz FIXED Mobile except aeronautical mobile (R) |  | P |
| 14990 - 15005 kHz STANDARD FREQUENCY AND TIME SIGNAL (15000 kHz) S5.111 | 14990 - 15005 kHz STANDARD FREQUENCY AND TIME SIGNAL (15000 kHz) |  | P |
| 15005 - 15010 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research | 15005 - 15010 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 15010 - 15100 kHz AERONAUTICAL MOBILE (OR) | 15010 - 15100 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 15100 - 15600 kHz BROADCASTING | 15100 - 15600 kHz BROADCASTING |  | NG |
| 15600 - 15800 kHz BROADCASTING S5.134, S5.146 | 15600 - 15800 kHz FIXED BROADCASTING | S5.134, S5.146 RN006 | P |
| 15800 - 16360 kHz FIXED S5.153 | 15800 - 16360 kHz FIXED |  | P |
| 16360 - 17410 kHz MARITIME MOBILE S5.109, S5.110, S5.132, S5.145 | 16360 - 17410 kHz MARITIME MOBILE | S5.109, S5.110, S5.132, S5.145 RN009 | P |
| 17410 - 17480 kHz FIXED | 17410 - 17480 kHz FIXED |  | P |
| 17480 - 17550 kHz BROADCASTING S5.134, S5.146 | 17480 - 17550 kHz BROADCASTING FIXED | S5.134, S5.146 RN006 | P |
| 17550 - 17900 kHz BROADCASTING | 17550 - 17900 kHz BROADCASTING |  | NG |
| 17900 - 17970 kHz AERONAUTICAL MOBILE (R) | 17900 - 17970 kHz AERONAUTICAL MOBILE (R) |  | P |
| 17970 - 18030 kHz AERONAUTICAL MOBILE (OR) | 17970 - 18030 kHz AERONAUTICAL MOBILE (OR) |  | P |
| 18030 - 18052 kHz FIXED | 18030 - 18052 kHz FIXED |  | P |
| 18052 - 18068 kHz FIXED Space Research | 18052 - 18068 kHz FIXED Space Research |  | P |
| 18068 - 18168 kHz AMATEUR AMATEUR-SATELLITE S5.154 | 18068 - 18168 kHz AMATEUR AMATEUR-SATELLITE FIXED | S5.154 | NG |
| 18168 - 18780 kHz FIXED Mobile except aeronautical mobile | 18168 - 18780 kHz FIXED Mobile except aeronautical mobile |  | P |
| 18780 - 18900 kHz MARITIME MOBILE | 18780 - 18900 kHz MARITIME MOBILE | RN009 | P |
| 18900 - 19020 kHz BROADCASTING S5.134, S5.146 | 18900 - 19020 kHz BROADCASTING | S5.134, S5.146 RN006 | P |
| 19020 - 19680 kHz FIXED | 19020 - 19680 kHz FIXED |  | P |
| 19680 - 19800 kHz MARITIME MOBILE S5.132 | 19680 - 19800 kHz MARITIME MOBILE | S5.132 RN009 | P |
| 19800 - 19990 kHz FIXED | 19800 - 19990 kHz FIXED |  | P |
| 19990 - 19995 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research S5.111 | 19990 - 19995 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 19995 - 20010 kHz STANDARD FREQUENCY AND TIME SIGNAL (20 000 kHz) S5.111 | 19995 - 20010 kHz STANDARD FREQUENCY AND TIME SIGNAL (20 000 kHz) |  | P |
| 20010 - 21000 kHz FIXED Mobile | 20010 - 21000 kHz FIXED Mobile |  | P |
| 21000 - 21450 kHz AMATEUR AMATEUR-SATELLITE | 21000 - 21450 kHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 21450 - 21850 kHz BROADCASTING | 21450 - 21850 kHz BROADCASTING |  | NG |
| 21850 – 21870 kHz FIXED S5.155, S5.155A | 21850 - 21870 kHz FIXED AERONAUTICAL MOBILE (R) | S5.155, S5.155A | P |
| 21870 – 21924 kHz FIXED S5.155B | 21870 - 21924 kHz FIXED | S5.155B | P |
| 21924 – 22000 kHz AERONAUTICAL MOBILE (R) | 21924 - 22000 kHz AERONAUTICAL MOBILE (R) |  | P |
| 22000 – 22855 kHz MARITIME MOBILE S5.132, S5.156 | 22000 - 22855 kHz MARITIME MOBILE | S5.132 RN010 | P |
| 22855 – 23000 kHz FIXED S5.156 | 22855 - 23000 kHz FIXED |  | P |
| 23000 – 23200 kHz FIXED Mobile except aeronautical mobile (R) S5.156 | 23000 - 23200 kHz FIXED Mobile except aeronautical mobile (R) |  | P |
| 23200 – 23350 kHz FIXED S5.156A AERONAUTICAL MOBILE (OR) | 23200 - 23350 kHz FIXED AERONAUTICAL MOBILE (OR) | S5.156A RN011 | P |
| 23350 – 24000 kHz FIXED MOBILE except aeronautical mobile S5.157 | 23350 - 24000 kHz FIXED MOBILE except aeronautical mobile | S5.157 | P |
| 24000 – 24890 kHz FIXED LAND MOBILE | 24000 - 24890 kHz FIXED LAND MOBILE |  | P |
| 24890 – 24990 kHz AMATEUR AMATEUR-SATELLITE | 24890 - 24990 kHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 24990 – 25005 kHz STANDARD FREQUENCY AND TIME SIGNAL (25 000 kHz) | 24990 - 25005 kHz STANDARD FREQUENCY AND TIME SIGNAL (25 000 kHz) |  | P |
| 25005 – 25010 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research | 25005 - 25010 kHz STANDARD FREQUENCY AND TIME SIGNAL Space Research |  | P |
| 25010 – 25070 kHz FIXED MOBILE except aeronautical mobile | 25010 - 25070 kHz FIXED MOBILE except aeronautical mobile |  | P |
| 25070 – 25210 kHz MARITIME MOBILE | 25070 - 25210 kHz MARITIME MOBILE | RN010 | P |
| 25210 – 25550 kHz FIXED MOBILE except aeronautical mobile | 25210 - 25550 kHz FIXED MOBILE except aeronautical mobile |  | P |
| 25550 – 25670 kHz RADIO ASTRONOMY S5.149 | 25550 - 25670 kHz RADIO ASTRONOMY | S5.149 | NG |
| 25670 – 26100 kHz BROADCASTING | 25670 - 26100 kHz BROADCASTING |  | ###### NG |
| 26100 – 26175 kHz MARITIME MOBILE S5.132 | 26100 - 26175 kHz MARITIME MOBILE | S5.132 RN010 | P |
| 26175 – 27500 kHz FIXED MOBILE except aeronautical mobile S5.150 | 26175 - 27500 kHz FIXED MOBILE except aeronautical mobile | S5.150 RN012, RN013, RN035 | P |
| 27.5 - 28 MHz METEOROLOGICAL AIDS FIXED MOBILE | 27.5 - 28 MHz METEOROLOGICAL AIDS FIXED MOBILE |  | P |
| 28 - 29.7 MHz AMATEUR AMATEUR-SATELLITE | 28 - 29.7 MHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 29.7 – 30.005 MHz FIXED MOBILE | 29.7 – 30.005 MHz FIXED MOBILE | RN035 | P |
| 30.005 – 30.01 MHz SPACE OPERATION (satellite identification) FIXED MOBILE SPACE RESEARCH | 30.005 – 30.01 MHz SPACE OPERATION (satellite identification) FIXED MOBILE SPACE RESEARCH | RN035 | P |
| 30.01 – 37.5 MHz FIXED MOBILE | 30.01 – 37.5 MHz FIXED MOBILE | RN015, RN035, RN038 | P |
| 37.5 – 38.25 MHz FIXED MOBILE Radio Astronomy S5.149 | 37.5 – 38.25 MHz FIXED MOBILE Radio Astronomy | S5.149 RN015 RN035 | P |
| 38.25 – 39.986 MHz FIXED MOBILE | 38.25 – 39.986 MHz FIXED MOBILE | RN015, RN035 RN038 | P |
| 39.986 – 40.02 MHz FIXED MOBILE Space Research | 39.986 – 40.02 MHz FIXED MOBILE Space Research | RN035, RN038 | P |
| 40.02 – 40.98 MHz FIXED MOBILE S5.150 | 40.02 – 40.98 MHz FIXED MOBILE | S5.150 RN035 | P |
| 40.98 - 41.015 MHz FIXED MOBILE Space Research S5.160, S5.161 | 40.98 - 41.015 MHz FIXED MOBILE Space Research | RN035 | P |
| 41.015 - 44 MHz FIXED MOBILE S5.160, S5.161 | 41.015 - 44 MHz FIXED MOBILE | RN035 | P |
| 44 - 47 MHz FIXED MOBILE S5.162, S5.162A | 44 - 47 MHz FIXED MOBILE | S5.162A RN035 | P |
| 47 - 68 MHz BROADCASTING S5.162A, S5.163, S5.164 S5.165, S5.169, S5.171 | 47 - 48.5 MHz BROADCASTING Land Mobile Fixed 48.5 - 56.5 MHz BROADCASTING 56.5 - 58 MHz BROADCASTING Fixed Land Mobile 58 - 68 MHz BROADCASTING | S5.162A, S5.163 RN014, RN017 | P NG P NG |
| 68 - 74.8 MHz FIXED MOBILE except aeronautical mobile S5.149, S5.174, S5.175, S5.177, S5.179 | 68 - 73 MHz BROADCASTING 73 – 74 MHz BROADCASTING FIXED MOBILE 74-74.6 MHz FIXED MOBILE 74.6 - 74.8 MHz FIXED MOBILE AERONAUTICAL RADIONAVIGATION | S5.149, S5.175 S5.177, S5.179 RN017 | P |
| 74.8 - 75.2 MHz AERONAUTICAL RADIONAVIGATION S5.180, S5.181 | 74.8 - 75.2 MHz AERONAUTICAL RADIONAVIGATION | S5.180 | P |
| 75.2 - 87.5 MHz FIXED MOBILE except aeronautical mobile S5.175, S5.179, S5.184, S5.187 | 75.2 - 75.4 MHz FIXED MOBILE AERONAUTICAL RADIONAVIGATION 75.4 - 76 MHz FIXED MOBILE 76 - 87.5 MHz BROADCASTING | S5.175, S5.179 RN014, RN016 | P |
| 87.5 - 100 MHz BROADCASTING S5.190 | 87.5 - 100 MHz BROADCASTING | RN014, RN016 RN017 | P |
| 100 - 108 MHz BROADCASTING S5.192, S5.194 | 100-108 MHz BROADCASTING | RN017 | NG |
| 108 - 117.975 MHz AERONAUTICAL RADIONAVIGATION S5.197 | 108 - 117.975 MHz AERONAUTICAL RADIONAVIGATION | RN018 | G |
| 117.975 - 136 MHz AERONAUTICAL MOBILE (R) S5.111, S5.198, S5.199, S5.200, S5.201 | 117.975 - 132 MHz AERONAUTICAL MOBILE (R) Aeronautical mobile-satellite (R) 132-136 MHz AERONAUTICAL MOBILE Aeronautical mobile-satellite (R) | S5.198, S5.199, S5.200, S5.201 RN018 | P |
| 136 - 137 MHz AERONAUTICAL MOBILE (R) S5.202, S5.203, S5.203A, S5.203B | 136 - 137 MHz AERONAUTICAL MOBILE | S5.202, S5.203 RN018 | P |
| 137 - 137.025 MHz SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) S5.209, S5.208A Fixed Mobile except aeronautical mobile (R) S5.204, S5.205, S5.206, S5.207, S5. 208 | 137 - 137.025 MHz AERONAUTICAL MOBILE (OR) SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile (R) | S5.206, S5.208 S5.208A, S5.209 RN018 | P |
| 137.025 - 137.175 MHz SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) S5.209,S5.208A Fixed Mobile except aeronautical mobile (R) S5.204, S5.205, S5.206, S5.207, S5.208 | 137.025 - 137.175 MHz AERONAUTICAL MOBILE (OR) SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) Fixed Mobile except aeronautical mobile (R) | S5.206, S5.208 S5.208A, S5.209 RN018 | P |
| 137.175 - 137.825 MHz SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) S5.209, S5.208A Fixed Mobile except aeronautical mobile (R) S5.204, S5.205, S5.206, S5.207, S5.208 | 137.175 - 137.825 MHz AERONAUTICAL MOBILE (OR) SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile (R) | S5.206, S5.208 S5.208A, S5.209 RN018 | P |
| 137.825 – 138 MHz SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) S5.209, S5.208A Fixed Mobile except aeronautical mobile (R) S5.204, S5.205, S5.206, S5.207, S5.208 | 137.825 - 138 MHz AERONAUTICAL MOBILE (OR) SPACE OPERATION (space-to-Earth) SPACE RESEARCH (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) Fixed Mobile except aeronautical mobile (R) | S5.206, S5.208 S5.208A, S5.209 RN018 | P |
| 138 - 143.6 MHz AERONAUTICAL MOBILE (OR) S5.210, S5.211, S5.212, S5.214 | 138 - 143.6 MHz AERONAUTICAL MOBILE (OR) | RN018, RN035 | G |
| 143.6 - 143.65 MHz AERONAUTICAL MOBILE (OR) SPACE RESEARCH (space-to-Earth) S5.211, S5.212, S5.214 | 143.6 - 143.65 MHz AERONAUTICAL MOBILE (OR) SPACE RESEARCH (space-to-Earth) | RN018 | G |
| 143.65 - 144 MHz AERONAUTICAL MOBILE (OR) S5.210 - 212, S5.214 | 143.65 - 144 MHz AERONAUTICAL MOBILE (OR) | RN018 | G |
| 144 - 146 MHz AMATEUR AMATEUR-SATELLITE S5.216 | 144 - 146 MHz AMATEUR AMATEUR-SATELLITE | RN018 | NG |
| 146 - 148 MHz FIXED MOBILE except aeronautical mobile (R) | 146 - 148 MHz FIXED MOBILE except aeronautical mobile (R) | RN018 | G |
| 148 – 149.9 MHz FIXED MOBILE except aeronautical mobile (R) MOBILE-SATELLITE (Earth-to-space) S5.209 S5.218, S5.219, S5.221 | 148 - 149.9 MHz FIXED MOBILE except aeronautical mobile (R) MOBILE-SATELLITE (Earth-to-space) | S5.209, S5.218, S5.219, S5.221 RN018 | G |
| 149.9 – 150.05 MHz RADIONAVIGATION- SATELLITE S5.224B MOBILE-SATELLITE (Earth-to-space) S5.209, S5.224A | 149.9 - 150.05 MHz RADIONAVIGATION- SATELLITE MOBILE-SATELLITE (Earth-to-space) | S5.209, S5.220, S5.222, S5.223, S5.224A, S5.224B RN018 | P |
| 150.05 - 153 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY S5.149 | 150.05 - 153 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY | S5.149 RN018, RN019 | P |
| 153 - 154 MHz FIXED MOBILE except aeronautical mobile (R) METEOROLOGICAL Aids | 153 - 154 MHz FIXED MOBILE except aeronautical mobile (R) METEOROLOGICAL Aids | RN018, RN019 | P |
| 154 - 156.7625 MHz FIXED MOBILE except aeronautical mobile (R) S5.226, S5.227 | 154 - 156.7625 MHz FIXED MOBILE except aeronautical mobile (R) | S5.226, S5.227 RN018 RN019 | P |
| 156.7625 – 156.8375 MHz MARITIME MOBILE (distress and calling) S5.111, S5.226 | 156.7625 - 156.8375 MHz MARITIME MOBILE (distress and calling) FIXED MOBILE except aeronautical mobile | S5.226 RN018 | P |
| 156.8375 – 174 MHz FIXED MOBILE except aeronautical mobile S5.226, S5.229 | 156.8375 - 162.7625 MHz FIXED MOBILE except aeronautical mobile 162.7625 – 163.2 MHz FIXED MOBILE except aeronautical mobile 163.2 - 168.5 MHz FIXED MOBILE except aeronautical mobile 168.5 - 174 MHz FIXED MOBILE except aeronautical mobile | S5.226 RN018, RN019, RN020, RN021 RN035 | P G P G |
| 174 - 223 MHz BROADCASTING S5.235, S5.237, S5.243 | 174 - 223 MHz BROADCASTING | RN014, RN023 RN035 | P |
| 223 - 230 MHz BROADCASTING Fixed Mobile S5.243, S5.246, S5.247 | 223 - 230 MHz BROADCASTING Fixed Mobile | RN014, RN023 | P |
| 230 - 235 MHz FIXED MOBILE S5.247, S5.251, S5.252 | 230 - 235 MHz FIXED MOBILE | RN018, RN023 | P |
| 235 - 267 MHz FIXED MOBILE S5.111,S5.199, S5.252,S5.254, S5.256 | 235 -240 MHz FIXED MOBILE 240 – 267 MHz FIXED MOBILE | S5.199, S5.254, S5.256 RN018, RN023 | P G |
| 267 - 272 MHz FIXED MOBILE Space Operation (space–to-Earth) S5.254, S5.257 | 267 - 272 MHz FIXED MOBILE Space Operation (space-to-Earth) | S5.254, S5.257 RN018 | G |
| 272 - 273 MHz SPACE OPERATION (space–to-Earth) FIXED MOBILE S5.254 | 272 - 273 MHz FIXED MOBILE | S5.254 RN018 | G |
| 273 - 312 MHz FIXED MOBILE S5.254 | 273 - 312 MHz FIXED MOBILE | S5.254 RN018, RN024 | P |
| 312 - 315 MHz FIXED MOBILE Mobile-satellite (Earth-to-space) S5.254 S5.255 | 312 - 315 MHz FIXED MOBILE Mobile-satellite (Earth-to-space) | S5.254, S5.255 RN018 | G |
| 315 - 322 MHz FIXED MOBILE S5.254 | 315 - 322 MHz FIXED MOBILE | S5.254 RN018 | G |
| 322 - 328.6 MHz FIXED MOBILE RADIO ASTRONOMY S5.149 | 322 - 328.6 MHz FIXED MOBILE RADIO ASTRONOMY | S5.149 RN018 | P |
| 328.6 - 335.4 MHz AERONAUTICAL RADIONAVIGATION S5.258, S5.259 | 328.6 - 335.4 MHz AERONAUTICAL RADIONAVIGATION | S5.258 RN018 | G |
| 335.4 - 387 MHz FIXED MOBILE S5.254 | 335.4 - 387 MHz FIXED MOBILE | S5.254 RN018, RN024, RN025 | P |
| 387 - 390 MHz FIXED MOBILE Mobile-satellite (space-to-Earth) S5.254, S5.255, S5.208A | 387 - 390 MHz FIXED MOBILE Mobile-satellite (space-to-Earth) | S5.254, S5.255, S5.208A RN018 | G |
| 390 - 399.9 MHz FIXED MOBILE S5.254 | 390 - 399.9 MHz FIXED MOBILE | S5.254 RN018, RN025, RN026 | P |
| 399.9 - 400.05 MHz RADIONAVIGATION - SATELLITE S5.222, S5.224B, S5.260 MOBILE-SATELLITE (Earth-to-space) S5.209, S5.224A S5.220 | 399.9 - 400.05 MHz RADIONAVIGATION - SATELLITE MOBILE-SATELLITE (Earth-to-space) | S5.209, S5.220, S5.222, S5.223, S5.224A, S5.224B, S5.260 RN018, RN026 | P |
| 400.05 - 400.15 MHz STANDARD FREQUENCY AND TIME SIGNAL-SATELLITE (400.1 MHz) S5.261, S5.262 | 400.05 - 400.15 MHz STANDARD FREQUENCY AND TIME SIGNAL-SATELLITE (400.1 MHz) FIXED MOBILE | S5.261, S5.262 RN018, RN026 | P |
| 400.15 - 401 MHz METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) S5.263 MOBILE-SATELLITE (space-to-Earth) S5.209, S5.208A Space Operation (space-to-Earth) S5.262, S5.264 | 400.15 - 401 MHz METEOROLOGICAL AIDS METEOROLOGICAL-SATELLITE (space-to-Earth) SPACE RESEARCH (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Space Operation (space-to-Earth) FIXED MOBILE | S5.208A, S5.209 S5.262, S5.263 S5.264 RN018, RN026 | P |
| 401 - 402 MHz METEOROLOGICAL AIDS SPACE OPERATION (space-to-Earth) EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) Fixed Mobile except aeronautical mobile | 401 - 402 MHz METEOROLOGICAL AIDS SPACE OPERATION (space-to-Earth) EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) Fixed Mobile except aeronautical mobile | RN018, RN026, | P |
| 402 - 403 MHz METEOROLOGICAL AIDS EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) Fixed Mobile except aeronautical mobile | 402 - 403 MHz METEOROLOGICAL AIDS EARTH EXPLORATION-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) Fixed Mobile except aeronautical mobile | RN018, RN026, RN035 | P |
| 403 - 406 MHz METEOROLOGICAL AIDS Fixed Mobile except aeronautical mobile | 403 - 406 MHz METEOROLOGICAL AIDS Fixed Mobile except aeronautical mobile | RN018, RN026, RN035 | P |
| 406 - 406.1 MHz MOBILE-SATELLITE (Earth-to-space) S5.266, S5.267 | 406 - 406.1 MHz MOBILE-SATELLITE (Earth-to-space) | S5.266 S5.267 RN018 | P |
| 406.1 - 410 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY S5.149 | 406.1 - 410 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY | S5.149 RN018, RN026, RN027 | P |
| 410 - 420 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) S5.268 | 410 - 420 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) | S5.268 RN018, RN028 | P |
| 420 - 430 MHz FIXED MOBILE except aeronautical mobile Radiolocation S5.269, S5.270, S5.271 | 420 - 430 MHz FIXED MOBILE except aeronautical mobile Radiolocation | RN018, RN028 | P |
| 430 - 440 MHz AMATEUR RADIOLOCATION S5.138, S5.271, S5.272, S5.273, S5.274, S5.275, S5.276, S5.277, S5.280, S5.281, S5.282, S5.283 | 430 - 440 MHz AMATEUR FIXED RADIOLOCATION | S5.138, S5.277 RN018, RN026, RN035 | P |
| 440 – 450 MHz FIXED MOBILE except aeronautical mobile Radiolocation S5.269, S5.270, S5.271, S5.284, S5.285, S5.286 | 440 - 450 MHz FIXED MOBILE except aeronautical mobile Radiolocation | S5.286 RN018, RN026 | G |
| 450 - 455 MHz FIXED MOBILE S5.209, S5.271, S5.286 S5.286A, S5.286B, S5.286C, S5.286D, S5.286E | 450 - 455 MHz FIXED MOBILE | S5.286A, S5.286 RN018, RN029 | P |
| 455 - 456 MHz FIXED MOBILE S5.209, S5.271, S5.286A, S5.286B, S5.286C, S5.286E | 455 - 456 MHz FIXED MOBILE | S5.209, S5.286A RN018, RN029 | NG |
| 456 - 459 MHz FIXED MOBILE S5.271, S5.287, S5.288 | 456 - 459 MHz FIXED MOBILE | S5.287 RN018, RN029, RN030 | P |
| 459 - 460 MHz FIXED MOBILE S5.271, S5.286B | 459 - 460 MHz FIXED MOBILE | RN018 | G |
| 460 - 470 MHz FIXED MOBILE Meteorological-satellite (space-to-Earth) S5.287, S5.288, S5.289, S5.290 | 460 - 470 MHz FIXED MOBILE Meteorological-satellite (space-to-Earth) | S5.287, S5.289 RN018, RN029, RN030 | P |
| 470 - 790 MHz BROADCASTING S5.149, S5.291A, S5.294, S5.296, S5.300, S5.302, S5.304, S5.306, S5.311, S5.312 | 470-645 MHz BROADCASTING 645-790 MHz BROADCASTING AERONAUTICAL RADIONAVIGATION | S5.149, S5.306, S5.311, S5.312 RN014, RN018, RN031, RN035 | NG P |
| 790 - 862 MHz FIXED BROADCASTING S5.312, S5.314, S5.315, S5.316, S5.319, S5.321 | 790 - 862 MHz BROADCASTING AERONAUTICAL RADIONAVIGATION FIXED Land mobile | S5.312, S5.314 RN014, RN018, RN032, RN033, RN035 | P |
| 862 - 890 MHz FIXED MOBILE except aeronautical mobile S5.317A BROADCASTING S5.322 S5.319, S5.323 | 862 - 890 MHz FIXED MOBILE except aeronautical mobile AERONAUTICAL RADIONAVIGATION | S5.323, S5.317A, RN018, RN032, RN033, RN034, RN035, RN036 | P |
| 890 – 942 MHz FIXED MOBILE except aeronautical mobile S5.317A BROADCASTING S5.322 Radiolocation S5.323 | 890 - 942 MHz FIXED MOBILE except aeronautical mobile AERONAUTICAL RADIONAVIGATION Radiolocation | S5.323, S5.317A, RN018, RN036, RN037, RN038 | P |
| 942 – 960 MHz FIXED MOBILE except aeronautical mobile S5.317A BROADCASTING S5.322 S5.323 | 942 - 960 MHz FIXED MOBILE except aeronautical mobile AERONAUTICAL RADIONAVIGATION | S5.323, S5.317A, RN018, RN036, RN037, RN038 | P |
| 960 - 1215 MHz AERONAUTICAL RADIONAVIGATION S5.328 S5.328A | 960 - 1215 MHz AERONAUTICAL RADIONAVIGATION | S5.328, S5.328A RN018, RN039 | P |
| 1215 – 1240 MHz RADIOLOCATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) S5.329, S5.329A EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) S5.330, S5.331, S5.332 | 1215 – 1240 MHz RADIOLOCATION RADIONAVIGATION- SATELLITE (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) | S5.329, S5.329A S5.332 | P |
| 1240 – 1260 MHz RADIOLOCATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) S5.329, S5.329A EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) Amateur S5.330, S5.331, S5.332, S5.334, S5.335 | 1240 - 1260 MHz RADIOLOCATION RADIONAVIGATION- SATELLITE (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) Amateur | S5.329, S5.329A S5.332 | P |
| 1260 – 1300 MHz RADIOLOCATION EARTH EXPLORATION-SATELLITE (active) RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) S5.329, S5.329A SPACE RESEARCH (active) Amateur S5.282, S5.330, S5.331, S5.332, S5.333, S5.334, S5.335 | 1260 - 1300 MHz RADIOLOCATION EARTH EXPLORATION-SATELLITE (active) RADIONAVIGATION-SATELLITE (space-to-Earth) (space-to-space) SPACE RESEARCH (active) Amateur | S5.329, S5.329A S5.332, S5.333, S5.335A | P |
| 1300 – 1350 MHz AERONAUTICAL RADIONAVIGATION S5.337 RADIONAVIGATION-SATELLITE (Earth-to-space) Radiolocation S5.149, S5.337A | 1300 - 1350 MHz AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (Earth-to-space) Radiolocation | S5.149, S5.337 S5.337A | P |
| 1350 – 1400 MHz FIXED MOBILE RADIOLOCATION S5.149, S5.338, S5.339 | 1350 - 1400 MHz FIXED MOBILE RADIOLOCATION | S5.149, S5.339 RN039A | P |
| 1400 – 1427 MHz EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 1400 - 1427 MHz EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.341 | P |
| 1427 – 1429 MHz SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical mobile S5.341 | 1427 - 1429 MHz SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical mobile | S5.341 RN039A | P |
| 1429 – 1452 MHz FIXED MOBILE except aeronautical mobile S5.341, S5.342 | 1429 - 1452 MHz FIXED MOBILE except aeronautical mobile | S5.341 RN039A | P |
| 1452 – 1492 MHz FIXED MOBILE except aeronautical mobile BROADCASTING S5.345, S5.347 BROADCASTING- SATELLITE S5.345, S5.347 S5.341, S5.342 | 1452 - 1492 MHz FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING- SATELLITE | S5.341, S5.345 | P |
| 1492 - 1525 MHz FIXED MOBILE except aeronautical mobile S5.341 S5.342 | 1492 - 1525 MHz FIXED MOBILE except aeronautical mobile | S5.341 RN039A | P |
| 1525 - 1530 MHz SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) S5.351A Earth Exploration-Satellite Mobile except aeronautical mobile S5.349 S5.341, S5.342, S5.350, S5.351, S5.352A, S5.354 | 1525 - 1530 MHz SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) Earth Exploration-Satellite Mobile except aeronautical mobile | S5.341, S5.351, S5.354, S5.351A | P |
| 1530 – 1535 MHz SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) S5.353A S5.351A Earth Exploration-Satellite Fixed Mobile except aeronautical mobile S5.341, S5.342, S5.351, S5.354 | 1530 - 1535 MHz SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Earth Exploration-Satellite Fixed Mobile except aeronautical mobile | S5.341, S5.351, S5.353A, S5.354, S5.351A | P |
| 1535 – 1559 MHz MOBILE-SATELLITE (space-to-Earth) S5.351A S5.341, S5.351, S5.353A, S5.354, S5.355 S5.356, S5.357, S5.357A, S5.359, S5.362A | 1535 - 1559MHz MOBILE-SATELLITE (space-to-Earth) | S5.341, S5.351, S5.351A, S5.353A, S5.354, S5.356, S5.357, S5.357A, S5.359 RN040 | P |
| 1559 – 1610 MHz AERONAUTICAL RADIONAVIGATION RADIONAVIGATION- SATELLITE (space-to-Earth) (space-to-space) S5.329A S5.341, S5.362C, S5.362B, S5.363 | 1559 - 1610 MHz FIXED AERONAUTICAL RADIONAVIGATION RADIONAVIGATION- SATELLITE (space-to-Earth) (space-to-space) | S5.329A, S5.341, S5.362B RN041, RN042 | P |
| 1610 – 1610.6 MHz MOBILE-SATELLITE (Earth-to-space) S5.351A AERONAUTICAL RADIONAVIGATION S5.341, S5.355, S5.359, S5.363, S5.364, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1610 - 1610.6 MHz FIXED MOBILE-SATELLITE (Earth-to-space) AERONAUTICAL RADIONAVIGATION | S5.351A S5.341, S5.359, S5.364, S5.366, S5.367, S5.368, S5.371, S5.372 RN042, RN043 | P |
| 1610.6 - 1613.8 MHz MOBILE-SATELLITE (Earth-to-space) S5.351A RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION S5.149, S5.341, S5.355, S5.359, S5.363, S5.364, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1610.6 - 1613.8 MHz FIXED MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY AERONAUTICAL RADIONAVIGATION | S5.351A, S5.149, S5.341, S5.359, S5.364, S5.366, S5.367, S5.368, S5.371, S5.372 RN042, RN043 | P |
| 1613.8 - 1626.5 MHz MOBILE-SATELLITE (Earth-to-space) S5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) S5.341, S5.355, S5.359, S5.363, S5.364, S5.365, S5.366, S5.367, S5.368, S5.369, S5.371, S5.372 | 1613.8 - 1626.5 MHz FIXED MOBILE-SATELLITE (Earth-to-space) AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) | S5.351A, S5.341, S5.359, S5.364, S5.365, S5.366, S5.367, S5.368, S5.371, S5.372 RN042, RN043 | P |
| 1626.5 - 1660 MHz MOBILE-SATELLITE (Earth-to-space) S5.351A S5.341, S5.351, S5.353A, S5.354, S5.355, S5.357A, S5.359, S5.362A, S5.374, S5.375, S5.376 | 1626.5 - 1660 MHz FIXED MOBILE-SATELLITE (Earth-to-space) | S5.351A, S5.341, S5.351, S5.353A, S5.354, S5.357A, S5.359 RN040 | P |
| 1660 - 1660.5 MHz MOBILE-SATELLITE (Earth-to-space) S5.351A RADIO ASTRONOMY S5.149, S5.341, S5.351, S5.354, S5.362A, S5.376A | 1660 - 1660.5 MHz MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY | S5.351A, S5.149, S5.341, S5.351, S5.354, S5.376A | P |
| 1660.5 - 1668.4 MHz RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile S5.149, S5.341, S5.379, S5.379A | 1660.5 - 1668.4 MHz RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile | S5.149, S5.341, S5.379A | P |
| 1668.4 - 1670 MHz METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY S5.149, S5.341 | 1668.4 - 1670 MHz METEOROLOGICAL AIDS FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY | S5.149, S5.341 | P |
| 1670 – 1675 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE S5.380 S5.341 | 1670 - 1675 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE | S5.341, S5.380 RN044 | P |
| 1675 – 1690 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile S5.341 | 1675 - 1690 MHz METEOROLOGICAL AIDS FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | S5.341 | P |
| 1690 - 1700 MHz METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) Fixed Mobile except aeronautical mobile S5.289, S5.341, S5.382 | 1690 – 1700 MHz METEOROLOGICAL AIDS METEOROLOGICAL- SATELLITE (space-to-Earth) FIXED MOBILE except aeronautical mobile | S5.289, S5.341, S5.382 | P |
| 1700 - 1710 MHz FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile S5.289, S5.341 | 1700 - 1710 MHz FIXED METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | S5.289, S5.341 | P |
| 1710 - 1930 MHz FIXED MOBILE S5.380, S5.384A, S5.388A S5.149, S5.341, S5.385, S5.386, S5.387, S5.388 | 1710 - 1770 MHz FIXED MOBILE 1770 - 1790 MHz FIXED METEOROLOGICAL- SATELLITE (Earth-to-space) (space-to-Earth) MOBILE 1790 - 1930 MHz FIXED MOBILE | S5.149, S5.341, S5.380, S5.384A, S5.388A S5.385, S5.388 RN035, RN044, RN044A, RN045, RN046 | P |
| 1930 – 1980 MHz FIXED MOBILE S5.388A S5.388 | 1930 - 1980 MHz FIXED MOBILE | S5.388, S5.388A, RN046, RN047 | P |
| 1980 – 2010 MHz FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) S5.388, S5.389A, S5.389B, S5.389F | 1980 - 2010 MHz FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) | S5.388, S5.389A RN043, RN047 | P |
| 2010 – 2025 MHz FIXED MOBILE S5.388A S5.388 | 2010 - 2025 MHz FIXED MOBILE | S5.388, S5.388A, RN047 | NG |
| 2025 – 2110 MHz SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE S5.391 SPACE RESEARCH (Earth-to-space) (space-to-space) S5.392 | 2025 - 2110 MHz SPACE OPERATION (Earth-to-space) (space-to-space) EARTH EXPLORATION-SATELLITE (Earth-to-space) (space-to-space) FIXED MOBILE SPACE RESEARCH (Earth-to-space) (space-to-space) | S5.391, S5.392 RN039A, RN047 | NG |
| 2110 - 2120 MHz FIXED MOBILE S5.388A SPACE RESEARCH (deep space) (Earth-to-space) S5.388 | 2110 - 2120 MHz FIXED MOBILE SPACE RESEARCH (deep space) (Earth-to-space) | S5.388, S5.388A | NG |
| 2120 - 2160 MHz FIXED MOBILE S5.388A S5.388 | 2120 - 2160 MHz FIXED MOBILE | S5.388, S5.388A, RN046 | NG |
| 2160 - 2170 MHz FIXED MOBILE S5.388A S5.388, S5.392A | 2160 - 2170 MHz FIXED MOBILE | S5.388, S5.388A, RN046 | NG |
| 2170 - 2200 MHz FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) S5.388, S5.389A, S5.389F, S5.392A | 2170 - 2200 MHz FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) | S5.388, S5.389A RN043 | NG |
| 2200 - 2290 MHz SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE S5.391 SPACE RESEARCH (space-to-Earth)(space-to-space) S5.392 | 2200 - 2290 MHz SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE SPACE RESEARCH (space-to-Earth) (space-to-space) | S5.391, S5.392 RN039A | NG |
| 2290 - 2300 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space) (space-to-Earth) | 2290 - 2300 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (deep space) (space-to-Earth) |  | NG |
| 2300 - 2450 MHz FIXED MOBILE Amateur Radiolocation S5.150, S5.282, S5.395 | 2300 - 2450 MHz FIXED MOBILE Amateur Radiolocation | S5.150, S5.282 RN035, RN048 | P |
| 2450 - 2483.5 MHz FIXED MOBILE Radiolocation S5.150, S5.397 | 2450 - 2483.5 MHz FIXED MOBILE Radiolocation | S5.150 RN035 | P |
| 2483.5 - 2500 MHz FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) S5.351A Radiolocation S5.150, S5.371, S5.397, S5.398, S5.399, S5.400, S5.402 | 2483.5 – 2500 MHz FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) Radiolocation | S5.150, S5.351A, S5.398, S5.399, S5.402 RN043 | NG |
| 2500 - 2520 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile S5.384A MOBILE-SATELLITE (space-to-Earth) S5.403, S5.351A S5.405, S5.407, S5.412, S5.414 | 2500 - 2520 MHz FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (space-to-Earth) | S5.351A, S5.384A, S5.403, S5.409, S5.410, S5.411, S5.414 RN048 | NG |
| 2520 - 2655 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile S5.384A BROADCASTING- SATELLITE S5.413, S5.416 S5.339, S5.403, S5.405, S5.412, S5.417, S5.418 S5.418C S5.418B | 2520-2655 MHz FIXED MOBILE except aeronautical mobile BROADCASTING- SATELLITE | S5.384A, S5.339, S5.403, S5.409, S5.410, S5.411, S5.418B, S5.418C, RN048 | NG |
| 2655 - 2670 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile S5.384A BROADCASTING- SATELLITE S5.413, S5.416 Earth Exploration-Satellite (passive) Radio Astronomy Space Research (passive) S5.149, S5.412, S5.417, S5.420 | 2655 - 2670 MHz FIXED MOBILE except aeronautical mobile BROADCASTING- SATELLITE Earth Exploration-Satellite (passive) Radio Astronomy Space Research (passive) | S5.149, S5.409, S5.410, S5.411, S5.413, S5.416, S5.420, S5.384A, RN048 | NG |
| 2670 - 2690 MHz FIXED S5.409, S5.410, S5.411 MOBILE except aeronautical mobile S5.384A MOBILE-SATELLITE (Earth-to-space) S5.351A Earth Exploration-Satellite (passive) Radio Astronomy Space Research (passive) S5.149, S5.419, S5.420 | 2670 - 2690 MHz FIXED MOBILE except aeronautical mobile MOBILE-SATELLITE (Earth-to-space) Earth Exploration-Satellite (passive) Radio Astronomy Space Research (passive) | S5.149, S5.409, S5.410, S5.411, S5.419, S5.420 S5.384A, S5.351A, RN048 | NG |
| 2690 - 2700 MHz EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.421, S5.422 | 2690 - 2700 MHz EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) FIXED | S5.340, S5.422 RN048 | NG |
| 2700 - 2900 MHz AERONAUTICAL RADIONAVIGATION S5.337 Radiolocation S5.423, S5.424 | 2700 - 2900 MHz AERONAUTICAL RADIONAVIGATION Radiolocation | S5.337, S5.423 | P |
| 2900 - 3100 MHz RADIONAVIGATION S5.426 Radiolocation S5.425, S5.427 | 2900 - 3100 MHz RADIONAVIGATION Radiolocation | S5.425, S5.426, S5.427 | P |
| 3100 – 3300 MHz RADIOLOCATION Earth exploration- satellite (active) Space research (active) S5.149, S5.428 | 3100 – 3300 MHz RADIOLOCATION Earth exploration- satellite (active) Space research (active) | S5.149 | P |
| 3300 – 3400 MHz RADIOLOCATION S5.149, S5.429, S5.430 | 3300 – 3400 MHz RADIOLOCATION | S5.149 | P |
| 3400 – 3600 MHz FIXED FIXED-SATELLITE (space-to-Earth) Mobile Radiolocation S5.431 | 3400 – 3600 MHz FIXED FIXED-SATELLITE (space-to-Earth) Mobile Radiolocation |  | NG |
| 3600 – 4200 MHz FIXED FIXED-SATELLITE (space-to-Earth) Mobile | 3600 – 4200 MHz FIXED FIXED-SATELLITE (space-to-Earth) Mobile |  | NG |
| 4200 – 4400 MHz AERONAUTICAL RADIONAVIGATION S5.438 S5.437, S5.439, S5.440 | 4200 – 4400 MHz AERONAUTICAL RADIONAVIGATION | S5.438, S5.440 | P |
| 4400 – 4500 MHz FIXED MOBILE | 4400 – 4500 MHz FIXED MOBILE |  | NG |
| 4500 – 4800 MHz FIXED FIXED-SATELLITE (space-to-Earth) S5.441 MOBILE | 4500 – 4800 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | S5.441 | NG |
| 4800 – 4990 MHz FIXED MOBILE S5.442 Radio Astronomy S5.149, S5.339, S5.443 | 4800 – 4990 MHz FIXED MOBILE Radio Astronomy | S5.149, S5.339, S5.442 | NG |
| 4990 – 5000 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY Space Research (passive) S5.149 | 4990 – 5000 MHz FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY Space Research (passive) | S5.149 | NG |
| 5000 – 5150 MHz AERONAUTICAL RADIONAVIGATION S5.367, S5.444, S5.444A, S5.443A, S5.443B | 5000 – 5150 MHz AERONAUTICAL RADIONAVIGATION | S5.367, S5.444, S5.444A, S5.443A, S5.443B | P |
| 5150 – 5250 MHz AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) S5.447A S5.446, S5.447, S5.447B, S5.447C | 5150 – 5250 MHz AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) | S5.446, S5.447A, S5.447B, S5.447C RN035 | P |
| 5250 – 5255 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH S5.447D S5.448, S5.448A | 5250 – 5255 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH | S5.447A, S5.447B, S5.447C, S5.447D, S5.448A RN035 | P |
| 5255 – 5350 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) S5.448, S5.448A | 5255 – 5350 MHz EARTH EXPLORATION- SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) | S5.448A RN035 | P |
| 5350 – 5460 MHz ###### AERONAUTICAL RADIONAVIGATION S5.449 EARTH EXPLORATION-SATELLITE (active) S5.448B ###### Radiolocation | 5350 – 5460 MHz ###### AERONAUTICAL RADIONAVIGATION ###### EARTH EXPLORATION- SATELLITE (active) ###### Radiolocation | S5.448B, S5.449 | P |
| 5460 – 5470 MHz RADIONAVIGATION S5.449 ###### Radiolocation | 5460 – 5470 MHz RADIONAVIGATION ###### Radiolocation | S5.449 | P |
| 5470 – 5650 MHz ###### MARITIME RADIONAVIGATION ###### Radiolocation S5.450, S5.451, S5.452 | 5470 – 5650 MHz ###### MARITIME ###### RADIONAVIGATION ###### Radiolocation | S5.452 RN035 | NG |
| 5650 – 5725 MHz RADIOLOCATION Amateur Space Research (deep space) S5.282, S5.451, S5.453, S5.454, S5.455 | 5650 – 5670 MHz RADIOLOCATION Amateur Space Research (deep space) 5670 – 5725 MHz FIXED RADIOLOCATION Amateur Space Research (deep space) | S5.282, S5.455, RN035 | P NG |
| 5725 – 5830 MHz FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur S5.150, S5.451, S5.453, S5.455, S5.456 | 5725 – 5830 MHz FIXED-SATELLITE (Earth-to-space) RADIOLOCATION FIXED Amateur | S5.150, S5.455 RN035 | NG |
| 5830 – 5850 MHz FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) S5.150, S5.451, S5.453, S5.455, S5.456 | 5830 – 5850 MHz FIXED-SATELLITE (Earth-to-space) RADIOLOCATION FIXED Amateur Amateur-satellite (space-to-Earth) | S5.150, S5.455 | NG |
| 5850 – 5925 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.150 | 5850 – 5925 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.150 | NG |
| 5925 – 6700 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.149, S5.440, S5.458 | 5925 – 6700 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.149, S5.440, S5.458 | NG |
| 6700 – 7075 MHz FIXED FIXED-SATELLITE (Earth-to-space) (space-to-Earth) S5.441 MOBILE S5.458, S5.458A, S5.458B, S5.458C | 6700 – 7075 MHz FIXED FIXED-SATELLITE (Earth-to-space) (space-to-Earth) MOBILE | S5.441, S5.458, S5.458A, S5.458B, S5.458C | NG |
| 7075 – 7250 MHz FIXED MOBILE S5.458, S5.459, S5.460 | 7075 – 7250 MHz FIXED MOBILE | S5.458, S5.460 | NG |
| 7250 – 7300 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE S5.461 | 7250 – 7300 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | S5.461 | NG |
| 7300 – 7450 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile S5.461 | 7300 – 7450 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | S5.461 | NG |
| 7450 – 7550 MHz FIXED FIXED-SATELLITE (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile S5.461A | 7450 – 7550 MHz FIXED FIXED-SATELLITE (space-to-Earth) METEOROLOGICAL- SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | S5.461A | NG |
| 7550 – 7750 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | 7550 – 7750 MHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile | RN049 | NG |
| 7750 – 7850 MHz FIXED MOBILE except aeronautical mobile METEOROLOGICAL- SATELLITE (space-to-Earth) S5.461B | 7750 – 7850 MHz FIXED MOBILE except aeronautical mobile METEOROLOGICAL- SATELLITE (space-to-Earth) | S5.461B | NG |
| 7850 – 7900 MHz FIXED MOBILE except aeronautical mobile | 7850 – 7900 MHz FIXED MOBILE except aeronautical mobile |  | NG |
| 7900 – 8025 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.461 | 7900 – 8025 MHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.461 | NG |
| 8025 – 8175 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.463 S5.462A | 8025 – 8175 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.462A, S5.463 | NG |
| 8175 – 8215 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) METEOROLOGICAL- SATELLITE (Earth-to-space) MOBILE S5.463 S5.462A | 8175 – 8215 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) METEOROLOGICAL- SATELLITE (Earth-to-space) MOBILE | S5.462A, S5.463 | NG |
| 8215 – 8400 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE S5.463 S5.462A | 8215 – 8400 MHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.462A, S5.463 | . NG |
| 8400 – 8500 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) S5.465, S5.466 S5.467 | 8400 – 8500 MHz FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) | S5.465 | NG |
| 8500 – 8550 MHz RADIOLOCATION S5.468, S5.469 | 8500 – 8550 MHz RADIOLOCATION LAND MOBILE RADIONAVIGATION | S5.469 RN049 | P |
| 8550 – 8650 MHz RADIOLOCATION SPACE RESEARCH (active) EARTH EXPLORATION- SATELLITE (active) S5.468, S5.469, S5.469A | 8550 – 8650 MHz RADIOLOCATION SPACE RESEARCH (active) EARTH EXPLORATION- SATELLITE (active) LAND MOBILE RADIONAVIGATION | S5.469, S5.469A RN049 | P |
| 8650 – 8750 MHz RADIOLOCATION S5.468, S5.469 | 8650 – 8750 MHz RADIOLOCATION LAND MOBILE RADIONAVIGATION | S5.469 RN049 | P |
| 8750 – 8850 MHz RADIOLOCATION AERONAUTICAL RADIONAVIGATION S5.470 S5.471 | 8750 – 8850 MHz RADIOLOCATION AERONAUTICAL RADIONAVIGATION | S5.470 | P |
| 8850 – 9000 MHz RADIOLOCATION MARITIME RADIONAVIGATION S5.472 S5.473 | 8850 – 9000 MHz RADIOLOCATION RADIONAVIGATION | S5.472, S5.473 | P |
| 9000 – 9200 MHz RADIONAVIGATION Radiolocation S5.471 | 9000 – 9200 MHz RADIONAVIGATION Radiolocation | S5.337 | P |
| 9200 – 9300 MHz RADIOLOCATION MARITIME RADIONAVIGATION S5.472 S5.473, S5.474 | 9200 – 9300 MHz RADIOLOCATION RADIONAVIGATION | S5.472, S5.473, S5.474 RN035 | P |
| 9300 – 9500 MHz RADIONAVIGATION S5.476 Radiolocation S5.427, S5.474, S5.475 | 9300 – 9500 MHz RADIONAVIGATION Radiolocation | S5.427, S5.474, S5.475, S5.476 RN035 | P |
| 9500 – 9800 MHz RADIOLOCATION RADIONAVIGATION SPACE RESEARCH (active) EARTH EXPLORATION- SATELLITE (active) S5.476A | 9500 – 9800 MHz RADIOLOCATION RADIONAVIGATION SPACE RESEARCH (active) EARTH EXPLORATION- SATELLITE (active) | S5.476A RN035 | P |
| 9800 – 10000 MHz RADIOLOCATION ###### Fixed S5.477, S5.478, S5.479 | 9800 – 10000 MHz RADIOLOCATION ###### Fixed | S5.479 RN035 | P |
| 10 – 10.45 GHz FIXED MOBILE RADIOLOCATION Amateur S5.479 | 10 – 10.45 GHz FIXED MOBILE RADIOLOCATION Amateur | S5.479 RN039A, RN050 | NG |
| 10.45 – 10.5 GHz RADIOLOCATION Amateur Amateur-satellite S5.481 | 10.45 – 10.5 GHz RADIOLOCATION Amateur Amateur-satellite | RN050 | P |
| 10.5 – 10.55 GHz FIXED MOBILE Radiolocation | 10.5 – 10.55 GHz FIXED MOBILE Radiolocation | RN035, RN039A, RN050 | NG |
| 10.55 – 10.6 GHz FIXED MOBILE except aeronautical mobile Radiolocation | 10.55 – 10.6 GHz FIXED MOBILE except aeronautical mobile Radiolocation | RN035, RN039A, RN050 | NG |
| 10.6 – 10.68 GHz EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation S5.149, S5.482 | 10.6 – 10.68 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation | S5.149, S5.482 RN039A, RN050 | NG |
| 10.68 – 10.7 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.483 | 10.68 – 10.7 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.483 | NG |
| 10.7 – 11.7 GHz FIXED FIXED-SATELLITE (space- to-Earth) S5.441, S5.484A (Earth-to-space) S5.484 MOBILE except aeronautical mobile | 10.7 – 11.7 GHz FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) MOBILE except aeronautical mobile | S5.441, S5.484, S5.484A RN051 | NG |
| 11.7 – 12.5 GHz FIXED BROADCASTING BROADCASTING- SATELLITE MOBILE except aeronautical mobile S5.487, S5.487A, S5.492 | 11.7 – 12.5 GHz FIXED BROADCASTING BROADCASTING- SATELLITE MOBILE except aeronautical mobile | S5.487, S5.487A, S5.492 | NG |
| 12.5 – 12.75 GHz FIXED-SATELLITE (space-to-Earth) S5.484A (Earth-to-space) S5.494, S5.495, S5.496 | 12.5 – 12.75 GHz FIXED-SATELLITE (space-to-Earth) (Earth-to-space) | S5.484A | NG |
| 12.75 – 13.25 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.441 MOBILE Space Research (deep space) (space-to-Earth) | 12.75 – 13.25 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Space Research (deep space) (space-to-Earth) | S5.441 RN051, RN052 | NG |
| 13.25 – 13.4 GHz AERONAUTICAL RADIONAVIGATION S5.497 EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) S5.498A, S5.499 | 13.25 – 13.4 GHz AERONAUTICAL RADIONAVIGATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) | S5.497, S5.498A | P |
| 13.4 – 13.75 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH S5.501A Standard frequency and time signal-satellite (Earth-to-space) S5.499, S5.500, S5.501, S5.501A, S5.501B | 13.4 – 13.75 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH Standard frequency and time signal-satellite (Earth-to-space) | S5.501A, S5.501B RN035, RN053 | P |
| 13.75 – 14 GHz FIXED-SATELLITE (Earth-to-space) S5.484A RADIOLOCATION Standard frequency and time signal-satellite (Earth-to-space) Space Research S5.499, S5.500, S5.501, S5.502, S5.503, S5.503A | 13.75 – 14 GHz FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Standard frequency and time signal-satellite (Earth-to-space) Space Research | S5.484A, S5.502, S5.503, S5.503A, RN035 | NG |
| 14 – 14.25 GHz FIXED-SATELLITE (Earth-to-space) S5.484A, S5.506 RADIONAVIGATION S5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research S5.505 | 14 – 14.25 GHz FIXED-SATELLITE (Earth-to-space) RADIONAVIGATION Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research | S5.484A, S5.504 | NG |
| 14.25 – 14.3 GHz FIXED-SATELLITE (Earth-to-space) S5.484A, S5.506 RADIONAVIGATION S5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research S5.505, S5.508, S5.509 | 14.25 – 14.3 GHz FIXED-SATELLITE (Earth-to-space) RADIONAVIGATION Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research | S5.484A, S5.504 | NG |
| 14.3 – 14.4 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.484A, S5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radionavigation-satellite | 14.3 – 14.4 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radionavigation-satellite | S5.484A | NG |
| 14.4 – 14.47 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.484A, S5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research (space-to-Earth) | 14.4 – 14.47 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space Research (space-to-Earth) | S5.484A RN051 | NG |
| 14.47 – 14.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.484A, S5.506 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radio Astronomy S5.149 | 14.47 – 14.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radio Astronomy | S5.149, S5.484A RN051 | NG |
| 14.5 – 14.8 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.510 MOBILE Space Research | 14.5 – 14.8 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Space Research | RN051 | NG |
| 14.8 – 15.35 GHz FIXED MOBILE Space Research S5.339 | 14.8 – 15.35 GHz FIXED MOBILE Space Research | S5.339 RN051 | NG |
| 15.35 – 15.4 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.511 | 15.35 – 15.4 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | NG |
| 15.4 – 15.43 GHz AERONAUTICAL RADIONAVIGATION S5.511D | 15.4 - 15.43 GHz AERONAUTICAL RADIONAVIGATION | S5.511D | P |
| 15.43 – 15.63 GHz FIXED-SATELLITE (space-to-Earth) (Earth-to-space) S5.511A AERONAUTICAL RADIONAVIGATION S5.511C | 15.43 - 15.63 GHz FIXED-SATELLITE (space-to-Earth) (Earth-to-space) AERONAUTICAL RADIONAVIGATION | S5.511A, S5.511C | P |
| 15.63 - 15.7 GHz AERONAUTICAL RADIONAVIGATION S5.511D | 15.63 - 15.7 GHz AERONAUTICAL RADIONAVIGATION | S5.511D | P |
| 15.7 – 16.6 GHz RADIOLOCATION S5.512, S5.513 | 15.7 - 16.6 GHz RADIOLOCATION |  | P |
| 16.6 – 17.1 GHz RADIOLOCATION Space Research (deep space) (Earth-to-space) S5.512, S5.513 | 16.6 - 17.1 GHz RADIOLOCATION Space Research (deep space) (Earth-to-space) |  | P |
| 17.1 – 17.2 GHz RADIOLOCATION S5.512, S5.513 | 17.1 - 17.2 GHz RADIOLOCATION | RN035 | P |
| 17.2 – 17.3 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) S5.512, S5.513, S5.513A | 17.2 - 17.3 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) | S5.513A RN035 | P |
| 17.3 - 17.7 GHz FIXED-SATELLITE (Earth-to-space) S5.516 Radiolocation S5.514 | 17.3 - 17.7 GHz FIXED-SATELLITE (Earth-to-space) Radiolocation | S5.516 | NG |
| 17.7 – 18.1 GHz FIXED FIXED-SATELLITE (space-to-Earth) S5.484A (Earth-to-space) S5.516 MOBILE | 17.7 – 18.1 GHz FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) MOBILE | S5.484A, S5.516 RN054 | NG |
| 18.1 – 18.4 GHz FIXED FIXED-SATELLITE (space-to-Earth) S5.484A (Earth-to-space) S5.520 MOBILE S5.519, S5.521 | 18.1 – 18.4 GHz FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) MOBILE | S5.484A, S5.519, S5.520 RN054 | NG |
| 18.4 – 18.6 GHz FIXED FIXED-SATELLITE (space-to-Earth) S5.484A MOBILE | 18.4 – 18.6 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | S5.484A RN054 | NG |
| 18.6 – 18.8 GHz FIXED FIXED-SATELLITE (space-to-Earth) S5.522B MOBILE except aeronautical mobile EARTH EXPLORATION-SATELLITE (passive) Space Research (passive) S5.522A, S5.522C | 18.6 – 18.8 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile EARTH EXPLORATION-SATELLITE (passive) Space Research (passive) | S5.522A, S5.522B RN054 | NG |
| 18.8 – 19.3 GHz FIXED FIXED-SATELLITE (space-to-Earth) S5.523A MOBILE | 18.8 – 19.3 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE | S5.523A RN054 | NG |
| 19.3 – 19.7 GHz FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) S5.523B, S5.523C, S5.523D, S5.523E MOBILE | 19.3 – 19.7 GHz FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) MOBILE | S5.523B, S5.523C, S5.523D, S5.523E RN054 | NG |
| 19.7 – 20.1 GHz FIXED-SATELLITE (space-to-Earth) S5.484A Mobile-satellite (space-to-Earth) S5.524 | 19.7 – 20.1 GHz FIXED-SATELLITE (space-to-Earth) Mobile-satellite (space-to-Earth) | S5.484A | NG |
| 20.1 – 20.2 GHz FIXED-SATELLITE (space-to-Earth) S5.484A MOBILE-SATELLITE (space-to-Earth) S5.524, S5.525, S5.526, S5.527, S5.528 | 20.1 – 20.2 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) | S5.484A, S5.525, S5.526, S5.527, S5.528 | NG |
| 20.2 – 21.2 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Standard frequency and time signal-satellite (space-to-Earth) S5.524 | 20.2 – 21.2 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) Standard frequency and time signal-satellite (space-to-Earth) |  | NG |
| 21.2 – 21.4 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | 21.2 – 21.4 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | RN055 | NG |
| 21.4 – 22 GHz FIXED MOBILE BROADCASTING- SATELLITE S5.530 | 21.4 – 22 GHz FIXED MOBILE BROADCASTING- SATELLITE | S5.530 RN055 | NG |
| 22 – 22.21 GHz FIXED MOBILE except aeronautical mobile S5.149 | 22 – 22.21 GHz FIXED MOBILE except aeronautical mobile | S5.149 RN055 | NG |
| 22.21 – 22.5 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) S5.149, S5.532 | 22.21 – 22.5 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.149, S5.532 RN055 | NG |
| 22.5 – 22.55 GHz FIXED MOBILE | 22.5 – 22.55 GHz FIXED MOBILE | RN055 | NG |
| 22.55 – 23.55 GHz FIXED INTER-SATELLITE MOBILE S5.149 | 22.55 – 23.55 GHz FIXED INTER-SATELLITE MOBILE | S5.149 RN055 | NG |
| 23.55 – 23.6 GHz FIXED MOBILE | 23.55 – 23.6 GHz FIXED MOBILE | RN055 | NG |
| 23.6 – 24 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 23.6 – 24 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | NG |
| 24 – 24.05 GHz AMATEUR AMATEUR-SATELLITE S5.150 | 24 – 24.05 GHz AMATEUR AMATEUR-SATELLITE | S5.150 RN035 | NG |
| 24.05 – 24.25 GHz RADIOLOCATION Amateur Earth Exploration-Satellite (active) S5.150 | 24.05 – 24.25 GHz RADIOLOCATION Amateur Earth Exploration-Satellite (active) | S5.150 RN035 | P |
| 24.25 – 24.45 GHz FIXED | 24.25 – 24.45 GHz FIXED |  | NG |
| 24.45 – 24.65 GHz FIXED INTER-SATELLITE | 24.45 – 24.65 GHz FIXED INTER-SATELLITE |  | NG |
| 24.65 – 24.75 GHz FIXED INTER-SATELLITE | 24.65 – 24.75 GHz FIXED INTER-SATELLITE |  | NG |
| 24.75 – 25.25 GHz FIXED | 24.75 – 25.25 GHz FIXED |  | NG |
| 25.25 – 25. 5 GHz FIXED INTER-SATELLITE S5.536 MOBILE Standard frequency and time signal-satellite (Earth-to-space) | 25.25 – 25. 5 GHz FIXED INTER-SATELLITE MOBILE Standard frequency and time signal-satellite (Earth-to-space) | S5.536 | NG |
| 25.5 – 27 GHz EARTH EXPLORATION- SATELLITE (space-to-Earth) S5.536A, S5.536B FIXED INTER-SATELLITE S5.536 MOBILE Standard frequency and time signal-satellite (Earth-to-space) | 25.5 – 27 GHz EARTH EXPLORATION- SATELLITE (space-to-Earth) FIXED INTER-SATELLITE MOBILE Standard frequency and time signal-satellite (Earth-to-space) | S5.536, S5.536A, S5.536B | NG |
| 27 – 27. 5 GHz FIXED INTER-SATELLITE S5.536 MOBILE | 27 – 27. 5 GHz FIXED INTER-SATELLITE MOBILE | S5.536 | NG |
| 27.5 – 28. 5 GHz FIXED S5.537A FIXED-SATELLITE (Earth-to-space) S5.484A, S5.539 MOBILE S5.538, S5.540 | 27.5 – 28. 5 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.484A, S5.538, S5.539, S5.540 | NG |
| 28.5 – 29.1 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.484A, S5.523A, S5.539 MOBILE Earth Exploration-Satellite (Earth-to-space) S5.541 S5.540 | 28.5 – 29.1 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Earth Exploration-Satellite (Earth-to-space) | S5.484A, S5.523A, S5.539, S5.540, S5.541 | NG |
| 29.1 – 29.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.523C, S5.523E, S5.535A, S5.539, S5.541A MOBILE Earth Exploration-Satellite (Earth-to-space) S5.541 S5.540 | 29.1 – 29.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Earth Exploration-Satellite (Earth-to-space) | S5.523C, S5.523E, S5.535A, S5.539, S5.540, S5.541A | NG |
| 29.5 – 29.9 GHz FIXED-SATELLITE (Earth-to-space) S5.484A, S5.539 Earth Exploration-Satellite (Earth-to-space) S5.541 Mobile-satellite (Earth-to-space) S5.540, S5.542 | 29.5 – 29.9 GHz FIXED-SATELLITE (Earth-to-space) Earth Exploration-Satellite (Earth-to-space) Mobile-satellite (Earth-to-space) | S5.484A, S5.539, S5.540, S5.541 | NG |
| 29.9 – 30 GHz FIXED-SATELLITE (Earth-to-space) S5.484A, S5.539 MOBILE-SATELLITE (Earth-to-space) Earth Exploration-Satellite (Earth-to-space) S5.541, S5.543 S5.525, S5.526, S5.527, S5.538, S5.540, S5.542 | 29.9 – 30 GHz FIXED-SATELLITE (Earth-to-space) MOBILE-SATELLITE (Earth-to-space) Earth Exploration-Satellite (Earth-to-space) | S5.484A, S5.525, S5.526, S5.527, S5.538, S5.539, S5.540, S5.543 | NG |
| 30 – 31 GHz FIXED-SATELLITE (Earth-to-space) MOBILE-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (space-to-Earth) S5.542 | 30 – 31 GHz FIXED-SATELLITE (Earth-to-space) MOBILE-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (space-to-Earth) |  | NG |
| 31 – 31.3 GHz FIXED S5.543A MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space Research S5.544, S5.545 S5.149 | 31 – 31.3 GHz FIXED MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space Research | S5.149, S5.544 | NG |
| 31.3 – 31.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 31.3 – 31.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | NG |
| 31.5 – 31.8 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile S5.149, S5.546 | 31.5 – 31.8 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) FIXED MOBILE except aeronautical mobile | S5.149, S5.546 | NG |
| 31.8 – 32 GHz RADIONAVIGATION FIXED S5.547A SPACE RESEARCH (deep space) (space-to-Earth) S5.547, S5.547B, S5.548 | 31.8 – 32 GHz RADIONAVIGATION FIXED SPACE RESEARCH (deep space) (space-to-Earth) | S5.547, S5.547A S5.548 | P |
| 32 – 32.3 GHz INTER-SATELLITE FIXED S5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) S5.547, S5.547C, S5.548 | 32 – 32.3 GHz INTER-SATELLITE FIXED RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) | S5.547, S5.548 | P |
| 32.3 – 33 GHz INTER-SATELLITE FIXED S5.547A RADIONAVIGATION S5.547, S5.547D, S5.548 | 32.3 – 33 GHz INTER-SATELLITE FIXED RADIONAVIGATION | S5.547, S5.548 | NG |
| 33 – 33.4 GHz RADIONAVIGATION FIXED S5.547A S5.547, S5.547E | 33 – 33.4 GHz RADIONAVIGATION FIXED | S5.547, S5.547A | P |
| 33.4 – 34.2 GHz RADIOLOCATION S5.549 | 33.4 – 34.2 GHz RADIOLOCATION |  | P |
| 34.2 – 34.7 GHz RADIOLOCATION SPACE RESEARCH (deep space) (Earth-to-space) S5.549 | 34.2 – 34.7 GHz RADIOLOCATION SPACE RESEARCH (deep space ) (Earth-to-space) |  | P |
| 34.7 – 35.2 GHz RADIOLOCATION Space Research S5.550 S5.549 | 34.7 – 35.2 GHz RADIOLOCATION Space Research |  | P |
| 35.2 – 35.5 GHz METEOROLOGICAL AIDS RADIOLOCATION S5.549 | 35.2 – 35.5 GHz METEOROLOGICAL AIDS RADIOLOCATION |  | P |
| 35.5 – 36 GHz EARTH EXPLORATION- SATELLITE (active) METEOROLOGICAL AIDS RADIOLOCATION SPACE RESEARCH (active) S5.549, S5.551A | 35.5 – 36 GHz EARTH EXPLORATION- SATELLITE (active) METEOROLOGICAL AIDS RADIOLOCATION SPACE RESEARCH (active) | S5.551A | NG |
| 36 – 37 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) S5.149 | 36 – 37 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | S5.149 | NG |
| 37 – 37.5 GHz FIXED MOBILE SPACE RESEARCH (space-to-Earth) S5.547 | 37 – 37.5 GHz FIXED MOBILE SPACE RESEARCH (space-to-Earth) | S5.547 | NG |
| 37.5 – 38 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (space-to-Earth) Earth Exploration-Satellite (space-to-Earth) S5.547, S5.551AA | 37.5 – 38 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE SPACE RESEARCH (space-to-Earth) Earth Exploration-Satellite (space-to-Earth) | S5.547, S5.551AA | NG |
| 38 – 39.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth Exploration-Satellite (space-to-Earth) S5.547, S5.551AA | 38 – 39.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth Exploration-Satellite (space-to-Earth) | S5.547, S5.551AA | NG |
| 39.5 – 40 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth Exploration-Satellite (space-to-Earth) S5.547, S5.551AA | 39.5 – 40 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth Exploration-Satellite (space-to-Earth) | S5.547, S5.551AA | NG |
| 40 – 40.5 GHz EARTH EXPLORATION-SATELLITE (Earth-to-space) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) SPACE RESEARCH (Earth-to-space) Earth Exploration-Satellite (space-to-Earth) | 40 – 40.5 GHz EARTH EXPLORATION- SATELLITE (Earth-to-space) FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) SPACE RESEARCH (Earth-to-space) Earth Exploration-Satellite (space-to-Earth) |  | NG |
| 40.5 – 41 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile S5.547 | 40.5 – 41 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING- SATELLITE Mobile | S5.547 RN048 | NG |
| 41 – 42 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile S5.547, S5.551G | 41 – 42 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile | S5.547, S5.551G, RN048 | NG |
| 42 – 42.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile S5.551G, S5.547, S5.551AA | 42 – 42.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) BROADCASTING BROADCASTING-SATELLITE Mobile | S5.551G, S5.547, S5.551AA, RN048 | NG |
| 42.5 – 43.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.552 MOBILE except aeronautical mobile RADIO ASTRONOMY S5.149, S5.547 | 42.5 – 43.5 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile RADIO ASTRONOMY | S5.149, S5.547, S5.552 | NG |
| 43.5 – 47 GHz MOBILE S5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE S5.554 | 43.5 – 47 GHz MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE | S5.553, S5.554 | P |
| 47 – 47.2 GHz AMATEUR AMATEUR-SATELLITE | 47 – 47.2 GHz AMATEUR AMATEUR-SATELLITE |  | NG |
| 47.2 – 50.2 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.552 MOBILE S5.149, S5.340, S5.552A, S5.555 | 47.2 – 50.2 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | S5.149, S5.340, S5.552, S5.552A, S5.555 | NG |
| 50.2 – 50.4 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) S5.340, S5.555A | 50.2 – 50.4 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) | S5.340, S5.555A | NG |
| 50.4 – 51.4 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Mobile-satellite (Earth-to-space) | 50.4 – 51.4 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Mobile-satellite (Earth-to-space) |  | NG |
| 51.4 – 52.6 GHz FIXED MOBILE S5.547, S5.556 | 51.4 – 52.6 GHz FIXED MOBILE | S5.547, S5.556 | NG |
| 52.6 – 54.25 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) S5.340, S5.556 | 52.6 – 54.25 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) | S5.340, S5.556 | NG |
| 54.25 – 55.78 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.556A SPACE RESEARCH (passive) S5.556B | 54.25 – 55.78 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.556A | NG |
| 55.78 – 56.9 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED S5.557A INTER-SATELLITE S5.556A MOBILE S5.558 SPACE RESEARCH (passive) S5.547, S5.557 | 55.78 – 56.9 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | S5.547, S5.556A S5.557A S5.558 | NG |
| 56.9 – 57 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED INTER-SATELLITE S5.558A MOBILE S5.558 SPACE RESEARCH (passive) S5.547, S5.557 | 56.9 – 57 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | S5.547, S5.558, S5.558A | NG |
| 57 – 58.2 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED INTER-SATELLITE S5.556A MOBILE S5.558 SPACE RESEARCH (passive) S5.547, S5.557 | 57 – 58.2 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED INTER-SATELLITE MOBILE SPACE RESEARCH (passive) | S5.547, S5.556A S5.558 | NG |
| 58.2 – 59 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) S5.547, S5.556 | 58.2 – 59 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) | S5.547, S5.556 | NG |
| 59 – 59.3 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) FIXED INTER-SATELLITE S5.556A MOBILE S5.558 RADIOLOCATION S5.559 | 59 – 59.3 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) FIXED INTER-SATELLITE MOBILE RADIOLOCATION | S5.556A, S5.558 S5.559 | NG |
| 59.3 – 64 GHz FIXED INTER-SATELLITE S5.556A MOBILE S5.558 RADIOLOCATION S5.559 S5.138 | 59.3 – 64 GHz FIXED INTER-SATELLITE MOBILE RADIOLOCATION | S5.138, S5.556A S5.558, S5.559 RN035 | P |
| 64 – 65 GHz FIXED INTER-SATELLITE MOBILE except aeronautical mobile S5.547, S5.556 | 64 – 65 GHz FIXED INTER-SATELLITE MOBILE except aeronautical mobile | S5.547, S5.556 | NG |
| 65 – 66 GHz EARTH EXPLORATION- SATELLITE SPACE RESEARCH INTER-SATELLITE FIXED MOBILE except aeronautical mobile S5.547 | 65 – 66 GHz EARTH EXPLORATION- SATELLITE SPACE RESEARCH INTER-SATELLITE FIXED MOBILE except aeronautical mobile | S5.547 | NG |
| 66 – 71 GHz MOBILE S5.553, S5.558 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE INTER-SATELLITE S5.554 | 66 – 71 GHz MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE INTER-SATELLITE | S5.553, S5.554 S5.558 | NG |
| 71 – 74 GHz FIXED FIXED-SATELLITE (space – to-Earth) MOBILE MOBILE-SATELLITE (space - to-Earth) | 71 – 74 GHz FIXED FIXED-SATELLITE (space - to-Earth) MOBILE MOBILE-SATELLITE (space - to-Earth) |  | NG |
| 74 – 75.5 GHz FIXED FIXED-SATELLITE (space - to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space Research (space-to-Earth) S5.561 | 74 – 75.5 GHz FIXED FIXED-SATELLITE (space - to-Earth) MOBILE BROADCASTING BROADCASTING- SATELLITE Space Research (space-to-Earth) | S5.561 | NG |
| 75.5 – 76 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space Research (space-to-Earth) S5.561, S5.559A | 75.5 – 76 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space Research (space-to-Earth) | S5.561, S5.559A | NG |
| 76 – 77.5 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space Research (space-to-Earth) S5.149 | 76 – 77.5 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space Research (space-to-Earth) | S5.149 RN035 | P |
| 77.5 – 78 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy Space Research (space-to-Earth) S5.149 | 77.5 – 78 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy Space Research (space-to-Earth) | S5.149 | P |
| 78 – 79 GHz RADIOLOCATION Amateur Amateur-satellite Radio Astronomy Space Research (space-to-Earth) S5.149, S5.560 | 78 – 79 GHz RADIOLOCATION Amateur Amateur-satellite Radio Astronomy Space Research (space-to-Earth) | S5.149, S5.560 | P |
| 79 – 81 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space Research (space-to-Earth) S5.149 | 79 – 81 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space Research (space-to-Earth) | S5.149 | P |
| 81 – 84 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space Research (space-to-Earth) S5.149, S5.560A | 81 – 84 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space Research (space-to-Earth) | S5.149, S5.560A | NG |
| 84 – 86 GHz FIXED FIXED-SATELLITE (Earth-to-space) S5.561A MOBILE RADIO ASTRONOMY S5.149 | 84 – 86 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | S5.149 | NG |
| 86 – 92 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 86 – 92 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | NG |
| 92 – 94 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION S5.149 | 92 – 94 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | S5.149 | P |
| 94 – 94.1 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) S5.562, S5.562A | 94 – 94.1 GHz RADIOLOCATION EARTH EXPLORATION- SATELLITE (active) SPACE RESEARCH (active) | S5.562, S5.562A | P |
| 94.1 – 95 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION S5.149 | 94.1 – 95 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | S5.149 | P |
| 95 – 100 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE S5.149, S5.554 | 95 – 100 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION -SATELLITE | S5.149, S5.554 | P |
| 100 – 102 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 100 – 102 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.341 | P |
| 102 – 105 GHz FIXED MOBILE RADIO ASTRONOMY S5.149, S5.341 | 102 – 105 GHz FIXED MOBILE RADIO ASTRONOMY | S5.149, S5.341 | P |
| 105 – 109. 5 GHz FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.562B S5.149, S5.341 | 105 – 109. 5 GHz FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.149, S5.341 S5.562B | P |
| 109. 5 - 111. 8 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 109. 5 - 111. 8 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.341 | P |
| 111. 8 – 114. 25 GHz FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.562B S5.149, S5.341 | 111. 8 – 114. 25 GHz FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.149, S5.341 S5.562B | P |
| 114. 25 – 116 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341 | 114. 25 – 116 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.341 | P |
| 116 – 119.98 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.562C SPACE RESEARCH (passive) S5.341 | 116 – 119.98 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.341, S5.562C | P |
| 119.98 – 120.02 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.562C SPACE RESEARCH (passive) S5.341 | 119.98 – 120.02 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.341, S5.562C | P |
| 120.02 - 122.25 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.562C SPACE RESEARCH (passive) S5.138 | 120.02 - 122.25 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.138, S5.562C RN035 | P |
| 122.25 - 123 GHz FIXED INTER-SATELLITE MOBILE S5.558 Amateur S5.138 | 122.25 - 123 GHz FIXED INTER-SATELLITE MOBILE Amateur | S5.138, S5.558 RN035 | P |
| 123 - 126 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy S5.554 | 123 - 126 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy | S5.554 | P |
| 126 – 130 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy S5.562D S5.149, S5.554 | 126 – 130 GHz FIXED-SATELLITE (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy | S5.149, S5.554 | P |
| 130 – 134 GHz EARTH EXPLORATION- SATELLITE (active) S5.562E FIXED INTER-SATELLITE MOBILE S5.558 RADIO ASTRONOMY S5.149, S5.562A | 130 – 134 GHz EARTH EXPLORATION- SATELLITE (active) FIXED INTER-SATELLITE MOBILE RADIO ASTRONOMY | S5.149, S5.558, S5.559, S5.562E, S5.562A | P |
| 134 – 136 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy | 134 – 136 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy |  | NG |
| 136 – 141 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite S5.149 | 136 – 141 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite | S5.149 | P |
| 141– 148.5 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION S5.149 | 141– 148.5 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | S5.149 | P |
| 148.5 – 151.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 148.5 – 151.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | P |
| 151.5 – 155.5 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION S5.149 | 151.5 – 155.5 GHz FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | S5.149 | P |
| 155.5 – 158.5 GHz EARTH EXPLORATION- SATELLITE (passive) S5.562F FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.562B S5.149, S5.562G | 155.5 – 158.5 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.149, S5.385, S5.562F, S5.562G, S5.562B | P |
| 158.5 – 164 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) | 158.5 – 164 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  | P |
| 164 – 167 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 164 – 167 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | P |
| 167 – 168 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE S5.558 | 167 – 168 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE | S5.558 | P |
| 168 – 170 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE S5.558 S5.149 | 168 – 170 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE | S5.149, S5.558 | P |
| 170 – 174.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE S5.558 S5.149, S5.562D | 170 – 174.5 GHz FIXED FIXED-SATELLITE (space-to-Earth) INTER-SATELLITE MOBILE | S5.149, S5.558 | P |
| 174.5 – 174.8 GHz FIXED INTER-SATELLITE MOBILE S5.558 | 174.5 – 174.8 GHz FIXED INTER-SATELLITE MOBILE | S5.558 | P |
| 174.8 – 182 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.562H SPACE RESEARCH (passive) | 174.8 – 182 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.562H | P |
| 182 – 185 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.563 | 182 – 185 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | P |
| 185 – 190 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE S5.562H SPACE RESEARCH (passive) | 185 – 190 GHz EARTH EXPLORATION- SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | S5.562H | P |
| 190 – 191.8 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) S5.340 | 190 – 191.8 GHz EARTH EXPLORATION- SATELLITE (passive) SPACE RESEARCH (passive) | S5.340 | P |
| 191.8 – 200 GHz FIXED INTER-SATELLITE MOBILE S5.558 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE S5.149,S5.341, S5.554 | 191.8 – 200 GHz FIXED INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION- SATELLITE | S5.149, S5.341, S5.554, S5.558 | P |
| 200 – 209 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.341, S5.563A | 200 – 209 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.341, S5.563A | P |
| 209 – 217 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.149, S5.341 | 209 – 217 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | S5.149, S5.341 | P |
| 217 – 226 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) S5.562B S5.149, S5.341 | 217 – 226 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.562B S5.149, S5.341 | P |
| 226 – 231.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340 | 226 – 231.5 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340 | P |
| 231.5 – 232 GHz FIXED MOBILE Radiolocation | 231.5 – 232 GHz FIXED MOBILE Radiolocation |  | P |
| 232 – 235 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation | 232 – 235 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Radiolocation |  | P |
| 235 – 238 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED-SATELLITE (space-to-Earth) SPACE RESEARCH (passive) S5.563A, S5.563B | 235 – 238 GHz EARTH EXPLORATION- SATELLITE (passive) FIXED-SATELLITE (space-to-Earth) SPACE RESEARCH (passive) | S5.563A, S5.563B | P |
| 238 – 240 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIOLOCATION RADIONAVIGATION RADIONAVIGATION –SATELLITE | 238 – 240 GHz FIXED FIXED-SATELLITE (space-to-Earth) MOBILE RADIOLOCATION RADIONAVIGATION RADIONAVIGATION -SATELLITE |  | P |
| 240 – 241 GHz FIXED MOBILE RADIOLOCATION | 240 – 241 GHz FIXED MOBILE RADIOLOCATION |  | P |
| 241 – 248 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite S5.138, S5.149 | 241 – 248 GHz RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite | S5.138, S5.149 RN035 | P |
| 248 – 250 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy S5.149 | 248 – 250 GHz AMATEUR AMATEUR-SATELLITE Radio Astronomy | S5.149 | NG |
| 250 – 252 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) S5.340, S5.563A | 250 – 252 GHz EARTH EXPLORATION- SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | S5.340, S5.563A | P |
| 252 – 265 GHz FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY RADIONAVIGATION RADIONAVIGATION-SATELLITE S5.149, S5.554 | 252 – 265 GHz FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY RADIONAVIGATION RADIONAVIGATION-SATELLITE | S5.149, S5.554 | P |
| 265 – 275 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY S5.149, S5.563A | 265 – 275 GHz FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | S5.149, S5.563A | P |
| 275 – 1000 GHz (Not allocated) S5.565 | 275 – 1000 GHz (Not allocated) | S5.565 |  |

# Annex 1

## Relevant footnotes (Extract from Article S5 of the Radio Regulations)

| S5.53 | Administrations authorizing the use of frequencies below 9 kHz shall ensure that no harmful interference is caused thereby to the services to which the bands above 9 kHz are allocated. |
| --- | --- |
| S5.54 | Administrations conducting scientific research using frequencies below 9 kHz are urged to advise other administrations that may be concerned in order that such research may be afforded all practicable protection from harmful interference. |
| S5.56 | The stations of services to which the bands 14-19.95 kHz and 20.05-70 kHz and in Region 1 also the bands 72-84 kHz and 86-90 kHz are allocated may transmit standard frequency and time signals. Such stations shall be afforded protection from harmful interference. In Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Republic, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the frequencies 25 kHz and 50 kHz will be used for this purpose under the same conditions. |
| S5.57 | The use of the bands 14-19.95 kHz, 20.05-70 kHz and 70-90 kHz (72-84 kHz and 86-90 kHz in Region 1) by the maritime mobile service is limited to coast radiotelegraph stations (A1A and F1B only). Exceptionally, the use of class J2B or J7B emissions is authorized subject to the necessary bandwidth not exceeding that normally used for class A1A or F1B emissions in the band concerned. |
| S5.60 | In the bands 70-90 kHz (70-86 kHz in Region 1) and 110-130 kHz (112-130 kHz in Region 1), pulsed radionavigation systems may be used on condition that they do not cause harmful interference to other services to which these bands are allocated. |
| S5.62 | Administrations which operate stations in the radionavigation service in the band 90-110 kHz are urged to coordinate technical and operating characteristics in such a way as to avoid harmful interference to the services provided by these stations. |
| S5.64 | Only classes A1A or F1B, A2C, A3C, F1C or F3C emissions are authorized for stations of the fixed service in the bands allocated to this service between 90 kHz and 160 kHz (148.5 kHz in Region 1) and for stations of the maritime mobile service in the bands allocated to this service between 110 kHz and 160 kHz (148.5 kHz in Region 1). Exceptionally, class J2B or J7B emissions are also authorized in the bands between 110 kHz and 160 kHz (148.5 kHz in Region 1) for stations of the maritime mobile service. |
| S5.73 | The band 285-325 kHz (283.5-325 kHz in Region 1) in the maritime radionavigation service may be used to transmit supplementary navigational information using narrow-band techniques, on condition that no harmful interference is caused to radiobeacon stations operating in the radionavigation service. |
| S5.74 | Additional allocation: in Region 1, the frequency band 285.3-285.7 kHz is also allocated to the maritime radionavigation service (other than radiobeacons) on a primary basis. |
| S5.75 | Different category of service: in Armenia, Azerbaijan, Belarus, Georgia, Moldova, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan, Ukraine and the Black Sea areas of Bulgaria and Romania, the allocation of the band 315-325 kHz to the maritime radionavigation service is on a primary basis under the condition that in the Baltic Sea area, the assignment of frequencies in this band to new stations in the maritime or aeronautical radionavigation services shall be subject to prior consultation between the administrations concerned. |
| S5.76 | The frequency 410 kHz is designated for radio direction-finding in the maritime radionavigation service. The other radionavigation services to which the band 405-415 kHz is allocated shall not cause harmful interference to radio direction-finding in the band 406.5-413.5 kHz. |
| S5.79 | The use of the bands 415-495 kHz and 505-526.5 kHz (505-510 kHz in Region 2) by the maritime mobile service is limited to radiotelegraphy. |
| S5.79A | When establishing coast stations in the NAVTEX service on the frequencies 490 kHz, 518 kHz and 4 209.5 kHz, administrations are strongly recommended to coordinate the operating characteristics in accordance with the procedures of the International Maritime Organization (IMO) (see Resolution 339 (Rev.WRC-97)). |
| S5.82 | In the maritime mobile service, the frequency 490 kHz is, from the date of full implementation of the GMDSS (see Resolution 331 (Rev.WRC-97)), to be used exclusively for the transmission by coast stations of navigational and meteorological warnings and urgent information to ships, by means of narrow-band direct-printing telegraphy. The conditions for use of the frequency 490 kHz are prescribed in Articles S31 and S52. In using the band 415-495 kHz for the aeronautical radionavigation service, administrations are requested to ensure that no harmful interference is caused to the frequency 490 kHz. |
| S5.83 | The frequency 500 kHz is an international distress and calling frequency for Morse radiotelegraphy. The conditions for its use are prescribed in Articles S31 and S52, and in Appendix S13. |
| S5.84 | The conditions for the use of the frequency 518 kHz by the maritime mobile service are prescribed in Articles S31 and S52 and in Appendix S13. |
| S5.90 | In the band 1 605-1 705 kHz, in cases where a broadcasting station of Region 2 is concerned, the service area of the maritime mobile stations in Region 1 shall be limited to that provided by ground-wave propagation. |
| S5.92 | Some countries of Region 1 use radiodetermination systems in the bands 1 606.5-1 625 kHz, 1 635-1 800 kHz, 1 850-2 160 kHz, 2 194-2 300 kHz, 2 502-2 850 kHz and 3 500-3 800 kHz, subject to agreement obtained under No. S9.21. The radiated mean power of these stations shall not exceed 50 W. |
| S5.93 | Additional allocation: in Angola, Armenia, Azerbaijan, Belarus, Georgia, Hungary, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Nigeria, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, the Russian Federation, Tajikistan, Chad, Turkmenistan and Ukraine, the bands 1 625-1 635 kHz, 1 800-1 810 kHz and 2 160-2 170 kHz and, in Bulgaria, the bands 1 625-1 635 kHz and 1 800-1 810 kHz, are also allocated to the fixed and land mobile services on a primary basis, subject to agreement obtained under No. S9.21. |
| S5.96 | In Germany, Armenia, Austria, Azerbaijan, Belarus, Denmark, Estonia, Finland, Georgia, Hungary, Ireland, Israel, Jordan, Kazakstan, Latvia, Liechtenstein, Lithuania, Malta, Moldova, Norway, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, the United Kingdom, the Russian Federation, Sweden, Switzerland, Tajikistan, Turkmenistan and Ukraine, administrations may allocate up to 200 kHz to their amateur service in the bands 1 715-1 800 kHz and 1 850-2 000 kHz. However, when allocating the bands within this range to their amateur service, administrations shall, after prior consultation with administrations of neighbouring countries, take such steps as may be necessary to prevent harmful interference from their amateur service to the fixed and mobile services of other countries. The mean power of any amateur station shall not exceed 10 W. |
| S5.98 | Alternative allocation: in Angola, Armenia, Azerbaijan, Belarus, Belgium, Bulgaria, Cameroon, the Congo, Denmark, Egypt, Eritrea, Spain, Ethiopia, Georgia, Greece, Italy, Kazakstan, Lebanon, Lithuania, Moldova, the Netherlands, Syria, Kyrgyzstan, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Turkey and Ukraine, the band 1 810-1 830 kHz is allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. |
| S5.100 | In Region 1, the authorization to use the band 1 810-1 830 kHz by the amateur service in countries situated totally or partially north of 40° N shall be given only after consultation with the countries mentioned in Nos. S5.98 and S5.99 to define the necessary steps to be taken to prevent harmful interference between amateur stations and stations of other services operating in accordance with Nos. S5.98 and S5.99. |
| S5.103 | In Region 1, in making assignments to stations in the fixed and mobile services in the bands 1 850-2 045 kHz, 2 194-2 498 kHz, 2 502-2 625 kHz and 2 650-2 850 kHz, administrations should bear in mind the special requirements of the maritime mobile service. |
| S5.104 | In Region 1, the use of the band 2 025-2 045 kHz by the meteorological aids service is limited to oceanographic buoy stations. |
| S5.108 | The carrier frequency 2 182 kHz is an international distress and calling frequency for radiotelephony. The conditions for the use of the band 2 173.5-2 190.5 kHz are prescribed in Articles S31 and S52 and in Appendix S13. |
| S5.109 | The frequencies 2 187.5 kHz, 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz are international distress frequencies for digital selective calling. The conditions for the use of these frequencies are prescribed in Article S31. |
| S5.110 | The frequencies 2 174.5 kHz, 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz are international distress frequencies for narrow-band direct-printing telegraphy. The conditions for the use of these frequencies are prescribed in Article S31. |
| S5.111 | The carrier frequencies 2 182 kHz, 3 023 kHz, 5 680 kHz, 8 364 kHz and the frequencies 121.5 MHz, 156.8 MHz and 243 MHz may also be used, in accordance with the procedures in force for terrestrial radiocommunication services, for search and rescue operations concerning manned space vehicles. The conditions for the use of the frequencies are prescribed in Article S31 and in Appendix S13. The same applies to the frequencies 10 003 kHz, 14 993 kHz and 19 993 kHz, but in each of these cases emissions must be confined in a band of 3 kHz about the frequency. |
| S5.113 | For the conditions for the use of the bands 2 300-2 495 kHz (2 498 kHz in Region 1), 3 200-3 400 kHz, 4 750-4 995 kHz and 5 005-5 060 kHz by the broadcasting service, see Nos. S5.16 to S5.20, S5.21 and S23.3 to S23.10. |
| S5.115 | The carrier (reference) frequencies 3 023 kHz and 5 680 kHz may also be used, in accordance with Article S31 and Appendix S13 by stations of the maritime mobile service engaged in coordinated search and rescue operations. |
| S5.116 | Administrations are urged to authorize the use of the band 3 155-3 195 kHz to provide a common worldwide channel for low power wireless hearing aids. Additional channels for these devices may be assigned by administrations in the bands between 3 155 kHz and 3 400 kHz to suit local needs. |
| S5.127 | The use of the band 4 000-4 063 kHz by the maritime mobile service is limited to ship stations using radiotelephony (see No. S52.220 and Appendix S17). |
| S5.129 | On condition that harmful interference is not caused to the maritime mobile service, the frequencies in the bands 4 063-4 123 kHz and 4 130-4 438 kHz may be used exceptionally by stations in the fixed service communicating only within the boundary of the country in which they are located with a mean power not exceeding 50 W. |
| S5.130 | The conditions for the use of the carrier frequencies 4 125 kHz and 6 215 kHz are prescribed in Articles S31 and S52 and in Appendix S13. |
| S5.131 | The frequency 4 209.5 kHz is used exclusively for the transmission by coast stations of meteorological and navigational warnings and urgent information to ships by means of narrow-band direct-printing techniques. |
| S5.132 | The frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the international frequencies for the transmission of maritime safety information (MSI) (see Appendix S17). |
| S5.133 | Different category of service: in Armenia, Azerbaijan, Belarus, Georgia, Kazakstan, Latvia, Lithuania, Moldova, Uzbekistan, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 5 130-5 250 kHz to the mobile, except aeronautical mobile, service is on a primary basis (see No. S5.33). |
| S5.134 | The use of the bands 5 900-5 950 kHz, 7 300-7 350 kHz, 9 400-9 500 kHz, 11 600-11 650 kHz, 12 050-12 100 kHz, 13 570-13 600 kHz, 13 800-13 870 kHz, 15 600-15 800 kHz, 17 480-17 550 kHz and 18 900-19 020 kHz by the broadcasting service is limited to single-sideband emissions with the characteristics specified in Appendix S11 or to any other spectrum-efficient modulation techniques recommended by ITU-R. Access to these bands shall be subject to the decisions of a competent conference. |
| S5.136 | The band 5 900-5 950 kHz is allocated, until 1 April 2007, to the fixed service on a primary basis, as well as to the following services: in Region 1 to the land mobile service on a primary basis, in Region 2 to the mobile except aeronautical mobile (R) service on a primary basis, and in Region 3 to the mobile except aeronautical mobile (R) service on a secondary basis, subject to application of the procedure referred to in Resolution 21 (Rev.WRC-95). After 1 April 2007, frequencies in this band may be used by stations in the above-mentioned services, communicating only within the boundary of the country in which they are located, on the condition that harmful interference is not caused to the broadcasting service. When using frequencies for these services, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations. |
| S5.137 | On condition that harmful interference is not caused to the maritime mobile service, the bands 6 200-6 213.5 kHz and 6 220.5-6 525 kHz may be used exceptionally by stations in the fixed service, communicating only within the boundary of the country in which they are located, with a mean power not exceeding 50 W. At the time of notification of these frequencies, the attention of the Bureau will be drawn to the above conditions. |
| S5.138 | The following bands: 6 765-6 795 kHz (centre frequency 6 780 kHz), 433.05-434.79 MHz (centre frequency 433.92 MHz) in Region 1 except in the countries mentioned in No. S5.280, 61-61.5 GHz (centre frequency 61.25 GHz), 122-123 GHz (centre frequency 122.5 GHz), and 244-246 GHz (centre frequency 245 GHz) are designated for industrial, scientific and medical (ISM) applications. The use of these frequency bands for ISM applications shall be subject to special authorization by the administration concerned, in agreement with other administrations whose radiocommunication services might be affected. In applying this provision, administrations shall have due regard to the latest relevant ITU-R Recommendations. |
| S5.139 | Different category of service: in Armenia, Azerbaijan, Belarus, Georgia, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 6 765-7 000 kHz to the land mobile service is on a primary basis (see No. S5.33). |
| S5.143 | The band 7 300-7 350 kHz is allocated, until 1 April 2007, to the fixed service on a primary basis and to the land mobile service on a secondary basis, subject to application of the procedure referred to in Resolution 21 (Rev.WRC-95). After 1 April 2007, frequencies in this band may be used by stations in the above-mentioned services, communicating only within the boundary of the country in which they are located, on condition that harmful interference is not caused to the broadcasting service. When using frequencies for these services, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations. |
| S5.145 | The conditions for the use of the carrier frequencies 8 291 kHz, 12 290 kHz and 16 420 kHz are prescribed in Articles S31 and S52 and in Appendix S13. |
| S5.146 | The bands 9 400-9 500 kHz, 11 600-11 650 kHz, 12 050-12 100 kHz, 15 600-15 800 kHz, 17 480-17 550 kHz and 18 900-19 020 kHz are allocated to the fixed service on a primary basis until 1 April 2007, subject to application of the procedure referred to in Resolution 21 (Rev.WRC-95). After 1 April 2007, frequencies in these bands may be used by stations in the fixed service, communicating only within the boundary of the country in which they are located, on condition that harmful interference is not caused to the broadcasting service. When using frequencies in the fixed service, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations. |
| S5.147 | On condition that harmful interference is not caused to the broadcasting service, frequencies in the bands 9 775-9 900 kHz, 11 650-11 700 kHz and 11 975-12 050 kHz may be used by stations in the fixed service communicating only within the boundary of the country in which they are located, each station using a total radiated power not exceeding 24 dBW. |
| S5.149 | In making assignments to stations of other services to which the bands: 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz, 1 610.6-1 613.8 MHz, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz, 2 655-2 690 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 4 825-4 835 MHz, 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz, 10.6-10.68 GHz, 14.47-14.5 GHz, 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz, 42.5-43.5 GHz, 42.77-42.87 GHz, 43.07-43.17 GHz, 43.37-43.47 GHz, 48.94-49.04 GHz, 76-86 GHz, 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz, 111.8-114.25 GHz, 128.33-128.59 GHz, 129.23-129.49 GHz, 130-134 GHz, 136-148.5 GHz, 151.5-158.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz, 172.31-172.65 GHz, 173.52-173.85 GHz, 195.75-196.15 GHz, 209-226 GHz, 241-250 GHz, 252-275 GHz are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. S4.5 and S4.6 and Article S29). |
| S5.150 | The following bands: 13 553-13 567 kHz (centre frequency 13 560 kHz), 26 957-27 283 kHz (centre frequency 27 120 kHz), 40.66-40.70 MHz (centre frequency 40.68 MHz), 902-928 MHz in Region 2 (centre frequency 915 MHz), 2 400-2 500 MHz (centre frequency 2 450 MHz), 5 725-5 875 MHz (centre frequency 5 800 MHz), and 24-24.25 GHz (centre frequency 24.125 GHz) are also designated for industrial, scientific and medical (ISM) applications. Radiocommunication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. S15.13. |
| S5.151 | The bands 13 570-13 600 kHz and 13 800-13 870 kHz are allocated, until 1 April 2007, to the fixed service on a primary basis and to the mobile except aeronautical mobile (R) service on a secondary basis, subject to application of the procedure referred to in Resolution 21 (Rev.WRC-95). After 1 April 2007, frequencies in these bands may be used by stations in the above-mentioned services, communicating only within the boundary of the country in which they are located, on the condition that harmful interference is not caused to the broadcasting service. When using frequencies in these services, administrations are urged to use the minimum power required and to take account of the seasonal use of frequencies by the broadcasting service published in accordance with the Radio Regulations. |
| S5.152 | Additional allocation: in Armenia, Azerbaijan, China, Côte d’Ivoire, Georgia, Iran (Islamic Republic of), Kazakstan, Moldova, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 14 250-14 350 kHz is also allocated to the fixed service on a primary basis. Stations of the fixed service shall not use a radiated power exceeding 24 dBW. |
| S5.154 | Additional allocation: in Armenia, Azerbaijan, Georgia, Kazakstan, Moldova, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 18 068-18 168 kHz is also allocated to the fixed service on a primary basis for use within their boundaries, with a peak envelope power not exceeding 1 kW. |
| S5.155 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Hungary, Kazakstan, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Republic, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 21 850-21 870 kHz is also allocated to the aeronautical mobile (R) services on a primary basis. |
| S5.155A | In Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Republic, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the use of the band 21 850-21 870 kHz by the fixed service is limited to provision of services related to aircraft flight safety. |
| S5.155B | The band 21 870-21 924 kHz is used by the fixed service for provision of services related to aircraft flight safety. |
| S5.156A | The use of the band 23 200-23 350 kHz by the fixed service is limited to provision of services related to aircraft flight safety. |
| S5.157 | The use of the band 23 350-24 000 kHz by the maritime mobile service is limited to inter-ship radiotelegraphy. |
| S5.162A | Additional allocation: in Germany, Austria, Belgium, Bosnia and Herzegovina, China, Vatican, Denmark, Spain, Estonia, Finland, France, Ireland, Iceland, Italy, Latvia, The Former Yugoslav Republic of Macedonia, Liechtenstein, Lithuania, Luxembourg, Moldova, Monaco, Norway, the Netherlands, Poland, Portugal, Slovakia, the Czech Republic, the United Kingdom, the Russian Federation, Sweden and Switzerland the band 46-68 MHz is also allocated to the radiolocation service on a secondary basis. This use is limited to the operation of wind profiler radars in accordance with Resolution 217 (WRC-97). |
| S5.163 | Additional allocation: in Armenia, Azerbaijan, Belarus, Estonia, Georgia, Hungary, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Slovakia, the Czech Republic, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 47-48.5 MHz and 56.5-58 MHz are also allocated to the fixed and land mobile services on a secondary basis. |
| S5.175 | Alternative allocation: in Armenia, Azerbaijan, Belarus, Georgia, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 68-73 MHz and 76-87.5 MHz are allocated to the broadcasting service on a primary basis. The services to which these bands are allocated in other countries and the broadcasting service in the countries listed above are subject to agreements with the neighbouring countries concerned. |
| S5.177 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Kazakstan, Latvia, Moldova, Uzbekistan, Poland, Kyrgyzstan, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 73-74 MHz is also allocated to the broadcasting service on a primary basis, subject to agreement obtained under No. S9.21. |
| S5.179 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, China, Georgia, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Kyrgyzstan, Slovakia, the Czech Republic, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 74.6-74.8 MHz and 75.2-75.4 MHz are also allocated to the aeronautical radionavigation service, on a primary basis, for ground-based transmitters only. |
| S5.180 | The frequency 75 MHz is assigned to marker beacons. Administrations shall refrain from assigning frequencies close to the limits of the guardband to stations of other services which, because of their power or geographical position, might cause harmful interference or otherwise place a constraint on marker beacons. |
| S5.198 | Additional allocation: the band 117.975-136 MHz is also allocated to the aeronautical mobile-satellite (R) service on a secondary basis, subject to agreement obtained under No. S9.21. |
| S5.199 | The bands 121.45-121.55 MHz and 242.95-243.05 MHz are also allocated to the mobile-satellite service for the reception on board satellites of emissions from emergency position-indicating radiobeacons transmitting at 121.5 MHz and 243 MHz (see Appendix S13). |
| S5.200 | In the band 117.975-136 MHz, the frequency 121.5 MHz is the aeronautical emergency frequency and, where required, the frequency 123.1 MHz is the aeronautical frequency auxiliary to 121.5 MHz. Mobile stations of the maritime mobile service may communicate on these frequencies under the conditions laid down in Article S31 and Appendix S13 for distress and safety purposes with stations of the aeronautical mobile service. |
| S5.201 | Additional allocation: in Angola, Armenia, Azerbaijan, Belarus, Bulgaria, Estonia, Georgia, Hungary, the Islamic Republic of Iran, Iraq, Japan, Kazakstan, Latvia, Moldova, Mongolia, Mozambique, Uzbekistan, Papua New Guinea, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 132-136 MHz is also allocated to the aeronautical mobile (OR) service on a primary basis. In assigning frequencies to stations of the aeronautical mobile (OR) service, the administration shall take account of the frequencies assigned to stations in the aeronautical mobile (R) service. |
| S5.202 | Additional allocation: in Saudi Arabia, Armenia, Azerbaijan, Belarus, Bulgaria, the United Arab Emirates, Georgia, Iran (Islamic Republic of), Jordan, Latvia, Moldova, Oman, Uzbekistan, Poland, Syria, Kyrgyzstan, Slovakia, the Czech Republic, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 136-137 MHz is also allocated to the aeronautical mobile (OR) service on a primary basis. In assigning frequencies to stations of the aeronautical mobile (OR) service, the administration shall take account of the frequencies assigned to stations in the aeronautical mobile (R) service. |
| S5.203 | In the band 136-137 MHz, existing operational meteorological satellites may continue to operate, under the conditions defined in No. S4.4 with respect to the aeronautical mobile service, until 1 January 2002. Administrations shall not authorize new frequency assignments in this band to stations in the meteorological-satellite service. |
| S5.206 | Different category of service: in Armenia, Azerbaijan, Belarus, Bulgaria, Egypt, Finland, France, Georgia, Greece, Kazakstan, Lebanon, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Syria, Slovakia, the Czech Republic, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 137-138 MHz to the aeronautical mobile (OR) service is on a primary basis (see No. S5.33). |
| S5.208 | The use of the band 137-138 MHz by the mobile-satellite service is subject to coordination under No. S9.11A. |
| S5.208A | In making assignments to space stations in the mobile-satellite service in the bands 137-138 MHz, 387-390 MHz and 400.15-401 MHz, administrations shall take all practicable steps to protect the radio astronomy service in the bands 150.05-153 MHz, 322-328.6 MHz, 406.1-410 MHz and 608-614 MHz from harmful interference from unwanted emissions. The threshold levels of interference detrimental to the radio astronomy service are shown in Table 1 of Recommendation ITU-R RA.769-1. |
| S5.209 | The use of the bands 137-138 MHz, 148-150.05 MHz, 399.9-400.05 MHz, 400.15-401 MHz, 454-456 MHz and 459-460 MHz by the mobile-satellite service is limited to non-geostationary-satellite systems. |
| S5.218 | Additional allocation: the band 148-149.9 MHz is also allocated to the space operation service (Earth-to-space) on a primary basis, subject to agreement obtained under No. S9.21. The bandwidth of any individual transmission shall not exceed ± 25 kHz. |
| S5.219 | The use of the band 148-149.9 MHz by the mobile-satellite service is subject to coordination under No. S9.11A. The mobile-satellite service shall not constrain the development and use of the fixed, mobile and space operation services in the band 148-149.9 MHz. |
| S5.220 | The use of the bands 149.9-150.05 MHz and 399.9-400.05 MHz by the mobile-satellite service is subject to coordination under No. S9.11A. The mobile-satellite service shall not constrain the development and use of the radionavigation-satellite service in the bands 149.9-150.05 MHz and 399.9-400.05 MHz. |
| S5.221 | Stations of the mobile-satellite service in the band 148-149.9 MHz shall not cause harmful interference to, or claim protection from, stations of the fixed or mobile services operating in accordance with the Table of Frequency Allocations in the following countries: Albania, Algeria, Germany, Saudi Arabia, Australia, Austria, Bahrain, Bangladesh, Barbados, Belarus, Belgium, Benin, Bosnia and Herzegovina, Brunei Darussalam, Bulgaria, Cameroon, China, Cyprus, Congo, Korea (Rep. of), Croatia, Cuba, Denmark, Egypt, the United Arab Emirates, Eritrea, Spain, Estonia, Ethiopia, Finland, France, Gabon, Ghana, Greece, Guinea, Guinea Bissau, Hungary, India, Iran (Islamic Republic of), Ireland, Iceland, Israel, Italy, Jamaica, Japan, Jordan, Kazakstan, Kenya, Kuwait, Latvia, The Former Yugoslav Republic of Macedonia, Lebanon, Libya, Liechtenstein, Lithuania, Luxembourg, Malaysia, Mali, Malta, Mauritania, Moldova, Mongolia, Mozambique, Namibia, Norway, New Zealand, Oman, Uganda, Uzbekistan, Pakistan, Panama, Papua New Guinea, Paraguay, the Netherlands, the Philippines, Poland, Portugal, Qatar, Syria, Kyrgyzstan, Slovakia, Romania, the United Kingdom, the Russian Federation, Senegal, Sierra Leone, Singapore, Slovenia, Sri Lanka, South Africa, Sweden, Switzerland, Swaziland, Tanzania, Chad, Thailand, Togo, Tonga, Trinidad and Tobago, Tunisia, Turkey, Ukraine, Viet Nam, Yemen, Yugoslavia, Zambia, and Zimbabwe. |
| S5.222 | Emissions of the radionavigation-satellite service in the bands 149.9-150.05 MHz and 399.9-400.05 MHz may also be used by receiving earth stations of the space research service. |
| S5.223 | Recognizing that the use of the band 149.9-150.05 MHz by the fixed and mobile services may cause harmful interference to the radionavigation-satellite service, administrations are urged not to authorize such use in application of No. S4.4. |
| S5.224A | The use of the bands 149.9-150.05 MHz and 399.9-400.05 MHz by the mobile-satellite service (Earth-to-space) is limited to the land mobile-satellite service (Earth-to-space) until 1 January 2015. |
| S5.224B | The allocation of the bands 149.9-150.05 MHz and 399.9-400.05 MHz to the radionavigation-satellite service shall be effective until 1 January 2015. |
| S5.226 | The frequency 156.8 MHz is the international distress, safety and calling frequency for the maritime mobile VHF radiotelephone service. The conditions for the use of this frequency are contained in Article S31 and Appendix S13. In the bands 156-156.7625 MHz, 156.8375-157.45 MHz, 160.6-160.975 MHz and 161.475-162.05 MHz, each administration shall give priority to the maritime mobile service on only such frequencies as are assigned to stations of the maritime mobile service by the administration (see Articles S31 and S52, and Appendix S13). Any use of frequencies in these bands by stations of other services to which they are allocated should be avoided in areas where such use might cause harmful interference to the maritime mobile VHF radiocommunication service. However, the frequency 156.8 MHz and the frequency bands in which priority is given to the maritime mobile service may be used for radiocommunications on inland waterways subject to agreement between interested and affected administrations and taking into account current frequency usage and existing agreements. |
| S5.227 | In the maritime mobile VHF service the frequency 156.525 MHz is to be used exclusively for digital selective calling for distress, safety and calling. The conditions for the use of this frequency are prescribed in Articles S31 and S52, and Appendices S13 and S18. |
| S5.254 | The bands 235-322 MHz and 335.4-399.9 MHz may be used by the mobile-satellite service, subject to agreement obtained under No. S9.21, on condition that stations in this service do not cause harmful interference to those of other services operating or planned to be operated in accordance with the Table of Frequency Allocations. |
| S5.255 | The bands 312-315 MHz (Earth-to-space) and 387-390 MHz (space-to-Earth) in the mobile-satellite service may also be used by non-geostationary-satellite systems. Such use is subject to coordination under No. S9.11A. |
| S5.256 | The frequency 243 MHz is the frequency in this band for use by survival craft stations and equipment used for survival purposes (see Appendix S13). |
| S5.257 | The band 267-272 MHz may be used by administrations for space telemetry in their countries on a primary basis, subject to agreement obtained under No. S9.21. |
| S5.258 | The use of the band 328.6-335.4 MHz by the aeronautical radionavigation service is limited to Instrument Landing Systems (glide path). |
| S5.260 | Recognizing that the use of the band 399.9-400.05 MHz by the fixed and mobile services may cause harmful interference to the radionavigation satellite service, administrations are urged not to authorize such use in application of No. S4.4. |
| S5.261 | Emissions shall be confined in a band of ± 25 kHz about the standard frequency 400.1 MHz. |
| S5.262 | Additional allocation: in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Bulgaria, Colombia, Costa Rica, Cuba, Egypt, the United Arab Emirates, Ecuador, Georgia, Hungary, Iran (Islamic Republic of), Iraq, Israel, Jordan, Kazakstan, Kuwait, Liberia, Malaysia, Moldova, Nigeria, Uzbekistan, Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, Slovakia, Romania, the Russian Federation, Singapore, Somalia, Tajikistan, Turkmenistan, Ukraine and Yugoslavia, the band 400.05-401 MHz is also allocated to the fixed and mobile services on a primary basis. |
| S5.263 | The band 400.15-401 MHz is also allocated to the space research service in the space-to-space direction for communications with manned space vehicles. In this application, the space research service will not be regarded as a safety service. |
| S5.264 | The use of the band 400.15-401 MHz by the mobile-satellite service is subject to coordination under No. S9.11A. The power flux-density limit indicated in Annex 1 of Appendix S5 shall apply until such time as a competent world radiocommunication conference revises it. |
| S5.266 | The use of the band 406-406.1 MHz by the mobile-satellite service is limited to low power satellite emergency position-indicating radiobeacons (see also Article S31 and Appendix S13). |
| S5.267 | Any emission capable of causing harmful interference to the authorized uses of the band 406-406.1 MHz is prohibited. |
| S5.268 | Use of the band 410-420 MHz by the space research service is limited to communications within 5 km of an orbiting, manned space vehicle. The power flux-density at the surface of the Earth produced by emissions from extra-vehicular activities shall not exceed –153 dB(W/m2) for 0°    5°, -153 + 0.077 ( – 5) dB(W/m2) for 5°    70° and –148 dB(W/m2) for 70°    90o, where  is the angle of arrival of the radio-frequency wave and the reference bandwidth is 4 kHz. No. S4.10 does not apply to extra-vehicular activities. In this frequency band the space research (space-to-space) service shall not claim protection from, nor constrain the use and development of, stations of the fixed and mobile services. |
| S5.277 | Additional allocation: in Angola, Armenia, Azerbaijan, Belarus, Cameroon, Congo, Djibouti, Georgia, Hungary, Israel, Kazakstan, Latvia, Mali, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, the Russian Federation, Rwanda, Tajikistan, Chad, Turkmenistan and Ukraine, the band 430-440 MHz is also allocated to the fixed service on a primary basis. |
| S5.280 | In Germany, Austria, Bosnia and Herzegovina, Croatia, The Former Yugoslav Republic of Macedonia, Liechtenstein, Portugal, Slovenia, Switzerland and Yugoslavia, the band 433.05-434.79 MHz (centre frequency 433.92 MHz) is designated for industrial, scientific and medical (ISM) applications. Radiocommunication services of these countries operating within this band must accept harmful interference which may be caused by these applications. ISM equipment operating in this band is subject to the provisions of No. S15.13. |
| S5.286 | The band 449.75-450.25 MHz may be used for the space operation service (Earth-to-space) and the space research service (Earth-to-space), subject to agreement obtained under No. S9.21. |
| S5.286A | The use of the bands 454-456 MHz and 459-460 MHz by the mobile-satellite service is subject to coordination under No. S9.11A. |
| S5.287 | In the maritime mobile service, the frequencies 457.525 MHz, 457.550 MHz, 457.575 MHz, 467.525 MHz, 467.550 MHz and 467.575 MHz may be used by on-board communication stations. Where needed, equipment designed for 12.5 kHz channel spacing using also the additional frequencies 457.5375 MHz, 457.5625 MHz, 467.5375 MHz and 467.5625 MHz may be introduced for on-board communications. The use of these frequencies in territorial waters may be subject to the national regulations of the administration concerned. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174 (see Resolution 341 (WRC-97)). |
| S5.289 | Earth exploration-satellite service applications, other than the meteorological-satellite service, may also be used in the bands 460-470 MHz and 1 690-1 710 MHz for space-to-Earth transmissions subject to not causing harmful interference to stations operating in accordance with the Table. |
| S5.306 | Additional allocation: in Region 1, except in the African Broadcasting Area (see Nos. S5.10 to S5.13), and in Region 3, the band 608-614 MHz is also allocated to the radio astronomy service on a secondary basis. |
| S5.311 | Within the frequency band 620-790 MHz, assignments may be made to television stations using frequency modulation in the broadcasting-satellite service subject to agreement between the administrations concerned and those having services, operating in accordance with the Table, which may be affected (see Resolutions 33 (Rev. WRC-97) and 507). Such stations shall not produce a power flux-density in excess of the value –129 dB(W/m2) for angles of arrival less than 20° (see Recommendation 705) within the territories of other countries without the consent of the administrations of those countries. |
| S5.312 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Hungary, Kazakstan, Latvia, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 645-862 MHz is also allocated to the aeronautical radionavigation service on a primary basis. |
| S5.314 | Additional allocation: in Austria, Italy, Moldova, Uzbekistan, the United Kingdom and Swaziland, the band 790-862 MHz is also allocated to the land mobile service on a secondary basis. |
| S5.317A | Administrations wishing to implement International Mobile Telecommunications-2000 (IMT-2000) may use those parts of the band 806-960 MHz which are allocated to the mobile service on a primary basis and are used or planned to be used for mobile systems (see Resolution 224 (WRC-2000)). This identification does not preclude the use of these bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. |
| S5.323 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Hungary, Kazakstan, Latvia, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 862-960 MHz is also allocated to the aeronautical radionavigation service on a primary basis. Such use is subject to agreement obtained under No. S9.21 with administrations concerned and limited to ground-based radiobeacons in operation on 27 October 1997 until the end of their lifetime. |
| S5.328 | The use of the band 960-1 215 MHz by the aeronautical radionavigation service is reserved on a worldwide basis for the operation and development of airborne electronic aids to air navigation and any directly associated ground-based facilities. |
| S5.328A | Additional allocation: the band 1 164-1 215 MHz is also allocated to the radionavigation-satellite service (space-to-Earth) (space-to-space) on a primary basis. The aggregate power flux-density produced by all the space stations of all radionavigation-satellite systems at the Earth’s surface shall not exceed the provisional value of –115 dB(W/m2) in any 1 MHz band for all angles of arrival. Stations in the radionavigation-satellite service shall not cause harmful interference to, nor claim protection from, stations of the aeronautical-radionavigation service. The provisions of Resolution 605 (WRC-2000) apply. |
| S5.329 | Use of the radionavigation-satellite service in the band 1 215-1 300 MHz shall be subject to the condition that no harmful interference is caused to, and no protection is claimed from, the radionavigation service authorized under No. S5.331. See also Resolution 606 (WRC-2000). |
| S5.329A | Use of systems in the radionavigation-satellite service (space-to-space) operating in the bands 1 215-1 300 MHz and 1 559-1 610 MHz is not intended to provide safety service applications, and shall not impose any additional constraints on other systems or services operating in accordance with the Table of Frequency Allocations. |
| S5.332 | In the band 1 215-1 260 MHz, active spaceborne sensors in the earth exploration-satellite and space research services shall not cause harmful interference to, claim protection from, or otherwise impose constraints on operation or development of the radiolocation service, the radionavigation-satellite service and other services allocated on a primary basis. |
| S5.333 | In the band 1 260-1 300 MHz, active spaceborne sensors in the Earth exploration-satellite and space research services shall not cause harmful interference to, claim protection from, or otherwise impose constraints on operation or development of the radiolocation service and other services allocated by footnotes on a primary basis. |
| S5.335A | In the band 1 260-1 300 MHz, active spaceborne sensors in the Earth exploration-satellite and space research services shall not cause harmful interference to, claim protection from, or otherwise impose constraints on operation or development of the radiolocation service and other services allocated by footnotes on a primary basis. |
| S5.337 | The use of the bands 1 300-1 350 MHz, 2 700-2 900 MHz and 9 000-9 200 MHz by the aeronautical radionavigation service is restricted to ground-based radars and to associated airborne transponders which transmit only on frequencies in these bands and only when actuated by radars operating in the same band. |
| S5.337A | The use of the band 1 300-1 350 MHz by earth stations in the radionavigation-satellite service and by stations in the radiolocation service shall not cause harmful interference to, nor constrain the operation and development of, the aeronautical-radionavigation service. |
| S5.339 | The bands 1 370-1 400 MHz, 2 640-2 655 MHz, 4 950-4 990 MHz and 15.20-15.35 GHz are also allocated to the space research (passive) and earth exploration-satellite (passive) services on a secondary basis. |
| S5.340 | All emissions are prohibited in the following bands: 1 400-1 427 MHz, 2 690-2 700 MHz, except those provided for by Nos. S5.421 and S5.422, 10.68-10.7 GHz, except those provided for by No. S5.483, 15.35-15.4 GHz, except those provided for by No. S5.511, 23.6-24 GHz, 31.3-31.5 GHz, 31.5-31.8 GHz, in Region 2, 48.94-49.04 GHz, from airborne stations, 50.2-50.4 GHz, except those provided for by No. S5.555A, 52.6-54.25 GHz, 86-92 GHz, 100-102 GHz, 109.5-111.8 GHz, 114.25-116 GHz 148.5-151.5 GHz, 164-167 GHz, 182-185 GHz, except those provided for by No. S5.563, 190-191.8 GHz, 200-209 GHz, 226-231.5 GHz, 250-252 GHz. |
| S5.341 | In the bands 1 400-1 727 MHz, 101-120 GHz and 197-220 GHz, passive research is being conducted by some countries in a programme for the search for intentional emissions of extraterrestrial origin. |
| S5.345 | Use of the band 1 452-1 492 MHz by the broadcasting-satellite service, and by the broadcasting service, is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92). |
| S5.351 | The bands 1 525-1 544 MHz, 1 545-1 559 MHz, 1 626.5-1 645.5 MHz and 1 646.5-1 660.5 MHz shall not be used for feeder links of any service. In exceptional circumstances, however, an earth station at a specified fixed point in any of the mobile-satellite services may be authorized by an administration to communicate via space stations using these bands. |
| S5.351A | For the use of the bands 1 525-1 544 MHz, 1 545-1 559 MHz, 1 610-1 626.5 MHz, 1 626.5-1 645.5 MHz, 1 646.5-1 660.5 MHz, 1 980-2 010 MHz, 2 170-2 200 MHz, 2 483.5-2 500 MHz, 2 500-2 520 MHz and 2 670-2 690 MHz by the mobile-satellite service, see Resolutions 212 (Rev.WRC-97) and 225 (WRC-2000). |
| S5.353A | In applying the procedures of Section II of Article S9 to the mobile-satellite service in the bands 1 530-1 544 MHz and 1 626.5-1 645.5 MHz, priority shall be given to accommodating the spectrum requirements for distress, urgency and safety communications of the Global Maritime Distress and Safety System (GMDSS). Maritime mobile-satellite distress, urgency and safety communications shall have priority access and immediate availability over all other mobile satellite communications operating within a network. Mobile-satellite systems shall not cause unacceptable interference to, or claim protection from, distress, urgency and safety communications of the GMDSS. Account shall be taken of the priority of safety-related communications in the other mobile-satellite services. (The provisions of Resolution 222 (WRC-2000) shall apply.) |
| S5.354 | The use of the bands 1 525-1 559 MHz and 1 626.5-1 660.5 MHz by the mobile-satellite services is subject to coordination under No. S9.11A. |
| S5.356 | The use of the band 1 544-1 545 MHz by the mobile-satellite service (space-to-Earth) is limited to distress and safety communications (see Article S31). |
| S5.357 | Transmissions in the band 1 545-1 555 MHz from terrestrial aeronautical stations directly to aircraft stations, or between aircraft stations, in the aeronautical mobile (R) service are also authorized when such transmissions are used to extend or supplement the satellite-to-aircraft links. |
| S5.357A | In applying the procedures of Section II of Article S9 to the mobile-satellite service in the bands 1 545-1 555 MHz and 1 646.5-1 656.5 MHz, priority shall be given to accommodating the spectrum requirements of the aeronautical mobile-satellite (R) service providing transmission of messages with priority 1 to 6 in Article S44. Aeronautical mobile-satellite (R) service communications with priority 1 to 6 in Article S44 shall have priority access and immediate availability, by pre-emption if necessary, over all other mobile-satellite communications operating within a network. Mobile-satellite systems shall not cause unacceptable interference to, or claim protection from, aeronautical mobile-satellite (R) service communications with priority 1 to 6 in Article S44. Account shall be taken of the priority of safety-related communications in the other mobile-satellite services. (The provisions of Resolution 222 (WRC-2000) shall apply.) |
| S5.359 | Additional allocation: in Germany, Saudi Arabia, Armenia, Austria, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Cameroon, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Jordan, Kazakstan, Kuwait, Latvia, Lebanon, Libya, Lithuania, Mali, Morocco, Mauritania, Moldova, Mongolia, Nigeria, Uganda, Uzbekistan, Pakistan, Poland, Syria, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Senegal, Swaziland, Tajikistan, Tanzania, Tunisia, Turkmenistan and Ukraine, the bands 1 550-1 559 MHz, 1 610-1 645.5 MHz and 1 646.5-1 660 MHz are also allocated to the fixed service on a primary basis. Administrations are urged to make all practicable efforts to avoid the implementation of new fixed-service stations in these bands. |
| S5.362B | Additional allocation: The band 1 559-1 610 MHz is also allocated to the fixed service on a primary basis until 1 January 2005 in Germany, Armenia, Azerbaijan, Belarus, Benin, Bosnia and Herzegovina, Bulgaria, Spain, France, Gabon, Georgia, Greece, Guinea, Guinea-Bissau, Hungary, Kazakstan, Latvia, Lithuania, Moldova, Mongolia, Nigeria, Uganda, Uzbekistan, Pakistan, Poland, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Senegal, Swaziland, Tajikistan, Tanzania, Turkmenistan and Ukraine, and until 1 January 2010 in Saudi Arabia, Cameroon, Jordan, Kuwait, Lebanon, Libya, Mali, Morocco, Mauritania, Syria and Tunisia. After these dates, the fixed service may continue to operate on a secondary basis until 1 January 2015, at which time this allocation shall no longer be valid. Administrations are urged to take all practicable steps to protect the radionavigation-satellite service and the aeronautical radionavigation service and not authorize new frequency assignments to fixed-service systems in this band. |
| S5.364 | The use of the band 1 610-1 626.5 MHz by the mobile-satellite service (Earth-to-space) and by the radiodetermination-satellite service (Earth-to-space) is subject to coordination under No. S9.11A. A mobile earth station operating in either of the services in this band shall not produce a peak e.i.r.p. density in excess of -15 dB(W/4 kHz) in the part of the band used by systems operating in accordance with the provisions of No. S5.366 (to which No. S4.10 applies), unless otherwise agreed by the affected administrations. In the part of the band where such systems are not operating, the mean e.i.r.p. density of a mobile earth station shall not exceed –3 dB(W/4 kHz). Stations of the mobile-satellite service shall not claim protection from stations in the aeronautical radionavigation service, stations operating in accordance with the provisions of No. S5.366 and stations in the fixed service operating in accordance with the provisions of No. S5.359. Administrations responsible for the coordination of mobile-satellite networks shall make all practicable efforts to ensure protection of stations operating in accordance with the provisions of No. S5.366. |
| S5.365 | The use of the band 1 613.8-1 626.5 MHz by the mobile-satellite service (space-to-Earth) is subject to coordination under No. S9.11A. |
| S5.366 | The band 1 610-1 626.5 MHz is reserved on a worldwide basis for the use and development of airborne electronic aids to air navigation and any directly associated ground-based or satellite-borne facilities. Such satellite use is subject to agreement obtained under No. S9.21. |
| S5.367 | Additional allocation: The bands 1 610-1 626.5 MHz and 5 000-5 150 MHz are also allocated to the aeronautical mobile-satellite (R) service on a primary basis, subject to agreement obtained under No. S9.21. |
| S5.368 | With respect to the radiodetermination-satellite and mobile-satellite services the provisions of No. S4.10 do not apply in the band 1 610-1 626.5 MHz, with the exception of the aeronautical radionavigation-satellite service. |
| S5.369 | Different category of service: in Angola, Australia, Burundi, China, Côte d’Ivoire, Eritrea, Ethiopia, India, the Islamic Republic of Iran, Israel, Jordan, Lebanon, Liberia, Libya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep.of the Congo, Syria, Senegal, Sudan, Swaziland, Togo and Zambia, the allocation of the band 1 610-1 626.5 MHz to the radiodetermination-satellite service (Earth-to-space) is on a primary basis (see No. S5.33), subject to agreement obtained under No. S9.21 from countries not listed in this provision. |
| S5.371 | Additional allocation: in Region 1, the bands 1 610-1 626.5 MHz (Earth-to-space) and 2 483.5-2 500 MHz (space-to-Earth) are also allocated to the radiodetermination-satellite service on a secondary basis, subject to agreement obtained under No. S9.21. |
| S5.372 | Harmful interference shall not be caused to stations of the radio astronomy service using the band 1 610.6-1 613.8 MHz by stations of the radiodetermination-satellite and mobile-satellite services (No. S29.13 applies). |
| S5.374 | Mobile earth stations in the mobile-satellite service operating in the bands 1 631.5-1 634.5 MHz and 1 656.5-1 660 MHz shall not cause harmful interference to stations in the fixed service operating in the countries listed in No. S5.359. |
| S5.375 | The use of the band 1 645.5-1 646.5 MHz by the mobile-satellite service (Earth-to-space) and for inter-satellite links is limited to distress and safety communications (see Article S31). |
| S5.376 | Transmissions in the band 1 646.5-1 656.5 MHz from aircraft stations in the aeronautical mobile (R) service directly to terrestrial aeronautical stations, or between aircraft stations, are also authorized when such transmissions are used to extend or supplement the aircraft-to-satellite links. |
| S5.376A | Mobile earth stations operating in the band 1 660-1 660.5 MHz shall not cause harmful interference to stations in the radio astronomy service. |
| S5.379A | Administrations are urged to give all practicable protection in the band 1 660.5-1 668.4 MHz for future research in radio astronomy, particularly by eliminating air-to-ground transmissions in the meteorological aids service in the band 1 664.4-1 668.4 MHz as soon as practicable. |
| S5.380 | The bands 1 670-1 675 MHz and 1 800-1 805 MHz are intended for use, on a worldwide basis, by administrations wishing to implement aeronautical public correspondence. The use of the band 1 670-1 675 MHz by stations in the systems for public correspondence with aircraft is limited to transmissions from aeronautical stations and the use of the band 1 800-1 805 MHz is limited to transmissions from aircraft stations. |
| S5.382 | Different category of service: in Saudi Arabia, Armenia, Austria, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Bulgaria, the Congo, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Guinea, Hungary, Iraq, Israel, Jordan, Kazakstan, Kuwait, the Former Yugoslav Republic of Macedonia, Lebanon, Mauritania, Moldova, Mongolia, Oman, Uzbekistan, Poland, Qatar, Syria, Kyrgyzstan, Romania, Russian Federation, Somalia, Tajikistan, Tanzania, Turkmenistan, Ukraine, Yemen and Yugoslavia, the allocation of the band 1 690-1 700 MHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. S5.33), and in the Democratic People’s Republic of Korea, the allocation of the band 1 690-1 700 MHz to the fixed service is on a primary basis (see No. S5.33) and to the mobile, except aeronautical mobile, service on a secondary basis. |
| S5.384A | The bands, or portions of the bands, 1 710-1 885 MHz and 2 500-2 690 MHz, are identified for use by administrations wishing to implement International Mobile Telecommunications-2000 (IMT-2000) in accordance with Resolution 223 (WRC-2000). This identification does not preclude the use of these bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. |
| S5.385 | Additional allocation: the band 1 718.8-1 722.2 MHz is also allocated to the radio astronomy service on a secondary basis for spectral line observations. |
| S5.388 | The bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications-2000 (IMT-2000). Such use does not preclude the use of these bands by other services to which they are allocated. The bands should be made available for IMT-2000 in accordance with Resolution 212 (Rev.WRC-97). (See also Resolution 223 (WRC-2000).) |
| S5.388A | In Regions 1 and 3, the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz and, in Region 2, the bands 1 885-1 980 and 2 110-2 160 MHz may be used by high altitude platform stations as base stations to provide International Mobile Telecommunications-2000 (IMT-2000), in accordance with Resolution 221 (WRC-2000). The use by IMT-2000 applications using high altitude platform stations as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations. |
| S5.389A | The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. S9.11A and to the provisions of Resolution 716 (WRC-95). The use of these bands shall not commence before 1 January 2000; however the use of the band 1 980-1 990 MHz in Region 2 shall not commence before 1 January 2005. |
| S5.391 | In making assignments to the mobile service in the bands 2 025-2 110 MHz and 2 200-2 290 MHz, administrations shall not introduce high-density mobile systems, as described in Recommendation ITU-R SA.1154, and shall take that Recommendation into account for the introduction of any other type of mobile system. |
| S5.392 | Administrations are urged to take all practicable measures to ensure that space-to-space transmissions between two or more non-geostationary satellites, in the space research, space operations and Earth exploration-satellite services in the bands 2 025-2 110 MHz and 2 200-2 290 MHz, shall not impose any constraints on Earth-to-space, space-to-Earth and other space-to-space transmissions of those services and in those bands between geostationary and non-geostationary satellites. |
| S5.398 | In respect of the radiodetermination-satellite service in the band 2 483.5-2 500 MHz, the provisions of No. S4.10 do not apply. |
| S5.399 | In Region 1, in countries other than those listed in No. S5.400, harmful interference shall not be caused to, or protection shall not be claimed from, stations of the radiolocation service by stations of the radiodetermination satellite service. |
| S5.400 | Different category of service: in Angola, Australia, Bangladesh, Burundi, China, Eritrea, Ethiopia, India, the Islamic Republic of Iran, Jordan, Lebanon, Liberia, Libya, Madagascar, Mali, Pakistan, Papua New Guinea, Dem. Rep. of the Congo, Syria, Sudan, Swaziland, Togo and Zambia, the allocation of the band 2 483.5-2 500 MHz to the radiodetermination-satellite service (space-to-Earth) is on a primary basis (see No. S5.33), subject to agreement obtained under No. S9.21 from countries not listed in this provision. |
| S5.402 | The use of the band 2 483.5-2 500 MHz by the mobile-satellite and the radiodetermination-satellite services is subject to the coordination under No. S9.11A. Administrations are urged to take all practicable steps to prevent harmful interference to the radio astronomy service from emissions in the 2 483.5-2 500 MHz band, especially those caused by second-harmonic radiation that would fall into the 4 990-5 000 MHz band allocated to the radio astronomy service worldwide. |
| S5.403 | Subject to agreement obtained under No. S9.21, the band 2 520-2 535 MHz (until 1 January 2005 the band 2 500-2 535 MHz) may also be used for the mobile-satellite (space-to-Earth), except aeronautical mobile-satellite, service for operation limited to within national boundaries. The provisions of No. S9.11A apply. |
| S5.409 | Administrations shall make all practicable efforts to avoid developing new tropospheric scatter systems in the band 2 500-2 690 MHz. |
| S5.410 | The band 2 500-2 690 MHz may be used for tropospheric scatter systems in Region 1, subject to agreement obtained under No. S9.21. |
| S5.411 | When planning new tropospheric scatter radio-relay links in the band 2 500-2 690 MHz, all possible measures shall be taken to avoid directing the antennae of these links towards the geostationary-satellite orbit. |
| S5.413 | In the design of systems in the broadcasting-satellite service in the bands between 2 500 MHz and 2 690 MHz, administrations are urged to take all necessary steps to protect the radio astronomy service in the band 2 690-2 700 MHz. |
| S5.414 | The allocation of the frequency band 2 500-2 520 MHz to the mobile-satellite service (space-to-Earth) shall be effective on 1 January 2005 and is subject to coordination under No. S9.11A. |
| S5.416 | The use of the band 2 520-2 670 MHz by the broadcasting-satellite service is limited to national and regional systems for community reception, subject to agreement obtained under No. S9.21. The power flux-density at the Earth’s surface shall not exceed the values given in Article S21, Table S21-4. |
| S5.418B | Use of the band 2 630-2 655 MHz by non-geostationary-satellite systems for which complete Appendix S4 coordination information, or notification information, has been received after 2 June 2000, is subject to the application of the provisions of No. S9.12. Resolution 539 (WRC-2000) applies. |
| S5.418C | Use of the band 2 630-2 655 MHz by geostationary-satellite networks for which complete Appendix S4 coordination information, or notification information, has been received after 2 June 2000 is subject to the application of the provisions of No. S9.13 with respect to non-geostationary-satellite systems in the broadcasting-satellite service (sound), and No. S22.2 does not apply. Resolution 539 (WRC-2000) applies. |
| S5.419 | The allocation of the frequency band 2 670-2 690 MHz to the mobile-satellite service shall be effective from 1 January 2005. When introducing systems of the mobile-satellite service in this band, administrations shall take all necessary steps to protect the satellite systems operating in this band prior to 3 March 1992. The coordination of mobile-satellite systems in the band shall be in accordance with No. S9.11A. |
| S5.420 | The band 2 655-2 670 MHz (until 1 January 2005 the band 2 655-2 690 MHz) may also be used for the mobile-satellite (Earth-to-space), except aeronautical mobile-satellite, service for operation limited to within national boundaries, subject to agreement obtained under No. S9.21. The coordination under No. S9.11A applies. |
| S5.422 | Additional allocation: in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, Brunei Darussalam, Congo, Côte d’Ivoire, Cuba, Egypt, the United Arab Emirates, Eritrea, Ethiopia, Gabon, Georgia, Guinea, Guinea-Bissau, Iran (Islamic Republic of), Iraq, Israel, Jordan, Lebanon, Malaysia, Mali, Mauritania, Moldova, Mongolia, Nigeria, Oman, Uzbekistan, Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, the Dem. Rep. of the Congo, Romania, the Russian Federation, Somalia, Tajikistan, Tunisia, Turkmenistan, Ukraine, Yemen and Yugoslavia, the band 2 690-2 700 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985. |
| S5.423 | In the band 2 700-2 900 MHz, ground-based radars used for meteorological purposes are authorized to operate on a basis of equality with stations of the aeronautical radionavigation service. |
| S5.425 | In the band 2 900-3 100 MHz, the use of the shipborne interrogator-transponder system (SIT) shall be confined to the sub-band 2 930 -2 950 MHz. |
| S5.426 | The use of the band 2 900-3 100 MHz by the aeronautical radionavigation service is limited to ground-based radars. |
| S5.427 | In the bands 2 900-3 100 MHz and 9 300-9 500 MHz, the response from radar transponders shall not be capable of being confused with the response from radar beacons (racons) and shall not cause interference to ship or aeronautical radars in the radionavigation service, having regard, however, to No. S4.9. |
| S5.438 | Use of the band 4 200-4 400 MHz by the aeronautical radionavigation service is reserved exclusively for radio altimeters installed on board aircraft and for the associated transponders on the ground. However, passive sensing in the earth exploration-satellite and space research services may be authorized in this band on a secondary basis (no protection is provided by the radio altimeters). |
| S5.440 | The standard frequency and time signal-satellite service may be authorized to use the frequency 4 202 MHz for space-to-Earth transmissions and the frequency 6 427 MHz for Earth-to-space transmissions. Such transmissions shall be confined within the limits of  2 MHz of these frequencies, subject to agreement obtained under No. S9.21. |
| S5.441 | The use of the bands 4 500-4 800 MHz (space-to-Earth), 6 725-7 025 MHz (Earth-to-space) by the fixed-satellite service shall be in accordance with the provisions of Appendix S30B. The use of the bands 10.7-10.95 GHz (space-to-Earth), 11.2-11.45 GHz (space-to-Earth) and 12.75-13.25 GHz (Earth-to-space) by geostationary-satellite systems in the fixed-satellite service shall be in accordance with the provisions of Appendix S30B. The use of the bands 10.7-10.95 GHz (space-to Earth), 11.2-11.45 GHz (space-to-Earth) and 12.75-13.25 GHz (Earth-to-space) by a non-geostationary-satellite system in the fixed-satellite service is subject to application of the provisions of No. S9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite system in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the fixed-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-GSO FSS systems and of the complete coordination or notification information, as appropriate, for the GSO networks, and No. S5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated. |
| S5.442 | In the bands 4 825-4 835 MHz and 4 950-4 990 MHz, the allocation to the mobile service is restricted to the mobile, except aeronautical mobile, service. |
| S5.443A | Additional allocation: The band 5 000-5 010 MHz is also allocated to the radionavigation-satellite service (Earth-to-space) on a primary basis. See Resolution 603 (WRC-2000). |
| S5.443B | Additional allocation: The band 5 010-5 030 MHz is also allocated to the radionavigation-satellite service (space-to-Earth) (space-to-space) on a primary basis. In order not to cause harmful interference to the microwave landing system operating above 5 030 MHz, the aggregate power flux-density produced at the Earth’s surface in the band 5 030-5 150 MHz by all the space stations within any radionavigation-satellite service system (space-to-Earth) operating in the band 5 010-5 030 MHz shall not exceed –124.5 dB(W/m2) in a 150 kHz band. In order not to cause harmful interference to the radio astronomy service in the band 4 990-5 000 MHz, the aggregate power flux-density produced in the 4 990-5 000 MHz band by all the space stations within any RNSS (space-to-Earth) system operating in the 5 010-5 030 MHz band shall not exceed the provisional value of –171 dB(W/m2) in a 10 MHz band at any radio astronomy observatory site for more than 2% of the time. For the use of this band, Resolution 604 (WRC-2000) applies. |
| S5.444 | The band 5 030-5 150 MHz is to be used for the operation of the international standard system (microwave landing system) for precision approach and landing. The requirements of this system shall take precedence over other uses of this band. For the use of this band, No. S5.444A and Resolution 114 (WRC-95) apply. |
| S5.444A | Additional allocation: the band 5 091-5 150 MHz is also allocated to the fixed-satellite service (Earth-to-space) on a primary basis. This allocation is limited to feeder links of non-geostationary mobile-satellite systems and is subject to coordination under No. S9.11A. In the band 5 091-5 150 MHz, the following conditions also apply: - prior to 1 January 2010, the use of the band 5 091-5 150 MHz by feeder links of non-geostationary-satellite systems in the mobile-satellite service shall be made in accordance with Resolution 114 (WRC-95); - prior to 1 January 2010, the requirements of existing and planned international standard systems for the aeronautical radionavigation service which cannot be met in the 5 000-5 091 MHz band, shall take precedence over other uses of this band; - after 1 January 2008, no new assignments shall be made to stations providing feeder links of non-geostationary mobile-satellite systems; - after 1 January 2010, the fixed-satellite service will become secondary to the aeronautical radionavigation service. |
| S5.446 | Additional allocation: in the countries listed in Nos. S5.369 and S5.400, the band 5 150-5 216 MHz is also allocated to the radiodetermination-satellite service (space-to-Earth) on a primary basis, subject to agreement obtained under No. S9.21. In Region 2, the band is also allocated to the radiodetermination-satellite service (space-to-Earth) on a primary basis. In Regions 1 and 3, except those countries listed in Nos. S5.369 and S5.400, the band is also allocated to the radiodetermination-satellite service (space-to-Earth) on a secondary basis. The use by the radiodetermination-satellite service is limited to feeder links in conjunction with the radiodetermination-satellite service operating in the bands 1 610-1 626.5 MHz and/or 2 483.5-2 500 MHz. The total power flux-density at the Earth’s surface shall in no case exceed –159 dBW/m2 in any 4 kHz band for all angles of arrival. |
| S5.447A | The allocation to the fixed-satellite service (Earth-to-space) is limited to feeder links of non-geostationary-satellite systems in the mobile-satellite service and is subject to coordination under No. S9.11A. |
| S5.447B | Additional allocation: the band 5 150-5 216 MHz is also allocated to the fixed-satellite service (space-to-Earth) on a primary basis. This allocation is limited to feeder links of non-geostationary-satellite systems in the mobile-satellite service and is subject to provisions of No. S9.11A. The power flux-density at the Earth’s surface produced by space stations of the fixed-satellite service operating in the space-to-Earth direction in the band 5 150-5 216 MHz shall in no case exceed –164 dB(W/m2) in any 4 kHz band for all angles of arrival. |
| S5.447C | Administrations responsible for fixed-satellite service networks in the band 5 150-5 250 MHz operated under Nos. S5.447A and S5.447B shall coordinate on an equal basis in accordance with No. S9.11A with administrations responsible for non-geostationary-satellite networks operated under No. S5.446 and brought into use prior to 17 November 1995. Satellite networks operated under No. S5.446 brought into use after 17 November 1995 shall not claim protection from, and shall not cause harmful interference to, stations of the fixed-satellite service operated under Nos. S5.447A and S5.447B. |
| S5.447D | The allocation of the band 5 250-5 255 MHz to the space research service on a primary basis is limited to active spaceborne sensors. Other uses of the band by the space research service are on a secondary basis. |
| S5.448A | The use of the frequency band 5 250-5 350 MHz by the earth exploration-satellite (active) and space research (active) services shall not constrain the future development and deployment of the radiolocation service. |
| S5.448B | The earth exploration-satellite (active) service operating in the band 5 350-5 460 MHz shall not cause harmful interference to, or constrain the use and development of, the aeronautical radionavigation service. |
| S5.449 | The use of the band 5 350-5 470 MHz by the aeronautical radionavigation service is limited to airborne radars and associated airborne beacons. |
| S5.452 | Between 5 600 MHz and 5 650 MHz, ground-based radars used for meteorological purposes are authorized to operate on a basis of equality with stations of the maritime radionavigation service. |
| S5.455 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Cuba, Georgia, Hungary, Kazakstan, Latvia, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 5 670-5 850 MHz is also allocated to the fixed service on a primary basis. |
| S5.458 | In the band 6 425-7 075 MHz, passive microwave sensor measurements are carried out over the oceans. In the band 7 075-7 250 MHz, passive microwave sensor measurements are carried out. Administrations should bear in mind the needs of the Earth exploration-satellite (passive) and space research (passive) services in their future planning of the bands 6 425-7 025 MHz and 7 075-7 250 MHz. |
| S5.458A | In making assignments in the band 6 700-7 075 MHz to space stations of the fixed-satellite service, administrations are urged to take all practicable steps to protect spectral line observations of the radio astronomy service in the band 6 650-6 675.2 MHz from harmful interference from unwanted emissions. |
| S5.458B | The space-to-Earth allocation to the fixed-satellite service in the band 6 700-7 075 MHz is limited to feeder links for non-geostationary satellite systems of the mobile-satellite service and is subject to coordination under No. S9.11A. The use of the band 6 700-7 075 MHz (space-to-Earth) by feeder links for non-geostationary satellite systems in the mobile-satellite service is not subject to No. S22.2. |
| S5.458C | Administrations making submissions in the band 7 025-7 075 MHz (Earth-to-space) for geostationary-satellite systems in the fixed-satellite service after 17 November 1995 shall consult on the basis of relevant ITU-R Recommendations with the administrations that have notified and brought into use non-geostationary-satellite systems in this frequency band before 18 November 1995 upon request of the latter administrations. This consultation shall be with a view to facilitating shared operation of both geostationary-satellite systems in the fixed-satellite service and non-geostationary-satellite systems in this band. |
| S5.460 | Additional allocation: the band 7 145-7 235 MHz is also allocated to the space research (Earth-to-space) service on a primary basis, subject to agreement obtained under No. S9.21. The use of the band 7 145-7 190 MHz is restricted to deep space; no emissions to deep space shall be effected in the band 7 190-7 235 MHz. |
| S5.461 | Additional allocation: the bands 7 250-7 375 MHz (space-to-Earth) and 7 900-8 025 MHz (Earth-to-space) are also allocated to the mobile-satellite service on a primary basis, subject to agreement obtained under No. S9.21. |
| S5.461A | The use of the band 7 450-7 550 MHz by the meteorological-satellite service (space-to-Earth) is limited to geostationary-satellite systems. Non-geostationary meteorological-satellite systems in this band notified before 30 November 1997 may continue to operate on a primary basis until the end of their lifetime. |
| S5.461B | The use of the band 7 750-7 850 MHz by the meteorological-satellite service (space-to-Earth) is limited to non-geostationary satellite systems. |
| S5.462A | In Regions 1 and 3 (except for Japan), in the band 8 025-8 400 MHz, the earth exploration-satellite service using geostationary satellites shall not produce a power flux-density in excess of the following provisional values for angles of arrival (), without the consent of the affected administration: –174 dB(W/m2) in a 4 kHz band for 0    5 –174 + 0.5 ( – 5) dB(W/m2) in a 4 kHz band for 5    25 –164 dB(W/m2) in a 4 kHz band for 25    90 These values are subject to study under Resolution 124 (WRC-97). |
| S5.463 | Aircraft stations are not permitted to transmit in the band 8 025-8 400 MHz. |
| S5.465 | In the space research service, the use of the band 8 400-8 450 MHz is limited to deep space. |
| S5.469 | Additional allocation: in Armenia, Azerbaijan, Belarus, Bulgaria, Georgia, Hungary, Lithuania, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the band 8 500-8 750 MHz is also allocated to the land mobile and radionavigation services on a primary basis. |
| S5.469A | In the band 8 550-8 650 MHz, stations in the earth exploration-satellite service (active) and space research service (active) shall not cause harmful interference to, or constrain the use and development of, stations of the radiolocation service. |
| S5.470 | The use of the band 8 750-8 850 MHz by the aeronautical radionavigation service is limited to airborne Doppler navigation aids on a centre frequency of 8 800 MHz. |
| S5.472 | In the bands 8 850-9 000 MHz and 9 200-9 225 MHz, the maritime radionavigation service is limited to shore-based radars. |
| S5.473 | Additional allocation: in Armenia, Austria, Azerbaijan, Belarus, Bulgaria, Cuba, Georgia, Hungary, Moldova, Mongolia, Uzbekistan, Poland, Kyrgyzstan, Slovakia, the Czech Republic, Romania, the Russian Federation, Tajikistan, Turkmenistan and Ukraine, the bands 8 850-9 000 MHz and 9 200-9 300 MHz are also allocated to the radionavigation service on a primary basis. |
| S5.474 | In the band 9 200-9 500 MHz, search and rescue transponders (SART) may be used, having due regard to the appropriate ITU-R Recommendation (see also Article S31). |
| S5.475 | The use of the band 9 300-9 500 MHz by the aeronautical radionavigation service is limited to airborne weather radars and ground-based radars. In addition, ground-based radar beacons in the aeronautical radionavigation service are permitted in the band 9 300-9 320 MHz on condition that harmful interference is not caused to the maritime radionavigation service. In the band 9 300-9 500 MHz, ground-based radars used for meteorological purposes have priority over other radiolocation devices. |
| S5.476 | In the band 9 300-9 320 MHz in the radionavigation service, the use of shipborne radars, other than those existing on 1 January 1976, is not permitted until 1 January 2001. |
| S5.476A | In the band 9 500-9 800 MHz, stations in the earth exploration-satellite service (active) and space research service (active) shall not cause harmful interference to, or constrain the use and development of, stations of the radionavigation and radiolocation services. |
| S5.479 | The band 9 975-10 025 MHz is also allocated to the meteorological-satellite service on a secondary basis for use by weather radars. |
| S5.482 | In the band 10.6-10.68 GHz, stations of the fixed and mobile, except aeronautical mobile, services shall be limited to a maximum equivalent isotropically radiated power of 40 dBW and the power delivered to the antenna shall not exceed –3 dBW. These limits may be exceeded subject to agreement obtained under No. S9.21. However, in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Bangladesh, Belarus, China, the United Arab Emirates, Georgia, India, Indonesia, the Islamic Republic of Iran, Iraq, Japan, Kazakstan, Kuwait, Latvia, Lebanon, Moldova, Nigeria, Uzbekistan, Pakistan, the Philippines, Qatar, Syria, Kyrgyzstan, Russian Federation, Tajikistan, Turkmenistan and Ukraine, the restrictions on the fixed and mobile, except aeronautical mobile, services are not applicable. |
| S5.483 | Additional allocation: in Saudi Arabia, Armenia, Azerbaijan, Bahrain, Belarus, Bosnia and Herzegovina, China, Colombia, Korea (Rep. of), Costa Rica, Egypt, the United Arab Emirates, Georgia, Iran (Islamic Republic of), Iraq, Israel, Japan, Jordan, Kazakstan, Kuwait, Latvia, Lebanon, Moldova, Mongolia, Uzbekistan, Qatar, Kyrgyzstan, the Dem. People’s Rep. of Korea, Romania, the Russian Federation, Tajikistan, Turkmenistan, Ukraine, Yemen and Yugoslavia, the band 10.68-10.7 GHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. Such use is limited to equipment in operation by 1 January 1985. |
| S5.484 | In Region 1, the use of the band 10.7-11.7 GHz by the fixed-satellite service (Earth-to-space) is limited to feeder links for the broadcasting-satellite service. |
| S5.484A | The use of the bands 10.95-11.2 GHz (space-to-Earth), 11.45-11.7 GHz (space-to-Earth), 11.7-12.2 GHz (space-to-Earth) in Region 2, 12.2-12.75 GHz (space-to-Earth) in Region 3, 12.5-12.75 GHz (space-to-Earth) in Region 1, 13.75-14.5 GHz (Earth-to-space), 17.8-18.6 GHz (space-to-Earth), 19.7-20.2 GHz (space-to-Earth), 27.5-28.6 GHz (Earth-to-space), 29.5-30 GHz (Earth-to-space) by a non-geostationary-satellite system in the fixed-satellite service is subject to application of the provisions of No. S9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite systems in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the fixed-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-GSO FSS systems and of the complete coordination or notification information, as appropriate, for the GSO networks, and No. S5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated. |
| S5.487 | In the band 11.7-12.5 GHz in Regions 1 and 3, the fixed, fixed-satellite, mobile, except aeronautical mobile, and broadcasting services, in accordance with their respective allocations, shall not cause harmful interference to, or claim protection from, broadcasting-satellite stations operating in accordance with the provisions of the Regions 1 and 3 Plan in Appendix S30. |
| S5.487A | Additional allocation: in Region 1, the band 11.7-12.5 GHz, in Region 2, the band 12.2-12.7 GHz and, in Region 3, the band 11.7-12.2 GHz, are also allocated to the fixed-satellite service (space-to-Earth) on a primary basis, limited to non-geostationary systems and subject to application of the provisions of No. S9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite systems in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the broadcasting-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-GSO FSS systems and of the complete coordination or notification information, as appropriate, for the GSO networks, and No. S5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated. |
| S5.492 | Assignments to stations of the broadcasting-satellite service which are in conformity with the appropriate regional Plan or included in the Regions 1 and 3 List in Appendix S30 may also be used for transmissions in the fixed-satellite service (space-to-Earth), provided that such transmissions do not cause more interference, or require more protection from interference, than the broadcasting-satellite service transmissions operating in conformity with the Plan or the List, as appropriate. |
| S5.497 | The use of the band 13.25-13.4 GHz by the aeronautical radionavigation service is limited to Doppler navigation aids. |
| S5.498A | The Earth exploration-satellite (active) and space research (active) services operating in the band 13.25-13.4 GHz shall not cause harmful interference to, or constrain the use and development of, the aeronautical radionavigation service. |
| S5.501A | The allocation of the band 13.4-13.75 GHz to the space research service on a primary basis is limited to active spaceborne sensors. Other uses of the band by the space research service are on a secondary basis. |
| S5.501B | In the band 13.4-13.75 GHz, the Earth exploration-satellite (active) and space research (active) services shall not cause harmful interference to, or constrain the use and development of, the radiolocation service. |
| S5.502 | In the band 13.75-14 GHz, an earth station in the fixed-satellite service shall have a minimum antenna diameter of 4.5 m and the e.i.r.p. of any emission should be at least 68 dBW and should not exceed 85 dBW. In addition the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW. The protection of assignments to receiving space stations in the fixed-satellite service operating with earth stations that, individually, have an e.i.r.p. of less than 68 dBW shall not impose constraints on the operation of the radiolocation and radionavigation stations operating in accordance with the Radio Regulations. No. S5.43A does not apply. See Resolution 733 (WRC-2000). |
| S5.503 | In the band 13.75-14 GHz, geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 shall operate on an equal basis with stations in the fixed-satellite service; after that date, new geostationary space stations in the space research service will operate on a secondary basis. Until those geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 cease to operate in this band: - the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in geostationary-satellite orbit shall not exceed 71 dBW in the 6 MHz band from 13.772 to 13.778 GHz; - the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in non-geostationary-satellite orbit shall not exceed 51 dBW in the 6 MHz band from 13.772 to 13.778 GHz. Automatic power control may be used to increase the e.i.r.p. density in the 6 MHz band in this frequency range to compensate for rain attenuation, to the extent that the power-flux density at the fixed-satellite service space station does not exceed the value resulting from use by an earth station of an e.i.r.p. of 71 dBW or 51 dBW, as appropriate, in the 6 MHz band in clear-sky conditions. |
| S5.503A | Until 1 January 2000, stations in the fixed-satellite service shall not cause harmful interference to non-geostationary space stations in the space research and Earth exploration-satellite services. After that date, these non-geostationary space stations will operate on a secondary basis in relation to the fixed-satellite service. Additionally, when planning earth stations in the fixed-satellite service to be brought into service between 1 January 2000 and 1 January 2001, in order to accommodate the needs of spaceborne precipitation radars operating in the band 13.793-13.805 GHz, advantage should be taken of the consultation process and the information given in Recommendation ITU-R SA.1071. |
| S5.504 | The use of the band 14-14.3 GHz by the radionavigation service shall be such as to provide sufficient protection to space stations of the fixed-satellite service. |
| S5.511A | The band 15.43-15.63 GHz is also allocated to the fixed-satellite service (space-to-Earth) on a primary basis. Use of the band 15.43-15.63 GHz by the fixed-satellite service (space-to-Earth and Earth-to-space) is limited to feeder links of non-geostationary systems in the mobile-satellite service, subject to coordination under No. S9.11A. The use of the frequency band 15.43-15.63 GHz by the fixed-satellite service (space-to-Earth) is limited to feeder links of non-geostationary systems in the mobile-satellite service for which advance publication information has been received by the Bureau prior to 2 June 2000. In the space-to-Earth direction, the minimum earth station elevation angle above and gain towards the local horizontal plane and the minimum coordination distances to protect an earth station from harmful interference shall be in accordance with Recommendation ITU-R S.1341. In order to protect the radio astronomy service in the band 15.35-15.4 GHz, the aggregate power flux-density radiated in the 15.35-15.4 GHz band by all the space stations within any non-GSO MSS feeder-link (space-to-Earth) system operating in the 15.43-15.63 GHz band shall not exceed the level of –156 dB(W/m2) in a 50 MHz bandwidth, into any radio astronomy observatory site for more than 2% of the time. |
| S5.511C | Stations operating in the aeronautical radionavigation service shall limit the effective e.i.r.p. in accordance with Recommendation ITU-R S.1340. The minimum coordination distance required to protect the aeronautical radionavigation stations (No. S4.10 applies) from harmful interference from feeder-link earth stations and the maximum e.i.r.p. transmitted towards the local horizontal plane by a feeder-link earth station shall be in accordance with Recommendation ITU-R S.1340. |
| S5.511D | Fixed-satellite service systems for which complete information for advance publication has been received by the Bureau by 21 November 1997 may operate in the bands 15.4-15.43 GHz and 15.63-15.7 GHz in the space-to-Earth direction and 15.63-15.65 GHz in the Earth-to-space direction. In the bands 15.4-15.43 GHz and 15.65-15.7 GHz, emissions from a non-geostationary space station shall not exceed the power flux-density limits at the Earth's surface of –146 dB(W/m2/MHz) for any angle of arrival. In the band 15.63-15.65 GHz, where an administration plans emissions from a non-geostationary space station that exceed –146 dB(W/m2/MHz) for any angle of arrival, it shall coordinate under No. S9.11A with the affected administrations. Stations in the fixed-satellite service operating in the band 15.63-15.65 GHz in the Earth-to-space direction shall not cause harmful interference to stations in the aeronautical radionavigation service (No. S4.10 applies). |
| S5.513A | Spaceborne active sensors operating in the band 17.2-17.3 GHz shall not cause harmful interference to, or constrain the development of, the radiolocation and other services allocated on a primary basis. |
| S5.516 | The use of the band 17.3-18.1 GHz by geostationary-satellite systems in the fixed-satellite service (Earth-to-space) is limited to feeder links for the broadcasting-satellite service. The use of the band 17.3-17.8 GHz in Region 2 by systems in the fixed-satellite service (Earth-to-space) is limited to geostationary satellites. For the use of the band 17.3-17.8 GHz in Region 2 by feeder links for the broadcasting-satellite service in the band 12.2-12.7 GHz, see Article S11. The use of the bands 17.3-18.1 GHz (Earth-to-space) in Regions 1 and 3 and 17.8-18.1 GHz (Earth-to-space) in Region 2 by non-geostationary-satellite systems in the fixed-satellite service is subject to application of the provisions of No. S9.12 for coordination with other non-geostationary-satellite systems in the fixed-satellite service. Non-geostationary-satellite systems in the fixed-satellite service shall not claim protection from geostationary-satellite networks in the fixed-satellite service operating in accordance with the Radio Regulations, irrespective of the dates of receipt by the Bureau of the complete coordination or notification information, as appropriate, for the non-GSO FSS systems and of the complete coordination or notification information, as appropriate, for the GSO networks, and No. S5.43A does not apply. Non-geostationary-satellite systems in the fixed-satellite service in the above bands shall be operated in such a way that any unacceptable interference that may occur during their operation shall be rapidly eliminated. |
| S5.519 | Additional allocation: the band 18.1-18.3 GHz is also allocated to the meteorological-satellite service (space-to-Earth) on a primary basis. Its use is limited to geostationary satellites and shall be in accordance with the provisions of Article S21, Table S21-4. |
| S5.520 | The use of the band 18.1-18.4 GHz by the fixed-satellite service (Earth-to-space) is limited to feeder links of geostationary-satellite systems in the broadcasting-satellite service. |
| S5.522A | The emissions of the fixed service and the fixed-satellite service in the band 18.6-18.8 GHz are limited to the values given in Nos. S21.5A and S21.16.2, respectively. |
| S5.522B | The use of the band 18.6-18.8 GHz by the fixed-satellite service is limited to geostationary systems and systems with an orbit of apogee greater than 20 000 km. |
| S5.523A | The use of the bands 18.8-19.3 GHz (space-to-Earth) and 28.6-29.1 GHz (Earth-to-space) by geostationary and non-geostationary fixed-satellite service networks is subject to the application of the provisions of No. S9.11A and No. S22.2 does not apply. Administrations having geostationary-satellite networks under coordination prior to 18 November 1995 shall cooperate to the maximum extent possible to coordinate pursuant to No. S9.11A with non-geostationary-satellite networks for which notification information has been received by the Bureau prior to that date, with a view to reaching results acceptable to all the parties concerned. Non-geostationary-satellite networks shall not cause unacceptable interference to geostationary fixed-satellite service networks for which complete Appendix S4 notification information is considered as having been received by the Bureau prior to 18 November 1995. |
| S5.523B | The use of the band 19.3-19.6 GHz (Earth-to-space) by the fixed-satellite service is limited to feeder links for non-geostationary-satellite systems in the mobile-satellite service. Such use is subject to the application of the provisions of No. S9.11A, and No. S22.2 does not apply. |
| S5.523C | No. S22.2 of the Radio Regulations shall continue to apply in the bands 19.3-19.6 GHz and 29.1-29.4 GHz, between feeder links of non-geostationary mobile-satellite service networks and those fixed-satellite service networks for which complete Appendix S4 coordination information, or notification information, is considered as having been received by the Bureau prior to 18 November 1995. |
| S5.523D | The use of the band 19.3-19.7 GHz (space-to-Earth) by geostationary fixed-satellite service systems and by feeder links for non-geostationary-satellite systems in the mobile-satellite service is subject to the application of the provisions of No. S9.11A, but not subject to the provisions of No. S22.2. The use of this band for other non-geostationary fixed-satellite service systems, or for the cases indicated in Nos. S5.523C and S5.523E, is not subject to the provisions of No. S9.11A and shall continue to be subject to Articles S9 (except No. S9.11A) and S11 procedures, and to the provisions of No. S22.2. |
| S5.523E | No. S22.2 of the Radio Regulations shall continue to apply in the bands 19.6-19.7 GHz and 29.4-29.5 GHz, between feeder links of non-geostationary mobile-satellite service networks and those fixed-satellite service networks for which complete Appendix S4 coordination information, or notification information, is considered as having been received by the Bureau by 21 November 1997. |
| S5.525 | In order to facilitate interregional coordination between networks in the mobile-satellite and fixed-satellite services, carriers in the mobile-satellite service that are most susceptible to interference shall, to the extent practicable, be located in the higher parts of the bands 19.7-20.2 GHz and 29.5-30 GHz. |
| S5.526 | In the bands 19.7-20.2 GHz and 29.5-30 GHz in Region 2, and in the bands 20.1-20.2 GHz and 29.9-30 GHz in Regions 1 and 3, networks which are both in the fixed-satellite service and in the mobile-satellite service may include links between earth stations at specified or unspecified points or while in motion, through one or more satellites for point-to-point and point-to-multipoint communications. |
| S5.527 | In the bands 19.7-20.2 GHz and 29.5-30 GHz, the provisions of No. S4.10 do not apply with respect to the mobile-satellite service. |
| S5.528 | The allocation to the mobile-satellite service is intended for use by networks which use narrow spot-beam antennas and other advanced technology at the space stations. Administrations operating systems in the mobile-satellite service in the band 19.7-20.1 GHz in Region 2 and in the band 20.1-20.2 GHz shall take all practicable steps to ensure the continued availability of these bands for administrations operating fixed and mobile systems in accordance with the provisions of No. S5.524. |
| S5.530 | In Regions 1 and 3, the allocation to the broadcasting-satellite service in the band 21.4-22 GHz shall come into effect on 1 April 2007. The use of this band by the broadcasting-satellite service after that date and on an interim basis prior to that date is subject to the provisions of Resolution 525 (WARC-92). |
| S5.532 | The use of the band 22.21-22.5 GHz by the Earth exploration-satellite (passive) and space research (passive) services shall not impose constraints upon the fixed and mobile, except aeronautical mobile, services. |
| S5.535A | The use of the band 29.1-29.5 GHz (Earth-to-space) by the fixed-satellite service is limited to geostationary-satellite systems and feeder links to non-geostationary-satellite systems in the mobile-satellite service. Such use is subject to the application of the provisions of No. S9.11A, but not subject to the provisions of No. S22.2, except as indicated in Nos. S5.523C and S5.523E where such use is not subject to the provisions of No. S9.11A and shall continue to be subject to Articles S9 (except No. S9.11A) and S11 procedures, and to the provisions of No. S22.2. |
| S5.536 | Use of the 25.25-27.5 GHz band by the inter-satellite service is limited to space research and Earth exploration-satellite applications, and also transmissions of data originating from industrial and medical activities in space. |
| S5.536A | Administrations installing earth exploration-satellite earth stations cannot claim protection from stations in the fixed and mobile services operated by neighbouring administrations. In addition, earth stations operating in the earth exploration-satellite service should take into account Recommendation ITU-R SA.1278. |
| S5.536B | In Germany, Saudi Arabia, Austria, Belgium, Brazil, Bulgaria, China, the Republic of Korea, Denmark, Egypt, United Arab Emirates, Spain, Estonia, Finland, France, Hungary, India, Islamic Republic of Iran, Ireland, Israel, Italy, Jordan, Kenya, Kuwait, Lebanon, Libya, Liechtenstein, Lithuania, Moldova, Norway, Oman, Uganda, Pakistan, the Philippines, Poland, Portugal, Syria, Slovakia, Czech Republic, Romania, the United Kingdom, Singapore, Sweden, Switzerland, Tanzania, Turkey, Viet Nam and Zimbabwe, earth stations operating in the Earth exploration-satellite service in the band 25.5-27 GHz shall not claim protection from, or constrain the use and deployment of, stations of the fixed and mobile services. |
| S5.538 | Additional allocation: the bands 27.500-27.501 GHz and 29.999-30.000 GHz are also allocated to the fixed-satellite service (space-to-Earth) on a primary basis for the beacon transmissions intended for up-link power control. Such space-to-Earth transmissions shall not exceed an equivalent isotropically radiated power (e.i.r.p.) of 10 dBW in the direction of adjacent satellites on the geostationary-satellite orbit. In the band 27.500-27.501 GHz, such space-to-Earth transmissions shall not produce a power flux-density in excess of the values specified in Article S21, Table S21-4 on the Earth’s surface. |
| S5.539 | The band 27.5-30 GHz may be used by the fixed-satellite service (Earth-to-space) for the provision of feeder links for the broadcasting-satellite service. |
| S5.540 | Additional allocation: the band 27.501-29.999 GHz is also allocated to the fixed-satellite service (space-to-Earth) on a secondary basis for beacon transmissions intended for up-link power control. |
| S5.541 | In the band 28.5-30 GHz, the earth exploration-satellite service is limited to the transfer of data between stations and not to the primary collection of information by means of active or passive sensors. |
| S5.541A | Feeder links of non-geostationary networks in the mobile-satellite service and geostationary networks in the fixed-satellite service operating in the band 29.1-29.5 GHz (Earth-to-space) shall employ uplink adaptive power control or other methods of fade compensation, such that the earth station transmissions shall be conducted at the power level required to meet the desired link performance while reducing the level of mutual interference between both networks. These methods shall apply to networks for which Appendix S4 coordination information is considered as having been received by the Bureau after 17 May 1996 and until they are changed by a future competent world radiocommunication conference. Administrations submitting Appendix S4 information for coordination before this date are encouraged to utilize these techniques to the extent practicable. |
| S5.543 | The band 29.95-30 GHz may be used for space-to-space links in the Earth exploration-satellite service for telemetry, tracking, and control purposes, on a secondary basis. |
| S5.544 | In the band 31-31.3 GHz the power flux-density limits specified in Article S21, Table S21-4 shall apply to the space research service. |
| S5.546 | Different category of service: in Saudi Arabia, Armenia, Azerbaijan, Belarus, Egypt, the United Arab Emirates, Spain, Estonia, Finland, Georgia, Hungary, Iran (Islamic Republic of), Israel, Jordan, Latvia, Lebanon, Moldova, Mongolia, Uzbekistan, Poland, Syria, Kyrgyzstan, Romania, the United Kingdom, the Russian Federation, Tajikistan, Turkmenistan, Turkey and Ukraine, the allocation of the band 31.5-31.8 GHz to the fixed and mobile, except aeronautical mobile, services is on a primary basis (see No. S5.33). |
| S5.547 | The bands 31.8-33.4 GHz, 37-40 GHz, 40.5-43.5 GHz, 51.4-52.6 GHz, 55.78-59 GHz and 64-66 GHz are available for high-density applications in the fixed service (see Resolutions 75 (WRC-2000) and 79 (WRC-2000)). Administrations should take this into account when considering regulatory provisions in relation to these bands. Because of the potential deployment of high-density applications in the fixed-satellite service in the bands 39.5-40 GHz and 40.5-42 GHz, administrations should further take into account potential constraints to high-density applications in the fixed service, as appropriate (see Resolution 84 (WRC-2000)). |
| S5.547A | Administrations should take practical measures to minimize the potential interference between stations in the fixed service and airborne stations in the radionavigation service in the 31.8-33.4 GHz band, taking into account the operational needs of the airborne radar systems. |
| S5.551A | In the band 35.5-36.0 GHz, active spaceborne sensors in the earth exploration-satellite and space research services shall not cause harmful interference to, claim protection from, or otherwise impose constraints on operation or development of the radiolocation service, the meteorological aids service and other services allocated on a primary basis. |
| S5. 551AA | In the bands 37.5-40 GHz and 42-42.5 GHz, non-GSO fixed-satellite service systems should employ power control or other methods of downlink fade compensation of the order of 10 dB, such that the satellite transmissions are at power levels required to meet the desired link performance while reducing the level of interference to the fixed service. The use of downlink fade compensation methods are under study by ITU-R (see Resolution 84 (WRC-2000)). |
| S5.551G | In order to protect the radio astronomy service in the band 42.5-43.5 GHz, the aggregate power flux-density in the 42.5-43.5 GHz band produced by all the space stations in any non-GSO FSS (space-to-Earth) or BSS (space-to-Earth) system operating in the 41.5-42.5 GHz band shall not exceed –167 dB(W/m2) in any 1 MHz band at the site of a radio astronomy station for more that 2% of the time. The power flux-density in the band 42.5-43.5 GHz produced by any GSO FSS (space-to-Earth) or BSS (space-to-Earth) station operating in the band 42.0-42.5 GHz shall not exceed –167 dB(W/m2) in any 1 MHz band at the site of a radio astronomy station. These limits are provisional and will be reviewed in accordance with Resolution 128 (Rev.WRC-2000). |
| S5.552 | The allocation of the spectrum for the fixed-satellite service in the bands 42.5-43.5 GHz and 47.2-50.2 GHz for Earth-to-space transmission is greater than that in the band 37.5-39.5 GHz for space-to-Earth transmission in order to accommodate feeder links to broadcasting satellites. Administrations are urged to take all practicable steps to reserve the band 47.2-49.2 GHz for feeder links for the broadcasting-satellite service operating in the band 40.5-42.5 GHz. |
| S5.552A | The allocation to the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz is designated for use by high altitude platform stations. The use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz is subject to the provisions of Resolution 122 (WRC-97). |
| S5.553 | In the bands 43.5-47 GHz and 66-71 GHz, stations in the land mobile service may be operated subject to not causing harmful interference to the space radiocommunication services to which these bands are allocated (see No. S5.43). |
| S5.554 | In the bands 43.5-47 GHz, 66-71 GHz, 95-100 GHz, 123-130 GHz, 191.8-200 GHz and 252-265 GHz, satellite links connecting land stations at specified fixed points are also authorized when used in conjunction with the mobile-satellite service or the radionavigation-satellite service. |
| S5.555 | Additional allocation: the band 48.94-49.04 GHz is also allocated to the radio astronomy service on a primary basis. |
| S5.555A | The band 50.2-50.4 GHz is also allocated, on a primary basis, to the fixed and mobile services until 1 July 2000. |
| S5.556 | In the bands 51.4-54.25 GHz, 58.2-59 GHz and 64-65 GHz, radio astronomy observations may be carried out under national arrangements. |
| S5.556A | Use of the bands 54.25-56.9 GHz, 57-58.2 GHz and 59-59.3 GHz by the inter-satellite service is limited to satellites in the geostationary-satellite orbit. The single-entry power flux-density at all altitudes from 0 km to 1 000 km above the Earth's surface produced by a station in the inter-satellite service, for all conditions and for all methods of modulation, shall not exceed -147 dB(W/m2/100 MHz) for all angles of arrival. |
| S5.557A | In the band 55.78-56.26 GHz, in order to protect stations in the Earth exploration-satellite service (passive), the maximum power density delivered by a transmitter to the antenna of a fixed service station is limited to –26 dB(W/MHz). |
| S5.558 | In the bands 55.78-58.2 GHz, 59-64 GHz, 66-71 GHz, 122.25-123 GHz, 130-134 GHz, 167-174.8 GHz and 191.8-200 GHz, stations in the aeronautical mobile service may be operated subject to not causing harmful interference to the inter-satellite service (see No. S5.43). |
| S5.558A | Use of the band 56.9-57 GHz by inter-satellite systems is limited to links between satellites in geostationary-satellite orbit and to transmissions from non-geostationary satellites in high-Earth orbit to those in low-Earth orbit. For links between satellites in the geostationary-satellite orbit, the single entry power flux-density at all altitudes from 0 km to 1 000 km above the Earth's surface, for all conditions and for all methods of modulation, shall not exceed –147 dB(W/m2/100 MHz) for all angles of arrival. |
| S5.559 | In the band 59-64 GHz, airborne radars in the radiolocation service may be operated subject to not causing harmful interference to the inter-satellite service (see No. S5.43). |
| S5.559A | The band 75.5-76 GHz is also allocated to the amateur and amateur-satellite services on a primary basis until the year 2006. |
| S5.560 | In the band 78-79 GHz radars located on space stations may be operated on a primary basis in the Earth exploration-satellite service and in the space research service. |
| S5.560A | The 81-81.5 GHz band is also allocated to the amateur and amateur-satellite services on a secondary basis. |
| S5.561 | In the band 74-76 GHz, stations in the fixed, mobile and broadcasting services shall not cause harmful interference to stations of the fixed-satellite service or stations of the broadcasting-satellite service operating in accordance with the decisions of the appropriate frequency assignment planning conference for the broadcasting-satellite service. |
| S5.562 | The use of the band 94-94.1 GHz by the Earth exploration-satellite (active) and space research (active) services is limited to spaceborne cloud radars. |
| S5.562A | Transmissions from space stations of the Earth exploration-satellite service (active) that are directed into the main beam of a radio astronomy antenna have the potential to damage some radio astronomy receivers. Space agencies operating the transmitters and the radio astronomy stations concerned should mutually plan their operations so as to avoid such occurrences to the maximum extent possible. |
| S5.562B | Use of this allocation is limited to space-based radio astronomy only. |
| S5.562C | Use of the band 116-122.25 GHz by the inter-satellite service is limited to satellites in the geostationary-satellite orbit. The single-entry power flux-density produced by a station in the inter-satellite service, for all conditions and for all methods of modulation, at all altitudes from 0 km to 1 000 km above the Earth’s surface and in the vicinity of all geostationary orbital positions occupied by passive sensors, shall not exceed –148 dB(W/(m2  MHz)) for all angles of arrival. |
| S5.562E | The allocation to the Earth exploration-satellite service (active) is limited to the band 133.5-134 GHz. |
| S5.562F | In the band 155.5-158.5 GHz, the allocation to the Earth exploration-satellite (passive) and space research (passive) services shall terminate on 1 January 2018. |
| S5.562G | The date of entry into force of the allocation to the fixed and mobile services in the band 155.5-158.5 GHz shall be 1 January 2018. |
| S5.562H | Use of the bands 174.8-182 GHz and 185-190 GHz by the inter-satellite service is limited to satellites in the geostationary-satellite orbit. The single-entry power flux-density produced by a station in the inter-satellite service, for all conditions and for all methods of modulation, at all altitudes from 0 km to 1 000 km above the Earth’s surface and in the vicinity of all geostationary orbital positions occupied by passive sensors, shall not exceed –144 dB(W/(m2  MHz)) for all angles of arrival. |
| S5.563A | In the bands 200-209 GHz, 235-238 GHz, 250-252 GHz and 265-275 GHz, ground-based passive atmospheric sensing is carried out to monitor atmospheric constituents. |
| S5.563B | The band 237.9-238 GHz is also allocated to the Earth exploration-satellite service (active) and the space research service (active) for spaceborne cloud radars only. |
| S5.565 | The frequency band 275-1 000 GHz may be used by administrations for experimentation with, and development of, various active and passive services. In this band a need has been identified for the following spectral line measurements for passive services: - radio astronomy service: 275-323 GHz, 327-371 GHz, 388-424 GHz, 426-442 GHz, 453-510 GHz, 623-711 GHz, 795-909 GHz and 926-945 GHz; - Earth exploration-satellite service (passive) and space research service (passive): 275-277 GHz, 294-306 GHz, 316-334 GHz, 342-349 GHz, 363-365 GHz, 371-389 GHz, 416-434 GHz, 442-444 GHz, 496-506 GHz, 546-568 GHz, 624-629 GHz, 634-654 GHz, 659-661 GHz, 684-692 GHz, 730-732 GHz, 851-853 GHz and 951-956 GHz. Future research in this largely unexplored spectral region may yield additional spectral lines and continuum bands of interest to the passive services. Administrations are urged to take all practicable steps to protect these passive services from harmful interference until the date when the allocation table is established in the above-mentioned frequency band. |

# Annex 2

## National footnotes

| RN001 | Frequencies 78 kHz and 120 kHz with 10 kHz bandwidth can be used by cable multi-channel broadcasting equipment under condition that all necessary measures are undertaken in this equipment to protect it from interference from other radio services. |
| --- | --- |
| RN002 | Within the band 148.5 - 283.5 kHz separate frequencies are used by aeronautical radionavigation service provided that no interference is caused to broadcasting service and safety of flights. |
| RN003 | Frequency band 405 – 415 kHz can be used by aeronautical mobile service on a secondary basis. |
| RN004 | Frequency band 505 - 526.5 kHz can be used by aeronautical mobile service provided that no interference is caused to maritime mobile and aeronautical radionavigation services. |
| RN005 | Within the band 526.5 – 1606.5 kHz separate frequencies can be used by aeronautical radionavigation service provided that no interference is caused to broadcasting service and safety of flights. |
| RN006 | Frequency bands 5900 - 5950 kHz, 7300 - 7350 kHz, 9400 - 9500 kHz, 11600 - 11650 kHz, 12050 - 12100 kHz, 13570 - 13600 kHz, 13800 - 13870 kHz, 15600 - 15800 kHz, 17480 - 17550 kHz and 18900 - 19020 kHz are reserved for broadcasting service in conformity with provisions of the Radio Regulations. Conditions, under which broadcasting service can use these bands, will be adopted additionally. |
| RN007 | Not used |
| RN008 | Not used |
| RN009 | Frequency bands 12330 - 13200 kHz, 16360 - 17410 kHz, 18780 - 18900 kHz and 19680 - 19800 kHz can be used by land mobile stations provided that no interference is caused to maritime mobile service. |
| RN010 | Frequency bands 22720 - 22855 kHz, 25110 - 25210 kHz and 26100 - 26175 kHz can be used by fixed and land mobile services provided that no interference is caused to maritime mobile service. |
| RN011 | Frequency band 23200 - 23350 kHz can be used by land mobile service provided that no interference is caused to fixed and aeronautical mobile services. |
| RN012 | Frequency band 26.96 - 27.41 MHz can be used by low power radiotelephony stations in conformity with ERC Decision ERC/DEC/(96)02. |
| RN013 | Frequencies 26945 kHz and 26960 kHz can be used by alarm systems with power up to 2 W on a secondary basis. |
| RN014 | Frequency channels used for television broadcasting in Moldova (R1..R69): BAND I Carrier frequencies video/sound 48.5 - 56.5 MHz channel 1 TV 49.75 / 56.25 MHz 58.0 - 66.0 MHz channel 2 TV 59.25 / 65.75 MHz BAND II 76.0 - 84.0 MHz channel 3 TV 77.25 / 83.75 MHz 84.0 - 92.0 MHz channel 4 TV 85.25 / 91.75 MHz 92.0 - 100 MHz channel 5 TV 93.25 / 99.75 MHz BAND III 174 – 182 MHz channel 6 TV 175.25 / 181.75 MHz 182 – 190 MHz channel 7 TV 183.25 / 189.75 MHz 190 – 198 MHz channel 8 TV 191.25 / 197.75 MHz 198 – 206 MHz channel 9 TV 199.25 / 205.75 MHz 206 – 214 MHz channel 10 TV 207.25 / 213.75 MHz 214 – 222 MHz channel 11 TV 215.25 / 221.75 MHz 222 – 230 MHz channel 12 TV 223.25 / 229.75 MHz BAND IV 470 - 478 MHz channel 21 TV 471.25 / 477.75 MHz 478 - 486 MHz channel 22 TV 479.25 / 485.75 MHz ................................................ 574 - 582 MHz channel 34 TV 575.25 / 581.75 MHz BAND V 582 - 590 MHz channel 35 TV 583.25 / 589.75 MHz 614 - 622 MHz channel 39 TV 615.25 / 621.75 MHz ................................................ 854 - 862 MHz channel 69 TV 855.25 / 861.75 MHz |
| RN015 | Frequency band 31.25 – 39.25 MHz is an intermediate frequencies band for TV sets (SECAM) (31.5 – intermediate frequency for sound carrier; 38 MHz - intermediate frequency for video carrier). |
| RN016 | TV channels R4 and R5 are in use up to the end of lifetime of the TV transmitters. New R4 or R5 assignments are prohibited. |
| RN017 | Frequency bands 66 – 74 MHz and 87.5 – 108 MHz are designated for mono and stereo FM broadcasting. |
| RN018 | Parts of the bands 110 – 174 MHz and 230 – 1000 MHz can be used for cable broadcasting networks provided that requirements for external interference are respected and no interference is caused to other services operating in accordance with the Table. Use of such networks, satisfying the requirements, can not serve as a basis for any requests with respect to interference caused by other services and can not claim restriction on operation of the latter, except in frequency bands, used in the given area by ether television and FM broadcasting. |
| RN019 | Within frequency bands 150.05 – 156.7625 MHz and 156.8375 – 168.5 MHz it is permitted to use alarm systems on a primary basis with the following conditions: - maximum transmitter power is 5 W; - channel spacing is 12.5 kHz or 25 kHz. - frequencies shall be coordinated in accordance with procedure in force. |
| RN020 | Frequency bands 159.425 – 159.9 MHz paired with 164.025 – 164.5 MHz and 160.825 – 161.4 MHz paired with 165.425 – 166 MHz are designated for non-governmental trunked land mobile systems with the following conditions: - channel spacing is 25 kHz with a perspective transition to the 12.5 kHz channel spacing; - frequency separation Tx/Rx is 4.6 MHz. |
| RN021 | Frequency band 163.2 – 164.2 MHz is used by the land mobile stations of civil aviation in airports to provide technological and internal communications. |
| RN022 | Not used |
| RN023 | Parts of the frequency band 174 – 240 MHz can be used for digital terrestrial broadcasting. |
| RN024 | Within frequency bands 299.6 – 300.0 MHz, 300.525 – 301.125 MHz, 305.825 – 307.0 MHz, 308.0 – 308.4 MHz, 335.6 – 336.0 MHz, 336.525 – 337.125 MHz, 341.825 – 343.0 MHz and 344.0 – 344.4 MHz separate frequencies can be used by non-governmental users subject to agreement with competent state security entities. |
| RN025 | Frequency band 380 – 385 MHz paired with 390 – 395 MHz is designated to be used in conformity with ERC Decision ERC/DEC/(96)01. |
| RN026 | Frequency bands 396 – 406 MHz, 406.1 – 409 MHz and 436 – 449 MHz are used by line-of-sight radio relay links. |
| RN027 | Frequency band 406.1 – 410 MHz can be used by radio astronomy service. |
| RN028 | Within frequency band 410 – 430 MHz priority is given to assigning frequencies to trunked mobile systems under the following conditions: - channel spacing is 25 kHz or 12.5 kHz; - frequency separation Tx/Rx is 10 MHz. |
| RN029 | Frequency band 453.0 –457.5 MHz paired with 463.0 – 467.5 MHz is designated for analog cellular networks (NMT-450 and similar). Network operators can also implement fixed wireless access in these bands. |
| RN030 | In frequency bands 457.4 – 458.2 MHz paired with 467.4 – 468.2 MHz separate frequencies can be assigned for railway communications. (See CEPT Recommendation T/R 22-01). |
| RN031 | In frequency band 726 – 790 MHz separate frequencies are used by radionavigation equipment till the end of its lifetime and implementation of the new systems in other bands. |
| RN032 | Frequency band 824 – 830 MHz paired with 869 – 875 MHz is designated for implementation of cellular mobile system for non-public use. |
| RN033 | Separate frequencies within frequency band 833 – 885 MHz are used by radiolocation stations for air traffic control systems till the end of their lifetime. |
| RN034 | In frequency 864.1 – 868.1 MHz it is permitted to use cordless telephones in conformity with ERC Decision ERC/DEC/(96)18, on a secondary basis. |
| RN035 | It is permitted to use short range devices on a secondary basis in the bands and with technical parameters indicated in corresponding annexes of CEPT Recommendation T/R 70-03 |
| RN036 | Separate frequencies within the frequency band 873.6 – 960 MHz are used by the ground based stations of aeronautical radionavigation service till the end of their lifetime. |
| RN037 | Parts of the band 890 – 914 MHz paired with 935 – 959 MHz are designated for GSM mobile cellular radiotelephony. (See CEPT Recommendation T/R 20-08). Network operators can also implement fixed wireless access in these bands. |
| RN038 | Within frequency bands 914 – 915 MHz paired with 959 – 960 MHz and 30.075 – 30.3 MHz paired with 39.775 – 40 MHz it is permitted to use cordless telephones on a secondary basis with the following parameters: 914 – 915 MHz (remote unit Tx)/959 – 960 MHz (base unit Tx) max. erp = 10 mW channel spacing = 25 kHz frequency separation = 45 MHz capacity = 40 channels 30.075 – 30.3 (base unit Tx)/39.775 – 40 MHz (remote unit Tx) max. erp = 50 mW channel spacing = 25 kHz frequency separation = 9.7 MHz capacity = 10 channels (automatic scanning) |
| RN039 | Frequency 1030 MHz is used by terrestrial air traffic control equipment and frequency 1090 MHz is used by on-board air traffic control equipment |
| RN039A | Frequency bands 1350 – 1375 MHz paired with 1492 – 1517 MHz, 1375 – 1400 MHz paired with 1427 – 1452 MHz, 2025 – 2110 MHz paired with 2200 – 2290 MHz and 10.15 – 10.3 GHz paired with 10.5 – 10.65 GHz are designated for implementation of point-multi-point systems in conformity with CEPT Recommendations CEPT/ERC/REC 13-01, CEPT/ERC/REC 12-05. |
| RN040 | Frequency bands 1550 – 1559 MHz, 1626.5 – 1645.5 MHz and 1646.5 - 1660 MHz are used by radio relay stations provided that no interference is caused to stations of mobile satellite service. |
| RN041 | Frequency band 1559 – 1610 MHz is used by radio relay stations provided that no interference is caused to stations of radionavigation-satellite service. |
| RN042 | In the band 1592 – 1622.5 MHz separate frequencies with 1 MHz bandwidth are used by on-board equipment to avoid aircraft collisions. |
| RN043 | It is permitted to use frequency bands 1610-1626.5 MHz, 1980-2010 MHz, 2170-2200 MHz, 2483.5-2500 MHz by Satellite Personal Communication Systems (S-PCS) in conformity with ERC Decision ERC/DEC(97)03. Ministries and Departments having fixed service systems in the above-mentioned bands shall transfer existing systems in other frequency bands destined to that service by 1.01.2000. |
| RN044 | Frequency band 1670 - 1675 MHz paired with 1800 – 1805 MHz is designated for Terrestrial Flight Telecommunications System (TFTS), in conformity with ERC Decision ERC/DEC(92)01. |
| RN044A | Parts of the band 1710 – 1785 MHz paired with 1805 – 1880 MHz can be used for implementation of Digital Communication System DCS1800 in conformity with ERC Decision ERC/DEC/(95)03. In some localities these parts can be used subject to solving the problem of transferring of existing radio relay links in other bands. |
| RN045 | It is permitted to use Digital European Cordless Telecommunications system (DECT) in the band 1880 – 1900 MHz in conformity with ERC Decision ERC/DEC/(94)03. In some localities these parts can be used subject to solving the problem of transferring of existing radio relay links in other bands. New assignments in the band 1880 – 1900 MHz for other stations are not permitted. |
| RN046 | Frequency bands 1900-1930 MHz, 1965–1980 MHz and 2125-2170 MHz will be used from 1 January 2002 for implementation Universal Mobile Telecommunications System (UMTS) in conformity with ERC Decision ERC/DEC/(97)07. Further spectrum could be made available from the bands in Decides 2 by 1 January 2005 subject to market demand; |
| RN047 | Frequency band 1940 – 2060 MHz can be used by existing aeronautical radionavigation applications (radio altimeters) till the end of their lifetime. Elaboration and purchasing abroad new aeronautical radionavigation applications in this band is prohibited. |
| RN048 | In the frequency bands 2500 – 2700 MHz and 40.5 – 42.5 GHz it is permitted implementation of microwave video distribution systems (MMDS, MVDS). Frequency band 2500 – 2700 MHz can be used on a primary basis subject to agreement between Ministry of Transport and Communications and Ministry of Defense for every location. In case of disagreement from the Ministry of Defense this band will be used in given location on a secondary basis. In the frequency band 2320 – 2400 MHz it is permitted implementation of microwave video distribution systems (MMDS, MVDS), also, in Chisinau on a secondary basis. Frequency band 40.5 – 42.5 GHz is used on a primary basis. |
| RN049 | Parts of the bands 7550 – 7750 MHz and 8500 – 8700 MHz can be used for radio links of fixed and mobile television cameras with transmitter power up to 1 W subject to coordination of locations of these stations in conformity with procedure in force. |
| RN050 | Frequency band 10.2 – 10.68 GHz can be used by low power radiolocation stations, which are using Doppler's effect for measurements of velocity of moving terrestrial objects. |
| RN051 | Separate frequencies in the bands 10.7 – 11.7 GHz, 12.75 – 13.25 GHz and 14.4 – 15.35 GHz can be used for line-of-sight radio relay stations. |
| RN052 | Parts of the frequency band 12.75 – 13.25 GHz can be used by mobile television cameras with transmitter power up to 1 W. |
| RN053 | Separate frequencies in the band 13.56 – 13.62 GHz can be used by low power radiolocation stations, which are using Doppler's effect for measurements of velocity of moving terrestrial objects. |
| RN054 | Frequency band 17.7 – 19.7 GHz can be used by line-of-sight radio relay stations. |
| RN055 | Parts of the band 21.2 – 23.6 GHz can be used by line-of-sight radio relay stations. |

# Annex 3

## Table of frequency bands

## designated for industrial, scientific, medicinal and household applications*

| No. | Nominal frequencies and permitted deviations | Permitted Frequency bands | Permitted applications |
| --- | --- | --- | --- |
| 1 | ## 18 kHz  7.5 % | 16.7 - 19.4 kHz | Industrial |
| 2 | 22 kHz  7.5 % | 20.4 - 23.7 kHz | Industrial, scientific, medicinal |
| 3 | 44 kHz  10 % | 40 - 48 kHz | Industrial, scientific, medicinal |
| 4 | 66 kHz + 12 % - 10 % | 59 - 74 kHz | Industrial, scientific, medicinal |
| 5 | 440 kHz  2.5 % | 429 - 451 kHz | Industrial, scientific, medicinal |
| 6 | 880 kHz  1.0 % | 871 - 889 kHz | Industrial, scientific, medicinal |
| 7 | 1760 kHz  2.5 % | 1720 - 1800 kHz | Industrial, scientific, medicinal |
| 8 | 2640 kHz  1.0 % | 2610 - 2670 kHz | Industrial, medicinal |
| 9 | 5280 kHz  2.5 % | 5150 - 5410 kHz | Industrial, scientific, medicinal |
| 10 | 6780 kHz  0.2 % | 6767 - 6794 kHz | Industrial, scientific, medicinal |
| 11 | 13560 kHz  1.0 % | 13424 - 13696 kHz | Industrial |
| 12 | 13560 kHz  0.05 % | 13553.2 - 13566.8 kHz | Scientific, medicinal |
| 13 | 27120 kHz  1.0 % | 26850 - 27390 kHz | Industrial |
| 14 | 27120 kHz  0.6 % | 26957 - 27283 kHz | Scientific, medicinal |
| 15 | 40.68 MHz  1.0 % | 40.3 - 41.1 MHz | Industrial, scientific |
| 16 | 40.68 MHz  0.05 % | 40.66 - 40.70 MHz | Medicinal |
| 17 | 81.36 MHz  1.0 % | 80.6 - 82.2 MHz | Industrial |
| 18 | 433.92 MHz  0.2 % | 433.05 - 434.79 MHz | Industrial, scientific, medicinal |
| 19 | 915 MHz  1.4 % | 902 - 928 MHz | Industrial, scientific, medicinal |
| 20 | 2450 MHz  2.0 % | 2400 - 2500 MHz | Industrial, scientific, medicinal, household |
| 21 | 5800 MHz  1.3 % | 5725 - 5875 MHz | Industrial, scientific, medicinal |
| 22 | 24.125 GHz  0.5 % | 24.0 – 24.25 GHz | Industrial, scientific, medicinal |
| 23 | 42.3 GHz  2.5 % | 41.3 – 43.4 GHz | Industrial, scientific, medicinal |
| 24 | 46.2 GHz  2.5 % | 45.0 – 47.4 GHz | Industrial, scientific, medicinal |
| 25 | 48.4 GHz  2.5 % | 47.2 – 49.6 GHz | Industrial, scientific, medicinal |
| 26 | 61.25 GHz  0.4 % | 61.0 – 61.5 GHz | Industrial, scientific, medicinal |
| 27 | 122.5 GHz  0.4 % | 122.0 – 123.0 GHz | Industrial, scientific, medicinal |
| 28 | 245 GHz  0.4 % | 244.0 – 246.0 GHz | Industrial, scientific, medicinal |

Order of frequency use

- Frequencies indicated in the table are designated for use in high frequency industrial, scientific, medicinal and household applications on a secondary basis without getting decisions on allocation of these frequencies under the following conditions:

- high frequency applications shall correspond to Norms on permitted levels of industrial interference;

- high frequency applications, on which Norms on permitted levels of industrial interference are not established, shall not cause interference to radioelectronic applications.

- On necessity to use frequencies other than those indicated in the table, it is necessary to get a decision following procedure in force, except for those frequencies used in high frequency applications to which power on the load is less than 5 W without open radiation.

- Frequency 81.36 MHz  1% could be used in high frequency applications under condition that value of interference field from these applications will not exceed 46 dB relative to 1 mkV at the distance of 30 m from the installation.

- This table does not abrogate decisions granted earlier for designing, manufacturing and purchasing abroad of in high frequency industrial, scientific, medicine and household applications, which are using frequencies and permitted deviations other than those mentioned in the table.

........................

* Industrial, scientific and medicinal application (of radio frequency energy): It is work of equipment or devices, which are designated for generation and local utilization of radio frequency energy for industrial, scientific, medicinal, household or similar purposes, except telecommunications.

*	(R): route.

**	(OR): off-route