QUESTION ITU-R 211/1

Unwanted emissions

(1997)

The ITU Radiocommunication Assembly,

considering

a)	that Radiocommunication Task Group 1/3 has studied the effects, measurements and limits to be applied to spurious emissions only;

b)	that unwanted emissions consist of both spurious and out-of-band emissions (Article 1, No. 140 of the Radio Regulations (RR));

c)	that out-of-band emissions are those emissions on a frequency, or frequencies, immediately outside the necessary bandwidth which result from the modulation process (Article 1, No. 138 of the RR);

d)	that spurious emissions are emissions on a frequency, or frequencies, which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information.

Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions (Article 1, No. 139 of the RR);

e)	that considerations of out-of-band emissions and necessary bandwidths are included by necessity in Recommendation ITU-R SM.328 (Spectra and bandwidth of emissions);

f)	that Recommendation ITU-R SM.329 has defined a transition region between out-of-band emissions and spurious emissions in a transmitted radio frequency spectrum;

g)	that considerable work remains to complete, verify and test the Monte Carlo analysis to be utilized to calculate the effects of spurious emissions on victim receivers for all frequency bands and all services, including such diverse factors in the analysis as propagation anomalies, loading of channels, and percentage of time that harmful interference may be present;

h)	that protection of radio astronomy, space research (deep space) and space services using passive sensors from the effects of all unwanted emissions remains as a crucial consideration;

j)	that Recommendation ITU-R SM.326 contains additional information about intermodulation products and their limits,

decides that the following Question should be studied

1	How should the limits of out-of-band emissions be specified (e.g., as power supplied to the antenna transmission line, in terms of power relative to the fundamental, in terms of absolute levels, or in terms of field strength measured at a distance)?

2	What limits should be applied to out-of-band emissions from the transmitters of all services and types of modulation operating on fundamental frequencies in the range of 9 kHz to 300 GHz?

3	Taking into account No. 301 of Article 5, and No. 343 of Article 6 of the RR, should special consideration be given out-of-band emissions near the edge of an allocated band, especially if the allocated band is adjacent to a radio astronomy band (e.g., at 1 660-1 670 MHz)?

4	How can the continuing work on the Monte Carlo model of spurious emission effects on potential victim receivers include recent progress made in other administrations performing this same or similar research, and can it be expanded to include out-of-band effects?

5	How can the conflicting requirements between on the one hand, the active space services and, on the other hand, radio astronomy service and space services using passive sensors be satisfied while preserving the sensitive and long time-frame observations necessary to scientific progress in these passive services from unwanted emissions?

5.1	How can the conflicting requirements between, on the one hand, the space services, and on the other hand, space research service (deep space) be satisfied while preserving the sensitive observations necessary to scientific progress in these services from unwanted emissions?

6	What improvement could be brought to the Recommendation ITU-R SM.329 on spurious emissions in order to foresee the technological evolution of wireless technology and its impact on the protection of present and future radio services?

7	What action could be taken on the determination of Category A limits for space services?

8	What are the impact of the unwanted emission limits on the affected terrestrial and space radio services?

further decides

1	that the results of the above studies should be included in (a) Recommendation(s);

2	that the above studies should be completed by 2003 at the latest.

_______________