REPORT  ITU-R  BT.2042-1

Technologies in the area of extremely high resolution imagery

(Question ITU-R 40/6)

(2004-2006)

# 1	Introduction

Throughout this Report a hierarchy of spatial resolutions, which is recommended in Recommendation ITU-R BT.1201 and also given in Table 1, is adopted to classify spatial resolution of pictures in extremely high resolution imagery (EHRI). The limitation of available technologies in this area used to force us to stay mainly in still (non-real-time picture) image applications for higher resolutions. Recently real-time systems for higher resolution systems are reported though those are still in the experimental stage. Basically real-time applications in this area can be defined in terms of frame repetition rates independent of the spatial resolution hierarchy.

The attention of the reader is drawn to Table 17 where some questions have been raised that need further study.

TABLE  1

A hierarchy of spatial resolution in EHRI

|  | EHRI-0 | EHRI-1 | EHRI-2 | EHRI-3 |
| --- | --- | --- | --- | --- |
| Spatial resolution (number of samples) | 1 920  1 080 | 3 840  2 160 | 5 760  3 240 | 7 680  4 320 |

The hierarchy is based on the well accepted 16:9 picture aspect ratio.

EHRI-1 to 3 are simple integer multiples of EHRI-0 pixel counts, namely 1 920  1 080, in horizontal and vertical directions, i.e. the multiplier is the suffix value plus 1.

The EHRI hierarchy in Table 1 is in spatial domain and is independent of the temporal axis. In the real-time case, images are classified by specifying the frame rate in the temporal axis.

## 1.1	EHRI systems under development in Japan

Recent findings on EHRI technology development have proved that real-time systems in the area of EHRI-1, 2 hierarchy defined in Table 1 are possible. They are still under development and dissemination of devices for EHRI and products to support practical applications is considered to be still several years away. However, an advent of a killer application of EHRI will surely accelerate the development of essential devices and thus system components.

TABLE  2

EHRI hierarchy and major system parameters
EHRI systems under development in Japan (September 2002)

| Affiliation | System parameters |  |  |  |  |  | Developed hardware |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EHRI hierarchy | Aspect ratio | Horizontal resolution (pixels) | Vertical resolution (pixels) | Frame rate (Hz) | Scanning |  |
| CRL and JVC | EHRI-1 | 16:9 | 3 840 | 2 160 | 30/60 | Progressive | Camera and display |
| NTT | EHRI-1 | 16:9 | 3 840 | 2 160 | 24/48/(96) | Progressive | Display |
| NHK | EHRI-1 | 16:9 | 3 840 | 2 160 | 60 | Progressive | Camera (developed one year before in 2001) |
| NHK | EHRI-3 | 16:9 | 7 680 | 4 320 | 60 | Progressive | Camera and display |
| CRL: Communications Research Laboratory JVC: Victor Company of Japan NTT: Nippon Telegraph and Telephone Corporation NHK: Japan Broadcasting Corporation NOTE 1 – The experimental systems are reported to ITU-R as a contribution in September 2002. |  |  |  |  |  |  |  |

CRL and JVC have jointly developed a camera and display system with 2 000 scanning lines called Quadruple HDTV. The camera system employs three CMOS sensors of 3 888  2 192 pixels and outputs the video signals in four channels of HDTV signals. The projector employs three LCD panels of 3 840  2 048 pixels. The light output of the projector is 5 200 lm and the contrast ratio is more than 750:1. The resolution of this system corresponds to 2  2 times of 1 920  1 080 pixels.

NTT has also developed a digital cinema system that can store, transmit, and display images of 2 000 scanning lines, with 10-bit each for R, G, and B components. The projector of the system is the same as that of CRL-JVC. Image sources of the system are 35 mm motion films of 24 Hz and the system operates at a frame rate of 24 Hz or 48 Hz. The projector displays the images with a refresh rate of 96 Hz in order to avoid the flicker disturbance. The resolution of this system also corresponds to 2  2 times of 1 920  1 080 pixels.

NHK has developed an EHRI-3 system including a video camera and a projector display succeeding their previous system based on EHRI-1. In order to realize this system, four panels for both CCD and LCD are employed. As the maximum number of panel pixels currently available is 3 840  2 048 for both CCD and LCD, four panels (two panels for greens, one for red and one for blue) are combined to realize a resolution of 8 k  4 k pixels. The two green panels are arranged by the diagonal-pixel-offset method to achieve the resolution. The resolution of this system corresponds to 4  4 times of 1 920  1 080 pixels.

## 1.2	1 920  1 080/60 Hz progressive technologies in Japan in the year 2003

### 1.2.1	1 920  1 080/60P current technology status

–	Camera system bases on 2/3 inch CCD technology

A 1 920  1 080/60P (60 frames/s) camera with three CCD devices for each RGB colour, has been developed as an experimental progressive scan HDTV camera in NHK of Japan in 2003. The horizontal and vertical resolutions of this camera are about 1 000 TV-lines each, and the vertical MTF (modulation transfer function) response is about 57% on 700 TVL and 30% on 1 000 TVL.

–	60P display devices available as products

It had been long believed that it is difficult to realize 1 080/60 Hz progressive CRT monitors since the response of horizontal deflection of CRT tube needs certain amount of time to settle itself in a stable condition. A novel technique can overcome this problem without changing the response of the deflection circuit of monitors. With a little bit higher response of the video circuit and the use of higher memory readout speed, the picture part of video signal can be squeezed in time domain and will leave a wider horizontal blanking period in the video signal. With this technique 1 920  1 080 60 Hz progressive scanning is realized. A professional monitor product is available from one of the broadcast products manufactures in Japan using this scheme. The scanning specification of the CRT monitor covers not only 24P but also 60P.

–	1 080/60P interface

To make a 1 080/60P system feasible, interface for the system components is considered to be essential. Fortunately there is an SMPTE standard, SMPTE 372M-2002, to use for the links between the equipment. The title of this SMPTE standard is “Dual Link 292M Interface for 1 920  1 080 Picture Raster”. The SMPTE standard uses two HD-SDI connections to transmit 2.970 Gbit/s data. The specification includes 1 920  1 080 60P/4:2:2/10-bit interconnections. Here, each link is specified in Recommendation ITU-R BT.1120 and can carry a 10-bit serial data stream defined in Recommendation ITU-R BT.709.

### 1.2.2	The technologies and products within the foreseeable range

–	Projectors available before the end of 2004

The availability of 1 080/60P projectors is a product planning issue, and not so much a technological issue. The processing speed is a key technological issue for projecting progressive signals. However, this issue is not difficult, and is rather straightforward. It does not require a novel technique to achieve.

The real issue is to develop a projector that meets the demand and the competitive pricing of the market. One of the broadcast products manufactures in Japan is currently planning to release a full 1 920  1 080 projector before the end of 2004. This multi-scan projector covers 50P and 60P projection in its specification.

–	CCD and CMOS devices for 1 080/60P cameras

For acquisition purposes, it is necessary for us to be provided with 60 Hz progressive cameras to have a genuine 60 Hz progressive environment. It is a well-known claim that an optical sensor for the 1 080/60P camera will be realized with the refinement of a current CCD device. Around this frame rate the CMOS optical sensor which can provide higher processing speed need not be required. It is also understood that a camera system with the CMOS device will also be available in parallel with CCD based 60 Hz progressive cameras.

–	Storage devices

The data rate of 1 920  1 080/60 Hz progressive format is two times higher than that of 1 920  1 080/60 Hz interlace. In order to record 1 920  1 080/4:2:2/10 bit/60 Hz progressive signals on tape it is necessary for a digital VTR to handle approximately 1.24  2 Gbps of data for net video only. Compression technology is widely applied to video recording and the picture quality is well accepted. Under the current product line-up of VTRs in several manufactures there are recorders which can record 880 Mbps of net video rate. The combination of these technologies makes a recorder for 1 080/60 Hz progressive quite feasible. One of the broadcast products manufacturers in Japan has released the specifications of a VTR product which is a portable VTR of the HDCAM series of products. The VTR can record 1 920  1 080/4:2:2/10 bit/60 Hz progressive signals with a compression factor of 2.7.

### 1.2.3	Summary

Japan contributed a progress report to the Radiocommunication Study Group 6 block meetings in the year 2002 on the subject of EHRI. In this Report several EHRI systems are reported to be progressive and have adopted the frame rate of 60 Hz. The systems reported are under experiment but several products which support 60 Hz progressive are already available. As the voices of customers accumulate toward 60 Hz progressive applications, it is a natural tendency that the family of products suitable to those applications should increase. There are clear technology trends to respond to such expectation.

### 1.2.4	Bibliograpy

Document 6-9/52, Document 6P/137 – Progress report on extremely high resolution imagery applicable to digital cinema, Contribution by Japan.

Contribution Document to AHG on D-cinema in September 2001 – Ultra-high definition video camera, by NHK Science & Technical Research Laboratories.

SMPTE 372M-2002 – Dual Link 292M Interface for 1 920  1 080 Picture Raster, Society for Motion Picture and Television Engineers.

# 2	Overview of current EHRI technologies

## 2.1	Still and picture-by-picture image processing (current practice in programme making)

It is well known that in films of recent release digital film optical effects are often used intensively and the advanced picture processing makes the films very attractive to the majority of audiences. The digital film optical effects, i.e. electronic processing on film, set a new stage for film-making, efficiently replacing the previous film optical processes by the cost-effective and well-established studio post-production techniques. These are compositing with computer-generated graphics, film matting and compositing by blue-screen keyer, retouching of scenes to remove unwanted landscapes and colour and gradation changes for old and decayed films.

There are several such systems available in the market and they are successfully used. The whole system comprises a CCD film scanner, an output film recorder and a signal processing facility based on high-speed workstations. Workstations and relevant software packages are usually used to realize these effects. The equipment can process film quality pictures in the area of EHRI; that is more than 40 times conventional TV signal resolutions.

## 2.2	Computer graphics (CG)

Various high quality graphic images are generated on computers. The images are generated in non-real-time, and there are no serious problems involved in this technology area. If disk storage capacity to store the images is large enough and a high-speed computer is used, parameters such as spatial resolution, screen aspect ratio, temporal resolution and others, can be set, in principle according to the demands. However, creation of moving images on a real-time basis is difficult to realize with current technology. It depends on the complexity of the image to be produced and the CG technology used. Image generation by a simple CG technology makes some applications, such as virtual reality systems, flight simulators and game machines, possible in real-time.

For current HDTV programme production, approximately 0.25 h is required using an 800 MIPS computer to generate one frame of a human image. If an EHRI-3 level of image is to be produced with the same technology, four hours will be needed to generate a 4  4 times higher resolution image. Availability of huge CPU power in terms of MIPS and an adoption of dedicated graphics engines are always the key for generating high resolution images in CG.

# 3	Technologies and devices for EHRI realization

## 3.1	Display devices

The number of HDTV display monitors for high-grade home use in Japan has begun to increase following the successful introduction of digital satellite broadcasting service for HDTV. The price of such monitors is becoming significantly lower compared to the past.

Personal computers are also becoming popular not only in the office but also in each individual home all over the world. The phenomenon has coincided with the wide penetration of the Internet. The GUI for the “Windows” machines requires much higher display capabilities than VGA (640  480), such as XGA (1 024  768), and SXGA (1 280  1 024). Displays for typical workstations hold a resolution of SXGA or UXGA (1 600  1 200). Toward the year 2005 WUXGA (1 920  1 200) and QXGA (3 200  2 400) TFT liquid crystal display (LCD) monitors will be available in the market and will be used in certain applications.

With the advent of multimedia age and especially after the emergence of interactive applications on TV, requirements for a display have changed. Such a display has to have a characteristic of both TV and PC display. Those two are different in the following respects:

–	Gamma non-linearity of a cathode ray tube (CRT) display is pre-equalized before broadcasting, while pictures generated by a PC do not have any pre-equalization. Simultaneous display of those two different pictures on the same screen is therefore a compromise.

–	Uniformity of picture resolution across the screen is the essential requirement for any PC monitor. On the other hand a TV monitor does not require uniformity but rather requires higher luminosity. Those two characteristics are difficult to maintain on the same display monitors.

–	Generally, TV displays have around 10% over-scan. PC displays do not have any. Besides the CRT, there are several other alternative new display technologies available now. Projection-type displays and panel displays have been developed to provide a larger screen size, which is important for sharing common pictures among a large number of audiences. CRT, liquid crystal on silicon (LCOS), and digital micro-mirror device (DMD) technologies are used for projection-type displays. Plasma display panel (PDP) technology is used widely for flat panel displays. Various sizes of liquid crystal (LC) panel displays are becoming popular. Fifteen inch XGA, 17 inch SXGA, and 24 inch WUXGA LC panel displays are available for computer display. TV display applications of the LC panel are also becoming popular and 28 inch panels are currently available for the applications.

Both CRT and PDP use electro-luminescent effect of G/B/R phosphors. On the other hand, the LC device controls the amount of light which is generated by a light bulb, and the DMD device, by switching mirrors on and off, reflects the light projected on it to a lens block. For colour display, red, blue and green lights are separated from a single light source by “dichroic prism” and are subsequently led to the modulation block of each projector.

### 3.1.1	CRT displays

For CRT displays with an image size of about 20 inches, a resolution of around 1000 lines is achievable at a shadow mask pitch of about 0.3 mm. In high-level workstations, a pitch of 0.15 mm has already been achieved. The mask pitch depends on many technical factors, like the thickness of the mask and manufacturing conditions. With the present technology level the limit is estimated to be about 0.16 mm in the 40 inch size of CRT. Current spot size of the electron beam is around 1-2 mm. To have higher resolution it is necessary to reduce the size of the spot to around 0.5-1 mm.

It is also necessary to increase the driving speed of CRT deflection circuitry. This is achieved by reducing the width of the deflection yoke wire and by lowering the loss at the core. To reduce deflection errors a digital compensation circuit will be necessary.

Table 3 shows typical high-resolution applications of the CRT display and some of the parameters of available products in the market.

TABLE  3

Some CRT display products available for high-resolution applications

| Area of application |  | Medical | Graphic display | HDTV display monitor |
| --- | --- | --- | --- | --- |
| Display | Size | 21 inch | 32 inch | 30 inch |
|  | Aspect ratio |  | 1:1 | 19:6 |
| Pixel number |  | 2 048 2 560 | 2 048  2 048 | 1 920  1 080 |
| Phosphor pitch |  | None colour continuous (black and white) | 0.31 mm |  |
| Contrast ratio |  | 10 bit D/A |  |  |
| Scanning | Horizontal | 186 kHz | 126.8 kHz |  |
|  | Vertical | 72 Hz non-interlace | 60 Hz non-interlace |  |
| Physical size | Depth |  | 1 000 mm |  |
|  | Weight |  | 100 kg |  |

### 3.1.2	Projection-type displays

By way of projecting light there are several technologies and thus product models available on the market. Following are typical examples of various types of the projectors in the area of high resolution applications.

TABLE  4

Some projection-type display products available for high-resolution applications

| Model name | MARQUEE 9500LC | JVC DLA-M4000L | Sony VPL-FE100J | Barco ELM R12 |
| --- | --- | --- | --- | --- |
| Projection type | 3 CRT and 3 proj.-lenses | 3 D-ILA devices 1 600 W Xenon | 3 LC panels 120 W UHP  4 | 3 DLP devices 3 000 W Xenon |
| Light-mod. device | 9 inch CRT | 0.9 inch (1 397, 760) D-ILA | 1.8 inch (1 310  720) Poli-silicon device | 1.1 inch DMD (1 310  720) |
| Resolution | 2 500  2 000 | 1 365  1 024 | 1 280  1 024 | 1 280  1 024 |
| Light output (lumen) | 1 300 | 4 000 | 3 500 | 12 000 |
| Screen size (inch) |  | 60~600 | 40~500 | ~800 |
| Contrast | – | – | – | 500:1 |
| Scanning frequency (horizontal/vertical) | 15~152 kHz/ 38~180 Hz | 15~82 kHz/ 50~78 Hz | 15~100 kHz/ 50~120 Hz |  |
| Power consumption | 650 W | 2 200 W | 770 W | 3 750 W |
| Weight (kg) | 80 | 70 | 34.5 | 143 |

### 3.1.3	Display panels

In the urban area of a heavily populated city, such as Tokyo, a large-size display panel for advertisement or for public announcement is often observed attached to the outside surface of the buildings in a busy square. Although the display is large, it is primarily designed for SDTV or lower quality pictures. The number of light-emitting devices required on the display is the primary limitation.

Plasma display panel technologies have been studied for a long time. After a long survey period, quite recently, 50 inch full colour panels are available and 60 inch panels are announced these days. They have 640-1 024 vertical resolution.

For the LCD of a direct-view type the availability of a larger size liquid crystal panel is a fundamental problem in terms of technology and cost involved. For a high resolution image display, a larger size screen is requested by most of the viewers.

## 3.2	Acquisition technology

### 3.2.1	Electronic picture camera

The marginal spatial resolution of a typical lens system is assumed to be about 100 lines/mm. Therefore, the achievable vertical resolution by a 1 inch lens system (CCD scanning area of 14  7.8 mm) is 7.8  100  2  1 560 lines, and it is considered that an optical system that is larger than 1 inch size would be required in a system above a EHRI-1 level (3 840  2 160). NHK, the public broadcaster of Japan is studying extremely high resolution camera systems. The objective is to realize a camera system producing in excess of 4 000 scanning lines. Table 6 shows specifications for their current camera system under development.

TABLE  5

Some panel display products available for high-resolution applications

|  | PDP | PDP |
| --- | --- | --- |
| Display size (inch) | 42 | 60 |
| Aspect ratio | 16:9 | 16:9 |
| Pixel pitch (mm) | 0.90  0.51 | 0.972  0.972 |
| Number of pixels | 1 024 1 024 | 1 366 768 |
| Quantization levels | 256 | 256 |
| Number of colours (104) | 1 677 | 1 670 |
| Luminance (cd/m2) | 500 | 450 |
| Contrast | 500:1 | More than 500:1 |
| View angle (degrees) | 160 | Not available |
| Power (W) | 250 | Not available |
| Comments | Available products | Under development |

TABLE  6

Intermediate specifications for a future camera system by NHK of Japan

| Horizontal pixel (/line) | 4400 |
| --- | --- |
| Number of vertical lines | 2250 |
| Number of pixels (active) | 800  104 |
| Aspect ratio | 16:9 |
| Frames (/s) | 60 |
| Scanning system | Progressive |
| Imaging system | RGB 3CCD |
| CCD imaging size (mm) | 32.2  17.2 (2.5 inch equivalent) |
| Lens system | Fix focal lens (f: 50 mm) |

Higher resolution requires a smaller pixel size with the same size of image pick-up device. The low sensitivity which comes from a smaller pixel size is alleviated by enlarging the light-receiving surface, adopting a high sensitivity device, and reducing the device noise level. As for the number and size of pixels, 2 million-pixel (2/3 in optical system) CCDs have been available for HDTV television. The wider surface of the image pick-up devices can cover up to EHRI-1 but some new technologies would be required for further increase of resolution. The reduction of the S/N ratio of a camera lowers the compression rate. Thus, lowering the noise level is of prime importance.

### 3.2.2	Telecine

Three different image pick-up methods are currently used in telecine. These are image pick-up tube camera or area sensor, flying spot scanner, and laser scanner. Most of the problems originating in high resolution imagery with these techniques come in real-time telecine operations. If the systems are operating in non-real-time, almost all the problems will disappear because scanning operations can be performed more slowly.

### 3.2.3	Electronic still camera

The image quality of silver salt photography using 35 mm film is almost equivalent to that of the EHRI-1 class. Handling of much higher resolution is possible by enlarging the size of the film used.

A still image CCD of 100 50 mm2 size with 51 million pixels, which corresponds to higher resolution than EHRI-3, has been realized. It has 10 080 horizontal elements and 5 040 vertical elements and can function up to 5 frames/s.

In 2001, 3 million pixel electronic still cameras are widely available in consumer electronics shops.

## 3.3	Transmission technology

### 3.3.1	Optical transmission

In optical transmission using 1.55 μm wavelength a rate of more than 2.5 Gbit/s and a relay distance of more than 100 km per span have been achieved. As the optical transmission system has a very large transmission capacity compared to other transmission schemes, it will form the fundamental transmission infrastructure for digital imaging in future.

Table 7 shows several potentially important fields of concern in developing for optical transmission technology to convey future high bit-rate signals in the EHRI real-time applications. It is obvious that some innovative break-through technologies are needed but dense wavelength division multiplexing (DWDM) technology in optical transmission has already been established. Large capacity optical networks based on DWDM are becoming widely available in various parts of the world.

TABLE  7

Issues on technology development of optical relay transmission

|  | In the case where 150 Mbit/s is the applied transmission ratio for real-time EHRI-0 and 1(1) | In the case where 600 Mbit/s is the applied transmission ratio for real-time EHRI-2 and 3(1) |
| --- | --- | --- |
| Optical relay transmission technology | Optical transmission technique up to 100 Gbit/s | Optical transmission technique up to Tbit/s bit level |
|  | Coherent light wave transmission technology | Coherent light wave transmission technology |
|  | Light modulation technology | Light modulation technology |
|  | DWDM (10 waves) | DWDM (100 waves) |
|  | Light amplification technology | – |
| (1) See Table 15 for definitions of the real-time EHRI-0 -1 -2 -3. |  |  |

### 3.3.2	Satellite broadcasting

As for satellite broadcasting, frequencies from 21.4-22 GHz (600 MHz) assigned by the World Administrative Radio Conference for Dealing with Frequency Allocations in Certain Parts of the Spectrum (Malaga-Torremolinos, 1992) (WARC-92) and/or much higher band may be used. In this case, from the viewpoint of the bandwidth in the travelling-wave tube (TWT), etc., it is possible to realize broadcasting satellite repeaters at about a 300 MHz bandwidth. However, in actual broadcasting, from the aspects of electric power generation and the scale of the satellite, attenuation and atmospheric absorption of the 21 GHz band radiowave must be overcome by changing the radiation power according to the amount of rainfall of each region. The following technical developments are necessary for this purpose:

–	highly efficient, lightweight, and high-output TWT,

–	space synthetic antenna,

–	electric power synthetic technology,

–	radiation power control technology.

### 3.3.3	CATV

Compared with the present analogue transmission over CATV networks, transmission of EHRI signals over CATV will need some of the following new measures:

–	use of multiple analogue TV channels,

–	realization of a high quality transmission channel,

–	much higher speed and broader bands,

–	use of digital and optical technology.

Table 8 lists examples of a possible combination of bandwidth and modulation levels for each member of the EHRI transmission hierarchy.

TABLE  8

Bandwidth and modulation levels for EHRI transmission

| Real-time EHRI transmission hierarchy(1) (after compression) | Combination between a bandwidth and modulation levels |
| --- | --- |
| EHRI-0 (50 Mbit/s) | 12 MHz/64-QAM |
| EHRI-0 and 1 (65-130 Mbit/s) | 24-36 MHz/64-QAM 18 MHz/256-QAM |
| EHRI-2 and 3 (500 Mbit/s) | 100 MHz/256-QAM (optical fibre cable required) |
| (1) See Table 15 for definition of the real-time transmission hierarchy. |  |

## 3.4	Storage technology

### 3.4.1	Tape streamers

The technology trend extrapolated from some of the current tape streamers (8 mm, 1/2 inch) shows that the maximum data storage capacity can be anticipated at around 400 Gbytes and 1 000 Gbytes (see Table 9).

TABLE  9

Maximum data capacity of some tape streamers in the year 2005

| Tape streamer | 8 mm cassette | 1/2 inch cassette |
| --- | --- | --- |
| Available data capacity (Gbytes) | 400 | 1 000 |

Real-time recording of EHRI signals on magnetic tape may not be feasible. Compression is considered to be mandatory to reduce the total amount of data and also the data rate which is otherwise too high to record. Table 10 shows the estimated recording capacity of each data streamer format under consideration.

In Table 10, real-time recording of an EHRI-3 signal clearly indicates that it needs compression whose ratio is higher than 1/30 from the view point of recording capacity. The estimated values are based solely on the total capacity of available media for the streamers. It is also important to consider the data rate for actual recording of the EHRI data streams but this point was left for more detailed discussions.

TABLE  10

Estimated recording capacity of tape streamers by the year 2005

| EHRI hierarchy(1) | Real-time EHRI 60 frames/s bit rate (Gbit/s) | Tape streamer Cassette type | Real-time EHRI (hour) |  |  | Still picture (No. of sheets) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Compression ratio |  |  | Compression ratio |
|  |  |  | 1/60 | 1/30 | 1/4 | 1/10 |
| EHRI-0 2 million pixels | 2.5 4:2:2 10 bit/pixel | 8 mm 1/2 inch | 21.3 53.3 | 10.7 26.8 | 1.4 3.5 | 7.68  105 1.92  106 |
| EHRI-1 8 million pixels | 10 4:2:2 10 bit/pixel | 8 mm 1/2 inch | 5.3 13.3 | 2.67 6.68 | 0.35 0.88 | 1.92  105 4.8  105 |
| EHRI-2 19 million pixels | 40 4:4:4 12 bit/pixel | 8 mm 1/2 inch | 1.3 3.25 | 0.67 1.68 | 0.09 0.23 | 4.8  104 1.2  105 |
| EHRI-3 33 million pixels | 72 4:4:4 12 bit/pixel | 8 mm 1/2 inch | 0.74 1.85 | 0.37 0.93 | 0.05 0.12 | 2.6  104 6.7  104 |
| (1) See Table 15 for definition of the real-time hierarchy. |  |  |  |  |  |  |

### 3.4.2	Disks

The technology trend extrapolated from the current disk technologies shows that four to nine times increase in recording capacity by the year 2005 can be expected. Table 11 indicates available recording capacity for each size disk currently on the market.

TABLE  11

Recording capacity to be obtained by the year 2005

| Storage media | Size (mm) | Current recording capacity (Gbyte) | Future recording capacity (Gbyte) |
| --- | --- | --- | --- |
| MD | 64 | 0.14 | 0.56-1.25 |
| CD-ROM, CD-R | 120 | 0.64 | 2.56-5.76 |
| DVD-ROM, DVD-R | 120 | 4.7 | 18.8-42.3 |

Real-time recording of EHRI signals on disks may not be feasible in terms of recording time and available data rate. Compression is considered to be mandatory to reduce the total amount of data and also the data rate which is otherwise too high to record. Table 12 shows estimated recording capacity for each disk format under consideration.

TABLE  12

| Calculated recording capacity of video disks at the year 2005 EHRI hierarchy(1) | Real-time EHRI 60 frames/s bit rate (Gbit/s) | DiskStorage media | Real-time EHRI (hour) |  |  | Still picture (No. of sheets) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Compression ratio |  |  | Compression ratio |
|  |  |  | 1/60 | 1/30 | 1/4 | 1/10 |
| EHRI-0 2 million pixels | 2.5 4:2:2 10 bit/pixel | MD CD DVD | 0.06 0.3 2.3 | 0.03 0.1 1.1 | – 0.02 0.15 | 2  103 9  103 8.1  104 |
| EHRI-1 8 million pixels | 10 4:2:2 10 bit/pixel | MD CD DVD | 0.01 0.06 0.56 | 0.01 0.03 0.28 | – – 0.04 | 5  102 2  103 2  104 |
| EHRI-2 18 million pixels | 40 4:2:2 12 bit/pixel | MD CD DVD | – 0.02 0.14 | – 0.01 0.07 | – – | 1  102 6  102 5.1  103 |
| EHRI-3 32 million pixels | 72 4:2:2 12 bit/pixel | MD CD DVD | – 0.01 0.08 | – – 0.04 | – – – | 7  10 3  102 2.8  103 |
| (1) See Table 15 for destination of the real-time hierarchy. |  |  |  |  |  |  |

Table 12 shows the recording capacity when technology improvement is expected to be nine times the present level. Table 12 clarifies that in motion images, a compression ratio less than 1/30 will make the recording time too short, and 1/60 compression with EHRI-0 will realize a recording time close to that of current analogue LD.

## 3.5	Coding and image processing technology

### 3.5.1	General

Ultra-definition television in the real-time EHRI category contains enormous amounts of data. While maintaining high image quality, effective and economical reduction of the bit rate to fit to the available bandwidth of transmission and storage media is quite important.

Table 13 shows the magnitude of compression rate expected at each processing stage in the total bit-rate reduction scheme.

TABLE  13

Picture data compression rate of each element in the total compression scheme

| Compression rate in the special frequency domain: discrete cosine transform Compression in the temporal domain: motion compensation Compression by the statistical characteristics of data: variable length coding | 5-10 2-3 1.3-1.5 |
| --- | --- |
| Average compression ratio | 15-30 |

### 3.5.2	MPEG-4 studio profile

MPEG-4 has a wider perspective. It can be applied not only for high compression applications for band-limited transmission based on a new object coding scheme but also for high quality picture compression; i.e. picture compression based on 10/12 bit per pixel coding, 4:4:4 components coding, and higher resolution coding.

Table 14 shows a proposed definition of the levels of the MPEG-4 studio profile.

TABLE  14

Definition of levels of the MPEG-4 studio profiles

| Level |  | Maximum picture size | Maximum total sample rate(1) | Maximum bit rate | Other aspects |
| --- | --- | --- | --- | --- | --- |
| Low (compatible with MPEG-2 high) |  | H: 1 920 pixels V: 1 088 lines Frame rate: 30 Hz | 125 337 600 = 1 920  1 088  30  2 | 300 Mbit/s | 4:2:2 10 bit |
| Main | 422 | H: 2 048 pixels V: 2 048 lines Frame rate: 60 Hz | 250 675 200 = 1 920  1 088  60  2 > 2 048  2 048  30  2 | 600 Mbit/s | 4:2:2 10 bit |
|  | 444 |  | 376 012 800 = 1 920  1 088  60  3 > 2 048  2 048  30  3 | 800 Mbit/s | 4:2:2 4:4:4 (YPbPr and RGB) 10 bit |
| High | 422 | H: 4 096 pixels V: 4 096 lines Frame rate: 120 Hz | 805 306 368 = 4 096  4 096  24  2 > 1 920 1 088 120 2 | 1.2 Gbit/s | 4:2:2 10 bit/12 bit |
|  | 444 |  | 1 207 959 552 = 4 096  4 096  24  3 > 1 920  1 088  120  3 | 2.5 Gbit/s | 4:2:2 4:4:4 (YPbPr and RGB) 10 bit/12 bit |
| (1) The performance rating of MPEG-2 decoder is evaluated by the maximum luminance sample rate. In case of MPEG-4, the total sample rate will be a proper measure for chip performance since MPEG-4 studio profile is likely to handle more chrominance samples as 4:4:4. |  |  |  |  |  |

–	Low level: This level is basically compatible with the MPEG-2 High Level. The difference resides in the 10 bit support in MPEG-4. This level is useful to convert and reuse the assets coded by MPEG-2 HL@4:2:2P in studio applications.

– 	Main level: This level is intended to cover DTV production and telecine applications. The production system for DTV will require the 1 920  1 080  60 progressive form of programme sources. The telecine machine should support higher resolution picture format such as 2048 2 048 24/25/30, 10 bit per pixel.

– 	High level: This level is to support super motion systems and high end telecine format. The super motion system will support 120 Hz in future. The high end telecine machine should cover high resolution formats such as 4 096  4 096  24, 10 or 12 bit.

The proposed structure of MPEG-4 studio profiles can be illustrated in Fig. 1.

The core profile is a minimum set of the studio profile and includes the simple tools for production requirements. This profile should provide the compatibility with MPEG-24:2:2 profile.

Table 15 shows the compression ratio required for transmission for each real-time EHRI image.

Viewers will seldom notice image quality degradation after secondary distribution if the compression ratio is around 15 to 30, as was indicated in Table 13. Additional reduction of the bit rates shall be possible by utilizing human visual sensitivity characteristics or filtering. Therefore, compression up to 1/25 to 1/50 is considered possible to achieve secondary distribution quality. However, as far as the image quality of contribution is concerned, a compression ratio of about 1/6 might be the limit.

In the case of top-level EHRI hierarchy, it is necessary to realize a compression ratio of 300-500 to send the signals through a transmission path. As to this level of compression, some form of technology breakthrough is required. A knowledge-based coding, which is still in the research phase, is one candidate.

TABLE  15

Required compression ratio for transmission

| Image hierarchy | MPEG-2 4:2:2 profile | Real-time EHRI-0 | Real-time EHRI-1 | Real-time EHRI-2 | Real-time EHRI-3 |
| --- | --- | --- | --- | --- | --- |
| Number of effective pixels | 720  512 (for 525) 608 (for 625) | 1 920  1 080 | 3 840  2 160 | 5 760  3 240 | 7 680  4 320 |
| Sampling frequency ratio | 4:2:2 | 4:2:2 | 4:2:2 | 4:4:4 | 4:4:4 |
| Gradation (luminance colour difference) (bit) | 8 | 10 | 10 | 12 | 12 |
| Frame rate/s | 30 | 60 | 60 | 60 | 60 |
| Source signal bit rate (Gbit/s) | 0.216 | 2.5 | 10 | 40 | 72 |
| Transmission rate (Mbit/s) | 5-50 | 60-80 | 100-150 | 150-600 | 150-600 |
| Compression ratio | 20-40 | 30-40 | 70-100 | 70-270 | 120-480 |

# 4	 Relationship between EHRI and LSDI

Large screen digital imagery (LSDI) is a family of digital imagery systems applicable to programmes such as dramas, plays, sporting events, concerts, cultural events, etc., from capture to large screen presentation in high-resolution quality in appropriately equipped theatres, halls, and other venues. For further information about the definition and application of LSDI, refer to Report ITU-R BT.2053 – Large screen digital imagery.

ITU-R has recommended a series of baseband image formats for LSDI applications. Table 16 shows their correspondence with EHRI formats. An expanded hierarchy of LSDI systems uses the image format of EHRI-1(3 840  2 160) or EHRI-3(7 680  4 320) as described in Recommendation ITU-R BT.1769 – Parameter values for an expanded hierarchy of LSDI image formats for production and international programme exchange. Large portions of the technical areas of expanded LSDI and EHRI overlap. The technical information on LSDI, including the expanded one, is also described in Report ITU-R BT.2053.

TABLE 16

Correspondences between EHRI and TV and LSDI formats

| EHRI hierarchy(1) | Image format | TV Recommendation | LSDI Recommendation |
| --- | --- | --- | --- |
| EHRI-0 | 1 920  1 080 | Rec. ITU-R BT.709 | Rec. ITU-R BT.1680 |
| EHRI-1 | 3 840  2 160 |  | Rec. ITU-R BT.1769 |
| EHRI-2 | 5 760  3 240 |  |  |
| EHRI-3 | 7 680  4 320 |  | Rec. ITU-R BT.1769 |
| (1) The classification of EHRI is given in § 1 of this Report. |  |  |  |

# 5	Parameters

TABLE 17

A set of parameters for EHRI

| Parameters | Values |
| --- | --- |
| Screen aspect ratio | 16:9 is the fundamental ratio but other ratio values may also be possible for various applications |
| Spatial resolution | 1 920  1 080 and/or its integer multiples are preferable on 16:9 screens. Squareness of the pixel is also important |
| Temporal resolution |  |
| Gradation | 8 bits for moving images and 10 bits for still images are essential. It may be necessary to utilize 12-bit gradation to correspond with sophisticated signal manipulations such as image compositions, video editing and secondary uses |
| Colorimetry | It seems the colorimetry described in Recommendation ITU-R BT.709 is appropriate for a while, but it may be necessary to utilize a new method which can realize a wider range of colour reproduction |