## ANNEX 2

## Definition of format for submitting path profile information

The path profile to be submitted with data relating to terrestrial links (Tables I and III) are specified in the following way: (preferred: DOS format ASCII File)

| ____________× PFL LAT |  | LON | ...file name, containing path code |
| --- | --- | --- | --- |
| TX CNTRY ___ ___×___ |  | ___×___ | ...country code, latitude, longitude |
| RX CNTRY ___ ___×___ |  | ___×___ | ...country code, latitude, longitude |
| DATE PROFILE TAKEN: |  | _____×___ | ...when was source map/DTBS created |
| SOURCE MAP- SCALE: 1: |  | _______ | ...if profile was retrieved from map |
| SOURCE DTBS-RES. (km): |  | ______ | ...if dig. terrain database was used |
| First point TX or RX: __ |  |  | ...pfl starts at trans or rcv site |
| Total path length (km): |  | _____×__ | ...distance TX to RX in km |
| Number of points: |  | ______ | ...number of points reported |

--------------------------------------------------------------------------

| =Distance= from first point (km) | Ground height amsl (m) | Coverage code (0-99) | Height of cover (m) |  |
| --- | --- | --- | --- | --- |

--------------------------------------------------------------------------

| 0.000 | ____._ | __ | ___ | ...actual profile starts here |
| --- | --- | --- | --- | --- |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |
| ___.___ | ____._ | __ | ___ |  |

Only the first two columns (distance and ground height) are mandatory. The coverage code and the height of cover (trees, buildings) are optional (see Annex 3).

The name of the ASCII file reflects the path code. This is a 4-character table name followed by the four digit path number. For example, for Table III-1, this is a four-digit number preceded by "3-1-". (Exception: Table III-1a, where the file name contains the seven character long path number preceded by "3".)

All path profile files have the extension ".PFL".

The file consists of 15 lines of header information followed by the profile information (one line per data point). The file can be created easily by setting up a spreadsheet with a column width of 12 for the first (A) column and 10 for the second (B), third column (C) and fourth column (D).

The following fields are used:

Profile file name (8 characters + ".PFL")

| Transmitter country code: | – – | (see Annex 1) |
| --- | --- | --- |
| Transmitter latitude: | – – – × – – – – | (decimal degrees; North = positive) |
| Transmitter longitude: | – – – × – – – – | (decimal degrees; 0-360, East = positive) |
| Receiver country code: | – – | (see Annex 1) |
| Receiver latitude: | – – – × – – – – | (decimal degrees; North = positive) |
| Receiver longitude: | – – – × – – – – | (decimal degrees; 0-360, East = positive) |
| Date profile taken: | YYYY.MM.DD | (this relates to the date the map or the digital terrain database was created – if not known, fill in the date when the profile has been extracted) |
| First point TX or RX: | – | (enter T if 0 km means transmit site, enter R if 0 km means receive site) |
| Total path length (km): | – – – – × – | (distance TX to RX in km) |
| Number of points: | – – – – – – | (number of points in the profile) |

-----------------------------------

| =Distance= from first point (km) | Ground height amsl (m) | Coverage code (0-99) |
| --- | --- | --- |
| ------- × ----------- × ------------- |  |  |
| XXXX.XXX XXXX.XX |  |  |

The lines from 15 onwards should only contain numeric information, namely:

1.	the distance from the origin in kilometres with 3 decimals; the decimal point should be located in the 8th character position from the left margin;

2.	the height of the ground (not tree canopy or top of buildings) above mean sea level in metres with 2 decimals; the decimal point should be located in the 20th character position from the left margin;

3.	(optional) the terrain cover classification with a two-digit integer number at positions 30 and 31 from the left margin;

4.	(optional) the height of the terrain cover.