Guide to ITU-R Recommendations for 
broadcast sound production

(under the responsibility of Working Party 6P)

General Recommendations for broadcast sound production

Recommendation ITU-R BS.708 – Determination of the electro-acoustical properties of studio monitor headphones, contains information regarding the following:

–	requirement for amplitude-frequency response;

–	response deviation limits;

–	methods of frequency response measurement.

This Recommendation is intended only as an aid in assessing the electro-acoustic performance of studio monitoring headphones. It does not attempt to advise on the relative merits of loudspeaker monitoring and headphone monitoring, nor on the many other aspects of importance in headphone selection such as cost, comfort, durability, repairability, feedback susceptibility, electrical impedance, electro-acoustical efficiency, consistency between units, non-linear distortion and maximum sound level.

Recommendation ITU-R BS.775 – Multichannel stereophonic sound system with and without accompanying picture, contains the following information:

–	standard geometry for loudspeakers and listening position;

–	specification for 5/2/5 channel encoding/transmission/decoding system which maximizes compatibility with 2-channel and single-channel systems;

–	an optional listening arrangement with enhanced surround sound field coverage using three front loudspeakers and four surround loudspeakers.

This Recommendation may be used as an aid in the design and use of production areas for multichannel sound production and for listening areas for assessment and review of multichannel sound productions.

Recommendation ITU-R BS.1423 – Guidelines for producing multichannel soundtracks using surround matrix techniques, contains information regarding:

–	procedures for quality checking on the matrix encode-decode operation;

–	requirements for monitoring decoders;

–	recommendations for preservation of source recordings and their primary (unmatrixed) recording format;

–	recommendations for labelling of master recordings and copies, both within the programme and on media and/or containers.

This Recommendation is intended to outline good operating practices in multichannel sound production for broadcasting to ensure that technical production quality is maintained throughout the production process from acquisition to distribution, transmission and archiving.

Recommendation ITU-R BS.646 – Source encoding for digital sound signals in broadcasting studios, contains operating parameters and performance guidelines for analogue to digital conversion in sound equipment. It covers sample rates, converter resolution and linearity, sample clock stability and clock synchronization requirements for edit systems and picture synchronization.

This Recommendation is intended as a guide to selection and assessment of performance of digital sound production equipment and digital sound production systems for broadcasting.