| Regional Radiocommunication Conference (RRC-06) (Second Session) Geneva, 15 May – 16 June 2006 |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Document CNG2/2(Rev.1)-E |
|  |  |  | 24 May 2006 |
|  |  |  | English only |

|  | CNG2 |
| --- | --- |
| CNG2 GUIDELINES FOR IMPROVEMENT OF 1ST ITERATION RESULTS |  |

The 1st afternoon session of 22 May 2006 of CNG2 setup the guidelines to improve the results of the first iteration with regards to CNG2

The following proposals were discussed and approved by CNG2. Depending on the interference situation,  it is proposed to proceed with the following actions:

| 1. ERP reduction in cases of interference: | VHF Band | UHF Band |
| --- | --- | --- |
| Large city | <= 35 dBW | <= 40 dBW |
| Medium sized city | <= 25 dBW | <= 30 dBW |
| Small city and border area | < =20 dBW | <= 25 dBW |

2.	Reduction of antenna heights, by limiting the maximum antenna height above ground level: Start by border areas with an antenna height of 75 m, run the exercises and correct the data according to the results and the coverage obtained.

3.	Antenna directivity: in border areas, use directional antennas directed towards the interior of the country.

4.	Administrative Declarations : Submit internal/external administrative declarations, in the cases where there is no real incompatibility, ex. When there is a mountain separating the two stations.

5.	Small exercices: it is recommended to  Administrations wishing to  modify the caractheristics of their  stations, to proceed with the running of exercices to ensure that these modifications contribute to the improvement of the results before submitting them for the 2nd iteration.

6.	Frequency channels  It is also recommended to Administrations to notify for the 2nd iteration the channel that was successfully assigned , instead of specifying the whole band. However, for the stations that did not get a channel, it is recommended to notify the desired band.

7.	Absent Administrations: It was decided to adopt the approach described in document  RRC06/DT/5 (Rev1), and to consider as compatible the requirements of absent administrations that are below a margin of 3 dB.