#### Greenland (country code +299)

- Communication of 7.VII.2017:

Tele-Post, Nuuk, announces the following numbering plan for Greenland.

General information:

•	The capital city of Greenland is Nuuk

•	Local time: UTC –3 hours (Summer time: UTC –2 hours)

•	The country code of Greenland is +299

•	Subscriber number consist of six (6) digits

Six-digit subscriber numbers for fixed and mobile network services:

| Locality fixed network service | Six-digit subscriber numbers | Test numbers* |
| --- | --- | --- |
| Nuuk | 31 xxxx – 37 xxxx | 32 10 00 |
| South | 6 xxxxx | 64 26 29 |
| West | 8 xxxxx | 86 47 55 |
| North/East | 9 xxxxx | 94 33 00 |
| Service numbers | 11x-18x |  |

| Non geographic numbers | Six-digit subscriber numbers | Test numbers* |
| --- | --- | --- |
| Fixed | 38 xxxx – 39 xxxx |  |
| V-SAT | 19 91 xx – 19 92 xx |  |
| Mobile service | 21 xxxx – 29 xxxx 42 xxxx – 49 xxxx 51 xxxx – 59 xxxx | 55 52 99 |

*	An 800 Hz tone will indicate that the call has been completed successfully. 
(The level is 8 dB below test level (–8 dBm0)).

Code screening and number lengths for routing to Greenland:

Option 1:

Code screening up to 1 digit after country code 299 and min/max number length screening

(country code 299 is included in the number length)

| Digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Invalid | 6/6 | 9/9 | 9/9 | 9/9 | 9/11 | 9/9 | 9/9 | 9/9 | 9/9 |

Option 2:

Code screening up to 2 digits after country code 299 and min/max number length screening

(country code 299 is included in the number length)

|  | Second digit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | Invalid *) | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |
| 1 | Invalid | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/9 |
| 2 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 3 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 4 | Invalid | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 5 | 9/11 **) | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 6 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 7 | 9/9 | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |
| 8 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 9 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |

*) 00 is international prefix

**)  Mainly Voice Mail Routing GSM.

Contact:

Tele-Post
Postboks 1002
3900 Nuuk
Tel: +299 341255
Fax: +299 322255
E-mail: roaming@telepost.gl
URL: www.tele.gl

##### Communication of 22.VII.2009:

Tele-Post, Nuuk, announces updates to the National Numbering Plan (NNP) of Greenland.

###### Greenland National Numbering Plan

General information:

	Capital city:	Nuuk

	Local time:	UTC –3 hours (summer time: UTC –2 hours)

	Country code:	+299

	Subscriber number:	six (6) digits

###### Six (6)-digits subscriber numbers for fixed and mobile network services

| Locality (fixed network service)/ (mobile network services) | Six (6)-digits subscriber numbers | Test numbers* |
| --- | --- | --- |
| Nuuk | 31 XX XX – 36 XX XX | 32 10 00 34 10 00 |
| VoIP | 38 XX XX – 39 XX XX |  |
| Nanortalik | 61 XX XX | 61 36 99 |
| Qaqortoq | 64 XX XX | 64 12 29 |
| Narsaq | 66 XX XX | 66 10 13 |
| Paamiut | 68 XX XX | 68 14 99 |
| Ivittuut | 69 1X XX | 69 10 99 |
| Maniitsoq | 81 XX XX | 81 25 55 |
| Kangerlussuaq | 84 XX XX | 84 14 99 |
| Sisimiut | 85 XX XX – 86 XX XX | 86 50 55 |
| Kangaatsiaq | 87 XX XX | 87 10 09 |
| Aasiaat | 89 XX XX | 89 23 99 |
| Qasigannguit | 91 XX XX | 91 13 99 |
| Qeqertasuaq | 92 XX XX | 92 14 99 |
| Ilulissat | 94 XX XX | 94 33 00 |
| Uummannaq | 95 XX XX | 95 13 99 |
| Upernavik | 96 XX XX | 96 13 99 |
| Qaanaaq | 97 XX XX | 97 10 99 |
| Tasiilaq | 98 XX XX | 98 10 99 |
| Locality (fixed network service)/ (mobile network services) | Six (6)-digits subscriber numbers | Test numbers* |
| Illoqqortoormiut | 99 XX XX | 99 10 99 |
| V-SAT | 19 91 00 – 19 92 99 |  |
| Mobile service (GSM) | 22 XX XX – 29 XX XX 42 XX XX – 49 XX XX 52 XX XX – 59 XX XX |  |
| Service numbers | 11X – 18X |  |
| * An 800-Hz tone will indicate that the call has been completed successfully (the level is 8 dB below test level (–8 dBm0)). |  |  |

###### Code screening and number lengths 
for routing to Greenland

Option 1: Code screening up to one (1) digit after country code +299 and minimum/maximum number length screening (the country code +299 is included in the number length).

| Digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Invalid | 6/6 | 9/9 | 9/9 | 9/9 | 9/11 | 9/9 | 9/9 | 9/9 | 9/9 |

Option 2: Code screening up to two (2) digits after country code +299 and minimum/maximum number length screening (the country code +299 is included in the number length).

| ♣ | Second digit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | Invalid*) | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |
| 1 | Invalid | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/6 | 6/9 |
| 2 | Invalid | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 3 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 4 | Invalid | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 5 | 9/11 **) | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |

| ♣ | Second digit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First digit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 6 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 7 | 9/9 | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |
| 8 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| 9 | Invalid | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 | 9/9 |
| *) 0 is the international prefix. **) Maximum length is eleven (11) for Voice Mail Routing GSM. |  |  |  |  |  |  |  |  |  |  |

All numbers are eleven (11) digits, except:

| 299500010 | HLR | Home Location Register |
| --- | --- | --- |
| 299500011 | VLR | Visitor Location Register |
| 299500011 | MSC | Mobile Switching Centre |
| 299500012 | EIR | Equipment Identification Register |
| 299500005 | SMSC | Short Message System Centre |
| 299505050 | Mobilsvar | Voicemail |

Contact:

| Mr Erik Rosing Tele-Post Postboks 1002 3900 NUUK Tel: +299 34 1255 Fax: +299 32 2966 E-mail: ero@tele.gl | Network Operation Centre (NOC) Tel: +299 34 13 77 (direct line) Tel: +299 34 12 55, extension 13 77 (via switchboard) Fax: +299 31 23 47 E-mail: noc@tele.gl |
| --- | --- |