#### Inmarsat (country code +870)

#### Communication of 15.VII.2008:

Inmarsat Global Limited reminds all recognized operating agencies (ROAs) of the cessation of the use of the E.164 codes “871, 872, 873 and 874” effective 31st December 2008. All callers to Inmarsat equipment are now being encouraged to use the E.164 code “870”, with immediate effect.

The following table illustrates the numbering breakdown for all types of Inmarsat equipment that would use the E.164 code “870”.

ROAs are requested to ensure that they are able to accommodate these numbering ranges and confirm to Inmarsat, using the contact below, that they have such routing in place.

Inmarsat numbering, using E.164 country code “870”

| (1) | (2) |  | (3) | (4) |
| --- | --- | --- | --- | --- |
| NDC (National Destination Code) or leading digits of N(S)N (National (Significant) Number) Inmarsat T-digit(s) | N(S)N number length |  | Usage of E.164 Number | Additional information |
|  | Maximum length | Minimum length |  |  |
| 1 | n/a | n/a | Reserved for future use | n/a (was used for Inmarsat-A) |
| 2 | n/a | n/a | Reserved for future use | n/a |
| 30 – 37 | 9 digits | 9 digits | Inmarsat B | Maritime mobile normal calls |
| 38 | 9 digits | 9 digits | Inmarsat B | Land mobile normal calls |
| 39 | 9 digits | 9 digits | Inmarsat B | Land and maritime high-speed data calls |
| 50 – 57 | 9 digits | 9 digits | Inmarsat aeronautical | Primary addressing |
| 58 | 9 digits | 9 digits | Inmarsat aeronautical | Alternate addressing |
| 59 | 9 digits | 9 digits | Inmarsat aeronautical | Special facilities |
| 60 | 9 digits | 9 digits | GAN | Maritime mobile ISDN calls |
| 61 – 67 | 9 digits | 9 digits | Inmarsat M | Maritime mobile normal calls |
| 68 - 69 | 9 digits | 9 digits | Inmarsat M | Land mobile normal calls |
| 70 - 75 | n/a | n/a | Reserved for future use | n/a |
| 76 | 9 digits | 9 digits | Inmarsat mini-M GAN Inmarsat Fleet | Maritime or land mobile normal calls |
| 77 | 9 digits | 9 digits | Inmarsat BGAN Inmarsat Fleet broadband Inmarsat Swift broadband Inmarsat Satellite phone | Circuit switched voice and GPRS |
| 78 | 9 digits | 9 digits | Inmarsat BGAN Inmarsat Fleet broadband Inmarsat Swift broadband Inmarsat Satellite phone | Circuit switched data +3.1 kHz audio |
| 79 | n/a | n/a | Reserved for future use | n/a |
| 8 | n/a | n/a | Reserved for future use | n/a (was used for Inmarsat-A) |
| 9 | n/a | n/a | Reserved for future use | n/a |

Contact:

Mr Chris Wortham
Registered Numbering Contact
Inmarsat Global Limited
99, City Road
LONDON  EC1Y  1AX
United Kingdom
Tel:	+44 20 7728 1017
Fax:	+44 20 7728 1689
E-mail:	chris_wortham@inmarsat.com
URL:	www.inmarsat.com