#### Lesotho (country code +266)

##### Communication of 29.VIII.2006:

The Lesotho Telecommunications Authority (LTA), Maseru, announces the entry into service of the following National Numbering Plan (NNP) in Lesotho.

Introduction

The Lesotho numbering plan is a numbering scheme which is based on two principles, namely:

a)	Network Selection
b)	Service Provision

1.	International Country Code:   266

2.	International Access Code (within Lesotho):   “”  00

3.	Numbering scheme type

Lesotho utilizes a “closed” numbering scheme of uniform eight (8)-digit National (Significant) Number (N(S)N). Also, the Network Destination Codes (NDCs) or Network Identification Codes (NICs) (a.k.a. known as network access codes) select the respective network destinations for traffic routing (except for toll-free, premium rate services and short codes).

At present, three (3) NDCs/levels have been allocated to the three existing network operators, namely:

a)	Level 2 for Telecom Lesotho (TL), usage:	+266 2 XXX XXXX
b)	Level 5 for Vodacom Lesotho (VCL), usage:	+266 5 XXX XXXX
c)	Level 6 for Econet Ezi-Cel Lesotho (EEL), usage:	+266 6 XXX XXXX

###### Table of Allocation

| Level | Application | Comment |
| --- | --- | --- |
| 0 | Access codes | Only “00” (””) currently assigned international access |
| 1 | Special service codes | Used at the discretion of the operator. These are mostly internal to the operator network |
| 2 | NIC assigned to fixed network operator | Telecom Lesotho |
| 3 | – | Reserved for future use |
| 4 | – | Reserved for future use |
| 5 | NIC assigned to 1st mobile network operator | Vodacom Lesotho (Pty) Ltd |
| 6 | NIC assigned to 2nd mobile network operator | Econet Ezi-Cel Lesotho |
| 7 | – | Reserved for future use |
| 8 | Free-phone & shared revenue services |  |
| 9 | Premium rate services |  |

4.	Toll-free/Free phone services

Level 8 is allocated for toll-free numbers/services as in ITU-T Recommendation E.164 and is utilized thus:

a)	800 2 XXXX for Telecom Lesotho
b)	800 5 XXXX for Vodacom Lesotho (Pty) Ltd
c)	800 6 XXXX for Econet Ezi-Cel Lesotho

5.	Short Codes

Level 1 is allocated for short codes services provision by operators.

Contact:

Lesotho Telecommunications Authority (LTA)
Moposo House, 6th Floor
Kingsway Road
P.O. Box 15896
MASERU 100
Lesotho
Tel: 	+266 22 325 595
Fax: 	+266 22 310 984
E-mail: 	admin@lta.org.ls
URL:	www.lta.org.ls