RECOMMENDATION  ITU-R  BR.1375-1*

High-definition television (HDTV) recording

(Question ITU-R 108/11)

(1998-2001)

The ITU Radiocommunication Assembly,

considering

a)	that the common image format, having 1 080  1 920 square pixels, is now recommended as the image capture format for HDTV;

b)	that the digital broadcasting service for HDTV programmes is gaining in momentum. There are several services in operation or under plan in many countries;

c)	that various analogue and digital format video tape recorders (VTRs) for HDTV tape recording have been developed;

d)	that many countries hold large archives of valuable, irreplaceable HDTV programmes, based on the signal formats defined in Recommendation ITU-R BT.709, Part 1;

e)	that analogue HDTV open-reel recording formats are considered to be obsolete for use in programme production today, but the programmes recorded on those formats in the past are often valuable;

f)	that digital recording of HDTV programmes is now widely used in HDTV production and post-production;

g)	that there are several types of equipment available for digital HDTV recording today, that differ in the recording medium, in the recording format, or in the recorder model, e.g.: recorders that use or do not use bit-rate compression, recorders that use open-reel tape or cassettes, deck or portable/camcorder, etc.;

h)	that bit-rate compression technique based on DCT and VLC coding can provide highly efficient methods of recording HDTV programmes, whose source data rate exceeds 1 Gbit/s;

j)	that the operational and handling characteristics of digital HDTV VTRs have improved to the point that the VTRs can be used not only in the studios but also in the field, with a level of convenience similar to that of conventional VTRs;

k)	that HDTV cameras using two-million pixel CCD imaging devices and digital signal processing with advanced VLSI chip(s) can provide high quality HDTV pictures while offering the operating features and small size of conventional cameras;

l)	that the use of 24 frame formats will be used to supplement and augment film production,

recommends

1	that digital HDTV recording should be preferred for HDTV programme origination and production, and for storage for programme exchange;

2	that analogue HDTV tape recordings should preferably be transferred to a digital recording format for postprocessing and archives;

3	that the 1080 x 1920 common image format specified in Recommendation ITU-R BT.709, Part 2 shall be preferred for HDTV recording, while the other image formats specified in Recommendation ITU-R BT.709, Part 1 will retain their value especially in view of the need to ensure the possibility to reuse archived HDTV programme assets;

4	that HDTV programmes intended for extensive and complex postprocessing should preferably be recorded on a digital format that does not use lossy compression, while HDTV programmes intended for limited or no postprocessing should preferably be recorded on a digital format, which may use a modest amount of compression.

NOTE 1 – The Tables in Annex 1 indicate specifications of commercially available recording devices for programme production in the HDTV formats specified in the latest version of Recommendation ITU-R BT.709.

ANNEX  1

Tables 1 to 6 describe the major features and specifications for the available HDTV storage devices:

Table 1:  Tape recorder for 1125/60 (59.94) – Overview

Table 2:  Disk recorder for 1125/60 (59.94) – Overview

Table 3:  HDTV digital VTR for 1125/60 (59.94) – Details

Table 4:  Tape recorder for 1125/50, 1250/50 – Overview

Table 5:  HDTV digital VTR for 1125/50, 1250/50 – Details

Table 6:  HDTV analogue VTR for 1125/60 – Details

TABLE  1

Tape recorder for 1125/60 (59.94) – Overview

| Digital/analogue |  | Digital |  |  |  |  |  |  | Analogue |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compression/non-compression |  | Compression |  |  |  |  | Non-compression |  |  |
| Package type |  | Cassette |  |  |  |  | Cassette | Open reel | Cassette |
| Format |  | A | B |  | C | D | D6 | 1'-C(mod) | UNIHI |
| Commercial name(1) |  | HDCAM(2) | HD-D5 |  | DVCPRO HD | D-9 HD | GBR, DCR | HDD/HDDP | HDV, AU-HD |
| Recording/playback time (maximum) |  | 40/124 min | 23/63/124 min |  | 46 min | 62 min | 64 min | 96 min | 63 min |
|  | Luminance | 23 MHz | 1920 s/line |  | 20 MHz |  | 1920 s/line |  | 20 MHz |
| Video specification | Chrominance | 7 MHz | 960 s/line |  | 10 MHz |  | 960 s/line |  | 7 MHz |
|  | Number of lines | 1080 (1035) | 1080 |  |  |  |  |  | 1036 |
|  | Digital or analogue | PCM |  |  |  |  | PCM |  | PCM |
| Audio specification | Number of channels | 4 |  |  | 8 | 4/8 | 5 stereo pairs | 8 (Digital)  1 (Analogue) | 4 |
|  | Sampling frequency | 48 kHz, 20 bits |  |  | 48 kHz, 16 bits | 48 kHz, 20/16 bits | 48 kHz, 20/24 bits | 48 kHz, 16 bits | 48 kHz, 16 bits |
| Ancillary data |  |  |  |  |  |  |  | 5 VBI lines |  |
| Media | Size of cassette (mm) | S: 156  96 L: 254  145 |  | S: 161  98 M: 212  124 L: 296  167 | 125  78 | 188  104 | 366  206 | 14 inch reel | 205  121.5 |
|  | Substance | Metal particle |  |  |  |  | Metal particle |  | Metal particle |
| Application example |  | Camcorder |  | Portable | Camcorder |  | Studio deck |  | Portable studio deck |
| Notes |  | 1/2 inch Intra field/frame Field editable DCT (1/7) |  | 1/2 inch (D5) Intra field DCT (1/4) | 6.35 mm Intra frame DCT (1/10) | 1/2 inch Intra frame DCT (1/10) | 3/4 inch tape |  | 1/2 inch tape |
| PCM: pulse code modulation VBI: vertical blanking interval S: small, M: medium, L: large (1) Each format as commercially known is listed here. (2) HDCAM is a handy camera-recorder type VTR. Studio decks also exist. |  |  |  |  |  |  |  |  |  |

TABLE  2

Disk recorder for 1125/60 (59.94) – Overview

| TABLE 3 Digital/analogue |  |  | Digital |  |  |  | Analogue |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compression/non-compression |  |  | Compression |  | Non-compression |  | Removable |  |  |
| Handling |  |  | Removable | (Non-removable) | (Removable) | Non-removable |  |  |  |
| Recording and/or playback |  |  | Recording and playback |  |  | Recording and playback | Playback only | Write-once | Recording and playback |
| Format |  |  | MO |  |  | HDD (RAID) | Optical | Wobbling | MO |
| Commercial name(1) |  |  |  |  |  |  | HDL-2000 | HDL-5800 |  |
| Recording/playback time |  |  | 32 min |  |  |  | 15 min (CLV) | 20 min (CLV) |  |
| Video specification |  | Luminance | 44.55 MHz, 8 bits |  |  |  | 20 MHz |  |  |
|  |  | Chrominance | 14.85 MHz, 8 bits |  |  |  | 6 MHz |  |  |
|  |  | Number of lines | 1088 |  |  |  |  |  |  |
| Audio specification |  | Digital or Analogue | PCM |  |  |  | PCM |  |  |
|  |  | Number of channels | 4 |  |  |  | 2 |  |  |
|  |  | Sampling frequency | 48 kHz, 20 bits |  |  |  | 48 kHz, 16 bits |  |  |
| Ancillary data |  |  | 256 Kbytes/frame |  |  |  |  |  |  |
| Media | Size of cassette (mm) |  | 300 |  |  |  | 300 |  |  |
|  | Substance |  | Magnetic coat |  |  |  | Aluminium coated | Alloy coat |  |
| Notes |  |  | Intra frame 94 Mbit/s drive, double sided |  |  |  | Single sided |  |  |
| CLV: constant linear velocity MO: magneto optical HDD (RAID): hard disk drive (redundant array of inexpensive disks) (1) Each format as commercially known is listed here. |  |  |  |  |  |  |  |  |  |

HDTV digital VTR for 1125/60 (59.94) – Details

| Format |  | 1 inch | D-6 | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Commercial name(1) |  | HDD/HDDP | GBR, DCR | HDCAM | HD-D5 | DVCPRO HD | D-9 HD |
| Sampling frequency | Video (MHz) | 74.25 |  |  |  |  |  |
|  | Audio (kHz) | 48 |  |  |  |  |  |
| Quantization | Video (bits) | 8 |  |  | 10/8 | 8 |  |
|  | Audio (bits) | 20 | 20/24 | 20 |  | 16 | 20/16 |
| Number of audio channels |  | 8 (Digital)  1 (Analogue) | 10 | 4 |  | 8 | 4 (20 bits) 8 (16 bits) |
| Compression (video) |  | – | – | Intra field/frame DCT (1/7)(2) | Intra field DCT (1/4) | Intra frame DCT (1/10)(3) |  |
| Channel coding |  | 8-8 map | 8-12 map | S-NRZI | 8-14 map | 24-25 I-NRZI |  |
| Total rate (Mbit/s) |  | 1.188 | 1.212 | 185 | 301 | 167 | 198 |
| Video rate (Mbit/s) |  | 958.5 |  | 140 | 235 | 100 |  |
| Number of recording RF channels |  | 8 |  | 4 (camcorder) /2 (studio) | 4 | 4 (camcorder) /2 (studio) | 4 |
| ECC | Inner | 110, 104 | 227, 211 | 231, 219 | 95, 87 | 85, 77 |  |
|  | Outer | 64, 60 | 254, 240 | 250, 226 | 128, 120 | 149, 138 |  |
| Drum diameter (mm) |  | 134.6 | 96.5 | 81.4 | 76.0 | 21.7 | 62 |
| Drum rotation (rps) |  | 120 | 150 | 45 (camcorder) /90 (studio) | 90 | 149.85 (camcorder) /299.70 (studio) | 75 |
| Number of tracks (/field) |  | 16 | 40 | 6 | 12 | 40/frame | 20/frame |
| Tape speed (mm/s) |  | 805.2 | 497 | 96.8 | 167.228 | 135.28 | 115.474 |
| Track pitch (m) |  | 37 | 22 | 21.7 | 20.0 | 18 | 20.0 |
| Minimum wavelength (m) |  | 0.69 | 0.81 | 0.49 | 0.63 | 0.49 | 0.587 |
| Width of tape (mm) |  | 25.4 | 19.01 | 12.65 |  | 6.35 | 12.65 |
| Media substance |  | Metal particle |  |  |  |  |  |
| Tape Hc (kA/m) |  | 115 | 127 | 135 | 143 | 184 |  |
| Cassette size (mm) |  | 11.75/14 inch reel | S: 172  109 M: 254  150 L: 366  206 | S: 156  96 L: 254  145 | S: 161  98 M: 212  124 L: 296  167 | 125  78 | 188  104 |
| Recording time (min) |  | 63/94 | 8/28/64 | 40/124 | 32/63/124 | 46 | 62 |
| ECC: error correction code (1) Each format as commercially known is listed here. (2) 5/8 Horizontal prefiltering and 1/4.4 DCT. (3) 2/3 Horizontal prefiltering and 1/6.6 DCT. |  |  |  |  |  |  |  |

TABLE  4

Tape recorder for 1125/50, 1250/50 – Overview

| Digital/analogue |  | Digital |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Compression/non-compression |  | Compression |  |  |  | Non-compression |
| Package type |  | Cassette |  |  |  |  |
| Format |  | A | B | C |  | D6 |
| 1125 or 1250 |  | 1125/50 |  |  |  |  |
| Commercial name(1) |  | HDCAM | HD-D5 | DVCPRO HD |  |  |
| Recording/playback time (maximum) |  | 124 min | 27/75/148 min | 46 min |  | 64 min |
| Video specification | Luminance (MHz) | 23 | 30 | 23 |  |  |
|  | Chrominance (MHz) | 7 | 15 | 11 |  |  |
|  | Number of lines | 1080 |  |  |  |  |
| Audio specification | Digital or analogue | PCM |  |  |  |  |
|  | Number of channels | 4 | 8 |  |  | 5 stereo pairs |
|  | Audio sampling | 48 kHz, 24 bits |  | 48 kHz, 16 bits |  |  |
| Ancillary data |  |  |  |  |  |  |
| Media | Size of cassette (mm) | S: 156  96 L: 254  145 | S: 161  98 M: 212  124 L: 296  167 | 125  78 |  | S: 172  109 M: 254  150 L: 366  206 |
|  | Substance | Metal particle |  |  |  |  |
| Application example |  | Camcorder | Portable | Camcorder |  | Studio deck |
| Notes |  | 1/2 inch Intra field/frame Field editable DCT (1/7) | 1/2 inch Intra field DCT (1/4) | 6.35 mm Intra frame DCT (1/6.6) |  | 3/4 inch tape |
| (1) Each format as commercially known is listed here. |  |  |  |  |  |  |

TABLE  5

HDTV digital VTR for 1125/50, 1250/50 – Details

| Format |  |  | D-6 | A | B | C |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1125/1250 | 1250/50 | 1125/50 |  |  |
| Commercial name(1) |  |  |  | HDCAM | HD-D5 | DVCPRO HD |
| Sampling frequency | Video (MHz) |  | 74.25 |  |  |  |
|  | Audio (kHz) |  | 48 |  |  |  |
| Quantization | Video (bits) |  | 8 |  | 8/10 map | 8 |
|  | Audio (bits) |  | 20/24 | 20 | 24 | 16 |
| Number of audio channels |  |  | 10 | 4 | 8 |  |
| Compression (video) |  |  | Non- compression | Intra field/frame DCT (1/7) | Intra field DCT (1/4) | Intra frame DCT (1/10) |
| Channel coding |  |  | 8-12 map | S-NRZI | 8-14 map | 24/25 I-NRZI |
| Total rate (Mbit/s) |  |  | 1.212 | 154 | 269 | 167 |
| Video rate (Mbit/s) |  |  | 958.5 | 117 | 196 | 100 |
| Number of recording RF channels |  |  | 8 | 4/2 | 4 | 4 (camcorder) /2 (studio) |
| ECC | Outer |  | 227, 211 | 231, 219 | 95, 87 | 85, 87 |
|  | Inner |  | 254, 240 | 250, 226 | 128, 120 | 149, 138 |
| Drum diameter (mm) |  |  | 96.5 | 81.4 | 76.0 | 21.7 |
| Drum rotation (rps) |  |  | 150 | 37.5/75 | 75 | 150 (camcorder) /300 (studio) |
| Number of tracks (/field) |  |  | 48 | 6 | 12 | 48/frame |
| Tape speed (mm/s) |  |  | 497 | 80.7 | 139.496 | 135.415 |
| Track pitch (m) |  |  | 22 | 21.7 | 20 | 18 |
| Minimum wavelength (m) |  |  | 0.81 | 0.49 | 0.59 | 0.49 |
| Width of tape (mm) |  |  | 19.01 | 12.65 |  | 6.35 |
| Media substance |  |  | Metal particle |  |  |  |
| Tape Hc (kA/m) |  |  | 127 | 132 | 144 | 184 |
| Cassette size (mm) |  |  | S: 172  109 M: 254  150 L: 366  206 | S: 156  96 L: 254  145 | S:161  98 M: 212  124 L: 296  167 | 125  78 |
| Recording time (min) |  |  | 8/28/64 | 48/149 | 27/75/148 | 46 |
| (1) Each format as commercially known is listed here. |  |  |  |  |  |  |

TABLE  6

HDTV analogue VTR for 1125/60 – Details

| Format |  | 1125/60 |  |
| --- | --- | --- | --- |
| Commercial name(1) |  | 1 inch | UNIHI |
| Video | Luminance (MHz) | 20 |  |
|  | Chrominance (MHz) | 10 | 7 |
| Audio sampling (kHz) |  | – | 48 |
| Number of audio channels |  | 2 analogue channels | 4 |
| Number of recording RF channels |  | 4 | 2 |
| Drum diameter (mm) |  | 134.6 | 76 |
| Drum rotation (rsp) |  | 60 | 90 |
| Number of tracks (/field) |  | 4 | 6 |
| Tape speed (mm/s) |  | 483 | 119.709 |
| Track pitch (m) |  | 89 | 24.8 |
| Width of tape (mm) |  | 25.4 | 12.650 |
| Media substance |  |  | Metal particle |
| Tape Hc (kA/m) |  |  |  |
| Cassette size (mm) |  | – | 205  121.5 |
| Recording time (min) |  | 63 |  |
| (1) Each format as commercially known is listed here. |  |  |  |

*	This Recommendation should be brought to the attention of the International Electrotechnical Commission (IEC).