RECOMMENDATION  ITU-R  BR.1422

OPERATIONAL  PRACTICES  FOR  TELEVISION  USE  OF  FILM  SOUNDTRACKS
ENCODED  WITH  NOISE  REDUCTION  AND  MATRIX  SURROUND

(Question ITU-R 218/10)

(1999)

Rec. ITU-R BR.1422

The ITU Radiocommunication Assembly,

considering

a)	that a large and growing number of 35 mm feature films are produced in multichannel sound;

b)	that a large number of the multichannel feature films are identified as Dolby Stereo or Dolby Stereo SR, and that films identified as Dolby Stereo utilize Dolby A-type noise reduction (companding), and films identified as Dolby Stereo SR use Dolby SR-type noise reduction;

c)	that Dolby Stereo and Dolby Stereo SR systems use the photographic analogue stereo tracks of 35 mm films;

d)	that films marked Dolby Stereo are compatible with expanders of the older Dolby A noise reduction systems;

e)	that films marked Dolby Stereo SR are compatible with the more recent Dolby SR noise reduction systems;

f)	that films using the Dolby Stereo SR yield lower noise in reproduction;

g)	that the stereo audio source can exist either in its original magnetic format or as a photographic analogue stereo track on 35 mm film;

h)	that a large number of broadcasters are transmitting or planning to transmit these feature films to television viewers;

j)	that a large number of home television viewers are equipped to reproduce Dolby multichannel sound in their homes;

k)	that a larger existing base of viewers are still equipped with mono and stereo reproduction equipment, which will be receiving multichannel sound;

l)	that new multichannel sound transmission must be compatible with the existing mono and stereo equipment;

m)	that production of multichannel sound requires correct technological procedures before transmission;

n)	that Recommendation ITU-R BR.1287 gives some general guidance on the use of multichannel film soundtracks in broadcasting;

o)	that the European Broadcasting Union (EBU) Technical Recommendation R59-1999 gives some general guidance on the use of multichannel film soundtracks in broadcasting,

recommends

1	that whenever possible, a stereo magnetic master or magnetic soundtrack should be used for sound reproduction. In the case where only the optical soundtrack copy is available, such a copy should be cleaned before transfer;

2	that as a principle, the reproduction of a stereo photographic soundtrack from a negative film should be avoided because of higher distortion and noise level. Such a track should always be converted into a positive form;

3	that a stereo optical track should always be reproduced using a stereo optical pickup. If only a mono signal is required, it should be derived as a downmix from the two channel stereo signal after the two channels have been individually decoded with the appropriate noise reduction system;

4	that the equipment for reproduction or expansion used corresponds to the type of noise reduction used to produce the film. If two versions of a film are available, it is advisable to always select the copy that yields the lowest noise;

5	that older noise reduction reproduction systems could play back films produced with lower noise yielding systems, the reverse should not be practised;

6	that since television reception does not allow as high a dynamic range as the cinema, if any correction of dynamic range is necessary, it should be done after noise reduction expansion and applied simultaneously on both encoded channels.