RECOMMENDATION  ITU-R  BS.597-1*

Channel spacing for sound broadcasting in band 7 (HF)

(1982-1986)

The ITU Radiocommunication Assembly,

considering

a)	that the coverage area achieved in practice depends among other things, upon the channel spacing;

b)	that there are technical advantages in the adoption of uniform carrier spacing with the nominal carrier frequencies being integral multiples of the channel spacing;

c)	that the intermediate frequency (or frequencies) of receivers should be chosen so as to be an integral multiple of the channel spacing;

d)	that the choice of channel spacing should also take into account the possible adoption of SSB (single sideband) in the future;

e)	that these advantages can only be fully realized if a uniform value for channel spacing is adopted for band 7 (HF),

recommends

1	that a uniform value of 10 kHz should be used worldwide for channel spacing in band 7 (HF);

2	that the nominal carrier frequencies should be an integral multiple of the channel spacing;

3	that interleaving channels with carrier frequencies separated by 5 kHz from those of the main channels may be used for coverage of geographically separated areas;

4	that, wherever possible, the intermediate frequency (or frequencies) in receivers should be an integral multiple of the channel spacing;

5	that a value of 5 kHz should be used worldwide for channel spacing for any SSB system that may be selected for future use in band 7 (HF).

*	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2002 in accordance with Resolution ITU-R 44.