RECOMMENDATION  ITU-R  BT.1117

STUDIO  FORMAT  PARAMETERS  FOR  ENHANCED  16:9  625-LINE  TELEVISION  SYSTEMS
(D-  AND  D2-MAC,  PALPLUS,  ENHANCED  SECAM)

(Question ITU-R 42/11)

(1994)

Rec. ITU-R BT.1117

The ITU Radiocommunication Assembly,

considering

a)	that there are already broadcast services offering 16:9 programmes;

b)	that there are proposals to introduce new systems of television broadcasting with improved quality of picture and sound, including a wider aspect ratio;

c)	that most broadcasting organizations are committed to maintaining a service to their viewers with receiving installations equipped only for terrestrial reception;

d)	that many broadcasting organizations will wish to enhance the quality of their existing services;

e)	that enhancements of existing terrestrial standards must remain compatible with current channel allocations;

f)	that enhancements of existing terrestrial standards must maintain a high degree of picture compatibility;

g)	that the principal enhancements identified as means of delivering improved images and sound by enhanced television emission include:

picture	–	wider aspect ratio,

–	reduced cross effects,

–	ghost cancellation,

–	enhanced resolution;

sound	–	digital, multi-channel sound;

h)	that so far, completely satisfactory decoding of PAL and SECAM encoded signals has not been achieved;

j)	that it is desirable that there be the maximum commonality of studio format parameters for different transmission/emission standards, for example, the MAC/packet systems;

k)	that changes to studio/production format and practices can improve the compatibility of an enhanced television signal and thus facilitate the introduction of enhanced systems;

l)	that the provision of an improved studio format will assist the process of up-conversion to HDTV,

recommends

1.	that organizations intending to produce programmes for enhanced television service should adopt component standards using some, or all, of the techniques given in Annex 1.

invites

1.	administrations to make contributions on this matter with a view to completion of this Recommendation.

ANNEX  1

# 1.	Use of conventional 625-lines production systems

## 1.1	Video signal format

### 1.1.1	Digital systems

The question of whether 625-line, 16:9 broadcast systems, both present and planned, would require any change to digital standards used in production has been studied.

In particular, consideration has been given to the need for an increased sampling frequency of 18 MHz for 16:9, 625-line production.

Given the specifications of the D/D2-MAC, the anticipated performance of the PALplus system and the probable performance of an enhanced SECAM system, some administrations consider that present Recommendations ITU-R BT.601 and ITU-R BT.656 are adequate for 16:9, 625-line productions for these emission systems. Therefore in the present situation in Europe, no action is needed to establish a further 625-line digital component standard.

The Asia-Pacific Broadcasting Union (ABU) is of the view that further consideration should also be given to the extension of Recommendations ITU-R BT.601 and ITU-R BT.656 to encompass an 18 MHz sampling rate for the 16:9 aspect ratio.

### 1.1.2	Analogue systems

Use of analogue component systems has proved to be perfectly appropriate provided that they are routed through equipments with a bandwidth which is greater or equal to that of the emission system that has to be fed. (See Note 1).

Note 1 – In the interim some broadcasters may wish to use analogue composite systems before they can convert their facilities to digital component ones. If so, all reasonable steps shall be taken to use some form of improved, “clean” composite coding and decoding methods.

### 1.1.3	Modified analogue and digital composite format solution

An alternative which has been implemented is to use a modified composite signal format. This exploits the additional bandwidth provided by D2 and D3 digital composite video tape recording formats to provide a luminance bandwidth of 6.6 MHz for PAL equivalent to (a) bandwidth(s) of about 5.0 MHz in a conventional 4:3 system. The method employs a novel form of colour coding that permits absolute freedom from cross effects while maintaining compatibility with existing PAL studio equipment, and Nyquist filtering to maintain picture quality through repeated transcoding to and from component formats.

## 1.2	Adjustments to conventional production equipment

### 1.2.1	Cameras

CCD cameras switchable between 16:9 and 4:3 formats are now commonly available. The new 16:9 sensors have the same image diagonal as the earlier 4:3 sensors. Lenses for the latter format can be used without any change in zoom range (or angle of view). When switching to the 4:3 format the image diagonal is reduced and consequently the angle of view.

Lenses for studio-type cameras compensating for this viewing angle reduction are now available. These lenses have a range extender turret equipped with a unit for reduction of image size in the same ratio as the diagonal is reduced.

When switching between the two formats, the settings of the image enhancer unit must be changed accordingly to maintain optimum image quality.

The scanning of camera tubes can be modified to provide the new aspect ratio. In order to maintain the same lens performance with the 16:9 aspect ratio, the image diagonal must be kept unchanged. To maintain optimum picture quality with the 16:9 aspect ratio, it may be necessary to optimize the adjustments of a number of the camera functions. Some of these adjustments may be time consuming. If the same set of tubes are used for both aspect ratios, it is likely that scanning marks will be visible in the picture.

### 1.2.2	Telecine

For flying spot telecines, the methods of changing between 4:3 and 16:9 operation are simple. Either a change of gate is used (16 mm - Super 16 mm) or the telecine is re-scanned. Problems may arise with raster burning if the same scanning tube is used for both ratios.

Modern types of CCD telecines with a suitable line-array sensor provide an easy and reliable change-over between 4:3 and 16:9 formats. Separate optical blocks together with integrated sizing and zooming facilities can be used for the replay of any of the various current film formats.

### 1.2.3	Processing

Equipment such as vision mixers, digital video effect generators (DVEs), caption generators and graphic systems can be used with the 16:9 aspect ratio without any modification to the hardware. Only the software must be updated to accommodate the geometry of the new image format.

### 1.2.4	Video tape recorders

In the case of analogue component VTRs the bandwidth is about 5.5 MHz. Taking into account the change of aspect ratio, this becomes equivalent to a resolution of about 4 MHz in a conventional system with a 4:3 aspect ratio. This resolution is deemed to be sufficient for the enhanced emission systems considered here. Consequently existing analogue component VTRs can be used for this application.

In the case of digital component VTRs with 720 samples per active line, the resolution is the same as the one envisaged for the emission systems considered here. Consequently existing digital component VTRs are perfectly able to be used for this application in Europe.

ABU is of the view that consideration should also be given to the adoption of 960 samples per active line.

In the case of digital D2 and D3 composite recorders, bandwidths in excess of 8.8 MHz for PAL are available for a single composite signal. The video bandwidth achievable using these formats depends on the nature of the modified composite signal described in § 1.1.3.

### 1.2.5	Monitoring

Picture monitors with 4:3 aspect ratio CRTs may possibly be adjusted to give a letter-box 16:9 aspect ratio picture. It is likely that the monitor must be completely realigned if such a scanning readjustment is implemented.

Picture monitors switchable between the two aspect ratios are now available. A requirement for these monitors must be that no realignment is necessary when switching between the formats.

Waveform monitoring equipment can be used for both aspect ratios without any modifications.

# 2.	Down conversion from HDTV sources

The use of HDTV equipment gives a single source which can be used for all the 16:9 services currently envisaged, from enhanced to high definition.

Whilst for enhanced definition services this may have substantially more “headroom” than required, this approach has the advantage that only one step of re-equipment is likely to be needed. Thus the recorded programmes would have adequate quality for the near-term analogue and the long-term digital HDTV as well as for 35 mm film distribution and archival purposes.

The HDTV studio source would be down converted to match the input requirement of the enhanced TV system encoders. Such a method of production and down conversion has already been successfully implemented.

The advantages of using HDTV studio sources for enhanced services, compared with lower definition sources, depend on the time-scales for the establishment of the studio facilities and the implementation of HDTV services.

The choice may also be influenced by the range of services for which the programmes are likely to be used, i.e. solely for one particular enhanced service or for a number of services at various levels of definition.

# 3.	Improved film production methods

Film offers a suitable recording and storage medium for future productions of wide screen programmes.

For the production of 625/50 programmes in the 16:9 aspect ratio on film, the most cost-effective solution would be to take the standard 16 mm camera aperture, with an aspect ratio of 1.37:1, and mask it down for a 16:9 presentation on a widescreen display. Should wide screen transmission for television be the predominant aim, the Super 16 mm format, with a nominal aspect ratio of 1.66:1, should preferably be used for image capture, as this format allows a larger image area to be recorded on the film. The standard 16 mm camera aperture would need to be modified to Super 16 mm aperture and the lens axis would need to be re-centred. Modern 16 mm film cameras enable a simple and quick change-over between standard and Super 16 mm mode of operation.

In order to be prepared for future broadcast of film programmes in higher quality television systems (e.g. HDTV), production on 35 mm film would be the better choice. A compromise approach, for the adaptation of the recorded image format for presentation both on 16:9 and 4:3 TV receivers, can be the “shoot and protect” concept. Two possibilities for such dual-purpose TV productions are currently used for film productions in European broadcasting organizations:

–	the “Academy” camera aperture, with an aspect ratio of 1.37:1 (the areas above and below a 16:9 area are protected);

–	or a camera aperture with an aspect ratio of 1.66:1 (the areas on either side of the central 4:3 area are protected).

The selection of the wanted image area for either 16:9 or 4:3 transmission can be done at the stage of film-to-tape transfer.

See also Recommendations ITU-R BR.782, ITU-R BR.783 and ITU-R BR.716.

_________________