RECOMMENDATION  ITU-R  BT.1118

ENHANCED  COMPATIBLE  WIDESCREEN  TELEVISION  BASED
ON  CONVENTIONAL  TELEVISION  SYSTEMS

(Question ITU-R 42/11)

(1994)

Rec. ITU-R BT.1118

The ITU Radiocommunication Assembly,

considering

a)	that both broadcasters and viewers have a large investment in equipment for conventional television, and broadcasters are therefore committed to maintaining the existing service;

b)	that many broadcasting organizations may wish to enhance the quality of their existing services to meet competition;

c)	that there are proposals to introduce new systems of television broadcasting with improved quality of picture and sound, including wider aspect ratio;

d)	that the quality goal for such new services has been defined in Recommendation ITU-R BT.1127;

e)	that the introduction of such new enhanced services will be supported by some existing production equipment;

f)	that modern technology allows cost-effective integration of enhancement technology into new consumer receivers and new studio equipment;

g)	that despite the advent of advanced television services in the future, enhanced systems could have sufficient life expectancy to justify investment in all areas of the broadcasting system;

h)	that a modular approach to enhancement will allow flexibility for broadcasters, manufacturers and consumers,

recommends

that for enhanced television:

1.	the display format should be 16:9. (As an interim arrangement the aspect ratio of transmitted images could be optimized to suit the populations of existing 4:3 receivers and new 16:9 receivers.)

2.	the transmission of wide aspect ratio images should use the letter-box technique (as an interim arrangement, the window technique may be used) when displayed on conventional 4:3 receivers;

3.	any additional signals which may be required to convey information to receivers for the enhancements being transmitted should be carried within the nominal 6, 7 or 8 MHz RF channel occupied by the conventional signal;

4.	the minimum protection ratios and the transmission power required by the enhanced television signal, either for conventional or enhanced receivers, should be the same as for conventional signals;

5.	when enhancements to existing television systems are made, some or all of the modular enhancements and features listed in Annex 1 should be used,

further recommends

1.	that enhanced terrestrial television broadcasting should not cause interference subjectively greater than that considered acceptable with current broadcasting services in the VHF/UHF bands;

2.	that the enthusiasm among creative staff for producing in 16:9 should be recognized and promoted;

3.	that the circumstances of developing countries should be given careful consideration in terms of priorities.

Note 1 – Administrations are invited to conduct further studies with the aim of making detailed proposals.

ANNEX  1

Modules and features of enhanced television

When enhancements to existing NTSC, PAL or SECAM television transmission systems are made, some or all of the following modular enhancement methods and features should be used.

# 1.	Picture enhancement techniques

## 1.1	Coder requirements for accommodating wide aspect ratio signals

–	Method of format conversion from 16:9 to 4:3,

–	method of encoding additional information:

–	method of deriving additional information,

–	method of processing the additional information,

–	method of including the additional information in the broadcast signal.

## 1.2	Coder requirements for reducing cross-effects and making optimal use of the signal spectrum

–	Method of luminance filtering,

–	method of chrominance filtering,

–	method of modulation,

–	method of combining luminance and chrominance.

## 1.3	Ghost cancellation

–	Insertion test waveform,

–	position in the signal.

## 1.4	Coder requirements for increased resolution

–	Method to derive extra information,

–	method of including the extra information in the channel.

## 1.5	Data signalling

–	Details of the data waveform,

–	position in the signal,

–	data to be transmitted.

# 2.	Sound enhancement methods

–	Details of sound coding technique,

–	method of including the sound signal.

# 3.	Other features

–	Methods to reduce impairments that might be created on the 4:3 conventional receivers by the enhancement signals should also be taken into consideration;

–	the possibility of the need for in-vision subtitles in the lower black part of the screen for existing 4:3 receivers without teletext decoders should be taken into account;

–	the possibility of transmitting such an enhanced signal through the existing satellite-broadcasting service and other media should be taken into account.

The main features of such a system, emphasizing the modular approach, are shown in the block diagram given in Fig. 1.

FIGURE 1/BT.1118...[D01] = 20.5cm

Four Appendices to this Annex describe the present state of enhanced television in a number of areas:

Appendix 1:	Enhanced television for 625-line PAL systems

Appendix 2:	Enhanced television in SECAM

Appendix 3:	EDTV-II state-of-the-art

Appendix 4:	Status of enhanced television in North America.

Some enhancement methods are discussed in Annex 2 for 625-line systems.

APPENDIX  1
TO  ANNEX  1

Enhanced television for 625-line PAL systems

# 1.	PALplus state-of-the-art

The PALplus system development is on schedule. The system design is finished. Field tests, including terrestrial transmissions in several European countries, cable tests in the Netherlands and satellite link tests have been executed with good results. Generally, whenever a good quality PAL signal is received, the PALplus decoded picture quality is equally good. So the decoded PALplus picture is not affected more severely than the PAL picture due to propagation impairments. Furthermore no unexpected problems occurred in PAL contribution and distribution networks. Regular test transmissions are scheduled for 1994 by some of the German broadcasters. The receiver manufacturers have started integrated circuit (IC) development and commercial market introduction of PALplus receivers is scheduled for September 1995 (Internationale Funk-Austellung (IFA'95) Berlin, 1995).

In the following section a description of the possible enhancement modules is given. The name of the PALplus system is reserved up to now to only include the picture enhancement techniques 2.1 and 2.2. It requires, however, that widescreen signalling (2.5 data signalling) be present. Ghost cancellation is not specific to PALplus and can equally be applied to PAL signals.

# 2.	Picture enhancement techniques

## 2.1	Accommodation for wide aspect ratio signals

The PALplus system uses a letter-box to transmit a 16:9 picture within a 4:3 broadcast frame. The black bars at the top and bottom are filled with the vertical helper signal, which has a very low visibility in order to be compatible on 4:3 receivers. The letter-box comprises 430 lines out of 574 full active lines and is generated by means of a vertical sample rate down conversion (factor 3/4) using a quadrature mirror filtering technique.

The helper signal is used for luminance only. The vertical conversion can be done either in intra-frame mode (for film-mode input material) or intra-field mode (for non-film material or so-called camera-mode).

The helper signal is attenuated to fit in the amplitude range of ±150 mV and is placed around blanking level. It is companded by means of a non-linear companding form to overcome the otherwise applicable noise penalty of compressing the amplitude. The signal is modulated AM/VSB onto the colour subcarrier to ensure that the signal is free of DC-components. This further reduces the visibility to a very low level.

## 2.2	Motion adaptive colour-plus

In film mode perfect luminance/chrominance separation is established using a technique called “colour-plus”. For camera mode this technique has been modified to be motion adaptive, employing an independent motion detector in both encoder and decoder, based on the chrominance signals. In case of movement the detectors softly switch on a pixel-by-pixel basis to low-pass filter separation; whilst for stationary parts the colour-plus principle is applied.

## 2.3	Ghost cancellation

The ghost cancelling reference (GCR) for 625-line PAL countries is given in Recommendation ITU-R BT.1124.

The ghost cancellation is not included within the PALplus system. It is optional since it is equally applicable to both normal PAL and PALplus signals.

## 2.4	Increased resolution

Except for the restored vertical resolution, which is conveyed using the vertical helper signal (see § 1.1) and the luminance/chrominance separation, yielding a clean horizontal frequency band for the range above approximately 3.5 MHz, there are no additional resolution-increasing techniques applied within the PALplus system.

## 2.5	Widescreen signalling

The widescreen signalling for 625-line PAL countries is given in Recommendation ITU-R BT.1119 and is required for the PALplus system.

# 3.	Sound enhancement methods

The NICAM system is recommended for conveying digital stereo sound. There is now no additional special digital sound system in development for those 625-line PAL countries employing the analogue two-carrier sound system, since it turned out to be too costly.

APPENDIX  2

TO  ANNEX  1

Enhanced television in SECAM

Introduction

Work has been conducted for more than ten years to improve the quality of SECAM transmitted pictures and the technology seems to be ready to implement some of the results.

# 1.	Picture enhancement techniques

## 1.1	Coder requirements for accommodating wide aspect ratio signals

### 1.1.1	Method for format conversion from 16:9 to 4:3 (detailed specification under study)

The enhanced SECAM uses a letter-box to transmit a 16:9 picture within a 4:3 broadcast frame.

### 1.1.2	Method of encoding additional information (specification under study)

The same principle of helper transmission has been studied for the SECAM system as for PALplus. Based on the fact that the SECAM system uses a chroma frequency modulation for spectrum sharing with the luminance signals, a different visibility of the helper signal in the black bars occurred on the compatible receiver. In order to achieve a comparable amount of visibility of the helper signal as in the PALplus system, some additional provisions have to be implemented for the SECAM system. It has been proven that the helper transmission is feasible for the SECAM system with, however, a slightly higher grade of complexity in the helper decoder compared with the PALplus standard.

The vertical helper methods used in PALplus are not directly applicable in SECAM, because the same AM/VSB decoders are not available in the receivers. Further study is being carried out.

## 1.2	Coder requirements for reducing cross-effects and making optimal use of the signal spectrum

### 1.2.1	Method of luminance filtering

Comb filter techniques are applicable in the encoder and in the decoder in order to nearly exploit the whole luminance bandwidth without chroma impairments. However, comb filters show specific artefacts in critical scenes, which sometimes lead to a picture degradation.

### 1.2.2	Method of chrominance filtering

An alternative to comb filter technique is the adoption of the luminance/chrominance separation technique used for PALplus.

### 1.2.3	Method of chrominance modulation

Shall be different to the PALplus system; further study is required.

### 1.2.4	Method of combining luminance and chrominance

The latest specification of the PALplus signal comprises a motion adaptive luminance/chrominance separation, which is called motion adaptive colour-plus. This method makes use of 180° phase difference of the colour subcarrier between adjacent lines of the two fields within one frame. By means of the phase difference, it is possible to avoid cross-talk between the luminance and chrominance signals.

Studies on the implementation of the motion adaptive colour-plus technique for SECAM have been carried out recently. The result revealed that an implementation is feasible and that a clean luminance/chrominance separation could be achieved. Nevertheless it shall be different from the PALplus system; further study is required.

## 1.3	Ghost cancellation

The GCR for 625-line PAL/SECAM countries is given in Recommendation ITU-R BT.1124.

The ghost cancellation is not included within the enhanced SECAM system. It is optional since it is equally applicable to both normal SECAM and enhanced SECAM signals.

## 1.4	Coder requirement for increased resolution

–	Method to derive extra information (to be defined);

–	method of including the extra information in the channel (to be defined).

## 1.5	Data signalling

The widescreen signalling for 625-line PAL/SECAM countries is given in Recommendation ITU-R BT.1119 and will be required for the enhanced SECAM system.

# 2.	Sound enhancement methods

Experiments have demonstrated the possibility of NICAM implementation in the SECAM/L system. A current European Telecommunications Standards (ETS) proposal specifies the modifications to the system L which are necessary to incorporate the NICAM signal. The nominal bandwidth of the video main sideband should be reduced to approximately 5.1 MHz. The video levels in the radiated signal are reduced by a factor of 90%, in order to have a 10% residual radiated vision carrier. The NICAM carrier frequency is located at 5.85 MHz above the vision carrier frequency. The type of modulation and bit rate as well as spectrum filters are the same as given for the B, G and H standards. The NICAM system is recommended for conveying digital stereo sound.

# 3.	Conclusion

It will be possible:

–	after defining certain specifications and data for enhanced SECAM, and

–	after thorough field testing,

to say that enhanced television in SECAM is feasible and takes full advantage of most of the techniques used in PALplus.

APPENDIX  3
TO  ANNEX  1

EDTV-II state-of-the-art

# 1.	Target specifications

At the Telecommunications Technology Council held in June 1992 in Japan, the items for the improvement and the target parameter values were tentatively approved as follows.

## 1.1	Format: letter box

## 1.2	Aspect ratio: 16:9

## 1.3	For luminance component

Horizontal resolution (still portions of pictures): approximately 640 lines per width.

Vertical resolution (still portions of pictures): approximately 480 lines per height.

Horizontal resolution (moving portions of pictures): approximately 448 lines per width.

Vertical resolution (moving portions of pictures): more than 240 lines per height and/or approximately 360 lines per height (further study is needed).

## 1.4	For chrominance components

Horizontal resolution of I signal: approximately 160 lines per width.

Horizontal resolution for Q signal: more than 50 lines per width.

# 2.	Current state-of-the-art

At this moment, intensive efforts are being conducted to finalize the tentative standard for hardware testing. The main items under study are as follows:

–	reducing the interference for 4:3 receiver viewers in the upper and lower black bar areas which carries helper signals;

–	the subjective assessment of the picture quality for the tentative standard;

–	estimation of the service area;

–	signalling.

APPENDIX  4
TO  ANNEX  1

Status of enhanced television in North America

At this stage no more work is being considered on NTSC-enhanced TV in the United States of America and Canada, unless digital advanced television systems currently under study prove completely unsuccessful. The United States of America allows the use of those video-processing techniques to improve the reproduction of received pictures that do not alter the transmission parameters (defined for NTSC-M in Recommendation ITU-R BT.470) nor change the characteristics of the radio-frequency emission of the TV broadcasting station. The United States of America and Canada have adopted the use of ghost cancelling reference (GCR) for 525-line NTSC as given in System C of Recommendation ITU-R BT.1124.

Cable Television Laboratories have also investigated viewers’ preferred screen format when watching 4:3 NTSC on 16:9 widescreen receivers. Three screen formats were studied:

–	“standard”; 4:3 picture with black strips along the right and left sides of the 16:9 screen;

–	“extended”; 4:3 picture blown up to fill the width of the 16:9 screen extending beyond top and bottom edges; viewer has option of scrolling picture up or down;

–	“compressed”; 4:3 image linearly distorted to fill the 16:9 screen.

The subjects perceived that each of the three screen formats had disadvantages.

ANNEX  2

Enhancement methods for 625-line systems

# 1.	Picture enhancement techniques

## 1.1	Wide aspect ratio coding

Two practical approaches to accommodating the wide aspect ratio frame in a conventional 4:3 signal are:

### 1.1.1	The letter-box format

Widescreen displays with a 16:9 aspect ratio are being made in Europe for enhanced television services. The most straightforward way to convey the 16:9 signal in the 625-line PAL/SECAM channel, and at the same time preserve the correct geometry for the viewer with a standard 4:3 receiver, is to use a letter-box format. In this format approximately 432 of the 575 active lines are used to carry the 16:9 active picture area.

In the widescreen receiver, approximately 432 lines are arranged, by up-conversion or other means, to fill the full height of the display. In the standard 4:3 receiver the viewer sees black bands above and below the active picture.

### 1.1.2	Window options

There are several possible choices for a compromise “letter-box” window solution, and one of these could be the mean aspect ratio between 4:3 and 16:9, i.e., 14:9. In this case, the black bands displayed on a conventional 4:3 aspect ratio receiver would be reduced from the 12.5% of a 16:9 letter-box to some 6% of full picture height at top and bottom. However, in practice, the viewer would be likely to see only about 2% black bands, due to the typical receiver overscan.

In the case of the widescreen receiver, options could exist to allow the 14:9 picture to be displayed filling the full width of the visible screen by incorporating a small amount of geometry error – typically 8%.

## 1.2	Reduction of cross-effects

Two approaches have been considered for reducing cross-effects in the PAL system, known in broad terms as band segregation and phase segregation.

Band segregation is the simplest method to understand because the luminance and chrominance components are separated in the frequency domain. A motion adaptive technique has been developed in Europe known as “motion adaptive colour-plus” which has been designed to allow optimum chrominance/luminance separation with both electronic camera and film sources. In the phase segregation method, high-frequency luminance and chrominance signals are effectively carried on orthogonally-phased subcarriers. Both methods involve complex multi-dimensional comb filter processing.

It remains to be seen whether similar techniques can be applied in the SECAM system.

## 1.3	Ghost cancellation

Proposals for ghost cancellation require the use of a test line or “training” signal. Various possibilities exist for the form of the training signal. Refer to Recommendation ITU-R BT.1124.

## 1.4	Increased resolution

In the circumstance where the source signal standard contains more resolution than conventional composite signals, there are proposals to include this information in the compatible enhanced signal. Such information might be accommodated by spectrum folding techniques and conveyed in the signal by coding methods such as:

–	quadrature modulation of the vision RF carrier;

–	signals carried in the video spectrum to take advantage of the Fukinuki hole in vision;

–	signals carried in the vertical blanking interval;

–	signals carried at the extremities of each active picture line, that would not normally be visible on conventional televisions due to overscan.

## 1.5	Data signalling

At its simplest, data signalling is desirable that would indicate to the receiver the aspect ratio of the transmission. The enhanced encoder would in addition need to convey other information to the decoder. For example, the status of the coder processing methods, the film or camera mode, if such improvements are incorporated in the system, to the decoder. Refer to Recommendation ITU-R BT.1119.

# 2.	Sound coding

It is expected that improved quality sound coding, by digital means, will form an important enhancement to the broadcast composite signal. The choice of technique needs to be made in association with other requirements in the signal.

_________________