RECOMMENDATION  ITU-R  BT.1122-1*,**

User requirements for emission and secondary
distribution systems for SDTV, HDTV and
hierarchical coding schemes

(1994-1995)

The ITU Radiocommunication Assembly,

considering

a)	that Recommendations ITU-R BT.601 and ITU-R BT.709 define the parameters of Y, CB, CR-based digital luminance and colour difference signals;

b)	that signals conforming to that standard are required to be transmitted through digital secondary distribution networks or emission systems (terrestrial, satellite, cable, etc.);

c)	that coding algorithms have been devised and standards established or proposed to enable such transmission to be effected using bit rate reduction techniques;

d)	that prototype codec equipment using these algorithms is being developed and needs to be assessed;

e)	that general advice on methods of assessment is contained within ITU-R texts, and that, in particular, subjective evaluation methods are defined in Recommendations ITU-R BT.500 and ITU-R BT.710;

f)	that such assessment will need to take account of basic picture quality and the failure characteristic in the presence of errors on the transmission and emission link;

g)	that both the design of codecs and their assessment will need to take account of user requirements;

h)	that in order to be complete, the user requirements should specify the test procedures and test material that should be used to check that the requirements are being met;

j)	that many organizations are requesting an early establishment of user requirements,

recommends

1	that the following user requirements should govern the specification design and testing of systems for the secondary distribution and emission of Y, CB, CR-based SDTV, HDTV and hierarchical television signals.

TABLE  1

User requirements for secondary distribution and emission

| User requirements | Specification |
| --- | --- |
| Functional requirements |  |
| Input television signal format | SDTV: 4:2:2 level of Rec. ITU-R BT.601 HDTV: studio standard pictures of Rec. ITU-R BT.709 |
| Input audio signal format | To be specified |
| Number of audio channels | To be specified |
| Ancillary data | To be specified |
| Access control | To be specified |
| HDTV/SDTV compatibility | Not required for some HDTV applications |
| Scalability | To be specified |
| Performance requirements |  |
| Requirements assumed for a complete broadcast chain |  |
| Codecs in cascade | Cascade of one contribution codec, one primary distribution codec and one secondary distribution codec. |
| Basic quality | Quality difference  18% of the DSCQS scale(1) for at least 4 sequences chosen from Recommendations ITU-R BT.710, ITU-R BT.802, ITU-R BT.1128 and ITU-R BT.1210, must be met and by at least 75% of the sequences chosen, the rest must achieve  36% |
| Change in overall delay after major disturbance | Less than 20 s |
| Total delay from origination to home | To be specified |
| Requirements for secondary distribution |  |
| Good reception condition | Quality difference  12% of the DSCQS scale(1) for at least 4 sequences chosen from Recommendations ITU-R BT.710, ITU-R BT.802, ITU-R BT.1128 and ITU-R BT.1210, must be met and by at least 75% of the sequences chosen, the rest must achieve  30% |
| Poor reception condition (2) – applicable to hierarchical coded systems | Quality difference  36% of the DSCQS scale(1) for at least 4 sequences chosen from Recommendations ITU-R BT.710, ITU-R BT.802, ITU-R BT.1128 and ITU-R BT.1210, must be met and by at least 75% of the sequences chosen, the rest must achieve  50% for the same test material and under the same viewing conditions |
| Lower resolution display (3) | Quality difference  12% of the DSCQS scale(1) for at least 4 sequences chosen from Recommendations ITU-R BT.710, ITU-R BT.802, ITU-R BT.1128 and ITU-R BT.1210, must be met by at least 75% of the sequences chosen, the rest must achieve  30% when compared with a down converted picture |
| Functional requirements |  |
| Basic audio quality | To be specified |
| Vision failure characteristics | To be specified |
| Image recovery time | 500 ms after a break of 50 ms |
| Audio recovery time | To be specified |
| Vision/audio failure characteristics | Vision failure first |
| Audio video relative delay | Less than  2 ms |
| Design requirements |  |
| Hardware complexity | Coder/decoder asymmetry to be favoured, i.e., a complex coder but simple receiver |
| DSCQS: double stimulus continuous quality scale. (1) Performance requirements – all quality assessment ratings in this section are carried out using the procedures given in Recommendations ITU-R BT.500, ITU-R BT.710 and ITU-R BT.1129 using the subjective assessment methods indicated. (2) Picture quality under rugged receiving conditions – under difficult receiving conditions, in some systems, a lower resolution picture is extracted and up-converted for display. For this mode of operation, quality assessment should use the higher resolution picture as reference. (3) Display at lower resolution – this condition applies when the receiver is capable of displaying only at lower resolution. For this mode of operation, quality assessment should use a down-converted version of the higher resolution picture as reference. |  |

*	 	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 9.

*	* 	Radiocommunication Study Group 6 made editorial amendments to this Recommendation in 2007 in accordance with Resolution ITU-R 44.