Recommendation ITU-R BT.1368-8 (05/2009)

Planning criteria for digital terrestrial television services in the VHF/UHF bands

BT Series Broadcasting service (television)

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  BT.1368-8*

Planning criteria for digital terrestrial television
services in the VHF/UHF bands

(1998-1998-2000-2002-2004-2005-2006-2007-2009)

Scope

This Recommendation defines planning criteria for various methods of providing digital terrestrial television services in the VHF/UHF bands.

The ITU Radiocommunication Assembly,

considering

a)	that systems are being developed for the transmission of digital terrestrial television services in the VHF/UHF bands;

b)	that the VHF/UHF television bands are already occupied by analogue television services;

c)	that the analogue television services will remain in use for a considerable period of time;

d)	that the availability of consistent sets of planning criteria agreed by administrations will facilitate the introduction of digital terrestrial television services,

recommends

1	that the relevant protection ratios (PRs) and the relevant minimum field strength values given in Annexes 1, 2 and 3, and the additional information given in Annexes 4, 5, 6 and 7 should be used as the basis for frequency planning for digital terrestrial television services.

Introduction

This Recommendation contains the following Annexes:

Annex 1 –	Planning criteria for ATSC digital terrestrial television systems in the VHF/UHF bands

Annex 2 –	Planning criteria for DVB-T digital terrestrial television systems in the VHF/UHF bands

Annex 3 –	Planning criteria for ISDB-T digital terrestrial television systems in the VHF/UHF bands

Annex 4 –	Other planning factors

Annex 5 –	Subjective comparison method (SCM) with a reference interferer for assessment protection ratios for analogue television systems

Annex 6 –	Test methods for protection ratio measurements for wanted digital terrestrial signals

Annex 7 –	Tropospheric and continuous interference

General

The RF protection ratio is the minimum value of wanted-to-unwanted signal ratio, usually expressed in decibels at the receiver input.

The reference level of the digital signal is defined as the r.m.s. value of the emitted signal power within the channel bandwidth. It should preferably be measured with a thermal power meter. All protection ratio values for wanted digital signals are to be measured with a 60 dBm receiver input power.

The reference level of the analogue vision-modulated signal is defined as the r.m.s. value of the vision carrier at peaks of the modulation envelope. All protection ratio values for wanted analogue signals are measured with a receiver input power of 39 dBm (70 dB(V) at 75 ).

# 1	Wanted digital terrestrial television systems

The protection ratios for digital terrestrial television systems apply to both continuous and tropospheric interference. The protection ratios refer to the centre frequency of the wanted digital terrestrial television system.

Because a digital television receiver needs to operate successfully in the presence of high level analogue signals on nearby channels, a high degree of receiver front-end linearity is required.

The protection ratios for digital terrestrial television systems as the interfering system are those for the case where the wanted and unwanted signals are not synchronized and/or do not have a common programme source. Results relevant to single frequency networks (SFN) are yet to be developed.

For the digital terrestrial television system, ATSC, the protection ratios are measured for a BER = 3 × 10–6 at the input of the MPEG-2 demultiplexer.

For the digital terrestrial television systems (digital video broadcasting-terrestrial (DVB-T) and integrated service digital broadcasting-terrestrial (ISDB-T)) the protection ratios are measured between the inner and outer codes, before Reed Solomon decoding, for a BER  2 × 104; this corresponds to a BER < 1 × 10–11 at the input of the MPEG-2 demultiplexer. For domestic receivers it may not be possible to measure the BER before Reed-Solomon decoding. The BER for such cases is under study.

To reduce the number of measurements and tables, it is proposed that protection ratio measurements for DVB-T systems should preferably be made with the following three modes shown in Table 1. Protection ratio values for the different required operational modes for fixed, portable or mobile reception can be calculated from the given measured values. A formula for calculation is still under study.

TABLE 1

Proposed preferable DVB-T mode types for measurements on protection ratios

| Modulation | Code rate | C/N (1) (dB) | Bit rate (2) (Mbit/s) |
| --- | --- | --- | --- |
| QPSK | 2/3 | 6.9 |  7 |
| 16-QAM | 2/3 | 13.1 |  13 |
| 64-QAM | 2/3 | 18.7 |  20 |
| (1) The figures are given for a Gaussian channel (including a typical implementation margin) for a BER < 1 × 10–11. (2) For a guard interval of 1/4. |  |  |  |

# 2	Wanted analogue terrestrial television systems

Measurements of protection ratios for the vision signal of a wanted analogue terrestrial television system should preferably be made with the subjective comparison method with a sine-wave reference interferer described in Annex 5.

The values of protection ratio quoted apply to interference produced by a single source. Except where otherwise stated, the ratios apply to tropospheric, T, interference and correspond closely to a slightly annoying impairment condition. They are considered to be acceptable only if the interference occurs for a small percentage of the time, not precisely defined but generally considered to be between 1% and 10%. For substantially non-fading unwanted signals, it is necessary to provide a higher degree of protection and ratios appropriate to continuous, C, interference should be used (see Annex 7).

When the wanted signal is an analogue television signal, two or more protection ratio values should be considered, one for the protection ratio of the vision signal and others for the protection ratios of sound signals. The most stringent value should then be used.

Significantly strong wanted input signals can require higher protection ratio values because of non-linear effects in the receiver.

For 625-line systems, the reference impairment levels are those which correspond to co-channel protection ratios of 30 dB and 40 dB, when two-thirds line offset is used, see Recommendation ITU-R BT.655. These conditions approximate to impairment grades 3 (slightly annoying) and 4 (perceptible but not annoying) and apply to tropospheric, T, and continuous, C, interference, respectively.

CONTENTS

Page

Annex 1 – Planning criteria for ATSC digital television systems in the VHF/UHF bands		12

1	Protection ratios for ATSC wanted digital terrestrial television signals		12

1.1	Protection of an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal		12

1.2	Protection of ATSC digital terrestrial television interfered with by analogue terrestrial television		13

1.2.1	Protection from co-channel interference		13

1.2.2	Protection from lower adjacent channel (N – 1) interference		13

1.2.3	Protection from upper adjacent channel (N + 1) interference		13

1.2.4	Protection from other channel interference		14

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals		14

2.1	Protection ratios for 525-line television systems		14

2.1.1	Protection for vision signals interfered with by ATSC digital television		14

2.2	Protection ratios for 625-line television systems		15

2.2.1	Protection of wanted vision signals interfered with by ATSC digital terrestrial television		15

2.2.1.1	Protection from co-channel interference		15

2.2.1.2	Protection from lower adjacent channel interference		15

2.2.1.3	Protection from upper adjacent channel interference		15

3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals		16

3.1	Protection for NTSC sound signals (BTSC MTS system and SAP) interfered with by an ATSC digital television signal		16

4	Minimum field strengths for ATSC terrestrial digital television		16

Appendix 1 to Annex 1 – Derivation by the figure of merit method		17

Annex 2 – Planning criteria for DVB-T digital television systems in the VHF/UHF bands		19

1	Protection ratios for DVB-T wanted digital terrestrial television signals		19

1.1	Protection of a DVB-T digital terrestrial television signal interfered with by a DVB-T digital terrestrial television signal		19

1.2	Protection of DVB-T digital terrestrial television interfered with by analogue terrestrial television		20

1.2.1	Protection from co-channel interference		20

1.2.2	Protection from lower adjacent channel (N – 1) interference		21

Page

1.2.3	Protection from upper adjacent channel (N + 1) interference		22

1.2.4	Protection from overlapping channel interference		22

1.3	Protection of DVB-T digital terrestrial television signal from CW or FM signals		23

1.4	Protection of DVB-T digital terrestrial television signals from T-DAB signals		24

1.5	Protection of DVB-T from wideband signals other than terrestrial broadcasting		25

1.5.1	Protection ratios for DVB-T interfered with by the fixed service (relocatable system)		25

1.5.2	Protection ratios for DVB-T interfered with by code-division multiple access (CDMA)		25

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals		26

2.1	Protection ratios for 625-line television systems		26

2.1.1	Protection of wanted vision signals interfered with by DVB-T digital terrestrial television signal		26

2.1.1.1	Protection from co-channel interference		27

2.1.1.2	Protection from lower adjacent channel interference		27

2.1.1.3	Protection from upper adjacent channel interference		28

2.1.1.4	Protection from image channel interference		28

2.1.1.5	Protection from overlapping interference		29

3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals		30

3.1	Protection for FM, AM and NICAM sound signals of analogue television systems interfered with by DVB-T digital terrestrial television signals		31

4	Protection ratios for a T-DAB signal interfered with by an unwanted digital terrestrial television signal		32

5	Minimum field strengths for DVB-T terrestrial digital television, fixed reception		33

6	Minimum median field strength for mobile DVB-T reception		33

6.1	Required average C/N for mobile reception		33

6.2	Receiver noise figure		36

7	Minimum median field strength for hand held pedestrian indoor, pedestrian outdoor and mobile DVB-H reception		36

7.1	Channel models for hand held pedestrian  indoor and outdoor  reception		36

7.2	Channel model for mobile reception		38

7.3	Required average C/N for hand held indoor  and outdoor reception		38

7.4	Required average C/N for hand held indoor and outdoor reception		39

7.5	Receiver noise figure		40

Page

Appendix 1 to Annex 2 – Calculation of minimum field strength and minimum median equivalent field strength		40

Annex 3 – Planning criteria for ISDB-T digital television system in the VHF/UHF bands		41

1	Protection ratios for ISDB-T wanted digital terrestrial television signals		41

1.1	Protection of an ISDB-T digital terrestrial television signal interfered with by an ISDB-T digital television signal		42

1.2	Protection of ISDB-T digital terrestrial television interfered with by analogue terrestrial television		43

1.2.1	Protection from co-channel interference		43

1.2.2	Protection from lower adjacent channel (N  1) interference		43

1.2.3	Protection from upper adjacent channel (N + 1) interference		44

2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ISDB-T digital terrestrial television signals		44

2.1	Protection ratios for 525-line television systems		44

2.1.1	Protection for NTSC vision signals interfered with by ISDB-T digital television signals		44

3	Protection ratios for sound signals of wanted analogue terrestrial television systems interfered with by ISDB-T unwanted digital terrestrial television system		45

3.1	Protection for NTSC sound signals interfered with by an ISDB-T digital television signal		45

4	Minimum field strengths for ISDB-T terrestrial digital television		45

5	Minimum median field strength for hand-held pedestrian indoor, pedestrian outdoor and mobile ISDB-T reception		45

5.1	Channel models for hand-held pedestrian indoor and outdoor reception		45

5.2	Channel model for mobile reception		46

5.3	Required average C/N for hand-held indoor and outdoor reception		47

5.4	Required average C/N for mobile reception		47

5.5	Receiver noise figure		48

Appendix 1 to Annex 3 – Calculation of minimum field strength and minimum median equivalent field strength		49

Annex 4 – Other planning factors		51

1	Field strength distribution with location		51

2	Reception using portable equipment inside buildings and vehicles		52

2.1	Height loss: Lh		52

Page

2.2	Building entry loss: Lb		52

2.3	Vehicle entry loss: Lv		53

3	Receiving antenna discrimination		53

4	Antennas for portable and mobile receivers		53

4.1	Antennas for portable reception		53

4.2	Antennas for hand-held reception		53

4.3	Antennas for mobile reception		54

Annex 5 – Subjective comparison method (SCM) with a reference interferer for assessment of protection ratios for analogue television systems		54

1	Introduction		54

2	SCM for assessment of protection ratios using a sine-wave reference		55

2.1	General description		55

2.2	Realization of the reference interferer		56

2.3	Test conditions		56

2.4	Presentation of the results		56

3	Table of important parameters		57

Annex 6 – Test methods for protection ratio measurements for wanted digital terrestrial signals		57

1	Background		57

2	Subjective failure point (SFP) method for protection ratio measurements		57

Annex 7 – Tropospheric and continuous interference		58

List of tables

Page

Table 1 –		Proposed preferable DVB-T mode types for measurements on protection ratios..........................................................................................................		3

Table 2 –		Co-channel protection ratios (dB) for an ATSC signal interfered with by an ATSC signal		12

Table 3 –		Protection ratios (dB) for ATSC interfered with by an ATSC signal in the lower (N – 1) and upper (N + 1) adjacent channels		12

Table 4 –		Protection ratios (dB) for an ATSC signal interfered with by an ATSC signal in channel N  2 and other out-of-band channels		12

Page

Table 5 –		Co-channel protection ratios (dB) for an ATSC signal interfered with by an analogue television signal		13

Table 6 –		Protection ratios (dB) for lower adjacent channel (N – 1) interference for an ATSC signal interfered with by an analogue television signal including sound		13

Table 7 –		Protection ratios (dB) for upper adjacent channel (N + 1) interference for an ATSC signal interfered with by an analogue television signal		13

Table 8 –		Protection ratios (dB) for an ATSC 6 MHz signal interfered with by an M/NTSC signal at other out-of-band channels		14

Table 9 –		Protection ratios (dB) for a wanted analogue vision signal (NTSC, 6 MHz) interfered with by an unwanted ATSC signal		14

Table 10 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an unwanted ATSC 6 MHz signal		15

Table 11 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an ATSC 6 MHz signal (lower adjacent channel)		15

Table 12 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an ATSC 6 MHz signal (upper adjacent channel)		15

Table 13 –	Derivation by the figure of merit method ATSC 6 MHz system		16

Table 14 –	Co-channel protection ratios (dB) for a DVB-T signal interfered with by a DVB-T signal		19

Table 15 –	Protection ratios (dB) for a DVB-T signal interfered with by a DVB-T signal in the lower (N – 1) and upper (N + 1) adjacent channels		20

Table 16 –	Co-channel protection ratios (dB) for DVB-T 7 MHz and 8 MHz signals interfered with by analogue television (non-controlled frequency condition) signals		20

Table 17 –	Protection ratios (dB) for lower adjacent channel (N – 1) interference for DVB-T 7 MHz and 8 MHz signals interfered with by analogue television signals including sound		21

Table 18 –	Protection ratios (dB) for upper adjacent channel (N + 1) interference for DVB-T 7 MHz and 8 MHz signals interfered with by an analogue television signal		22

Table 19 –	Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an overlapping PAL B signal including sound		22

Table 20 –	Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an overlapping 7 MHz analogue TV signal including sound		22

Page

Table 21 –	Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an overlapping 8 MHz analogue TV signal including sound		23

Table 22 –	Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an overlapping 8 MHz analogue TV signal including sound		23

Table 23 –	Co-channel protection ratios (dB) for a DVB-T 8 MHz 64-QAM code rate 2/3 signal interfered with by a CW or a FM carrier (non-controlled frequency offset)		23

Table 24 –	Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3 signal interfered with by a CW or an FM carrier (non-controlled frequency offset)		24

Table 25 –	Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3 signal interfered with by a CW carrier (controlled frequency offset)		24

Table 26 –	Co-channel protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by a T-DAB signal		24

Table 27 –	Protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by a T-DAB signal in the lower (N – 1) or upper (N + 1) adjacent channels		25

Table 28 –	Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal interfered with by emissions of the fixed service		25

Table 29 –	Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal interfered with by emissions of CDMA-1X		26

Table 30 –	Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal interfered with by emissions of CDMA-3X		26

Table 31 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an unwanted DVB-T 8 MHz signal		27

Table 32 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by an unwanted DVB-T 7 MHz signal		27

Table 33 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by DVB-T 7 MHz and 8 MHz signals (lower adjacent channel)		27

Table 34 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by DVB-T 7 MHz and 8 MHz signals (upper adjacent channel)		28

Table 35 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by a DVB-T 8 MHz signal (image channel)		28

Table 36 –	Protection ratios (dB) for a wanted analogue vision signal interfered with by a DVB-T 7 MHz signal (image channel)		28

Page

Table 37 –	Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals interfered with by a DVB-T 7 MHz signal (overlapping channels)		29

Table 38 –	Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals interfered with by a DVB-T 8 MHz signal (overlapping channels)		30

Table 39 –	Co-channel protection (dB) ratios for a wanted sound signal interfered with by DVB-T digital terrestrial television signals		31

Table 40 –	Protection ratios (dB) for a wanted FM sound signal interfered with by a DVB-T 7 MHz signal (overlapping channels)		32

Table 41 –	Protection ratios (dB) for a wanted AM sound signal interfered with by a DVB-T 8 MHz signal for different frequency offsets (upper adjacent channel)		32

Table 42 –	Protection ratios (dB) for a T-DAB signal interfered with by a DVB-T 8 MHz signal		32

Table 43 –	Protection ratios (dB) for a T-DAB signal interfered with by a DVB-T 7 MHz signal		33

Table 44 –	Calculation of minimum field strength DVB-T 8 MHz system		33

Table 45 –	Required average C/N, speed limits for mobile reception for the non-diversity case		35

Table 46 –	Required average C/N, speed limits for mobile reception for the diversity case		35

Table 47 –	Channel profile for measurement of required average C/N for mobile reception of DVB-T reception “typical urban”		36

Table 48 –	Doppler spectrum definitions for PI- and PO-channels		37

Table 49 –	Definition of PI-channel		37

Table 50 –	Definition of PO-channel		37

Table 51 –	C/N (dB) for 5% MFER in PI and PO channel		38

Table 52 –	DVB-H C/N (dB) in mobile channel for 5% MFER		39

Table 53 –	Co-channel protection ratios (dB) for ISDB-T 6 MHz interfered with by ISDB-T 6 MHz		42

Table 54 –	Protection ratios (dB) for ISDB-T 6 MHz interfered with by ISDB-T 6 MHz in lower adjacent channel (N - 1)		42

Table 55 –	Protection ratios (dB) for ISDB-T 6 MHz interfered with by ISDB-T 6 MHz in upper adjacent channel (N + 1)		42

Page

Table 56 –	Co-channel protection ratios (dB) for ISDB-T 6 MHz interfered with by analogue television		43

Table 57 –	Protection ratios (dB) for lower adjacent channel (N - 1) interference for ISDB-T 6 MHz interfered with by NTSC signals including sound		43

Table 58 –	Protection ratios (dB) for upper adjacent channel (N + 1) interference for ISDB-T 6 MHz interfered with by NTSC 6 MHz signal		44

Table 59 –	Protection ratios (dB) for analogue vision signal (NTSC, 6 MHz) interfered with by ISDB-T signal		44

Table 60 –	Sound quality related to the vision protection ratio of grade 3 when NTSC 6 MHz signal interfered with by ISDB-T 6 MHz signal		45

Table 61 –	Derivation by the voltage method ISDB-T 6 MHz system		46

Table 62 –	C/N (dB) for 5% ESR in PI and PO channel		47

Table 63 –	ISDB-T C/N (dB) in mobile channel for 5% ESR		48

Table 64 –	Building entry loss variations in the UHF Bands IV/V		52

Table 65 –	Antenna gain (dBd) for portable reception		53

Table 66 –	Antenna gain (dBd) for hand-held reception		54

Table 67 –	Antenna gain (dBd) for mobile reception		54

Table 68 –	Basic terms and relations for the SCM		57

List of figures

Page

Figure 1 –	Required average C/N in a mobile propagation channel		34

Figure 2 –	Ratio (dB) of the field strength for a given percentage of the receiving locations to the field strength for 50% of the receiving locations		51

Figure 3 –	SCM for assessment of protection ratios		55

Annex 1

Planning criteria for ATSC digital television systems in the VHF/UHF bands

# 1	Protection ratios for ATSC wanted digital terrestrial television signals

Tables 2 to 4 and 5 to 8 show protection ratios for an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal, and by an analogue terrestrial television signal, respectively.

## 1.1	Protection of an ATSC digital terrestrial television signal interfered with by an ATSC digital terrestrial television signal

TABLE 2

Co-channel protection ratios (dB) for an ATSC signal 
interfered with by an ATSC signal

| Wanted signal | Unwanted signal |
| --- | --- |
|  | ATSC 6 MHz |
| ATSC 6 MHz | 15 19(1) |
| (1) Based on equally partitioned noise and interference. |  |

TABLE 3

Protection ratios (dB) for ATSC interfered with by an ATSC
signal in the lower (N – 1) and upper (N + 1) adjacent channels

| Channel | N – 1 | N + 1 |
| --- | --- | --- |
| ATSC 6 MHz | –27 | –27 |

The protection ratios are given in dB and apply to both continuous and tropospheric interference.

TABLE 4

Protection ratios (dB) for an ATSC signal interfered with by an ATSC
signal in channel N  2 and other out-of-band channels

| Channel | N  2 and other out-of-band channels |
| --- | --- |
| ATSC 6 MHz | –58 |

## 1.2	Protection of ATSC digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE 5

Co-channel protection ratios (dB) for an ATSC signal interfered with
by an analogue television signal

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |  |
| --- | --- | --- |
|  | M/NTSC | PAL B |
| ATSC 6 MHz | 2(1) 7 | 9 |
| ATSC 6 MHz with 1/2 rate concatenated trellis coding | 1 | 3 |
| ATSC 6 MHz with 1/4 rate concatenated trellis coding | –2 | 0 |
| (1) Using a comb filter in the digital television receiver and C/N of 19 dB. |  |  |

### 1.2.2	Protection from lower adjacent channel (N – 1) interference

TABLE 6

Protection ratios (dB) for lower adjacent channel (N – 1) interference
for an ATSC signal interfered with by an analogue television signal
including sound

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |
| --- | --- |
|  | M/NTSC |
| ATSC 6 MHz | –48 |

### 1.2.3	Protection from upper adjacent channel (N + 1) interference

TABLE 7

Protection ratios (dB) for upper adjacent channel (N + 1) interference 
for an ATSC signal interfered with by an analogue television signal

| Wanted signal | Unwanted signal (Analogue TV signal including sound carriers) |
| --- | --- |
|  | M/NTSC |
| ATSC 6 MHz | –49 |

### 1.2.4	Protection from other channel interference

TABLE 8

Protection ratios (dB) for an ATSC 6 MHz signal interfered with by 
an M/NTSC signal at other out-of-band channels

| Wanted signal | Unwanted signal | Unwanted channels | Protection ratio |
| --- | --- | --- | --- |
| ATSC | M/NTSC | N  2 to N  8 | –58 |

# 2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals

Table 9 and Tables 10 to 12 show protection ratios for wanted 525-line and 625-line, respectively, analogue television signals interfered with by ATSC digital terrestrial television signals.

## 2.1	Protection ratios for 525-line television systems

### 2.1.1	Protection for vision signals interfered with by ATSC digital television

In this section the protection ratios for an analogue wanted signal interfered by an unwanted ATSC digital signal apply only to the interference to the vision and colour carriers.

TABLE 9

Protection ratios (dB) for a wanted analogue vision signal (NTSC, 6 MHz)
interfered with by an unwanted ATSC signal

| Unwanted digital channel | Tropospheric interference grade 3 | Continuous interference grade 4 |
| --- | --- | --- |
| N – 1 (lower) | –16 |  |
| N (co-channel) | 34 |  |
| N + 1 (upper) | –17 |  |
| N + 14 (image) | –33 |  |
| N + 15 (image) | –31 |  |
| N  2 | –24 |  |
| N  3 | –30 |  |
| N  4 | –25 |  |
| N  7 | –34 |  |
| N  8 | –32 |  |

## 2.2	Protection ratios for 625-line television systems

### 2.2.1	Protection of wanted vision signals interfered with by ATSC digital terrestrial television

In this section the protection ratios for an analogue wanted signal interfered by an unwanted digital signal relate only to the interference to the vision signal.

The protection ratio values given are related to an out-of-channel spectrum attenuation of the unwanted DVB-T transmitter of 40 dB.

#### 2.2.1.1	Protection from co-channel interference

TABLE 10

Protection ratios (dB) for a wanted analogue vision signal interfered with by
an unwanted ATSC 6 MHz signal

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | 38 | 45 |

#### 2.2.1.2	Protection from lower adjacent channel interference

TABLE 11

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an ATSC 6 MHz signal (lower adjacent channel)

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz signal (lower adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | – 7 | – 1 |

#### 2.2.1.3	Protection from upper adjacent channel interference

TABLE 12

Protection ratios (dB) for a wanted analogue vision signal interfered with by
an ATSC 6 MHz signal (upper adjacent channel)

| Wanted signal: analogue system | Unwanted signal: ATSC 6 MHz signal (upper adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL | –7 | 0 |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted ATSC digital terrestrial television signals

## 3.1	Protection for NTSC sound signals (BTSC MTS system and SAP) interfered with by an ATSC digital television signal (see Note 1)

In the case of an unwanted upper adjacent digital channel N + 1 the audio signals degrade before the vision signal. The protection ratio value for the interference into the BTSC MTS and SAP sound signals was measured with 12 dB. (Vision protection ratio for N + 1 is 17 dB.) The 12 dB sound protection ratio figure is related to the wanted NTSC vision carrier level.

NOTE 1 – BTSC MTS:	broadcast television system committee multichannel television sound; SAP:  sound audio programme.

# 4	Minimum field strengths for ATSC terrestrial digital television

TABLE 13

Derivation by the figure of merit method ATSC 6 MHz system*

| Planning parameter(1) | Low VHF 54-88 MHz | High VHF 174-216 MHz | UHF 470-806 MHz |
| --- | --- | --- | --- |
| Frequency (MHz) | 69 | 194 | 615 |
| C/N (dB) | 19.5(2) | 19.5(2) | 19.5(2) |
| k (dB) | –228.6 | –228.6 | –228.6 |
| B (dB(Hz)) (6 MHz) | 67.8 | 67.8 | 67.8 |
| G1m2 (dB) | – 1.8 | 7.3 | 17.2 |
| GD (dB) | 6 | 8 | 10 |
| GI (dB) | 8.2 | 10.2 | 12.2 |
| Transmission line loss (dB) line | 1.1 | 1.9 | 3.3 |
| Antenna 300/75 balun loss (dB) balun | 0.5 | 0.5 | 0.5 |
| Receiver noise figure (dB) | 5 | 5 | 10 |
| Trx (K) | 627.1 | 627.1 | 2 610 |
| Tline (K) | 65.0 | 102.9 | 154.4 |
| LNA noise figure (dB) | 5 | 5 | 5 |
| LNA gain (dB) | 20 | 20 | 20 |
| TLNA (dB) | 627.1 | 627.1 | 627.1 |
| Tbalun (K) | 31.6 | 31.6 | 31.6 |
| Ta (K) | 9 972.1 | 569.1 | Negligible |
| Ta balun (K) | 8 885.1 | 507.1 | Negligible |
| Tline/ G (K) | 0.8 | 1.6 | 3.3 |
| Trx/ G (K) | 8.1 | 9.7 | 55.8 |
| Te (K) | 9 552.6 | 1 176.8 | 717.8 |
| 10 log(Te) (dB(K)) | 39.8 | 30.7 | 28.6 |

TABLE 13 (end)

| Planning parameter(1) | Low VHF 54-88 MHz | High VHF 174-216 MHz | UHF 470-806 MHz |
| --- | --- | --- | --- |
| GA (dB) | 7.7 | 9.7 | 11.7 |
| Erx (dB(V/m))(2), (3) (TBC) | 35 | 33 | 39 |
| * The values in the Table were calculated assuming C/N with typical multipath reception impairment and equal partitioning for noise and interference. The receiving system model is a typical receiving installation located near the edge of coverage and consists of an externally mounted antenna, a low noise amplifier (LNA) mounted at the antenna, an interconnecting downlead cable and an ATSC receiver. (1) For definitions see Appendix 1 to Annex 1. (2) Figures should be adjusted downward (towards better performance) by 6 dB for 1/2 rate concatenated trellis coding or 9 dB for 1/4 rate concatenated trellis coding. (3) For formula see Appendix 1 to Annex 1. |  |  |  |

Appendix 1 
to Annex 1

Derivation by the figure of merit method

Required field strength

Erx (dB(V/m))  =   (dB(W/m2)) +10 log(120 )

C/N  =   – Glm2 + GA / Te – k – Brf

Erx (dB(V/m))	=	 (dB(W/m2))  25.8 (dB)  120 (dB)

=	145.8 + C/N + Glm2 – GA / Te + 10 log(k) +10 log(Brf )

Erx:	required field strength at the receive system antenna

 :	power flux-density at the receive system antenna

C/N :	carrier-to-noise ratio

Glm2:	gain of 1 m2

GA /Te :	figure of merit of the receive system

k :	Boltzmann’s constant (J/K)

Brf :	system equivalent noise bandwidth.

Receive system figure of merit

(For receiving system model with LNA)

GA / Te    (G  L ) / (balun Ta  Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))

Receiver noise temperature

Trx    (10NF / 10 – 1)    290

LNA noise temperature

TLNA    (10NF / 10 – 1)    290

Transmission line noise temperature

Tline    (1 – line)    290

Balun noise temperature

Tbalun    (1 – balun)    290

Antenna noise temperature

Ta    10(6.63 – 2.77(log f ))    290     (for dipole antenna)

where f is expressed in MHz.

Antenna noise temperature (referred to LNA input)

 Ta    Ta(balun)

System noise temperature

Te    (balun Ta  Tbalun  TLNA  Tline /(line GLNA)  Trx / (line GLNA))

Te (dB(K))		10 log(balun Ta  Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))

or		10 log(Tbalun  TLNA  Tline / (line GLNA)  Trx / (line GLNA))  Next

when Ta is not known.

Gain of 1 m2

G1m2    10 log(4  /2)

Data

GI :	antenna gain (isotropic) (dB)

L :	transmission line loss (dB)

line :	transmission line loss (numeric ratio)

Ta :	antenna noise temperature (K)

Trx :	receiver noise temperature (K)

nf :	noise factor (numeric ratio)

NF :	noise figure (dB)

T0 :	reference temperature  290 K

 :	wavelength of frequency of operation

GA :	system gain (dB)

Te :	system noise temperature (K)

Next :	dB value representing the contribution due to external noise

k :	Boltzmann’s constant 1.38  10–23 (–228.6 dB) (J/K)

B :	system equivalent noise bandwidth (dB(Hz))

balun :	antenna 300/75 Balun loss (numeric ratio)

LNA :	low noise amplifier

TLNA :	LNA noise temperature (K)

Annex 2

Planning criteria for DVB-T digital television systems 
in the VHF/UHF bands

# 1	Protection ratios for DVB-T wanted digital terrestrial television signals

Tables 14 to 15, 16 to 22, 23 to 25 and 26 to 27 show protection ratios for the DVB-T wanted digital terrestrial television signals interfered with:

	by DVB-T digital terrestrial television signals,

	by analogue terrestrial television signals,

	by a single continuous wave (CW) or FM carrier

	by terrestrial digital audio broadcasting (T-DAB) signals, respectively.

## 1.1	Protection of a DVB-T digital terrestrial television signal interfered with by a DVB-T digital terrestrial television signal

TABLE 14

Co-channel protection ratios (dB) for a DVB-T signal
interfered with by a DVB-T signal

| Modulation | Code rate | Gaussian channel | Rice channel | Rayleigh channel |
| --- | --- | --- | --- | --- |
| QPSK | 1/2 | 5 | 6 | 8 |
| QPSK | 2/3 | 7 | 8 | 11 |
| 16-QAM | 1/2 | 10 | 11 | 13 |
| 16-QAM | 2/3 | 13 | 14 | 16 |
| 16-QAM | 3/4 | 14 | 15 | 18 |
| 64-QAM | 1/2 | 16 | 17 | 19 |
| 64-QAM | 2/3 | 19 | 20 | 23 |
| 64-QAM | 3/4 | 20 | 21 | 25 |

Protection ratios are given for three types of propagation channels (i.e. Gaussian, Ricean and Rayleigh). For fixed and portable reception, the values relevant to the Ricean and Rayleigh channels, respectively, should be adopted.

The same protection ratios should be applied for DVB-T systems with 6, 7 and 8 MHz bandwidth.

Protection ratios are rounded to the nearest integer.

For overlapping channel, in absence of measurement information, and if the overlapping bandwidth between the wanted and unwanted signals is less than 1 MHz, the protection ratio, PR, should be extrapolated from the co-channel ratio figure as follows:

PR    CCI    10 log10(BO/BW )

where:

CCI :	co-channel protection ratio

BO :	bandwidth (MHz) in which the two DVB-T signals are overlapping

BW :	bandwidth (MHz) of the wanted signal

PR 	30 dB should be used when the above formula gives PR <  30 dB.

However, further studies are needed on this subject.

TABLE 15

Protection ratios (dB) for a DVB-T signal interfered with by a DVB-T signal
in the lower (N – 1) and upper (N  1) adjacent channels

| Channel | N – 1 | N  1 |
| --- | --- | --- |
| PR | – 30 | – 30 |

The protection ratio is given in dB and applies to both continuous and tropospheric interference.

The values given apply to the case where wanted and unwanted DVB-T signals have the same channel width. Other combinations of channel width need further studies.

It is known from measurements of existing receivers that they permit lower protection ratios.

But for planning purposes it is an advantage to have this value.

## 1.2	Protection of DVB-T digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE 16

Co-channel protection ratios (dB) for DVB-T 7 MHz and 8 MHz signals
interfered with by analogue television (non-controlled
frequency condition) signals

| Modulation | Code rate | Gaussian channel | Rice channel | Rayleigh channel |
| --- | --- | --- | --- | --- |
| QPSK | 1/2 | –12 |  | –12 |
| QPSK | 2/3 | –8 |  | –8 |
| QPSK | 3/4 | –4 |  |  |
| QPSK | 5/6 | 3 |  |  |
| QPSK | 7/8 | 9 |  |  |
| 16-QAM | 1/2 | –8 |  | –8 |
| 16-QAM | 2/3 | –3 |  | 3 |
| 16-QAM | 3/4 | 0 |  | 5 |
| 16-QAM | 5/6 | 9 |  |  |
| 16-QAM | 7/8 | 16 |  |  |

TABLE 16 (end)

| Modulation | Code rate | Gaussian channel | Rice channel | Rayleigh channel |
| --- | --- | --- | --- | --- |
| 64-QAM | 1/2 | –3 |  | 3 |
| 64-QAM | 2/3 | 3 |  | 6 |
| 64-QAM | 3/4 | 9 |  | 15 |
| 64-QAM | 5/6 | 15 |  |  |
| 64-QAM | 7/8 | 20 |  |  |

NOTE 1 – The PAL/SECAM values are valid for the following sound carrier modes:

–	MONO FM with a single sound carrier at 10 dB referred to the vision carrier;

–	DUAL FM and FM + NICAM with two sound carriers at 13 dB and 20 dB level;

–	AM + NICAM with two sound carriers at respectively 10 dB and 27 dB level.

According to the available measurements, the same protection ratio values are applicable for 2k and 8k modes.

In all Tables except Table 25, the so-called non-controlled conditions are used.

Actual measurements of protection ratio values will reflect the cyclic variation that occurs when the offset between a wanted DVB-T signal and an unwanted analogue signal is varied over a frequency range equivalent to the spacing between carriers of coded orthogonal frequency division multiplex (COFDM) systems. The protection ratios given represent a conservative, but realistic, value that covers the expected offset performance of existing receivers. The adoption of fine offset between COFDM signals and interfering analogue TV signals will permit the achievement of up to 3 dB improvement in protection ratio. The required transmitter frequency stability is similar to that of analogue precision offset, that means a range of some  1 Hz.

Protection ratios for DVB-T 6 MHz are missing due to lack of measurement results.

### 1.2.2	Protection from lower adjacent channel (N – 1) interference

TABLE 17

Protection ratios (dB) for lower adjacent channel (N – 1) interference for
DVB-T 7 MHz and 8 MHz signals interfered with by analogue 
television signals including sound

| Wanted signal |  | Unwanted signal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Constellation | Code rate | PAL B | PAL G, B1 | PAL I | PAL D, K | SECAM L | SECAM D, K |
| QPSK | 1/2 |  | –44 |  |  |  |  |
| QPSK | 2/3 | –44 | –44 |  |  |  |  |
| 16-QAM | 1/2 |  | –43 | – 43 |  |  |  |
| 16-QAM | 2/3 | –42 | –42 |  |  |  |  |
| 16-QAM | 3/4 |  | –38 |  |  |  |  |
| 64-QAM | 1/2 |  | –40 | –38 |  |  |  |
| 64-QAM | 2/3 | –35 | –35 | –34 |  | –35 | –37 |
| 64-QAM | 3/4 |  | –32 |  |  |  |  |

All values applicable for fixed and portable reception conditions.

### 1.2.3	Protection from upper adjacent channel (N  1) interference

TABLE 18

Protection ratios (dB) for upper adjacent channel (N  1) interference for DVB-T
7 MHz and 8 MHz signals interfered with by an analogue television signal

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Constellation | Code rate | PAL/SECAM |
| QPSK | 2/3 | – 47 |
| 16-QAM | 2/3 | – 43 |
| 64-QAM | 2/3 | –38 |

### 1.2.4	Protection from overlapping channel interference

TABLE 19

Protection ratios (dB) for a DVB-T 8 MHz signal interfered
with by an overlapping PAL B signal including sound

| Unwanted signal: PAL B analogue TV system | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 9.75 | 9.25 | 8.75 | 8.25 | 6.75 | 3.95 | 3.75 | 2.75 | 0.75 | 2.25 | 3.25 | 4.75 | 5.25 |
| PR | 37 | 14 | 8 | 4 | 2 | 1 | 3 | 3 | 3 | 2 | 1 | 29 | 36 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

TABLE 20

Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an 
overlapping 7 MHz analogue TV signal including sound

| Unwanted signal: 7 MHz analogue TV system | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 9.25 | 8.75 | 8.25 | 7.75 | 6.25 | 3.45 | 3.25 | 2.25 | 1.25 | 0 | 1.75 | 2.75 | 4.25 | 4.75 |
| PR | 35 | 12 | 11 | 5 | 3 | 1 | 4 | 1 | 0 | 2 | 5 | 5 | 36 | 38 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

TABLE 21

Protection ratios (dB) for a DVB-T 7 MHz signal interfered with by an 
overlapping 8 MHz analogue TV signal including sound

| Unwanted signal: 8 MHz analogue TV system | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 10.25 | 9.75 | 9.25 | 8.75 | 7.25 | 3.45 | 3.25 | 2.25 | 1.25 | 0 | 1.75 | 2.75 | 4.25 | 4.75 |
| PR | 35 | 12 | 11 | 5 | 3 | 1 | 4 | 1 | 0 | 2 | 5 | 5 | 36 | –38 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

TABLE 22

Protection ratios (dB) for a DVB-T 8 MHz signal interfered with by an 
overlapping 8 MHz analogue TV signal including sound

| Unwanted signal: 8 MHz analogue TV system | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | –10.75 | –10.25 | –9.75 | –9.25 | 7.75 | 3.45 | 3.25 | 2.25 | 1.25 | 0 | 2.25 | 3.25 | 4.75 | 5.25 |
| PR | 35 | 12 | 11 | 5 | 3 | 1 | 4 | 1 | 0 | 2 | 5 | 5 | 36 | –38 |

The frequency difference f is the vision carrier frequency of the analogue television signal minus the centre frequency of the DVB-T signal.

## 1.3	Protection of DVB-T digital terrestrial television signal from CW or FM signals

TABLE 23

Co-channel protection ratios (dB) for a DVB-T 8 MHz 64-QAM code rate 2/3
signal interfered with by a CW or a FM carrier
(non-controlled frequency offset)

| Unwanted signal: CW or FM carrier | Wanted signal: DVB-T, 8 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 12 | 4.5 | 3.9 | 0 | 3.9 | 4.5 | 12 |
| PR | 38 | 33 | 3 | 3 | 3 | 33 | 38 |

TABLE 24

Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3
signal interfered with by a CW or an FM carrier
(non-controlled frequency offset)

| Unwanted signal: CW or FM carrier | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  f (MHz) | 10.5 | 4.0 | 3.4 | 0 | 3.4 | 4.0 | 10.5 |
| PR | 38 | 33 | 3 | 3 | 3 | 33 | 38 |

The given protection ratio Tables can be used for interfering signals with narrow bandwidth e.g. analogue sound carriers or non-broadcast services.

TABLE 25

Co-channel protection ratios (dB) for a DVB-T 7 MHz 64-QAM code rate 2/3
signal interfered with by a CW carrier
(controlled frequency offset)

| Unwanted signal: CW carrier | Wanted signal: DVB-T, 7 MHz, 64-QAM, code rate 2/3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f (MHz) | –8 | – 4 | –3 | 0 | 3 | 4 | 8 |
| PR | – 48 | – 41 | –8 | –9 | –6 | –39 | – 48 |

The given protection ratio Tables can be used for interfering signals with narrow bandwidth e.g. analogue sound carriers or non-broadcast services. It should be noted that the fine structure of the protection ratio versus frequency offset between the OFDM signal and the interfering CW signal exhibits a cyclic variation. The values shown in Table 25 are for the optimum offset.

## 1.4	Protection of DVB-T digital terrestrial television signals from T-DAB signals

TABLE 26

Co-channel protection ratios (dB) for a DVB-T 7 MHz
and 8 MHz signal interfered with by a T-DAB signal

| Wanted signal DVB-T |  | PR |
| --- | --- | --- |
| Constellation | Code rate |  |
| QPSK | 1/2 | 10 |
| QPSK | 2/3 | 12 |
| QPSK | 3/4 | 14 |
| 16-QAM | 1/2 | 15 |
| 16-QAM | 2/3 | 18 |
| 16-QAM | 3/4 | 20 |
| 64-QAM | 1/2 | 20 |

TABLE 26 (end)

| Wanted signal DVB-T |  | PR |
| --- | --- | --- |
| Constellation | Code rate |  |
| 64-QAM | 2/3 | 24 |
| 64-QAM | 3/4 | 26 |
| 64-QAM | 7/8 | 31 |

NOTE 1 – The given protection ratios for DVB-T signals represent the worst case of interference from T-DAB.

TABLE 27

Protection ratios (dB) for a DVB-T 7 MHz and 8 MHz signal interfered with by 
a T-DAB signal in the lower (N – 1) or upper (N  1) adjacent channels

| Channel | N – 1 | N  1 |
| --- | --- | --- |
| PR | –30 | –30 |

The protection ratio is given in dB.

## 1.5	Protection of DVB-T from wideband signals other than terrestrial broadcasting

### 1.5.1 	Protection ratios for DVB-T interfered with by the fixed service (relocatable system)

TABLE 28

Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3 signal 
interfered with by emissions of the fixed service

| Df (MHz) | –12 | –4.5 | –3.75 | 0 | 3.75 | 4.5 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PR (dB) | –45 | –27 | 1 | 4 | 1 | –27 | –45 |
| Df: difference between the centre frequencies. |  |  |  |  |  |  |  |

Technical characteristics of the interferer

–	Modulation 2-FSK

–	Bandwidth: 750 kHz (3 dB)

### 1.5.2	Protection ratios for DVB-T interfered with by code-division multiple access (CDMA)

It should be noted that the Tables 29 and 30 refer to the situation where one single channel of the CDMA system is interfering with DVB-T.

TABLE 29

Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3
signal interfered with by emissions of CDMA-1X

| Df (MHz) | –12 | –4.5 | –3.75 | 0 | 3.75 | 4.5 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PR (dB) | –38 | –20 | –3 | 10 | –3 | –20 | –38 |
| Df: difference between the centre frequencies. |  |  |  |  |  |  |  |

Characteristics of the interfering signal

–	Modulation:		QPSK

–	Bandwidth:		1.25 MHz (99%)

TABLE 30

Protection ratios for DVB-T 8 MHz 64-QAM code rate 2/3
signal interfered with by emissions of CDMA-3X

| Df (MHz) | –12 | –4.5 | –3.75 | 0 | 3.75 | 4.5 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PR (dB) | –38 | 8 | 13 | 18 | 13 | 8 | –38 |
| Df: difference between the centre frequencies. |  |  |  |  |  |  |  |

Characteristics of the interfering signal

–	Modulation:		QPSK

–	Bandwidth:		4 MHz (99%)

# 2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals

Tables 31 to 38 show protection ratios for a wanted 625-line analogue television signal interfered with by a DVB-T digital terrestrial television signal.

## 2.1	Protection ratios for 625-line television systems

### 2.1.1	Protection of wanted vision signals interfered with by DVB-T digital terrestrial television signal

In this section the protection ratios for an analogue wanted signal interfered by an unwanted DVB-T digital signal relate only to the interference to the vision signal.

The protection ratio values given are related to an out-of-channel spectrum attenuation of the unwanted DVB-T transmitter of 40 dB.

#### 2.1.1.1	Protection from co-channel interference

TABLE 31

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an unwanted DVB-T 8 MHz signal

| Wanted signal: analogue system | Unwanted signal: DVB-T, 8 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B, D, D1, G, H, K/PAL | 34 | 40 |
| I/PAL | 37 | 41 |
| B, D, K, L/SECAM | 35 | 41 |

TABLE 32

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
an unwanted DVB-T 7 MHz signal

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 MHz |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B/PAL, B/SECAM | 35 | 41 |

#### 2.1.1.2	Protection from lower adjacent channel interference

TABLE 33

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
DVB-T 7 MHz and 8 MHz signals (lower adjacent channel)

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 or 8 MHz (lower adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| B, D, D1, G, H, I, K/PAL | –9 | –5 |
| B, D, K, L/SECAM | –5 | –1 |

#### 2.1.1.3	Protection from upper adjacent channel interference

TABLE 34

Protection ratios (dB) for a wanted analogue vision signal interfered with by 
DVB-T 7 MHz and 8 MHz signals (upper adjacent channel)

| Wanted signal: analogue system | Unwanted signal: DVB-T, 7 or 8 MHz (upper adjacent channel) |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| PAL and SECAM | –8 | –5 |

#### 2.1.1.4	Protection from image channel interference

TABLE 35

Protection ratios (dB) for a wanted analogue vision signal interfered with by
a DVB-T 8 MHz signal (image channel)

| Wanted analogue system | Unwanted DVB-T channel | Tropospheric interference | Continuous interference |
| --- | --- | --- | --- |
| D1, G/PAL | N  9 | –19 | –15 |
| I/PAL | N  9 |  |  |
| L/SECAM (1) | N  9 | –24 | –22 |
| D, K/SECAM (1) | N  8, N  9 | –16 | –11 |
| D, K/PAL | N  8, N  9 |  |  |
| (1) Provisional values still under study. |  |  |  |

TABLE 36

Protection ratios (dB) for a wanted analogue vision signal interfered with by
a DVB-T 7 MHz signal (image channel)

| Wanted analogue system | Unwanted DVB-T channel | Tropospheric interference | Continuous interference |
| --- | --- | --- | --- |
| B/PAL | N  10, N  11 | –22 | –18 |

#### 2.1.1.5	Protection from overlapping interference

TABLE 37

Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals* 
interfered with by a DVB-T 7 MHz signal
(overlapping channels)

| Frequency of the centre of the unwanted DVB-T signal minus the vision carrier frequency of the wanted analogue television signal (MHz) | Protection ratio |  |
| --- | --- | --- |
|  | Tropospheric interference | Continuous interference |
| –7.75 | –16 | –11 |
| (N – 1) – 4.75 | –9 | –5 |
| – 4.25 | –3 | 4 |
| –3.75 | 13 | 21 |
| –3.25 | 25 | 31 |
| –2.75 | 30 | 37 |
| –1.75 | 34 | 40 |
| –0.75 | 35 | 41 |
| (N ) 2.25 | 35 | 41 |
| 4.25 | 35 | 40 |
| 5.25 | 31 | 38 |
| 6.25 | 28 | 35 |
| 7.25 | 26 | 33 |
| 8.25 | 6 | 12 |
| (N  1) 9.25 | –8 | –5 |
| 12.25 | –8 | –5 |
| * For all SECAM systems similar values are expected. The values are still under study. |  |  |

TABLE 38

Protection ratios (dB) for analogue B, D, D1, G, H, K/PAL vision signals*
interfered with by a DVB-T 8 MHz signal
(overlapping channels)

| Frequency of the centre of the unwanted DVB-T signal minus the vision carrier frequency of the wanted analogue television signal (MHz) | Protection ratio |  |
| --- | --- | --- |
|  | Tropospheric interference (1) | Continuous interference (1) |
| – 8.25 | –16 | –11 |
| (N – 1) –5.25 | –9 | –5 |
| – 4.75 | – 4 | 3 |
| – 4.25 | 12 | 20 |
| –3.75 | 24 | 30 |
| –3.25 | 29 | 36 |
| –2.25 | 33 | 39 |
| –1.25 | 34 | 40 |
| (N ) 2.75 | 34 | 40 |
| 4.75 | 34 | 39 |
| 5.75 | 30 | 37 |
| 6.75 | 27 | 34 |
| 7.75 | 25 | 32 |
| 8.75 | 5 | 11 |
| (N  1) 9.75 | –8 | –5 |
| 12.75 | –8 | –5 |
| * For all SECAM systems similar values are expected. The values are still under study. (1) The values for tropospheric and continuous interference have been arrived at from Table 37 by calculation. |  |  |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television signals interfered with by unwanted DVB-T digital terrestrial television signals

Tables 39 to 41 in this Annex show protection ratios for wanted FM, AM and NICAM television sound carriers interfered with by unwanted digital terrestrial television signals.

All protection ratios in this section refer to the level of the wanted television sound carriers. The reference level of the sound carriers is the r.m.s. value of the unmodulated carrier.

The sound quality for tropospheric interference corresponds to grade 3, for continuous interference to grade 4.

The reference (S/Ns) for FM sound signals are:

–	40 dB (approximates to impairment grade 3) – tropospheric case;

–	48 dB (approximates to impairment grade 4) – continuous case.

The reference S/Ns are measured as S/N peak-to-peak weighted, given in Recommendation ITU-R BS.468 and Recommendation ITU-R BS.412.

The reference FM sound signal level corresponds to a maximum frequency deviation of  50 kHz.

The reference BERs for NICAM digital sound signals are:

–	BER  1  10–4 (approximates to impairment grade 3), tropospheric case;

–	BER  1  10–5 (approximates to impairment grade 4), continuous case.

In the case of a two-sound carrier transmission, each of the two-sound signals must be considered separately. Multiplex modulated sound signals may require higher protection.

## 3.1	Protection for FM, AM and NICAM sound signals of analogue television systems interfered with by DVB-T digital terrestrial television signals

TABLE 39

Co-channel protection (dB) ratios for a wanted sound signal
interfered with by DVB-T digital terrestrial television signals

| Protection ratio related to the wanted sound carrier |  | Unwanted signal |  |
| --- | --- | --- | --- |
| Wanted sound signal |  | DVB-T 7 MHz | DVB-T 8 MHz |
| FM | Tropospheric case | 6 | 5 |
|  | Continuous case | 16 | 15 |
| AM | Tropospheric case | 21 | 20 |
|  | Continuous case | 24 | 23 |
| NICAM | Tropospheric case | 5 | 4 |
| PAL B/G | Continuous case | 6 | 5 |
| NICAM | Tropospheric case |  |  |
| System I | Continuous case |  |  |
| NICAM | Tropospheric case | 12 | 11 |
| System L | Continuous case | 13 | 12 |

TABLE 40

Protection ratios (dB) for a wanted FM sound signal interfered with by 
a DVB-T 7 MHz signal (overlapping channels)

|  |  | Frequency of the 3 dB point of DVB-T signal minus sound carrier frequency |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency of the DVB-T signal relative to an FM carrier | Protection ratio related to the wanted sound carrier | –500 kHz | –250 kHz | –50 kHz | 0.0 kHz | 50 kHz | 250 kHz | 500 kHz |
| DVB-T below FM | Tropospheric case | 0 | 0 | 0 | 5 | 5 | 6 | 6 |
|  | Continuous case | 9 | 9 | 9 | 14 | 14 | 15 | 16 |
| DVB-T above FM | Tropospheric case | 5 | 5 | 4 | 3 | –9 | –22 | –32 |
|  | Continuous case | 15 | 15 | 14 | 12 | –6 | –16 | –27 |
| NOTE 1 – The protection ratio figures are related to an out-of-channel spectrum attenuation of 40 dB. NOTE 2 – This Table is still under study. |  |  |  |  |  |  |  |  |

TABLE 41

Protection ratios (dB) for a wanted AM sound signal interfered with by 
a DVB-T 8 MHz signal for different frequency offsets
(upper adjacent channel)

| Protection ratio related to the wanted sound carrier | Centre frequency of the DVB-T signal minus sound carrier frequency |  |  |
| --- | --- | --- | --- |
|  | With negative offset | No offset | With positive offset |
|  | 4.250 – 0.166 MHz = 4.084 MHz | 4.250 MHz | 4.250 + 0.166 MHz = 4.416 MHz |
| Tropospheric case | –1 | –2 | – 4 |
| Continuous case | 1 | 0 | –2 |

# 4	Protection ratios for a T-DAB signal interfered with by an unwanted digital terrestrial television signal

TABLE 42

Protection ratios (dB) for a T-DAB signal interfered with by
a DVB-T 8 MHz signal

| 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (1) (MHz) | –5 | – 4.2 | – 4 | –3 | 0 | 3 | 4 | 4.2 | 5 |
| PR | –50 | – 1 | 0 | 1 | 1 | 1 | 0 | –1 | –50 |
| (1)  f : centre frequency of the DVB-T signal minus centre frequency of the T-DAB signal. |  |  |  |  |  |  |  |  |  |

TABLE 43

Protection ratios (dB) for a T-DAB signal interfered with by
a DVB-T 7 MHz signal

| 64-QAM, code rate 2/3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  f (1) (MHz) | – 4.5 | – 3.7 | – 3.5 | – 2.5 | 0 | 2.5 | 3.5 | 3.7 | 4.5 |
| PR | – 49 | 0 | 1 | 2 | 2 | 2 | 1 | 0 | – 49 |
| (1)  f : centre frequency of the DVB-T signal minus centre frequency of the T-DAB signal. |  |  |  |  |  |  |  |  |  |

# 5	Minimum field strengths for DVB-T terrestrial digital television, fixed reception

The formula for calculating minimum field strength is given in Appendix 1 to Annex 2.

TABLE 44

Calculation of minimum field strength DVB-T 8 MHz system

| Frequency (MHz) | 200 |  |  | 550 |  |  | 700 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| System variant guard interval 1/4 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 | QPSK 2/3 | 16-QAM 2/3 | 64-QAM 2/3 |
| Receiver noise figure, F (dB) | 5 | 5 | 5 | 7 | 7 | 7 | 7 | 7 | 7 |
| Receiver carrier/noise ratio(1) (C/N) (dB) | 8 | 14 | 20 | 8 | 14 | 20 | 8 | 14 | 20 |
| Feeder loss Af (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 5 | 5 | 5 |
| Antenna gain, G (dB) | 5 | 5 | 5 | 10 | 10 | 10 | 12 | 12 | 12 |
| Minimum field strength for fixed reception, Emin (dB(µV/m))(2) | 27 | 33 | 39 | 33 | 39 | 45 | 35 | 41 | 47 |
| (1) For Rice channel. (2) For formula, see Appendix 1 to Annex 2. |  |  |  |  |  |  |  |  |  |

# 6	Minimum median field strength for mobile DVB-T reception

The equations for calculating the minimum median field strength are given in Appendix 1 to this Annex. The input values to the calculation are found in this section and in Annex 4. Mobile reception should be calculated with a location probability of 99%.

## 6.1	Required average C/N for mobile reception

For a given DVB-T mode the required average C/N for a certain quality level is a function of Doppler frequency only, and a graph like the one presented in Fig. 1 can be drawn.

FIGURE 1

Required average C /N in a mobile propagation channel

The minimum required average C/N values (C/Nmin), Doppler frequency for an average C/N equal to C/Nmin + 3 dB and the maximum Doppler (speed) limits for mobile reception are given in Table 45 and Table 46. The speed limits for C/Nmin + 3 dB are given for three frequencies (200 MHz, 500 MHz and 800 MHz). The average C/N value, C/Nmin + 3 dB, is suitable for calculation of required field strength. Table 45 shows values for the required average C/N and the speed limits in the non-diversity case. Table 46 contains the corresponding values for the diversity case. The values are based on the typical channel profile “typical urban” shown in Table 47. Quality criteria is the subjective failure point (SFP) corresponding to erroneous seconds ratio, –ESR = 5%, and packet error ratio, PER = 1  10−4.

TABLE 45

Required average C/N, speed limits for mobile reception for the non-diversity case

| Guard interval = 1/32 |  |  | 2k |  |  |  |  |  | 8k |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Speed at Fd , 3 dB (km/h) |  |  |  |  |  | Speed at Fd , 3 dB (km/h) |  |  |  |  |  |
| Modulation | Bit rate (Mbit/s) | Code rate | C/Nmin (dB) | Fd, max (Hz) | Fd at C/Nmin + 3 dB | 200 MHz | 500 MHz | 800 MHz | C/Nmin (dB) | Fd, max (Hz) | Fd at C/Nmin + 3 dB | 200 MHz | 500 MHz | 800 MHz |
| QPSK | 6.03 | 1/2 | 13.0 | 318 | 259 | 1 398 | 559 | 349 | 13.0 | 76 | 65 | 349 | 140 | 87 |
| QPSK | 8.04 | 2/3 | 16.0 | 247 | 224 | 1 207 | 483 | 302 | 16.0 | 65 | 53 | 286 | 114 | 71 |
| 16-QAM | 12.06 | 1/2 | 18.5 | 224 | 182 | 985 | 394 | 246 | 18.5 | 59 | 47 | 254 | 102 | 64 |
| 16-QAM | 16.09 | 2/3 | 21.5 | 176 | 147 | 794 | 318 | 199 | 21.5 | 41 | 35 | 191 | 76 | 48 |
| 64-QAM | 18.10 | 1/2 | 23.5 | 141 | 118 | 635 | 254 | 159 | 23.5 | 35 | 29 | 159 | 64 | 40 |
| 64-QAM | 24.13 | 2/3 | 27.0 | 82 | 65 | 349 | 140 | 87 | 27.0 | 24 | 18 | 95 | 38 | 24 |

TABLE 46

Required average C/N, speed limits for mobile reception for the diversity case

| Guard interval = 1/32 |  |  | 2k |  |  |  |  |  | 8k |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Speed at Fd , 3 dB (km/h) |  |  |  |  |  | Speed at Fd , 3 dB (km/h) |  |  |  |  |  |
| Modulation | Bit rate (Mbit/s) | Code rate | C/Nmin (dB) | Fd, max (Hz) | Fd at C/Nmin + 3 dB | 200 MHz | 500 MHz | 800 MHz | C/Nmin (dB) | Fd, max (Hz) | Fd at C/Nmin + 3 dB | 200 MHz | 500 MHz | 800 MHz |
| QPSK | 6.03 | 1/2 | 7.0 | 560 | 518 | 2 795 | 1 118 | 699 | 7.0 | 140 | 129 | 699 | 280 | 175 |
| QPSK | 8.04 | 2/3 | 10.0 | 494 | 447 | 2 414 | 966 | 604 | 10.0 | 129 | 106 | 572 | 229 | 143 |
| 16-QAM | 12.06 | 1/2 | 12.5 | 447 | 365 | 1 969 | 788 | 492 | 12.5 | 118 | 94 | 508 | 203 | 127 |
| 16-QAM | 16.09 | 2/3 | 15.5 | 353 | 294 | 1 588 | 635 | 397 | 15.5 | 82 | 71 | 381 | 152 | 95 |
| 64-QAM | 18.10 | 1/2 | 17.5 | 282 | 235 | 1 271 | 508 | 318 | 17.5 | 71 | 59 | 318 | 127 | 79 |
| 64-QAM | 24.13 | 2/3 | 21.0 | 165 | 129 | 699 | 280 | 175 | 21.0 | 47 | 35 | 191 | 76 | 48 |

TABLE 47

Channel profile for measurement of required average C/N for
mobile reception of DVB-T reception “typical urban”

| Tap number | Delay (µs) | Power (dB) | Doppler category |
| --- | --- | --- | --- |
| 1 | 0 | –3 | Rayleigh |
| 2 | 0.2 | 0 | Rayleigh |
| 3 | 0.5 | –2 | Rayleigh |
| 4 | 1.6 | –6 | Rayleigh |
| 5 | 2.3 | –8 | Rayleigh |
| 6 | 5 | –10 | Rayleigh |

The values for the bit rate correspond to the shortest guard interval 1/32 which is the least critical case in terms of Doppler. It is to be expected that when the guard interval increases the maximum speed decreases. For instance with 1/4 guard interval, the maximum Doppler, Fd max, decreases to about 85%.

The performance in a mobile channel depends to large extent on the design of the DVB-T receiver. Improvements may be achieved with receivers particularly designed for mobile reception.

DVB-H uses the DVB-T transmission system as the physical layer and adds extra error correction and time-slicing mechanism on the link layer. The maximum Doppler frequency (speed) in mobile reception will be improved due to the additional time interleaving. C/N values for DVB-H reception need to be developed.

## 6.2	Receiver noise figure

Noise figure of 5 dB is for integrated vehicle mobile receivers. A lower noise figure is possible when the antenna is internally matched to the first amplifier stage without a need for a loop through connection.

# 7	Minimum median field strength for hand held pedestrian indoor, pedestrian outdoor and mobile DVB-H reception

The equations for calculating the minimum median field strength are given in Appendix 1 to this Annex. The input values to the calculation are found in this section and in Annex 4. Mobile reception should be calculated with a location probability of 99%.

## 7.1	Channel models for hand held pedestrian indoor and outdoor reception

The pedestrian indoor (PI) and pedestrian outdoor (PO) channel models have been developed for describing the slowly moving hand held reception indoors and outdoors. The channel models are based on measurements in DVB-H Single Frequency Networks and have paths from two different transmitter locations. Definitions of the taps for the channels are given in Table 49 and Table 50. The indicated Doppler frequency of 1.5 Hz is corresponding 3 km/h velocity at middle of UHF-band. The Doppler spectra of various taps are defined in Table 48.

TABLE 48

Doppler spectrum definitions for PI- and PO-channels

| Spectrum for the 1st tap | Spectrum for taps 2-12 |
| --- | --- |
| 0.1G(f;0.08fD)+δ(f−0.5fD) | G(f;0.08fD) |
| where: G(f;σ)=exp−f22σ2 |  |

TABLE 49

Definition of PI-channel

| Path | Delay (s) | Power (dB) | Doppler Spectrum | Fd (Hz) | STD Norm. |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.0 | 0.0 | See Table 2 | 1.69 | 0.08 |
| 2 | 0.1 | –6.4 | Gauss | 1.69 | 0.08 |
| 3 | 0.2 | –10.4 | Gauss | 1.69 | 0.08 |
| 4 | 0.4 | –13.0 | Gauss | 1.69 | 0.08 |
| 5 | 0.6 | –13.3 | Gauss | 1.69 | 0.08 |
| 6 | 0.8 | –13.7 | Gauss | 1.69 | 0.08 |
| 7 | 1.0 | –16.2 | Gauss | 1.69 | 0.08 |
| 8 | 1.6 | –15.2 | Gauss | 1.69 | 0.08 |
| 9 | 8.1 | –14.9 | Gauss | 1.69 | 0.08 |
| 10 | 8.8 | –16.2 | Gauss | 1.69 | 0.08 |
| 11 | 9.0 | –11.1 | Gauss | 1.69 | 0.08 |
| 12 | 9.2 | –11.2 | Gauss | 1.69 | 0.08 |

TABLE 50

Definition of PO-channel

| Path | Delay (s) | Power (dB) | Doppler Spectrum | Fd (Hz) | STD Norm. |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.0 | 0.0 | See Table 2 | 1.69 | 0.08 |
| 2 | 0.2 | –1.5 | Gauss | 1.69 | 0.08 |
| 3 | 0.6 | –3.8 | Gauss | 1.69 | 0.08 |
| 4 | 1.0 | –7.3 | Gauss | 1.69 | 0.08 |
| 5 | 1.4 | –9.8 | Gauss | 1.69 | 0.08 |
| 6 | 1.8 | –13.3 | Gauss | 1.69 | 0.08 |
| 7 | 2.3 | –15.9 | Gauss | 1.69 | 0.08 |
| 8 | 3.4 | –20.6 | Gauss | 1.69 | 0.08 |

TABLE 50 (end)

| Path | Delay (s) | Power (dB) | Doppler Spectrum | Fd (Hz) | STD Norm. |
| --- | --- | --- | --- | --- | --- |
| 9 | 4.5 | –19.0 | Gauss | 1.69 | 0.08 |
| 10 | 5.0 | –17.7 | Gauss | 1.69 | 0.08 |
| 11 | 5.3 | –18.9 | Gauss | 1.69 | 0.08 |
| 12 | 5.7 | –19.3 | Gauss | 1.69 | 0.08 |

## 7.2 	Channel model for mobile reception

The channel model for mobile reception is given in Table 45. This typical urban model is valid for both DVB-T and DVB-H.

## 7.3	Required average C/N for hand held indoor and outdoor reception

The DVB-H receiver shall have the performance given in Table 51 when noise (N) is applied together with the wanted carrier (C) in a signal bandwidth of 7.61 MHz. Degradation point criteria is 5% MPE-FEC frame error rate (5% MFER). The C/N performance figures are based on the state of the art receivers on the market added with a 2 dB margin.

TABLE 51

C/N (dB) for 5% MFER in PI and PO channel

| Modulation | Code rate | MPE-FEC code rate | PI | PO |
| --- | --- | --- | --- | --- |
| QPSK | 1/2 | 1/2 | 6.6 | 7.6 |
| QPSK | 1/2 | 2/3 | 6.8 | 7.8 |
| QPSK | 1/2 | 3/4 | 7.0 | 8.0 |
| QPSK | 1/2 | 5/6 | 7.2 | 8.2 |
| QPSK | 1/2 | 7/8 | 7.4 | 8.4 |
| QPSK | 2/3 | 2/3 | 9.8 | 10.8 |
| QPSK | 2/3 | 3/4 | 10.0 | 11.0 |
| QPSK | 2/3 | 5/6 | 10.2 | 11.2 |
| QPSK | 2/3 | 7/8 | 10.4 | 11.4 |
| 16-QAM | 1/2 | 2/3 | 12.8 | 13.8 |
| 16-QAM | 1/2 | 3/4 | 13.0 | 14.0 |
| 16-QAM | 1/2 | 5/6 | 13.2 | 14.2 |
| 16-QAM | 1/2 | 7/8 | 13.4 | 14.4 |
| 16-QAM | 2/3 | 2/3 | 15.8 | 16.8 |
| 16-QAM | 2/3 | 3/4 | 16.0 | 17.0 |
| 16-QAM | 2/3 | 5/6 | 16.2 | 17.2 |
| 16-QAM | 2/3 | 7/8 | 16.4 | 17.4 |

TABLE 51 (end)

| Modulation | Code rate | MPE-FEC code rate | PI | PO |
| --- | --- | --- | --- | --- |
| 64-QAM | 1/2 | 5/6 | 17.7 | 18.7 |
| 64-QAM | 1/2 | 7/8 | 17.9 | 18.9 |
| 64-QAM | 2/3 | 2/3 | 20.6 | 21.6 |
| 64-QAM | 2/3 | 3/4 | 20.8 | 21.8 |
| 64-QAM | 2/3 | 5/6 | 21.0 | 22.0 |

## 7.4	Required average C/N for hand held indoor and outdoor reception

The DVB-H receiver shall have the performance given in Table 52 when noise (N) and Doppler shift (Fd) is applied together with the wanted carrier (C) in mobile channel defined in Table 45. The figures are given for guard interval 1/4. The C/N performance is based on the state of the art DVB-H receivers with added 2 dB margin. The Doppler performance is derived from a use case analysis where the target speed with 8k mode at 750 MHz is 130 km/h. This corresponds to a Doppler frequency of 100 Hz. The 4k and 2k Doppler performance is obtained by multiplying the 8k performance by 2 and 4. Degradation point criteria is 5% MPE-FEC frame error rate (5% MFER).

TABLE 52

DVB-H C/N (dB) in mobile channel for 5% MFER

| Guard interval = 1/4 |  |  | 2k |  | Speed at Fd, 3 dB km/h |  | 4k |  | Speed at Fd, 3 dB km/h |  | 8k |  | Speed at Fd, 3 dB km/h |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modula-tion | Code rate | MPE-FEC CR | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz |
| QPSK | 1/2 | 1/2 | 8.5 | 400 | 911 | 579 | 8.5 | 200 | 456 | 290 | 8.5 | 100 | 228 | 145 |
|  |  | 2/3 | 9.0 | 400 | 911 | 579 | 9.0 | 200 | 456 | 290 | 9.0 | 100 | 228 | 145 |
|  |  | 3/4 | 9.5 | 400 | 911 | 579 | 9.5 | 200 | 456 | 290 | 9.5 | 100 | 228 | 145 |
|  |  | 5/6 | 10.0 | 400 | 911 | 579 | 10.0 | 200 | 456 | 290 | 10.0 | 100 | 228 | 145 |
|  |  | 7/8 | 10.5 | 400 | 911 | 579 | 10.5 | 200 | 456 | 290 | 10.5 | 100 | 228 | 145 |
| QPSK | 2/3 | 2/3 | 12.0 | 400 | 911 | 579 | 12.0 | 200 | 456 | 290 | 12.0 | 100 | 228 | 145 |
|  |  | 3/4 | 12.5 | 400 | 911 | 579 | 12.5 | 200 | 456 | 290 | 12.5 | 100 | 228 | 145 |
|  |  | 5/6 | 13.5 | 400 | 911 | 579 | 13.5 | 200 | 456 | 290 | 13.5 | 100 | 228 | 145 |
|  |  | 7/8 | 14.5 | 400 | 911 | 579 | 14.5 | 200 | 456 | 290 | 14.5 | 100 | 228 | 145 |
| 16-QAM | 1/2 | 2/3 | 15.0 | 400 | 911 | 579 | 15.0 | 200 | 456 | 290 | 15.0 | 100 | 228 | 145 |
|  |  | 3/4 | 15.5 | 400 | 911 | 579 | 15.5 | 200 | 456 | 290 | 15.5 | 100 | 228 | 145 |
|  |  | 5/6 | 16.5 | 400 | 911 | 579 | 16.5 | 200 | 456 | 290 | 16.5 | 100 | 228 | 145 |
|  |  | 7/8 | 17.5 | 400 | 911 | 579 | 17.5 | 200 | 456 | 290 | 17.5 | 100 | 228 | 145 |
| 16-QAM | 2/3 | 2/3 | 18.0 | 380 | 866 | 550 | 18.0 | 190 | 433 | 275 | 18.0 | 95 | 216 | 138 |
|  |  | 3/4 | 18.5 | 380 | 866 | 550 | 18.5 | 190 | 433 | 275 | 18.5 | 95 | 216 | 138 |
|  |  | 5/6 | 19.5 | 380 | 866 | 550 | 19.5 | 190 | 433 | 275 | 19.5 | 95 | 216 | 138 |
|  |  | 7/8 | 20.5 | 380 | 866 | 550 | 20.5 | 190 | 433 | 275 | 20.5 | 95 | 216 | 138 |

TABLE 52 (end)

| Guard interval = 1/4 |  |  | 2k |  | Speed at Fd, 3 dB km/h |  | 4k |  | Speed at Fd, 3 dB km/h |  | 8k |  | Speed at Fd, 3 dB km/h |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modula-tion | Code rate | MPE-FEC CR | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz | C/Nmin dB | Fd, 3 dB Hz | 474 MHz | 746 MHz |
| 64-QAM | 1/2 | 5/6 | 21.5 | 200 | 456 | 290 | 21.5 | 100 | 228 | 145 | 21.5 | 50 | 114 | 73 |
|  |  | 7/8 | 22.5 | 200 | 456 | 290 | 22.5 | 100 | 228 | 145 | 22.5 | 50 | 114 | 73 |
| 64-QAM | 2/3 | 2/3 | 25.0 | 120 | 273 | 174 | 25.0 | 60 | 137 | 87 | 25.0 | 30 | 68 | 43 |
|  |  | 3/4 | 25.5 | 120 | 273 | 174 | 25.5 | 60 | 137 | 87 | 25.5 | 30 | 68 | 43 |
|  |  | 5/6 | 27.0 | 120 | 273 | 174 | 27.0 | 60 | 137 | 87 | 27.0 | 30 | 68 | 43 |

## 7.5	Receiver noise figure

DVB-H receivers are expected to have a full interoperability with GSM-900 cellular radios and therefore have a GSM-reject filter in front of the DVB-H receiver. The total system noise figure of the receiver and the filter is 6 dB.

Appendix 1
to Annex 2

Calculation of minimum field strength and minimum 
median equivalent field strength

The minimum field strength and minimum median equivalent field strength values calculated using the following equations:

Pn	= 	F + 10 log (k T0 B)

Ps min	= 	C/N + Pn

Aa	= 	G + 10 log (1.642/4 )

φmin	= 	Ps min – Aa + Lf

Emin	= 	φmin + 120 + 10 log (120 )

= 	φmin + 145.8

Emed	= 	Emin + Pmmn + Cl 	for roof top level fixed reception

Emed	= 	Emin + Pmmn + Cl + Lh	for portable outdoor and mobile reception

Emed	= 	Emin + Pmmn + Cl + Lh + Lb	for portable indoor and mobile hand-held reception

Cl	= 	µ  t

t	= 	σb2+σm2

where:

Pn :	receiver noise input power (dBW)

F :	receiver noise figure (dB)

k :	Boltzmann’s constant (k = 1.38  10–23 (J/K))

T0 :	absolute temperature (T0 = 290 (K))

B :	receiver noise bandwidth (B = 7.61  106 (Hz))

Ps min :	minimum receiver input power (dBW)

C/N :	RF S/N at the receiver input required by the system (dB)

Aa :	effective antenna aperture (dBm2)

G :	antenna gain related to half dipole (dBd)

 :	wavelength of the signal (m)

φmin :	minimum pfd at receiving place (dB(W/m2))

Lf  :	feeder loss (dB)

Emin :	equivalent minimum field strength at receiving place (dB(V/m))

Emed :	minimum median equivalent field strength, planning value (dB(V/m))

Pmmn :	allowance for man-made noise (dB)

Lh :	height loss (reception point at 1.5 m above ground level) (dB)

Lb :	building or vehicle entry loss (dB)

Cl :	location correction factor (dB)

t :	total standard deviation (dB)

m :	standard deviation macro-scale (m = 5.5 (dB))

b :	standard deviation building entry loss (dB)

µ :	distribution factor being 0.52 for 70%, 1.28 for 90%, 1.64 for 95% and 2.33 for 99%.

Annex 3

Planning criteria for ISDB-T digital television system in the VHF/UHF bands

# 1	Protection ratios for ISDB-T wanted digital terrestrial television signals

Tables 53 to 55 and 56 to 58 show protection ratios for an ISDB-T wanted digital terrestrial television signal interfered with by an ISDB-T digital terrestrial television signal and by an analogue terrestrial television signal, respectively.

## 1.1	Protection of an ISDB-T digital terrestrial television signal interfered with by an ISDB-T digital television signal

TABLE 53

Co-channel protection ratios (dB) for ISDB-T 6 MHz
interfered with by ISDB-T 6 MHz

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 10 | 17 | 23 |
| 5/6 | 9 | 16 | 22 |
| 3/4 | 9 | 15 | 21 |
| 2/3 | 8 | 14 | 20 |
| 1/2 | 6 | 12 | 17 |

TABLE 54

Protection ratios (dB) for ISDB-T 6 MHz interfered with by
ISDB-T 6 MHz in lower adjacent channel (N  1)

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 28 | 27 | 24 |
| 5/6 | 28 | 27 | 25 |
| 3/4 | 29 | 27 | 26 |
| 2/3 | 29 | 28 | 26 |
| 1/2 | 29 | 28 | 27 |

TABLE 55

Protection ratios (dB) for ISDB-T 6 MHz interfered with by
ISDB-T 6 MHz in upper adjacent channel (N + 1)

| Coding rate | Modulation |  |  |
| --- | --- | --- | --- |
|  | DQPSK | 16-QAM | 64-QAM |
| 7/8 | 29 | 28 | 27 |
| 5/6 | 29 | 28 | 27 |
| 3/4 | 29 | 28 | 27 |
| 2/3 | 30 | 29 | 27 |
| 1/2 | 30 | 29 | 28 |

## 1.2	Protection of ISDB-T digital terrestrial television interfered with by analogue terrestrial television

### 1.2.1	Protection from co-channel interference

TABLE 56

Co-channel protection ratios (dB) for ISDB-T 6 MHz
interfered with by analogue television

|  | Protection ratio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | DQPSK |  |  |  |  | QPSK |  |  |  |  |
| Code rate | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 |
| M/NTSC | 5 | 3 | 1 | 2 | 6 | 16 | 11 | 8 | 0 | 2 |
| Modulation | 16-QAM |  |  |  |  | 64-QAM |  |  |  |  |
| Code rate | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 | 1/2 | 2/3 | 3/4 | 5/6 | 7/8 |
| M/NTSC | 11 | 5 | 1 | 6 | 10 | 6 | 1 | 5 | 9 | 14 |
| NOTE 1 – The sound carrier level is 6 dB lower than NTSC vision carrier level. NOTE 2 – The values in this Table give the reception threshold. Taking into account the performance variation in domestic receivers, long-term degradation of reception condition etc., a margin of several dBs should preferably be added in actual frequency planning. |  |  |  |  |  |  |  |  |  |  |

### 1.2.2	Protection from lower adjacent channel (N – 1) interference

TABLE 57

Protection ratios (dB) for lower adjacent channel (N – 1) interference for ISDB-T 6 MHz interfered with by NTSC signals including sound

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Modulation | Coding rate | M/NTSC |
| DQPSK | 1/2 | 34 |
| DQPSK | 2/3 | 34 |
| DQPSK | 3/4 | 33 |
| 16-QAM | 1/2 | 34 |
| 16-QAM | 2/3 | 33 |
| 16-QAM | 3/4 | 32 |
| 64-QAM | 2/3 | 32 |
| 64-QAM | 3/4 | 31 |
| 64-QAM | 5/6 | 29 |
| 64-QAM | 7/8 | 29 |

### 1.2.3	Protection from upper adjacent channel (N  1) interference

TABLE 58

Protection ratios (dB) for upper adjacent channel (N  1) interference for
ISDB-T 6 MHz interfered with by NTSC 6 MHz signal

| Wanted signal |  | Unwanted signal |
| --- | --- | --- |
| Modulation | Coding rate | M/NTSC |
| DQPSK | 1/2 | 35 |
| DQPSK | 2/3 | 35 |
| DQPSK | 3/4 | 34 |
| 16-QAM | 1/2 | 35 |
| 16-QAM | 2/3 | 34 |
| 16-QAM | 3/4 | 33 |
| 64-QAM | 2/3 | 33 |
| 64-QAM | 3/4 | 33 |
| 64-QAM | 5/6 | 32 |
| 64-QAM | 7/8 | 31 |

# 2	Protection ratios for wanted analogue terrestrial television signals interfered with by unwanted ISDB-T digital terrestrial television signals

Table 59 shows protection ratios for a 525-line analogue television signal interfered with by an ISDB-T digital terrestrial television signal.

The protection ratio values given are related to a spectrum shoulder attenuation of the unwanted digital signal of 38 dB.

## 2.1	Protection ratios for 525-line television systems

### 2.1.1	Protection for NTSC vision signals interfered with by ISDB-T digital television signals

TABLE 59

Protection ratios (dB) for analogue vision signal (NTSC, 6 MHz)
interfered with by ISDB-T signal

| Unwanted digital channel | Tropospheric interference | Continuous interference |
| --- | --- | --- |
| N – 1 (lower) | 6 | 3 |
| N (co-channel) | 39 | 44 |
| N + 1 (upper) | 6 | 3 |

# 3	Protection ratios for sound signals of wanted analogue terrestrial television systems interfered with by ISDB-T unwanted digital terrestrial television system

## 3.1	Protection for NTSC sound signals interfered with by an ISDB-T digital television signal

As shown in Table 60, the sound signal of NTSC broadcasting is robust compared with the vision signal when interfered with by an ISDB-T signal. Therefore protection ratios for NTSC broadcasting are determined by the protection ratios for the vision signal, which are shown in Table 59.

TABLE 60

Sound quality related to the vision protection ratio of grade 3 when
NTSC 6 MHz signal interfered with by ISDB-T 6 MHz signal

|  | Measurement condition (see Note 3) | Sound quality |
| --- | --- | --- |
| Co-channel interference | D/U  39 dB | > grade 4 (S/N = 54 dB) |
| Upper adjacent channel interference | D/U  6 dB |  grade 4 (S/N  53 dB) |
| Lower adjacent channel interference | D/U  6 dB |  grade 4 (S/N  52 dB) |
| NOTE 1 – The sound carrier level is 6 dB lower than NTSC vision carrier level. NOTE 2 – The reference FM sound signal level corresponds to a maximum frequency deviation of  25 kHz. NOTE 3 – D/U ratio that gives vision quality of grade 3 (corresponding to protection ratio for tropospheric interference). |  |  |

# 4	Minimum field strengths for ISDB-T terrestrial digital television

To reduce the number of tables for the minimum field strengths of an ISDB-T system, the derivation of minimum field strengths is made with the typical transmission modes shown in Table 61. The minimum field strengths for the different modes can be calculated from the given values in Table 61.

# 5	Minimum median field strength for hand-held pedestrian indoor, pedestrian outdoor and mobile ISDB-T reception

The equations for calculating the minimum median field strength are given in Appendix 1 to this Annex, and the values that were used in the calculation are in this section and Annex 4. To calculate the minimum median field strength for pedestrian indoor, pedestrian outdoor, and mobile ISDB-T reception, the value of the location probability should be determined by the administration that has the transmitting stations on its territory.

## 5.1	Channel models for hand-held pedestrian indoor and outdoor reception

The pedestrian indoor (PI) and the pedestrian outdoor (PO) channel models are used as channel models for hand-held reception. Definitions of the taps for the channels are given in Tables 49 and 50. The Doppler spectra of various taps are defined in Table 48.

TABLE 61

Derivation by the voltage method ISDB-T 6 MHz system

| Frequency (MHz) | Low VHF |  |  |  | High VHF |  |  |  |  | UHF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 100 |  |  |  | 200 |  |  |  |  | 600 |  |  |  |  |
| System | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 |  | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 | DQPSK 1/2 | QPSK 1/2 | 16-QAM 3/4 | 64-QAM 7/8 |  |
| Noise bandwidth, B (MHz) | 5.6 | 5.6 | 5.6 | 5.6 |  | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 | 5.6 |  |
| Receiver noise figure, F (dB) | 5 | 5 | 5 | 5 |  | 5 | 5 | 5 | 5 | 7 | 7 | 7 | 7 |  |
| Receiver noise input voltage, UN (1) (dB(V)) | 7.1 | 7.1 | 7.1 | 7.1 |  | 7.1 | 7.1 | 7.1 | 7.1 | 9.1 | 9.1 | 9.1 | 9.1 |  |
| Receiver carrier/noise ratio(2) (C/N) (dB) | 6.2 | 4.9 | 14.6 | 22.0 |  | 6.2 | 4.9 | 14.6 | 22.0 | 6.2 | 4.9 | 14.6 | 22.0 |  |
| Urban noise (dB) | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |  |
| Minimum receiver input voltage, Umin (dB(V))(1) | 14.3 | 13.0 | 22.7 | 30.1 |  | 14.3 | 13.0 | 22.7 | 30.1 | 15.3 | 14.0 | 23.7 | 31.1 |  |
| Conversion factor (1) K (dB) | 6.4 | 6.4 | 6.4 | 6.4 |  | 12.4 | 12.4 | 12.4 | 12.4 | 21.9 | 21.9 | 21.9 | 21.9 |  |
| Feeder loss, Lf (dB) | 3 | 3 | 3 | 3 |  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |  |
| Antenna gain, G (dB) | 3 | 3 | 3 | 3 |  | 5 | 5 | 5 | 5 | 10 | 10 | 10 | 10 |  |
| Minimum field strength for fixed reception, Emin (dB(V/m))(1) | 20.7 | 19.4 | 29.1 | 36.5 |  | 24.7 | 23.4 | 33.1 | 40.5 | 30.2 | 28.9 | 38.6 | 46.0 |  |
| (1) For formula, see Appendix 1 to Annex 3. (2) For noise bandwidth noted above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.2	Channel model for mobile reception

The typical urban model is used as the channel model for mobile reception. The channel model for mobile reception is given in Table 47.

## 5.3	Required average C/N for hand-held indoor and outdoor reception

The ISDB-T receiver shall have the performance given in Table 62 when noise (N) is applied together with the wanted carrier (C) in a signal bandwidth of 5.57 MHz. The values for the C/N ratio in both the non-diversity case and the diversity case are shown in Table 62. Degradation point criterion is an erroneous second ratio (ESR) of 5%.

TABLE 62

C/N (dB) for 5% ESR in PI and PO channel

| Mode | Modulation | Code rate | For the non-diversity case |  | For the diversity case |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PI | PO | PI | PO |
| 2 k | QPSK | 1/2 | 10 | 10.5 | 5 | 5 |
|  | QPSK | 2/3 | 13 | 13.5 | 7.5 | 7.5 |
|  | 16-QAM | 1/2 | 15.5 | 16 | 11 | 11 |
|  | 16-QAM | 2/3 | 19 | 20 | 13.5 | 13.5 |
|  | 64-QAM | 1/2 | 20.5 | 20.5 | 16 | 16 |
|  | 64-QAM | 2/3 | 24.5 | 24.5 | 19 | 19 |
|  | 64-QAM | 3/4 | 27 | 27 | 20.5 | 20.5 |
| 4 k | QPSK | 1/2 | 10 | 10 | 5 | 5.5 |
|  | QPSK | 2/3 | 13 | 13 | 7.5 | 8 |
|  | 16-QAM | 1/2 | 15.5 | 15.5 | 10.5 | 11 |
|  | 16-QAM | 2/3 | 19 | 19.5 | 13 | 13.5 |
|  | 64-QAM | 1/2 | 20.5 | 20.5 | 16 | 16 |
|  | 64-QAM | 2/3 | 24.5 | 25 | 19 | 19 |
|  | 64-QAM | 3/4 | 27 | 27 | 20.5 | 20.5 |
| 8 k | QPSK | 1/2 | 10 | 10 | 5 | 5.5 |
|  | QPSK | 2/3 | 13.5 | 13.5 | 7.5 | 7.5 |
|  | 16-QAM | 1/2 | 15.5 | 16 | 11 | 11 |
|  | 16-QAM | 2/3 | 19.5 | 19.5 | 13.5 | 13.5 |
|  | 64-QAM | 1/2 | 20.5 | 21 | 16 | 16 |
|  | 64-QAM | 2/3 | 24.5 | 24.5 | 19 | 19 |
|  | 64-QAM | 3/4 | 27 | 27 | 20.5 | 21 |

## 5.4	Required average C/N for mobile reception

The ISDB-T receiver shall have the performance given in Table 63 when noise (N) and Doppler shift (Fd) is applied together with the wanted carrier (C) in the mobile channel based on the channel profile, “TYPICAL URBAN”, which is shown in Table 47. The speed limits for C/Nmin + 3 dB are given for two frequencies (470 and 770 MHz). The average C/N value, C/Nmin + 3 dB, is suitable for calculating the required field strength. The values for the required average C/N ratio and the speed limits in the non-diversity case are shown in Table 63a), and the corresponding values for the diversity case are shown in Table 63b). The figures are given for a guard interval of 1/8. Degradation point criterion is given by an ESR of 5%.

TABLE 63

ISDB-T C/N (dB) in mobile channel for 5% ESR

a)   Non diversity case

| Guard interval = 1/8 |  | 2 k |  | Speed at Fd, 3 dB km/h |  | 4 k |  | Speed at Fd, 3 dB km/h |  | 8 k |  | Speed at Fd, 3 dB km/h |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modula-tion | Code rate | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz |
| QPSK | 1/2 | 8 | 360 | 827 | 505 | 8.5 | 170 | 391 | 238 | 8.5 | 89 | 205 | 125 |
| QPSK | 2/3 | 11.5 | 310 | 712 | 435 | 12 | 140 | 322 | 196 | 11.5 | 74 | 170 | 104 |
| 16-QAM | 1/2 | 13.5 | 270 | 620 | 379 | 13.5 | 130 | 299 | 182 | 14.5 | 67 | 154 | 94 |
| 16-QAM | 2/3 | 17.5 | 200 | 460 | 281 | 17.5 | 100 | 230 | 140 | 18 | 50 | 115 | 70 |
| 64-QAM | 1/2 | 19 | 180 | 414 | 252 | 19 | 89 | 205 | 125 | 19.5 | 42 | 97 | 59 |
| 64-QAM | 2/3 | 23.5 | 110 | 253 | 154 | 24 | 60 | 138 | 84 | 24.5 | 28 | 64 | 39 |
| 64-QAM | 3/4 | 26.5 | 100 | 230 | 140 | 27 | 49 | 113 | 69 | 27.5 | 23 | 53 | 32 |

b)   Diversity case

| Guard interval = 1/8 |  | 2 k |  | Speed at Fd, 3 dB km/h |  | 4 k |  | Speed at Fd, 3 dB km/h |  | 8 k |  | Speed at Fd, 3 dB km/h |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modula-tion | Code rate | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz | C/Nmin dB | Fd, 3 dB Hz | 470 MHz | 770 MHz |
| QPSK | 1/2 | 4 | 420 | 965 | 589 | 4 | 210 | 483 | 295 | 4 | 100 | 230 | 140 |
| QPSK | 2/3 | 6 | 390 | 896 | 547 | 6.5 | 190 | 437 | 266 | 6 | 96 | 221 | 135 |
| 16-QAM | 1/2 | 9 | 350 | 804 | 491 | 9 | 170 | 391 | 238 | 9.5 | 85 | 195 | 119 |
| 16-QAM | 2/3 | 12 | 280 | 643 | 393 | 12 | 140 | 322 | 196 | 12 | 70 | 161 | 98 |
| 64-QAM | 1/2 | 14.5 | 230 | 529 | 323 | 14.5 | 110 | 253 | 154 | 15 | 57 | 131 | 80 |
| 64-QAM | 2/3 | 18.5 | 180 | 414 | 252 | 18 | 91 | 209 | 128 | 18.5 | 43 | 99 | 60 |
| 64-QAM | 3/4 | 20 | 160 | 368 | 224 | 20 | 79 | 182 | 111 | 20 | 38 | 87 | 53 |

The performance in a mobile channel depends to a large extent on the design of the ISDB-T receiver. Using receivers designed for mobile reception may lead to a higher level of performance.

## 5.5	Receiver noise figure

The values of the noise figure are 5 dB in the VHF band and 7 dB in the UHF band for ISDB-T receivers.

Appendix 1
to Annex 3

Calculation of minimum field strength and minimum 
median equivalent field strength

The minimum field strength and minimum median equivalent field-strength values were calculated by using the following equations:

Pn	= 	F + 10 log (k T0 B)

UN	= 	Pn+ 120 + 10 log R

Ps min	= 	C/N + Pn

Umin	= 	Ps min + 120 + 10 log R

Aa	= 	G + 10 log (1.64 2/4 )

φmin	= 	Ps min – Aa + Lf

Emin	= 	φmin + 120 + 10 log (120 )

= 	φmin + 145.8

= 	Ps min – Aa + Lf  + 120 + 10 log (120 )

= 	Umin  –10 log R  – G  – 10 log (1.64 2/4 ) + Lf  + 10 log (120 )

= 	Umin  + 20 log (2 /)  – G  + Lf

= 	Umin  +  K  – G  + Lf  			     K = 20 log (2 /)

Emed	= 	Emin + Pmmn + Cl 	for rooftop level fixed reception

Emed	= 	Emin + Pmmn + Cl + Lh	for portable outdoor and mobile reception

Emed	= 	Emin + Pmmn + Cl + Lh + Lb	for portable indoor and mobile hand-held reception

Cl	= 	µ  t

t	= 	σb2+σm2

where:

Pn :	receiver intrinsic noise power (dBW)

F :	receiver noise figure (dB)

k :	Boltzmann’s constant (k = 1.38  10−23 (J/K))

T0 :	absolute temperature (T0 = 290 (K))

B :	receiver noise bandwidth (B = 5.57  106, 6.50  106, 7.43  106 (Hz))

Un :	receiver noise input voltage (dB(V))

R :	antenna inpedance (R = 73.1())

Ps min :	minimum receiver input power (dBW)

Umin :	minimum receiver input voltage (dB(V))

C/N :	RF S/N at the receiver input required by the system (dB)

Aa :	effective antenna aperture (dBm2)

G :	antenna gain related to half dipole (dBd)

 :	wavelength of signal (m)

φmin :	minimum pfd at receiving place (dB(W/m2))

Lf :	feeder loss (dB)

Emin :	equivalent minimum field strength at receiving place (dB(V/m))

K :	conversion factor (dB)

Emed :	minimum median equivalent field strength, planning value (dB(V/m))

Pmmn :	allowance for man-made noise (dB) (median values of man-made noise power are shown in Recommendation ITU-R P.372-9)

Lh :	height loss (reception point at 1.5 m above ground level) (dB)

Lb :	building or vehicle entry loss (dB)

Cl :	location correction factor (dB)

t :	total standard deviation (dB)

m :	standard deviation macro-scale (m = 5.5 (dB))

b :	standard deviation building entry loss (dB)

µ :	distribution factor being 0.52 for 70%, 1.28 for 90%, 1.64 for 95% and 2.33 for 99%.

Annex 4

Other planning factors

# 1	Field strength distribution with location

It is to be expected that the distributions of field strength with location for digital television signals will not be the same as those applicable to analogue television signals. Recommendation ITU-R P.1546 includes the standard deviation for the analogue and for the digital case in Table 1, 2 and 3 for 100 MHz, 600 MHz and 2 000 MHz, respectively.

The results of propagation studies for digital systems are given in Fig. 2 for the VHF and UHF bands. The figure corresponds to a standard deviation of 5.5 dB. These results may also be used to derive propagation prediction curves for location percentages other than 50%. Refer to Recommendation ITU-R P.1546 for the location percentages other than 50% for analogue and digital systems, where the digital system bandwidth is greater than 1.5 MHz.

FIGURE 2

Ratio (dB) of the field strength for a given percentage of the receiving
locations to the field strength for 50% of the receiving locations

# 2	Reception using portable equipment inside buildings and vehicles

## 2.1	Height loss: Lh

For land paths, the curves in Recommendation ITU-R P.1546 give field-strength values for a receiving antenna height above ground equal to the representative height of ground cover around the receiving antenna location. Subject to a minimum height value of 10 m, examples of reference heights are 20 m for an urban area, 30 m for a dense urban area and 10 m for a suburban area. (For sea paths, the notional value is 10 m.)

If the receiving antenna height is different from the representative height, a correction is applied to the field strength taken from the curves of Recommendation ITU-R P.1546 according to a procedure given in the Recommendation.

## 2.2	Building entry loss: Lb

Losses due to penetration into a building depend significantly on the building material, angle of incidence and frequency. Consideration should also be given as to whether reception is in an interior room or in one located near an exterior wall. The building entry loss is defined as the difference (dB) between the mean field strength inside a building at a given height above ground level and the mean field strength outside the same building at the same height above ground level. Whilst no single comprehensive formula is available for computing building entry loss, useful statistical information based on measured losses in several types of building, at frequencies from about 500 MHz to 5 GHz, are given in Recommendation ITU-R P.679. Once inside the building, propagation loss due to walls and floors are dealt with in Recommendation ITU-R P.1238.

A large spread of building entry losses have been measured. Table 64 gives three classes of the relative possibilities to achieve indoor reception and the corresponding mean and standard deviation values of the building entry losses, for the same outdoor field strength, based on UHF measurements.

TABLE 64

Building entry loss variations in the UHF Bands IV/V

| Classification of the relative possibilities to achieve indoor reception | Mean building entry loss (dB) | Standard deviation (dB) |
| --- | --- | --- |
| High | 7 | 5 |
| Medium | 11 | 6 |
| Low | 15 | 7 |

Examples of buildings with different relative possibilities to achieve indoor reception:

High:

–	suburban residential building without metallized glass windows,

–	room with a window on the exterior wall in an apartment in an urban environment.

Medium:

–	exterior rooms in an urban environment with metallized glass windows,

–	inner rooms in an apartment in an urban environment.

Low:

–	inner rooms in office buildings.

If more precise values based on local measurements are available, these could be used as a basis for planning a specific service.

## 2.3	Vehicle entry loss: Lv

For reception with a hand-held device inside a vehicle, the vehicle body loss should be taken into account. A typical vehicle entry loss for the UHF Bands IV/V, based on cellular radio experience, is 6 dB.

# 3	Receiving antenna discrimination

Information concerning the directivity and polarization discrimination of domestic receiving antennas is given in Recommendation ITU-R BT.419.

# 4	Antennas for portable and mobile receivers

## 4.1	Antennas for portable reception

A spread in antenna gain has been measured for different types of antenna. The following antenna gain values are typical:

TABLE 65

Antenna gain (dBd) for portable reception

| Band | Gain (dBd) |
| --- | --- |
| VHF Band III | –2 |
| UHF Band IV | 0 |
| UHF Band V | 0 |

No polarization discrimination is expected.

## 4.2	Antennas for hand-held reception

The antenna in a small hand-held terminal has to be an integral part of the terminal construction and will therefore be small when compared to the wavelength. Current understanding of the design problem indicates that the worst-case antenna gain is in the lowest part of the UHF band. The antenna gain for three frequencies in the UHF band is given in Table 66. Nominal antenna gain between these frequencies can be obtained by linear interpolation.

TABLE 66

Antenna gain (dBd) for hand-held reception

| Frequency (MHz) | Gain (dBd) |
| --- | --- |
| 474 | –12 |
| 698 | –9 |
| 858 | –7 |

Generally, no polarization discrimination is expected from this type of portable reception antenna and the radiation pattern in the horizontal plane is omnidirectional.

## 4.3	Antennas for mobile reception

The practical standard antenna for vehicle reception is l/4 monopole, which uses the metallic roof as a ground plane. The antenna gain for conventional incident wave angles depends on the position of the antenna on the roof. For passive antenna systems the values in Table 67 can be expected.

TABLE 67

Antenna gain (dBd) for mobile reception

| Band | Gain (dBd) |
| --- | --- |
| VHF Band III | –5 |
| UHF Band IV | –2 |
| UHF Band V | –1 |

The polarization discrimination is theoretically about 4 to 10 dB depending on the roof position of the antenna.

Annex 5

Subjective comparison method (SCM) with a reference interferer
for assessment of protection ratios for analogue television systems

# 1	Introduction

Subjective methods for assessment of impairment grades involve extensive tests, are time consuming, require large numbers of observers and consider the full impairment grade range.

For assessing protection ratios only two fixed impairment types are necessary, approximately grade 3 for tropospheric and grade 4 for continuous, see Table 68.

This Annex gives a method of assessing protection ratios for wanted analogue TV systems based on the subjective comparison of the impairment of an interferer with that of a reference interferer. Usable and reliable results are produced with only a small number of observers and one still picture.

This subjective comparison method is appropriate for the evaluation of interference from any unwanted digital or analogue transmission system into a wanted analogue television channel. The application of a defined fixed reference interferer results in a reproducible set of figures with a low deviation (approximately  1 dB standard deviation). Only a small number of observers – three to five experts or non-experts – is necessary.

There are two reference interferers which may be used:

–	sine-wave interference

–	Gaussian-noise interferer.

Tests have shown that for unwanted digital television systems a noise reference interferer can improve the assessment decision by the observer. The use of noise reference interferer shows the same results as the defined sine-wave interferer. The disadvantage is that a more complicated test arrangement may be necessary. Further tests are necessary, especially by fixing the equivalent noise reference.

(For the time being the sine-wave reference interferer should be used until an agreement on a common test procedure and an agreement on a harmonized unified noise reference figure has been obtained.)

# 2	SCM for assessment of protection ratios using a sine-wave reference

## 2.1	General description

Figure 3 shows the test arrangement for the subjective comparison method with sine-wave interferer. The lower three blocks are the main signal path, the wanted video source, the television transmitter and the TV receiver under test. The reference video interferer is a simple sine-wave signal. The amplitude of the sine-wave generator is switchable between tropospheric interference and continuous interference. The unwanted RF interferer is added to the wanted signal path. The amplitude and frequency of the interferer are calculated from the RF reference interferer given in Recommendation ITU-R BT.655, Annex 1, § 2.3.

FIGURE 3

SCM for assessment of protection ratios

The intensity of the RF interferer can be changed with an attenuator controlled by the observer. The RF interferer is adjusted to produce the same impairment grade as the reference interferer by comparing the interfered pictures on the TV screen.

The RF protection ratio is the difference between the wanted and the unwanted signal levels at the receiver input. The test arrangement can be adjusted in such a way that the value in dB shown at the attenuation box gives, directly, the protection ratio.

## 2.2	Realization of the reference interferer

For 625-line systems the reference impairment levels are those which correspond to co-channel protection ratios of 30 dB and 40 dB with a frequency offset between the wanted and unwanted vision carriers close to two-thirds of the line frequency but adjusted for maximum impairment. The precise frequency difference is 10 416 Hz. These conditions approximate impairment grades 3 (slightly annoying) and 4 (perceptible, but not annoying) and apply to tropospheric (1% of time) and continuous interference (50% of time), respectively. The impairment grade of the given video baseband reference interferer is independent of the analogue television system and independent of the RF modulation parameters such as modulation polarity, residual carrier etc.

The RF reference interferer can be realized as a simple sine-wave signal at baseband frequency as shown in Fig. 3. The sine-wave reference interferer has a fixed frequency of 10 416 Hz for 625-line systems or 10 500 Hz for 525-line systems –60 Hz and 10 510 Hz for 525-line systems –59.94 Hz, an amplitude of either 60 mVp-p or 20 mVp-p referring to a black-to-white level of 700 mVp-p or a CVBS level of 1 Vp-p. These amplitudes correspond to the RF protection ratios of 30 dB and 40 dB respectively (2/3 line offset). The frequency stability of the sine-wave generator must be within  1 Hz.

## 2.3	Test conditions

Wanted video signal:	only an electronic test picture is required (e.g. FuBK, Philips or others).

Viewing conditions:		as given in Recommendation ITU-R BT.500.

Viewing distance:		five times the picture height.

Test receiver:			up to five different domestic sets, not older than five years, for co-channel measurements a professional receiver can be used.

Receiver input signal:	39 dBm (70 dB(V) at 75 ).

Observers:				five observers, experts or non-experts, are necessary. For initial tests less than five observers are possible. Each single test should be made with one observer only. Observers should be introduced to the method of assessment.

## 2.4	Presentation of the results

The results should be presented together with the following information:

–	mean and standard deviation of the statistical distribution of the protection ratio values;

–	test configuration, test picture, type of picture source;

–	number of observers;

–	reference interferer type;

–	the spectrum of the unwanted signal (RF interferer), including the out-of-channel range;

–	the used RF level for the wanted signal at the receiver input (for domestic receivers an input voltage of 39 dBm (70 dB(V) at 75  should be used);

–	when domestic sets are used, type, display size and year of production.

# 3	Table of important parameters

TABLE 68

Basic terms and relations for the SCM

| Quality impairment | Grade 3 | Grade 4 |
| --- | --- | --- |
| Interference type | Tropospheric | Continuous |
| Time allowance | 1% to 5% of time | 50% of time |
| Subjective impairment | Slightly annoying | Perceptible, but not annoying |
| Reference interferer (mVp-p) | 60 | 20 |
| RF protection ratio (dB) | 30 | 40 |

Annex 6

Test methods for protection ratio measurements for 
wanted digital terrestrial signals

# 1	Background

Initial studies of the protection ratios for the DVB-T system were based on a target BER of 2  10–4 measured between the inner and outer codes, before Reed-Solomon decoding. For the case of a noise-like interferer, this has been taken to correspond to a quasi-error-free (QEF) picture quality with the BER  1  10–11 at the input of the MPEG-2 demultiplexer.

# 2	Subjective failure point (SFP) method for protection ratio measurements

For domestic receivers it may not be possible to measure the BER and therefore a new method called the SFP method has been proposed for protection ratio measurements in a unified manner. The quality criterion for protection ratio measurements is to find a limit for a just error-free picture at the TV screen. The RF protection ratio for the wanted DVB-T signal is a value of wanted-to-unwanted signal ratio at the receiver input, determined by the SFP method, and rounded to the next higher integer value.

The SFP method corresponds to the picture quality where no more than one error is visible in the picture for an average observation time of 20 s. The adjustment of the wanted and unwanted signal levels for the SFP method is to be carried out in small steps, usually in steps of 0.1 dB. For a “noise-like” interferer the difference in a value of wanted-to-unwanted signal ratio between the QEF method with a BER of 2  10–4 and the SFP method is less than 1 dB. All protection ratio values for wanted digital TV signals are measured with a receiver input power of 60 dBm.

It is proposed that the SFP method should be adopted for assessment of all DTTB systems. (For the digital system ISDB-T this method will be studied in Japan.)

Annex 7

Tropospheric and continuous interference

When using the protection ratios in planning, it is necessary to determine whether, in particular circumstances, the interference should be considered as tropospheric or continuous. This can be done by comparing the nuisance fields for the two conditions, the nuisance field being defined as the field strength of the interfering transmitter (at its pertinent e.r.p.) enlarged by the relevant protection ratio.

Thus, the nuisance field for continuous interference:

EC    E(50, 50)    P    AC

and the nuisance field for tropospheric interference:

ET    E(50, t)    P    AT

where:

E(50, t) :	field strength (dB(V/m)) of the interfering transmitter, normalized to 1 kW, and exceeded during t % of the time

P :	e.r.p. (dB(1 kW)) of the interfering transmitter

A :	protection ratio (dB)

C and T :	continuous and tropospheric interference, respectively.

The protection ratio for continuous interference is applicable when the resulting nuisance field is stronger than that resulting from tropospheric interference, that is, when EC  ET.

This means that AC should be used in all cases when:

E(50, 50)    AC    E(50, t)    AT

*	The material and terms used in this Recommendation in no way modify/replace/supersede those used in the GE06 Agreement.

ISDB-T is used in Japan, Brazil and other countries not part of the GE06 Agreement.

ISDB-T is used in Japan, Brazil and other countries not part of the GE06 Agreement.