RECOMMENDATION  ITU-R  F.1099

RADIO-FREQUENCY  CHANNEL  ARRANGEMENTS  FOR  HIGH-CAPACITY  DIGITAL
RADIO-RELAY  SYSTEMS  IN  THE  5  GHz (4 400-5 000 MHz)  BAND

(Question ITU-R 136/9)

(1994)

Rec. ITU-R F.1099

The ITU Radiocommunication Assembly,

considering

a)	that high-capacity digital radio-relay systems of more than the order of 90 Mbit/s conveying plesiochronous or synchronous digital hierarchy signals are required in the 5 GHz radio-frequency bands;

b)	that the centre gaps of the individual channel arrangements and the guard spacing at the edges of the band can be chosen by non-occupancy of a suitable number of RF-channel positions in a homogeneous basic pattern;

c)	that the uniform basic pattern spacing should not be unjustifiably small nor so large as to jeopardize efficient use of the available spectrum;

d)	that the absolute frequencies of the basic pattern should be defined by a single reference frequency,

recommends

1.	that the preferred radio-frequency channel arrangement for high-capacity digital radio-relay systems of the order of 90 Mbit/s or more conveying plesiochronous or synchronous digital hierarchy signals (see Note 1), operating in the 5 GHz band, should be selected from a homogeneous pattern with the following characteristics:

Centre frequencies fp of the radio-frequency channels within the basic pattern

fp  =  5 000  –  10 p	MHz

p: integral 1, 2, 3 ... (see Note 2);

2.	that all the go channels should be in one half of the band and all the return channels should be in the other half of the band;

3.	that the channel spacing XS, the centre gap YS, the guard spaces Z1S and Z2S at the edges of the band and the antenna polarization should be agreed between the administrations concerned;

4.	that the alternated or co-channel arrangement plan should be used, examples of which are shown in Fig. 1;

5.	that radio-frequency channel arrangements derived from recommends 1 for the 5 GHz band and given in Annexes 1 and 2 should be regarded as part of this Recommendation.

Note 1 – Actual gross bit rates including overhead may be as much as 5% or higher than net transmission rates.

Note 2 – Due regard should be taken of the fact that in some countries where additional radio-frequency channels interleaved between those of main patterns are required, the values of the centre frequencies of these radio-frequency channels should be given by the following equation (see Annexes 1 and 2).

fp  =  4 995  –  10 p	MHz

FIGURE 1...[D01] = 3 CM

ANNEX  1

Radio-frequency channel arrangement for the band 4 400-5 000 MHz
with channel spacings of 40 or 60 MHz

# 1.	40 MHz radio-frequency channel arrangement

1.1	The following radio-frequency channel arrangement provides seven go and seven return channels with a transmission capacity up to 2 ´ 155 Mbit/s for radio systems with a suitable higher level modulation and spectrum efficiency up to 7.75 bit/s/Hz. The radio-frequency channel arrangement should be as shown in Fig. 2 and should be derived as follows:

Let	f0	be the frequency (MHz) of the centre of the band of frequencies occupied, f0 = 4 700,

fn	be the centre frequency (MHz) of one radio-frequency channel in the lower half of the band,

be the centre frequency (MHz) of one radio-frequency channel in the upper half of the band,

then the frequencies (MHz) of individual channels are expressed by the following relationships:

lower half of the band:	fn = f0 – 310 + 40 n

upper half of the band:	 = f0 – 10 + 40 n

where:

n = 1, 2, 3, 4, 5, 6 or 7.

1.2	All the go channels should be in one half of the band and all the return channels should be in the other half of the band.

1.3	Different polarizations should be used in an alternated pattern for radio-frequency channels in the same half of the band or where it is possible, band re-use in the co-channel mode may be utilized.

FIGURE 2...[D02] = 3 CM

# 2.	60 MHz radio-frequency channel arrangements

This section describes examples of radio-frequency channel arrangements derived from recommends 1 and Note 2 of the main text of this Recommendation. The systems characterized by parameters in Table 1 achieve high spectrum utilization efficiency of the order of 5 bit/s/Hz or 10 bit/s/Hz employing 16-QAM or 256-QAM, respectively.

The systems representing examples 1a, 1b and 2 are designed to transmit SDH signals, using a multi-carrier transmission method. In case of the multi-carrier systems “Number of channels” means the number of transmitters (or receivers), each of which accommodates three or six carriers within 60 MHz. Also, XS, YS and ZS are stated for the centre frequency of the transmitter (or receiver) (see Figs. 3 and 4).

TABLE  1

|  | Example 1a | Example 1b |  | Example 2(1) |
| --- | --- | --- | --- | --- |
| System capacity | (Synchronous) |  | (Synchronous) |  |
|  | 156 Mbit/s(2) | 311 Mbit/s(2) |  | 311 Mbit/s(2) |
| Modulation | 16-QAM | 256-QAM |  | 256-QAM |
| Polarization | Co-channel |  | Co-channel |  |
| Transmission method | 3-carrier per channel |  | 6-carrier per channel |  |
| Centre frequency of the carriers fn (MHz) | fn = 5 000 – 10 m |  | fn = 4 995 – 10 m |  |
|  | m = 2, 4, 6... 28 (upper) |  | m = 1, 2, 3... 27, 28 (upper) |  |
|  | m = 32, 34, 36... 58 (lower) |  | m = 31, 32... 57, 58 (lower) |  |
| Number of channels | 10(2) |  | 10(2) |  |
| (1) Example 2 is applicable to hops under very severe propagation conditions. (2) The capacity of the innermost radio-frequency channels is limited to two-thirds of the full capacity. |  |  |  |  |

FIGURE 3...[D03] = 3 CM

FIGURE 4...[D04] = 3 CM

ANNEX  2

Radio-frequency chanelling arrangements for the band 4 540-4 900 MHz
with channel spacings of 20 or 40 MHz

This Annex describes a radio-frequency channelling plan for digital radio systems in the band 4 540-4 900 MHz. The arrangement provides for up to four go and four return channels each accommodating either 4 ´ 45 Mbit/s, 6 ´ 45 Mbit/s or the SDH bit rate at 2 ´ 155 Mbit/s. A 512-QAM modulation scheme allows for system operation at STM-1 or 2 ´ STM-1. An alternative arrangement provides up to eight go and eight return channels each accommodating either 2 ´ 45 Mbit/s, 3 ´ 45 Mbit/s or the synchronous digital hierarchy bit rate at 155 Mbit/s.

1.	The radio-frequency channel arrangement is shown in Fig. 5 and is derived as follows:

FIGURE 5...[D05] = 3 CM

Let f0 be the frequency at the centre of the band:

f0 = 4 720 MHz,

fn :		centre frequency of one radio-frequency channel in the lower half of the band (MHz),

:		centre frequency of one radio-frequency channel in the upper half of the band (MHz),

then the centre frequencies of the individual channels are expressed by the following relationships:

lower half of the band:	fn = f0 – 195 + 40 n

upper half of the band:	 = f0 – 5 + 40 n

where:

n = 1, 2, 3, 4.

2.	An alternative arrangement is shown in Fig. 6 and the assignments are expressed as follows:

FIGURE 6...[D06] = 3 CM

The centre frequencies of the individual channels are expressed by the following relationships:

lower half of the band:	fn = f0 – 185 + 20 n

upper half of the band:	 = f0 + 5 + 20 n

where:

f0 = 4 720 MHz

n = 1, 2, 3, 4, 5, 6, 7, 8.

3.	All the go channels should be in one half of the band and all the return channels should be in the other half of the band.

_________________