Recommendation ITU-R F.1404-1 (05/2002)

Minimum propagation attenuation due to atmospheric gases for use in frequency sharing studies between systems in the fixed service and systems in the broadcasting-satellite, mobile-satellite and space science services

F Series Fixed service

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2009

 ITU 2009

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  F.1404-1*, **

Minimum propagation attenuation due to atmospheric gases for use
in frequency sharing studies between systems in the fixed
service and systems in the broadcasting-satellite,
mobile-satellite and space science services

(Questions ITU-R 111/5 and ITU-R 113/5)

(1999-2002)

Scope

This Recommendation provides the method for estimating the minimum slant-path attenuation due to atmospheric gasses, including water vapour, for use in frequency sharing studies between systems in the fixed service and systems in the broadcasting-satellite, mobile-satellite and space science services. The method is developed on a band-by-band basis to provide the minimum attenuation for the driest month and at the frequency that gives the lowest attenuation in each band.

The ITU Radiocommunication Assembly,

considering

a)	that slant path attenuation between a terrestrial station and a space station (geostationary (GSO) or non-geostationary (non-GSO)) resulting from absorption due to atmospheric gases including water vapour is an important factor in frequency sharing studies between systems in the fixed service (FS) and systems in the broadcasting-satellite service (BSS), the mobile-satellite service (MSS) and space science service;

b)	that slant path attenuation depends on the distribution along the path of meteorological parameters such as temperature, pressure and humidity, and thus varies with the geographic location of the site, the month of the year, the height of an FS station above sea level and the elevation angle of the slant path;

c)	that such slant path attenuation can be estimated by the method described in Annex 1 to Recommendation ITU-R P.676, but that it is desirable to provide a simple procedure to estimate the attenuation;

d)	that for the purpose of frequency sharing studies, it is necessary to define the parameters in the driest month at sea level for each climate area, based on Recommendation ITU-R P.835;

e)	that slant path attenuation is a complicated function of the frequency and that for each frequency band a representative frequency giving the lowest attenuation should be chosen,

recommends

1	that for frequency sharing studies between FS systems and BSS, MSS and space science service systems in each frequency band, slant path attenuation resulting from absorption due to atmospheric gases including water vapour should be estimated at a representative frequency which gives the lowest attenuation in that band (see Note 1);

2	that the method of Annex 1 should be utilized for the estimation of slant path attenuation due to atmospheric absorption (see Notes 2, 3 and 4).

NOTE 1 – The information in this Recommendation is solely for the purpose of frequency sharing studies, because it deals with the slant path attenuation in the driest month.

NOTE 2 – When more details are required, these may be obtained from Recommendation ITU-R P.676.

NOTE 3 – The information in this Recommendation is based on Recommendation ITU-R P.676-3 (Geneva, 1997) and Recommendation ITU-R P.835-2 (Geneva, 1997).

NOTE 4 – Recommendation ITU-R SF.1395 presents approximate formulae of minimum slant path attenuation due to atmospheric absorption for the frequency bands shared by the FS and the FSS.

ANNEX  1

Estimation of slant path propagation attenuation due to atmospheric gases
for use in frequency sharing studies between FS systems and
BSS, MSS and space science service systems

# 1	Introduction

Slant path attenuation between a terrestrial station and a space station (GSO or non-GSO) resulting from absorption due to atmospheric gases including water vapour is an important factor in frequency sharing studies between FS systems and BSS, MSS and space science service systems. The slant path attenuation depends on the distribution along the path of meteorological parameters such as temperature, pressure and humidity, and thus varies with the geographic location of the site, the month of the year, the height of an FS station above sea level and the elevation angle of the slant path and the operating frequency. The procedure for calculating the slant path attenuation is the line-by-line procedure given in Annex 1 to Recommendation ITU-R P.676.

The detailed calculations of atmospheric attenuation may utilize local information of average water vapour content in the driest month and of other meteorological parameters along with the atmospheric models of Recommendation ITU-R P.835. Where this information is not available, the following results provide a simple procedure for estimating atmospheric attenuation.

The formulae given in § 2 consider each of the frequency bands which are allocated to the FS and one of the BSS, MSS or space science services on a shared basis and are presented for five representative geographical areas of the world (northern and southern hemispheres).

# 2	Estimation of slant path attenuation

For the purpose of this simplified estimation, an FS station is identified as being within one of three climate areas depending only on the latitude (absolute value) of the station:

–	low-latitudes within 22.5° of the Equator;

–	mid-latitudes greater than 22.5°, but less than 45° from the Equator;

–	high-latitudes of 45° or more from the Equator.

Table 1 shows the climate parameters for each of these areas. Note that the sea-level water vapour density for the low-latitude area is lower than that prescribed in Recommendation ITU-R P.835 corresponding to the dry season. The attenuation values for these areas have been determined as a function of the elevation angle of the actual transmission path from the FS station to the position of a space station (GSO or non-GSO). The numerical formulae for atmospheric attenuation which approximate the theoretical values are given in the following sections, where:

AL (h, ), AM (h, ) and AH (h, ):	total atmospheric absorption loss (dB) for the low-latitude, mid-latitude and high-latitude areas, respectively;

h and :				FS antenna altitude above sea level (km) and elevation angle (degrees), respectively.

TABLE  1

Parameters at sea level for the climate areas

| Climate area | Temperature (K) | Atmospheric pressure (hPa) | Water vapour density (g/m3) |
| --- | --- | --- | --- |
| Low-latitude | 300.4 | 1 012.0 | 10.0 |
| Mid-latitude | 272.7 | 1 018.9 | 3.5 |
| High-latitude | 257.4 | 1 010.8 | 1.23 |

The method in Annex 1 to Recommendation ITU-R P.676 was used for integration. The height profiles of temperature, pressure and water vapour density as defined in Recommendation ITU-R P.835 were used in calculating the loss. The approximation was carried out for 0  h  3 km and 0    90.

The actual elevation angle may be determined from the elevation angle developed under free space propagation conditions using the method in Recommendation ITU-R F.1333. For actual elevation angles below 0, the attenuation for 0 should be used.

NOTE 1 – In some situations, it may become necessary to estimate the attenuation at a specific frequency based on the following formulae. For example, if it is necessary to find the attenuation in the low-latitude area at 36.5 GHz, it is possible to estimate this attenuation as an interpolation of the attenuation at 36.0 GHz (see equation (9a)) and that at 37.0 GHz (see equation (10a)). However, for such interpolation to be accurate, the two adjacent representative frequencies should be reasonably close to each other. In addition, special caution is required near 22.24 GHz (water vapour resonance frequency) where the linear interpolation may not apply.

## 2.1	Frequency band 11.7-12.75 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 11.7 GHz.

AL (h, )		3.84 / [1  0.8598   h (0.2815  0.3031 )  0.1148 h2]	(1a)

AM (h, )		3.23 / [1  0.7585   h (0.4154  0.2232 )]	(1b)

AH (h, )		3.12 / [1  0.7487   h (0.3792  0.2102 )]	(1c)

## 2.2	Frequency band 18.6-18.8 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 18.6 GHz.

AL (h, )		15.16 / [1  0.9258   0.03625 2  h (0.2981  0.4352 ) 
 h2 (0.2429  0.1330 )]		(2a)

AM (h, )		7.98 / [1  0.9103   h (0.2862  0.4112 )  0.1469 h2]	(2b)

AH (h, )		5.67 / [1  0.8172   h (0.2017  0.3017 )  0.1057 h2]	(2c)

## 2.3	Frequency band 21.2-21.4 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 21.2 GHz.

AL (h, )		38.08 / [1  0.8485   0.06485 2 – 0.002121 3  0.1669  10– 4 4
 h (0.2934  0.3816 )  h2 (0.09441  0.1701 )  0.04082 h3]	(3a)

AM (h, )		16.70 / [1  0.8126   0.02719 2  h (0.2395  0.2772 )
 h2 (0.1180  0.08558 )]		(3b)

AH (h, )			9.66 / [1  0.6721   0.04348 2  h (0.07322  0.3655 )  0.1177 h2]	(3c)

## 2.4	Frequency band 21.4-22.0 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 21.4 GHz.

AL (h, )		40.39 / [1  0.8413   0.06418 2 – 0.002095 3  0.1646  10– 4 4
 h (0.2871  0.3732 )  h2 (0.09311  0.1638 )  0.03859 h3]	(4a)

AM (h, )		17.59 / [1  0.8066   0.02682 2  h (0.2354  0.2699 )
 h2 (0.1135  0.08342 )]		(4b)

AH (h, )		10.08 / [1  0.6205   0.04369 2  h (0.06793  0.3605 )  0.1155 h2]	(4c)

## 2.5	Frequency band 22.21-22.5 GHz

In this frequency band, the attenuation is at its maximum at 22.24 GHz due to water vapour resonance and lowest at 22.5 GHz and, therefore, the following formulae give the attenuation at 22.5 GHz.

AL (h, )		47.88 / [1  0.78405   0.10659 2 – 0.0091566 3
 0.30002  10–3 4 – 0.40272  10–5 5  0.18706  10–7 6
 h (0.29782  0.30275 )  h2 (0.066824  0.17983 )  0.038747 h3]	(5a)

AM (h, )	=	20.36 / [1  0.7223   0.06031 2 – 0.001980 3  0.1572  10– 4 4
 h (0.2053  0.2374 )  h2 (0.1101  0.08933 )]	(5b)

AH (h, )		1.55 / [1  0.6073   0.04379 2  h (0.05750  0.3490 )  0.1102 h2]	(5c)

## 2.6	Frequency band 23.6-24.0 GHz

In this frequency band, the attenuation is smaller at higher frequencies and, therefore, the following formulae give the attenuation at 24.0 GHz.

AL (h, )		40.20 / [1  0.8774   0.06742 2 – 0.002221 3  0.1759  10– 4 4
 h (0.3193  0.4177 )  h2 (0.1014  0.1945 )  0.05008 h3]	(6a)

AM (h, )		17.88 / [1  0.8377   0.02861 2  h (0.2587  0.3070 )
 h2 (0.1362  0.09479 )]		(6b)

AH (h, )		10.51 / [1  0.6504   0.04326 2  h (0.08915  0.3870 )  0.1285 h2]	(6c)

## 2.7	Frequency band 25.25-27.5 GHz

In this frequency band, the attenuation is smaller at higher frequencies and, therefore, the following formulae give the attenuation at 27.5 GHz.

AL (h, )		22.73 / [1  0.9463   0.03455 2  h (0.3232  0.4519 )
 h2 (0.2486  0.1317 )]		(7a)

AM (h, )			11.96 / [1  0.8121   0.03055 2  h (0.2619  0.4728 )  0.1490 h2]	(7b)

AH (h, )		8.77 / [1  0.8259   h (0.2163  0.3037 )  0.1067 h2]	(7c)

## 2.8	Frequency band 31.0-31.3 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 31.0 GHz.

AL (h, )		19.54 / [1  0.9323   0.02553 2  h (0.3416  0.4413 )
 h2 (0.1980  0.08016 )]		(8a)

AM (h, )		11.76 / [1  0.81370  0.02033 2  h (0.2740  0.3935 )  0.1203 h2]	(8b)

AH (h, )		9.52 / [1  0.8160   h (0.2378  0.2722 )  0.08949 h2]	(8c)

## 2.9	Frequency band 31.8-33.0 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 31.8 GHz.

AL (h, )		19.55 / [1  0.9263   0.02442 2  h (0.3399  0.4324 )
 h2 (0.1898  0.07463 )]		(9a)

AM (h, )			12.04 / [1  0.8112   0.01934 2  h (0.2740  0.3825 )  0.1155 h2]	(9b)

AH (h, )		9.90 / [1  0.8140   h (0.2401  0.2679 )  0.08673 h2]	(9c)

## 2.10	Frequency band 36.0-37.0 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give attenuation at 36.0 GHz.

AL (h, )		21.60 / [1  0.8102   0.05726 2 – 0.001887 3  0.1488  10– 4 4
 h (0.2731  0.5166 )  0.1884 h2]	(10a)

AM (h, )		15.00 / [1  0.8197    0.01342 2  h (0.3078  0.2651 )
 h2 (0.07561  0.03399 )]		(10b)

AH (h, )		12.80 / [1  0.7376   0.01588 2  h (0.2185  0.2806 )  0.07660 h2]	(10c)

## 2.11	Frequency band 37.0-38.0 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 37.0 GHz.

AL (h, )		22.63 / [1  0.8064   0.05519 2 – 0.001808 3  0.1416  10– 4 4
 h (0.2740  0.4986 )  0.1789 h2]	(11a)

AM (h, )		16.03 / [1  0.8146   0.01315 2  h (0.3044  0.2598 )
 h2 (0.07308  0.03276 )]		(11b)

AH (h, )			13.85 / [1  0.7369   0.01556 2  h (0.2197  0.2771 )  0.07495 h2]	(11c)

## 2.12	Frequency band 39.5-40.0 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 39.5 GHz.

AL (h, )		26.03 / [1  0.7941   0.05051 2 – 0.001631 3  0.1259  10– 4 4
 h (0.2739  0.4541 )  0.1562 h2]	(12a)

AM (h, )		19.39 / [1  0.8019   0.01254 2  h (0.2957  0.2470 )
 h2 (0.06718  0.03002 )]		(12b)

AH (h, )		17.46 / [1  0.7615   0.01187 2  h (0.2619  0.2041 )
 h2 (0.05213  0.02735 )]		(12c)

## 2.13	Frequency band 40.0-40.5 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 40.0 GHz.

AL (h, )		26.87 / [1  0.7912   0.04963 2 – 0.001599 3  0.1230  10– 4 4
 h (0.2735  0.4451 )  0.1517 h2]	(13a)

AM (h, )		20.23 / [1  0.7993   0.01243 2  h (0.2939  0.2444 )
 h2 (0.06605  0.02951 )]		(13b)

AH (h, )		18.33 / [1  0.7608   0.01179 2  h (0.2620  0.2033 )
  h2 (0.05148  0.02706 )]		(13c)

## 2.14	Frequency band 40.5-42.5 GHz

In this frequency band, the attenuation is larger at higher frequencies and, therefore, the following formulae give the attenuation at 40.5 GHz.

AL (h, )		27.78 / [1  0.7880   0.04877 2 – 0.001566 3  0.1202  10– 4 4
 h (0.2729  0.4361 )  0.1473 h2]	(14a)

AM (h, )		0.76 / [1  0.6980   0.04731 2 – 0.001508 3  0.1157  10– 4 4
  h (0.2497  0.3257 )  0.07995 h2]	(14b)

AH (h, )		18.92 / [1  0.6577   0.04678 2 – 0.001484 3  0.1139  10– 4 4
  h (0.2200  0.2811 )  0.06507 h2]	(14c)

## 2.15	Frequency band 55.78-59.0 GHz

In this frequency band, the attenuation is generally larger at higher frequencies and, therefore, the following formulae give the attenuation at 55.78 GHz.

AL (h, )		2 217.1 / [1  0.40513   0.011553 2 – 0.00046820 3  0.84110  10–5 4
– 0.77381  10–7 5  0.27719  10–9 6  h (0.089500  0.042363 
 00025474 2 – 0.41230  10–5   0.0093864 h2]	(15a)

AM (h, )			2 338.1 / [1  0.43368   0.011352 2 – 0.00045777 3  0.81145  10– 5 4
– 0.74110  10–7 5  0.26432  10–9 6  h (0.10951  0.050614 
 0.00028913 2 – 0.47829  10–5 3)  0.0099073 h2]	(15b)

AH (h, )		2 414.2 / [1  0.44582   0.011815 2 – 0.00048462 3  0.87303  10–5 4
– 0.80607  10–7 5  0.28980  10–9 6  h (0.12263  0.054149 
 0.00032477 2 – 0.52680  10–5 3  0.0087766 h2]	(15c)

In the band 55.78-56.26 GHz, in order to protect stations in the Earth exploration-satellite service (passive), the maximum power density delivered by a transmitter to the antenna of a FS station is limited to 26 dB(W/MHz) (see No. 5.557A of the Radio Regulations).

## 2.16	Frequency band 64.0-66.0 GHz

In this frequency band, the attenuation is smaller at higher frequencies and, therefore, the following formulae give the attenuation at 66.0 GHz.

AL (h, )		528.4 / [1  0.568865   0.0640672 2 – 0.00696532 3
 0.385420  10–3 4 – 0.114133  10– 4 5  0.181220  10–6 6
– 0.145280  10–8 7  0.461010  10–11 8
 h (0.178140  0.117782   0.00785552 2 – 0.228606  10–3 3
 0.159694  10–5 4  h2 0.0367537  0.0186594 )]	(16a)

AM (h, )		522.9 / [1  0.596648   0.0698675 2 – 0.00806908 3
 0.466138  10–3 4 – 0.141814  10– 4 5  0.229255  10–6 6
– 0.186157  10–8 7  0.596475  10–11 8
 h (0.205676  0.125103   0.0107935 2 – 0.326445  10–3 3
+ 0.235065  10–5  4)  h2 (0.0399720  0.0251223 )]	(16b)

AH (h, )		531.9 / [1  0.616560   0.0701934 2 – 0.00821842 3
 0.476119  10–3 4 – 0.143928  10– 4 5  0.230683  10–6 6
– 0.185825  10–8 7  0.591348  10–11 8
 h (0.224143  0.119089   0.0133543 2 – 0.416213  10–3 3
 0.308010  10–5  4)  h2 (0.0388456  0.0290534 )]	(16c)

*	This Recommendation should be brought to the attention of Radiocommunication Study Groups 3 (Working Party (WP) 3J), 6 (WP 6S), 7 (WP 7D) and 8 (WP 8D).

*	*	Radiocommunication Study Group 5 made editorial amendments to this Recommendation in 2009 in accordance with Resolution ITU-R 1.