RECOMMENDATION  ITU-R  F.275-3*

Pre-emphasis characteristic for frequency modulation radio-relay systems
for telephony using frequency-division multiplex

(1959-1966-1970-1982)

The ITU Radiocommunication Assembly,

considering

a)	that the pre-emphasis characteristic should preferably be such that the effective (r.m.s.) deviation due to the frequency-division multiplex telephony signal is the same with and without pre-emphasis (Recommendation ITU-R F.404);

b)	that, in a frequency-modulation system for frequency-division multiplex telephony, the thermal noise is highest in the top channel and decreases with decreasing baseband frequency;

c)	that, in a phase-modulation system, or in a frequency-modulation system with pre-emphasis of 6 dB per octave, the thermal noise is constant over the whole baseband;

d)	that the thermal noise in the highest channel of a phase-modulation system is approximately 4.8 dB better than the corresponding channel of a frequency-modulation system, assuming that the two types of system are adjusted to have the same total frequency deviation;

e)	that the reduction in frequency deviation with decreasing baseband frequency in a phase-modulation system makes such a system more sensitive to low frequency interference and to the effects of non-linearity in the system;

f)	that the loss of advantage in the top channel is quite small and the effects due to non-linearity are not excessive if the range of pre-emphasis is restricted to about 8 dB;

g)	that agreement on the pre-emphasis characteristic is desirable to facilitate international connection at radio frequencies or intermediate frequencies;

h)	that the pre-emphasis network may be inserted at different places in various types of equipment,

recommends

1	that, where pre-emphasis is used in radio-relay systems for frequency-division multiplex telephony, the same normalized attenuation-frequency characteristic should be used for systems with capacities up to and including 2 700 channels;

2	that the preferred pre-emphasis characteristic is given by the expression:

Relative deviation produced by the test tone (dB)   = 5 – 10 log10 	(1)

where fr (the resonant frequency of the network)  1.25 fmax, fmax is the highest telephone channel baseband frequency of the system, and f is the baseband frequency.

The variation of deviation with frequency is shown in Fig. 1. Table 1 shows fmax and fr for the frequency-division multiplex systems which are the subject of Recommendation ITU-R F.380 and which are mentioned in Recommendation ITU-R F.404;

TABLE  1

Characteristics frequencies for pre-emphasis and de-emphasis networks for the
frequency-division multiplex systems which are the subject of Recommendation ITU-R F.380 and which are mentioned in Recommendation ITU-R F.404

| Maximum number of telephone traffic channels (1) | fmax (2) (kHz) | fr (3) (kHz) | fc (4) (kHz) |
| --- | --- | --- | --- |
| 24 60 120 300 600 960 1 260 1 800 2 700 | 108 300 552 1 300 2 660 4 188 5 636 8 204 12 388 | 135 375 690 1 625 3 325 5 235 7 045 10 255 15 485 | 66.226 183.96 338.49 797.16 1 631.1 2 568.1 3 456.0 5 030.7 7 596.3 |
| (1) This figure is the nominal maximum traffic capacity of the system and applies also when only a smaller number of telephone channels are in service. (2) Nominal maximum frequency of the band occupied by telephone channels. (3) Nominal resonant frequency of the pre-emphasis or de-emphasis network. (4) Cross-over frequency at which the deviations with pre-emphasis and without pre-emphasis are equal. |  |  |  |

3	that the tolerance on the frequency response of the pre-emphasis characteristics, and also on the de-emphasis characteristics should be such that, within the nominal upper and lower limits of the baseband, the departure of the characteristic of a practical network from the theoretical characteristic should be confined within a variation of  (0.1  0.05 f / fmax) dB, f being the baseband frequency, and fmax the nominal maximum frequency of the baseband. This corresponds to component tolerances of about  1% for resistors and about  0.5% for capacitors and inductors. Further, the magnitude of the departure should exhibit no rapid variations within this frequency range.

NOTE 1 – It is recognized that it may be desirable to achieve the pre-emphasis characteristic by inserting a network at different places in various types of equipment. An example of a pre-emphasis and de-emphasis network, to work between a constant-voltage source and an open-circuit load, is shown in Figs. 2(a) and 2(b), respectively, and to work between matched resistive input and output impedances is shown in Figs. 3(a) and 3(b), respectively.

NOTE 2 – In the expression for the relative deviation as indicated in § 2, it should be noted that the frequency at which the deviation with pre-emphasis corresponds to that without pre-emphasis (Recommendation ITU-R F.404) is 0.61320 fmax. It may be convenient to adopt this frequency for testing the loss between baseband terminal points of systems when these are not in service.

NOTE 3 – It is recognized that it may sometimes be desirable to use a different pre-emphasis characteristic by agreement between the administrations concerned.

*	Radiocommunication Study Group 9 made editorial amendments to this Recommendation in 2001 in accordance with Resolution ITU-R 44.