RECOMMENDATION  ITU-R  F.634-3

ERROR  PERFORMANCE  OBJECTIVES  FOR  REAL  DIGITAL  RADIO-RELAY
LINKS  FORMING  PART  OF  A  HIGH-GRADE  CIRCUIT  WITHIN  AN  
INTEGRATED  SERVICES  DIGITAL  NETWORK

(Question ITU-R 134/9)

(1986-1990-1991-1994)

Rec. ITU-R F.634-3

The ITU Radiocommunication Assembly,

considering

a)	that the error performance objectives of a high-grade 2 500 km hypothetical reference digital path for digital radio-relay systems, operating above the second hierarchical level, are given in Recommendation ITU-R F.594, which applies to 64 kbit/s;

b)	that network performance objectives for digital sections are given in ITU-T Recommendation G.921;

c)	that real paths which form part of the high-grade portion of an integrated services digital network (ISDN) sometimes differ in composition from the hypothetical reference digital path (see Recommendation ITU-R F.556, Fig. 1), and share radio-relay links shorter than 2 500 km with other digital links;

d)	that conformity with digital radio performance Recommendations cannot be easily established by direct measurements on real systems due to the seasonal and annual variations in propagation conditions, and that practical advice on how to apply performance Recommendations is therefore necessary;

e)	that, therefore, it is necessary to give objectives for allowable bit error ratios as a guide in the design and planning of real radio-relay links forming part of the high-grade portion of an integrated services digital network,

recommends

1.	that when establishing real digital radio links intended to form part of a high-grade circuit within an ISDN, the following error performance criteria should be respected for a link with length, L, of between 280 km and 2 500 km (see Notes 1, 2, 3 and 10):

1.1	BER > 1 ´ 10–3 for no more than (L/2 500) ´ 0.054% of any month; integration time 1 s (see Note 4);

1.2	BER > 1 ´ 10–6 for no more than (L/2 500) ´ 0.4% of any month; integration time 1 min (see Notes 4, 7 and 8);

1.3	errored seconds for no more than (L/2 500) ´ 0.32% of any month (see Notes 4, 5 and 9);

1.4	residual bit error ratio:

mmmmmm(see Note 6)

2.	that the BER performance criteria are to be respected at the system bit rate. The errored second criterion should, for the present, be respected at the 64 kbit/s level (see Notes 5 and 8);

3.	that Annex 1 should be used for guidance on the factors which need to be taken into account in determining performance requirements for real digital radio-relay links forming a part of a high grade circuit within an ISDN.

Note 1 – International links are sometimes composed of radio-relay systems with a capacity equal to or lower than the second hierarchical level. Real radio-relay links discussed here can include these small capacity systems.

Note 2 – Performance objectives for circuits shorter than 280 km are still under study.

Note 3 – For the error performance objectives for circuits longer than 2 500 km see §1.1 of Annex 1.

Note 4 – The term “any month”, as used in this Recommendation, is defined in Recommendation ITU-R PN.581. Where measurements are used to ensure compliance with this Recommendation, then the propagation conditions also need to be assessed and related to propagation data representative of “any month” conditions.

Note 5 – The relationship between the errored seconds of a 64 kbit/s channel and the corresponding parameters which may be measured directly at the bit rate of the radio-relay system is still under study. For the time being, the errored seconds should be measured only at the 64 kbit/s interface. However, it should be noted that if the objectives given in § 1.1, 1.2 and 1.4 are satisfied, then the objective given in § 1.3 is usually satisfied, taking into account the typical cumulative error ratio distribution for high-grade systems.

Note 6 – The provisional method of measurement of the residual bit error ratio involves taking the BER measurements over a period of one month using a 15 min integration time, discarding the 50% of 15 min intervals which contain the worst BER measurements, and taking the worst of the remaining measurements. The RBER limits and method of measurement are under study and require verification. Annex 1 discusses RBER and refers to other methods of its measurement.

Note 7 – Seconds when the BER ³ 1 ´ 10–3 should be excluded, when measuring degraded minute performance.

Note 8 – Measurements of the 10–6 BER criterion at the system bit rate, using different integration times, indicate that the errors occurring within a 1 min period may be clustered. The degraded minute objective may therefore become a more stringent requirement to be respected than the severely errored seconds objective (see Annex 1) and possibly more stringent than the degraded minute objective at 64 kbit/s. This subject requires further study.

Note 9 – The errored second allowance includes all performance degradations other than unavailability.

Note 10 – It should be noted that this Recommendation includes allowances for all degradations to performance, additional to those caused by fading.

Note 11 – The Recommendation applies only when the system is considered to be available in accordance with Recommendation ITU-R F.557 and includes periods of high bit error ratio exceeding 10–3 which persist for periods of less than 10 consecutive seconds. Periods of high bit error ratio which persist for 10 consecutive seconds duration or longer are taken into account by Recommendation ITU-R F.557.

Note 12 – It should be noted that:

–	the requirements of this Recommendation are intended to meet the relevant performance objectives of ITU-T Recommendations G.821 and ITU-T G.921 under all normally envisaged operating conditions;

–	ITU-T Recommendation G.821 remains the overriding performance objective of the network.

ANNEX  1

Factors to be taken into account in determining performance
requirements for real digital radio-relay links forming
part of a high-grade circuit within an ISDN

This Annex considers factors which need to be taken into account in determining performance requirements for real digital radio-relay links forming part of a high-grade circuit within an ISDN.

# 1.	Performance objectives for real digital links

## 1.1	General consideration

For the objectives of bit error performance, a hypothetical reference digital connection has been defined in ITU-T Recommendation G.821. It is sub-divided into portions of different circuit quality, termed “high grade”, “medium grade” and “local grade”. The admissible transmission impairment is allocated to these portions in fixed percentages. The apportionments for the local grade and medium grade portions are considered as block allowances, whereas that for the high-grade portion is regarded as an allocation proportional to distance.

In conformity with ITU-T Recommendation G.821, the hypothetical reference digital path (HRDP) given in Recommendation ITU-R F.556, and the associated error performance objectives given in Recommendation ITU-R F.594 applying to high-grade digital radio-relay systems, form the first stage of the sub-division. To enable each administration to specify performance values for transmission systems, further sub-division into hypothetical reference digital sections (HRDS) is necessary. For digital systems of the 2 Mbit/s hierarchy, these values are given in ITU-T Recommendation G.921. Four quality classifications of HRDS have been defined and each has been assigned a fixed percentage of quality impairment. The class 1 section corresponds to the circuit classification “high grade”, but can also be used in the “medium grade” portion. Classes 2 to 4 apply to the medium grade portion only (see Recommendation ITU-R F.696). Information on the “local grade” part, which represents only the subscriber line, is given in Recommendation ITU-R F.697.

Recommendation ITU-R F.594 gives performance objectives for the hypothetical reference digital path (HRDP) as defined in Recommendation ITU-R F.556. Real links will differ both in length and composition from the HRDP, and it is therefore desirable to provide planning objectives for the allowable bit error ratios of such paths, particularly those which are shorter in length than the HRDP.

In general for real links there are two problems:

–	to find a method of apportioning the objectives given in Recommendation ITU-R F.594 for systems which are intended to be connected in tandem in order to form paths similar to the HRDP;

–	to take into account ITU-T Recommendations G.821 and ITU-T G.921 in determining objectives for digital radio-relay links used in medium and local grade portions of an ISDN connection.

It should be noted that in some countries real links can be longer than 2 500 km.

In these cases the objectives given in recommends 1.2 to 1.4 may be applied to links with a real length L (L > 2 500 km), but the SES objective evaluated for the total link length L should not be exceeded for more than [0.05 + (L/2 500) ´ 0.004] % of any month.

Since the performance of digital radio-relay systems is dependent upon fading, it is generally agreed that the behaviour of any section of the HRDP will be statistically independent. If this is assumed, then mathematically the HRDP performance could be determined by the convolution of the probability density functions of all sections. This process is however not a practical one, since the probability density function is not known in sufficient detail.

The example given in Fig. 1 illustrates a further important principle coming from the application of ITU-T Recommendation G.102. This Recommendation draws attention to the need for a margin between network performance objectives and equipment design objectives which are essentially the performance predictions for each hop (see also § 2.5.5 of ITU-T Recommendation G.102). In Fig. 1, the predicted outage for the hop (see Report 784 (Düsseldorf, 1990)) accounts for only a proportion of the objective, 10% is allocated to interference from the fixed-satellite service, whilst the remainder is used as a margin to take account of inaccuracies in the outage prediction method and for other non-deterministic effects.

FIGURE  1/F.634 [D02] = 14 cm

It is necessary to check that all the objectives of this Recommendation are satisfied during route design including severely errored seconds (SES), degraded minutes (DM), errored seconds (ES) and residual bit-error ratio (RBER).

Procedures for predicting performance are described in Recommendation ITU-R F.1093 and the models generally evaluate the percentage of time that a bit-error ratio (BER) of 10–3 will be exceeded during a worst month for multipath fading. Performance measurement has shown that the majority of this time is composed of event durations which are less than 10 s. For this reason, as the worst case, the predicted multipath outage could be considered as severely errored seconds rather than unavailable time (see Recommendation ITU-R F.557 for the definition of unavailable time).

Measurements indicate that in some cases the DM objective of this Recommendation may be more difficult to respect than the SES objective.

A procedure for predicting degraded minute performance may therefore be necessary.

## 1.2	DM and SES objectives at system bit rate

Measurements at the system bit rate indicate that the percentage of time for which BER exceeds 1 ´ 10–6 is four to seven times lower if evaluated with a 1 s instead of a 1 min integration time. One reason for the different percentages of time lies in the fact that elimination of seconds which exceed a BER of 10–3 still leaves significant numbers of burst errors in the adjacent seconds. Measurements on a number of single hop field trials and multi-hop operational systems confirming this behaviour have been reported (see Appendix 1 to Annex 1).

Measurements of SES at the system bit rate should take into account Annex D of ITU-T Recommendation G.821.

The percentage of DM at 64 kbit/s can be taken directly at primary bit rate and above.

Various theoretical and experimental studies have indicated that the direct translation of SES is accurate within a few per cent, or:

% SES64  =  % SES system bit rate

The percentage of SES normalized to 64 kbit/s can be assessed from measurements made at the system bit rate as follows:

% SES64  =  Y%  +  Z%

where:

Y :	percentage SES at the system bit rate

Z : 	percentage of non-SES at the system bit rate containing one or more loss of frame alignment at the system bit rate.

The factor Z% represents an allowance for error bursts which are extended or cause loss of frame alignment during the demultiplexing process between the system bit rate and 64 kbit/s.

It should be noted that Z could be dependent on transmission equipment design and will not necessarily include all non-SES at the measurement bit rate which ultimately cause SES at the 64 kbit/s level. Certain error bursts which can pass transparently through demultiplexers at the hierarchical level of the source of the error burst without causing a loss of frame alignment at that level, can cause losses of frame alignment in demultiplexers at lower hierarchical levels. These events cannot, therefore, be accurately assessed as SES in the measurement at the source of the event.

In the case of digital radio-relay systems the factor Z% is thought to lie between 1 and 5% of the measured performance Y%. There is an urgent need for real measurements of Z% to be undertaken.

Other studies have reported a translation function for DM and SES based on the probability distribution of errors at the system bit rate.

Further evidence is needed to demonstrate that this alternative method is valid for a wide range of system types and in a variety of regions.

## 1.3	Relationship between ES objective at 64 kbit/s and corresponding parameters at the system bit rate

The relationship between the ES of a 64 kbit/s channel and the corresponding parameter which may be measured directly at the system bit rate is under study.

For the time being the following relationship (provisionally agreed in Telecommunication Standardization Study Group 13) is preferred.

The percentage of ES parameter at 64 kbit/s is given by:

where:

n :	number of errors in the ith second at the system bit rate

N :	system bit rate divided by 64 kbit/s

J :	integer number of 1 s periods (excluding unavailability time) within the total measurement period

the ratio (n/N)i for the ith second is:

n / Nmmifmm0  <  n  <  N

or

1mmifmmn  ³  N.

This relationship is conservative, since it assumes that the errors occurring at system bit rate are uniformly distributed among the 64 kbit/s channels.

In practice, due to the non-uniform distribution of errors the actual result at 64 kbit/s is better than the value calculated with the expression given above.

Field measurements indicate agreement with a less severe translation which assumes a Poisson distribution of the errors occurring at line bit rate in the 64 kbit/s channels.

Further studies are required in order to verify whether the adoption of the conservative hypothesis produces substantial additional constraints to the radio system design.

A practical way of using the above formulae is to compute the probability of an ES at 64 kbit/s, (n/N)i, for each second i and summing these probabilities over all the seconds J in the measurement period (e.g. one month). The result will be an estimate of the total number of ES at 64 kbit/s. Division by J and multiplication by 100 gives the percentage of ES in the measurement period. The accuracy of this process depends on the translation that takes place, every second, using the above formula. Measurements have demonstrated that this process gives answers that are within about ± 25% of the true number of ES obtained by actually demultiplexing the signal to 64 kbit/s. It is necessary that the monitoring circuit perform the above described operations in real time, namely the counting of bit errors in 1 s intervals at the system bit rate, truncating the numbers at n = N, and then accumulating and scaling them.

There is strong experimental evidence that compatible ES objectives at higher rates are difficult to establish because they are heavily dependent on the particular kind of radio equipment and path. It is therefore recommended that the ES objective only be applied at 64 kbit/s (see Note 5 of this Recommendation).

Use may be made of a selective demultiplexing method which extracts from the high bit rate signal a secondary error signal by sampling at 64 kbit/s which permits an estimation of system performance at the 64 kbit/s rate to be made.

## 1.4	RBER objective

In this Recommendation, for high-grade real radio-relay systems a residual bit error ratio (RBER) is specified in order to avoid the problem that the degraded minute objective for a 2 500 km HRDP could be exceeded, due to residual errors in concatenated digital sections accumulating to form additional DM. This RBER also serves as a basis for both equipment and system design and a number of administrations have shown that the use of RBER together with the other BER objectives measured at the system bit rate, assures that all the objectives of Recommendation ITU-R F.594 are met.

The RBER is the error ratio in the absence of fading and includes allowance for system inherent errors, environmental and ageing effects and long-term interference. A value of 5.0 ´ 10–9 for the RBER is specified as appropriate in this Recommendation for a 2 500 km high-grade real circuit. Such a value is consistent with a range of figures that can be derived theoretically from the performance objectives of Recommendation ITU-R F.594 on the assumption of various error statistics.

A difficulty arises when defining a measurement procedure for RBER, as it cannot be easily verified that fading is absent on all hops within a section during the measurement period.

A provisional method of measuring RBER has been given in Note 6 to this Recommendation in which 50% of the 15 min intervals taken over one month and containing the worst BER measurements are discarded, in order to eliminate periods affected by significant fading. It should be noted that periods containing errors due to effects other than fading may also be discarded by this procedure, and in some circumstances, periods affected by fading may be included in the measurement procedure.

In determining the percentage of intervals to be discarded, account should be taken of the need to ensure that the errors in concatenated digital sections do not accumulate to form additional degraded minutes within the overall HRDP. If measurements are made over a shorter period than one month, it may be appropriate to discard a higher percentage of intervals. The percentage of intervals to be discarded therefore requires further study.

Currently, a 15 min integration period has been provisionally adopted for the measurement of RBER. However, particularly in the case of low capacity systems (see Note 1 to this Recommendation), other values may also be suitable.

There are a number of proposals from administrations for alternative RBER measurement methods, including different integration periods and also variations in the percentage of intervals to be discarded. In order to determine the most appropriate RBER measurement criteria, administrations are invited to perform practical measurements and submit additional information concerning the measurement conditions (e.g. interference, environmental, etc.) as a matter of urgency. The various proposals are summarized below:

–	a proposal based on a theoretical study is that the RBER threshold set in this Recommendation should be respected for at least 80% of any month. The proposed integration time is 1 h;

–	another administration proposes to choose a variable period long enough to measure with a fixed accuracy (i.e. fixed number of errors) the RBER during the periods least subject to fading, for example from 1000 to 1400 h;

–	the possibility of calculating the RBER from the monthly results of bit errors per second by elimination of the DM and SES occurring in the available time is also being investigated. The method proceeds on the assumption that the DM and SES are experienced only in the presence of fading;

–	another possible method for measuring RBER uses a sliding 24-hour period containing 48 half-hour intervals. Any half-hour period that contains one or more ES with a BER greater than 10–6 is eliminated from further consideration, the rationale being that these ES might be due to propagation effects. The two worst remaining half-hour periods are again eliminated in order to exclude other remaining propagation effects. The RBER is then taken as the BER of the next or third worst half-hour period;

–	measurements taken on a long system during the worst fading months over a two-year period show that the RBER performance objectives of this Recommendation were met for 75% of any month with an integration time of 1 h, provided the SES are excluded from the measurement. Alternately the requirement was met for 95% of any month with an integration time of 1 min; in this latter case the SES were included in the measurement;

–	one administration has reported results of applying the method of measurement proposed in Note 6 of this Recommendation and the other alternative methods and concluded that the traditional method – excluding the worst half of the 15 min periods within the month – is prone to include fading activity in some cases. However, when the fading activity is sufficient to cause this problem, there is a good chance that the ES objective will not be met.

For a real high-grade circuit, the RBER has to be linearly apportioned to derive the corresponding values for the digital sections or to provide system design objectives. However, since such an RBER objective includes many sources of error including interference, ageing and environmental effects, the subject is still under study.

Administrations are invited to submit additional information on the two problems outlined above.

## 1.5	Network performance objectives (NPO) and equipment design objectives (EDO)

ITU-T Recommendation G.801 describes the application of digital transmission models for the specification and sub-division of transmission parameters. It shows a way in which a national reference model can be used to arrive at network performance objectives (NPO) and equipment design objectives (EDO) for real transmission paths using objectives for hypothetical reference digital connections (HRDX), hypothetical reference digital links (HRDL) and hypothetical reference digital sections (HRDS). This procedure is applicable to all transmission media and thus to radio-relay systems.

With the aid of HRDS, each administration can establish its own reference model tailored to the prevailing national network conditions. It has to be ensured, however, that this model meets the objectives of ITU-T Recommendation G.821. To obtain a representative network model, the individual network parts should be composed of digital sections such that as many as possible of the real links are taken into consideration. A block allowance can then be determined for each model network part, from the chosen arrangement of digital sections and their associated allocation percentages. This block allowance represents a network performance objective for the network parts concerned and has to be met by real transmission paths. In terms of design, any radio-relay systems employed in these parts must be capable of fulfilling the applicable NPO. In this method, the radio-relay systems to be used (e.g. frequency

band, bit rate, etc.) and the maximum possible number of hops are defined for each part of the network. Equipment design objectives could then be specified by taking into account environmental effects, equipment ageing, etc. If, in practice, real links are installed which do not exhaust the values given for a particular part of the network model – e.g. have less hops – it is proposed that the link should still be allocated the full block allowance. The idea is to reduce the constraints at the time of the initial concept of the system. For example, the use of space diversity, use of adaptive time domain equalizers could be reduced, smaller sized antennas and lower power transmitters could be used and appropriate performance allowances for oversized hop lengths could be provided.

## 1.6	Intermittent failure mechanisms

A system can experience an intermittent malfunction before total failure occurs. It is important, in maintaining good service, to detect such a condition as early as possible and take corrective action.

# 2.	Error performance objectives for periods shorter than one month

The criteria given in ITU-T Recommendation G.821 (and hence Recommendation ITU-R F.594 and this Recommendation) are normally too long for use as maintenance limits or for circuit provisioning tests. Measurements over much shorter periods (e.g. one day) may be necessary in order to determine whether a circuit is fit for service or should receive maintenance attention.

Radio systems differ from line systems in that the significant performance degradations (i.e. those due to fading) tend to be concentrated into a few days whereas for line systems they tend to be randomly distributed throughout the month. For this reason the mathematical models being examined by the ITU-T to describe the distribution of performance degradations within a month may not be suitable for radio systems. This subject requires active study.

Field measurements have shown that radio systems cannot respect daily performance objectives which are linearly sub-divided with distance and time from the 2 500 km HRDP. The measurements show that a 24-hour performance objective of 20% of the one month objective can be met for SES and ES criteria, but that the DM criteria cannot be used as a reliable indicator of system performance over periods of less than one month. Further measurements are required to confirm the validity of these limits. However, the ITU-T Recommendation G.821 parameters are not ideally suited for monitoring radio system performance over short periods, particularly where fading causes bursty error distributions. Any performance assessment derived from a single 24-hour measurement period will contain a significant degree of uncertainty. It is therefore proposed that any limits used for periods shorter than one month should only be used in conjunction with other supporting performance related data; for example:

–	RBER measurements,

–	other system parameters (e.g. AGC levels and trends),

–	local meteorological data,

–	performance of other radio links in the area,

–	historical performance of radio system.

# 3.	Interference from the fixed-satellite service (FSS)

Many of the frequency bands used by radio relay systems are now shared with the FSS. Interference from the FSS generally may take two forms:

–	from earth stations in the bands used for up links (e.g. 6 GHz, 14 GHz);

–	from satellites in the bands used for down links (e.g. 4 GHz, 11 GHz, 18 GHz).

The level of this interference may be sufficient to cause performance and/or availability degradations to digital radio systems. The limits for the degradations to a 2 500 km HRDP quoted in Recommendation ITU-R SF.615 are given in Table 1.

TABLE  1

Recommended limits for performance and availability degradations due to interference 
from sharing with the FSS

| Criterion | Objective for 2 500 km HRDP (Recommendations ITU-R F.594 and ITU-R F.557) | Allowance for band sharing |
| --- | --- | --- |
| 1. DM 2. SES 3. ES 4. Unavailability | 0.4% of any month 0.054% of any month 0.32% of any month 0.3% of a period probably greater than one year | 0.04% of any month 0.0054% of any month 0.032% of any month 0.03% of a period probably greater than one year |

Some allowance needs to be made for these degradations when setting real route design objectives for systems operating in the shared bands. It is therefore necessary to consider how to apportion the degradations in Table 1 to shorter links.

Sharing models have previously assumed that only one or two stations within a 2 500 km HRDP will be significantly affected by this kind of interference. This would imply that the majority of the degradations listed in Table 1 could be allocated to one or two hops and that sharing criteria (e.g. power flux-density limits, equivalent isotropic radiated power limits) can be derived on this basis.

However, this may in future be an unrealistic assumption since the number of earth stations is generally increasing (especially for systems operating above 10 GHz) and these now are often sited close to major cities. In consequence the number of radio relay receivers affected will also generally increase. This could lead to subsequent performance degradations which were not taken into account in the initial route design.

The most prudent basis for apportioning these FSS degradations within the HRDP would be on a hop by hop basis. Thus the 10% degradation would be included in the performance budget at the route design stage as illustrated in Fig. 1 for the case of SES.

APPENDIX  1

to ANNEX  1

Error performance monitoring in Australia

The unavailability and error performance of the major intercapital 140 Mbit/s 16-QAM radio-relay routes in Australia are monitored to assess compliance with ITU-T, ITU-R and National objectives. This Appendix summarizes results of in-service monitoring over a period of one year (mid 1986 to mid 1987). The error performance of a 2 Mbit/s pseudo-random sequence injected at the second order digital multiplex point is monitored either for unidirectional transmission or bidirectional transmission with a loopback at the far end. The results are analyzed on a 4 weekly (28 day) basis and it should be noted that the multiplex equipment and in some instances short optical fibre capital city digital entrance links are included.

The four routes monitored and their respective monthly error performance objectives are as follows:

| Route | Distance | ITU-R objectives(%) |  |  |
| --- | --- | --- | --- | --- |
|  | (km) | ES | SES | DM |
| 1 – Perth-Adelaide 2 – Adelaide-Melbourne 3 – Melbourne-Sydney 4 – Sydney-Brisbane | 2 628 ´ 2 970 ´ 2 980 860 | 0.64 0.25 0.13 0.11 | 0.108 0.042 0.021 0.019 | 0.80 0.31 0.16 0.14 |

| Measurement | Route 1 |  |  | Route 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| period | ES(1) | SES | DM | ES(1) | SES | DM |
| 11 12 13 14 15 16 17 18 19 10 11 12 13 | 0.25 0.46 0.51 0.16 0.42 0.19 0.28 0.49 0.15 0.13 0.06 0.35 0.01 | 0.007 0.015 0.037 0.035 0.022 0.009 0.016 0.010 0.010 0.006 0.006 0.043 0.003 | 0.43(2) 0.75(2) 1.54(2) 0.96(2) 0.50(2) 0.53(2) 0.59(2) 0.39(2) 0.61(2) 0.33(2) 0.36(2) 1.12(2) 0.10(2) | 0.02(2) 0.02(2) 0.00(2) 0.01(2) 0.01(2) 0.20(2) 0.01(2) 0.04(2) 0.26(2) 0.19(2) 0.27(2) 0.09(2) 0.01(2) | 0.001 0.002 0.000 0.002 0.002 0.014 0.001 0.005 0.015 0.006 0.007 0.004 0.000 | 0.08(2) 0.10(2) 0.06(2) 0.07(2) 0.09(2) 0.70(2) 0.10(2) 0.31(2) 0.76(2) 0.33(2) 0.55(2) 0.26(2) 0.02(2) |

–	Data not available.

| Measurement | Route 3 |  |  | Route 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| period | ES(1) | SES | DM | ES(1) | SES | DM |
| 11 12 13 14 15 16 17 18 19 10 11 12 13 | 0.012 0.003 0.003 0.001 0.001 0.010 0.042 0.000 0.003 0.002 0.004 0.007 0.009 | 0.002 0.001 0.002 0.000 0.000 0.000 0.001 0.000 0.001 0.000 0.001 0.001 0.000 | – 0.05 0.03 0.01 0.01 0.01 0.01 0.00 0.01 0.01 0.02 0.02 0.01 | 0.14(2) 0.34(2) 0.09(2) 0.05(2) 0.24(2) 0.07(2) 0.07(2) 0.01(2) –(2) –(2) –(2) –(2) –(2) | 0.001 0.003 0.003 0.003 0.003 0.002 0.002 0.005 – – – – – | 0.03(2) 0.29(2) 3.70(2) 1.58(2) 1.29(2) 3.01(2) 0.08(2) 0.06(2) –(2) –(2) –(2) –(2) –(2) |

(1)	Percentage of seconds with errors at 2 Mbit/s.

(2)	Objectives exceeded.

An analysis of the results shows the following:

–	severely errored seconds (SES) objectives are met on all the four routes by a factor at least 2.5.

The ES measurements shown in the tables above are made at 2 Mbit/s and therefore are not directly related to the errored seconds objective given in this Recommendation (specified at 64 kbit/s). The measured ES exceeds the 64 kbit/s ES objective in a total of 5 periods on two of the four routes by a maximum factor of 3. Given the configuration of the routes monitored and that a demultiplexing factor which would improve the ES performance at 64 kbit/s is applicable, it can be concluded that the radio-relay systems on the four routes monitored have no difficulty in meeting ES objectives.

The degraded minute (DM) objective which is exceeded for a total of 13 periods, involving three of the four routes, has been found to be a more stringent criterion than SES.

–	The elimination of SES only from DM is insufficient to successfully decouple the errors caused by the onset of and exit from deep fades, thereby creating DM.

_________________