RECOMMENDATION 635-2

RADIO-FREQUENCY  CHANNEL  ARRANGEMENTS  BASED  ON  A
HOMOGENEOUS  PATTERN  FOR  RADIO-RELAY  SYSTEMS
OPERATING  IN  THE  4  GHz  BAND

(Question 136/9)

(1986-1990-1992)

Rec. 635-2

The CCIR,

considering

a)	that high-capacity digital radio-relay systems of the order of 90 Mbit/s, 140 Mbit/s or 200 Mbit/s or synchronous digital hierarchy bit rates are required in the 4 GHz radio-frequency bands;

b)	that the lower band limits of the 4 GHz radio-frequency bands are not uniform and vary internationally from 3 400 to 3 800 MHz;

c)	that efficient use of bands of different width can be achieved by radio-frequency channel arrangements matched to the width of the band available;

d)	that a high degree of compatibility between RF channels of different arrangements can be achieved by selecting all channel centre frequencies from a uniform basic pattern;

e)	that the centre gaps of the individual channel arrangements and the guard spacing at the edges of the band can be chosen by non-occupancy of a suitable number of RF-channel positions in a homogeneous basic pattern;

f)	that the uniform basic pattern spacing should not be unjustifiably small (i.e. the number of RF-channel positions too high) nor so large as to jeopardize efficient use of the available spectrum;

g)	that the absolute frequencies of the basic pattern should be defined by a single reference frequency,

recommends

1.	that the preferred radio-frequency channel arrangement for high-capacity digital radio-relay systems of the order of 90 Mbit/s, 140 Mbit/s or 200 Mbit/s or synchronous digital hierarchy bit rates (Note 1), operating in the 4 GHz band (Note 1), should be selected from a homogeneous pattern with the following characteristics:

Centre frequencies fn of the radio-frequency channels within the basic pattern

(1)

where:

m :	integral number depending on available frequency band: 1, 2, 3, . . . (Note 2);

2.	that all the go channels should be in one half of the band and all the return channels should be in the other half of the band;

3.	that the channel spacing XS, the centre gap YS, the guard spaces Z1S and Z2S at the edges of the band and the antenna polarization should be agreed between the administrations concerned;

4.	that the alternated or co-channel arrangement plan should be used, examples of which are shown in Fig. 1 (see also Note 3).

Figure  1     [D01]  =  10.5 cm

Note 1 – Actual gross bit rates including overhead may be as much as 5% or more higher than net transmission rates.

Note 2 – Due regard should be taken of the fact that in some countries where additional radio-frequency channels interleaved between those of the main patterns are required, the values of the centre frequencies of these radio-frequency channels should be 5 MHz below those of the corresponding main channel centre frequencies as shown in the following equation:

Note 3 – Due regard should be taken of the fact that in some countries the band 3 700-4 200 is used. A radio-frequency channel arrangement, using this band and based on the homogeneous pattern is given in Annex 1, § 4.

ANNEX  1

Frequency arrangements derived from a homogeneous
frequency pattern for the 4 GHz band

Radio-frequency channel arrangements derived from recommends 1 for the 4 GHz band are described below.

# 1.	90 MHz co-channel radio-frequency channel arrangements

The radio channelling plan shown in Fig. 2 for the frequency band 3 600-4 200 MHz is based upon the use of 140 Mbit/s systems employing reduced bandwidth quaternary phase shift keyed (RB 4-PSK) modulation.

# 2.	60/80 MHz radio-frequency channel arrangements

Table 1 describes radio-frequency channel arrangements for the band 3 600-4 200 MHz which are used for 16-QAM or 256-QAM systems.

Figure  2     [D02]  =  8 cm

The radio-frequency channel arrangements for a single-carrier transmission method are used for 16-QAM 200 Mbit/s systems. In these systems, antennas with good cross-polarization characteristics are separately used for transmitting and receiving in commercial equipment to attain the high spectrum utilization efficiency of 5 bit/s/Hz with cross-polarized operation.

The frequency channel arrangements for the 3-carrier transmission method are used for both the 16-QAM system and the 256-QAM system. The 16-QAM system transmits 155 Mbit/s in a 60 MHz bandwidth using 3 carriers and attains a spectrum utilization efficiency of 5 bit/s/Hz with cross-polarization. The 256-QAM system transmits 2 × 155 Mbit/s in a 60 MHz bandwidth using 3 carriers and attains a spectrum utilization efficiency of 10 bit/s/Hz with cross-polarization.

The frequency channel arrangements for the 6-carrier transmission method are used for the 256-QAM systems. These systems transmit 2 × 155 Mbit/s in a 60 MHz bandwidth using 6 carriers and are applicable to hops under very severe propagation conditions.

In these 3 or 6-carrier systems, a single antenna can be shared for transmitting and receiving signals. A cross-polarization interference canceller is also employed.

# 3.	40 MHz alternated radio-frequency channel arrangement

In countries where the lower band limit is 3 600 MHz, the following frequency arrangement ensures the compatibility between 140 Mbit/s 16-QAM radio systems and 155 Mbit/s or 2 × 155 Mbit/s radio systems with a suitable higher level modulation and spectrum efficiency up to 7.25 bit/s/Hz. The frequency channel arrangement shown in Fig. 3 gives an alternated pattern of 7 go and 7 return channels with centre frequencies as follows:

fn :	centre frequency of one radio-frequency channel in the go (return) channel of the band (MHz);

fn  =  4 200 – 10 m, where m  =  58, 54, 50, 46, 42, 38, 34

:	centre frequency of one radio-frequency channel in the return (go) channel of the band (MHz);

=  4 200 – 10 m, where m  =  26, 22, 18, 14, 10, 6, 2.

µTABLE  1

Radio-frequency channel arrangements for the 4 GHz band

| Modulation (Capacity per channel) | 16-QAM (200 Mbit/s) | 16-QAM (155.52 Mbit/s) 256-QAM (311.04 Mbit/s) | 256-QAM (311.04 Mbit/s) |
| --- | --- | --- | --- |
| Frequency band (MHz) | 3 600-4 200 | 3 600-4 200 | 3 600-4 200 |
| Centre frequency of the band f0 (MHz) | 3 900 | 3 900 | 3 900 |
| Centre frequency of the carriers fn (MHz) | f0 ± (30 + 40 n) n = 0, 1, . . ., 6 | f0 ± 20 n n = 1, 2, . . ., 14 | f0 ± (15 + 10 n) n = 0, 1, . . ., 27 |
| Interleaved or co-channel | Interleaved | Co-channel | Co-channel |
| Transmission method | Single carrier transmission method | 3-carrier transmission method (20 MHz bandwidth/carrier) | 6-carrier transmission method (10 MHz bandwidth/carrier) |
| Number of channels | 7 | 10 (1) | 10 (1) |
| Channel bandwidth XS (MHz) X | 80 1.6 | 60 1.54 | 60 1.54 |
| Centre gap YS (MHz) Y | 60 1.2 | 60 1.54 | 60 1.54 |
| Guard space ZS (MHz) Z | 30 0.6 | 40 1.03 | 40 1.03 |

(1)	The capacity of the innermost radio frequency channels is limited to two-thirds of the full capacity.

§

Figure  3     [D03]  =  8.5 cm

# 4.	Description of the radio-frequency channel arrangement for the band 3.7-4.2 GHz

4.1	The radio-frequency channel arrangement for a band 500 MHz wide and for up to six go and six return channels (Group 1) and an interleaved pattern of six go and six return channels (Group 2), each accommodating up to 1 260 telephone channels or the equivalent, or up to 2 × 45 Mbit/s, is shown in Fig. 4 and is derived as follows:

Let:	fr	be the frequency of the lower edge of the band of frequencies occupied (MHz)

fn	be the centre frequency of one radio-frequency channel in the go (return) channel of the band (MHz)

be the centre frequency of one radio-frequency channel in the return (go) channel of the band (MHz);

then the frequencies in MHz of individual channels are expressed by the following relationships:

Group 1

go (return) channel,	fn	=	fr – 50 + 80 n

return (go) channel,		=	fr – 10 + 80 n

where:

n  =  1, 2, 3, 4, 5 and 6.

Group 2

go (return) channel	fn	=	fr – 70 + 80 (n – 6)

return (go) channel,		=	fr – 30 + 80 (n – 6)

where:

n  =  7, 8, 9, 10, 11 and 12.

Figure  4     [D04]  =  6 cm

4.2	In a section over which international connections are arranged, the go and return channels are in the same group and are adjacent channels in that group.

4.3	In any section, both the go and return channels of any one group are of one polarization.

4.4	In any section, the channels of each group are of different polarizations.

4.5	In general, the value of fr is 3 700 MHz.

Note 1 – Subject to agreement between administrations concerned, 1 800 telephone channels may be accommodated on each radio-frequency channel using either Group 1 or Group 2 frequencies.

_________________