RECOMMENDATION  ITU-R  F.701-1

RADIO-FREQUENCY  CHANNEL  ARRANGEMENTS  FOR  ANALOGUE  AND  DIGITAL  
POINT-TO-MULTIPOINT  RADIO  SYSTEMS  OPERATING  IN  FREQUENCY
BANDS  IN  THE  RANGE  1.427  TO  2.690  GHz  
(1.5, 1.8, 2.0, 2.2, 2.4 AND 2.6 GHz)

(Question ITU-R 125/9)

(1990-1994)

Rec. ITU-R F.701-1

The ITU Radiocommunication Assembly,

considering

a)	that both analogue and digital point-to-multipoint microwave TDMA systems are now widely used to provide telephone and other telecommunication subscriber services in rural and remote areas of many countries;

b)	that the overwhelming majority of such systems are operating in a number of specific frequency bands between 1.427 and 2.690 GHz as established by administrations;

c)	that some administrations permit sharing of certain of these frequency bands between point-to-point (P-P) and point-to-multipoint (P-MP) systems;

d)	that the radio-frequency channel arrangements for point-to-point systems are given in Recommendations ITU-R F.283, ITU-R F.382 and ITU-R F.746;

e)	that efficient use of bands of different width can be achieved by radio-frequency channel arrangements matched to the width of the band available;

f)	that a high degree of compatibility between RF channels of different arrangements could be achieved by selecting all channel centre frequencies from a uniform basic pattern;

g)	that the centre gaps of the individual channel arrangements and the guard spacing at the edges of the band can be chosen by non-occupancy of a suitable number of RF-channel positions in a homogeneous basic pattern;

h)	that the uniform basic pattern spacing should not be unjustifiably small (i.e. the number of RF-channel positions too high) nor so large as to jeopardize efficient use of the available spectrum;

j)	that the absolute frequencies of the basic pattern should be defined by a single reference frequency,

recommends

1.	that the preferred radio-frequency channel arrangement for point-to-multipoint radio-relay systems carrying of the order of 10 to 120 telephone channels or a bit rate of the order of 1 to 8 Mbit/s or the equivalent and operating in the frequency bands between 1.427 and 2.690 GHz should be selected from a pattern with the following characteristics:

centre frequencies fn of the radio-frequency channels within the basic pattern:

fn  =  fR  – 0.5 mmmmmmmMHz

where:

m :	integer whose maximum value depends on the available frequency band

fR :	reference frequency;

2.	that these arrangements be applicable to the following frequency bands with the stated values of reference frequency fR (see Notes 1 and 2):

| Frequency band (MHz) | fR (MHz) | Notes |  |
| --- | --- | --- | --- |
| 1 427-1 530 | 1 530 |  | 3 |
| 1 700-1 900 | 1 900 |  | 4 and 5 |
| 1 900-2 100 | 2 100 |  | 4 and 5 |
| 2 100-2 300 | 2 300 |  | 4 and 5 |
| 2 300-2 500 | 2 500 |  | 3 and 6 |
| 2 500-2 690 | 2 690 |  | 4 |

3.	that the channel spacing XS, the centre gap YS, the guard spaces ZS1 and ZS2, and antenna polarization should preferably be chosen to allow P-MP radio systems to coexist with P-P systems operating on channel plans described in Recommendations ITU-R F.283, ITU-R F.382 and ITU-R F.746 and should be agreed between the administrations concerned (see Note 1);

4.	that the alternated or co-channel arrangement plan should be used, examples of which are shown in Fig. 1.

FIGURE  1/F.701...[D01] = 10.5 CM

Note 1 – It is recognized that some administrations have developed frequency channel arrangements that differ from this Recommendation inasmuch as ZS1, ZS2 and YS were not always integer multiples of the channel spacing XS.

Note 2 – Some administrations have selected fR to be a frequency other than those shown in § 2.

Note 3 – A frequency channel arrangement for P-P systems in this band is described in Recommendation ITU-R F.746.

Note 4 – Frequency channel arrangements for P-P systems in these bands are described in Recommendation ITU-R F.283.

Note 5 – Frequency channel arrangements for P-P systems in these bands are described in Recommendation ITU-R F.382.

Note 6 – The band 2 400-2 500 MHz (centre frequency 2 450 MHz) is designated for industrial, scientific and medical (ISM) applications. Radio services in this band must accept harmful interference caused by these applications.

_________________