9B2:  System general characteristics

RECOMMENDATION  ITU-R  F.750-1*

ARCHITECTURES AND FUNCTIONAL ASPECTS OF RADIO-RELAY
SYSTEMS FOR SDH-BASED NETWORKS

(Question ITU-R 160/9)

(1992-1994)

Rec. ITU-R F.750-1

The ITU Radiocommunication Assembly,

considering

a)	that ITU-T Recommendations G.707, G.708 and G.709 specify the bit rates, the multiplexing structure and the detailed mappings associated with the synchronous digital hierarchy (SDH);

b)	that ITU-T Recommendations G.781, G.782 and G.783 specify the general characteristics and functions of synchronous multiplexing equipment and Recommendation G.784 specifies the management of SDH equipment and networks;

c)	that ITU-T Recommendations G.703 and G.957 specify the physical parameters of the electrical and optical interfaces of SDH equipment;

d)	that ITU-T Recommendations G.803 and G.831 specify the architectures and management capabilities of transport networks based on the SDH;

e)	that among the family of SDH equipment there will be synchronous radio-relay systems (SDH-DRRs);

f)	that there is a need to ensure a complete operational integration of the SDH-DRRs in a synchronous network;

g)	that Recommendation ITU-R F.751 specifies transmission characteristics and performance requirements of SDH digital radio-relay systems,

recommends

1.	that synchronous digital hierarchy digital radio-relay systems comply with the requirements described in Annex 1.

ANNEX  1

Contents

1.	Introduction

1.1	Scope

1.2	Abbreviations

1.3	Definitions

2.	Features and layering of the SDH-based networks

2.1	SDH description

2.2	SDH layering

2.3	Network node interfaces (NNI)

2.4	Functional blocks of SDH equipment

3.	Application of radio-relay systems to SDH-based networks

3.1	General considerations

3.2	Multiplex and regenerator sections

3.3	Functional block diagrams

3.4	Radio protection switching (RPS) arrangement

3.5	Synchronization

4.	Function and usage of section overhead bytes

4.1	Multiplex and regenerator section overheads (SOH)

4.2	Media specific bytes

4.3	Reduced SOH functionality for intra-office sections

5.	Radio-relay specific functions

6.	Sub-STM-1 transmission rate SDH radio-relay systems

6.1	Network interfaces

6.2	Multiplexing schemes

6.3	Multiplex and regenerator radio sections

6.4	Functional block diagrams

6.5	Sub-STM-1 radio-relay equipment interface (RREI)

6.6	Radio protection switching

6.7	Section overhead (SOH) for sub-STM-1 DRRs

6.8	Techniques for transport of media specific functions

7.	Operation and maintenance aspects

7.1	Management functions

7.2	Maintenance functions

7.3	TMN interfaces

Appendix 1 – RREI electrical characteristics

Appendix 2 – Migration strategy to SDH-based networks

# 1.	Introduction

## 1.1	Scope

This Annex defines the architectures and functional aspects of the SDH-DRRs aiming at their complete operational integration in a SDH-based network.

The architectures are defined in terms of functional blocks without any constraint on physical implementation.

## 1.2	Abbreviations

ADM		Add/drop multiplexer

ATM		Asynchronous transfer mode

ATPC		Automatic transmitter power control

AU		Administrative unit

AUG		Administrative unit group

BIP		Bit interleaved parity

C		Container

DCC		Data communication channel

DRR		Digital radio-relay system

DXC		Digital cross-connect

ECC		Embedded control channel

FEC		Forward error correction

FOTS		Fibre optics transmission system

HO		Higher order path

HOVC		Higher order virtual container

HPA		Higher order path adaptation

HPC		Higher order path connection

HPT		Higher order path termination

IOS		Intra-office section

IOST		Intra-office section termination

LOF		Loss of frame

LOP		Loss of pointer

LOS		Loss of signal

LOVC		Lower order virtual container

LPA		Lower order path adaptation

LPC		Lower order path connection

LPT		Lower order path termination

MCF		Message communications function

MS		Multiplex section

MSOH		Multiplex section overhead

MSP		Multiplex section protection

MST		Multiplex section termination

MST-RR		Multiplex section termination for sub-STM-1 radio-relay

MTPI		Multiplexer timing physical interface

MTS		Multiplexer timing source

MUX		Multiplexer

NNI		Network node interface

OAM		Operation, administration and maintenance

OH		Overhead

OLI		Optical line interface

OLT		Optical line termination

OR		Optical repeater

PDH		Plesiochronous digital hierarchy

PI		Physical interface

POH		Path overhead

RF		Radio frequency

RFCOH		Radio frame complementary overhead

RPI		Radio physical interface

RPS		Radio protection switching

RREI		Radio-relay equipment interface

RRPI		Radio-relay physical interface for sub-STM-1 radio-relay

RRR		Radio-relay regenerator

RRRP		Radio-relay reference point for sub-STM-1 radio-relay

RRT		Radio-relay terminal

RS		Regenerator section

RSOH		Regenerator section overhead

RST		Regenerator section termination

RST-RR		Regenerator section termination for sub-STM-1 radio-relay

SA		Section adaptation

SA-RR		Section adaptation for sub-STM-1 SDH radio-relay

SDH		Synchronous digital hierarchy

SEMF		Synchronous equipment management function

SMN		Synchronous management network

SMS		SDH management sub-network

SOH		Section overhead

SPI		SDH physical interface

STM-n		Synchronous transport module of order n

STM-RR		Synchronous transport module for sub-STM-1 radio-relay

TCM		Trellis coded modulation

TMN		Telecommunications management network

T, T¢		Baseband access points

TU		Tributary unit

TUG		Tributary unit group

VC		Virtual container

## 1.3	Definitions

The following definitions are relevant in the context of SDH-related Recommendations.

Add/drop mux (ADM)

These are Type III multiplexers as defined in ITU-T Recommendation G.782.

They provide the ability to access any of the constituent signals within a STM-n signal without demultiplexing and terminating the complete signal. The interface provided for the accessed signal could be either according to ITU-T Recommendation G.703 or an STM-m (m < n).

Administrative unit (AU)

An AU is the information structure which provides adaptation between the higher order path layer and the multiplex section layer (see ITU-T Recommendation G.708 for details).

Administrative unit group (AUG)

An AUG consists of a homogeneous, byte interleaved assembly of AU-3s or AU-4s.

Asynchronous transfer mode (ATM)

See ITU-T Recommendations I.150, I.311, I.321 and I.327.

Bit interleaved parity (BIP)

BIP-X is a code defined as a method of error monitoring (see ITU-T Recommendation G.708 for details).

Container (C)

A container is the information structure which forms the network synchronous information payload for a VC (see ITU-T Recommendation G.708 for details).

Data communication channel (DCC)

See ITU-T Recommendation G.782.

Digital cross-connect (DXC)

See ITU-T Recommendation G.782.

Embedded control channel (ECC)

See ITU-T Recommendation G.782.

Higher order virtual container (HOVC): VC-n (n = 3,4)

This element comprises either a single C-n (n = 3,4) or an assembly of TUGs (TUG-2s or TUG-3s), together with the VC POH appropriate to that level.

Higher order path (HO)

In a SDH network, the higher order path layers provide a server network from the lower order path layers (see ITU-T Recommendation G.782).

Higher order path adaptation (HPA)

The HPA function adapts a lower order VC (VC-1/2/3) to a higher order VC (VC-3/4) by processing the TU pointer which indicates the phase of the VC-1/2/3 POH relative to the VC-3/4 POH and assembling/dis-assembling the complete VC-3/4 (see ITU-T Recommendation G.783).

Higher order path connection (HPC)

The HPC function provides for flexible assignment of higher order VCs (VC-3/4) within an STM-n signal (see ITU-T Recommendation G.783).

Higher order path termination (HPT)

The HPT function terminates a higher order path by generating and adding the appropriate VC POH to the relevant container at the path source and removing the VC POH and reading it at the path sink (see ITU-T Recommendation G.783).

Inter-office section

See ITU-T Recommendation G.958.

Intra-office section (IOS)

See ITU-T Recommendations G.957 and G.958 and § 3.1 of Annex 1.

Intra-office section termination (IOST)

See ITU-T Recommendation G.958 and § 3.1.

Lower order virtual container (LOVC): VC-n (n = 1,2)

This element comprises a single C-n (n = 1,2) plus the lower order VC POH appropriate to that level.

Lower order path adaptation (LPA)

The LPA function adapts a PDH signal to an SDH network by mapping/de-mapping the signal into/out of a synchronous container. If the signal is asynchronous, the mapping process will include bit level justification.

Lower order path connection (LPC)

The LPC function provides for flexible assignment of lower order VCs in a higher order VC.

Lower order path termination (LPT)

The LPT function terminates a lower order path by generating and adding the appropriate VC POH to the relevant container at the path source and removing the VC POH and reading it at the path sink.

Message communications function (MCF)

See ITU-T Recommendation G.782.

Multiplex section overhead (MSOH)

MSOH comprises rows 5 to 9 of the SOH of the STM-n signal.

Multiplex section protection (MSP)

The MSP function provides capability for branching the signal onto another line system for protection purposes (see ITU-T Recommendations G.782 and G.783).

Multiplex section termination (MST)

The MST function generates and adds rows 5 to 9 of the SOH (see ITU-T Recommendation G.783).

Multiplex section termination for sub-STM-1 radio-relay (MST-RR)

See § 6.

Multiplexer timing physical interface (MTPI)

The MTPI function provides the interface between an external synchronization signal and the multiplex timing source (see ITU-T Recommendation G.783).

Multiplexer timing source (MTS)

The MTS function provides timing reference to the relevant component parts of multiplexing equipment and represents the SDH network element clock (see ITU-T Recommendation G.783).

Network node interface (NNI)

See ITU-T Recommendation G.708 and § 2.3 of Annex 1.

Operation, administration and maintenance (OAM)

See ITU-T Recommendation G.784.

Optical line interface (OLI)

See ITU-T Recommendation G.957.

Optical line termination (OLT)

See ITU-T Recommendation G.958.

Path overhead (POH)

The VC POH provides for integrity of communication between the points of assembly of a VC and its point of dis-assembly.

Plesiochronous digital hierarchy (PDH)

See ITU-T Recommendation G.703.

Radio frame complementary overhead (RFCOH)

The transmission capacity contained in the radio frame (see § 5.3.1 and Recommendation ITU-R F.751).

Radio physical interface (RPI)

A common description for the typical radio-relay functions, including modulator, demodulator, transmitter, receiver, possible radio framer, etc. (see § 6.4).

Radio protection switching (RPS)

See § 3.4.

Radio-relay equipment interface (RREI) for sub-STM-1 radio-relay

See Appendix 1.

Radio-relay physical interface (RRPI) for sub-STM-1 radio-relay

See § 6.5.

Radio-relay regenerator (RRR)

See § 3.1.

Radio-relay reference point for sub-STM-1 radio-relay (RRRP)

See § 6.2.

Radio-relay terminal (RRT)

See § 3.1.

Regenerator section (RS)

A regenerator section is part of a line system between two regenerator section terminations.

Regenerator section overhead (RSOH)

The RSOH comprises rows 1 to 3 of the SOH of the STM-n signal.

Regenerator section termination (RST)

The RST function generates and adds rows 1 to 3 of the SOH; the STM-n signal is then scrambled except for row 1 of the SOH (see ITU-T Recommendation G.783).

Regenerator section termination for sub-STM-1 radio-relay (RST-RR)

See § 6.4.

Section adaptation (SA)

The SA function processes the AU-3/4 pointer to indicate the phase of the VC-3/4 POH relative to the STM-n SOH. Byte multiplexes the AU groups to construct the complete STM-n frame (see ITU-T Recommendation G.783).

Section adaptation for sub-STM-1 radio-relay (SA-RR)

See § 6.4.

Section overhead (SOH)

SOH information is added to the information payload to create an STM-n. It includes block framing information and information for maintenance, performance monitoring and other operational functions.

SDH physical interface (SPI)

The SPI function converts an internal logic level STM-n signal into an STM-n line interface signal (see ITU-T Recommendation G.783).

Synchronous equipment management function (SEMF)

The SEMF converts performance data and implementation specific hardware alarms into object-oriented messages for transmission over DCCs and/or a Q interface (see ITU-T Recommendation G.782).

Synchronous management network (SMN)

This is a subset of the TMN. See ITU-T Recommendation G.784.

Synchronous digital hierarchy management sub-network (SMS)

This is a subset of the SMN. See ITU-T Recommendation G.784.

Synchronous transport module (STM)

A STM is the information structure used to support section layer connections in SDH. See ITU-T Recommendation G.708 for more details.

Synchronous transport module for sub-STM-1 radio-relay (STM-RR)

See § 6.2.

Telecommunications management network (TMN)

The purpose of a TMN is to support administrations in management of their telecommunications network. See ITU-T Recommendation M.30 for details.

Tributary unit (TU)

A TU is an information structure which provides adaptation between the lower order path layer and higher order path layer. See ITU-T Recommendation G.708 for details.

Tributary unit group (TUG)

One or more TUs, occupying fixed, defined positions in a higher order VC payload is termed as a tributary unit group.

T, T ¢

Access points of telecommunications equipment as defined in Recommendation ITU-R F.596.

Virtual container (VC)

A VC is the information structure used to support path layer connections in the SDH. See ITU-T Recommendation G.708 for details.

Type I/IA multiplexer:

–	Type I (see ITU-T Recommendation G.782)

This provides a simple G.703 to STM-n multiplex function. For example, 63 ´ 2 048 kbit/s signals could be multiplexed to form an STM-1 output or, 12 ´ 44 736 kbit/s signals could be multiplexed to form an STM-4. The location of each of the tributary signals in the aggregate signal is fixed and dependent on the multiplex structure chosen.

–	Type IA (see ITU-T Recommendation G.782)

The ability to provide flexible assignment of an input to any position in the STM-n frame can be provided by including a VC-1/2 and/or VC-3/4 path connection function.

# 2.	Features and layering of the SDH-based networks

## 2.1	SDH description

The synchronous digital hierarchy (SDH) is described in ITU-T Recommendations G.707 – Synchronous Digital Hierarchy Bit Rates, G.708 – Network node interface for the synchronous digital hierarchy, and G.709 – Synchronous multiplexing structure. These Recommendations embrace a new multiplexing method and frame structure which result in a basic rate of 155 520 kbit/s, known as STM-1. The next higher level bit rates are 622 080 kbit/s or STM-4 and 2 488 320 kbit/s or STM-16.

The frame structure of the STM-1 provides a payload area and a section overhead (SOH) as shown in Fig. 1. The multiplexing method is such that a variety of signals may be combined to form the payload by building up tributaries into packages within the STM frame. The section overhead is divided into a number of bytes for various system and network operator functions.

FIGURE 1/F.750-1...[D01] = 8 CM

Details of the SOH are given in § 4. The higher order signals (STM-n) are formed by byte interleaving lower order STM-1 signals (see ITU-T Recommendation G.708).

## 2.2	SDH layering

### 2.2.1	Layering

One of the basic principles which is described in ITU-T Recommendation G.803 is the concept of layering of transport networks.

Figure 2 describes the layer model of the transport network. Features of the layered model are as follows:

–	a circuit layer network, a path layer network and a transmission media layer network;

–	the relationship between any adjacent two layers is a server/client relationship;

–	each layer has its own OAM capability;

–	a circuit layer network provides telecommunications services to users. The circuit layer network is independent of the path layer network;

–	a path layer network is commonly used by the circuit layer networks for different services. The path layer network is independent of the transmission media layer network;

–	a transmission media layer network is dependent on the transmission medium such as optical fibre and radio. The transmission media layer is divided into a section layer and a physical media layer. A section layer can be further divided into a multiplex section layer and a regenerator section layer.

FIGURE 2/F.750-1...[D02] = 14 CM

### 2.2.2	Layering and the SDH frame structure

The SDH frame structure implies an organization of the network in logical layers, namely path and section layers.

The path layer consists of:

–	the lower-order VC layer (LOVC) based on the tributary unit,

–	the higher-order VC layer (HOVC) based on the administrative unit.

The section layer consists of:

–	the multiplexer section layer (MS), and

–	the regenerator section layer (RS).

The RS is media dependent, the MS may be media dependent with a restricted point-to-point topology, while the LOVC and HOVC are designed to be media independent with a wide, complex meshed topology.

## 2.3	Network node interfaces (NNI)

The connection between radio systems and other SDH network elements shall be at standardized interface points. The recommended connection is to make T, T¢ points (in Recommendation ITU-R F.596) coincide with the network node interface (NNI) points identified in ITU-T Recommendation G.708.

An example of the positions of the T, T¢ points and the NNIs is illustrated in Fig. 3.

FIGURE 3/F.750-1...[D03] = 9 CM

## 2.4	Functional blocks of SDH equipment

The use of functional blocks has been adopted by ITU-T Recommendations G.782 and G.783 to simplify the specification of SDH equipment. Decomposition of SDH DRRs into functional blocks complying with these Recommendations is discussed in § 3.3 and 6.4.

# 3.	Application of radio-relay systems to SDH-based networks

## 3.1	General considerations

The scope of this section is to underline the possible applications and topologies foreseen for SDH-DRRs.

The inter-operability of equipment from different sources is maintained as long as the functional requirements of SDH are properly adhered to.

The following main applications for SDH-DRRs are foreseen:

–	use of radio-relays to close an optical fibre ring (see an example in Fig. 4),

–	connection in tandem with fibre optic systems (see an example in Fig. 5),

–	multimedia protection (see an example in Fig. 6),

–	point-to-multipoint systems with integral multiplex functions.

Media independent multiplex sections are possible only if all different media use the MSOH for the same overhead functions (with no media-dependent functions).

When using multi-line protection switching (n + m), the protected radio section may be a multiplex section (see § 3.4).

FIGURE 4/F.750-1...[D04] = 11 CM

FIGURE 5/F.750-1...[D05] = 7 CM

FIGURE 6/F.750-1...[D06] = 10 CM

It should be noted that if SDH radio-relay systems include facilities for radio-protection switching, then they may need to access and recalculate the embedded block error monitoring present with the SOH in the B1 and B2 bytes. In this case, if the B2 bytes are recalculated, the radio-switching section should functionally be a multiplex section.

The sections before a radio multiplex section can be either an intra-office section (IOS) or an inter-office connection.

### 3.1.1	Interfaces

Radio-relay systems shall interface within the SDH network at NNI.

Radio-relay systems can provide either an electrical or an optical intra-office interface. The electrical interface is defined in ITU-T Recommendation G.703 while the intra-office optical interface is defined in ITU-T Recommendation G.957 (see Table 1/G.957).

The need for the utilization of inter-office interconnectivity is under study.

### 3.1.2	Mid-air interconnectivity

It is not practicable for radio systems to provide a radio-frequency interface for mid-air interconnectivity. Mid-air compatibility would require standardization of many additional parameters such as the modulation and coding method, filtering arrangements, diversity combining and protection switching methods and associated control algorithms, adaptive equalizers, overhead bit patterns, FEC, adaptive transmitter power control, etc. Such detailed specifications and standardization would stifle future innovation and do not leave the freedom for different modulation schemes to be used for different applications. Therefore, standardization of a mid-air interface is not required.

## 3.2	Multiplex and regenerator sections

Within a network based on the synchronous digital hierarchy, radio-relay systems shall form either a multiplex section or a regenerator section. In the former case both RSOH and MSOH within the STN-n signal are accessible. In the latter case the RSOH is accessible (see also § 3.3 and Figs. 8 and 10).

Both media-independent and media-dependent multiplex sections are possible.

## 3.3	Functional block diagrams

Figures 7 and 8 show the functional block diagrams of a multiplex section radio terminal and a regenerator section radio terminal respectively.

The partitioning in functional blocks is used to simplify and generalize the description and it does not imply any physical partitioning and/or implementation.

Case a) in Fig. 7 shows a radio terminal following an intra-office section, and case b) shows the radio terminal with an inter-office interface.

The necessity of having radio terminals with inter-office interfaces is under study.

Figure 7, case c), shows a radio terminal forming one end of a regenerator section.

The RPI (radio physical interface) includes the radio functions such as:

–	scrambling/descrambling,

–	error correction code (if implemented),

–	modem,

–	up/down conversions,

–	power amplification (including ATPC, if implemented),

–	filtering, etc.

Refer to Appendix 2 for a migration strategy from PDH to SDH-based networks.

FIGURE 7/F.750-1...[D07] = 21 CM

FIGURE 8/F.750-1...[D08] = 7 CM

## 3.4	Radio protection switching (RPS) arrangement

The multiplex section protection (MSP) defined in ITU-T Recommendation G.782 is not suitable for the improvement of transmission quality as required by radio-relay systems. Therefore, two separate levels of protection are possible:

–	network protection (MSP),

–	radio protection switching (RPS).

Because K1 and K2 bytes are used for network protection and their protocol is not suitable for radio switching, a communication channel for the control signals of a multi-line (n + m) radio protection switching is needed (see § 4).

Three different RPS arrangements are possible and are shown in Fig. 9. Case a) refers to a RPS switching at VC-4 level, while cases b) and c) refer to a RPS switching at STM-1 level.

If the STM-1 signals of the operating and standby channels are not synchronized, both in frequency and in phase, the switching operation causes synchronization losses on the standby channel and consequently an increase of the switching time. To avoid this, cases a) and b) of Fig. 9 incorporate SA functions. In case c) proper resynchronizing techniques have to be adopted with respect to the dynamics of the fading to reduce the global switching operation time.

In the case of twin path (1 + 1) RPS, the STM-1 signals on the operating and standby channels are synchronized both in frequency and in phase as the two channels are continuously fed in parallel by the same signal.

Since SA follows an MST function, cases a) and b) of Fig. 9 are multiplex sections. This also applies to Fig. 9c) when the MST/SA option is used.

In all cases of Fig. 9 the RPS function must be implemented in a manner so that it is transparent to the form of network protection employed. RPS action must not cause any consequential network switch if the latter is used.

# 3.5	Synchronization

Synchronization requirements of SDH digital radio networks are to follow the requirements of ITU-T Recommendation G.782.

Primary reference and slave clocks are specified in ITU-T Recommendations G.811 and G.812 respectively.

Timing references may be derived from external synchronization interfaces, tributary interfaces, or STM-n interfaces.

Requirements for jitter and wander performances for SDH radio-relay systems can be found in Recommendation ITU-R F.751.

FIGURE 9/F.750-1...[D09] = 22 CM

# 4.	Function and usage of section overhead bytes

The frame structure of STM-1 signals provides a payload area and a section overhead (SOH) as shown in Fig. 1. The multiplexing method is such that a variety of signals may be combined to form the payload by building up tributaries into packages within the STM-1 frame. The section overhead is divided into a number of bytes for various system and network operator functions.

## 4.1	Multiplex and regenerator section overheads (SOH)

The concepts of multiplex sections and regenerator sections are described in § 3. Associated with each of those sections is an overhead (MSOH and RSOH). Rules for access to specific rows are given in ITU-T Recommendation G.708 and in § 3.

Figure 10 shows the designation of these overhead bytes which are summarized as follows:

–	6 bytes (A1, A2) for frame alignment,

–	2 bytes (E1, E2) for order wire channels,

–	3 bytes (B2) for multiplex section bit error monitoring,

–	1 byte (C1) for STM identification,

–	1 byte (B1) for regenerator section bit error monitoring,

–	1 byte (F1) for user channel,

–	2 bytes (K1, K2) for automatic protection switching,

–	12 bytes (D1, ¼ D3, D4, ¼ D12) for data communication channels,

–	6 bytes reserved for national use,

–	6 bytes (Z1, Z2) not yet defined,

–	6 bytes for media specific usage,

–	26 bytes reserved for future international standardization.

SDH radio systems shall both transport and utilize the appropriate SOH functions in accordance with ITU-T Recommendation G.708, such that radio systems can be fully integrated into a managed transmission network.

## 4.2	Media specific bytes

ITU-T Recommendation G.708 allows for a total of six bytes for media specific usage in rows 1 to 3 of the SOH, designated S(2,2,1), S(2,3,1), S(2,5,1), S(3,2,1), S(3,3,1) and S(3,5,1). These bytes are shown in Fig. 10.

## 4.3	Reduced SOH functionality for intra-office sections

Intra-office sections offering reduced functionality are the subject of further study by the ITU-T and should be terminated by an intra-office section termination (see Fig. 4).

# 5.	Radio-relay specific functions

SDH-DRRs may provide auxiliary capacity necessary for radio specific functions, e.g. supervisory functions, order wire, radio protection switching, etc.

The radio specific functions and the possible techniques used to transport the related data channels, e.g. using the SOH bytes of the STM-1 or the STM-RR frames or radio frame complementary overhead (RFCOH), are discussed in § 4 of Recommendation ITU-R F.751.

FIGURE 10/F.750-1...[D10] = 15 CM

# 6.	Sub-STM-1 transmission rate SDH radio-relay systems

This section proposes the integration of sub-STM-1 transport capacity digital radio-relay (DRRs) systems with standard interfaces within a synchronous digital hierarchy (SDH) telecommunications network.

This area is of particular interest in cases where the required traffic payload is below that available within an STM-1 signal.

When the STM-1 signal is partly filled, there is the opportunity for the radio-relay to transport only part of the STM-1 signal with the necessary SOH overhead. This can provide saving in cost and radio spectrum and/or reduced modulation complexity.

Radio-relay systems at bit rates below STM-1 to be integrated in an SDH network have to guarantee functional transparency of SDH networks between two STM-1 network node interfaces.

This section addresses radio-relay systems at a bit rate below STM-1 necessary to transport VC-3 payload capacity.

## 6.1	Network interfaces

Sub-STM-1 SDH radio-relay systems shall employ network interfaces at the STM-1 level as described in ITU-T Recommendation G.708 and at the tributary rates described in ITU-T Recommendation G.703.

## 6.2	Multiplexing schemes

The SDH multiplexing route to form the sub-STM-1 synchronous transport module for radio-relay (STM-RR) is deduced from the SDH multiplexing route. The sub-STM-1 signal can be derived from the STM-1 signal, or from tributary rates below the C-4 level as defined in ITU-T Recommendation G.703. These routes can be seen in Fig. 11.

FIGURE 11/F.750-1...[D11] = 17 CM

The following definitions apply to Fig. 11:

–	Radio-relay reference point for sub-STM-1 radio-relay (RRRP): a functional reference point within a sub-STM-1 radio system where the STM-RR is assembled.

–	Synchronous transport module for sub-STM-1 radio-relay (STM-RR): a frame structure at 51.84 Mbit/s rate with overhead and payload mapping as recommended in Annex A of ITU-T Recommendation G.708.

The interconnections of STM-1 and STM-RR are shown in Fig. 12a) and Fig. 12b) respectively for AU-4-based SDH networks and AU-3-based SDH networks.

In the case of AU-4-based SDH networks, the information structure AU-3 does not represent an administrative unit and is not managed as such at network interfaces.

FIGURE 12/F.750-1...[D12] = 12 CM

## 6.3	Multiplex and regenerator radio sections

This section identifies three configurations for sub-STM-1 SDH radio-relay systems as shown in Figs. 13, 14 and 15. In each case the allocations of the multiplex section and regenerator section are shown. These functions are analogous to the MST and RST functions of ITU-T Recommendation G.782.

The configuration shown in Fig. 13 employs ITU-T Recommendation G.708 network node interfaces at each radio terminal, whilst providing an STM-RR transport capability.

FIGURE 13/F.750-1...[D13] = 9 CM

The configuration shown in Fig. 14 employs a single ITU-T Recommendation G.708 NNI, a STM-RR transport capability, and integral multiplexing functionality to provide tributary payload access.

FIGURE 14/F.750-1...[D14] = 8 CM

The configuration shown in Fig. 15 employs ITU-T Recommendation G.703 tributary payload access at each terminal with integral multiplexing with an STM-RR transport capability.

FIGURE 15/F.750-1...[D15] = 9 CM

## 6.4	Functional block diagrams

This section contains functional block diagrams for the system configurations identified in § 6.3 for sub-STM-1 SDH radio-relay systems.

The following definitions apply to Figs. 16 and 17, in addition to those functional block diagrams defined in ITU-T Recommendation G.782:

–	The radio protection switching: this function provides the capability of multi-line protection for radio equipment failure and radio propagation fading phenomena.

–	The section adaptation function for sub-STM-1 radio-relay (SA-RR): this function is analogous to the SA of ITU-T Recommendation G.782 and provides for the adaptation of the paths to be transported in the AU-3. In particular, the SA-RR function processes the AU-3 pointer to indicate the phase of the first column of VC-3 relative to STM-RR SOH.

–	Regenerator section termination for sub-STM-1 radio-relay (RST-RR) and multiplexer section termination for radio-relay (MST-RR): These functions are analogous respectively to RST and MST of ITU-T Recommendation G.782.

–	The radio physical interface (RPI) function: This interface is analogous to the SPI function in ITU-T Recommendation G.782. It refers to the radio path characteristics and it is not subject to international standardization.

## 6.5	Sub-STM-1 radio-relay equipment interface (RREI)

In some cases, it may be desirable to connect radio-relay equipment at sub-STM-1 interface rate of 51.84 Mbit/s. This interface applies at the RRRP and is not an NNI; rather it is intended as an optional interface between sub-STM-1 radio-relay equipment.

The electrical characteristics of the sub-STM-1 RREI are found in Appendix 1.

An application of the RREI is shown in Fig. 18, where inter-operability of equipment from different suppliers within MS can be pursued.

The functional block diagram of a sub-STM-1 regenerator is shown in Fig. 19. The RRPI function converts the internal logical level STM-RR into an RREI line interface signal.

FIGURE 16/F.750-1...[D16] = 15 CM

FIGURE 17/F.750-1...[D17] = 5 CM

FIGURE 18/F.750-1...[D18] = Figure à l'italiènne - Page plaine

FIGURE 19/F.750-1...[D19] = 6 CM

## 6.6	Radio protection switching

Sub-STM-1 radio-relay systems may have radio protection switching (RPS). If the multiplex section contains radio-relay equipment connected through the RREI, then RPS should be implemented independently on either side of the RREI.

A communication channel, if required, for the sub-STM-1 radio protection switching should be implemented using RFCOH, or depending on implementations, bytes C1, F1 and/or one of the data communication channels may be used. The K1 and K2 bytes are reserved for network protection switching.

## 6.7	Section overhead (SOH) for sub-STM-1 DRRs

Figure 20 shows the section overhead (SOH) bytes in the STM-RR. The STM-RR SOH information is classified into regenerator section overhead (RSOH) which is terminated at regenerator functions and multiplexer section overhead (MSOH) which passes transparently through regenerators and is terminated where STM-RR is assembled and dis-assembled.

FIGURE 20/F.750-1...[D20] = 11 CM

The description and the function of the STM-RR bytes are analogous to the corresponding bytes of the STM-1 SOH.

The need for radio specific SOH bytes for sub-STM-1 radio-relay applications has been identified.

Specific SOH bytes have not been assigned. However, depending on the sub-STM-1 radio-relay applications or implementations, some of the SOH bytes may be available because their standard function as in ITU-T Recommendation G.708 may not be necessary or may be achieved by other means, e.g. use of FEC indications for radio performance monitoring. Depending on the implementations, bytes C1, F1 and/or one of the data communication channels may be used.

However, RFCOH could also be used to perform media specific functions.

## 6.8	Techniques for transport of media specific functions

A description of possible radio-relay specific functions is found in § 4 of Recommendation ITU-R F.751. The technique adopted to provide these functions may be implementation dependent; two possibilities are the use of radio frame complementary overhead (RFCOH) and the use of STM-RR SOH as in § 6.7.

# 7.	Operation and maintenance aspects

The operation, administration and maintenance features of SDH radio systems should be designed in accordance with ITU-T Recommendations M.20 (Maintenance philosophy for telecommunications networks), M.30 (Principles for a telecommunications management network) and G.784 (SDH management).

## 7.1	Management functions

SDH radio-relay systems will be part of an overall managed telecommunications network. In particular, these radio systems will be part of a managed synchronous network.

ITU-T Recommendation G.784 allows the SDH management network (SMN) to consist of various managed SDH sub-networks. SDH radio-relay systems will be managed within an SDH management sub-network (SMS) as shown in Fig. 21.

FIGURE 21/F.750-1...[D21] = 11 CM

Details of an SMS consisting of a radio system connected to multiplex equipment are shown in Fig. 22. The network elements (NEs) to be managed are also shown.

FIGURE 22/F.750-1...[D22] = 16 CM

As an SDH-NE, the radio-relay terminal or repeater may have a work-station interface F and/or a Q interface. It may be linked to other NEs according to the architecture of Fig. 3.4 of ITU-T Recommendation G.784. One NE in the SMS should be a gateway NE to facilitate communication with a mediation device or the operations system.

## 7.2	Maintenance functions

Radio specific alarms and a standardized message set have to be defined within Q protocols (ITU-T Recommendations G.783, G.784 and G.831).

This section describes parameters which should be monitored in SDH digital radio systems.

The following represents the minimum set of primitives for radio specific alarms available at the S1 reference point (see Fig. 2.2 of ITU-T Recommendation G.783).

Transmitter status

This parameter should indicate whether the transmitter power level is in the range specified for the system under control. It will therefore have two values:

–	within range,

–	out of range.

This alarm can also contain other major hardware failures, e.g. local oscillator, up-converter, etc.

Receiver status

This parameter should indicate when the receiver RF signal has dropped under the value that produces a BER = 1 ´ 10-3 or worse.

Modulator failure

This parameter should indicate main failures of modulator hardware, e.g. scrambler, carrier level, data loss at modulator input, etc.

Demodulator failure

This parameter should indicate main failure of demodulator hardware, e.g. descrambler, carrier recovery loss, data loss at demodulator output, etc.

RPS status

This function should indicate main failure of the switch hardware, e.g. processor status. It would also be useful for this parameter to indicate the use of the protection channel.

## 7.3	TMN interfaces

SDH radio systems should provide at least one interface at each network element conforming to ITU-T Recommendation G.773.

Full implementation of the TMN interface is the subject of further study.

APPENDIX  1

RREI electrical characteristics

Nominal bit rate: 51.840 Mbit/s

Bit rate tolerance

During synchronized operation, the bit rate tolerance shall be that of the network clock. In a mode without any synchronization to a network clock (e.g. self-timed, free running), the bit rate tolerance shall not exceed 1 037 bit/s (20 ppm).

Line code: B3ZS

Termination

One coaxial line shall be used for each direction of transmission.

Impedance

A resistive test load of 75 W ± 5% shall be used at the interface for the evaluation of pulse shape and the electrical parameters specified below.

Power level

A wideband power measurement using a power level sensor with a working frequency range of at least four times the bit rate frequency shall be between -2.7 dBm and + 4.7 dBm, accounting for both transmitter variations and a range of connecting cable lengths between 68.6 m and 137 m. A filter with a characteristic equivalent to a Butterworth low pass filter with a cut-off frequency of 207.360 MHz shall be used.

Eye diagram

An eye diagram mask based on the maximum and minimum power levels and cable lengths given above is shown in Fig. 23 where the voltage amplitude has been normalized to one, and the time scale is specified in terms of the unit interval T. Exclusionary regions are shown as shaded areas on the figure. The corner points of these regions are listed below the figure.

FIGURE 23/F.750-1...[D23] = 12 CM

| Outer region corner points | Inner region corner points |
| --- | --- |

| Point | Time | Amplitude | Point | Time | Amplitude |
| --- | --- | --- | --- | --- | --- |

| A B C D E | - -0.50 -0.44 -0.18 0.08 0.50 | 0.37 0.80 1.00 1.00 0.37 | F G H I J K | - 0.28 0.00 -0.25 -0.04 0.04 0.11 | 0.12 0.16 0.24 0.80 0.80 0.22 |
| --- | --- | --- | --- | --- | --- |

Note 1 – Both inner and outer regions are symmetric about the zero amplitude axis.

DC offset

There shall be no DC power flow across the interface.

Frame structure

The signal shall have the frame structure and scrambling as defined in ITU-T Recommendations G.708 and G.709.

Jitter

This requires further study.

APPENDIX  2

Migration strategy to SDH-based networks

In the case of migration from an existing PDH (e.g. 140 Mbit/s) to SDH, one possibility is to have SDH-compatible radio systems during the transition period, which have the ability to transport SDH bit rates, although carrying PDH traffic. This can be done by providing special hardware marked by * in Fig. 24a), which will convert the PDH signal to an SDH signal according to the mapping rules of ITU-T Recommendation G.709. When the need to transport real SDH signals arises, the extra hardware will be discarded as shown in Fig. 24b) and replaced by a synchronous physical interface module.

The radio equipment would result in a radio system with full SDH functionality.

FIGURE 24/F.750-1...[D24] = 14 CM

_________________

* 	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 13.