RECOMMENDATION  757*

BASIC  SYSTEM  REQUIREMENTS  AND  PERFORMANCE  OBJECTIVES  FOR
CELLULAR  TYPE  MOBILE  SYSTEMS  USED  AS  FIXED  SYSTEMS

(Question 140/9)

(1992)

Rec. 757

The CCIR,

considering

a)	that cellular type mobile radiocommunication systems are already in wide use;

b)	that in some cases it may be desirable, for reasons of convenience and economy, to apply cellular type mobile radiocommunication systems for use as fixed systems in both rural and urban areas;

c)	that when used in fixed applications such radio links perform an equivalent access function to metallic subscriber lines;

d)	that when used in fixed applications the radio links provided may form part of an international connection;

e)	that the introduction of digital type cellular systems will make it possible to offer various types of service including the local grade portion of an ISDN;

f)	that such radio links will normally be a part of a mobile network,

recommends

1.	that cellular type mobile systems used in fixed applications provide services also available to subscribers by metallic lines. These services include:

–	individual subscriber telephone service;

–	pay-phone service of various kinds;

–	4-wire service with and without E&M signalling;

–	the capability to carry voice-band data signals including facsimile and other telematic services up to a data rate of 9.6 kbit/s;

2.	that digital cellular mobile systems used in fixed applications should provide the same ISDN access as digital mobile cellular systems;

3.	that, since cellular systems used as fixed systems may form part of an international connection, the relevant G-Series CCITT Recommendations should be met;

4.	that, giving due regard to economical considerations, the grade-of-service (lost call probability) offered by such a system to a subscriber shall not normally be worse than 5% and shall be calculated employing CCITT Recommendations E.506, E.541 and Supplement No. 1 to the E-Series Recommendations. As an objective, a service quality comparable to that already provided to fixed subscribers in urban areas should be offered, e.g. a grade-of-service better than 1%;

5.	that the error performance and availability objectives of digital systems should generally be in accordance with CCIR Recommendation 697;

6.	that analogue systems should be designed to provide voice circuits with a noise level less than 1 000 pWp (before companding improvement is taken into account) in the unfaded condition, concerning which further information is provided in Annex 1. As an objective the availability should be better than 99.9%;

7.	that the frequency bands selected by an administration be the same as those utilized by the cellular mobile service;

8.	that Annex 1 should be referred to for the application of cellular type mobile systems as fixed systems.

ANNEX  1

Application of cellular type mobile radiocommunication systems
for use as fixed systems

# 1.	Introduction

Cellular mobile radiocommunication systems are already in wide use. The technology for such systems is expanding rapidly.

It is technically feasible, and in some cases it may be desirable for reasons of convenience and economy, to apply cellular type mobile radiocommunication systems for use as fixed systems. This Annex describes basic system requirements for such applications. In general most applications deal with the connection of subscribers to the telephone exchange and thereby into the switched network.

For brevity, the application of cellular type mobile radiocommunication systems for use as fixed systems will be called simply “cellular type fixed systems”.

# 2.	General considerations

The service to be provided forms a permanent, integral part of the national telephone network and as such can be part of an international connection.

A number of administrations have already implemented such systems, for the provision of telephone service in rural areas. It is important, therefore, to establish the basic system requirements (e.g., performance objectives and frequency bands) which permit such integration as effectively as possible without degrading overall network performance.

The general goal in rural and remote areas is to establish an overall quality of service equal to that achieved by wireline systems in well served urban areas. A minimum objective towards this goal is to achieve service quality at least comparable to that offered in these urban areas, as proposed in the CCITT Handbook on Rural Telecommunications (Geneva, 1985) and in Recommendation 756.

In some cases, it may be effective to use cellular type fixed systems not only in rural areas but also in urban areas e.g. where the cable infrastructure is temporarily inadequate. The radio system has the advantage that it can be deployed rapidly compared with cable systems. Also it may be another attractive feature that the facilities can be easily converted for mobile use after the cable systems become available.

## 2.1	Basic approach

There are two basic approaches for cellular type fixed systems. One is to establish an entirely new cellular type system, optimized for and dedicated to fixed use, and the other approach is to make minimum changes to the existing or planned cellular mobile systems for adaptation to fixed use.

The former approach may be justified in some cases from the viewpoint of economic considerations. However, it should be taken into account that in many cases it may be desirable that a system can accommodate both mobile and fixed subscribers. The latter approach seems preferable for these cases. It is therefore desirable that future cellular mobile systems should include in their design considerations the possible application of the systems for fixed use.

One complicating factor that must be taken into account is whether the cellular system is operating entirely in a fixed service mode or whether it is a mobile system with some fixed users. On the one hand, if the fixed station is part of an existing mobile system, then the service can be provided with considerable economy, since the mobile system provides the basic infrastructure in which the fixed station works. On the other hand, the mobile system will have been designed to satisfy its own performance criteria set by the mobile environment, and may well limit the performance achievable by the fixed station. For example, one administration operates mobile systems with a carrier to interference ratio of 18 dB at the edges of the cell. This provides an acceptable level of performance for a mobile system, but could result in unacceptable performance in the fixed service where the radio link is intended to be part of the telephone network and radio is used instead of wire or cable only for convenience and economy. Another factor is that cellular systems are normally optimized for low traffic subscribers, 0.02 erlang, whereas fixed subscribers normally average between 0.05 and 0.09 erlang.

## 2.2	Frequency bands

Frequency spectrum is a limited natural resource. Therefore, the frequency bands suitable for mobile communication should be primarily used for the mobile services. For this reason, the application of cellular type mobile systems for fixed use may be justified mainly in rural areas where the demand for mobile communication is small and the provision of telecommunication services by means of wireline facilities is too costly.

If cellular mobile systems are adapted for use as fixed systems, frequency bands for cellular type fixed systems should be the same as those for cellular mobile systems.

Frequency bands commonly used for cellular mobile radio are in the 400 MHz and 800/900 MHz bands. Any of these bands are, in principle, suitable for the provision of a fixed service; however, the interference environment in any area where it is proposed to operate must now satisfy criteria for the fixed service as opposed to the mobile cellular service. Further study is required for preferred frequency bands for cellular type systems which have been optimized for fixed use.

# 3.	System requirements

Basic requirements are examined in this Annex.

## 3.1	Operational aspects

As a matter of principle, all kinds of telecommunication services offered through wireline facilities could be made available by cellular type fixed systems. Most of the services are already provided by cellular mobile systems. Among the services which are not usually provided by cellular mobile systems is the pilot number service (multiple lines) which is essential for key telephones and private branch exchanges.

Some features of cellular mobile systems are not necessary for cellular type fixed systems. Among them are roaming and hand-off capabilities. In addition, certain sub-systems of cellular mobile systems may require modifications for adaptation to the fixed use. Most important are the numbering plan and charging sub-system. In particular, in cases where a system accommodates both cellular mobile and cellular type fixed subscribers, the numbering and charging sub-systems should be capable of handling the two categories of subscribers, unless the regulation permits a common sub-system to be applicable to both cellular mobile and cellular type fixed subscribers.

One of the solutions for numbering and charging when cellular systems are introduced into an existing PSTN might be to adopt service control points (SCP) with common channel signalling.

## 3.2	Basic voice channel characteristics

### 3.2.1	General

A full description of the voice channel would involve the specification of all voice channel parameters:

–	circuit noise,

–	frequency response,

–	envelope delay,

–	impulse noise,

–	gain stability,

as well as impedance, return loss, balance, etc., at the interface.

Whilst all the above are important, only the circuit noise is discussed here, the other requirements being the subject of further study.

### 3.2.2	Cellular radio noise performance

Mobile cellular radio traditionally does not specify the noise level of a connection in absolute terms. Channel performance as a whole is specified in terms of Mean Opinion Scores (MOS) which describe the level of satisfaction expressed by a group of listeners to a given circuit, as described in CCITT Recommendations P.70 to P.79. Fixed service applications, on the other hand, deal with absolute values of circuit parameters, such as noise level, frequency response, etc. Hypothetical reference connections to allocate noise allowances to various parts of a circuit have not yet been established for cellular mobile systems.

Analogue cellular radios employ FM modulation and companding. In a mobile environment the performance is normally limited by the rapid fading associated with the movement of the subscriber. In a fixed environment the radio path is, by definition, comparatively stable and subject only to fading and may be designed in accordance with established principles. In particular, directional antennas are used by the subscriber station which increase system gain and help to reduce the effects of interference. Based on reported figures, a steady state (unfaded, high received signal level, no interference), noise level in the range 1 000 to 10 000 pWp is typical of cellular radio.

Companding in accordance with CCITT Recommendation G.162 is normally employed and subjective improvements of the order of 10 to 20 dB have been reported, reducing the steady state circuit noise to 100-1 000 pWp subjective equivalent. However, CCITT Recommendation G.143 provides guidance on the use of compandors and recommends that only a 10 dB “advantage” (improvement in subjective noise level) should be assumed for planning purposes and cautions on the possible effects of compandors used in series in the network.

### 3.2.3	Local loop noise performance requirements

Various performance figures for the noise contribution in the local loop have been put forward for analogue systems. For example, CCITT Recommendation G.103, which provides recommendations for hypothetical reference connections, allows 100 to 500 pWp for the connection to the local exchange. In as much as the rural connection can be part of an international connection, these figures are applicable to the subscriber to exchange connection in a rural area. This is essential if the objective of providing urban quality service is to be achieved. This would not be inconsistent with CCIR Recommendation 395 which, for example, establishes a noise power limit of 350 pWp for a 50 km link and 500 pWp for a 100 km link. These distances are typical for rural subscriber connections via radio. The CCITT Handbook on Rural Telecommunications makes reference to CCITT Recommendation G.103 for guidance but suggests “realistic” figures such as 4 000 to 10 000 pW0p and discusses figures of 1 000 pW0p used by some administrations. CCIR Recommendation 754 which deals with trunk connections in rural areas, uses a figure of 1 000 pWp derived from CCITT Recommendation G.123 (circuit noise in the national network). However, the 1985 CCITT Handbook on Rural

Telecommunications also cautions (on page 28) that, in the most complex reference connection, any appreciable increase of receiving end noise will degrade the system and result in user dissatisfaction. It also discusses the importance of maintaining a proper balance between noise and loss. It is clear that the CCIR and CCITT do not envisage high levels of noise for rural subscriber connections where it can be reasonably and economically avoided.

Narrow-band FM systems such as mobile cellular, while offering performance that may well be acceptable in a number of applications, are limited by the residual noise inherent in these systems. They are designed for an environment where the residual equipment noise is negligible in relation to the noise caused by interference and multipath fading. Wideband techniques, such as the TDMA systems discussed in CCIR Recommendation 756 can offer low noise performance (typically 100 to 200 pWp) owing to, in the case of digital systems, the use of standard A and m-law PCM voice encoding as specified in CCITT Recommendation G.165.

## 3.3	Service requirements

The basic service requirement is typically a 2-wire connection. However, in practice, a variety of additional services is also required. For example, in rural and remote areas, the provision of a pay-phone service is a necessity, while telex, facsimile, 4-wire with E&M signalling and a variety of data services represent a wide range of additional telecommunications services which may need to be provided by radio to subscribers.

## 3.4	Operating environment

In providing telecommunications services, consideration must be given to the likely location of the subscriber station. While it is possible to locate the subscriber terminal at the subscriber's premises, this is not necessarily the best location for the radio antenna. In hilly terrain, houses are most often built in valleys or where some shielding is provided from the weather. This must be taken into account in the system design by, for example, adapting cellular equipment to feed a 1 200 W loop (including the telephone set) when used in the fixed service.

Clearly there are two types of environment possible, protected and unprotected. In the unprotected case, complete environmental sealing is required, which includes protection against rain, dust, sand, corrosive atmosphere, insects, fungus, etc., and operation in high humidity and over temperature ranges from – 40° to + 55 °C is often specified. In the protected case, while environmental protection is less demanding, the equipment must still operate over wide temperature (– 10° to + 55 °C) and humidity (5 to 95%) ranges.

## 3.5	Availability

When used in the fixed service, the design of the radio path should permit an overall availability to an individual subscriber of not less than 99.9% for both propagation and equipment effects (CCIR Recommendation 697). This will achieve a quality of service comparable to that in urban areas served by wireline systems. Some links are often designed with a higher availability when the administration considers it necessary.

## 3.6	Traffic capacity – grade of service

The grade of service or lost call probability is frequently designed to be of the order of 1%, but it is seldom as high as 5%, while some administrations set requirements in the range 0.1 to 0.5%, in order not to degrade the National Network beyond the CCITT recommended objective of 1%. Care must be taken to allow for appropriate growth in the number of subscribers and the higher loss probability figures should, therefore, be avoided, since they will generally result in severe customer dissatisfaction. These probabilities are calculated in the usual manner, employing CCITT Recommendations E.506, E.541, and Supplement No. 1 to the E-Series Recommendations, as well as CCIR Recommendation 756. Factors to consider include:

–	the number of radio channels required,

–	the number of subscribers to be served, and

–	the traffic intensity per subscriber.

Average traffic intensities of 0.05 to 0.09 erlang per subscriber have been used frequently for rural subscribers. The loss probability for up to 6 RF channels is shown in graphical form in the 1985 CCITT Handbook on Rural Telecommunications, page 84, Figs. 7-4.

## 3.7	Power sources

In some rural areas, the commercial a.c. power is either unavailable or is less reliable than that in urban or suburban areas. Substantial attention must be given to provide reliable power sources for the subscriber units in rural areas.

# 4.	Digital systems

Most of the currently operational cellular mobile systems employ analogue modulation (see CCIR Recommendation 622).

Digital cellular mobile systems are currently under development and will be operating within the next few years. Therefore, it may be necessary to develop both analogue and digital cellular type fixed systems. In addition, CCIR Study Group 8 which is concerned with future public land mobile telecommunications systems (FPLMTS) foresees the application of FPLMTS to the fixed service.

In the development of digital cellular type fixed systems, due consideration should be given to the general trend of telecommunications towards the integrated services digital network (ISDN).

Generally, digital systems will offer improved performance compared with analogue systems, especially in terms of more efficient spectrum use and greater interference tolerance. These systems will use low bit rate voice encoding to achieve increased spectrum efficiency. This will cause additional throughput delay (of the order of 50 ms) which, when the link forms part of a long-distance or international call, could frequently cause the overall delay to exceed national or CCITT objectives and require the use of echo cancellers. Voice channel quality of such systems, using encoding schemes other than 64 kbit/s PCM or 32 kbit/s ADPCM, can only be determined using CCITT subjective testing methods (see § 3.2). In addition, systems using low bit rate voice encoding must be examined from the perspective of greater performance degradation caused by successive A/D conversion or re-encodings that exist within any telecommunications network.

A form of ISDN, similar in format to the 2B + D basic access, will also be likely to be possible. For fixed service applications, the performance of the radio link for such a local grade connection is considered in CCIR Recommendation 697.

Introduction of digital type cellular systems will make it possible to offer various types of services (for example kbit/s class data transmission services, or digital services associated with ISDN, etc.).

# 5.	Optimized fixed systems

Optimized fixed systems employing TDMA techniques are described in the 1985 CCITT Handbook on Rural Telecommunications and in CCIR Recommendation 756. These systems were designed specifically for the provision of telecommunications in rural and remote areas via radio and achieve a high level of performance and quality comparable or equal to that offered by wireline services in a well served urban area, but over a distance of up to several hundred kilometres.

Most of these systems operate in certain frequency bands between 1.427 and 2.690 GHz. A variety of telecommunication services (§ 3.3) is often provided in addition to a 2-wire basic telephone service and, in future, full ISDN 2B + D basic access will also be provided. This subject is being examined under Question 125/9. The use of radio repeaters extends the system reach to hundreds of kilometres, and provides the necessary connections to bring isolated subscriber groups to a distant exchange. Generally, most of these systems are designed to serve clusters of subscribers from one subscriber out-station, thus providing economical service to the cluster. Performance is normally specified in terms of detailed channel characteristics (noise, frequency response, envelope delay, etc.). Noise levels of the order of 100 to 200 pWp are typical and radio paths are designed to provide availabilities of the order of 99.9% or better.

# 6.	Summary

Cellular type fixed systems are capable of making telecommunication services available to subscribers in rural areas and, in particular, to subscribers in the developing countries.

Optimized fixed systems can offer, as should be expected, a higher level of performance and service features than might be achieved by the use of cellular radio. The level of performance offered by analogue cellular radio may be acceptable, in some cases, to an administration that requires a basic telephone service for a few widely scattered subscribers, especially if the service can be provided very economically within an existing mobile cell. An administration, however, should consider that the performance which can be achieved may degrade national or international connections beyond acceptable national or CCITT objectives, and that the proliferation of such methods may be undesirable in the telecommunication network as a whole. As is always the case, a full evaluation of suitable radio techniques must be made which will include the consideration of CCITT and CCIR Recommendations, comparisons of achievable versus required performance, cost, the lifetime of the equipment, maintenance, reliability, suitability for the local physical environment, services offered, etc.

CCITT GAS 7 (Volume II, Training Handbook of Rural Telecommunications, Geneva, 1989) addresses many of these topics. The study of the use of cellular mobile radio specifically for rural telecommunications is currently being addressed by GAS 7. The interest of the developing countries in this subject is also reflected in the Questions submitted by the Plan Committee for Latin America (Paramaribo, 1985) and by the Plan Committee for Asia and Oceania (Bali, 1986).

In view of the apparent growing use of analogue cellular mobile radio in the fixed service, it would be highly desirable if administrations could provide information on the effects they have noted on overall network performance.

_________________

*	This Recommendation should be brought to the attention of Study Group 8 and the Plan Committee of the CCITT.