RECOMMENDATION  ITU-R  M.1073

DIGITAL  CELLULAR  LAND  MOBILE
TELECOMMUNICATION  SYSTEMS

(Question ITU-R 107/8)

(1994)

Rec. ITU-R M.1073

The ITU Radiocommunication Assembly,

considering

a)	that digital signals in various formats are being used to improve the communications efficiency of the land mobile service;

b)	that digital transmission systems which are not compatible with existing land mobile systems should also be considered, including the transmission of digitally encoded speech signals;

c)	that mobile telephone services, i.e. services for public correspondence via radio stations connected to the switched public telephone network, are in operation in a number of countries and that their use is extending;

d)	that the various technical systems already in use or proposed for such services, are not necessarily compatible;

e)	that system compatibility is necessary in the case of international operation;

f)	that for international operation it is desirable to agree on the parameters of the system;

g)	Recommendation No. 310 of the World Administrative Radio Conference (Geneva, 1979) (WARC-79);

h)	Question ITU-R 52/8 on the integration of public radiocommunication services in the VHF/UHF frequency bands;

j)	the need to improve spectrum utilization efficiency and hence system capacity per MHz per unit area;

k)	the need for a flexible system structure able to match network investment to revenue growth, readily adapting to environmental factors and responding to new developments rather than restricting innovation;

l)	the increasing importance of the various types of data and telematic services;

m)	Question ITU-R 101/8 on digitized speech transmission, Question ITU-R 37/8 on cellular systems;

n)	Recommendation ITU-R M.622 on analogue cellular systems;

o)	ITU-T Recommendations and on-going work items that are relevant to this work,

recommends

that the following technical and operational characteristics of digital cellular land mobile telecommunication systems should be adopted for systems intended for international or regional use:

# 1.	General objectives

The general objectives of digital cellular land mobile telecommunication systems (DCLMTS) are to provide:

–	systems with high spectrum utilization efficiency, thereby accommodating more users within the limited spectrum resource than existing analogue cellular public land mobile telecommunication systems (PLMTS);

–	users with a wide range of services and facilities, both voice and non-voice, that are compatible with, and access, those offered by the public fixed networks (PSTN, ISDN, PDN, etc.);

–	services and facilities exclusive to mobile systems, including facilities for automatic roaming, locating and updating mobile users;

–	users with a variety of mobile stations consistent with their requirements, ranging from vehicle mounted to hand-held stations with voice and non-voice interfaces;

–	services of high quality and integrity at an economic cost;

–	mobile equipment and infrastructure at the reduced cost, weight, size and power drain offered by the adoption of digital processing and VLSI technology.

# 2.	Digital technology

Digital technology is introduced into the PLMTS in five major areas:

–	digital radio modulation/demodulation;

–	digital speech coding;

–	channel coding and digital signal processing;

–	digital control and data channels;

–	privacy and authentication.

# 3.	Service types

The basic telecommunication services offered by the DCLMTS can be divided into two types:

–	bearer services which give the user the capacity needed to transmit appropriate signals between certain access points;

–	teleservices which provide the user with the full capacity, including terminal equipment functions, to communicate with other users.

Supplementary services are also available in association with the basic services.

The services supported by the DCLMTS in each of these categories are related to those offered by the ISDN, but are for the time being confined to lower bit rate channels (typically less than 16 kbit/s) by the limitations of the radio channel. All the DCLMTS support some services in each category, but the range offered varies between systems.

## 3.1	Bearer services

Typical bearer services offered include:

–	synchronous, asynchronous and packet data at rates up to a maximum of 9.6 kbit/s;

–	unrestricted digital capability at specific bit rates (generally less than 16 kbit/s).

In general, connection of voice-band modems to the speech path of mobile stations is not supported. Equivalent service to that offered by the use of voice-band modems on the PSTN or ISDN can be provided via the bearer services listed above.

## 3.2	Teleservices

All the DCLMTS support telephony and facsimile teleservices. Some extend the teleservice offerings to include videotex, teletex, etc.

## 3.3	Supplementary services

The range of supplementary services supported by the DCLMTS varies depending on the system and also the particular implementation.

# 4.	Architecture common to all digital systems

## 4.1	Base station layout

The geographical distribution of base stations is organized around two types of structure:

–	regular cell structures using omnidirectional antennas and,

–	sector cell structures using directional antennas.

## 4.2	Channel design

Two basic categories of channels are defined for DCLMTS:

–	traffic channels (TCH) which are used for voice and data transmission (i.e. bearer services and teleservices);

–	control channels (CCH) which are used for signalling and control purposes, including handover.

The CCH are further divided into three broad types:

–	common control channels (CCCH) which are used for paging, random access, etc.;

–	broadcast control channels (BCCH) which are used for broadcast messages, and/or synchronization and frequency correction;

–	associated control channels (ACCH) which can be divided into slow ACCH (SACCH) and fast ACCH (FACCH) and provide control and signalling functions for individual users.

Some systems may also define other types of control channel for particular applications (e.g. stand-alone dedicated control channels).

The basic terminologies for some of these control channels can be found in the ITU-T Q.1000 Series of Recommendations.

## 4.3	Network architecture and assignment of functions

Figure 1 shows the basic system architecture for a DCLMTS, including the major functional components. The communication protocols are specified according to the 7-layer OSI model, while the interfaces between mobile switching centres (MSCs) and the interfaces to the ISDN, PSTN and PDN are all specified according to ITU-T Recommendations. The numbering plan also follows ITU-T Recommendations.

# 5.	Systems being installed or planned

General characteristics of the systems are given in Annex 1. Annexes 2 to 5 give a general description of specific systems.

FIGURE 1...[D01] = 18 CM

ANNEX 1

Systems being installed or planned

# 1.	Introduction

High capacity digital cellular systems are being developed in Europe, North America and Japan. Each of these systems has the basic objectives and characteristics outlined in the Recommendation. However each is being developed with a slightly different focus and with different constraints. This has produced three different system specifications. Each of these systems is introduced below and more detailed descriptions can be found in Annexes 2 through 5.

## 1.1	GSM

The development and specification of this Pan-European system in the 900 MHz band (GSM 900) and in the 1 800 MHz band (DCS 1 800) has been carried out through cooperation among more than 16 European countries. The system is operated by one or more operators in each country from the beginning of the nineties.

The international aspects of the GSM system were taken into account from the very beginning, making GSM a true international public land mobile network (PLMN). For example, GSM supports roaming between countries and full access to the international PSTN, ISDN and PDNs.

## 1.2	North America

In North America, two digital cellular standards have been developed, one based on TDMA (time division multiple access) and one based on CDMA (code division multiple access). These standards have been designed to meet the requirements to introduce new digital services and to provide additional traffic capacity while maintaining compatibility with the existing advanced mobile phone service (AMPS) analogue standard. The new digital standards will be used in North America, and a similar design can be used by over forty countries that have adopted, installed or specified AMPS or its derivative (TACS).

The dual mode digital and analogue North America Cellular Standards for PLMTS has evolved from the existing AMPS standard to provide both voice and data services with a ten-times traffic capacity improvement as the goal. The North America Digital Cellular Standards support international roaming and full access to the PSTN, ISDN and PDN.

## 1.3	Japan – Personal digital cellular

Due to the rapid increase in the number of mobile subscribers in Japan, the capacity of the existing analogue cellular system is approaching saturation. To accommodate more subscribers and provide a greater variety of services, digital cellular technology offers promising opportunities. The Japanese Personal Digital Cellular (PDC) provides new digitized traffic and control channels. The standard is applicable to both the 800/900 MHz and 1.5 GHz bands.

# 2.	Explanation of table

Annexes 2, 3, 4 and 5 give details of digital cellular systems emanating from Europe, North America and Japan respectively. Table 1 presents the core parameters for these four systems. In each case complete specifications are, or will be, available from the relevant authorities as indicated in the Annexes.

TABLE  1

Core parameters

| Feature | GSM 900/DCS 1 800 | North American D-AMPS | North American CDMA | Japan PDC |
| --- | --- | --- | --- | --- |

| Class of emission |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – traffic channels | 271KF7W | 40K0G7WDT | 1250K0B1W | 32K0G7WDE |
| --- | --- | --- | --- | --- |

| - – control channels | 271KF7W | 40K0G1D | 1250K0B1W | 32K0G1D |
| --- | --- | --- | --- | --- |

| Transmit frequency bands (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – base stations | 935-960 (GSM) 1 805-1 880 (DCS) | 869-894 | 869-894 | 810-826 1 477-1 501 |
| --- | --- | --- | --- | --- |

| - – mobile stations | 890-915 (GSM) 1 710-1 785 (DCS) | 824-849 | 824-849 | 940-956 1 429-1 453 |
| --- | --- | --- | --- | --- |

| Duplex separation (MHz) | 45 (GSM) 95 (DCS) | 45 | 45 | 130 (0.9 GHz) 48 (1.5 GHz) |
| --- | --- | --- | --- | --- |

| RF carrier spacing (kHz) | 200 | 30 | 1 250 | 25 interleaved 50 |
| --- | --- | --- | --- | --- |

| Total number of RF duplex channels | 124 (GSM) 374 (DCS) | 832 | 20 | 640 (0.9 GHz) 960 (1.5 GHz) |
| --- | --- | --- | --- | --- |

| Maximum base station e.r.p. (W) |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – peak RF carrier | 300 (GSM) 20 (DCS) | 300 | Not specified | Not specified |
| --- | --- | --- | --- | --- |

| Nominal mobile station transmit power (W) Peak, average | 8, 1.0 (GSM) 1, 0.125 (DCS) 5, 0.625 (GSM) 0.25, 0.031 (DCS) 2, 0.25 (GSM) 0.8, 0.1 (GSM) | 9, 3 4.8, 1.6 1.8, 0.6 To be defined | 0.2, 0.01 | 3 2 0.8 0.3 |
| --- | --- | --- | --- | --- |
| Cell radius (km) |  |  |  |  |

| - – minimum | 0.5 | 0.5 | Not specified | 0.5 |
| --- | --- | --- | --- | --- |

| - – maximum | 35 | 20 | 50 | 20 (up to 60) |
| --- | --- | --- | --- | --- |

| Access method | TDMA | TDMA | CDMA | TDMA |
| --- | --- | --- | --- | --- |
| Traffic channels/RF carrier |  |  |  |  |

| - – initial | 8 | 3 | 61 | 3 |
| --- | --- | --- | --- | --- |

| - – design capability | 16 | 6 | 122 | 6 |
| --- | --- | --- | --- | --- |

| Modulation | GMSK (BT = 0.3)f | p/4 differentially encoded QPSK (roll-off = 0.35) | QPSK (spreading) BPSK (outbound); 64-ary orthogonal (inbound) | p/4 shifted QPSK (roll-off = 0.5, root Nyquist filter) |
| --- | --- | --- | --- | --- |
| Transmission rate (kbit/s) | 270.833 | 48.6 | 9.6 per channel 614.4 per carrier | 42 |

TABLE  1 (continued)

| Feature | GSM 900/DCS 1 800 | North American D-AMPS | North American CDMA | Japan PDC |
| --- | --- | --- | --- | --- |

| Traffic channel structure |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – Full rate speech codec |  |  |  |  |
| --- | --- | --- | --- | --- |

| – bit rate (kbit/s) | 13 | 8 | 8.55 maximum | 6.7 |
| --- | --- | --- | --- | --- |

| – error protection | 9.8 kbit/s FEC + speech processing | 5 kbit/s FEC |  |  |
| --- | --- | --- | --- | --- |

| – coding algorithm | RPE-LTP | CELP | Variable rate CELP | VSELP |
| --- | --- | --- | --- | --- |

| - – Half rate speech codec |  |  |  |  |
| --- | --- | --- | --- | --- |

| – initial | To be defined | To be defined | No | No |
| --- | --- | --- | --- | --- |

| – future | Yes | Yes | Yes | Yes |
| --- | --- | --- | --- | --- |

| – Data |  |  |  |  |
| --- | --- | --- | --- | --- |

| – initial net rate (kbit/s) | up to 9.6 | 2.4, 4.8, 9.6 | up to 9.6 | To be defined |
| --- | --- | --- | --- | --- |

| – other rates (kbit/s) | up to 12 | To be defined | To be defined | To be defined |
| --- | --- | --- | --- | --- |

| Channel coding | Rate 1/2 convolutional code with interleaving plus error detection | Rate 1/2 convolutional code | Convolutional code with interleaving and error detection; rate 6/11 outbound; rate 1/3 inbound | Rate 9/17 convolutional code with 2 slot interleaving and error detection (speech traffic channel) |
| --- | --- | --- | --- | --- |

| Control channel structure |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – common control channel | Yes (3) | Shared with AMPS | Yes (configurable) | Yes |
| --- | --- | --- | --- | --- |

| - – associated control channel | Fast and slow | Fast and slow | Embedded dim and burst | Fast and slow |
| --- | --- | --- | --- | --- |

| - – broadcast control channel | Yes (3) | Yes | Yes (configurable) | Yes |
| --- | --- | --- | --- | --- |

| Delay spread equalization capability (ms) | 20 | 60 | Rake receiver (spread limited by code reuse) | (1) |
| --- | --- | --- | --- | --- |

| Handover |  |  |  |  |
| --- | --- | --- | --- | --- |

| - – mobile assisted | Yes | Yes | Yes | Yes |
| --- | --- | --- | --- | --- |

| - – inter-system capability with existing analogue system | No | Yes, between D-AMPS and AMPS | CDMA to AMPS | No |
| --- | --- | --- | --- | --- |

| International roaming capability | Yes >16 countries | Yes | Yes | Yes |
| --- | --- | --- | --- | --- |

| Design capability for multiple operators in same area | Yes | Yes | Yes | Yes |
| --- | --- | --- | --- | --- |

- (1) Equalizer not required; however, an equalizer is available as an option for certain propagation environments. Delay spread is not specified.

ANNEX 2

General description of the GSM system

# 1.	Introduction

The characteristics of the GSM system that are common to most of the digital cellular systems can be found in Annex 1. Therefore this Annex highlights only original aspects of the GSM system and, in fact, only parts of them.

The driving force of the GSM has been its international layout based on a common availability of virtually “clear” frequency bands. This situation offered a unique opportunity of optimizing the usage of new technologies, and therefore spectrum efficiency, with a rather limited number of constraints. A very advanced radio design was therefore possible.

The GSM system is applicable to both the 900 MHz band (GSM 900) and the 1 800 MHz band (DCS 1 800). Full detailed information on the specifications of the GSM system is given in the references list [ETSI].

# 2.	Services

In the process of drafting the GSM standard, the details of the implementation of each particular service together with the required interworking mechanisms have been specified in order to offer full access to the services while roaming, and to minimize the complexity of the mobile station.

## 2.1	Bearer services

The bearer services offered by the GSM PLMN include transparent and non-transparent data services for circuit as well as packet mode, up to a net data rate of 12 kbit/s.

## 2.2	Teleservices

Among the main teleservices supported by the GSM are:

–	speech, i.e. telephony and emergency calls,

–	short message service,

–	data message handling system access,

–	videotex,

–	facsimile.

## 2.3	Supplementary services

The supplementary services offered by GSM operators can be divided into four main groups:

–	call forwarding,

–	call completion,

–	advice of charge,

–	call restriction.

## 2.4	Security aspects

Further to the provision of a wide range of services, the GSM system has also been designed to ensure a high level of security. Therefore security features are provided to protect the access to the services and the privacy of user-related information. The following security features are implemented in the GSM system:

–	subscriber identity confidentiality: it ensures that the mobile subscriber identity (IMSI) cannot be disclosed;

–	subscriber identity authentication: it verifies that the subscriber identity sent by the mobile is the one claimed (not duplicated or impersonated);

–	user data confidentiality: it ensures that the user data, including speech, transferred on the radio path cannot be disclosed by unauthorized bodies;

–	signalling information element confidentiality: it is the property that a given piece of signalling information (subscriber and equipment identities, directory numbers, etc.) exchanged on the radio path cannot be used by unauthorized individuals or entities.

The IMSI is the information which uniquely identifies the subscriber, and that has to be present and valid to allow the operation of the mobile station.

Each mobile station has a unique identity that shall be implemented by the manufacturer: the international mobile equipment identity (IMEI).

The security functions for authentication of the subscriber related information, and all processes involving the authentication key are contained in a removable piece of the mobile station called the subscriber identity module (SIM).

# 3.	Overview of the system

The GSM system has been standardized by administrations, operators and manufacturers in over 16 European countries in order to provide full service access to international roamers. The standard of the GSM is described in terms of interfaces and functional entities.

Two interfaces are mandatory: the radio interface (Um) and the interface “A” between the mobile services switching centre (MSC) and the base station system (BSS). A further interface “A bis” within the BSS system is being specified but its implementation is not mandatory.

The functional architecture is given in Fig. 2. It shows:

–	the MSC, the home location register (HLR), and the visitor location register (VLR), where the networking and switching functions are performed;

–	the BSS which includes the base stations controller (BSC) and the base station transceivers;

–	the operations and maintenance centre (OMC);

–	the mobile station (MS).

The MAP is the mobile application part of ITU-T Signalling System No. 7 which has been specified to allow the routing of calls to MS which have roamed to different MSC areas or to different networks.

The MSC, HLR and VLR execute interworking with partner networks, call control and encryption of signalling and user speech and data. These functions also include authentication of the mobile user, location updating as roaming occurs, paging of the mobile to indicate incoming calls.

The BSS performs the radio channel management functions which include administration of the radio channel configurations, allocations of radio channels and link supervision, scheduling of messages on broadcast channels, choice of frequency hopping sequences whenever needed, and power controls.

# 4.	Technical radio characteristics

These characteristics are specified in GSM Recommendation Series-05 and 06.

## 4.1	RF equipment requirements

In accordance with GSM Recommendation 05.05 (see references list [ETSI]).

FIGURE 2...[D02] = 18 CM

## 4.2	Carrier spacing

A 200 kHz carrier spacing yields at least 18 dB adjacent RF channel selectivity within the system. The second adjacent RF channel at 400 kHz spacing yields at least 50 dB selectivity within the system. The corresponding third adjacent RF channel selectivity yields at least 58 dB.

Frequency hopping is a possible feature.

## 4.3	Class of emission

271KF7W according to Radio Regulations RR 4, i.e. Gaussian minimum shift keying GMSK (BT = 0.3) with a modulation rate of 270.83 kbit/s per carrier, using a time division multiple access (TDMA) scheme for eight basic physical channels.

## 4.4	Cell structure and carrier re-use

It is possible to use large cells (up to 35 km base-mobile distance) in rural areas as well as small cells (down to 1 km diameter) in urban areas.

Extended cell operation ranging up to 120 km base-mobile distance is also possible.

In areas of high peak traffic density (e.g. city centres) it is possible to build up a sector cell structure using directional antennas with a channel concentration at the traffic peak area.

Co-channel protection ratio down to C/I = 9 dB is acceptable by the system and yields a possible re-use corresponding to a 9-cell cluster (3-cell re-use patterns with three sectors per cell).

The receiver sensitivity, similar to that of existing analogue systems, allows an average transmit power about 9 dB lower than current analogue systems, given the same requirements for maximum cell sizes and the same RF device choices.

## 4.5	Time-slots and TDMA frames

A burst containing 148 bits, corresponding to 114 coded bits, is sent within a time-slot duration of 0.577 ms. A set of eight time-slots is used to build up a TDMA frame containing eight basic physical channels. Each physical channel has logical channels mapped on it, i.e. the traffic channels and control channels.

The useful information is distributed in the time-slots in a manner allowing recovery from total erasure of some time-slots.

Two multiframe structures are defined: one consisting of 26 TDMA frames (recurrence interval of 120 ms) for traffic channels and their associated control channels, and one for the other control channels comprising 51 TDMA frames (recurrence interval of 236 ms).

## 4.6	Traffic channels

### 4.6.1	Full and half-rate traffic channels

The system is able to support both full and half-rate traffic channels, corresponding respectively to the gross bit rates of 22.8 and 11.4 kbit/s. The half-rate channel is obtained by the use of only half of the time-slots used by the full-rate channel. A carrier therefore provides up to 8 full-rate or 16 half-rate traffic channels (or a combination of both) with their respective associated control channels.

### 4.6.2	Speech traffic channels

The full-rate speech codec, and the associated error correction and detection mechanisms have been defined in the GSM standard. Speech frames of 20 ms, each comprising 260 bits, provide a net bit rate of 13 kbit/s. The coding method, “regular pulse excited linear prediction coding with long-term prediction (RPE-LTP)”, has been designed to be robust in the presence of transmission errors, and to offer a quality close to that of the PSTN while using a limited bit rate.

Error correction (consisting of a 1/2 rate convolutional code) and interleaving schemes, to selectively protect the most important bits within the speech frame (70% of the bits) have been specified. Furthermore, an error detection mechanism has been included, associated with extrapolation techniques which have been described and/or recommended, in order to minimize the impairment of speech quality if speech frames are not correctly received. The usage of speech activity detectors has also been specified in the GSM system. Details can be found in the GSM standard.

### 4.6.3	Data traffic channels

Transparent and non-transparent data services of up to 9.6 kbit/s are supported by different rate adaptations, channel coding and interleaving schemes, on full-rate and/or half-rate channels.

Unrestricted digital bearer services with a net bit rate of 12 kbit/s are also supported.

### 4.6.4	Discontinuous transmission

All traffic channels may use, when possible, discontinuous transmission (i.e. the transmitter is silent when no relevant information is to be transmitted). In the case of speech this is possible due to the specification of speech activity detectors.

This feature, combined with frequency hopping which introduces interferer diversity, is expected to increase the system capacity. It will also prolong battery life in hand-held portable stations.

## 4.7	Control channels

Three categories of control channels are defined: broadcast, common and dedicated.

### 4.7.1	Broadcast channels

Broadcast channels are divided into frequency correction, synchronization and broadcast control channels.

### 4.7.2	Common control channels

Common control channels are divided into paging, random access and access grant channels.

### 4.7.3	Dedicated control channels

Dedicated control channels are divided into slow and fast associated control channels, as well as stand-alone dedicated control channels with their associated control channels. Also under this category a cell broadcast channel is defined to carry short messages service cell broadcast.

Short message service, mobile terminated and mobile originated point-to-point calls, are supported by the stand-alone dedicated control channel or the slow associated control channel.

# 5.	Operational characteristics

## 5.1	Cell selection

Whilst in idle mode the mobile station is camped on a cell from which it can reliably decode downlink data, and with which it has a high probability of communicating on the uplink.

The cell selection is based on path loss criteria. If these criteria are not met, or if the mobile station fails to decode paging blocks or fails to access the uplink, the mobile station starts to re-select.

## 5.2	Location updating (roaming)

Roaming is performed in accordance with Recommendation ITU-R M.624.

The mobile station evaluates the received signal and initiates the location updating procedure when necessary.

Roaming is possible between mobile service switching centres (MSCs) and between countries.

## 5.3	Communication protocols

The communication protocols are layered according to the OSI model and are specified in the GSM Recommendations.

The network layer is divided into three sub-layers: call control, mobile management and radio resource management.

The link layer is based on LAPD protocols and makes use of the control channels. Messages between link layer peer entities are source coded into 23 octets, i.e. 184 bits.

## 5.4	Call setup

### 5.4.1	Mobile originated call setup

The procedure starts on the random access channel to set up a radio resource. Then authentication is done on the mobile management sub-layer. After ciphering and assignment has been confirmed, the call setup is confirmed on the call control sub-layer.

### 5.4.2	Mobile terminated call setup

After paging from the network the same procedure as § 5.4.1 is followed.

## 5.5	Handover

Handover is required to maintain a call in progress as a mobile passes from one cell coverage area to another and may also be employed to meet network management requirements, i.e. relief of congestion (network-directed handover).

The handover is done either from a channel on one cell to another channel on another cell, or between channels of the same cell.

The handover strategy employed by the network for radio link control determines the handover decision that will be made based on the measurement results reported by the mobile and base stations and the various parameters set for each cell. The exact handover strategies are determined by the network operator.

A procedure is implemented in the mobile station which monitors the downlink signal level and quality from its serving cell, the downlink signal level and the colour code of surrounding cells.

A procedure is implemented in the base station which monitors the uplink signal level and quality from each mobile station being served by the cell.

These radio link measurements are also used for RF power control.

Handover is possible between location areas and between different MSCs belonging to the same PLMN.

## 5.6	Radio link failure

The criteria for determining radio link failure are specified in order to ensure that calls which fail, either from loss of radio coverage or unacceptable interference, are satisfactorily handled by the network. Radio link failure results in either call re-establishment or release of the call in progress.

The criterion for determining radio link failure in the mobile station is based on the success rate of decoding messages on the downlink slow associated control channel.

## 5.7	Signalling between base station and MSC

The signalling follows a layered approach similar to ISDN in accordance with GSM Recommendations.

## 5.8	ISDN, PDN and PSTN interfaces

These interfaces are in accordance with ITU-T Recommendations Q.700 and Q.1000 Series.

## 5.9	Numbering plan

The numbering plan is in accordance with ITU-T Recommendations E.164, E.212 and E.213.

## 5.10	Signalling between MSCs

The signalling between MSCs uses ITU-T Signalling System No. 7 (ITU-T Recommendations E.214, Q.700 Series and GSM 09.02 or ITU-T Recommendation Q.1051).

REFERENCES

ETSI GSM specifications. European Telecommunications Standards Institute, Sophia Antipolis, F-06291 Valbonne Cedex, France.

ANNEX  3

General description of the North American TDMA digital cellular
public land mobile telecommunications system

# 1.	General description

## 1.1	Objectives

The North American digital cellular system for the public land mobile telecommunications system (PLMTS) is designed to provide digital voice and data services and to meet a significantly growing capacity requirement. The standard is suitable for new systems and is also compatible with the existing advanced mobile phone service (AMPS) system. RF carrier spacing is compatible with the AMPS spacing and thus both types can co-exist in the same radio environment.

## 1.2	Evolution strategy

Because of the compatibility of the RF signals, the system provides operators with a smooth evolutionary path for the introduction of digital services and additional traffic capacity to their existing PLMTS. The digital standard can be incorporated into existing networks to allow both digital and analogue traffic. Users with dual-mode terminals can receive service from operators who have added a digital capability, and from those operators who have only analogue facilities. Operators need not install digital equipment until their traffic growth warrants additional facilities. Because the digital and analogue traffic share the same band and control channel, the dual-mode terminals can economically share many radio and control components. Section 2 of this Annex outlines the technical features of the system. As the new standard is compatible with the existing AMPS system, only digital features are highlighted here. For a further description of the AMPS refer to Report ITU-R M.742.

For details of the digital cellular standard and dual-mode operation see EIA/TIA [1992; 1993a and b].

## 1.3	Capacity improvement

The system uses a linear modulation technique and low bit rate speech coding to achieve high spectral efficiency. This combination enables each 30 kHz radio frequency carrier to handle three traffic channels. Six traffic channels can be accommodated in the future with half-rate codecs. The linear modulation technique also operates at a lower carrier-to-interference ratio than the present analogue system. Capacity can be further increased by the use of improved frequency re-use patterns, cell sectoring and planning techniques. Together, these techniques provide greatly increased traffic capacity as Table 2 indicates.

The following assumptions were used in the preparation of the table:

–	a total of 25 MHz is available to the operator;

–	a 2% blocking probability is acceptable;

–	a lower C/I threshold is used for the digital system; and

–	the same ratio of traffic to signalling channels is used for both the analogue and digital systems.

TABLE  2

Capacity improvements in the North American digital cellular system
compared with the present analogue system

| Evolution stage |  |  |  |  |
| --- | --- | --- | --- | --- |

| Access technique | Traffic channels per radio-frequency carrier | Cell re-use pattern | Sectors per cell | Number of traffic channels | Erlang per cell | Improvement factor relative to (7 ´ 3)(1) | Improvement factor relative to (4 ´ 6)(1) |
| --- | --- | --- | --- | --- | --- | --- | --- |

| FDMA | 1 | 7 | 3 | - 395 | - 37 | - 1.0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - FDMA | 1 | 4 | 6 | - 395 | - 60 | - 1.6 | - 1.0 |
| - TDMA | 3 | 7 | 3 | - 1 185 | - 138 | - 3.7 |  |
| - TDMA | 3 | 4 | 6 | - 1 185 | - 236 | - 6.4 | - 3.9 |
| - TDMA | 6 | 7 | 3 | - 2 370 | - 302 | - 8.7 |  |
| - TDMA | 6 | 4 | 6 | - 2 370 | - 522 | - 14.1 | - 8.7 |
| - TDMA | 6 | 4 | 3 | - 2 370 | - 550 | - 14.9 | - 9.2 |
| - TDMA | 6 | 3 | 3 | - 2 370 | - 746 | - 21.3 | - 12.4 |

- (1) (m ´ n) m cell re-use pattern with n sectors.

# 2.	Technical outline

## 2.1	RF interface requirements

–	Modulation: p/4 differentially encoded QPSK.

The information is differentially encoded; symbols are transmitted as changes in phase rather than absolute phases.

Grey code is used in the symbol mapping; two di-bit symbols corresponding to adjacent signal phases differ only in a single bit.

## 2.2	Typical cell radius: 0.5-20 km

It is possible to have three and six sector cells using directional antennas with both digital and analogue channels for rural and urban areas.

## 2.3	Channel coding

–	Rate one-half convolutional code;

–	two levels of error protection;

–	cyclic redundancy code (CRC) to protect the most important bits for speech.

## 2.4	Time-slots and TDMA frames

Time-slots are arranged in the following order: transmit-receive-idle. The idle time-slots may be used for measuring and channel monitoring purposes.

The numbering scheme is one through six. Currently, six synchronization words are used to allow up to six users to share one TDMA channel.

Provisions are made to allow other conditions of transmission under base station control to support more users and different services on one TDMA channel.

## 2.5	Traffic channel structure

–	Speech: supports full-rate and half-rate speech codecs.

For a full-rate speech codec, 13 kbit/s is allocated to speech coding and forward correction. It is possible to support different speech codecs of different rates at the base station.

–	Data: it provides data services of 2.4, 4.8, 9.6 kbit/s. Higher rates will also be specified.

## 2.6	Control channel structure

–	Common control channel:

AMPS common control channel [EIA/TIA, 1989] will be shared by both digital and analogue mobile stations.

Additional messages for mobile stations in digital mode will be handled in the digital traffic channels to support the higher dual-mode cellular system capacity.

Digital control channel also will be specified.

–	Fast and slow control channels: these are dedicated in each channel for each user.

The supervisory and other control signals can be handled by fast and slow associated control channels.

The digital voice colour code will be a separate field to provide the function equivalent to the analogue supervisory audio tone (SAT), used to detect interference.

## 2.7	Handover

–	Inter-system and intra-system handovers are specified.

–	Handovers between digital and analogue channels are possible.

–	Mobile assisted handover:

This provides the ability for the mobile stations in the digital mode to measure and report both the received signal strength and channel quality over the current digital connection as well as the received signal strength on other channels as requested by the base station.

## 2.8	System architecture

–	Communication protocol:

The network communication protocol reference model is designed according to the OSI model.

–	Interfaces:

The interfaces between system functional blocks are modelled according to ITU-T Q.1000 Series Recommendations.

# 3.	Cellular inter-system operation

This section describes techniques that support cellular operation involving two or more MSCs. For details of the procedures for inter-system operation for the North American cellular system [EIA/TIA, 1991a, b, c, d and e; 1992; 1993a and b].

## 3.1	Functional overview

This section identifies the services that require inter-system operation and presents a network reference model that shows the functional entities of the system.

### 3.1.1	Cellular inter-system services

Currently, there are two major categories of inter-system services:

a)	Inter-system handover

Inter-system handover refers to the general provisions by which a call in progress on a radio channel under the control of a current serving MSC (mobile switching centre) may be automatically transferred to a different radio channel under the control of another MSC without interruption to the ongoing communication.

b)	Automatic roaming

Automatic roaming refers to the general provisions for automatically providing cellular services to the  cellular subscriber stations (CSSs) which are operating outside their home service area but within the aggregate service area of all participating MSCs. In the most general implementation these provisions include:

–	timely identification of the current serving MSC;

–	automatic service qualification of the roaming CSSs, including credit validation, feature privileges, and feature control;

–	automatic call delivery to the roaming CSSs.

The requirement that the procedures by which these operations are implemented be automatic is implicit in the intent of the TIA IS-41 Recommendations.

### 3.1.2	Network reference model

Figure 3 presents the functional entities and the associated interface reference points that may logically comprise a cellular network. The model, as defined herein, is intended to provide a level of abstraction that may facilitate the specification of messages and protocols within IS-41. As such, implementations may vary with respect to how the functional entities are distributed among various physical units. In cases where functional entities are combined in the same physical equipment the interface reference points become internal, and need not adhere to interface standards.

## 3.2	Inter-system handover

This section presents an overview for handover between two different MSCs. The term “inter-system handover” is used to describe a handover between two different MSCs. To perform inter-system handover means to switch a CSS telephone call that is in progress on the “serving” MSC, to a different MSC. The CSS is instructed to re-tune to a frequency that is controlled by a different MSC.

During a CSS telephone call, the MSC controlling the call is called the “serving” system. A neighbour system is called a “candidate” system if the neighbour is being considered as a possible server in a locate/handover sequence. A neighbour system is called a “target” system if the neighbour has been chosen by the serving system as the next serving system, but the handover has not yet occurred.

Inter-system handover is used when two MSCs are adjacent to one another and the neighbour system is better able to serve a CSS than the current serving system. The neighbour system and the serving system must have contiguous serving areas. Inter-system handover is triggered by the normal locate/handover sequence. In the serving system, the neighbour system appears as a “neighbour” to the serving system cell sites that are adjacent to the neighbour system.

When a cell site initiates the locate/handover sequence, and a neighbour system is to be used in the sequence, the messages and procedures presented in EIA/TIA [1991e] allow the two systems to communicate.

FIGURE 3...[D03] = 20 CM

## 3.3	Automatic roaming

This section presents an overview for automatic roaming. Automatic roaming refers to the process whereby a cellular system responds to the presence of a roamer by informing the home system of its presence, determines the roamer’s credit-worthiness and if required obtains the roamer’s service profile, permits a roamer to originate calls and operate subscribed features, and provides the requisite information for routing of calls to the roaming subscriber in question.

Automatic roaming involves the following automatic mechanisms:

–	making the identity of the current serving, or visited, system known to the home system;

–	establishing financial responsibility for the roaming subscriber;

–	establishing a valid roamer service profile in a visited system;

–	providing for call delivery to the roaming subscriber.

The transactions are automatic in the sense that they operate in a manner that requires minimal intervention on the part of the cellular subscriber and/or parties attempting to place a call to a cellular subscriber.

## 3.4	Data communications

The inter-system data communications required for inter-system handover and automatic roaming use the protocol of ITU-T Recommendation X.25 or the Message Transfer Part of the ITU-T Signalling System No. 7 protocol.

REFERENCES

EIA/TIA [September, 1989] Cellular System: Mobile Station – Land Station Compatibility Specification. EIA/TIA-553. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [December, 1991a] Cellular Radiocommunications Inter-system Operations: Functional Overview. EIA/TIA IS-41.1-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [December, 1991b] Cellular Radiocommunications Inter-system Operations: Inter-system Handover. EIA/TIA IS-41.2-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [December, 1991c] Cellular Radiocommunications Inter-system Operations: Automatic Roaming. EIA/TIA IS-41.3-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [December, 1991d] Cellular Radiocommunications Inter-system Operations: Operations, Administration and Maintenance. EIA/TIA IS-41.4-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [December, 1991e] Cellular Radiocommunications Inter-system Operations: Data Communications. EIA/TIA IS-41.5-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [April, 1992] Cellular System Dual Mode Mobile Station – Base Station Compatibility Standard. EIA/TIA IS-54-B. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [September, 1993a] Recommended Minimum Performance Standards for 800 MHz Dual Mode Mobile Stations. EIA/TIA IS-55-A. Electronic Industries Association/Telecommunications Industry Association, United States of America.

EIA/TIA [October, 1993b] Recommended Minimum Performance Standards for 800 MHz Base Stations Supporting Dual Mode Mobile Stations. EIA/TIA IS-56-A. Electronic Industries Association/Telecommunications Industry Association, United States of America.

ANNEX  4

General description of the Japanese personal digital cellular (PDC)
land mobile telecommunication system

# 1.	Introduction

The Japanese personal digital cellular (PDC) PLMTS is specified to provide various services and to accommodate a great number of subscribers.

The system is applicable to both the 800/900 MHz and the 1.5 GHz bands and supports data, facsimile and ISDN services. To realize efficient frequency utilization, the RF carrier spacing is 25 kHz in accordance with the existing analogue standard [RCR, 1992].

# 2.	Overview of the system

Figure 4 shows an example of the digital mobile communications network architecture and area configuration.

The digital mobile communications network is connected to the PSTN and another PLMN. It is also connected to the ISDN by ISDN user part (ISUP) and to the packet switched public data network (PSPDN) via the ISDN.

GMSC: gateway mobile services switching centre. This provides a gateway function between the fixed network and the mobile network.

VMSC: visitor mobile services switching centre. This provides a call connection capability both for mobile originated/terminated call setups and supplementary services.

HLR: home location register. This stores subscriber data and the location of home subscribers, e.g. the mobile station identification number and the  area where the subscribers belong are registered.

GLR: gateway location register. This is provided to temporarily store the data of a terminal moving in from other networks. This GLR complements the HLR in which the regular mobile communications service subscriber information is stored.

BS: base station. This provides the radio channel management functions.

MS: mobile station. This is an interface terminal and provides multi-service functions to the mobile subscriber.

# 3.	Main features [RCR, 1992]

## 3.1	RF interface requirements

–	Channel spacing:	25 kHz interleaved channel spacing, 50 kHz channel spacing;

–	modulation:	p/4 shifted QPSK (roll-off factor; 0.5, root-Nyquist filter);

–	access method:	TDMA:

–	3 time-slots/25 kHz (for full-rate);

–	6 time-slots/25 kHz (for half-rate);

–	transmission bit rate:	42 kbit/s.

## 3.2	Cell structure and carrier re-use

–	Typical cell radius: 0.5-20 km (up to 60 km by time alignment);

–	sector cell structure using directional antennas.

FIGURE 4...[D04] = PAGE PLAINE

## 3.3	Channel coding (speech traffic channel)

–	Rate 9/17 convolutional code;

–	two levels of error protection;

–	cyclic redundancy code (CRC) to protect the most important bits for speech.

## 3.4	Time-slots

–	Three for full-rate, six for half-rate.

## 3.5	Traffic channels

–	Speech:	supports full-rate and half-rate speech codecs.

–	full-rate speech codecs of 6.7 kbit/s;

–	up to 11.2 kbit/s are allocated to speech coding and forward error correction;

–	half-rate speech codecs will also be specified.

–	Data and other services:

–	data services of G3 facsimile, V.42 Annex, and higher rates will also be specified;

–	facsimile;

–	ISDN sub-rate (8 kbit/s).

## 3.6	Control channels

–	Broadcast control channels (BCCH): control channels for broadcast messages;

–	common control channels (CCCH): control channels for signalling, such as paging;

–	associated control channels (ACCH): slow ACCH and fast ACCH.

## 3.7	Cell selection

–	While in idle mode, the mobile station monitors the downlink signal level and colour code from its serving cell and surrounding cells.

## 3.8	Handover

–	Inter-system and intra-system handovers are specified;

–	mobile assisted handover:

–	this provides the ability for the mobile stations to measure and report both the received signal strength and channel quality over the current connection as well as the received signal strength on other channels, as requested by the base station.

## 3.9	Roaming

–	In accordance with Recommendation ITU-R M.624;

–	the mobile station evaluates the received signal and coding and initiates the location updating procedure when necessary;

–	roaming is possible between MSCs and between systems.

## 3.10	System architecture

–	Communication protocol: the network communication protocol reference model is designed according to the OSI model;

–	interfaces: the interfaces between system function blocks are designed according to ITU-T Recommendations.

## 3.11	Networking

–	ISDN and PSTN interfaces: in accordance with ITU-T Recommendations Q.700 Series;

–	numbering plan: in accordance with ITU-T Recommendations E.164, E.212 and E.213.

REFERENCES

RCR [1992] Personal digital cellular telecommunication system. Research and Development Centre for Radio Systems, Japan. RCR standard STD-27B/1992.12.10.

ANNEX 5

General description of the North American digital CDMA
wideband spread spectrum cellular system

# 1.	Introduction

## 1.1	Objectives

The North American CDMA wideband digital cellular system for public land mobile telecommunications system (PLMTS) is designed to provide digital voice and data services and to meet a significantly growing capacity requirement. The standard is suitable for new systems and is also compatible with the existing advanced mobile phone service (AMPS) system. RF carrier spacing for each CDMA channel is 1.25 MHz which is obtained by clearing the appropriate number of AMPS channels. In this manner, both types of service can co-exist in the same environment. Capacities are achieved with CDMA that exceed ten times that achievable with analogue AMPS in the same bandwidth.

## 1.2	Compatibility considerations

Because of the compatibility of the RF signals, the system provides operators with a smooth transition for the introduction of CDMA digital services and additional traffic capacity to their existing PLMTS. The CDMA digital standard can be incorporated into existing networks to allow both digital and analogue traffic. Users with dual-mode terminals can receive service from operators who have added a digital capability, and from those operators who have only analogue facilities. Operators need only install digital equipment to add CDMA channels when their traffic growth requires it or when they desire to add specialized services. Section 2 of this Annex outlines some of the technical features of the system. System specifications are summarized in Table 1. As the new standard is compatible with the existing AMPS system, only the digital features are highlighted here. For a further description of the AMPS, refer to Report ITU-R M.742.

The major distinction between CDMA wideband spread spectrum cellular and the narrow-band technologies is that in CDMA many signals share the same bandwidth. The processing gain inherent in de-spreading a single user’s signal overcomes the interference which is the superposition of the signals of the other users. The capacity limit of the system is reached when the interference level is large enough that the signal-to-noise ratio of one user is at the minimum acceptable value. Very high capacity is achieved by techniques that include power control, coding, variable rate vocoding, rake receivers able to combine multipath components, interleaving, etc.

## 1.3	Functional overview

One functional implementation of the CDMA wideband spread spectrum cellular system is the same as described in § 3.1 of Annex 3, including the accompanying Fig. 3.

For details of the CDMA wideband digital cellular standard, see TIA/EIA [1993; 1994]; [1], [2].

# 2.	Technical outline

## 2.1	Outbound link

### 2.1.1	RF interface

#### 2.1.1.1	Data modulation

Data modulation on the outbound link is coherent BPSK at a symbol rate of 19 200 symbols/s.

#### 2.1.1.2	Spreading modulation

The outbound link symbol stream is added, modulo 2, to an orthogonal cover sequence used for channelization, and then QPSK spread by a 1.2288 MHz pseudo-noise sequence with a period of 80/3 ms (32 768 chips). The radiated waveform is tightly band-limited to a bandwidth of 1.25 MHz. Distant base stations are distinguished from one another by the relative phases of their short code spreading sequences.

### 2.1.2	Channel structure

The outbound link is channelized by adding a cover sequence to each channel. The cover sequences have a period equal to the symbol duration, and are mutually orthogonal. The orthogonality of the cover sequences permits separation of 64 logical channels at the mobile station receiver. There are three types of overhead channels: pilot, sync, and paging. The remaining channels are available for traffic.

#### 2.1.2.1	Pilot channel

The pilot channel is spread but otherwise unmodulated. It serves as a phase reference for coherent demodulation of the other 63 channels. It also is used as a search target for acquisition of new base stations as the mobile stations move from one coverage area to another.

#### 2.1.2.2	Sync channel

The sync channel carries information which permits the mobile stations to determine system time and pilot offset of the base station in preparation for system access. The data rate on the sync channel is 1 200 bit/s.

#### 2.1.2.3	Paging channels

Each base station has one or more paging channels. The paging channels broadcast the identities of mobile stations for which there are incoming calls. Mobiles that hear their identifications on the paging channel respond via an access channel on the inbound link. The data rate on the paging channels is 4 800 or 9 600 bit/s, at the discretion of the operator.

#### 2.1.2.4	Traffic channels

Traffic channels carry coded speech, or other traffic. Variable-rate traffic on the outbound link reduces the mutual interference between channels. Data rates of 9 600, 4 800, 2 400, and 1 200 bit/s are available frame-by-frame on the traffic channel. The rate variation is achieved by repetition of symbols, with the energy per bit kept approximately constant.

### 2.1.3	Coding and interleaving

The outbound link is convolutionally coded and block interleaved. The coding is rate 1/2 with a constraint length of 9. The interleaving span is 26.666... ms on the sync channel and 20 ms on the paging and traffic channels.

All traffic channel frames transmitted at 4 800 and 9 600 bit/s are protected by a cyclic redundancy code (CRC), which can be used by the receiver to detect decoding failures.

Sync and paging channel messages are protected by 30-bit CRCs. Traffic channel messages are protected by 16-bit CRCs. These CRCs are in addition to the frame CRCs on the traffic channel.

### 2.1.4	Reverse-link power control sub-channel

The forward traffic channels are punctured at a 800 bit/s, with 2 symbols per power control bit, in order to do independent closed-loop power control of each mobile station. The power control sub-channel is not coded.

## 2.2	Inbound link

### 2.2.1	RF interface

#### 2.2.1.1	Data modulation

Data modulation on the inbound link is 64-ary orthogonal, using Walsh codes. The symbol rate is 4 800 symbols per second.

#### 2.2.1.2	Spreading modulation

The outbound link symbol stream is added, modulo 2, to a 1.2288 MHz cover sequence used for channelization, and then QPSK spread by a 1 2288 MHz pseudo-noise sequence with a period of 80/3 ms (32 768 chips). This is the same sequence as the short code used in the outbound link. The inbound cover sequence (“long code”) is a unique phase of a 42-bit maximal length linear feedback shift register sequence. The radiated waveform is tightly band-limited to a bandwidth of 1.25 MHz. Distant base stations are distinguished from one another by the relative phases of their short code spreading sequences.

### 2.2.2	Channel structure

Channelization of the inbound link is accomplished by assigning each mobile station a unique phase of the long code to use for covering its traffic transmissions. There are also pre-defined phases for common access channels. Unlike the outbound link, the inbound cover sequences are not orthogonal.

#### 2.2.2.1	Access channels

Access channels are pre-defined long code phases. They are used by the mobile stations to get the attention of the base station, either to respond to a page, to originate a call, or to perform a registration. The access channel data rate is 4 800 bit/s.

#### 2.2.2.2	Traffic channels

Traffic channels carry coded speech, or other traffic. Variable rate traffic on the inbound link reduces the mutual interference between channels. Data rates of 9 600, 4 800, 2 400, and 1 200 bit/s are available frame-by-frame on traffic channels. The rate variation is achieved by burst transmission. The transmitter power within the bursts remains constant with rate. The number of bursts transmitted within each frame depends on the rate.

### 2.2.3	Coding and interleaving

Coding on the inbound link is rate 1/3, constraint length 9. It is always block interleaved with a span of 20 ms.

All traffic channel frames transmitted at 4 800 and 9 600 bit/s are protected by a CRC, which can be used by the receiver to detect decoding failures.

Access channel messages are protected by 30-bit CRCs. Traffic channel messages are protected by 16-bit CRCs. These CRCs are in addition to the frame CRCs on the traffic channel.

### 2.2.4	Reverse link power control

The power transmitted by the mobile station is regulated to be near the minimum required for adequate error rate performance. The radiated power is determined by the received base station power, and by a closed-loop correction radiated by the base station at an 800 Hz rate.

## 2.3	Associated signalling channel

Signalling between mobile and base, after the transition to traffic, is accomplished by “blank-and-burst” or “dim-and-burst” in the traffic channel. Blank-and-burst pre-empts one or more traffic frames and substitutes the signalling message. Dim-and-burst is similar, except the speech coder is informed that it may not use full-rate. A full-rate frame, consisting of the half-rate or lower rate speech data and a half frame of signalling data. The dim-and-burst method has less impact on voice quality. In both cases the speech codec in the receiver knows that the frame was pre-empted, and can take mitigating actions, possibly different than those it would take in the event of a channel error.

## 2.4	Handover

### 2.4.1	Soft handover

The system supports seamless soft handover. This is accomplished by two or more multiple base stations radiating the outbound traffic for the mobile. The mobile, by measurement of pilot strengths from the various participating base stations, selects two or more signals to combine. Such soft combining improves coverage near the handover region, making the handover nearly undetectable by the users.

### 2.4.2	Hard handover

Hard handover is supported for changing the 1.25 MHz RF channel, and for other circumstances where soft handover is not possible. Changes of channel might be needed, for example, to move between public and private networks, or to avoid interference.

### 2.4.3	Digital to analogue handover

Handover from CDMA digital service to AMPS service is supported.

## 2.5	Registration and mobility management

Mobility management is supported by nine operator-selectable registration mechanisms. The nine types of registration are:

–	Power up: the mobile registers when it is turned on.

–	Power down: the mobile registers when it is turned off.

–	Elapsed time: the mobile station registers when a timer expires.

–	Distance moved: the mobile station registers when the distance between the current base station and the last base station in which it last registered exceeds a threshold.

–	Zone changes: the mobile station registers when it enters a new zone.

–	Parameter change: the mobile station registers when certain of its stored parameters change.

–	Direct order: the mobile station registers when ordered to do so.

–	Traffic channel: the base station can interrogate a mobile station that has been assigned to a traffic channel, thereby accomplishing a registration.

–	Implicit: any origination or page response constitutes an implied registration.

## 2.6	Security features

Both global and unique challenge-response authentication procedures are available in the standard to prevent various types of over-the-air service fraud. Certain sensitive message fields also can be encrypted. This can be used to protect things like credit card numbers entered by subscribers, for example.

REFERENCES

TIA/EIA [July, 1993] Mobile Station-Base Station Compatibility Standard for Dual-Mode Wideband Spread Spectrum Cellular System. TIA/EIA IS-95. Telecommunications Industry Association/Electronic Industries Association, United States of America.

TIA/EIA [April, 1994] Speech Service Option Standard for Wideband Spread Spectrum Digital Cellular System. PN-3119 Draft, to be published as TIA/EIA IS-96. Telecommunications Industry Association/Electronic Industries Association, United States of America.

[1]	“Recommended Minimum Performance Standards for Base Stations Supporting Dual-Mode Wideband Spread Spectrum Cellular Mobile Stations”, PN-3120 Draft, to be published as TIA/EIA IS-97.

[2]	“Recommended Minimum Performance Standards for Mobile Stations Supporting Dual-Mode Wideband Spread Spectrum Cellular Base Stations”, PN-3121 Draft, to be published as TIA/EIA IS-98.

_________________