RECOMMENDATION  ITU-R  M.1085*

TECHNICAL  AND  OPERATIONAL  CHARACTERISTICS  OF  WIND  PROFILER  RADARS

(Question ITU-R 102/8)

(1994)

Rec. ITU-R M.1085

The ITU Radiocommunication Assembly,

considering

a)	Recommendation No. 621 of the World Administrative Radio Conference for Dealing with Frequency Allocations in certains Parts of the Spectrum (Malaga-Torremolinos, 1992);

b)	that wind profiler radars are important meteorological systems used to measure wind direction and speed as a function of altitude;

c)	that many administrations plan to deploy wind profiler radars in operational networks in order to improve meteorological predictions and warnings, and support studies of the climate and increase the safety of navigation;

d)	the need for frequency bands in the vicinity of 50, 400, and 1 000 MHz to permit the full performance capability of wind profiler radar operations, as requested by the Secretary-General of the World Meteorological Organization (WMO);

e)	that wind profilers operating near 1 000 MHz may be installed at fixed locations or may be transportable, to support a variety of requirements including environmental emergencies, pollution monitoring, etc.;

f)	that above a frequency limit around 1 300 MHz the wind profiler radars performance decreases significantly;

g)	that the wind profiler radars could be designed to operate on assigned frequencies in a range up to ± 1% of the centre frequency;

h)	that wind profiler radars may have to share spectrum with other systems both current and future;

j)	that it would be desirable to have a limited number of frequencies authorized worldwide, in order to minimize research and development investment in the design of components;

k)	that technical standards would enhance compatibility with other systems within the same band. An example of a 400 MHz wind profiler radar standard is given in Annex 1;

l)	that studies are continuing with regard to quantitative limits on unwanted emissions for wind profiler radars,

recommends

1.	that the power to the antenna line should be limited to that necessary to obtain data at the maximum altitude for which the profiler was designed.

Table 1 provides representative values for operational equipment;

TABLE  1

| Bands (MHz) | Typical range (km)(1) | Peak power (kW) | Typical antenna size (m2) |
| --- | --- | --- | --- |

| 50 | 2-30 | Up to 50 | 2 500-10 000 |
| --- | --- | --- | --- |

| 400 | 0.5-16 | Up to 40 | 50-100 |
| --- | --- | --- | --- |

| 1 000 | 0.1-3 | Up to 5 | 3-10 |
| --- | --- | --- | --- |

(1)	The range for a given frequency band depends mainly on the product: (mean power) ´ (antenna effective surface).

2.	that the occupied bandwidth should be as close to the necessary bandwidth as is technically and economically feasible to provide the required range resolution, noting that reduced values of resolution are generally acceptable at higher altitudes. Table 2 provides representative values;

TABLE  2

| Bands around (MHz) | Necessary bandwith (MHz) | Height resolution (m) |
| --- | --- | --- |

| 50 and 400 | - 0.7 | - 1 000 |
| --- | --- | --- |

| 50 and 400 | - 1.0 | - 500 |
| --- | --- | --- |

| 50 and 400 | - 1.6 | - 250 |
| --- | --- | --- |

| 1 000 | - 3,0 | - 150 |
| --- | --- | --- |

| 1 000 | - 13,0 | - 30 |
| --- | --- | --- |

3.	that the unwanted emissions from wind profiler radars should be reduced as much as technically and economically feasible;

4.	that the antenna radiation pattern should minimize the levels of the side lobes, especially those at or near the horizon.

Table 3 provides representative antenna side-lobe gains for the specified angles above the horizon in the 50 and 400 MHz bands.

TABLE  3

| Angle above the horizon (degrees) | Antenna side-lobe gain (dBi) |
| --- | --- |

|  | Median | Maximum |
| --- | --- | --- |

| 0-5 | - –20 | - –8 |
| --- | --- | --- |

| 5-45 | - –5 | - 7 |
| --- | --- | --- |

| > 45 | - 0 | - 12 |
| --- | --- | --- |

In the 1 000 MHz band, the preceding antenna side-lobe gains may be increased by 6 dB depending on the type of antenna;

5.	that Administrations should develop appropriate frequency-distance (F/D) curves in accordance with Recommendation ITU-R SM.337 for specific wind profiler radar designs sharing with other systems;

6.	that the selection of wind profiler radar locations should take advantage of terrain and siting configuration to minimize the possibility of interaction with other systems; additional enhancements (e.g. fences, berms) and antenna orientation may improve compatibility;

7.	that time-sharing should not be considered as a technically adequate means of protecting sensitive safety-of-life systems such as the COSPAS-SARSAT;

8.	that in the vicinity of 50 MHz, bands should be chosen, where compatibility is possible, taking into account the necessary protection;

9.	that frequency assignments in the radiolocation bands around 400 and 1 000 MHz be considered, taking into account the necessary protection.

ANNEX  1

Example of a 400 MHz wind profiler radar standard

# 1.	Introduction

The following technical standard used by the United States Federal Government takes into account peak power and frequency of operation. This technical standard is given as an example for a non-FM pulsed wind profiler radar (WPR) in the band 440-450 MHz, specifically operating on the frequency 449 MHz. This standard is contained in the National Telecommunications and Information Administration (NTIA), Manual of Regulations and Procedures for Federal Radio-Frequency Management.

# 2.	Symbols and terms defined

B :		emission bandwidth (MHz)

F0 :		operating frequency (MHz). For non-FM pulse radars, the peak of the power spectrum

N :		total number of chips (sub-pulses) contained in the pulse. (N = 1 for non-FM and FM pulse radars)

PG :		processing gain (dB). For a non-FM pulsed radar PG = 10 log N

Pp :		peak power (dBm)

PRR :	pulse repetition rate (pulses/s)

Pt :		maximum spectral power density (dB(mW/kHz))

t :		emitted pulse duration (ms) at 50% amplitude (voltage) points. For coded pulses, the pulse duration is the interval between 50% amplitude points of one chip (sub-pulse). The 100% amplitude is the nominal flat top level of the pulse (see Fig.1)

tr :		emitted pulse rise time (ms) from the 10% to 90% amplitude points on the leading edge (see Fig. 1). For coded pulses it is the rise time of a sub-pulse; if the sub-pulse rise time is not discernible, assume that it is 40% of the time to switch from one phase or sub-pulse to the next

S :		roll-off slope (dB/decade)

FIGURE 1/M.1085...[D01] = 8 CM

Commensurate:	in general, the term “commensurate” is approximately equivalent to the term “equal” and it is used several times in § 3.5 (WPR receiver). Furthermore, in the sentence “the –60 dB receiver bandwidth shall be commensurate with the –60 dB emission bandwidth”, it is intended for the receiver to be sufficiently broadband to pass all of the desired signal, but not so wide as to render the receiver vulnerable to adjacent channel interference.

Roll-off slope:	defines the slopes in dB per decade of the emission at and beyond the 40 dB point. It is the upper limit of the permissible energy level beyond that point.

# 3.	Criteria

## 3.1	Emission bandwidth

The emission bandwidth for WPRs at the antenna input shall not exceed the following limits:

3.1.1	For non-FM pulse radars (including coded pulse radars):

B (– 40 dB) = 6.2 / (trt)½ or 64 / t, whichever is less.

3.1.2	For wind profiler radars, an operational justification shall be provided if the pulse rise time, tr, is less than 0.01 ms.

Note 1 – The –20 dB bandwidth for WPRs operating at 449 MHz shall not exceed 2 MHz.

## 3.2	Emission levels

WPR emission levels at the antenna input shall be no greater than the values obtainable from the curve in Fig. 2. At the frequencies ± B(–40 dB) / 2 displaced from F0, the level shall be at least 40 dB below the maximum value. At and beyond the frequencies ± B(–X dB) / 2 from F0, the level shall be at least the dB value below the maximum spectral power density given by:

X (dB) = 60 dB, or

X (dB) = Pt + 30, whichever is greater attenuation

Between the –40 dB and –X dB frequencies, the level shall be below the 40 dB per decade (S = 40) roll-off lines in Fig. 2. All harmonic frequencies shall be at a level that is at least 60 dB below the maximum spectral power density.

Note 1 – Pt may be measured or may for the purpose of these criteria be calculated from the following:

Pt  =  Pp  +  20 log(N  ·  t)  +  10 log (PRR)  –  PG  –  90

Note 1 – The roll-off slope, S, from the –40 dB to –X dB points is at 40 dB per decade. The –20 dB bandwidth is limited to 2 MHz for wind profiler radars operating at 449 MHz. The maximum emission spectrum level between the –40 dB and –X dB points for S dB per decade slope is described by the formula:

where:		½ B (– 40 dB)  £  çF  –  F0 ç  £  ½ B (– X dB)

and:		B (– X dB)  =  (10a)  B (– 40 dB)

FIGURE 2/M.1085...[D02] = 14 CM Note relative à la Figure

## 3.3	Antenna gain characteristics

The centre of the antenna main beam generated at any time shall be limited within a cone of half-angles that are 20° from the zenith. The side-lobe levels (excluding the main beam) in all azimuths shall not exceed the following values:

| Elevation angle (degrees) | Side-lobe levels (dBi) |
| --- | --- |

|  | Median | Maximum |
| --- | --- | --- |

| ³ 45 | - 0 | - 12 |
| --- | --- | --- |

| for 5 to 45 | - –5 | - 7 |
| --- | --- | --- |

| £ 5 | - –20 | - –8 |
| --- | --- | --- |

## 3.4	Frequency tolerance

WPR transmitters shall meet a frequency tolerance no greater that 10 parts/million.

## 3.5	WPR receiver

The –3 dB receiver bandwidth should be commensurate with the authorized emission bandwidth plus twice the frequency tolerance of the transmitter as specified in § 3.4. The –60 dB receiver bandwidth shall be commensurate with the –60 dB emission bandwidth. Receivers shall be capable of switching bandwidth limits to appropriate values whenever the transmitter bandwidth is switched (pulse shape changed). Receiver IF image frequency rejection shall be at least 50 dB. Rejection of other spurious responses shall be at least 60 dB. WPR receivers shall not exhibit any local oscillator radiation greater than –40 dBm at the antenna input terminals. Frequency stability of receivers shall be commensurate with, or better than, that of the associated transmitters.

## 3.6	EMC provision

WPRs shall have the capacity to tolerate incoherent pulsed interference of duty cycle less than 1.5% such that peak interfering signal levels 30 dB greater than the WPR receiver noise level at the IF output will not degrade WPR performance.

## 3.7	Measurement capability

In order to coordinate radar operations in the field, an accurate measurement of the operating frequency is necessary. An accuracy of 1.0 part per million is adequate. Of comparable importance is the capability to measure pulse rise time and spectrum occupancy. Accordingly, the instrumentation necessary to make a frequency measurement shall have at least 1.0 part per million and suitable oscilloscopes and spectrum analysers to measure time and frequency parameters necessary to determine conformance with these criteria. Measurement instruments shall have resolution bandwidths of at least 10 kHz to measure close in bandwidth limits, and otherwise 100 kHz bandwidth below 1 GHz and 1 MHz bandwidth at and above 1 GHz should be used.

## 3.8	Effective isotropically radiated power (e.i.r.p.)

The peak e.i.r.p. of any WPR operating at 449 MHz shall not exceed 110 dBm.

_________________

*	This Recommendation should be brought to the attention of Radiocommunication Study Groups 2, 4, 9 and 11.