RECOMMENDATION  ITU-R  M.1174-1*

TECHNICAL  CHARACTERISTICS  OF  EQUIPMENT  USED
FOR  ON-BOARD  VESSEL  COMMUNICATIONS  IN
THE  BANDS  BETWEEN  450  AND  470  MHz

(1995-1998)

Rec. ITU-R M.1174-1

# Summary

This Recommendation describes the technical characteristics for equipment operating in the maritime mobile services in accordance with the provisions of No. S5.287 of the Radio Regulations (RR) for on-board vessel communications. Provision is made for 25 kHz or 12.5 kHz channel spacing.

The ITU Radiocommunication Assembly,

considering

a)	that there is a need to describe the characteristics of equipment for on-board vessel communications in the bands between 450 and 470 MHz;

b)	that changes have recently been made to the frequency availability;

c)	Resolution 341 (WRC-97),

recommends

1	that transmitters and receivers used in the maritime mobile service for on-board vessel communications in the bands between 450 and 470 MHz should conform to the technical characteristics shown in Annex 1.

ANNEX  1

Technical characteristics of equipment used for on-board vessel communications
in the bands between 450 and 470 MHz

1	The equipment should be fitted with sufficient channels for satisfactory operation in the area of intended use.

2	The effective radiated power should be limited to the maximum required for satisfactory operations, but should in no case exceed 2 W.  Wherever practicable the equipment should be fitted with a suitable device to reduce readily the output power by at least 10 dB.

3	In the case of equipment installed at a fixed point on the ship, the height of its antenna should not be more than 3.5 m above the level of the bridge.

|  | 25 kHz channels | 12.5 kHz channels |
| --- | --- | --- |
| 4 | Only frequency modulation with a pre-emphasis of 6 dB/octave (phase modulation) should be used. | Only frequency modulation with a pre-emphasis of 6 dB/octave (phase modulation) should be used. |
| 5 | The frequency deviation corresponding to 100% modulation should approach  5 kHz as nearly as practicable. In no event should the frequency deviation exceed  5 kHz. | The frequency deviation corresponding to 100% modulation should approach  2.5 kHz as nearly as practicable. In no event should the frequency deviation exceed  2.5 kHz. |
| 6 | The frequency tolerance should be 5 parts in 106. | The frequency tolerance should be 2.5 parts in 106. |
| 7 (Note 1) | The audio-frequency band should be limited to 3 000 Hz. | The audio-frequency band should be limited to 2 600 Hz. |

NOTE 1 – The frequency deviation characteristics for 25 kHz and 12.5 kHz channelling are based on European Telecommunications Standards 300 086 published by the European Telecommunications Standards Institute (ETSI).

8	Control, telemetry and other non-voice signals should be coded in such a manner as to minimise the possibility of false response to interfering signals.

9	The frequencies specified in RR No. S5.287 for on-board communications may be used for single frequency and two-frequency simplex operation.

10	When used in the duplex mode the base transmitter frequency should be selected from the lower range for improved operability.

11	If the use of a repeater station is required on board a ship, the following frequency pairs should be used (see also RR No. S5.288):

457.525 MHz and 467.525 MHz

457.550 MHz and 467.550 MHz

457.575 MHz and 467.575 MHz

Frequencies

The frequencies in RR S5.287 (subject to national regulations) are:

For 25 kHz channel spacing:

457.525 MHz

457.550 MHz

457.575 MHz

467.525 MHz

467.550 MHz

467.575 MHz

For equipment designed to operate with 12.5 kHz channel spacing the additional frequencies are:

457.5375 MHz

457.5625 MHz

467.5375 MHz

467.5625 MHz

*	 	This Recommendation should be brought to the attention of the International Maritime Organization (IMO) and the International Maritime Radio Committee (CIRM).