RECOMMENDATION  ITU-R  M.1451

TRANSPORT  INFORMATION  AND  CONTROL  SYSTEMS:  FUNCTIONALITIES

(Question ITU-R 205/8)

(2000)

Rec. ITU-R M.1451

The ITU Radiocommunication Assembly,

considering

a)	that transport information and control systems (TICS) may significantly contribute to the improvement of public safety;

b)	that international standards would facilitate worldwide applications of TICS and provide for economies of scale in bringing TICS equipment and services to the public;

c)	that early international harmonization of TICS would have several benefits;

d)	that worldwide compatibility of TICS may be dependent on common radio spectrum allocations;

e)	that the International Organization for Standardization (ISO) has work under way on standardizing TICS (non-radio aspects) in ISO/TC204 which will contribute to the efforts in ITU-R,

noting

a)	that the RF objectives and requirements of TICS elements are defined in Recommendation ITU-R M.1310,

recommends

that TICS intended for worldwide and/or regional use should meet the following functional requirements:

# 1	TICS functions

The functions below are intended to provide a preliminary framework for the definition of TICS functions.

## 1.1	Advanced vehicle control systems

–	Monitoring vehicle status including vehicle operations and roadside conditions.

–	Providing automatic vehicle operations including automatic cruise control.

–	Providing automatic emergency notification and incident detection.

–	Enhancing driver's vision.

## 1.2	Advanced traffic management systems

–	Providing traffic monitoring and surveillance including travel demand management and the exchange of information between traffic management centres.

–	Providing device control including the remote control of such devices.

–	Providing incident management and generating appropriate responses.

–	Monitoring vehicle emissions.

## 1.3	Advanced traveller information systems

–	Providing trip planning, guidance and information services including yellow page services.

–	Providing traveller services at information kiosks.

–	Managing rideshares.

–	Value-added traveller information services including e-mail, short message (SMS), paging, and Internet.

## 1.4	Advanced public transportation systems

–	Monitoring and disseminating information on vehicle operation and schedule adherence.

–	Planning and scheduling transit services, including vehicle maintenance schedules.

–	Providing transit user roadside facilities.

## 1.5	Advanced fleet management systems

–	Managing commercial vehicle fleet operations, as well as driver operations.

–	Providing roadside check facilities.

–	Administering electronic credentials for cargo data/contents and border clearance.

## 1.6	Emergency management systems

–	Allocating emergency services in response to requests.

–	Providing operator interface for emergency data.

–	Providing emergency service map data.

## 1.7	Electronic payment services

–	Providing electronic toll payment.

–	Providing electronic parking payment.

–	Providing electronic fare collection.

–	Carrying out centralized payments processing.

–	Providing payment instrument interfaces.

# 2	Relationship of TICS functions to suitable RF options

## 2.1	Advanced vehicle control systems

| TICS functions | RF options |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Longitudinal collision avoidance | Lateral collision avoidance | Intersection collision avoidance | Vision enhancement systems | Pre-crash restraint deployment | Automated road systems | Safety readiness |
| Monitor vehicle status | – Short range vehicle-to-vehicle – Short range radar – LCX | – Short range vehicle-to-vehicle – Short range radar – LCX | – Short range vehicle-to-vehicle – LCX – DSRC | – Short range radar | – Short range radar | – Short range vehicle-to-vehicle – Short range radar – LCX – DSRC | – LCX – DSRC |
| Provide automatic vehicle operations | – Short range vehicle-to-vehicle – Short range radar – LCX | – Short range vehicle-to-vehicle – Short range radar – LCX | – Short range vehicle-to-vehicle – Short range radar – LCX – DSRC | – Short range radar – LCX | – Short range radar | – Short range vehicle-to-vehicle – Short range radar – LCX – DSRC | – DSRC |
| Provide automatic emergency notification and incident detection | – Wide area – LCX |  | – Wide area | – LCX |  | – Wide area – LCX | – LCX |
| Enhance driver's vision | – Short range vehicle-to-vehicle – Short range radar | – Short range vehicle-to-vehicle – Short range radar | – Wide area – DSRC | – Short range radar | – Short range radar | – LCX – DSRC | – Short range radar |
| LCX: leaky coaxial DSRC: dedicated short-range communications |  |  |  |  |  |  |  |

## 2.2	Advanced traffic management systems

| TICS functions | RF options |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Traffic network monitoring and control | Travel demand management | Incident detection and management | Emission testing and mitigation | Parking management |
| Provide traffic monitoring and surveillance | – Microwave – Radar | – Microwave | – Radar – Two-way mobile-to-base – LCX | – Microwave | – DSRC – Microwave |
| Provide device control | – Microwave | – Microwave | – Two-way mobile-to-base – LCX | – Microwave | – Broadcast – DSRC – Two-way mobile-to-base |
| Provide incident management and generate appropriate responses | – Microwave | – DSRC – Microwave – Two-way mobile-to-base | – Two-way mobile-to-base – LCX | – Microwave | – Broadcast – DSRC |
| Monitor vehicle emission | – Microwave | – DSRC – Microwave – Two-way mobile-to-base | – Two-way mobile-to-base – LCX | – DSRC – Two-way mobile-to-base – LCX | – DSRC – Two-way mobile-to-base |

## 2.3	Advanced traveller information systems

| TICS functions | RF options |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pre-trip travel information | En-route driver information | En-route transit information | Route guidance | Ride matching and reservation |
| Provide trip planning, guidance and information services | – Broadcast – Two-way mobile-to-base – DSRC – LCX | – Broadcast – Two-way mobile-to-base – DSRC – LCX | – Broadcast – Two-way mobile-to-base – DSRC – LCX | – Broadcast – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base |
| Provide traveller services at information kiosks | – Broadcast – Two-way mobile-to-base | – Broadcast – Two-way mobile-to-base | – Broadcast – Two-way mobile-to-base |  | – Two-way mobile-to-base |
| Managing rideshares |  |  |  |  | – Two-way mobile-to-base |
| Value-added traveller information services (e-mail, SMS, paging, Internet, etc.) | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base – DSRC – LCX |

## 2.4	Advanced public transportation systems

| TICS functions | RF options |  |
| --- | --- | --- |
|  | Public transportation management | Personalized public transportation |
| Monitor and disseminate information on vehicle operation and schedule adherence | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base |
| Plan and schedule transit services, including vehicle maintenance schedules | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base |
| Providing transit user roadside facilities | – Two-way mobile-to-base – DSRC – LCX | – Two-way mobile-to-base – DSRC |

## 2.5	Advanced fleet management systems

| TICS functions | RF options |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Vehicle administration | Safety monitoring and tracking | Fleet management | Vehicle pre-clearance | Automated roadside safety inspections | Hazardous material incident response |
| Manage commercial vehicle fleet operations, as well as driver operations | – Two-way mobile-to-base | – Two-way mobile-to-base – DSRC | – Two-way mobile-to-base – DSRC | – DSRC | – DSRC | – Two-way mobile-to-base |
| Provide roadside check facilities | – DSRC | – DSRC | – DSRC | – DSRC | – DSRC | – DSRC |
| Administer electronic credentials for cargo data/contents and border clearance | – Two-way mobile-to-base | – Two-way mobile-to-base – DSRC | – Two-way mobile-to-base – DSRC | – DSRC | – DSRC | – Two-way mobile-to-base |
| Manage special permitted vehicles | – Two-way mobile-to-base – DSRC | – Two-way mobile-to-base – DSRC | – Two-way mobile-to-base – DSRC | – DSRC | – DSRC – LCX | – Two-way mobile-to-base |
| Manage effective road network administration | – Two-way mobile-to-base – DSRC | – Two-way mobile-to-base | – Two-way mobile-to-base |  | – DSRC – LCX | – Two-way mobile-to-base – Wide area – Broadcast – LCX |

## 2.6	Emergency management systems

| TICS functions | RF options |  |  |
| --- | --- | --- | --- |
|  | Emergency notification and personal security | Public travel security | Emergency vehicle management |
| Allocate emergency services in response to requests | – Two-way mobile-to-base | – Two-way mobile-to-base | – Two-way mobile-to- base – Two-way mobile-to- mobile |
| Provide operator interface for emergency data |  |  |  |
| Provide emergency service map data |  |  | – Two-way mobile-to- base – Two-way mobile-to- mobile |
| Manage effective road network administration | – Two-way mobile-to-base – Wide area – Broadcast – LCX | – Two-way mobile-to-base – Wide area – Broadcast – DSRC | – Two-way mobile-to- base – Two-way mobile-to-mobile – LCX |
| Manage special permitted vehicles | – Two-way mobile-to-base – Wide area – Broadcast | – Two-way mobile-to-base – LCX | – Two-way mobile-to- base – LCX |

## 2.7	Electronic payment services

| TICS functions | RF options |
| --- | --- |
|  | Electronic payment services |
| Provide electronic toll payment | – Two-way mobile-to-base – DSRC |
| Provide electronic parking payment | – Two-way mobile-to-base – DSRC |
| Provide electronic fare collection | – Two-way mobile-to-base – DSRC |
| Carry out centralized payments processing | – Two-way mobile-to-base |
| Provide payment instrument interface | – Two-way mobile-to-base |