RECOMMENDATION  549-1

SIDE  TONE  REFERENCE  EQUIVALENT  OF  HANDSET  USED  ON  BOARD  A  SHIP
IN  THE  MARITIME  MOBILE-SATELLITE  SERVICE  AND  IN  AUTOMATED
VHF/UHF  MARITIME  MOBILE  RADIOTELEPHONE  SYSTEMS

(Study Programme 17A/8)

(1978-1982)

Rec. 549-1

The CCIR,

CONSIDERING

(a)	that the ambient acoustic noise to be expected on board ships is often significantly higher than on terrestrial telephone locations;

(b)	that room noise picked up by the side tone path of the handset should not introduce any further transmission impairment;

(c)	CCITT Recommendation G.121, Section E,

UNANIMOUSLY  RECOMMENDS

that the side tone reference equivalent of the handset in the ships’ installation should be at least 17 dB.

Note 1. – Noise levels in the range 50 to 80 dBA are to be expected on board ships, the actual level being dependent on locations on board ship, the ship’s size and structure.

Note 2. – The need for a side tone may be discussed in view of the degradations it can create. Even when the value 17 dB is attained, room noise at expected levels may still have an adverse effect (CCITT Recommendation G.121). The recommended value is a compromise between the need for limiting impairment due to room noise and the possible need for a certain “liveliness” in the handset.

Note 3. – The CCITT Recommendations on side tone could be subject to change as all aspects of side tone are under review.