Recommendation ITU-R P.2146-0 (08/2022)

Sea surface bistatic scattering

P Series Radiowave propagation

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2022

ITU 2022

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  P.2146-0

Sea surface bistatic scattering

(Question ITU-R 208-6/3)

(2022)

Scope

This Recommendation provides a method for predicting the bistatic scattering coefficient and coherent reflection coefficient for the sea surface. This model can be applied at any elevation angle, except grazing incidence, and is applicable for frequencies up to 100 GHz, and for wind speed between 0.5 m/s to 25 m/s.

Keywords

Bistatic scattering coefficient, coherent scattering, diffuse scattering, wind speed, upwind and crosswind mean square slopes, gravity wave, capillary wave, large-scale roughness, small-scale roughness

Acronyms/Abbreviations

ECMWF 	European Centre for Medium-Range Weather Forecast

ERA5	ECMWF Reanalysis version 5

Related ITU-R Recommendations and Handbook

Recommendation ITU-R P.372

Recommendation ITU-R P.452

Recommendation ITU-R P.527

Recommendation ITU-R P.528

Recommendation ITU-R P.676

Recommendation ITU-R P.680

Recommendation ITU-R P.1144

Recommendation ITU-R P.2148

NOTE – In every case, the latest revision/edition of the Recommendation in force should be used.

The ITU Radiocommunication Assembly,

considering

a)	that the proper planning of Earth surface observation systems and down-looking space systems, it is necessary to have appropriate models for predicting bistatic scattering coefficient of the sea surface;

b)	that the bistatic scattering coefficient may have one or two of the following components: coherent bistatic scattering coefficient component and diffuse (non-coherent) bistatic scattering coefficient component;

c)	that the diffuse bistatic scattering may be due to large-scale roughness driven by long gravity wave and small-scale roughness driven by short capillary wave;

d)	that both the long gravity wave and short capillary wave are driven by wind;

e)	that model has been developed that allows the prediction of bistatic scattering coefficients of the sea surface needed in assessing several propagation parameters such as fading depth, atmospheric noise due to emission from sea surface, interference power due to scattering from sea surface,

recommends

that the method in the Annex should be used to predict the bistatic scattering coefficient from the sea surface.

Annex

TABLE OF CONTENTS

Page

Policy on Intellectual Property Right (IPR)	ii

Annex	2

1	Introduction	4

2	Step 0: Input parameters	6

2.1	Propagation input parameters	6

2.2	Sea surface input parameters	7

3	Step 1: Determine the sea the water complex relative permittivity	8

4	Step 2: Determine the sea surface roughness parameters	9

4.1	Sea surface small-scale height spectral density (capillary wave) and sea height variance	9

4.2	Probability density of large-scale probability density of surface slopes (gravity wave)	9

5	Step 3: Determine the coherent bistatic scattering coefficient	10

6	Step 4: Determine the diffuse bistatic scattering due to large scale roughness	11

7	Step 5: Determine the diffuse bistatic scattering due to small-scale roughness	12

8	Sum the components of sea surface bistatic scattering coefficient	15

8.1	Sea surface backscattering coefficients	16

8.2	Sea surface bistatic scattering coefficients in the forward direction	18

Attachment A to Annex  Calculation of scattering coefficients between circularly polarized  and linearly polarized waves	22

A.1	Circularly Polarized (CP) incident to linear scattered power	22

A.2	Linear incident to CP scattered power	23

Attachment B to Annex  Calculation of scattering coefficients between circularly polarized waves	23

Attachment C to Annex  Simple approximations	24

Attachment D to Annex  Sea surface height spectra model	25

Attachment E to Annex  Interference power of a reflected signal from the sea surface into a receiver	26

E.1 	Introduction	26

E.2	Coherent received power	27

E.3	Diffuse received power	28

# 1	Introduction

This Recommendation provides a method for predicting the sea surface bistatic scattering coefficient as a function of sea surface salinity, temperature, wind speed, relative wind direction and inverse wave age. If local data for these parameters are not available, the default values of these parameters as provided in Table 1 should be used. This prediction method is applicable to any elevation angle, except grazing incidence, any frequency up to 100 GHz, and wind speeds from 0.5 m/s to 25 m/s.

The bistatic scattering coefficient has two components: 1) a diffuse (non-coherent) component, which is the fraction of the incident power from an incident direction scattered to an arbitrary receive direction, and 2) a coherent component, which is the fraction of the incident power scattered back to the incident (i.e. specular) direction.

The prediction method in §§ 1 to 8 assumes the source and receiver polarizations are linear. If the source and/or receiver polarizations are circular, the appropriate bistatic scattering coefficients can be calculated from the linear bistatic scattering coefficients using the methods in Attachments A, B or C to this Annex.

These prediction methods can be used to predict the interference power reflected by the sea surface between a source (e.g. a downlink transmitter) and a receiver receiving harmful interference (e.g. an Earth observation satellite receiver) using the method in Attachment E.

The sea surface is generally a randomly rough surface with roughness driven by wind. The bistatic scattering coefficient γpqks,ki is the fraction of power scattered along a direction ks with polarization ps due to incident wave illuminating the surface along the direction ki with polarization qi (see Fig. 1). The fraction of power is per unit solid angle and per unit area. Due to the complex nature of the surface roughness, the bistatic scattering coefficient is assumed to be composed of two components: a coherent component γpqcks,ki, and a diffuse (non-coherent) component γpqdifks,ki. These two components are added together to:

γpqks,ki=γpqcks,ki+γpqdifks,ki	(1)

The coherent component exists only along the specular direction of the surface (θs=θi, φs=φi).

FIGURE 1

Geometric configuration of bistatic scattering from sea surface

Figure 1 illustrates the geometry considered in the scattering model. The xy-plane is the horizontal sea surface and the zenith direction is the positive z axis. The positive x axis is the upwind direction, the positive y axis is the crosswind direction where the incidence zenith angle, θi and the scattering zenith angle, θs, are measured from the positive z axis. The incidence clock angle, φi, and the scattering clock angle, φs are measured in the xy-plane in the counter-clockwise direction relative to the upwind direction.

The sea surface is treated as a two-scale roughness surface having large-scale roughness and small-scale roughness laying over the long-scale roughness. The long-scale roughness is associated with the long gravity wave and the small-scale roughness is associated with the short capillary wave. Accordingly, the diffuse (non-coherent) component of the bistatic scattering coefficient γpqdifks,ki is the sum of two components as given in equation (2):

γpqdifks,ki=γpqlks,ki+γpqsks,ki	(2)

The first term in equation (2) is attributed to the sea surface long-scale roughness and the second term is attributed to the small-scale roughness.

The model for predicting the coherent and the diffuse (non-coherent) components of the sea surface bistatic scattering coefficient can be constructed based on the flow chart depicted in Fig. 2.

FIGURE 2

Flow chart of the sea surface bistatic scattering prediction model

The bistatic scattering coefficient of sea surface modelled here is for linearly polarized incident waves and linearly polarized scattered waves. The transformations from a linear polarization basis to other combinations of incident and scattered polarizations are provided in Attachment A and Attachment B:

–	Attachment A is for circularly polarized incident waves and linearly polarized scattered waves and vice versa.

–	Attachment B is for circularly polarized incident waves and circularly polarized scattered waves.

–	Attachment C provides simple approximations for the bistatic scattering coefficients in Attachment A and Attachment B.

Moreover,

–	Attachment D provides a sea surface height spectra model.

–	Attachment E provides a methodology for determining the interference power from a reflected/scattered signal of the sea surface into a receiver

# 2	Step 0: Input parameters

To predict values of the coherent and diffuse (non-coherent) components of the bistatic scattering coefficient for sea surface, two types of input parameters are required:

–	Propagation input parameters, and

–	Sea surface input parameters.

## 2.1	Propagation input parameters

The propagation input parameters are as follows:

–	The elevation angle ϵi and azimuth angle φi of the incident wave illuminating the sea surface. These two angles determine the incident propagation direction ki.

–	The polarization vector qi of the wave illuminating the sea surface.

–	The elevation angle ϵs and azimuth angle φs of the scattering direction ks along which the bistatic scattering coefficient is calculated.

–	The polarization vectors ps of the scattered field.

–	The frequency f at which the bistatic scattering coefficient is calculated. The frequency is given in GHz units. The frequency determines the RF wavenumber 
kk=2πf0.299792458radm.

Since only linear polarizations are considered in this Annex, qi = vi or hi  and ps = vs or hs with v standing for vertical (parallel) polarization, and h standing for horizontal (perpendicular) polarization. Throughout this recommendation the elevation angles ϵi,s are replaced by the corresponding zenith angles θi,s.

θi,s=π2-ϵi,s	(3)

## 2.2	Sea surface input parameters

The sea surface input parameters required for predicting bistatic scattering from sea surface are reported in Table 1 where default values may be used if local parameters are not known.

TABLE 1

Sea surface input parameters

| Parameter | Description | Default value |
| --- | --- | --- |
| Sea surface salinity | Required for computing the complex relative permittivity of sea surface | 35 ppt |
| Sea surface temperature | Required for computing the complex relative permittivity of sea surface | 0, 15, 30° C |
| Wind speed U10(m/s) | The wind speed at a height of 10 m above sea surface | See text below |
| Inverse wave age Ω | The sea is fully developed when Ω is close to 0.85, mature when Ω is close to 1, and young when Ω > 2 | 0.85 |
| Cut off roughness height wavenumber κd | The height wavenumber dividing the sea surface height spectrum into long gravity wave spectrum and short capillary wave spectrum | 0.5k where k is the wavenumber (1/m) |

Wind speed and coordinate system

Local values of the parameters in Table 1 should be used if available. Wind speed at 10 m above the sea surface depends on geographic location and season and varies over time. If available, local sea surface wind speed data (magnitude, direction and spatial and temporal correlation) applicable to the specific geographical location of interest should be used. If spatial or temporal correlation is not required, the global maps in Recommendation ITU-R P.2148 can be used.

If local sea surface wind speed data are not available, the local temporal wind vector can be derived from multiple sources of global data; e.g. the eastward and northward components of hourly wind speed at a height of 10 m above the surface of the Earth from 1979 are available from the ECMWF Copernicus Climate Data Store.

Some datasets (e.g. ECMWF ERA5 data) provide the v, northward (wind heading towards the north) component of wind speed, and the u, eastward (wind heading toward the east) component of wind speed. For a given value of u and v, the magnitude of the wind speed at a height of 10 m above the surface of the Earth, U10, is u2+v2. As shown in Fig. 3, the upwind direction is the opposite direction that the wind is heading; and the clockwise angle from north to the upwind direction, φw, is 270o-atan2(v,u). Hence, in Fig. 3, the positive x-axis, which corresponds to the upwind direction, is a clockwise angle of 270o-atan2(v,u) relative to north; or, equivalently, north is a counter clockwise angle of 270o-atan2(v,u) relative to the upwind direction.

If ϕi' and ϕs' are the incident and scattering clockwise angles relative to north, then φi and φs, the corresponding incident and scattering clockwise angles shown in Fig. 3 are: φi=φw-φi' 
and φs=φw-φs' .

FIGURE 3

Wind speed and coordinate systems

The following intermediate parameters are calculated from the above input parameters as part of the method:

–	Sea surface complex relative permittivity εr,

–	Sea surface height spectral density function Wsκ,ψ,

–	Sea surface height variance σ2, and

–	Sea surface upwind and cross wind mean square slopes mu2 and mc2.

# 3	Step 1: Determine the sea the water complex relative permittivity

The complex relative permittivity model described by equations (14) to (27) of Recommendation ITU-R P.527-6 is used in this Recommendation to obtain the sea water complex relative permittivity. This model formulates the sea water complex relative permittivity εr in terms of sea surface salinity, sea surface temperature and RF frequency. The default values of sea surface salinity and temperature are given in Table 1.

# 4	Step 2: Determine the sea surface roughness parameters

Sea surface roughness is described by the following:

–	The small-scale surface height spectral density which is required for calculating the diffuse bistatic scattering coefficient component due to small-scale roughness (§ 6).

–	The total sea surface rms height which is required for calculating the coherent reflectivity (§ 5).

–	The large-scale probability density of surface slopes which is required for calculating the diffuse bistatic scattering coefficient component due to large scale roughness (§ 7).

## 4.1	Sea surface small-scale height spectral density (capillary wave) and sea height variance

The small-scale spectrum is related to the sea surface directional spectrum, Wκ,ψ, as follows.

Wsκ,ψ=0if   κ<κdWκ,ψotherwise	(4)

where κd is the two-scale cut-off height wavenumber, and κ,ψ are the polar coordinates of the spatial wavenumbers in the upwind and crosswind directions associated with the rectangular coordinates of the surface slopes. Explicit expression for Wκ,ψ is given in Attachment D.

The total sea surface rms height σ, which is required for calculating the coherent reflectivity, is related to the wind speed, U10, as follows:

σ2=σ0+σ1U10+σ2U102+σ3U103+σ4U104+σ5U105if   U10≥1ms0.001515U10if   U10<1ms	(5)

With σ2 (the surface height variance) in square meters and U10 (wind speed at a height of 10 m above sea surface) in m/s, and:

σ0=-0.002913931483264

σ1=0.006483314256661

σ2=-0.002390537892927

σ3=0.000309146709141

σ4=0.000026373965831

σ5= 0.000000350137099

## 4.2	Probability density of large-scale probability density of surface slopes (gravity wave)

The large-scale probability density of surface slopes is represented by zero-mean bivariate Gaussian distribution:

PSu,Sc=12πmumcexp-12Su2mu2+Sc2mc2	(6)

where Su and Sc are the slopes along the upwind and crosswind directions at surface patches where local bistatic scattering coefficients are calculated. Furthermore, mu2and mc2 are the upwind and crosswind mean square slopes.

mu2=t=07atU10t	(7)

mc2=t=07btU10t	(8)

where the regression coefficients at(t=0,…,7) and bt (t=0,….,7) in equations (7) and (8) have the following dependence on frequency f in GHz:

at=dt,0+dt,1log ef+dt,2logef2+dt,3logef3+dt,4logef4      t=0,..,7	(9)

bt=zt,0+zt,1logef+zt,2logef2+zt,3logef3+zt,4logef4         t=0,..,7	(10)

The coefficients dt,m and zt,m are given in Table 2 and Table 3, respectively. Equations (7) and (8) are valid for U10 values between 0.5 m/s and 25 m/s and frequencies between 1 and 100 GHz.

TABLE 2

Values of the regression coefficients dt,m of equation (9)

| at | dt,0 | dt,1 | dt,2 | dt,3 | dt,4 |
| --- | --- | --- | --- | --- | --- |
| a0 | −0.001 316 803 829 | −0.000 766 377 24 | 0.000 178 465 995 | 0.000 163 583 254 | −2.722 372 719 5e-05 |
| a1 | 0.003 381 740 504 | 0.003 262 226 696 | 0.001 055 843 558 | −0.000 556 018 050 | 5.638 297 081 0e-05 |
| a2 | −8.387 091 908e-06 | −0.000 788 099 04 | −0.000 849 564 4 | 0.000 321 034 03 | −2.969 409 304 3e-05 |
| a3 | −7.172 344 345 1e-05 | 9.130 847 487e-05 | 0.000 180 310 43 | −6.039 065 778e-05 | 5.252 298 53e-06 |
| a4 | 9.781 960 983 7e-06 | −5.515 385 070e-06 | −1.831 052 853e-05 | 5.756 933 90e-06 | −4.820 426 74e-07 |
| a5 | −5.824 151 735 3e-07 | 1.831 590 630e-07 | 9.693 536 66e-07 | −2.928 018 73e-07 | 2.384 386 09e-08 |
| a6 | 1.662 701 734 3e-08 | −3.121 665 19e-09 | −2.590 444 81e-08 | 7.608 802 794e-09 | −6.063 116 61e-10 |
| a7 | −1.853 308 18e-10 | 2.084 451 182e-11 | 2.762 769 59e-10 | −7.94818760e-11 | 6.223 677 47e-12 |

TABLE 3

Values of the regression coefficients zt,m of equation (10)

| bt | zt,0 | zt,1 | zt,2 | zt,3 | zt,4 |
| --- | --- | --- | --- | --- | --- |
| b0 | −0.000 388 356 64 | −0.000 566 882 739 | −0.000 187 663 9 | 0.000 195 168 030 1 | −2.564 879 98e-05 |
| b1 | 0.000 711 554 432 3 | 0.001 274 333 859 | 0.001 582 455 599 | −0.000 564 251 194 | 5.158 545 58e-05 |
| b2 | 0.000 467 115 768 | 7.665 602 489e-05 | −0.000 999 944 82 | 0.000 304 430 724 | −2.608 628 437e-05 |
| b3 | −0.000 113 274 18 | −7.062 890 94e-05 | 0.000 204 604 176 | −5.704 760 441e-05 | 4.619 116 82e-06 |
| b4 | 1.144 869 515e-05 | 9.917 914 997 6e-06 | −2.031 787 86e-05 | 5.376 554 489e-06 | −4.184 881 982e-07 |
| b5 | −5.954 866 288 2e-07 | −6.127 030 44e-07 | 1.063 995 76e-06 | −2.717 537 12e-07 | 2.052 809 6e-08 |
| b6 | 1.566 749 978 4e-08 | 1.794 015 885e-08 | −2.826 461 77e-08 | 7.033 322 599e-09 | −5.186 932 2e-10 |
| b7 | −1.651 144 028 4e-10 | −2.032 492 61e-10 | 3.003 151 95e-10 | −7.323 652 942e-11 | 5.294 665 17e-12 |

# 5	Step 3: Determine the coherent bistatic scattering coefficient

The coherent component of the bistatic scattering coefficient is like polarized component (vv, or hh) and it exists only along the specular reflection direction:

γpqcks,ki=4πrppθi2exp-2kσcosθi2if  θs=θi,  φs=φi,  and p=q0otherwise	(11)

In the above, rppθi is Fresnel reflection coefficient for polarization p (p=v,h):

rhhθi=cosθi-εr-sin2θicosθi+εr-sin2θi	(12)

rvvθi=εrcosθi-εr-sin2θiεrcosθi+εr-sin2θi	(13)

# 6	Step 4: Determine the diffuse bistatic scattering due to large scale roughness

For getting the bistatic scattering coefficient due to large scale roughness (gravity wave) use the incident angles (θi, φi) and the scattering angles (θs,φs) provided in § 2.1 to calculate the following quantities.

qx=sinθscosφs-sinθicosφi	(14)

qy=sinθssinφs-sinθisinφi	(15)

qz=cosθs+cosθi	(16)

q2=qx2+qy2+qz2	(17)

ks⋅vi=-sinθscosθicosφs-φi-sinθicosθs	(18)

ks⋅hi=sinθssinφs-φi	(19)

ki⋅vs=sinθicosθscosφs-φi+sinθscosθi	(20)

ki⋅hs=-sinθisinφs-φi	(21)

D02=ki⋅vs2+ki⋅hs2	(22)

Introduce equations (16) and (17) into equation (23) to get the local incident angle θi' at the surface point contributing to the bistatic scattering coefficient:

cosθi'=qqz2qz	(23)

Introduce equation (23) into equations (12) and (13) using the proper value of the complex relative permittivity to get local Fresnel reflection rhh' and rvv'. Then introduce the local Fresnel coefficients rhh' and rvv' along with the scalar vector products of equations (18) to (21) into equations (24) to (27) to get the polarization factors Upqks,ki’s:

If D02≠0:

Uhhks,ki=ks⋅viki⋅vsrhh'+ks⋅hiki⋅hsrvv'D02	(24)

Uvhks,ki=-ks⋅viki⋅hsrhh'+ks⋅hiki⋅vsrvv'D02	(25)

Uhvks,ki=-ks⋅hiki⋅vsrhh'+ks⋅viki⋅hsrvv'D02	(26)

Uvvks,ki=ks⋅hiki⋅hsrhh'+ks⋅viki⋅vsrvv'D02	(27)

If D02=0:

Uhhks,ki=rhh'	(28)

Uvhks,ki=0	(29)

Uhvks,ki=0	(30)

Uvvks,ki=rvv'	(31)

Introduce equations (14) to (17) and (24) to (27) into equation (32) to get the sea surface diffuse component of the bistatic scattering coefficient γpqlks,ki due to long gravity wave:

γpqlks,ki=12mumcqqz4Upqks,ki2exp-12qz2qxmu2+qymc2	(32)

where mu and mc are the upwind and the crosswind mean squared surface slopes given in equations (7) and (8) as a function of wind speed U10 and frequency f.

# 7	Step 5: Determine the diffuse bistatic scattering due to small-scale roughness

The approach exploited in getting the diffuse bistatic scattering due to small-scale roughness starts by setting the maxima and minima of Su and Sc.

Su,max=6 mu

Su,min=-min6 mu,cotθi

Sc,max=6 mc

Sc,min=-6 mc

The slope region subtended by the above maxima and minima are divided into 64 × 64 points using Gaussian quadrature nodes. At an arbitrary node t, m the slopes can be written as:

Su,t=12Su,max-Su,minχt+Su,max+Su,min	(33)

Sc,m=12Sc,max-Sc,minςm+Sc,max+Sc,min	(34)

In equations (33) and (34), χt and ςm are the Gaussian Quadrature nodes (t,m=1,…,64) which correspond to the Xi of Section 3 of the Annex to Recommendation ITU-R P.1144-11, i.e. χt,ςm→Xi. Also Su,t→Xi' and Sc,m→Zi'.

Figure 4

Local scattering angles (θs', φs') and inclination angles of surface normal 
(zenith angle θn and azimuth angle φn)

At the above Gaussian node perform the following procedures:

–	Get the zenith angle θn and azimuth angle φn defining the inclination of the sea surface large-scale surface normal n (see Fig. 4) through introducing equations (32) and (33) into equations (35) to (37):

φn=tan-1Sc,mSu,t	(35)

cosθn=1Su,t2+Sc,m2+1	(36)

sinθn=cosθnSu,tcosφn+Sc,msinφn	(37)

–	Get the local scattering and incident angles θs', and φs', and θi', and φi' at this node point:

φs'=tan-1sinθssinφs-φnsinθscosθncosφs-φn+cosθssinθn	(38)

cosθs'=-sinθsSu,tcosφs+Sc,msinφs+cosθsSu,t2+Sc,m2+1	(39)

sinθs'=sinθscosθncosφs-φn+cosθssinθncosφs'+sinθssinφs-φnsinφs'	(40)

and

φi'=tan-1sinθisinφi-φnsinθicosθncosφi-φn-cosθisinθn	(41)

cosθi'=sinθiSu,tcosφi+Sc,msinφi+cosθiSu,t2+Sc,m2+1	(42)

sinθi'=sinθicosθncosφi-φn-cosθisinθncosφi'+sinθisinφi-φnsinφi'	(43)

–	Get the local incident and scattered angles enables getting the components of the incident horizontal polarization and scattered horizontal polarization:

hxi=sinθisinφi-cosθiSc,m	(44)

hyi=cosθiSu,t-sinθicosφi	(45)

hzi=sinθiSu,tsinφi-Sc,mcosφi	(46)

Di=hxi2+hyi2+hzi2	(47)

and

hxs=sinθssinφs+cosθsSc,m	(48)

hys=-cosθsSu,t+sinθscosφs	(49)

hzs=sinθsSu,tsinφs-Sc,mcosφs	(50)

Ds=hxs2+hys2+hzs2	(51)

–	Get the following scalar vector products in the incident direction:

vi'⋅vi=cosθn-cosθiSu,tcosφi+Sc,msinφi+sinθiDi	(52)

vi'⋅hi=cosθn-Su,tsinφi+Sc,mcosφiDi	(53)

hi'⋅vi=-1Dicosθihxicosφi+hyisinφi+hzisinθi	(54)

hi'⋅hi=1Dihyicosφi-hxisinφi	(55)

And the following scalar vector products in the scattering direction:

vs⋅vs'=cosθncosθsSu,tcosφs+Sc,msinφs+sinθsDs	(56)

vs⋅hs'=1Dscosθshxscosφs+hyssinφs-hzssinθs	(57)

hs⋅vs'=cosθn-Su,tsinφs+Sc,mcosφsDs	(58)

hs⋅hs'=1Dshyscosφs-hxssinφs	(59)

–	Get the polarization factors in the local frame gpq'’s through introducing equations (38) and (41) along with proper value of complex relative permittivity into equations (60) to (63):

ghh'=εr-1cosθs'+εr-sin2 θs'cosθi'+εr-sin2 θi'cosφs'-φi'	(60)

gvh'=-εr-1εr-sin2θs'εrcosθs'+εr-sin2θs'cosθi'+εr-sin2θi'sinφs'-φi'	(61)

ghv'=εr-1εr-sin2θi'cosθs'+εr-sin2θs'εrcosθi'+εr-sin2 θi'sinφs'-φi'	(62)

gvv'=εr-1εrsinθi'sinθs'-εr-sin2 θs'εr-sin2 θi'cosφs'-φi'εrcosθs'+εr-sin2θs'εrcosθi'+εr-sin2θi'	(63)

–	Get the polarization factors Gpq’s through introducing equations (60) to (63), and equations (52) to (59) into equations (64) to (67):

Gvv=vs⋅vs'gvv'+vs⋅hs'ghv'vi'⋅vi+vs⋅vs'gvh'+vs⋅hs'ghh'hi'⋅vi 	(64)

Gvh=vs⋅vs'gvv'+vs⋅hs'ghv'vi'⋅hi+vs⋅vs'gvh'+vs⋅hs'ghh'hi'⋅hi	(65)

Ghv=hs⋅vs'gvv'+hs⋅hs'ghv'vi'⋅vi+hs⋅vs'gvh'+hs⋅hs'ghh'hi'⋅vi	(66)

Ghh=hs⋅vs'gvv'+hs⋅hs'ghv'vi'⋅hi+hs⋅vs'gvh'+hs⋅hs'ghh'hi'⋅hi	(67)

The function U ensures that the Gaussian node t,m sees the incident direction and scattered direction simultaneously:

USu,t,θi=max⁡((1+Su,ttanθi) ,0)if 0≤θi'≤π2 and 0 ≤θs'≤π2 0otherwise	(68)

The above procedures need to be repeated to cover all the nodes of the slope.

To get sea surface diffuse bistatic scattering coefficient due to short waves, use the above formulations to calculate (69) and (70) at all nodes:

PSu,t,Sc,m=12πmumcexp-12Su,t2mu2+Sc,m2mc2	(69)

γpqcapks,ki=16πk2cosθs'cosθi'2Gpq2Wsκ,φi	(70)

Equation (69) is obtained from equation (6). In addition, Wsκ,φi in equation (70) is the short-wave spectral density function given in Attachment D calculated at ψ=φi, and height wavenumber κ given by:

κ=ksin2θs'+sin2θi'-2sinθs'sinθi'cosφs'-φi'	(71)

Then introduce equations (68) to (71) into equation (72) to get the sea surface diffuse bistatic scattering coefficient due to small-scale roughness (short capillary wave):

γpqsks,ki=Cm=164t=164γpqcapks,kiUSu,t,θiPSu,t,Sc,mwu,twc,m	(72)

In equation (72), wu,t and wc,m are the Gaussian quadrature weights which are given in Section 3 of the Annex to Recommendation ITU-R P.1144-11. The factor C is a constant given by:

C=14Su,max-Su,minSc,max-Sc,min	(73)

# 8	Sum the components of sea surface bistatic scattering coefficient

The coherent component (11) and diffuse components (32) and (72) of sea surface bistatic scattering coefficients are summed and exploited in exploring features of those coefficients in the backscattering direction and the forward (specular) direction.

γpqsks,ki=γpqlks,ki+γpqsks,ki+γpqcks,ki	(74)

## 8.1	Sea surface backscattering coefficients

The sea surface backscattering coefficients are of interest, especially to the active remote sensing communities. They have no coherent component, and they can be calculated from the sum of equations (32) and (72) by setting ks=-ki (i.e. θs=θi, and φs=π+φi). Figures 5 to 8 illustrate the dependence of the sea surface backscattering coefficients on incidence angle, polarization, wind speed and wind direction.

Figure 5

Backscattering coefficients along upwind direction as a function of incidence angle at different values of wind speed 
(frequency = 13.6 GHz, Ω = 0.84, temperature = 30° C, salinity = 35 ppt)

Figure 6

Backscattering coefficients as a function of wind speed at different angles of incidence 
(frequency = 13.6 GHz, Ω = 0.84, temperature = 30° C, salinity = 35 ppt)

Figure 7

Upwind backscattering coefficients as a function of incidence angle 
(frequency = 18.6 GHz, Ω = 0.84, temperature = 30° C, U10 = 18 m/s, salinity = 35 ppt)

Figure 8

Crosswind backscattering coefficients as a function of incidence angle 
(frequency = 18.6 GHz, Ω = 0.84, temperature = 30° C, U10= 18 m/s, salinity = 35 ppt)

## 8.2	Sea surface bistatic scattering coefficients in the forward direction

The sea surface bistatic scattering coefficients in the forward direction can be used for assessing the interference power received by a receive antenna. These sea surface bistatic scattering coefficients can also be used to calculate the fading depth due to sea surface reflections in Recommendation ITU-R P.680-3. They can be calculated as the sum of the coherent component (11) and the diffuse components. The coherent component exits only along the specular reflection direction (θs=θi, and φs=φi). The diffuse component is defined in equations (32) and (72) by setting 
θs=θi∧φs=φi.

Figures 9 to 14 are examples of the bistatic scattering coefficients in the forward direction. Figures 9 and 10 fix the scattering angle (the receive observation angle) and vary the angle of incidence (direction of the interfering signals). Figure 9 is calculated at 1.227 6 GHz (GPS L2 frequency), and Fig. 10 is calculated at 18.6 GHz. In each Figure, the bistatic scattering coefficients are calculated at two different wind speeds: U10 = 2 m/s, and U10 = 25 m/s.

Figures 11 to 14 fix the incidence angle (interferer angle) and vary the scattering angle. Figures 11 and 13 are calculated at 1.227 6 GHz, and Figs 12 and 14 are calculated at 18.6 GHz. Figures 11 and 12 are calculated at sea surface temperature of 30° C. Figures 13 and 14 are calculated at a sea surface temperature of 5° C. Moreover, in each of Figs 11 to 14, the bistatic scattering coefficients are calculated at two different wind speeds: U10 = 2 m/s, and U10 = 25 m/s.

Due to reciprocity, fixing either the angle of incidence or the scattering angle yields similar results. Accordingly, the fixed value of scattering angle in Figs 8 and 9 are different than the fixed value of the incident angle in Figs 11 to 14.

Comparing Fig. 11 against Fig. 13 and comparing Fig. 12 against Fig. 14 indicate that for temperature above freezing condition, the temperature has no pronounced effect on the bistatic scattering coefficient.

Figure 9

Forward bistatic scattering coefficients as a function of incidence angle at different values of wind speeds
(θs = 30°, frequency = 1.227 6 GHz, Ω = 0.84, temperature = 30° C, salinity = 35 ppt)

Figure 10

Forward bistatic scattering coefficients as a function of incidence angle at different values of wind speeds
(θs = 30°, frequency = 18.6 GHz, Ω = 0.84, temperature = 30° C, salinity = 35 ppt)

Figure 11

Forward bistatic scattering coefficients as a function of scattering angle at different values of wind speeds
(θi = 50°, frequency = 1.227 6 GHz, Ω = 0.84, temperature = 30 °C, salinity = 35 ppt)

Figure 12

Forward bistatic scattering coefficients as a function of scattering angle at different values of wind speeds
(θi = 50°, frequency = 18.6 GHz, Ω = 0.84, temperature = 30° C, salinity = 35 ppt)

Figure 13

Forward bistatic scattering coefficients as a function of scattering angle at different values of wind speeds
(θi = 50°, frequency = 1.227 6 GHz, Ω = 0.84, temperature = 5° C, salinity = 35 ppt)

Figure 14

Forward bistatic scattering coefficients as a function of scattering angle at different values of wind speeds
(θi = 50°, frequency = 18.6 GHz, Ω = 0.84, temperature = 5° C, salinity = 35 ppt)

Attachment A
to Annex

Calculation of scattering coefficients between circularly polarized 
and linearly polarized waves

This Attachment provides the calculations of the bistatic scattering coefficients between a) a circularly polarized incident wave and a linearly polarized scattered wave (i.e. γqc, where c is either right-hand circular polarization, R, or left-hand circular polarization, L, and q is either vertical, v, or horizontal, h, polarization) and b) a linearly polarized incident wave and a circularly polarized scattered wave (i.e. γcq, where q is either vertical, v, or horizontal, h, polarization and c is either right-hand circular polarization, R, or left-hand circular polarization, L).

## A.1	Circularly Polarized (CP) incident to linear scattered power

In this case, the coherent bistatic scattering coefficient γpccks,ki components due to sea surface can be obtained from equation (11) through replacing rppθi by rpcθi with

rvRθi=12  rvvθi 	(a.1)

rhRθi= -j2 rhhθi	(a.2)

rvLθi=  12 rvvθi	(a.3)

rhLθi=  j2 rhhθi	(a.4)

and  j=-1.

The bistatic scattering coefficient γpclks,ki due to long gravity wave can be obtained from equation (32) through replacing the polarization factors Upqks,ki by the polarization factors Upcks,ki with:

UvRks,ki=12  Uvvks,ki-jUvhks,ki 	(a.5)

UhRks,ki= 12 Uhvks,ki-jUhhks,ki	(a.6)

UvLks,ki=  12 Uvvks,ki+jUvhks,ki	(a.7)

UhLks,ki=  12 Uhvks,ki+jUhhks,ki	(a.8)

The bistatic scattering coefficient γpcshks,ki due to short capillary wave can be obtained from equation (72) through replacing the polarization factors Gpqks,ki in equation (70) by the polarization factors Gpcks,ki with:

GvRks,ki= 12 Gvvks,ki-jGvhks,ki 	(a.9)

GhRks,ki=  12Ghvks,ki-jGhhks,ki	(a.10)

GvLks,ki=  12 Gvvks,ki+jGvhks,ki	(a.11)

GhLks,ki=  12Ghvks,ki+jGhhks,ki	(a.12)

## A.2	Linear incident to CP scattered power

For this case, the coherent bistatic scattering coefficient γcqcks,ki components due to sea surface can be obtained from equation (11) through replacing rppθi by rcqθi with:

rRvθi=12  rvvθi 	(a.13)

rLvθi= 12 rvvθi	(a.14)

rRhθi=  j2 rhhθi	(a.15)

rLhθi=  -j2 rhhθi	(a.16)

The bistatic scattering coefficient γcqlks,ki due to long gravity wave can be obtained from equation (32) through replacing Upqks,ki by Ucqks,ki with:

URvks,ki=12  Uvvks,ki+jUhvks,ki 	(a.17)

ULvks,ki= 12 Uvvks,ki-jUhvks,ki	(a.18)

URhks,ki=  12 Uvhks,ki+jUhhks,ki	(a.19)

ULhks,ki=  12 Uvhks,ki-jUhhks,ki	(a.20)

The bistatic scattering coefficient γcqsks,ki due to short capillary wave can be obtained from equation (72) through replacing Gpqks,ki in equation (70) by Gcqks,ki with:

GRvks,ki= 12 Gvvks,ki+jGhvks,ki 	(a.21)

GLvks,ki=  12Gvvks,ki-jGhvks,ki	(a.22)

GRhks,ki=  12 Gvhks,ki+jGhhks,ki	(a.23)

GLhks,ki=  12Gvhks,ki-jGhhks,ki	(a.24)

Attachment B
to Annex

Calculation of scattering coefficients between circularly polarized waves

This Attachment provides the calculations of the bistatic scattering coefficients between a circularly polarized incident wave and a circularly polarized scattered wave (i.e. γPQ, where P is either right-hand circular polarization, R, or left-hand circular polarization, L, and Q is either right-hand circular polarization, R, or left-hand circular polarization, L).

The coherent bistatic scattering coefficient γcccks,ki components due to sea surface can be obtained from equation (11) through replacing rppθi by rPQθi with:

rRRθi=12rvvθi+rhhθi 	(b.1)

rRLθi=12rvvθi-rhhθi 	(b.2)

rLRθi=12rvvθi-rhhθi 	(b.3)

rLLθi=12rvvθi+rhhθi 	(b.4)

The bistatic scattering coefficient γPQlks,ki due to long gravity wave can be obtained from equation (32) through replacing Upqks,ki by Uccks,ki with:

URRks,ki= 12 Uvvks,ki+Uhhks,ki+j Uhvks,ki-Uvhks,ki	(b.5)

URLks,ki=   12Uvvks,ki-Uhhks,ki+j Uhvks,ki+Uvhks,ki	(b.6)

ULRks,ki=   12Uvvks,ki-Uhhks,ki-j Uhvks,ki+Uvhks,ki	(b.7)

ULLks,ki=  12 Uvvks,ki+Uhhks,ki-j Uhvks,ki-Uvhks,ki	(b.8)

The bistatic scattering coefficient γPQsks,ki due to short capillary wave can be obtained from equation (72) through replacing Gpqks,ki in equation (70) by Gccks,ki with:

GRRks,ki= 12 Gvvks,ki+Ghhks,ki+j Ghvks,ki-Gvhks,ki	(b.9)

GRLks,ki= 12 Gvvks,ki-Ghhks,ki+j Ghvks,ki+Gvhks,ki	(b.10)

GLRks,ki=  12Gvvks,ki-Ghhks,ki-j Ghvks,ki+Gvhks,ki	(b.11)

GLLks,ki=  12Gvvks,ki+Ghhks,ki-j Ghvks,ki-Gvhks,ki	(b.12)

Attachment C
to Annex

Simple approximations

For incidence angles θi and scattering angles θs below 60 degrees, and for azimuth scattering angles φs within ±5 degrees of the forward scattering plane (φs=φi=0o), the cross polarized terms in equations (a.5) to (a.24) could be neglected leading to the simplifications reported below. In so doing, yields an error of less than 5%. Within the forward scattering plane (φs= φi= 0o), for the above incident and scattering angles, the error is negligible.

| RHCP incident and vertical scattered power | γvR=12γvv |
| --- | --- |
| RHCP incident and horizontal scattered power | γhR=12γhh |
| LHCP incident and vertical scattered power | γvL=12γvv |
| LHCP incident and horizontal scattered power | γhL=12γhh |
| Vertical incident and RHCP scattered power | γRv=12γvv |
| Horizontal incident and RHCP scattered power | γRh=12γhh |
| Vertical incident and LHCP scattered power | γLv=12γvv |
| Horizontal incident and LHCP scattered power | γLh=12γhh |

Attachment D
to Annex

Sea surface height spectra model

The sea surface height spectrum is divided into two spectra: large-scale (gravity) spectrum and small-scale (capillary) spectrum, which are denoted by Wlκ,ψ and Wsκ,ψ, respectively. The large-scale spectrum is noted for reference but is not directly needed. The small-scale spectrum is related to the sea surface directional spectrum, Wκ,ψ, as follows:

Wsκ,ψ=0if   κ<κdWκ,ψotherwise	(d.1)

where κd is the two-scale cut-off height wavenumber, and κ,ψ are the polar coordinates of the height wavenumbers in the upwind and crosswind directions associated with the rectangular coordinates of the surface slopes Su,Sc. The directional spectrum Wκ,ψ is as follows:

Wκ,ψ=κ-1Sκφκ,ψ	(d.2)

where Sκ is the isotropic (omni-directional) spectrum function, and φκ,ψ is the angular spreading function. The isotropic spectrum, Sκ, is given by:

Sκ=Bl+Bhκ3Gγexp-1.25κpκ2	(d.3)

and Bl is the wave spectrum for long gravity waves:

Bl=0.003Ω0.5U10ΩCκexp-Ω10κκp-1	(d.4)

G=1.7if  Ω<11.7+6.0 logeΩif  1≤Ω<52.7Ω0.57if  Ω≥5	(d.5)

γ=exp-κκp-122ξ2	(d.6)

ξ=0.081+4Ω-3if  Ω<50.16if  Ω≥5	(d.7)

Cκ=g1+κκm2κ	(d.8)

κp=gΩU102,  g=9.81,  κm=364.52	(d.9)

Bh is wave spectrum for short capillary waves:

Bh=0.5αm0.232Cκexp-0.25κκm-12	(d.10)

αm=0.014u0.232	(d.11)

u=U100.0010.81+0.065U10	(d.12)

and Ω is the inverse wave age. The sea is considered to be:

–	fully developed when Ω has values close to 0.84,

–	mature when Ω has values close to 1, and

–	young when Ω has values > 2.

U10 is the wind speed (typically between 3 m/s and 33 m/s) at a height of 10 m above the sea surface, and u is the friction velocity (i.e. the wind speed at the sea surface). γ in equation (d.3) is the water surface tension, and g in equation (d.5) is the water acceleration due to gravity in m/s2.

The angular spreading function φκ,ψ in equation (d.2) provides the azimuth dependence of the directional spectrum Wκ,ψ given by:

φκ,ψ=12π1+∆κcos2ψ	(d.13)

where 𝜓 corresponds to the upwind direction, and ∆κ is the angular dependence amplitude:

∆κ=tanhloge24+4ΩCκU102.5+0.13u0.2320.232Cκ2.5	(d.14)

Note that Cκ in equation (d.4) goes to ∞ when κ goes to zero. However, when κ goes to zero, Sκ also goes to zero. As a result, when κ=0, set Sκ=0 without further calculation.

Attachment E
to Annex

Interference power of a reflected signal from the sea surface into a receiver

## E.1 	Introduction

The bistatic scattered power Prp from the sea surface received by a receive antenna with linear polarization p is the sum of two components: a coherent component Pcp, and a diffuse (incoherent) component Pdp.

Prp=Pcp+Pdp	(e.1)

The following sections provide calculations of the received power for: a) the general case and b) approximations for the specific case of a transmitter in GEO orbit and a receiver in LEO orbit.

## E.2	Coherent received power

For identical transmit (incident) and receive (scattered) linear polarizations, p, the received coherent power, Pcp (W), is:

Pcp=PtpGtpLtp4πRt+Rr2λ2GrpLrpLd4π γppc    (W)	(e.2)

where:

Ptp :	transmitted power with polarization p (W)

Gtp :	gain of the transmit antenna in the direction of the reflection point on the surface of the Earth (linear)

Rt :	range between the transmitter and the specular reflection point on the surface of the Earth (m)

Rr :	range between the specular reflection point on the surface of the Earth and the receiving antenna (m)

Grp :	gain of the receive antenna in the direction of the specular reflection point on the surface of the Earth (linear)

Ltp :	atmospheric loss (e.g. the gaseous attenuation) along the path from the interferer to the specular reflection point. The gaseous attenuation, Agas (dB), can be calculated using Annex 1 or Annex 2 of Recommendation ITU-R P.676-12, in which case Ltp=10-Agas/10

Lrp  :	atmospheric loss (e.g. the gaseous attenuation) along the path from the specular reflection point to the receiving antenna. The gaseous attenuation, Agas (dB), can be calculated using Annex 1 or Annex 2 of Recommendation ITU-R P.676-12, in which case Lrp=10-Agas/10

λ :	wavelength (m)

Ld  :	divergence loss due to the Earth’s curvature, given by:

Ld=11+2Reacosθ1+2Recosθa	(e.3)

where:

a :	average radius of the Earth (6 371 000 m)

Re :	effective range Re=RtRrRt+Rr  (m)

θ :	incident zenith angle.

### E.2.1	Transmitter in GEO orbit and a receiver in LEO orbit: coherent component

For the specular reflected component, Rt+Rr is the total range and 4πRt+Rr2 is the spreading loss. PtpGtp is the equivalent isotopically radiated power (e.i.r.p.). For a transmitter in GEO orbit and a receiver in low earth orbit (LEO), the approximation that Rt+Rr≅Rt introduces a few percent error in the coherent term. In this case, equation (e.2) becomes:

Pcp=PtpGtpLtp4πRt2λ2GrpLrpLd  γppc   (W)	(e.4)

γppc is the coherent bistatic scattering coefficient given by equation (11) for linearly polarized incident and linearly polarized scattered waves. The corresponding polarization transformations in Attachments A, B and C can be used for other incident and scattered polarization pairs.

## E.3	Diffuse received power

The received diffuse power, Pdp (W), can be calculated using the conventional bistatic radar equation (e.g. equation (73) of Recommendation ITU-R P.452-16). The diffuse scattered power received by an antenna with polarization p due and transmit polarization q is:

Pdp= Ptq λ24π3 GrpkiGtqksrt2rr2Ltqki  γpqdifks,kicosθs  Lrpks dA     (W)	(e.5)

where:

Ptq :	transmit power with polarization q (W)

ki : 	incident direction at the scattering differential area

ks : 	scattered direction at the scattering differential area

Grpki : 	gain of the transmit antenna in the incident direction ki (linear)

Gtqks :	gain of the receive antenna in the receive direction ks (linear)

dA : 	differential scattering area within the surface (m2)

rt : 	range between the transmitter and the scattering differential area on the surface of the Earth (m)

rr: 	range between the scattering differential area on the surface of the Earth and the receiver (m).

### E.3.1	Transmitter in GEO orbit and a receiver in LEO orbit: diffuse component

The following approximation assumes the gain of the transmit antenna is constant over the receive antenna footprint (a reasonable assumption for a GEO transmitter with a moderate gain antenna and a LEO receiver with a high gain antenna), and the distances from the transmit antenna to the surface of the Earth and from the surface of the Earth to the receive antenna do not vary significantly over the receive antenna footprint. Then, equation (e.5) can be approximated by:

Pdp≅PtqGtqλ2LtqkiLrpks4πRt2γpqdifks0,ki0	(e.6)

where:

ki0 :	incident direction at the scattering differential area along the transmitting antenna main beam

ks0 : 	scattered direction at the scattering differential area along the receive antenna main beam

Rt : 	range between the transmitter and the centre of the footprint, on the surface of the Earth, illuminated by the transmit antenna (m).

The polarization transformations in Attachments A, B and C can be used to calculate γpqdifrt0,rr0 for incident and scattered polarizations other than linear-linear or circular-circular.

The upwind direction is the opposite direction that the wind is heading.

The unit ppt is parts per thousand.

The function atan2(y, x) is defined as the angle between the positive x axis and the vector from the origin to the point (x, y).