SECTION  5G:	PROPAGATION  DATA  REQUIRED  FOR  THE  EVALUATION  OF  INTERFERENCE:  SPACE  AND  TERRESTRIAL  SYSTEMS

RECOMMENDATION  ITU-R  PN.452-6

PREDICTION  PROCEDURE  FOR  THE  EVALUATION  OF  MICROWAVE
INTERFERENCE  BETWEEN  STATIONS  ON  THE  SURFACE  OF
THE  EARTH  AT  FREQUENCIES  ABOVE  ABOUT  0.7  GHz

(Question ITU-R 208/3)

(1970-1974-1978-1982-1986-1992-1994)

Rec. ITU-R PN.452-6

The ITU Radiocommunication Assembly,

considering

a)	that due to congestion of the radio spectrum, frequency bands must be shared between different terrestrial services, between systems in the same service and between systems in the terrestrial and Earth-space services;

b)	that for the satisfactory coexistence of systems sharing the same frequency bands, interference propagation prediction procedures are needed that are accurate and reliable in operation and acceptable to all parties concerned;

c)	that interference propagation predictions are required to meet “worst-month” performance and availability objectives;

d)	that prediction methods are required for application to all types of path in all areas of the world,

recommends

1.	that, for frequencies above about 0.7 GHz, the microwave interference prediction procedure given in Annex 1 be used for the evaluation of the available propagation loss in interference calculations between stations on the surface of the Earth.

ANNEX  1

# 1.	Introduction

Congestion of the radio-frequency spectrum has made necessary the sharing of many of the microwave frequency bands between different radio services, and between the different operators of similar radio services. In order to ensure the satisfactory coexistence of the terrestrial and Earth-space systems involved, it is important to be able to predict with reasonable accuracy the levels of interference that might exist between them, using prediction procedures and models which are acceptable to all parties concerned, and which have demonstrated accuracy and reliability.

Many types and combinations of interference path may exist between stations on the surface of the Earth and between these stations and stations in space, and prediction methods are required for each scenario. This Annex addresses one of the more important sets of interference problems, i.e., those situations where there is a potential for interference between microwave radio stations located on the surface of the Earth.

The prediction procedure detailed below is appropriate to terrestrial microwave link stations and satellite earth stations operating in the frequency range of about 0.7 GHz to 30 GHz. The method includes a comprehensive set of propagation models which ensure that the predictions embrace all the significant propagation mechanisms that can arise. Techniques for analysing the radio-meteorological and topographical features of the path are provided so that it is possible to generate a prediction for any practical type of interference path.

# 2.	Interference propagation mechanisms

Microwave interference propagation may arise through a range of propagation mechanisms whose individual dominance depends on many factors, including climate, radio frequency, time percentage of interest, distance and path topography. At any one time a single mechanism or more than one may occur.

The eight principal interference propagation mechanisms are outlined here, of which the first four may be regarded as long-term, or continuous, interference mechanisms (Fig. 1), and the remaining four as short-term, or anomalous, mechanisms (Fig. 2) usually generating much higher interfering signal levels.

FIGURE  1/PN.452...[D01] = 3 CM

## 2.1	Line-of-sight

The most straightforward interference propagation mechanism is line-of-sight under well-mixed atmospheric conditions. An additional complexity can, however, come into play when sub-path diffraction causes an increase in signal level.

## 2.2	Diffraction

The accuracy to which this mechanism can be modelled often determines the density of microwave systems that can be achieved in a given area. The diffraction prediction capability must have sufficient utility to cover smooth-earth, discrete obstacle and irregular terrain situations.

## 2.3	Tropospheric scatter

This mechanism defines the “background” interference level for longer paths (e.g., 100-150 km). The prediction requirement for this mode in the interference context is somewhat different from that needed for troposcatter link design as it is time percentages below 50% that are of interest, often on paths with a highly asymmetrical geometry and with the common volumes being generated by antenna side lobes.

FIGURE  2/PN.452...[D02] = 3 CM

## 2.4	Scatter from terrain and buildings (not illustrated in Fig. 1)

This has not hitherto been a problem, but may become important in the future if crossing paths and high-density networks of paths become more commonplace. This mechanism is not currently covered by this prediction procedure.

## 2.5	Enhanced line-of-sight

The interference propagation mechanism of unobstructed transmission on line-of-sight paths may sometimes have levels raised by multipath and focusing effects.

## 2.6	Surface super-refraction and ducting

This is the most important short-term interference mechanism over water and in flat coastal land areas.

## 2.7	Elevated layer reflection and refraction

The treatment of reflection and/or refraction from layers at heights up to a few hundred metres is of major importance as these mechanisms enable signals to bypass the diffraction loss of the terrain very effectively under favourable path geometry situations.

# 2.8	Hydrometeor scatter

Hydrometeor scatter is particularly important as a possible source of interference because it may act virtually omni-directionally .

A basic problem in interference prediction (which is indeed common to all tropospheric prediction procedures) is the difficulty of providing a unified consistent set of practical methods covering a wide range of distances and time percentages; i.e., for the real atmosphere in which the statistics of one mechanism merge gradually into another as meteorological and/or path conditions change. Especially in these transitional regions, a given level of signal may occur

for a time percentage which is the sum of those in different mechanisms. The approach in this procedure has been deliberately to keep separate the prediction of interference levels from the different propagation mechanisms up to the point where they can be combined into an overall prediction for the path. This is especially the case for the two somewhat different circumstances of clear-air and hydrometeor scatter conditions.

# 3.	Clear-air interference prediction

## 3.1	Global and European procedures

The text below provides two complementary versions of a comprehensive clear-air interference prediction procedure which takes into account the clear-air propagation mechanisms outlined in § 2 above. The first version, the “global” procedure (§ 3.2), can be applied (for all practical purposes) on a world-wide basis, and uses radio-meteorological data to describe the location variability of interference propagation conditions. The second version, the “European” procedure (§ 3.3) offers improved precision for North-West Europe, from where a significant amount of measured data has allowed more accurate determination of propagation characteristics based on land, sea and coastal zone concepts. The two procedures are essentially the same apart from the method of defining the radio-meteorological influences affecting the anomalous propagation conditions on the path.

The procedures below are intended for computer implementation, although it is possible to hand-work the important clear-air calculations for individual paths if required. Maps are used to provide essential input data, but the intention is that these be realized as simple databases for use by computer software. Appendix 1, which contains the maps, also suggests one convenient method of creating suitable databases.

## 3.2	Deriving a prediction using the global procedure

### 3.2.1	Outline of the procedure

The global procedure allows average year or (quasi) worst-month predictions to be derived for any path bounded by 80° North and 70° South. The North-South limits are imposed by the current availability of radio-meteorological data. The procedure embraces all the clear-air propagation mechanisms mentioned in § 2 above (with the exception of terrain and building scatter). The variations in propagation conditions around the world are described by means of maps of long and short term radio refractive index conditions. (The resolution and accuracy of the data in these maps is currently being improved for areas where better data have become available since the original work.)

There are seven stages to deriving a prediction using the global method:

Step 1:	Decide whether an average year or worst-month prediction is required.

Step 2:	Assemble the basic input data.

Step 3:	Derive the annual or worst-month radio-meteorological data from the maps provided.

Step 4:	Analyse the path profile, and classify the path according to the path geometry.

Step 5:	Identify which individual propagation models need to be invoked.

Step 6:	Calculate the individual propagation predictions using each of the models identified in Step 5.

Step 7:	Combine the individual predictions to give the overall statistics.

### 3.2.2	Choice of average year or worst-month prediction

The decision as to whether annual or “worst-month” predictions are required is dictated by the quality (i.e., performance and availability) objectives of the interfered-with radio system at the receiving end of the interference path. As interference is often a bi-directional problem, two such sets of quality objectives may need to be evaluated in order to determine the worst-case situation upon which the prediction needs to be based. In the majority of cases the quality objectives will be couched in terms of a percentage “of any month”, and hence worst-month data will be needed.

True worst-month data can only be provided when there exists an extensive database of long-term radio measurements (ideally 10 years or more). No such database exists for interference propagation data from around the world, and thus the global method has to rely on quasi worst-month data derived from long-term meteorological measurements, on the assumption that reliable relationships exist between the observed meteorological conditions and the actual propagation conditions. However, this approximation of the worst-month situation has proved realistic in tests against such measured data as are available, and provides prediction results far closer to operational requirements than have previously been available.

### 3.2.3	Assembling the input data

The basic input data required for the global procedure is given in Table 1:

TABLE  1

Basic input data

It should be noted that, for the interfering and interfered-with stations:

| Parameter | Preferred resolution | Description |
| --- | --- | --- |
| f | 0.01 GHz | Frequency (GHz) |
| p | 0.001% | Required time percentage(s) for which the calculated basic transmission loss is not exceeded |
| jt, jr | 0.001° | Latitude of station (degrees) |
| yt, yr | 0.001° | Longitude of station (degrees) |
| htg, hrg | 1 m | Antenna centre height above ground level (m) |
| Gt, Gr | 0.1 dBi | Antenna gain in the direction of the horizon along the great-circle interference path (dBi) |

t :	interferer

r :	interfered-with.

In addition, where the path is partly over the sea or other large areas of water, certain additional information about the great-circle interference path needs to be determined, as indicated in Table 2. All other information required to complete the prediction is derived from these basic data during the execution of the procedure.

### 3.2.4	Radio-meteorological data for the path

The global model uses two radio-meteorological parameters to describe the variability of background and anomalous propagation conditions at the different locations around the world.

–	DN, the average radio-refractive index lapse-rate through the first 1 km of the atmosphere, provides the data upon which the appropriate effective earth radius can be based for path profile and diffraction obstacle analysis. Figures 5 and 6, respectively, provide world maps of average annual values of DN and maximum monthly mean values of DN (for worst month prediction).

–	The parameter b0% is the time percentage for which super-refractive lapse-rates with gradients of modulus exceeding 100 N-units/km can be expected in the lower atmosphere, and is used as a measure of the incidence of fully developed anomalous propagation in the area under consideration. Figures 7 and 8, respectively, provide the annual and maximum monthly mean time percentages, b0% relevant to these anomalous propagation conditions.

TABLE  2

Additional input data required for paths with one or more sea sections

(1)	The exact values of dct and dcr are only of importance if in the range 0 km < dct; dcr £ 3 km. If, in either or both cases, the distances are obviously in excess of 3 km then it is only necessary to note the > 3 km condition for future reference. Few interference paths will in fact need detailed evaluation of these two parameters.

| Parameter | Preferred resolution | Description |
| --- | --- | --- |
| db | 1 km | Aggregate length of the path sections over water (km) |
| w | – | Fraction of the total path over water: w = db /d (1) where d is the great-circle distance (km) calculated using equation (29) in Appendix 2. For totally overland paths w = 0 |
| dct(1) | 0.1 km | Distance from the first terminal (the interference source) to the coast along the great-circle interference path (km) |
| dcr(1) | 0.1 km | Corresponding distance for the second (interfered-with) station (km) |

The correct input data for the prediction models is given by the average of the path-end and path centre values of DN and b0% appropriate to the path in question from Figs. 5 and 7 for any (average year) or Figs. 6 and 8 (for the worst month) as decided in the procedural step in § 3.2.2. However, in areas where the values are reasonably constant the path centre value alone can safely be used.

Using the radio-meteorological data, the median effective earth radius factor k50 for the path can be determined:

(2)

(Note that DN is considered to be a positive quantity in this procedure.)

When diffraction calculations need to be undertaken (see Table 4 for conditions), the basic transmission loss variability with time percentage due to the diffraction propagation mechanism is assumed to be the result of changes in bulk atmospheric radio refractivity lapse rate. As the time percentage, p, reduces, the effective earth radius factor k( p) is assumed to increase according to the function:

(2a)

Note that this expression is only valid for p% £ 50%.

Using the value of k50 found from equation (2), and assuming a true earth radius of 6 375 km, the median value of effective earth radius ae can be determined:

(3)

Once again, under the specific conditions which make necessary the diffraction calculations, a time-varying value of effective earth radius, a(p), is needed during the derivation of the diffraction geometry for each required time percentage:

(3a)

### 3.2.5	Calculating predictions for appropriate individual mechanisms

There are four clear-air propagation models provided within this procedure, each dealing with a particular propagation mechanism. These are summarized in Table 3.

TABLE  3

Propagation models

| Propagation mechanisms | Text section |
| --- | --- |
| Line-of-sight (including small time percentage signal enhancement due to multipath and focusing effects) | 4.2 |
| Diffraction (embracing irregular terrain, smooth-Earth, and sub-path effects) | 4.3 |
| Tropospheric scatter | 4.4 |
| Ducting/layer reflection | 4.5 |

Depending on the path characteristics, calculations need to be performed using one or more of these models. The type of path must be determined using a path profile.

### 3.2.6	Analysis of the path profile

Values for a number of path-related parameters must be found via an initial analysis of the path profile for p = 50%. The main information on the derivation, construction and analysis of the path profile is given in Appendix 2.

Having analysed the profile under median atmospheric conditions according to the method in Appendix 2, the path will have been classified into one of the three geometrical categories indicated in Table 4. This classification then applies for the remainder of the procedure.

TABLE  4

Interference path classifications and propagation model requirements

(1)	Sub-path diffraction is defined by a terrain incursion into the first Fresnel zone.

| Classification | Models required (with conditions) |
| --- | --- |
| Line-of-sight with first Fresnel zone clearance | Line-of-sight |
| Line-of-sight with sub-path diffraction(1) | Line-of-sight; diffraction |
| Trans-horizon | Diffraction (d £ 200 km and b0 < 50%) Troposcatter (q ³ 8 mrad) Ducting/layer reflection (d > 20 km) |

The remaining path-related parameters necessary for the calculations, as indicated in Table 5, will also have been determined from the path profile.

### 3.2.7	Calculation of individual propagation predictions

Table 4 indicates, for each type of path, the propagation models that are appropriate, and the conditions under which each should be exercised, in order to derive the correct overall prediction. The necessary equations for these individual mechanism predictions are to be found in the text sections indicated in Table 3.

In order to build an overall prediction, the individual propagation mechanism predictions must be calculated and combined (see § 3.2.8) for each time percentage p% for which a prediction is required.

TABLE  5

Output parameters required from the path profile analysis

(1)	See Appendix 2 for definitions.

| Path type | Parameter | Description |
| --- | --- | --- |
| All | d | Great-circle path distance (km) |
| Trans-horizon | dlt, dlr | Distance from the transmit and receive antennas to their respective horizons (km) |
| Trans-horizon | qt, qr | Transmit and receive horizon elevation angles respectively (mrad) |
| Trans-horizon | q | Path angular distance (mrad) |
| Trans-horizon | hte, hre | Effective heights(1) of antennas above the terrain (m) |
| Trans-horizon | hm | Terrain roughness(1) (m) |

Note 1 – Other profile parameters required for evaluation of diffraction over a line-of-sight path with subpath obstructions, or for a trans-horizon path, are described in Recommendation ITU-R PN.526.

### 3.2.8	Determine overall prediction

The method of combining the individual mechanism predictions into an overall path prediction depends on the type of path, as shown in Table 6.

Once this has been completed for all the required time percentages, the prediction is complete.

Note that discontinuities in the model have been found to be possible for paths of approximately 20 km distance and 8 mrad angular distance for path losses not exceeded for 0.001% time.

TABLE  6

Methods of deriving overall predictions

| Path type | Action required |
| --- | --- |
| Line-of-sight | None; the correct overall prediction is that supplied by the line-of-sight model |
| Line-of-sight with sub-path diffraction | The prediction is obtained by summing the losses given by the line-of-sight and (sub-path) diffraction models, i.e.: Lb( p) = Lb0( p) + Lds( p)mmmmmmmdB (4) where: Lb0( p) : predicted basic transmission loss not exceeded for p% of time given by the line-of-sight model Lds( p) : prediction for p% of time given by the sub-path diffraction loss element of the diffraction model |
| Trans-horizon | The overall prediction can be obtained for each time percentage by adopting the lowest basic transmission loss from the propagation models exercised, however, a smoother transition between mechanisms, better emulating what is observed in nature, can be obtained by applying the following ancillary algorithm: mmmmdB (5) where: Lbs( p), Lbd ( p) and Lba( p) : individual predicted basic transmission loss for p% of time given by the troposcatter, diffraction and ducting/layer reflection propagation models respectively. Note 1 – Where a model has not been proposed for a path (because the conditions given in Table 4 were not met), the appropriate term should be omitted from equation (5). |

## 3.3	Deriving a prediction using the “European” procedure

### 3.3.1	Outline of the European procedure

The “European” version of the procedure is based on a large amount of measured data from North-West Europe. Until future versions of the maps of the radio-meteorological data used for the global procedure can provide a similar resolution and accuracy (particularly for worst month), this European version of the procedure offers increased prediction precision for interference paths within the area bounded (approximately) by 12° W, 20° E, 45° N and 70° N.

The seven stages to deriving a prediction described for the global procedure (§ 3.2.1) still apply in this case. However two cases (Steps 1 and 3 relating to radio-meteorology) are modified for the European version. The straightforward changes are described in § 3.3.2 and 3.3.4.

### 3.3.2	Choice of average year or worst-month predictions (replaces procedure of § 3.2.2 of global method)

The European version basically predicts the annual distribution of basic transmission loss. For average year predictions the required values of p% are used directly in the prediction equations.

Should worst-month data be required (as is probable) the appropriate predictions are derived by converting the required worst-month time percentage(s), pm%, to the equivalent annual value(s), p%, using an empirically derived model. This worst-month conversion model is based on long term measured statistics from inland, sea and coastal paths in North-West Europe and is known to provide accurate results.

The annual equivalent, p%, of the required worst-month time percentage, pm%, can be determined from:

(6)

where:

w :	fraction of the path over water.

This value of p is then used for the prediction calculations, the resulting values being appropriate to the required worst-month time percentage, pm%.

### 3.3.3	Assembling the input data

The basic input information required is identical to that described in § 3.2.3.

### 3.3.4	Radio-meteorological data for the path (modifies § 3.2.4 of the global method)

The European procedure provides empirically derived functions in place of the maps to describe the location variability of the propagation conditions. These functions have also been derived from a substantial database of microwave measurements, and offer a higher confidence level and better spatial resolution than can be provided by the map derived radio-meteorological data.

For the area of Europe covered by this version of the procedure:

(7)

For the purposes of deriving an accurate value of b0 the Earth’s surface is divided up into three radio-climatic zones as shown in Table 7. A map showing these zones is provided in Fig. 10.

TABLE  7

European radio-climatic zones

(1)	The above cold sea definition correctly implies that only large bodies of water need be considered. However, the inclusion or omission of one or more small lakes, or islands, in the path analysis will not cause significant errors because the model is relatively insensitive to the aggregated zone distances.

| Zone type | Code | Definition |
| --- | --- | --- |
| Coastal land | A1 | Coastal land and shore areas, i.e. land adjacent to the sea up to an altitude of 100 m relative to mean sea or water level, but limited to a distance of 50 km from the nearest sea area |
| Inland | A2 | All land, other than coastal and shore areas defined as “coastal land” above |
| Cold sea | B | “Cold” seas, oceans and other large bodies of water (i.e., covering a circle of at least 100 km in diameter(1)) situated at latitudes above 30° (with the exception of the Mediterranean and the Black Sea) |

To determine the value of the radio-meteorological parameter b0 it is necessary to determine two additional distances dtm and dlm along the great-circle path based on these zones. These are defined in Table 8.

TABLE  8

Additional path parameters for the European procedure

| Parameter | Definition |
| --- | --- |
| dtm | The longest continuous land (inland + coastal) section of the great-circle path (km) |
| dlm | The longest continuous inland section of the great-circle path (km) |

Once dtm and dlm are known, b0% can be determined using:

(8)

where:

for  dtm  £  40 km	(8a)

for  dtm  >  40 km	(8b)

for  dlm  £  40 km	(8c)

for  dlm  >  40 km	(8d)

# 4.	Clear-air propagation models

## 4.1	General

The procedures above (both global and European) involve four separate propagation models to provide the components of the overall prediction. These propagation models, which are identical for both versions of the overall procedure, are provided in this section.

## 4.2	Line-of-sight propagation (including short-term enhancements)

The basic transmission loss Lb0( p) not exceeded for time percentage, p%, due to line-of-sight propagation is given by:

(9)

where:

Es( p) :	short-term enhancement due to multipath and focusing effects

(9a)

Ag :	total gaseous absorption

(9b)

where:

go and gw(r) :	specific attenuations due to dry air and water vapour, respectively, and are found from the equations in Recommendation ITU-R PN.676

r :	water vapour density

(9c)

w being the fraction of the total path over water.

The above expression for Es(p) provides a straightforward assessment of the enhancement to be expected under line-of-sight conditions. A more detailed analysis method offering a potentially more accurate worst-month prediction of the enhancement term is provided in Recommendation ITU-R PN.530.

## 4.3	Diffraction

Given a terrain profile, the total loss due to diffraction effects Ld (p) not exceeded for p% time is found using the method described in Recommendation ITU-R PN.526 § 4.5, for an effective earth radius a (p) as given by equation (3a), for values of p from 50% down to a minimum of b0 as given by equation (8) (for the European procedure) or Figs. 7 and 8 (for the global procedure) as appropriate.

For diffraction loss at time percentages smaller than b0, the value of p should be set equal to b0. This procedure applies only when p is used to calculate effective earth radius.

Note that Recommendation ITU-R PN.526 § 4.5 can be used for a line-of-sight path with sub-path obstruction, or for a trans-horizon path.

The basic transmission loss not exceeded for p% time for a diffraction path is then given by:

(9d)

where Es( p) and Ag are as given by equations (9a) and (9b).

Note that Es( p) is included in the diffraction calculation to avoid a discontinuity for paths which are marginally line-of-sight.

## 4.4	Tropospheric scatter

Calculation of the basic transmission loss due to tropospheric scatter (troposcatter) is accomplished by a model which derives the basic transmission loss Lbs( p), for any required values of time percentage less than or equal to 50% of time. To ensure continuity with the other propagation models, this model is considered to be valid for 0.001% £ p £ 50%.

This troposcatter prediction model has been derived for interference prediction purposes and is not appropriate for the calculation of propagation conditions above 50% of time affecting the performance aspects of trans-horizon radio-relay systems.

The basic transmission loss due to troposcatter not exceeded for any time percentage, p%, below 50%, Lbs( p) (dB), is given by:

(10a)

where:

k( f ) :	frequency dependence:

(10b)

(10c)

d :	distance (km)

q :	scatter angle (angular distance) (mrad)

N0 :	path centre sea-level surface refractivity derived from the map in Fig. 9

Lc :	aperture to medium coupling loss (dB)

(10d)

Ag :	total gaseous absorption (dB) for the troposcatter path:

(10e)

where:

go and gw(r) :	specific attenuations due to dry air and water vapour, respectively, and are found from the equations in Recommendation ITU-R PN.676

r:	water vapour concentration (g/m3) (3 g/m3 should be used for the complete path for the troposcatter propagation mechanism).

## 4.5	Ducting/layer reflection

The anomalous propagation (ducting and layer reflection) prediction is based on a cumulative distribution Lba( p) with a reference point (b, Lbr) which undergoes (independent) translations along the time percentage and basic transmission loss axes, depending on the path input parameters. b is referred to below as the “reference time percentage” and Lbr the “reference basic transmission loss”. Figure 3 shows the principle of operation of the model and the factors influencing the alignment of the reference point against the two axes (in practice the shape of the distribution is also slightly modified by the path input conditions).

Derivation of a prediction for this mode is straightforward, and involves three calculations for b, Lbr, and the cumulative distribution Lba( p) respectively.

FIGURE  3/PN.452...[D03] = 3 CM

Calculation of the reference time percentage, b% is effected using:

(11)

where:

m2 :	correction for path geometry

(12)

(13a)

(13b)

m3 :	correction for terrain roughness

for  hm  £  10 m	(14a)

for  hm  >  10 m	(14b)

for  d  –  dlt  –  dlr  £  40 km	(14c)

for  d  –  dlt  –  dlr  >  40 km	(14d)

Calculation of the reference basic transmission loss, Lbr , is achieved using:

(15)

where:

(16)

q¢ :	modified angular distance:

(17)

where:

:	modified transmitter and receiver horizon elevation angles, respectively.

To avoid “double accounting” with the site-shielding losses, (Ast and Asr),  are defined thus:

Ac :	antenna-propagation mode coupling correction:

(18)

Act and Acr :	over-sea surface-duct coupling corrections for the interfering and interfered-with stations respectively

(19a)

(19b)

where ht and hr are the heights of the transmit and receive antennas, respectively, above mean sea level.

It is useful to note the very limited set of conditions under which equation (19a) is needed. To assist in cases where it is appropriate the Act;cr function of equation (19a) is shown graphically in Fig. 4.

Ast and Asr :	site-shielding diffraction losses for the interfering and interfered-with stations respectively

(19c)

(19d)

Ag :	total gaseous absorption:

(20)

where:

go and gw(r) :	specific attenuations due to dry air and water vapour, respectively, and are found from the equations in Recommendation ITU-R PN.676

r :	water vapour density (g/m3)

(20a)

FIGURE  4/PN.452...[D04] = 3 CM

Points on the cumulative distribution Lba( p) are then given by:

(21)

where:

(21a)

The remaining terms have been defined in Tables 1, 2 and 5 and Appendix 2.

## 5.	Hydrometeor-scatter interference prediction

## 5.1	Introduction

This model, for which software is available from the ITU Radiocommunication Bureau, predicts transmission loss statistics of an interfering signal from rainfall rate statistics. It is based on two fundamental assumptions:

–	scattering occurs only within rain cells having circular cross-sections whose diameters depend on the rainfall rates inside the cell. For any link geometry, one cell is assumed to be in a fixed position which is intended to represent the worst case. Within the rain cell, the rainfall rate, and hence the reflectivity, is constant up to a rain height whose mean value depends on geographic latitude, but with a specific distribution about this mean that reflects the annual variation in height. Above the rain height, a linear decrease of the reflectivity (in dB) is assumed;

–	attenuation occurs inside as well as outside the cell, but only below the rain height. Inside the cell, the well known dependence of specific attenuation on the rainfall rate is assumed.

The model is able to compute interference levels for both long-path (> 100 km) and short-path geometries (down to a few kilometres) with arbitrary elevation angles at both terminals, as well as side scatter (non great-circle) geometries, and side-lobe coupling. Because of the complexity of the path geometry, it is available in the form of an example computer implementation.

The physical basis of the method is given in Appendix 3, where the intersection of a narrow beam (e.g., an earth station), and a wide beam (e.g., a terrestrial station) is assumed.

In comparative testing against both direct and radar-derived measurements, the method has yielded good results for a variety of frequencies and geometries.

## 5.2	Transmission loss formula

The transmission loss, L (dB), due to hydrometeor scatter, for a given rainfall rate, R, and rain height, hFR, may be expressed as follows:

(22)

where:

dT :	distance between stations via the scattering volume (km)

f :	frequency (GHz)

zR :	unit volume rain scatter reflectivity factor below the (top of) rain height, hFR (mm6/m3):

(23)

R( p) :	point rainfall rate exceeded for time percentage, p, of interest

S :	allowance for the deviation from Rayleigh law scattering in rain at frequencies above 10 GHz (assume S = 0 above hFR):

(24a)

(24b)

js :	scatter angle, i.e., angle between the directions of propagation of the waves incident on, and outgoing from, the scatter volume (for example, js = 0° for forward scatter, and js = 180° for back scatter)

Ag :	gaseous attenuation by oxygen and water vapour, calculated using the formulae of Recommendation ITU-R PN.676 and a water vapour density of 7.5 g/m3

hE :	antenna efficiency (factor < 1) of the “earth” station

C :	effective scatter transfer function:

(25)

where:

hmin and hmax :	minimum and maximum heights for integration

gT0 :	on-axis antenna gain factor of the terrestrial station

gT :	antenna directivity (factor £ 1) of the terrestrial station in the direction towards the integration point (volume element) under consideration

z(h) :	reflectivity function in the scatter volume, normalized to zR (the value below hFR)

(26)

(27)

AT, AE :	rain attenuation (factors £ 1) for the paths from the terrestrial station to the integration point and from there to the earth station, respectively.

Equation (22) assumes the worst case (maximum) polarization coupling. For less than this ideal coupling, additional loss will occur (see Appendix 3).

To calculate the overall transmission loss, the integration of equation (25) has to be performed along the part of the earth station’s antenna beam that is either, within the rain cell, or within the antenna beam of the terrestrial station (shown dark on Fig. 13).

The antenna beam of the terrestrial station is assumed to have a Gaussian shape. The integration is extended to the –18 dB points of the antenna pattern, relative to the maximum gain found along the narrow beam. To derive an analytical expression for the integral, the directivity pattern of the antenna of T is approximated by a Gaussian function as follows:

(28)

where Dh1/2 is the difference of the heights of those two points on the antenna axis of the earth station where the directivity function of the antenna of the terrestrial station falls to 0.5 (–3 dB points).

## 5.3	Cumulative distribution of transmission loss

The (annual) cumulative distribution of transmission loss due to hydrometeor scatter is evaluated in two steps.

Firstly, for each combination of rainfall rate and rain height, the transmission loss is calculated as if there were a deterministic dependence on these parameters. It is assumed that this value of transmission loss happens with the same probability as the corresponding combination of rainfall rate and rain height, assuming statistical independence of these two parameters.

Secondly, the probabilities of all rainfall-rate/rain-height combinations leading to the same transmission loss values are summed to yield the total probability for that transmission loss. The resulting cumulative distribution is taken as the predicted distribution although there is no one-to-one correlation between the individual rainfall-rate/rain-height combinations and the transmission loss.

## 5.4	Worst-month predictions

Worst-month scaling factors, Q, for hydrometeor scatter have been found to be about 3 at 1% time, 4.5 at 0.01% time and about 7 at 0.001% time in Europe. These are based on a limited amount of data at frequencies of 11 GHz and above, and should be used prudently. It was found that the mean values were very close to the European rainfall rate factors. Therefore, in the absence of measured Q values for precipitation scatter, it is suggested that the Q values for rainfall rate be used instead (see Recommendation ITU-R PN.841 for tables of Q values).

As an example, based on results for an 11 GHz path of 131 km in the United Kingdom, a transmission loss of 141 dB was exceeded for all but 0.01% of time on an annual basis. Using a Q factor of 4.5, the transmission loss exceeded for all but 0.002% of the annual distribution, 139.3 dB, would occur for 0.01% of the worst-month, a decrease in loss of 1.7 dB.

In principle, these worst-month factors should take complete account of the melting layer, but model calculations have suggested that the interference effect of this layer may be significant if it is centred on the common volume of the intersecting main beams for a significant period of time during, for example, a “worst-month”. The effect is frequency dependent, being more significant at lower frequencies, such as 4-6 GHz, and less so at high frequencies. The effect was observed on measurements on a 131 km path at 11.2 GHz (composite rain climate C, D, E) which indicated that the presence of the melting layer in the summer increased the interference level by about 2-3 dB at percentages of time between 0.1% and 0.01%, relative to the expected enhancement in the seasonal statistics in the absence of the melting layer. At 5 GHz, the scaled interference enhancement due to the presence of the melting layer would be 3-4 dB. Worst-month enhancements in interference resulting from the presence of the melting layer would be slightly larger than these worst-season enhancements.

APPENDIX  1

TO  ANNEX  1

Radio-meteorological data required for the clear-air prediction procedure

# 1.	Introduction

The clear-air prediction procedures rely on radio-meteorological data to provide the basic location variability for the predictions. These data are provided in the form of maps which are contained in this Appendix.

# 2.	Maps of vertical variation of radio refractivity data

For the global procedure, the clear-air radio-meteorology of the path is characterized for the continuous (long-term) interference mechanisms by the average annual value of DN (the refractive index lapse-rate over the first 1 km of the atmosphere) and for the anomalous (short-term) mechanisms by the time percentage, b0%, for which the refractive

gradient of the lower atmosphere is below -100 N-units/km. These parameters provide a reasonable basis upon which to model the clear-air propagation mechanisms described in § 2 of Annex 1. For each of these quantities, data are available in this Appendix for annual and quasi worst-month predictions:

–	Figure 5 provides average year DN data,

–	Figure 6 provides the associated “maximum monthly mean” DN contours,

–	Figure 7 shows the average year values of b0%, whilst

–	Figure 8 provides the related “maximum monthly mean” values for b0%.

The source data for the maximum monthly mean map of b0% (Fig. 8) offered only seasonal (every third month) values, and thus the quasi worst-month values quoted are based on the highest of the four seasonal values for each location.

FIGURE  5/PN.452...[D05] = 12.5 CM

FIGURE  6/PN.452...[D06] = 3 CM

FIGURE  7/PN.452...[D07] = 3 CM

FIGURE  8/PN.452...[D08] = 13.5 CM

# 3.	Map of surface refractivity, N0

Figure 9 provides a map of average sea-level surface refractivity, N0, for the troposcatter model.

# 4.	Map of European radio-meteorological zones

For the European version of the procedure, it is necessary to classify each great-circle interference path into sections falling into the inland, coastal or sea zones defined in Table 7. In Fig. 10 a map showing these zones for North-West Europe is provided.

# 5.	Implementation of maps in computer database form

For computer implementation of the procedures, it is convenient to capture these maps in digital form and to convert them into simple databases that can be accessed by the software.

For the global refractive index maps, it is suggested that the contours be converted into two-dimensional arrays of 0.5° ´ 0.5° latitude and longitude. To avoid discontinuities in the prediction with small changes in location or distance the values for each array cell should be derived by interpolation between the contours. Note that the contours are drawn to logarithmically scaled intervals, and therefore logarithmic interpolation is recommended.

The radio-climatic zone map in Fig. 10 can be captured in two-dimensional array form, but for this application a resolution of 0.1° ´ 0.1° latitude and longitude is recommended. For the smaller geographical area this does not represent storage or access time problems to a computer. Note that large areas of the map are either sea or inland zone, and for efficient and fast computations there is ample scope for indexing these larger homogeneous areas within a database structure.

FIGURE  9/PN.452...[D09] = 11 CM

FIGURE  10/PN.452...[D10] = 3 CM

APPENDIX  2

TO  ANNEX  1

Path profile analysis

# 1.	Introduction

For path profile analysis, a path profile of terrain heights above mean sea level is required. The parameters that need to be derived from the path profile analysis for the purposes of the propagation models are given in Table 5.

# 2.	Construction of path profile

Based on the geographical coordinates of the interfering (jt, yt) and interfered-with (jr, yr) stations, terrain heights (above mean sea level) along the great-circle path should be derived from a topographical database or from appropriate large-scale contour maps. The preferred distance resolution of the profile is that giving an integer number of steps of approximately 0.25 km. Other distance increments can be used, up to a maximum of about 1 km, with some possible decrease in prediction accuracy. The profile should include the ground heights at the interfering and interfered-with station locations as the start and end points. To the heights along the path should be added the necessary Earth’s curvature, based on the value of ae found in equation (3).

For the purposes of this Recommendation the point of the path profile at the interferer is considered as point zero, and the point at the interfered-with station is considered as point n. The path profile therefore consists of n + 1 points. Figure 11 gives an example of a path profile of terrain heights above mean sea level, showing the various parameters related to the actual terrain.

FIGURE  11/PN.452...[D11] = 3 CM

Table 9 defines parameters used or derived during the path profile analysis.

TABLE  9

Path profile parameter definitions

| Parameter | Description |
| --- | --- |
| ae | Effective Earth’s radius (km) |
| d | Great-circle path distance (km) |
| di | Great-circle distance of the ith terrain point from the interferer (km) |
| dii | Incremental distance for regular path profile data (km) |
| f | Frequency (GHz) |
| l | Wavelength (m) |
| hts | Interferer antenna height above mean sea level (m) |
| hrs | Interfered-with antenna height above mean sea level (m) |
| qt | Horizon elevation angle above local horizontal (mrad), measured from the interfering antenna |
| qr | Horizon elevation angle above local horizontal (mrad), measured from the interfered-with antenna |
| q | Path angular distance (mrad) |
| hst | Height of the smooth-Earth surface (amsl) at the interfering station location (m) |
| hsr | Height of the smooth-Earth surface (amsl) at the interfered-with station location (m) |
| hi | Height of the ith terrain point amsl (m) Note 1 – h0 : ground height of interfering station hn : ground height of interfered-with station |
| hm | Terrain roughness (m) |
| hte | Effective height of interfering antenna (m) |
| hre | Effective height of interfered-with antenna (m) |

# 2.	Path length

For general cases the path length, d (km), can be found from the path profile data:

(29)

however, for regularly-spaced path profile data this simplifies to:

(30)

where:

dii :	incremental path distance (km).

# 3.	Path classification

The path profile must next be used to classify the path into one of three geometrical categories based on an effective Earth’s radius of ae. The interference path classifications are as indicated in Table 4.

## 3.1	Classification Step 1: Test for a trans-horizon path

A path is trans-horizon if the physical horizon elevation angle as seen by the interfering antenna (relative to the local horizontal) is greater than the angle (again relative to the interferer’s local horizontal) subtended by the interfered-with antenna.

The test for the trans-horizon path condition is thus:

(31)

where:

mmmmmmmrad	(32)

qi :	elevation angle to the i th terrain height

(33)

where:

hts :	interferer antenna height (m) above mean sea level (amsl)

hi :	height of the i th terrain point (m) amsl

di :	distance from interferer to the i th terrain element (km)

(34)

where:

hrs :	interfered-with antenna height (m) amsl

d :	total great-circle path distance (km)

ae :	median effective Earth’s radius appropriate to the path (equation (3)).

If the condition of equation (31) is met, then the remaining path profile analysis required for trans-horizon paths can be undertaken, see § 4.1. Under these conditions Step 2 of the path classification is not needed.

If the condition of equation (31) is not fulfilled, the path is line-of-sight, with or without incursion by the terrain of the first Fresnel zone.

## 3.2	Step 2: Test for line-of-sight with sub-path diffraction (i.e., without full first Fresnel zone clearance)

A non trans-horizon path is line-of-sight with sub-path diffraction, if the elevation angle over the physical horizon, as seen by the interfering antenna (relative to the local horizontal), and allowing for clearance equal to the first Fresnel ellipsoid radius at the horizon point, is greater than the angle (again relative to the interferer’s local horizontal) subtended by the interfered-with antenna.

The path has sub-path diffraction if:

(35)

where:

mmmmmmmrad	(36)

To complete this test an extra term is therefore required in equation (33) to allow for the first Fresnel ellipsoid. Recommendation ITU-R PN.526, § 2, gives the radius of this ellipsoid, Ri, m, at any point along the path:

(37)

where:

f :	frequency (GHz).

The appropriate radius, Ri m, is added to each terrain height, hi m, in equation (33) yielding equation (38). Allowing for first Fresnel zone clearance,  , the terminal antenna elevation angle (rad) to the i th point is obtained from the following equation:

(38)

If the condition of equation (35) is met, then the remaining path profile analysis required for sub-path diffraction cases can be undertaken (see § 4.2 below).

If the condition of equation (35) is not fulfilled, the path is line-of-sight and no further path profile analysis is needed.

# 4.	Derivation of parameters from the path profile

## 4.1	Trans-horizon paths

The parameters to be derived from the path profile are those contained in Table 9.

### 4.1.1	Interfering antenna horizon elevation angle, qt

The interfering antenna’s horizon elevation angle is the maximum antenna horizon elevation angle when equation (32) is applied to the n - 1 terrain profile heights.

(39)

with qmax as determined in equation (32).

### 4.1.2	Interfering antenna horizon distance, dlt

The horizon distance is the minimum distance from the transmitter at which the maximum antenna horizon elevation angle is calculated from equation (32).

(40)

### 4.1.3	Interfered-with horizon elevation angle, qr

The receive antenna horizon elevation angle is the maximum antenna horizon elevation angle when equation (32) is applied to the n – 1 terrain profile heights.

mmmmmmmrad	(41)

(42)

### 4.1.4	Interfered-with antenna horizon distance, dlr

The horizon distance is the minimum distance from the receiver at which the maximum antenna horizon elevation angle is calculated from equation (32).

(43)

### 4.1.5	Angular distance q (mrad)

(44)

### 4.1.6	“Smooth earth” model and effective antenna heights

#### 4.1.6.1	General

To determine the effective antenna heights, and to allow an appropriate assessment of the path roughness to be made, it is necessary to derive an effective “smooth-earth” surface as a reference plane over which the irregular terrain of the path is deemed to exist. Once this is derived the values of the terrain roughness parameter (§ 4.1.6.4) and effective antenna heights for the interfering and interfered-with stations can be obtained.

#### 4.1.6.2	Exceptions

For straightforward “sea” paths, i.e., w ³ 0.9, and where both antenna horizons fall on the sea surface, the derivation of the smooth-earth surface calculation can be omitted if required. In such case the reference plane can be taken to be a mean sea (or water) level over the whole path, the terrain roughness may be assumed to be 0 m, and the effective antenna heights are equal to the real heights above the sea surface.

For all other paths it is necessary to apply the smooth-earth terrain approximation procedure detailed in § 4.1 and to derive the effective antenna heights and the terrain roughness as in § 4.1.6.4.

#### 4.1.6.3	Deriving the smooth-earth surface

Derive a straight line approximation to the terrain heights above mean sea level of the form:

(45)

where:

hsi :	height (m) amsl of the least-squares fit surface at distance di (km) from the interference source

hst :	height (m) amsl, of the smooth-earth surface at the path origin, i.e., at the interfering station

m :	slope (m/km) of the least-squares surface relative to sea level

mmmmmmm/km	(46)

where:

hi :	real height of the i th terrain point (m) amsl

ha :	mean of the real path heights amsl from h0 to hn inclusive (m):

(47)

(48)

and hence the height of the smooth-earth surface at the interfered-with station, hsr, is given by:

(49)

Correction must then be made if the smooth-earth heights fall above the true ground height, i.e.:

(50a)

(50b)

If either or both of hst or hsr were modified by equations (50a) or (50b) then the slope, m, of the smooth-earth surface must also be corrected:

(51)

#### 4.1.6.4	Terrain roughness, hm

The terrain roughness parameter, hm (m) is the maximum height of the terrain above the smooth-earth surface in the section of the path between, and including, the horizon points.

mmmmmmm	(52)

where:

ilt :	index of the profile point at distance dlt from the transmitter

ilr :	index of the profile point at distance dlr from the receiver.

The smooth-earth surface and the terrain roughness parameter hm are illustrated in Fig. 12.

FIGURE  12/PN.452...[D12] = 3 CM

APPENDIX  3

TO  ANNEX  1

Physical basis of the hydrometeor-scatter model

Scattering is assumed to occur only within one fixed, cylindrical rain cell of circular cross-section. The diameter of the cell, dc, depends on rainfall rate R (mm/h) as:

(53)

The cell is centred at the intersection (or point of closest approach if the beams do not exactly intersect) of the two antenna axes (see Fig. 13). This is necessary as a consequence of the inclusion of short-path geometries. Within the cell and for a given rain rate, the reflectivity is taken to be constant up to the freezing level height (or rain height), hFR. Above hFR it decreases at a rate of 6.5 dB/km. The rain outside the rain cell is not assumed to contribute significantly to scattering.

FIGURE  13/PN.452...[D13] = 3 CM

Rain attenuation is assumed to act only below the rain height. Inside the cell the specific attenuation, gR , is applied according to the formula gR = k Ra . The coefficients k and a depend on the frequency, polarization and direction of propagation (elevation angle) of the wave and are given in Recommendation ITU-R PN.838. For the attenuation, GR, outside the rain cell (between the edge of the cell and a point at distance d) an exponential decrease is assumed as follows:

(54)

where:

rm :	the scale length for rain attenuation, is given by:

(55)

e :	elevation angle.

Equation (54) is valid if the whole path is below the rain height. If only part of it (between distances d1 and d2 from the edge of the cell) is below the rain height:

(56)

For those portions of the propagation path that are above hFR, zero rain attenuation is assumed.

Because the horizontal scale length increases rapidly with decreasing rainfall rate, the distinction between the cell and the surrounding rain will become blurred for rainfall rates below some 10 mm/h.

The rain height is treated as a random variable with a given distribution function. It is important to explicitly include the variability of the (top of) rain height, hFR, in the prediction method, this being a major factor. Due to the rain attenuation acting only below hFR, a common volume just above hFR may result in a considerably lower transmission loss (higher interference signal) than one below hFR, despite the decrease of the reflectivity above hFR. It is only in the case of a common volume sufficiently high above hFR that the absence of rain attenuation dominates the reflectivity decrease, and the transmission loss increases again. If the rain height is above the scatter volume for some part of the year and below it otherwise, the annual transmission loss statistics in the low-probability, low-loss range might be dominated by scattering from snow and ice, which would not be predicted if a fixed rain height above the scatter volume were used. This effect is the more pronounced for higher frequencies, and might be negligible at frequencies below 11 GHz.

The mean value of the distribution, hFRm, is taken to depend on the station’s geographic latitude, j (degrees). Recommendation ITU-R PN.839 gives the mean value of hFR.

The distribution function is given in Table 10, but locally-derived long-term values should be used if they are available.

TABLE 10

Cumulative distributions of rain height, relative to the median value

| Rain height difference (km) | Exceedence probability (%) |
| --- | --- |
| –2.125 | 100.0 |
| –1.875 | 199.1 |
| –1.625 | 196.9 |
| –1.375 | 191.0 |
| –1.125 | 180.0 |
| –0.875 | 168.5 |
| –0.625 | 156.5 |
| –0.375 | 144.2 |
| –0.125 | 133.5 |
| –0.025 | 128.5 |
| –0.125 | 124.0 |
| –0.375 | 116.3 |
| –0.625 | 110.2 |
| –0.875 | 116.1 |
| –1.125 | 113.4 |
| –1.375 | 111.8 |
| –1.625 | 110.9 |

The procedure should be used with the rainfall rate distribution of the relevant rain climatic zone as given in Recommendation ITU-R PN.837, unless a locally measured rainfall rate distribution is available. Each rainfall rate value will yield its own transmission loss value, which for low rainfall rates decreases with increasing rainfall rate, due to the increasing reflectivity. But for a common volume below the rain height, hFR, the rain attenuation also increases with increasing rainfall rate, and will eventually outweigh the increasing reflectivity. Hence, the minimum transmission loss (on a path below hFR) will be found at some intermediate rainfall rate which decreases as the frequency increases. Therefore, the conversion of the transmission loss as a function of the rainfall rate into a cumulative distribution function needs an extra step in the prediction (see § 5.3).

A “narrow-beam” approximation is used in which it is assumed that one of the antenna beams (i.e. that of the earth station) is much narrower than the other (in the vicinity of the intersection) and so, the effects of antenna gain and beamwidth cancel each other out. However, the efficiency of the narrow-beam antenna, hE, must be included. (A default value of 0.6 is used if no value is provided.)

A Gaussian-shaped pattern for the wide-beam antenna is assumed. To account for the effect of side-lobe coupling, the side lobes of the wide-beam antenna are simulated by a second Gaussian-shaped beam which is much lower and broader than, and is superimposed on, the main lobe. The gain of the second beam is 1 dB below the peak of the highest side lobe, and its width is so chosen that, at the position of the highest side lobe, its gain is 3 dB below that peak. In other words: gain, g*, and (double-sided) half-power beamwidth, j*, of the side-lobe pattern are defined by:

(57)

(58)

where:

g :	main-lobe axial gain (dBi)

dg :	relative gain of the highest side lobe (dB relative to g)

d j :	position (off-axis angle) of the highest side lobe (degrees).

The on-axis gain of the main lobe must be reduced slightly to account for the superimposed side-lobe beam, such that the sum of the two gains corresponds to the actual gain of the antenna.

Polarization coupling in equation (22) is assumed to be a maximum. In practice, rain scatter is anisotropic and the polarization of the scattered interfering signal may be different from that of the receiving antenna. The relation between these two polarizations will, in general, be complex, depending on the polarizations of the transmitting and receiving antennas, the overall geometry of the system and, to some extent, the shape of the scattering particles. If a preliminary calculation for cases of main-beam-to-main-beam coupling indicates the possibility of an interfering situation, it would in theory be possible to evaluate the coupling more precisely taking into account the above parameters.

_________________