RECOMMENDATION  620-1

PROPAGATION  DATA  REQUIRED  FOR  THE  CALCULATION  OF  COORDINATION
DISTANCES  IN  THE  FREQUENCY  RANGE 1-40 GHz

(1986-1992)

Rec. 620-1

The CCIR,

considering

a)	the terms of Resolution No. 60 of the World Administrative Radio Conference, (Geneva, 1979);

b)	that the coordination area is that area, around an earth station, so defined that any interference between the earth station in question and terrestrial stations outside this area may be considered as negligible;

c)	that the determination of the coordination area should be based on the best propagation data available and should be adequately conservative,

recommends

that, for the determination of the coordination area with respect to frequencies above 1 GHz, administrations use the propagation calculation methods set out in Annex 1.

ANNEX  1

# 1.	Introduction

This Annex provides propagation data for use in the calculation of a coordination area and sets out a straightforward method for the assessment of the propagation factors concerned in the determination of coordination distances.

The determination of coordination distance necessitates the comparison of the required transmission loss, based on system and interference model considerations, with the transmission loss contributed by the propagation medium. If the application of this method reveals that a potential coordination problem exists, then evaluation should be carried out as described in Recommendation 452.

# 2.	General considerations

The determination of coordination distance propagation characteristics for an earth station is based here on the assumption that:

–	the locations of terrestrial stations with which coordination is to be sought are not known;

–	in the interference path geometry, only information pertaining to the earth station is available;

–	for the geometry over the remainder of the interference path, cautious limiting assumptions must be made as shown in the following text.

In this Annex propagation phenomena are classified into two modes as follows:

–	Mode 1:  propagation phenomena in clear air, and affected by the presence of the Earth’s surface (diffraction, refraction, ducting). These phenomena are confined to propagation along the great-circle path;

–	Mode 2:  hydrometeor scatter which is not limited to the great-circle path, but is, as dealt with in this Annex, limited to earth stations operating with geostationary satellites.

For each azimuth from the earth station, and for each of the above two modes of propagation, it is necessary to determine a distance corresponding to the required transmission loss; the “coordination distance” will be the greater of the two distances found. Recommendation 359 provides full information on the required transmission loss.

Because of the risk of error in predicting transmission loss over short distances when the exact path geometry is unknown, the minimum coordination distance in all cases is considered to be 100 km.

The 100 km minimum coordination circle should be centred at a distance Dd along the earth station beam axis, where Dd is given by:

(1)

where:

es :  earth-station antenna main beam elevation angle (degrees).

The methods described in this Annex refer to the average annual statistics. If the worst-month case is needed, then convert the required worst month time percentage to the equivalent annual statistics using the method given in Recommendation 841 for the category GLOBAL.

# 3.	Determination of coordination distance for propagation Mode 1 – Great-circle propagation mechanisms

## 3.1	Radio-climatic zones

In the calculation of coordination distance for propagation Mode 1, the world is divided into four basic radio-climatic zones. These zones are defined as follows:

–	Zone A1:  coastal land and shore areas, i.e., land adjacent to a Zone B or Zone C area (see below), up to an altitude of 100 m (see Note 1) relative to mean sea or water level, but limited to a maximum distance of 50 km from the nearest Zone B or Zone C area as the case may be.

Note 1 – In the absence of precise information on the 100 m contour, an approximation (e.g., 300 feet) may be used. If no suitable contour is available, the 50 km distance limit from the coast line or shore line is applicable;

–	Zone A2:  all land, other than coastal land and shore defined as Zone A1 above;

–	Zone B:  “cold” seas, oceans and other large bodies of water (i.e., covering a circle at least 100 km in diameter) situated at latitudes above 30°, with the exception of the Mediterranean and the Black Sea;

–	Zone C:  “warm” seas, oceans and other large bodies of water (i.e., covering a circle at least 100 km in diameter) situated at latitudes below 30°, as well as the Mediterranean and the Black Sea.

## 3.2	Calculation of coordination distance for paths within a single radio-climatic zone

For the minimum permissible basic transmission loss, Lb(p) for p% of the time, the coordination distance in each radio-climatic zone is derived using the method described below.

Where the distance derived lies entirely within the boundary of the radio-climatic zone appropriate to the earth station, that distance is taken as the actual coordination distance for propagation Mode 1. If the distance extends beyond the boundary of one radio-climatic zone, the overall coordination distance is obtained using the method given in § 3.3.

Note 1 – Under certain conditions, depending on the time dependency of the minimum basic transmission loss criteria, the coordination distance for Mode 1 (as in Mode 2) may not correspond to the lowest time percentage. When the difference in the criteria between 0.005% and 20% of the time exceeds 15 dB, coordination distances should be evaluated over the complete time percentage range. The required distance will be the maximum of the values found.

The minimum permissible basic transmission loss is related to coordination distance by the following expression:

(2)

in which:

(2a)

with:

g :	rate of attenuation (dB/km)

d1 :	coordination distance for propagation Mode 1 (km)

Ah :	horizon angle correction (dB)

f :	frequency (GHz).

The horizon angle correction is given by:

with:

e :	horizon angle** (degrees).

From equation (2) the coordination distance d1 may be found as follows, using the value of Lb ( p) for small time percentages:

(4)

The value of g depends on the radio-climatic zone and the percentage of time p, and is the sum of four components:

(5)

in which:

gz :	rate of attenuation (dB/km) due to all effects except atmospheric gases

gw :	rate of attenuation (dB/km) due to atmospheric water vapour

go :	rate of attenuation (dB/km) due to dry air.

The rate of attenuation gz depends on the radio-climatic zone, the frequency, and the percentage of time, and is to be determined from:

(6)

where the values of C1, C2, C3 and C4 for each zone are according to Table 1, and p is the time percentage (0.001 < p < 1%) of interest.

The rate of attenuation for water vapour, gw, depends on the frequency and the density of water vapour in the air. gw may be neglected when f < 10 GHz.

TABLE  1

The values of C1, C2, C3 and C4 for the different zones

|  | C1 | C2 | C3 | C4 |
| --- | --- | --- | --- | --- |
| Zone A1 | 0.03 | 0.03 | 0.15 | 0.2 |
| Zone A2 | 0.04 | 0.05 | 0.16 | 0.1 |
| Zone B | 0.015 | 0.015 | 0.05 | 0.15 |
| Zone C | 0 | 0.015 | 0.04 | 0.15 |

The water vapour density, r (g/m3), depends on the radio-climatic zone, as follows:

Zone A2: 		r  =  7.5 g/m3

Zones A1 and B:	r  =  10 g/m3

Zone C: 			r  =  10 g/m3

The rate of attenuation for dry air, go depends on frequency.

gw  and  go can be found from the equations in Recommendation 676.

Thus the coordination distance for propagation Mode 1 is derived for the appropriate climatic zone, frequency, percentage of time and horizon angle using equations (2) to (6).

## 3.3	Mixed paths

If the distance being calculated extends through more than one radio-climatic zone (mixed path), the prediction is made as follows:

Successive path sections in different zones are designated by the suffixes i, j, k... For the path section in which the earth station is located:

(7)

where gi is the rate of attenuation in the first zone (i).

Now, in the direction considered, if the value di is greater than the actual distance Di in the first zone (i), it follows that:

(8)

and so dj is found. If the value dj is greater than the actual distance Dj of the path in the second zone ( j), it can then be stated that:

(9)

from which dk may be found. This method may be extended as necessary, and in the case given the total distance d1 may now be expressed as:

(10)

## 3.4	Maximum coordination distance for propagation Mode 1

The maximum coordination distance for propagation Mode 1 is to be taken as 1 200 km.

# 4.	Determination of the coordination contour for propagation Mode 2 – Scattering from hydrometeors

The determination of the coordination contour for scattering from hydrometeors (e.g., rain-scatter) is predicted on a path geometry which is substantially different from that of the great-circle propagation mechanisms. As a first approximation, energy is scattered isotropically by rain, so that interference may result for large scattering angles, and for beam intersections away from the great-circle path.

For this propagation mode the previous classification of the terrain into Zones A1, A2, B and C is no longer appropriate.

As noted in § 2, the minimum coordination distance is 100 km. This is considered to provide adequate protection for the general case of interference from hydrometeor scatter and specific cases need only be evaluated within this distance from the earth station. However, there may be particular combinations of system parameters, i.e., high interfering transmitter powers and/or a low permissible interfering power at the interfered-with receiver, which result in the latter station requiring additional protection from hydrometeor-scatter interference.

Therefore, where the required transmission loss exceeds the value of Table 2 for the frequency band and rain climate zone (see Appendix 1) appropriate to the earth station, the procedure given in Appendix 2 should be used to create the appropriately-extended Mode 2 contour.

TABLE  2

Transmission loss thresholds (dB) for Mode 2 extended contour calculations

| - Frequency band (GHz) | Rain climate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | A, B | C, D, E | F, G, H, J, K | L, M | N, P, Q |
| - 11 | 152 | 148 | 144 | 141 | 136 |
| - 14 | 140 | 136 | 132 | 129 | 125 |
| - 16 | 138 | 134 | 130 | 127 | 124 |
| - 18 | 136 | 132 | 129 | 126 | 124 |
| - 10 | 135 | 131 | 129 | 127 | 126 |
| - 12 | 134 | 131 | 129 | 127 | 126 |
| - 14 | 135 | 132 | 130 | 128 | 127 |
| - 18 | 138 | 136 | 134 | 132 | 131 |
| - 20 | 144 | 142 | 140 | 139 | 137 |
| - 22.4 | 153 | 151 | 149 | 148 | 146 |
| - 25 | 149 | 147 | 145 | 144 | 142 |
| - 28 | 147 | 145 | 143 | 141 | 139 |
| - 30 | 147 | 145 | 143 | 141 | 140 |
| - 35 | 151 | 149 | 147 | 145 | 143 |
| - 40 | 157 | 155 | 153 | 151 | 149 |

If a worst-month coordination contour is required, the transmission loss thresholds in Table 2 should be correspondingly reduced by 2 dB.

APPENDIX  1
TO  ANNEX  1

Classification of rain climates

As shown in Recommendation 837 the world has been divided into a number of rain climatic zones which show different precipitation characteristics. The curves shown in Fig. 1 represent consolidated rainfall-rate distributions, each applicable to several of these rain climates, and are based on rainfall rate distributions as defined in Recommendation 837.

Figure  1  [D01]  =  12.5 cm  =  489%

The distributions of Fig. 1 have been extended beyond 0.3% to such greater percentages of time pc at which the rainfall rate is assumed to approach zero, using the expression:

(11)

and using, for R(0.3%) and pc, the following values:

| Rain climatic zone | - R(0.3%) (mm/h) | - pc (%) |
| --- | --- | --- |
| - A, B | - 01.5 | - 2 |
| - C, D, E | - 03.5 | - 3 |
| - F, G, H, J, K | - 07.0 | - 5 |
| - L, M | - 09.0 | - 7.5 |
| - N, P, Q | - 25.0 | - 10 |

This approach is appropriate for the numerical evaluation of the rain-scatter distance. It is, however, of concern only for frequencies above 8 GHz.

For numerical evaluation of R( p) in the range 0.001< p <0.3%, the curves in Fig. 1 have been converted into equations (12) to (16).

# Climates	A, B

(12)

# Climates	C, D, E

(13)

# Climates	F, G, H, J, K

(14)

# Climates	L, M

(15)

# Climates	N, P, Q

(16)

APPENDIX  2
TO  ANNEX  1

Calculation of contours for propagation Mode 2

The material is presented in terms of the basic equations for the relationship between rainfall rate, transmission loss and rain-scatter distance. The equations allow transmission loss to be expressed versus rainfall rate for any given distance, with the cumulative time distributions of rainfall rate in various “rain climatic zones” given in Appendix 1.

The transmission loss may be calculated as a function of distance r (km) (see Note 1), frequency f (GHz) and surface rainfall rate R (mm/h) from:

(17)

where:

R :	surface rainfall rate (mm/h), as given in Appendix 1 for various rain-climatic zones

gT :	gain of the terrestrial station antenna (dB), assumed to be 42 dB

Ab  	is given by:

C	is given by:

g R	is given by:

gR  =  k R ammmmmmdB	(19b)

Note 1 – r is the distance between the region of maximum scattering and the location of an eventual terrestrial station.

Table 3 gives values for k and a for vertical polarization (which yields minimum specific attenuation).

TABLE  3

Values of k and a for vertical polarization
as a function of the frequency

| - Frequency (GHz) | - k | - a |
| --- | --- | --- |
| - 1 | - 0.000 0352 | - 0.880 |
| - 4 | - 0.000 591 | - 1.075 |
| - 6 | - 0.001 55 | - 1.265 |
| - 8 | - 0.003 95 | - 1.31 |
| - 10 | - 0.008 87 | - 1.264 |
| - 12 | - 0.016 8 | - 1.20 |
| - 14 | - 0.029 | - 1.15 |
| - 18 | - 0.055 | - 1.09 |
| - 20 | - 0.069 1 | - 1.065 |
| - 22.4 | - 0.090 | - 1.05 |
| - 25 | - 0.113 | - 1.03 |
| - 28 | - 0.150 | - 1.01 |
| - 30 | - 0.167 | - 1.00 |
| - 35 | - 0.233 | - 0.963 |
| - 40 | - 0.310 | - 0.929 |

Further:

(20)

(21)

E :	loss in scatter coupling for heights above the melting layer, given by:

where:

HFR :	mean annual rain height (km) in the region of the earth station, as defined in Recommendation 839.

The equations for gaseous specific attenuation, go (for dry air) and gw (for water vapour), are given in Recommendation 676. The water vapour specific attenuation gw is to be calculated for an assumed water vapour density of r = 7.5 g/m3.

Equation (17) allows transmission loss, L, to be obtained as a monotonic function of  rainfall rate, R, and with the hydrometeor scatter distance, r, as a parameter. The procedure to determine the hydrometeor scatter contour is as follows:

a)	The value of R should be found for the required time percentage, p, and the appropriate rain climate A to Q from the data in Appendix 1.

b)	Values of L are then calculated for incremental values of r, starting at 100 km. The correct value of r is that for which the corresponding value of L equals or exceeds the required transmission loss. This value of r is denoted dr.

c)	If the iterative calculation results in r equalling or exceeding the appropriate maximum distance given in Table 4, then the calculation is terminated and dr is assumed to have this maximum value.

d)	Determine a point at distance Dd along the earth-station beam azimuth. This can be obtained from the following formula:

TABLE  4

Maximum hydrometeor scatter distances (km)

| Latitude (degrees) | 0-30 | 30-40 | 40-50 | 50-60 | > 60 |
| --- | --- | --- | --- | --- | --- |
| Distance (km) | 350 | 360 | 340 | 310 | 280 |

(25)

where:

es :	earth-station antenna main beam elevation angle (degrees).

e)	Draw a circle of radius dr around this point. This is the hydrometeor scatter coordination contour (coordination contour for propagation Mode 2). The coordination distance for propagation Mode 2 on a given azimuth from the earth station is the distance from the earth station to the coordination contour on that azimuth, denoted d2.

As the only significant hydrometeor scatter is that occurring in the general vicinity of the earth station, the question of a mixed path loss does not arise.

_________________

*	*	Horizon angle is defined here as the angle viewed from the centre of the earth-station antenna, between the horizontal plane and a ray that grazes the visible physical horizon in the direction concerned. It is necessary to determine horizon angles for all azimuths around an earth station. In practice it will generally suffice to do this in azimuth increments of 5°. However, every attempt should be made to identify and take into consideration minimum horizon angles that may occur between those azimuths examined in 5° increments.