RECOMMENDATION  ITU-R  P.620-2

PROPAGATION  DATA  REQUIRED  FOR  THE  EVALUATION  OF  COORDINATION
DISTANCES  IN  THE  FREQUENCY  RANGE 1-40 GHz

(Question ITU-R 208/3)

(1986-1992-1995)

Rec. ITU-R P.620-2

The ITU Radiocommunication Assembly,

considering

a)	the terms of Resolution No. 60 of the World Administrative Radio Conference (Geneva, 1979);

b)	that the coordination area is that area, around an earth station, so defined that any interference between the earth station in question and terrestrial stations outside this area may be considered as negligible;

c)	that the determination of the coordination area should be based on the best propagation data available and should be adequately conservative,

recommends

1	that, for the determination of the coordination area with respect to frequencies above 1 GHz, administrations use the propagation calculation methods set out in Annex 1.

ANNEX  1

# 1	Introduction

This Annex provides propagation data for use in the calculation of a coordination area and sets out a straightforward method for the assessment of the propagation factors concerned in the determination of coordination distances.

The coordination area represents the area, outside of which, interference between the earth station and terrestrial stations operating within the conservative assumptions given in Tables 1 and 2 of Recommendation ITU-R IS.847 may be considered negligible. The determination of coordination distance therefore necessitates the comparison of the required transmission loss, based on system and interference model considerations, with the transmission loss contributed by the propagation medium. The required distance is that which makes these two losses equal.

It is important to note that the coordination area does not represent a zone within which the sharing of frequencies between the earth station and the terrestrial station is excluded. Such sharing is often possible, and the coordination area serves to assist this arrangement by indicating where the potential for interference between the earth station and any terrestrial stations needs to be evaluated using Recommendation ITU-R P.452.

In addition to providing the method of calculation for the primary coordination contour, this Recommendation now provides information in § 5 that enables the preparation of auxiliary contours to assist in the rapid elimination of the majority of potential interference cases during the subsequent coordination analysis for terrestrial stations falling within the primary contour.

The methods described in this Annex refer to average annual statistics. If the worst-month case is needed, the required worst-month time percentage should be converted to the equivalent annual statistics using the method given in Recommendation ITU-R P.841 for the category GLOBAL.

# 2	General considerations

The determination of coordination distance propagation characteristics for an earth station is based here on the assumption that:

–	the locations of terrestrial stations with which coordination is to be sought are not known;

–	in the interference path geometry, only information pertaining to the earth station is available;

–	for the geometry over the remainder of the interference path, cautious limiting assumptions must be made as shown in the following text.

In this Annex propagation phenomena are classified into two modes as follows:

–	mode 1:  propagation phenomena in clear air, and affected by the presence of the Earth’s surface (diffraction, refraction, ducting). These phenomena are confined to propagation along the great-circle path;

–	mode 2:  hydrometeor scatter which is not limited to the great-circle path, but is, as dealt with in this Annex, limited to earth stations operating with geostationary satellites.

For each azimuth from the earth station, and for each of the above two modes of propagation, it is necessary to determine a distance corresponding to the required transmission loss; the “coordination distance” will be the greater of the two distances found. Recommendation ITU-R IS.847 provides full information on the required transmission loss.

The coordination distance in any given direction is determined by a number of factors set out above and, based on propagation factors alone, the distances could extend from relatively close-in to the earth station to many hundreds of kilometres. However, for practical reasons it is necessary for both modes of propagation to set lower limits to the distances within which to search for potential interference situations. These distances are defined as the mode 1 and mode 2 minimum coordination distances respectively.

In the case of mode 1, the requirement for the precautionary lower limit comes about because of the risk of significant errors in determining basic transmission loss (especially due to the diffraction propagation mechanism) over relatively short distances when the details of the actual path geometry are not known. The coordination distance should therefore always be kept sufficiently large to ensure that the diffraction fields will be negligible under all likely practical path conditions.

For mode 2 the requirement arises because the assumptions inherent in the hydrometeor scatter propagation modelling become unreliable at very short distances. By keeping the coordination distances at or beyond a pre-set minimum the model is not exercised in the region where it could become misleading.

Values for the appropriate minimum coordination distances are provided for each mode in the relevant sections below. The overall coordination contour traced out by the coordination distances determined radially around the earth station could of course be at all points greater than the “minimum distances” if the propagation calculations so determine.

# 3	Determination of coordination distance for propagation mode 1 – great-circle propagation mechanisms

## 3.1	Radioclimatic data

For the calculation of the coordination distance for propagation mode 1, the world has been classified in terms of a radiometeorological parameter br, which indicates the relative incidence of clear-air anomalous propagation conditions. Figure 1 shows a world map of br values for use with the procedure and equations given below. The correct value of br to be used for coordination purposes is the highest value within a radius of 200 km around the particular earth station location. The latitude and longitude of the location of this point should be noted for use in equations (2) and (3). However, for latitudes above 60° N or below 60° S the value of br obtained from the map should not be allowed to exceed the corresponding value at 60° N or S on the earth station longitude.

FIGURE 1...[D01] = 13 CM

The relative point incidence of anomalous propagation, bp%, for the earth station location is then determined using:

(1)

where:

(2)

(3)

j :	latitude of highest br value within 200 km of earth station (degrees) (positive North)

y :	longitude of highest br value within 200 km of earth station (degrees) (positive East).

## 3.2	Procedure for the calculation of mode 1 coordination distance

The coordination distance for propagation mode 1 is that distance d1 (km) which will result in a value of available basic transmission loss which is equal to the minimum permissible basic transmission loss, Lb (p). Here p is the required time percentage, limited to the range 0.001% £ p £ 1%.

Let:

(4)

in which:

(5)

where:

f :	frequency (GHz)

Ah :	correction for the earth station horizon elevation angle q° (see Note 1) given by the expression:

(6)

NOTE 1 – The horizon angle is defined here as the angle, viewed from the centre of the earth-station antenna, between the horizontal plane and a ray that grazes the visible physical horizon in the direction concerned. It is necessary to determine horizon angles for all azimuths around an earth station. In practice it will generally suffice to do this in azimuth increments of 5°. However, every attempt should be made to identify and take into consideration minimum horizon elevation angles that may occur between those azimuths examined in 5° increments.

NOTE 2 – The maximum value of Ah is 30 dB; the use of larger values may not result in the protection being realized in practical situations.

Ak :	additional loss (dB) due to the presence of local “clutter”

Ac :	correction (dB) for direct coupling into over-sea ducts.

The values of Ac (dB) and Ak (dB) can be found for one or both ends of the path, as appropriate, from the equations in § 4.5 and 4.6 respectively of Recommendation ITU-R P.452.

Having determined L1, the required distance must be determined on the basis of an iterative calculation. Commencing at the minimum coordination distance, dmin (km), the distance from the earth station is incremented until the required basic transmission loss is achieved (see Note 3). The preferred distance increment, s (km), is 1 km. However, for fast preliminary calculations, values for s of up to 5 km could be used without undue loss of accuracy.

(7)

NOTE 3 – For computer implementations, modern numerical methods can offer faster techniques than this simple distance incrementing for finding d1. Where computational speed is important such techniques should be explored.

For iterations i = 0,1....n, calculate equations (10) to (18a) until the following condition is fulfilled:

(8)

The required coordination distance d1, is then given by:

(9)

where n is the lowest value of i that causes the condition of equation (8) to be met, and s is the incremental distance. The maximum coordination distance for propagation mode 1 is to be taken as 1 200 km. The loss L2 is evaluated from:

(10)

where:

di :	current distance (km) from earth station:

(11)

gg :	specific attenuation (dB/km) due to gaseous absorption:

(12)

go and gw can be found from the equations in Recommendation ITU-R P.676, using a value of 7.5 g/m3 for the water vapour concentration, r.

(13)

dt :	current aggregate land distance (km; Zone A1 + Zone A2) within the distance di:

(14)

(15)

where:

(15a)

where:

(16)

(17)

The parameter m1 depends on the degree to which the path is over land (inland and/or coastal) and water and is given by:

(18)

with:

(18a)

where dl is the current aggregate inland distance (km) (Zone A2) within the distance di.

The radioclimatic zones to be used for the derivation of dt and dl are defined in Table 1.

TABLE  1

Radioclimatic zones

| Zone type | Code | Definition |
| --- | --- | --- |
| Coastal land | A1 | Coastal land and shore areas, i.e., land adjacent to the sea up to an altitude of 100 m relative to mean sea or water level, but limited to a distance of 50 km from the nearest sea area. Where precise 100 m data is not available an approximate value, i.e., 300 ft, may be used |
| Inland | A2 | All land, other than coastal and shore areas defined as “coastal land” above |
| Sea | B | Seas, oceans and other large bodies of water (i.e., covering a circle of at least 100 km in diameter) |

###### Large bodies of inland water

A “large” body of inland water, to be considered as lying in Zone B, is defined for the administrative purpose of coordination as one having an area of at least 7 800 km2, but excluding the area of rivers. Islands within such bodies of water are to be included as water within the calculation of this area if they have elevations lower than 100 m above the mean water level for more than 90% of their area. Islands that do not meet these criteria should be classified as land for the purposes of the water area calculation.

###### Large inland lake or wetland areas

Large inland areas of greater than 7 800 km2 which contain many small lakes or a river network should be declared as “coastal” Zone A1 by administrations if the area comprises more than 50% water, and more than 90% of the land is less than 100 m above the mean water level.

Climatic regions pertaining to Zone A1, large inland bodies of water and large inland lake and wetland regions are difficult to determine unambiguously. Therefore administrations are requested to register with the ITU (BR) those regions within their territorial boundaries that they wish identified as belonging to one of these categories. In the absence of registered information to the contrary, all land areas will be considered to pertain to climate Zone A2.

For maximum consistency of results between administrations it is strongly recommended that the calculations of this procedure be based on the ITU-R Digitized World Map (IDWM) which is available from the ITU for mainframe or personal computer environments.

# 4	Determination of the coordination contour for propagation mode 2 – Scattering from hydrometeors

## 4.1	General

The determination of the coordination contour for scattering from hydrometeors (e.g., rain-scatter) is predicted on a path geometry which is substantially different from that of the great-circle propagation mechanisms. As a first approximation, energy is scattered isotropically by rain, so that interference may result for large scattering angles, and for beam intersections away from the great-circle path.

For this propagation mode the previous classification of the terrain into Zones A1, A2, B and C is no longer appropriate.

The following procedure, together with the supplementary information in Appendix 1, should be used to determine the mode 2 coordination distance.

## 4.2	Calculation of contours for propagation mode 2

The material is presented in terms of the basic equations for the relationship between rainfall rate, transmission loss and rain-scatter distance. The equations allow transmission loss to be expressed versus rainfall rate for any given distance, with the cumulative time distributions of rainfall rate in various “rain climatic zones” given in Appendix 1.

The transmission loss may be calculated as a function of distance r (km) (see Note 1), frequency f (GHz) and surface rainfall rate R (mm/h) from:

(19)

where:

R :	surface rainfall rate (mm/h), as given in Appendix 1 for various rain-climatic zones

gT :	gain of the terrestrial station antenna (dB), assumed to be 42 dB

and:

C is given by:

gR is given by:

(22)

NOTE 1 – r is the distance between the region of maximum scattering and the location of an eventual terrestrial station.

Table 2 gives values for k and a for vertical polarization (which yields minimum specific attenuation).

Further:

(23)

where:

(24)

E :	loss in scatter coupling for heights above the melting layer, given by:

where:

HFR :	mean annual rain height (km) in the region of the earth station, as defined in Recommendation ITU-R P.839.

TABLE  2

Values of k and a for vertical polarization
as a function of the frequency

| - Frequency (GHz) | - k | - a |
| --- | --- | --- |
| - 1 | - 0.000 0352 | - 0.880 |
| - 4 | - 0.000 591 | - 1.075 |
| - 6 | - 0.001 55 | - 1.265 |
| - 8 | - 0.003 95 | - 1.31 |
| - 10 | - 0.008 87 | - 1.264 |
| - 12 | - 0.016 8 | - 1.20 |
| - 14 | - 0.029 | - 1.15 |
| - 18 | - 0.055 | - 1.09 |
| - 20 | - 0.069 1 | - 1.065 |
| - 22.4 | - 0.090 | - 1.05 |
| - 25 | - 0.113 | - 1.03 |
| - 28 | - 0.150 | - 1.01 |
| - 30 | - 0.167 | - 1.00 |
| - 35 | - 0.233 | - 0.963 |
| - 40 | - 0.310 | - 0.929 |

The equations for gaseous specific attenuation, go (for dry air) and gw (for water vapour), are given in Recommendation ITU-R P.676. The water vapour specific attenuation gw is to be calculated for an assumed water vapour density of r = 7.5 g/m3.

Equation (19) allows transmission loss, L, to be obtained as a monotonic function of  rainfall rate, R, and with the hydrometeor scatter distance, r, as a parameter. The procedure to determine the hydrometeor scatter contour is as follows:

a)	The value of R should be found for the required time percentage, p, and the appropriate rain climate A to Q from the data in Appendix 1.

b)	Values of L are then calculated for incremental values of r, starting at 45 km. The correct value of r is that for which the corresponding value of L equals or exceeds the required transmission loss. This value of r is denoted dr.

c)	If the iterative calculation results in r equalling or exceeding the appropriate maximum distance given in Table 3, then the calculation is terminated and dr is assumed to have this maximum value.

TABLE  3

Maximum hydrometeor scatter distances (km)

| Latitude (degrees) | 0-30 | 30-40 | 40-50 | 50-60 | > 60 |
| --- | --- | --- | --- | --- | --- |
| Distance (km) | 350 | 360 | 340 | 310 | 280 |

d)	Determine a point at distance D d along the earth-station beam azimuth. This can be obtained from the following formula:

(28)

where:

es :	earth-station antenna main beam elevation angle.

e)	Draw a circle of radius dr around this point. This is the hydrometeor scatter coordination contour (coordination contour for propagation mode 2). The coordination distance for propagation mode 2 on a given azimuth from the earth station is the distance from the earth station to the coordination contour on that azimuth, denoted d2.

As the only significant hydrometeor scatter is that occurring in the general vicinity of the earth station, the question of a mixed path loss does not arise.

# 5	Auxiliary contours

### 5.1	General

The coordination contours, together with the supplementary coordination contours as described in Recommendation ITU-R IS.847 are based upon the most unfavourable assumptions regarding interference geometry. Such assumptions rarely apply in practice, and so auxiliary contours should be drawn to eliminate terrestrial stations from further consideration for which the extreme assumptions do not apply, e.g., in cases where the terrestrial station antenna gain or e.i.r.p. in the direction of the earth station is less than that assumed in § 3 and 4.

For mode 1, the derivation of auxiliary contours in terms of the reduced power levels is straightforward. The mode 2 auxiliary contours however are generated not for different power levels but for different values of the avoidance angle, this angle being the offset azimuth angle of the terrestrial station main beam axis away from the direction of the earth station. When an earth station is to be coordinated with a group of terrestrial stations, it is necessary to consider the actual parameters of the terrestrial stations and to generate a supplementary coordination contour corresponding to those parameters. If some of the terrestrial stations fall inside the supplementary contour, a further elimination can be achieved through the use of auxiliary contours that have to be generated for the actual parameters of the group of terrestrial stations. In other words, these contours are most usefully applied in conjunction with the supplementary contour rather than the main contour.

## 5.2	Clear-air conditions (mode 1)

The auxiliary contours should be drawn by introducing into equation (4) a reduced loss of 5, 10, 15, 20 dB, etc., in required transmission loss, down to the minimum coordination distance.

## 5.3	Hydrometeor scatter (mode 2)

The coordination contour for mode 2 propagation around an earth station is calculated assuming the worst possible scenario, namely that the terrestrial station and the earth station point directly towards one another, and that the two main beams intersect exactly. This produces a large coordination area within which detailed calculations of hydrometeor

scatter interference levels need to be performed. In practice, mode 2 propagation is far more likely to occur outside this great-circle plane than on it, and, furthermore, the antenna mainlobes are unlikely to intersect exactly. In either case, it is possible to generate auxiliary contours which would yield areas that are smaller than the coordination area. Appendix 2 provides a procedure for determining auxiliary contours on the basis of the avoidance angle (j). Any station which lies outside the relevant contour for its avoidance angle need not be considered as a significant source of interference.

In the case of mode 2, the minimum coordination distance is determined by different criteria from the mode 1 case and a distance of 45 km should be applied. The mode 2 auxiliary contours should be prepared for avoidance angles of 2°, 5°, 10°, 20° and 30°, with additional angles as appropriate. The auxiliary contours should be applied when considering terrestrial station antennas conforming to Recommendation ITU-R F.699.

APPENDIX  1

TO  ANNEX  1

Classification of rain climates

As shown in Recommendation ITU-R P.837 the world has been divided into a number of rain climatic zones which show different precipitation characteristics. The curves shown in Fig. 2 represent consolidated rainfall-rate distributions, each applicable to several of these rain climates, and are based on rainfall rate distributions as defined in Recommendation ITU-R P.837.

FIGURE 2...[D02] = 13 CM

The distributions of Fig. 2 have been extended beyond 0.3% to such greater percentages of time pc at which the rainfall rate is assumed to approach zero, using the expression:

(29)

and using, for R(0.3%) and pc, the following values:

| Rain climatic zone | - R(0.3%) (mm/h) | - pc (%) |
| --- | --- | --- |
| - A, B | - 1.5 | - 2 |
| - C, D, E | - 3.5 | - 3 |
| - F, G, H, J, K | - 7.0 | - 5 |
| - L, M | - 9.0 | - 7.5 |
| - N, P, Q | - 25.0 | - 10 |

This approach is appropriate for the numerical evaluation of the rain-scatter distance. It is, however, of concern only for frequencies above 8 GHz.

For numerical evaluation of R( p) in the range 0.001% < p < 0.3%, the curves in Fig. 2 have been converted into equations (30) to (34).

Climates A, B

(30)

Climates C, D, E

(31)

Climates F, G, H, J, K

(32)

Climates L, M

(33)

Climates N, P, Q

(34)

APPENDIX  2

TO  ANNEX  1

Calculation of contours for propagation mode 2

Table 4 gives the definition of terms used in this section. Figure 3 shows the plan view of the hydrometeor scatter projected on to the horizontal plane, where A is the earth station, B is the terrestrial station at an arbitrary position, X and Y represent the locations of the terrestrial station that correspond to the maximum and minimum distance away from the centre of the contour respectively and M is the maximum horizontal extent of the potential common volume (CV).

TABLE  4

Hydrometeor scatter terms

| HFR | Rain cell height |
| --- | --- |
| rb | Distance from the CV centre to the auxiliary contour |
| b | Horizontal distance between earth station and most distant CV possible |
| es | Earth station elevation angle |
| a | Polar angle of terrestrial station with respect to the centre of the common volume |
| y | Angle subtended by b at terrestrial station (“look angle”) |
| d | Required minimum protection angle |
| j | Avoidance angle (= y + d) |
| d | Distance from the earth station to a point on the auxiliary contour |
| qd | Azimuth relative to the main beam direction from the earth station to a point on the auxiliary contour |

The shaded area in Fig. 3 represents the critical region along the earth station beam which, if intersected by the terrestrial station main beam, would result in significant hydrometeor scatter interference via main lobe to main lobe coupling. This critical region, whose extent is annotated by b in the figure, is bounded by the earth station on one side and the maximum rain height (HFR) on the other. For a given point within the coordination zone, the angle subtended by this region is termed the look angle y. The protection angle d represents the angle of the terrestrial station beam away from the critical region. The avoidance angle j is the sum of these two angles y and d and it is this quantity j which will remain fixed along its own auxiliary contour.

The reference point of the contour is at the centre of the common volume (at a distance b/2 away from the earth station). Each contour is generated by varying the polar angle a and deriving corresponding values of rb, as a changes from 0° to 360°, the angles y and d will rise and fall, but their sum remains the same. The most favourable position for a terrestrial station occurs when a = 0 (position Y), at which point the protection angle reaches its maximum. The distance from the centre of the common volume to the terrestrial station is then a minimum, denoted by rb min in the figure. At the other extreme, when a = 90° (position X), the look angle reaches its maximum, the protection angle will be at its minimum and rb will have its largest value rb max for the contour.

###### The step-by-step algorithm

The following algorithm can be used to calculate the auxiliary mode 2 coordination contour for a given value of the avoidance angle j.

–	The limits of the minimum protection angle d0 are:

d0 min  =  1.0°

d0 max  =  48.0°

–	Compute b using:

(35)

–	Calculate the value of d0 that corresponds to the chosen j, as follows:

a)	Set d0 = d0 min.

b)	Calculate the transmitter sidelobe gain at this angle d0 off boresight, according to Recommendation ITU-R F.699.

c)	Use the resulting gain in place of the parameter gT in equation (19) to compute the maximum distance for the auxiliary contour, rb max, for the required transmission loss threshold.

d)	Calculate y0 using:

(36)

e) 	Calculate the avoidance angle, j¢, for the selected d0 using:

(37)

f)	If ½j¢ - j½ > 0.01 j then use the standard bisection technique to determine a new estimate for d0 and repeat from step b) until convergence is achieved as defined by ½j¢ - j½ £ 0.01j.

g)	Use the final value of d0 and rb max hereafter.

–	Derive rb min by:

a)	calculating the antenna sidelobe gain for the above value of j using Recommendation ITU-R F.699;

b)	using this sidelobe gain in place of the parameter gT in equation (19) to compute the distance for the auxiliary contour, for the required transmission loss threshold. This distance is rb min.

–	Generate the contour for values of a from 0o to 180o in steps of 1o as follows:

a)	Set rb  =  0.5 (rb min  +  rb max)

b)	Compute y from:

(38)

where:

and:

c)	Compute d  =  j  -  y.

d) 	Compute G(d) using Recommendation ITU-R F.699.

e) 	Using the resulting side lobe gain, G(d), in place of the parameter gT in equation (19), compute the distance,  for the required transmission loss threshold.

f)	If ½ - rb½ < 0.5 km, then the desired value has been found.

If not, a new value is given to rb :

rb  =  0.5 (rb  +  rb max)		for	  >  rb 
rb  =  0.5 (rb  +  rb min)		for	  £  rb

and steps b) to f) are repeated.

–	Once the value of rb has been found, calculate the distance, d and the azimuth qd from the location of the earth station to that contour point using:

(39)

–	The values of rb for a from 181° to 359° can be found by using the symmetry relationship:

(41)

FIGURE 3...[D03] = 20 CM

_________________