RECOMMENDATION  ITU-R  P.676-3

ATTENUATION  BY  ATMOSPHERIC  GASES

(Question ITU-R 201/3)

(1990-1992-1995-1997)

Rec. ITU-R P.676-3

The ITU Radiocommunication Assembly,

considering

a)	the necessity of estimating the attenuation by atmospheric gases on terrestrial and slant paths,

recommends

1	that, for general application, the procedures in Annex 1 be used to calculate gaseous attenuation at frequencies up to 1 000 GHz. (Software code in MATLAB is available from the Radiocommunication Bureau);

2	that, for approximate estimates of gaseous attenuation in the frequency range 1 to 350 GHz, the simpler procedure given in Annex 2 be used.

ANNEX  1

Line-by-line calculation of gaseous attenuation

# 1	Specific attenuation

The specific attenuation at frequencies up to 1 000 GHz due to dry air and water vapour, can be evaluated most accurately at any value of pressure, temperature and humidity by means of a summation of the individual resonance lines from oxygen and water vapour, together with small additional factors for the non-resonant Debye spectrum of oxygen below 10 GHz, pressure-induced nitrogen attenuation above 100 GHz and a wet continuum to account for the excess water vapour-absorption found experimentally. Figure 1 shows the specific attenuation using the model, calculated from 0 to 1 000 GHz at 1 GHz intervals, for a pressure of 1 013 hPa, temperature of 15 C for the cases of a water-vapour density of 7.5 g/m3 (Curve A) and a dry atmosphere (Curve B).

Near 60 GHz, many oxygen absorption lines merge together, at sea-level pressures, to form a single, broad absorption band, which is shown in more detail in Fig. 2. This Figure also shows the oxygen attenuation at higher altitudes, with the individual lines becoming resolved at lower pressures.

For quick and approximate estimates of specific attenuation at frequencies up to 350 GHz, in cases where high accuracy is not required, simplified algorithms are given in Annex 2 for restricted ranges of meteorological conditions.

FIGURE 1/P.676...[D01]  3 CM

FIGURE 2/P.676...[D01]  3 CM

The specific gaseous attenuation is given by:

(1)

where go and gw are the specific attenuations (dB/km) due to dry air and water vapour, respectively, and where f is the frequency (GHz) and N ( f ) is the imaginary part of the frequency-dependent complex refractivity:

(2)

Si is the strength of the ith line, Fi is the line shape factor and the sum extends over all the lines;  ( f ) and  are dry and wet continuum spectra.

The line strength is given by:

(3)

where:

p :	dry air pressure (hPa)

e :	water vapour partial pressure in hPa (total barometric pressure P  p  e)

 	300/T

T :	temperature (K).

Local values of p, e and T measured profiles (e.g. using radiosondes) should be used; however, in the absence of local information, the Reference Standard Atmospheres described in Recommendation ITU-R P.835 should be used.

The water-vapour partial pressure, e, may be obtained from the water-vapour density  using the expression:

e=ρT216.7	(4)

The coefficients a1, a2 are given in Table 1 for oxygen, those for water vapour, b1 and b2, are given in Table 2.

The line-shape factor is given by:

(5)

where fi is the line frequency and f is the width of the line:

(6)

and  is a correction factor which arises due to interference effects in oxygen lines:

(7)

The spectroscopic coefficients are given in Tables 1 and 2.

TABLE  1

Spectroscopic data for oxygen attenuation

| f0 | a1 | a2 | a3 | a4 | a5 | a6 |
| --- | --- | --- | --- | --- | --- | --- |
| 50.474238 | 0.94 | 9.694 | 8.60 | 0 | 1.600 | 5.520 |
| 50.987749 | 2.46 | 8.694 | 8.70 | 0 | 1.400 | 5.520 |
| 51.503350 | 6.08 | 7.744 | 8.90 | 0 | 1.165 | 5.520 |
| 52.021410 | 14.14 | 6.844 | 9.20 | 0 | 0.883 | 5.520 |
| 52.542394 | 31.02 | 6.004 | 9.40 | 0 | 0.579 | 5.520 |
| 53.066907 | 64.10 | 5.224 | 9.70 | 0 | 0.252 | 5.520 |
| 53.595749 | 124.70 | 4.484 | 10.00 | 0 | – 0.066 | 5.520 |
| 54.130000 | 228.00 | 3.814 | 10.20 | 0 | – 0.314 | 5.520 |
| 54.671159 | 391.80 | 3.194 | 10.50 | 0 | – 0.706 | 5.520 |
| 55.221367 | 631.60 | 2.624 | 10.79 | 0 | –1.151 | 5.514 |
| 55.783802 | 953.50 | 2.119 | 11.10 | 0 | – 0.920 | 5.025 |
| 56.264775 | 548.90 | 0.015 | 16.46 | 0 | 2.881 | – 0.069 |
| 56.363389 | 1 344.00 | 1.660 | 11.44 | 0 | – 0.596 | 4.750 |
| 56.968206 | 1 763.00 | 1.260 | 11.81 | 0 | – 0.556 | 4.104 |
| 57.612484 | 2 141.00 | 0.915 | 12.21 | 0 | –2.414 | 3.536 |
| 58.323877 | 2 386.00 | 0.626 | 12.66 | 0 | –2.635 | 2.686 |
| 58.446590 | 1 457.00 | 0.084 | 14.49 | 0 | 6.848 | – 0.647 |
| 59.164207 | 2 404.00 | 0.391 | 13.19 | 0 | – 6.032 | 1.858 |
| 59.590983 | 2 112.00 | 0.212 | 13.60 | 0 | 8.266 | –1.413 |
| 60.306061 | 2 124.00 | 0.212 | 13.82 | 0 | –7.170 | 0.916 |
| 60.434776 | 2 461.00 | 0.391 | 12.97 | 0 | 5.664 | –2.323 |
| 61.150560 | 2 504.00 | 0.626 | 12.48 | 0 | 1.731 | –3.039 |
| 61.800154 | 2 298.00 | 0.915 | 12.07 | 0 | 1.738 | –3.797 |
| 62.411215 | 1 933.00 | 1.260 | 11.71 | 0 | – 0.048 | – 4.277 |
| 62.486260 | 1 517.00 | 0.083 | 14.68 | 0 | – 4.290 | 0.238 |
| 62.997977 | 1 503.00 | 1.665 | 11.39 | 0 | 0.134 | – 4.860 |
| 63.568518 | 1 087.00 | 2.115 | 11.08 | 0 | 0.541 | –5.079 |
| 64.127767 | 733.50 | 2.620 | 10.78 | 0 | 0.814 | –5.525 |
| 64.678903 | 463.50 | 3.195 | 10.50 | 0 | 0.415 | –5.520 |
| 65.224071 | 274.80 | 3.815 | 10.20 | 0 | 0.069 | –5.520 |
| 65.764772 | 153.00 | 4.485 | 10.00 | 0 | – 0.143 | –5.520 |
| 66.302091 | 80.09 | 5.225 | 9.70 | 0 | – 0.428 | –5.520 |
| 66.836830 | 39.46 | 6.005 | 9.40 | 0 | – 0.726 | –5.520 |
| 67.369598 | 18.32 | 6.845 | 9.20 | 0 | –1.002 | –5.520 |
| 67.900867 | 8.01 | 7.745 | 8.90 | 0 | –1.255 | –5.520 |
| 68.431005 | 3.30 | 8.695 | 8.70 | 0 | –1.500 | –5.520 |
| 68.960311 | 1.28 | 9.695 | 8.60 | 0 | –1.700 | –5.520 |
| 118.750343 | 945.00 | 0.009 | 16.30 | 0 | – 0.247 | 0.003 |
| 368.498350 | 67.90 | 0.049 | 19.20 | 0.6 | 0 | 0 |
| 424.763124 | 638.00 | 0.044 | 19.16 | 0.6 | 0 | 0 |
| 487.249370 | 235.00 | 0.049 | 19.20 | 0.6 | 0 | 0 |
| 715.393150 | 99.60 | 0.145 | 18.10 | 0.6 | 0 | 0 |
| 773.839675 | 671.00 | 0.130 | 18.10 | 0.6 | 0 | 0 |
| 834.145330 | 180.00 | 0.147 | 18.10 | 0.6 | 0 | 0 |

TABLE  2

Spectroscopic data for water-vapour attenuation

| f0 | b1 | b2 | b3 | b4 | b5 | b6 |
| --- | --- | --- | --- | --- | --- | --- |
| 22.235080 | 0.1090 | 2.143 | 28.11 | 0.69 | 4.80 | 1.00 |
| 67.813960 | 0.0011 | 8.735 | 28.58 | 0.69 | 4.93 | 0.82 |
| 119.995941 | 0.0007 | 8.356 | 29.48 | 0.70 | 4.78 | 0.79 |
| 183.310074 | 2.3000 | 0.668 | 28.13 | 0.64 | 5.30 | 0.85 |
| 321.225644 | 0.0464 | 6.181 | 23.03 | 0.67 | 4.69 | 0.54 |
| 325.152919 | 1.5400 | 1.540 | 27.83 | 0.68 | 4.85 | 0.74 |
| 336.187000 | 0.0010 | 9.829 | 26.93 | 0.69 | 4.74 | 0.61 |
| 380.197372 | 11.9000 | 1.048 | 28.73 | 0.69 | 5.38 | 0.84 |
| 390.134508 | 0.0044 | 7.350 | 21.52 | 0.63 | 4.81 | 0.55 |
| 437.346667 | 0.0637 | 5.050 | 18.45 | 0.60 | 4.23 | 0.48 |
| 439.150812 | 0.9210 | 3.596 | 21.00 | 0.63 | 4.29 | 0.52 |
| 443.018295 | 0.1940 | 5.050 | 18.60 | 0.60 | 4.23 | 0.50 |
| 448.001075 | 10.6000 | 1.405 | 26.32 | 0.66 | 4.84 | 0.67 |
| 470.888947 | 0.3300 | 3.599 | 21.52 | 0.66 | 4.57 | 0.65 |
| 474.689127 | 1.2800 | 2.381 | 23.55 | 0.65 | 4.65 | 0.64 |
| 488.491133 | 0.2530 | 2.853 | 26.02 | 0.69 | 5.04 | 0.72 |
| 503.568532 | 0.0374 | 6.733 | 16.12 | 0.61 | 3.98 | 0.43 |
| 504.482692 | 0.0125 | 6.733 | 16.12 | 0.61 | 4.01 | 0.45 |
| 556.936002 | 510.0000 | 0.159 | 32.10 | 0.69 | 4.11 | 1.00 |
| 620.700807 | 5.0900 | 2.200 | 24.38 | 0.71 | 4.68 | 0.68 |
| 658.006500 | 0.2740 | 7.820 | 32.10 | 0.69 | 4.14 | 1.00 |
| 752.033227 | 250.0000 | 0.396 | 30.60 | 0.68 | 4.09 | 0.84 |
| 841.073593 | 0.0130 | 8.180 | 15.90 | 0.33 | 5.76 | 0.45 |
| 859.865000 | 0.1330 | 7.989 | 30.60 | 0.68 | 4.09 | 0.84 |
| 899.407000 | 0.0550 | 7.917 | 29.85 | 0.68 | 4.53 | 0.90 |
| 902.555000 | 0.0380 | 8.432 | 28.65 | 0.70 | 5.10 | 0.95 |
| 906.205524 | 0.1830 | 5.111 | 24.08 | 0.70 | 4.70 | 0.53 |
| 916.171582 | 8.5600 | 1.442 | 26.70 | 0.70 | 4.78 | 0.78 |
| 970.315022 | 9.1600 | 1.920 | 25.50 | 0.64 | 4.94 | 0.67 |
| 987.926764 | 138.0000 | 0.258 | 29.85 | 0.68 | 4.55 | 0.90 |

The dry air continuum arises from the non-resonant Debye spectrum of oxygen below 10 GHz and a pressure-induced nitrogen attenuation above 100 GHz.

(8)

where d is the width parameter for the Debye spectrum:

(9)

The wet continuum, , is included to account for the fact that measurements of water-vapour attenuation are generally in excess of those predicted using the theory described by equations (2) to (7), plus a term to include the effects of higher-frequency water-vapour lines not included in the reduced line base:

(10)

# 2	Path attenuation

## 2.1	Terrestrial paths

For a terrestrial path, or for slightly inclined paths close to the ground, the path attenuation, A, may be written as:

A     r0    (o    w) r0	dB	(11)

where r0 is path length (km).

## 2.2	Slant paths

This section gives a method to integrate the specific attenuation calculated using the line-by-line model given above, at different pressures, temperatures and humidities through the atmosphere. By this means, the path attenuation for communications systems with any geometrical configuration within and external to the Earth’s atmosphere may be accurately determined simply by dividing the atmosphere into horizontal layers, specifying the profile of the meteorological parameters pressure, temperature and humidity along the path. In the absence of local profiles, from radiosonde data, for example, the reference standard atmospheres in Recommendation ITU-R P.835 may be used, either for global application or for low (annual), mid (summer and winter) and high latitude (summer and winter) sites.

Figure 3 shows the zenith attenuation calculated at 1 GHz intervals with this model for the global reference standard atmosphere in Recommendation ITU-R P.835, with horizontal layers 1 km thick and summing the attenuations for each layer, for the cases of a moist atmosphere (Curve A) and a dry atmosphere (Curve B).

The total slant path attenuation, A(h, ), from a station with altitude, h, and elevation angle, , can be calculated as follows when  ³ 0:

Ah.j=gHsinFdH	(12)

where the value of F can be determined as follows based on Snell’s law in polar coordinates:

F=arccoscr+H´nH	(13)

where:

c    (r    h)    n (h)    cos 	(14)

where n(h) is the atmospheric radio refractive index, calculated from pressure, temperature and water-vapour pressure along the path (see Recommendation ITU-R P.835) using Recommendation ITU-R P.453.

On the other hand, when  < 0, there is a minimum height, hmin, at which the radio beam becomes parallel with the Earth’s surface. The value of hmin can be determined by solving the following transcendental equation:

(r    hmin)    n(hmin)    c	(15)

This can be easily solved by repeating the following calculation, using hmin  h as an initial value:

h'min=cnhmin−r	(16)

Therefore, A(h, ) can be calculated as follows:

Ah,j=gHsinFdH+gHsinFdH	(17)

FIGURE 3/P.676...[D03]  3 CM

In carrying out the integration of equations (12) and (17), care should be exercised in that the integrand becomes infinite at F  0. However, this singularity can be eliminated by an appropriate variable conversion, for example, by using u4  H – h in equation (12) and u4  H – hmin in equation (17).

A numerical solution for the attenuation due to atmospheric gases can be implemented with the following algorithm.

To calculate the total attenuation for a satellite link, it is necessary to know not only the specific attenuation at each point of the link but also the length of path that has that specific attenuation. To derive the path length it is also necessary to consider the ray bending that occurs in a spherical Earth.

FIGURE 4/P.676...[D04]  3 CM

Using Fig. 4 as a reference, an is the path length through layer n with thickness dn that has refractive index nn. an and bn are the entry and exiting incidence angles. rn are the radii from the centre of the Earth to the beginning of layer n. an can then be expressed as:

an=−rncosβn+124rn2cos2βn+8rndn+4dn2	(18)

The angle an can be calculated from:

αn=p−arccos−an2−2rndn−dn22anrn+2andn	(19)

b1 is the incidence angle at the ground station (the complement of the elevation angle q). bn  1 can be calculated from an using Snell’s law that in this case becomes:

βn+1=arcsinnnnn+1sinαn	(20)

where nn and nn  1 are the refractive indexes of layers n and n  1.

The remaining frequency dependent (dispersive) term has a marginal influence on the result (around 1%) but can be calculated from the method shown in the ITU-R Handbook on Radiometeorology.

The total attenuation can be derived using:

Agas=n=1kangn	dB	(21)

where n is the specific attenuation derived from equation (1).

To ensure an accurate estimate of the attenuation, the thickness of the layers should increase exponentially with height. Accurate results can be obtained with layer thickness that increases from 10 cm for the lower layer to 1 km at an altitude of 100 km.

For Earth-to-space applications the integration should be performed at least up to 30 km.

# 3	Dispersive effects

The effects of dispersion are discussed in the ITU-R Handbook on Radiometeorology, which contains a model for calculating dispersion based on the line-by-line calculation. For practical purposes, dispersive effects should not impose serious limitations on millimetric terrestrial communication systems operating with bandwidths of up to a few hundred MHz over short ranges (for example, less than about 20 km), especially in the window regions of the spectrum, at frequencies removed from the centres of major absorption lines. For satellite communication systems, the longer path lengths through the atmosphere will constrain operating frequencies further to the window regions, where both atmospheric attenuation and the corresponding dispersion are low.

ANNEX  2

Approximate estimation of gaseous attenuation
in the frequency range 1-350 GHz

This Annex contains simplified algorithms for quick, approximate estimation of gaseous attenuation for a limited range of meteorological conditions and a limited variety of geometrical configurations.

# 1	Specific attenuation

The specific attenuation due to dry air and water vapour, from sea level to an altitude of 5 km, can be estimated using the following simplified algorithms, which are based on curve-fitting to the line-by-line calculation, and agree with the more accurate calculations to within an average of about ±15% at frequencies removed from the centres of major absorption lines. The absolute difference between the results from these algorithms and the line-by-line calculation is generally less than 0.1 dB/km and reaches a maximum of 0.7 dB/km near 60 GHz. For altitudes higher than 5 km, and in cases where higher accuracy is required, the line-by-line calculation should be used.

For dry air, the attenuation o (dB/km) is given by:

go=7.27rtf2+0.351rp2rt2+7.5f−572+2.44rp2rt5f2rp2rt2´10−3	(22a)

for  f    57 GHz

go=2´10−4rt1.51−1.2´10−5f1.5+4f−632+1.5rp2rt5+0.28rt2f−118.752+2.84rp2rt2f2rp2rt2´10−3	(22b)

for 63 GHz   f    350 GHz

go=f−60f−6318go57−1.66rp2rt8.5f−57f−63+f−57f−6018go63	(22c)

for 57 GHz   f    63 GHz.

where:

f :	frequency (GHz)

rp 	p / 1013

rt 	288/(273  t)

p :	pressure (hPa)

t :	temperature (C).

For water vapour, the attenuation w (dB/km) is given by:

gw=3.27´10−2rt+1.67´10−3ρrt7rp+7.7´10−4f0.5+3.79f−22.2352+9.81rp2rt	(23)

for  f    350 GHz

where  is the water-vapour density (g/m3).

Figure 5 shows the specific attenuation from 1 to 350 GHz at sea-level for dry air and water vapour with a density of 7.5 g/m3. This figure was derived using the line-by-line calculation as described in Annex 1.

# 2	Path attenuation

## 2.1	Terrestrial paths

For a horizontal path, or for slightly inclined paths close to the ground, the path attenuation, A, may be written as:

A    g r0    (go    gw)r0         dB	(24)

where r0 is the path length (km).

FIGURE 5/P.676...[D05]  3 CM

## 2.2	Slant paths

This section contains simple algorithms for the calculation of gaseous attenuation along slant paths through the Earth’s atmosphere, by defining an equivalent height by which the specific attenuation calculated in § 1 may be multiplied to obtain the zenith path attenuation, from sea level up to altitudes of about 2 km. The path attenuation at elevation angles other than the zenith may then be determined using the procedures described later in this section.

For dry air, the equivalent height is given by:

ho    6	km	for	f    50	GHz	(25)

km	for 70  	f    350	GHz	(26)

and for water-vapour, the equivalent height is:

(27)

For  f    350 GHz

where:

hw0	:	water vapour equivalent height in the window regions

	1.6 km in clear weather

	2.1 km in rain.

These equivalent heights for water vapour were determined at a ground-level temperature of 15 C. For other temperatures the equivalent heights may be corrected by 0.1% or 1% per C in clear weather or rain respectively, in the window regions, and by 0.2% or 2% in the absorption bands (height increasing with increasing temperature).

The concept of equivalent height is based on the assumption of an exponential atmosphere specified by a scale height to describe the decay in density with altitude. Note that scale heights for both dry air and water vapour may vary with latitude, season and/or climate, and that water vapour distributions in the real atmosphere may deviate considerably from the exponential, with corresponding changes in equivalent heights. The values given above are applicable up to an altitude of 2 km.

The total zenith attenuation is then:

(28)

Figure 6 shows the total zenith attenuation at sea level for two cases: (A) the dry reference standard atmosphere in Recommendation ITU-R P.835 and (B) including the water-vapour model atmosphere in Recommendation ITU-R P.835. Between 50 and 70 GHz greater accuracy can be obtained from the 0 km curve in Fig. 7. This figure was derived using the line-by-line calculation as described in Annex 1.

### 2.2.1	Elevation angles between 10° and 90°

For elevation angles between 10 and 90, the path attenuation is obtained using the cosecant law:

(29)

where  is the elevation angle.

FIGURE 6/P.676...[D06]  3 CM

FIGURE 7/P.676...[D07]  3 CM

These formulae are applicable to cases of inclined paths between a satellite and an earth station situated at sea level. To determine the attenuation values on an inclined path between a station situated at altitude h1 and another at a higher altitude h2, the values ho and hw in equation (29) must be replaced by the following  and  values:

(30)

(31)

it being understood that the value  of the water vapour density used in equation (23) is the hypothetical value at sea level calculated as follows:

ρ=ρ1´exp(h12)	(32)

where 1 is the value corresponding to altitude h1 of the station in question, and the equivalent height of water vapour density is assumed as 2 km (see Recommendation ITU-R P.835).

Equations (30), (31) and (32) use different normalizations for the dry air and water vapour equivalent heights. While the mean air pressure referred to sea level can be considered constant around the world (equal to 1 013 hPa), the water vapour density not only has a wide range of climatic variability but is measured at the surface (i.e. at the height of the ground station). For values of surface water vapour density, see Recommendation ITU-R P.836.

### 2.2.2	Elevation angle between 0 and 10

In this case, the relations (29) to (32) must be replaced by more accurate formulae allowing for the real length of the atmospheric path. This leads to the following relations:

A=Recosj  gohoFtanjReho+gwhwFtanjRehwdB	(33)

where:

Re :	effective Earth radius including refraction, given in Recommendation ITU-R P.834, expressed in km (a value of 8 500 km is generally acceptable for the immediate vicinity of the Earth’s surface)

 :	elevation angle

F :	function defined by:

(34)

The formula (33) is applicable to cases of inclined paths between a satellite and an earth station situated at sea level. To determine the attenuation values on an inclined path between a station situated at altitude h1 and a higher altitude h2 (where both altitudes are less than 1 000 km above m.s.l.), the relation (33) must be replaced by the following:

A=gohRe+h1⋅F(x1)e−h1hocosj1−Re+h2F(x2)e−h2hocosj2

+gwhwRe+h1⋅F(x'1)e−h1hwcosj1−Re+h2F(x'2)e−h2hwcosj2         dB	(35)

where:

1 :	elevation angle at altitude h1

2    arc cos Re+h1Re+h2cosj1	(36a)

xi=tanjiRe+hiho	for i    1, 2	(36b)

x'i=tanjiRe+hihw	for i    1, 2	(36c)

it being understood that the value  of the water vapour density used in equation (23) is the hypothetical value at sea level calculated as follows:

ρ=ρ1⋅exp(h12)	(37)

where 1 is the value corresponding to altitude h1 of the station in question, and the equivalent height of water vapour density is assumed as 2 km (see Recommendation ITU-R P.835).

Values for 1 at the surface can be found in Recommendation ITU-R P.836. The different formulation for dry air and water vapour is explained at the end of § 2.2.

For elevation angles less than 0, the line-by-line calculation in Annex 1 must be used.

## 2.3	Slant path water-vapour attenuation

The above method for calculating slant path attenuation by water vapour relies on the knowledge of the profile of water-vapour pressure (or density) along the path. In cases where the total columnar water vapour content along the path, V, is known, an alternative method may be used. The total water-vapour attenuation in the zenith direction can be expressed as:

Aw    av V               dB	(38)

where V (kg/m2 or mm) and av (dB/kg/m2 or dB/mm) is the water-vapour mass absorption coefficient. The mass absorption coefficient can be calculated from the specific attenuation coefficient w, divided by the water-vapour density , which may be obtained from the water-vapour pressure using equation (4).

Values for the total columnar content V can be obtained either from radiosonde profiles or radiometric measurements. Statistics of V are given in Recommendation ITU-R P.836. For elevation angles other than the zenith, the attenuation must be divided by sin , where  is the elevation angle, assuming a uniform horizontally-stratified atmosphere, down to elevation angles of about 10°.