Recommendation ITU-R RA.1860 (01/2010)

Preferred frequency bands for radio astronomical measurements in the range 1-3 THz

RA Series Radio astronomy

Foreword

The role of the Radiocommunication Sector is to ensure the rational, equitable, efficient and economical use of the radio-frequency spectrum by all radiocommunication services, including satellite services, and carry out studies without limit of frequency range on the basis of which Recommendations are adopted.

The regulatory and policy functions of the Radiocommunication Sector are performed by World and Regional Radiocommunication Conferences and Radiocommunication Assemblies supported by Study Groups.

# Policy on Intellectual Property Right (IPR)

ITU-R policy on IPR is described in the Common Patent Policy for ITU-T/ITU-R/ISO/IEC referenced in Annex 1 of Resolution ITU-R 1. Forms to be used for the submission of patent statements and licensing declarations by patent holders are available from http://www.itu.int/ITU-R/go/patents/en where the Guidelines for Implementation of the Common Patent Policy for ITU-T/ITU-R/ISO/IEC and the ITU-R patent information database can also be found.

| Series of ITU-R Recommendations (Also available online at http://www.itu.int/publ/R-REC/en) |  |
| --- | --- |
| Series | Title |
| BO | Satellite delivery |
| BR | Recording for production, archival and play-out; film for television |
| BS | Broadcasting service (sound) |
| BT | Broadcasting service (television) |
| F | Fixed service |
| M | Mobile, radiodetermination, amateur and related satellite services |
| P | Radiowave propagation |
| RA | Radio astronomy |
| RS | Remote sensing systems |
| S | Fixed-satellite service |
| SA | Space applications and meteorology |
| SF | Frequency sharing and coordination between fixed-satellite and fixed service systems |
| SM | Spectrum management |
| SNG | Satellite news gathering |
| TF | Time signals and frequency standards emissions |
| V | Vocabulary and related subjects |

Note: This ITU-R Recommendation was approved in English under the procedure detailed in Resolution ITU-R 1.

Electronic Publication

Geneva, 2010

 ITU 2010

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without written permission of ITU.

RECOMMENDATION  ITU-R  RA.1860

Preferred frequency bands for radio astronomical
measurements in the range 1-3 THz

(Question ITU-R 145/7)

(2010)

Scope

This Recommendation describes spectral line and continuum radio astronomical observations conducted in the frequency range between 1 000 and 3 000 GHz and recommends that administrations provide assistance in the coordination of radio astronomy observations in this frequency range.

The ITU Radiocommunication Assembly,

considering

a)	that radio astronomers study radio emissions from cosmic sources at higher and higher frequencies;

b)	that the frequencies of observation are often determined by the spectral line frequencies of atoms and molecules that exist in astrophysical environments and that such frequencies are determined by nature;

c)	that the possibility of carrying out observations from the surface of the Earth depends on the “windows” at which the atmosphere is sufficiently transparent;

d)	that the development of radio astronomy at THz frequencies is leading to technological advances, particularly in receiving techniques, and promises further important results;

e)	that frequency bands of importance to the radio astronomy service below 1 000 GHz are addressed in Recommendation ITU-R RA.314;

f)	that astronomical use of frequencies between 10 THz and 1 000 THz are addressed in Recommendation ITU-R RA.1630;

g)	that there is increasing interest in the use of the range 1 000-3 000 GHz by the radio astronomy service;

h)	that the International Astronomical Union (IAU) is maintaining and updating the list of spectral lines of the greatest importance to radio astronomy at frequencies up to 3 000 GHz;

j)	that account should be taken of the Doppler shifts of the lines, due to the relative motion of cosmic source and observer;

k)	that radio astronomers also have a need to make continuum observations at frequencies above 1 000 GHz, and that the bands used for such observations from the ground are determined by the atmospheric windows where lower attenuation occurs;

l)	that the use of radio telescopes from space platforms, conducted under the space research service (passive), provides access to the entire radio spectrum, including portions of the spectrum not accessible from the Earth due to absorption in the atmosphere;

m)	that protections are afforded to the radio astronomy service in bands below 275 GHz by virtue of the Table of Frequency Allocations and in bands between 275 and 1 000 GHz by virtue of No. 5.565 of the Radio Regulations (RR),

recommends

1	that administrations provide assistance in the coordination of radio astronomy observations in bands between 1 000 GHz and 3 000 GHz, particularly those listed in Table 1 (for space-based observations) and Table 3 (for ground-based observations).

Annex 1

# 1	Spectral lines of astronomical interest

Table 1 lists the non-Doppler-shifted (“rest”) frequencies of the spectral lines emitted by a variety of molecules of astrophysical interest. From the rest frequency, an approximate range of frequencies over which the lines may be observed is provided, based upon typical Doppler shifts that are observed due to radial motion of the cosmic source toward or away from the observer. In the table, an assumed Doppler range of ±300 km/s is used. The ranges listed in Table 1 are suggested minimum bands.

It should be noted that because of high sensitivities of the radio astronomical observations so many spectral lines have been observed from extragalactic objects, up to z ~ 6 or higher (here, z denotes the “red-shift”, defined by z = (obs – 0)/0, where obs is the observed wavelength and 0 is the non-Doppler shifted wavelength). Therefore highly red-shifted spectral line observations need to be taken care of.

The source data for Table 1, which was approved by the General Assembly of the International Astronomical Union (IAU) in 2009, were derived from observational studies of submillimetre spectral line emission towards two molecule-rich regions of the Milky Way Galaxy,; from the JPL Molecular Spectroscopy database; and from a web-based catalogue of molecular spectral lines (the Cologne Database for Molecular Spectroscopy) maintained by the University of Cologne, Germany.

# 2	Atmospheric absorption

At millimetre and submillimetre wavelengths, an important factor for ground-based observing is the degree to which the atmosphere attenuates radio signals. Attenuation of signals in the 1 000-3 000 GHz frequency range is due mainly to absorption by water vapour, and to a lesser extent by oxygen, nitrogen and ozone.

For ground-based observatories, atmospheric attenuation reduces the observed signal level of emission coming from cosmic sources and adds thermal noise to the received signal. For this reason, millimetre and submillimetre radio astronomy stations are located at high, dry sites. Despite optimizing the observing location, the level of attenuation is so high over portions of the 1 000-3 000 GHz range that, with the exception of a few “windows”, many observations in this spectral region can only be conducted from space. Table 1 notes those spectral lines that would only be observable from space due to high atmospheric attenuation within the suggested minimum band.

Figure 1 is an example of the transmittance on a vertical path through the atmosphere (i.e. looking straight up along the shortest line of sight through the atmosphere), over the frequency range 1 000-3 000 GHz, at a good astronomical observing site. The data were calculated using the am atmospheric model, assuming the following inputs:

–	Site location: Cerro Sairecabur, approximately 35 km north-northwest of the location of the Atacama Large Millimeter/submillimeter Array (ALMA) site at Chajnantor, Chile.

–	Elevation: 5 525 m.

–	Barometric pressure at ground: 532 mbar.

–	Ground temperature: 285 K.

–	Atmospheric pressure/temperature profile: Based on data acquired over 187 radiosonde flights in the area over the period 1998-2002. The ozone profile was based on NASA ozone-sonde flights over Galapagos, Ecuador, during the same period.

–	Total precipitable water vapour (PWV) content: 0.25 mm. Based on extended measurement campaigns, this site was determined to have a PWV at or below this value approximately 25% of the time.

The calculated transmittance curve agrees well with observed atmospheric brightness temperatures obtained with Fourier transform spectrometer (FTS) measurements at the site.

Table 2 lists the frequency ranges over which the atmospheric transmittance of Fig. 2 exceeds 20%, 10%, 5%, and 1% (Fig. 2 is a graphical representation of the data in Table 2). Lower transmittances will decrease the received signal strength from cosmic sources due to atmospheric absorption, and increase the radio astronomy system noise temperature due to thermal emission from the atmosphere. Therefore, larger transmittances are highly desirable for ground-based radio astronomical measurements, although useful ground-based observations of relatively strong cosmic emissions may be made for transmittances as low as 5%, and possibly lower.

TABLE 1

Astrophysically most important spectral lines in the frequency range between 1 000 and 3 000 GHz

| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Water vapour (H218O) | 1 003.278 | 1 002.274 | 1 004.281 | G | 1 002.274 | 1 004.281 |
| Heavy water (HDO) | 1 009.945 | 1 008.935 | 1 010.955 | G | 1 008.935 | 1 010.955 |
| Azanylidenium (NH+) | 1 012.524 | 1 011.511 | 1 013.537 | G | 1 011.511 | 1 013.537 |
| Carbon monosulphide (CS) | 1 027.314 | 1 026.287 | 1 028.341 | G | 1 026.287 | 1 028.341 |
| Hydroxylium (OH+) | 1 033.119 | 1 032.085 | 1 034.152 | G | 1 032.085 | 1 034.152 |
| Carbon monoxide (CO) | 1 036.912 | 1 035.875 | 1 037.949 | G | 1 035.875 | 1 037.949 |
| Hydrogen cyanide (HCN) | 1 062.983 | 1 061.920 | 1 064.046 |  | 1 061.920 | 1 064.046 |
| Formylium (HCO+) | 1 069.694 | 1 068.624 | 1 070.764 |  | 1 068.624 | 1 070.764 |
| Carbon monosulphide (CS) | 1 076.078 | 1 075.002 | 1 077.154 |  | 1 075.002 | 1 077.154 |
| Sulfhydrylium (SH+) | 1 082.909 | 1 081.826 | 1 083.993 |  | 1 081.826 | 1 083.993 |
| Water vapour (H218O) | 1 095.627 | 1 094.532 | 1 096.723 |  | 1 094.532 | 1 098.462 |
| Carbon monoxide (C18O) | 1 097.163 | 1 096.066 | 1 098.260 |  |  |  |
| Water vapour (H2O) | 1 097.365 | 1 096.267 | 1 098.462 |  |  |  |
| Carbon monoxide (13CO) | 1 101.350 | 1 100.248 | 1 102.451 |  | 1 100.248 | 1 102.800 |
| Water vapour (H218O) | 1 101.698 | 1 100.597 | 1 102.800 |  |  |  |
| Water vapour (H2O) | 1 113.343 | 1 112.230 | 1 114.456 |  | 1 112.230 | 1 116.175 |
| Water cation (H2O+) | 1 115.059 | 1 113.943 | 1 116.175 |  |  |  |
| Carbon monosulphide (CS) | 1 124.820 | 1 123.696 | 1 125.945 |  | 1 123.696 | 1 125.945 |
| Water vapour (H218O) | 1 136.704 | 1 135.567 | 1 137.840 |  | 1 135.567 | 1 137.840 |
| Hydrogen cyanide (HCN) | 1 151.452 | 1 150.301 | 1 152.603 |  | 1 150.301 | 1 154.280 |
| Carbon monoxide (CO) | 1 151.985 | 1 150.833 | 1 153.137 |  |  |  |
| Water vapour (H2O) | 1 153.127 | 1 151.974 | 1 154.280 |  |  |  |
| Water vapour (H2O) | 1 158.324 | 1 157.165 | 1 159.482 |  | 1 157.165 | 1 159.886 |
| Formylium (HCO+) | 1 158.727 | 1 157.568 | 1 159.886 |  |  |  |
| Heavy water (HDO) | 1 161.953 | 1 160.791 | 1 163.115 |  | 1 160.791 | 1 165.935 |
| Water vapour (H2O) | 1 162.912 | 1 161.749 | 1 164.075 |  |  |  |
| Heavy water (HDO) | 1 164.770 | 1 163.605 | 1 165.935 |  |  |  |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Ammonia (NH3) | 1 168.452 | 1 167.283 | 1 169.620 |  | 1 167.283 | 1 169.620 |
| Carbon monosulphide (CS) | 1 173.539 | 1 172.366 | 1 174.713 |  | 1 172.366 | 1 174.713 |
| Heavy water (HDO) | 1 180.324 | 1 179.143 | 1 181.504 |  | 1 179.143 | 1 182.575 |
| Water vapour (H218O) | 1 181.394 | 1 180.213 | 1 182.575 |  |  |  |
| Water vapour (H218O) | 1 188.863 | 1 187.674 | 1 190.052 |  | 1 187.674 | 1 190.052 |
| Water vapour (H218O) | 1 199.006 | 1 197.807 | 1 200.205 |  | 1 197.807 | 1 200.205 |
| Carbon monoxide (C18O) | 1 206.725 | 1 205.519 | 1 207.932 |  | 1 205.519 | 1 208.846 |
| Water vapour (H2O) | 1 207.639 | 1 206.431 | 1 208.846 |  |  |  |
| Carbon monoxide (13CO) | 1 211.330 | 1 210.118 | 1 212.541 |  | 1 210.118 | 1 212.541 |
| Ammonia (NH3) | 1 214.859 | 1 213.644 | 1 216.073 |  | 1 213.644 | 1 218.476 |
| Ammonia (NH3) | 1 215.245 | 1 214.030 | 1 216.460 |  |  |  |
| Water vapour (H218O) | 1 216.850 | 1 215.634 | 1 218.067 |  |  |  |
| Heavy water (HDO) | 1 217.258 | 1 216.041 | 1 218.476 |  |  |  |
| Carbon monosulphide (CS) | 1 222.234 | 1 221.012 | 1 223.456 |  | 1 221.012 | 1 223.456 |
| Water vapour (H2O) | 1 228.789 | 1 227.560 | 1 230.018 |  | 1 227.560 | 1 231.633 |
| Heavy water (HDO) | 1 230.403 | 1 229.173 | 1 231.633 |  |  |  |
| Hydrogen cyanide (HCN) | 1 239.895 | 1 238.655 | 1 241.134 |  | 1 238.655 | 1 241.134 |
| Formylium (HCO+) | 1 247.735 | 1 246.487 | 1 248.982 |  | 1 246.487 | 1 248.982 |
| Heavy water (HDO) | 1 259.072 | 1 257.813 | 1 260.331 | G | 1 257.813 | 1 262.731 |
| Heavy water (HDO) | 1 261.469 | 1 260.208 | 1 262.731 | G |  |  |
| Carbon monoxide (CO) | 1 267.014 | 1 265.747 | 1 268.282 | G | 1 265.747 | 1 268.310 |
| Heavy water (HDO) | 1 267.043 | 1 265.776 | 1 268.310 | G |  |  |
| Carbon monosulphide (CS) | 1 270.903 | 1 269.632 | 1 272.174 | G | 1 269.632 | 1 272.174 |
| Heavy water (HDO) | 1 277.676 | 1 276.398 | 1 278.954 | G | 1 276.398 | 1 278.954 |
| Heavy water (HDO) | 1 291.642 | 1 290.351 | 1 292.934 | G | 1 290.351 | 1 294.666 |
| Heavy water (HDO) | 1 293.372 | 1 292.079 | 1 294.666 | G |  |  |
| Heavy water (HDO) | 1 297.805 | 1 296.507 | 1 299.103 | G | 1 296.507 | 1 299.103 |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Carbon monoxide (C18O) | 1 316.244 | 1 314.928 | 1 317.560 | G | 1 314.928 | 1 317.560 |
| Carbon monosulphide (CS) | 1 319.545 | 1 318.226 | 1 320.865 |  | 1 318.226 | 1 323.387 |
| Carbon monoxide (13CO) | 1 321.266 | 1 319.944 | 1 322.587 |  |  |  |
| Water vapour (H2O) | 1 322.065 | 1 320.743 | 1 323.387 |  |  |  |
| Hydrogen cyanide (HCN) | 1 328.308 | 1 326.980 | 1 329.637 | G | 1 326.980 | 1 329.637 |
| Formylium (HCO+) | 1 336.714 | 1 335.378 | 1 338.051 | G | 1 335.378 | 1 338.051 |
| Water vapour (H218O) | 1 340.739 | 1 339.398 | 1 342.080 | G | 1 339.398 | 1 342.080 |
| Heavy water (HDO) | 1 353.777 | 1 352.423 | 1 355.130 | G | 1 352.423 | 1 355.130 |
| Carbon monosulphide (CS) | 1 368.160 | 1 366.792 | 1 369.528 | G | 1 366.792 | 1 371.455 |
| Trihydrogen (H2D+) | 1 370.085 | 1 368.715 | 1 371.455 | G |  |  |
| Carbon monoxide (CO) | 1 381.995 | 1 380.613 | 1 383.377 | G | 1 380.613 | 1 383.377 |
| Heavy water (HDO) | 1 385.216 | 1 383.831 | 1 386.601 |  | 1 383.831 | 1 386.601 |
| Heavy water (HDO) | 1 392.919 | 1 391.526 | 1 394.312 |  | 1 391.526 | 1 394.312 |
| Water vapour (H218O) | 1 402.966 | 1 401.563 | 1 404.369 |  | 1 401.563 | 1 404.369 |
| Water vapour (H2O) | 1 410.618 | 1 409.207 | 1 412.029 |  | 1 409.207 | 1 412.029 |
| Hydrogen cyanide (HCN) | 1 416.691 | 1 415.275 | 1 418.108 |  | 1 415.275 | 1 418.162 |
| Carbon monosulphide (CS) | 1 416.745 | 1 415.329 | 1 418.162 |  |  |  |
| Formylium (HCO+) | 1 425.664 | 1 424.238 | 1 427.090 |  | 1 424.238 | 1 427.141 |
| Carbon monoxide (C18O) | 1 425.715 | 1 424.289 | 1 427.141 |  |  |  |
| Carbon monoxide (13CO) | 1 431.153 | 1 429.722 | 1 432.584 |  | 1 429.722 | 1 434.310 |
| Heavy water (HDO) | 1 432.877 | 1 431.444 | 1 434.310 |  |  |  |
| Water vapour (H218O) | 1 438.649 | 1 437.210 | 1 440.087 |  | 1 437.210 | 1 442.222 |
| Water vapour (H2O) | 1 440.782 | 1 439.341 | 1 442.222 |  |  |  |
| Heavy water (HDO) | 1 444.829 | 1 443.384 | 1 446.274 | G | 1 443.384 | 1 448.460 |
| Sulfhydryl (SH) | 1 447.012 | 1 445.564 | 1 448.460 | G |  |  |
| Nitrogen ion (N II) | 1 461.130 | 1 459.669 | 1 462.591 | G | 1 459.669 | 1 462.591 |
| Trihydrogen (D2H+) | 1 476.606 | 1 475.128 | 1 478.083 | G | 1 475.128 | 1 478.083 |
| Heavy water (HDO) | 1 491.927 | 1 490.435 | 1 493.419 | G | 1 490.435 | 1 493.419 |
| Carbon monoxide (CO) | 1 496.923 | 1 495.426 | 1 498.420 | G | 1 495.426 | 1 498.420 |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 1 500.990 | 1 499.489 | 1 502.491 | G | 1 499.489 | 1 502.491 |
| Hydrogen cyanide (HCN) | 1 505.042 | 1 503.537 | 1 506.547 | G | 1 503.537 | 1 508.768 |
| Heavy water (HDO) | 1 507.261 | 1 505.754 | 1 508.768 | G |  |  |
| Formylium (HCO+) | 1 514.582 | 1 513.067 | 1 516.096 | G | 1 513.067 | 1 516.096 |
| Heavy water (HDO) | 1 522.926 | 1 521.403 | 1 524.449 | G | 1 521.403 | 1 524.449 |
| Carbon monoxide (C18O) | 1 535.134 | 1 533.599 | 1 536.669 | G | 1 533.599 | 1 536.669 |
| Carbon monoxide (13CO) | 1 540.989 | 1 539.448 | 1 542.530 |  | 1 539.448 | 1 543.509 |
| Water vapour (H2O) | 1 541.967 | 1 540.425 | 1 543.509 |  |  |  |
| Heavy water (HDO) | 1 577.178 | 1 575.600 | 1 578.755 |  | 1 575.600 | 1 578.755 |
| Hydrogen cyanide (HCN) | 1 593.357 | 1 591.764 | 1 594.951 |  | 1 591.764 | 1 594.951 |
| Water vapour (H2O) | 1 602.219 | 1 600.617 | 1 603.821 |  | 1 600.617 | 1 607.568 |
| Formylium (HCO+) | 1 603.466 | 1 601.862 | 1 605.069 |  |  |  |
| Water vapour (H218O) | 1 605.962 | 1 604.356 | 1 607.568 |  |  |  |
| Carbon monoxide (CO) | 1 611.794 | 1 610.182 | 1 613.405 |  | 1 610.182 | 1 617.242 |
| Heavy water (HDO) | 1 614.294 | 1 612.679 | 1 615.908 |  |  |  |
| Heavy water (HDO) | 1 615.626 | 1 614.010 | 1 617.242 |  |  |  |
| Water vapour (H218O) | 1 620.852 | 1 619.231 | 1 622.472 |  | 1 619.231 | 1 622.472 |
| Heavy water (HDO) | 1 625.408 | 1 623.783 | 1 627.033 |  | 1 623.783 | 1 627.033 |
| Sulfhydrylium (SH+) | 1 632.518 | 1 630.884 | 1 634.151 |  | 1 630.884 | 1 636.274 |
| Water vapour (H218O) | 1 633.484 | 1 631.850 | 1 635.117 |  |  |  |
| Heavy water (HDO) | 1 634.639 | 1 633.005 | 1 636.274 |  |  |  |
| Carbon monoxide (C18O) | 1 644.497 | 1 642.852 | 1 646.141 |  | 1 642.852 | 1 646.141 |
| Heavy water (HDO) | 1 648.801 | 1 647.153 | 1 650.450 |  | 1 647.153 | 1 652.418 |
| Carbon monoxide (13CO) | 1 650.768 | 1 649.117 | 1 652.418 |  |  |  |
| Oxonium (H3O+) | 1 655.813 | 1 654.158 | 1 657.469 |  | 1 654.158 | 1 657.858 |
| Water vapour (H218O) | 1 655.868 | 1 654.212 | 1 657.523 |  |  |  |
| Water vapour (H218O) | 1 656.202 | 1 654.546 | 1 657.858 |  |  |  |
| Water vapour (H2O) | 1 661.008 | 1 659.347 | 1 662.669 |  | 1 659.347 | 1 665.247 |
| Oxonium (H3O+) | 1 663.584 | 1 661.920 | 1 665.247 |  |  |  |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Methylidynium (CH+) | 1 669.170 | 1 667.499 | 1 670.840 |  | 1 667.499 | 1 673.021 |
| Water vapour (H2O) | 1 669.905 | 1 668.235 | 1 671.575 |  |  |  |
| Water vapour (H218O) | 1 671.350 | 1 669.679 | 1 673.021 |  |  |  |
| Heavy water (HDO) | 1 678.578 | 1 676.899 | 1 680.256 |  | 1 676.899 | 1 686.290 |
| Hydrogen cyanide (HCN) | 1 681.636 | 1 679.955 | 1 683.318 |  |  |  |
| Heavy water (HDO) | 1 684.606 | 1 682.921 | 1 686.290 |  |  |  |
| Formylium (HCO+) | 1 692.313 | 1 690.621 | 1 694.006 |  | 1 690.621 | 1 694.006 |
| Water vapour (H2O) | 1 713.883 | 1 712.169 | 1 715.597 |  | 1 712.169 | 1 721.701 |
| Water vapour (H2O) | 1 716.770 | 1 715.053 | 1 718.486 |  |  |  |
| Water vapour (H2O) | 1 716.957 | 1 715.240 | 1 718.674 |  |  |  |
| Water vapour (H218O) | 1 719.250 | 1 717.531 | 1 720.969 |  |  |  |
| Water vapour (H218O) | 1 719.981 | 1 718.261 | 1 721.701 |  |  |  |
| Carbon monoxide (CO) | 1 726.603 | 1 724.876 | 1 728.329 |  | 1 724.876 | 1 728.329 |
| Carbon monoxide (C18O) | 1 753.800 | 1 752.046 | 1 755.554 |  | 1 752.046 | 1 755.554 |
| Heavy water (HDO) | 1 759.978 | 1 758.218 | 1 761.738 |  | 1 758.218 | 1 765.585 |
| Carbon monoxide (13CO) | 1 760.486 | 1 758.726 | 1 762.247 |  |  |  |
| Water vapour (H2O) | 1 762.043 | 1 760.281 | 1 763.805 |  |  |  |
| Ammonia (NH3) | 1 763.525 | 1 761.762 | 1 765.289 |  |  |  |
| Heavy water (HDO) | 1 763.558 | 1 761.795 | 1 765.322 |  |  |  |
| Ammonia (NH3) | 1 763.602 | 1 761.838 | 1 765.365 |  |  |  |
| Ammonia (NH3) | 1 763.821 | 1 762.058 | 1 765.585 |  |  |  |
| Hydrogen cyanide (HCN) | 1 769.877 | 1 768.107 | 1 771.647 |  | 1 768.107 | 1 771.647 |
| Formylium (HCO+) | 1 781.123 | 1 779.342 | 1 782.904 |  | 1 779.342 | 1 782.904 |
| Water vapour (H2O) | 1 794.789 | 1 792.994 | 1 796.584 |  | 1 792.994 | 1 796.584 |
| Water vapour (H218O) | 1 800.475 | 1 798.674 | 1 802.275 |  | 1 798.674 | 1 802.275 |
| Ammonia (NH3) | 1 808.936 | 1 807.127 | 1 810.744 |  | 1 807.127 | 1 812.188 |
| Ammonia (NH3) | 1 810.378 | 1 808.567 | 1 812.188 |  |  |  |
| Water vapour (H218O) | 1 815.853 | 1 814.038 | 1 817.669 |  | 1 814.038 | 1 820.348 |
| Heavy water (HDO) | 1 818.530 | 1 816.711 | 1 820.348 |  |  |  |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Water vapour (H218O) | 1 824.554 | 1 822.729 | 1 826.378 |  | 1 822.729 | 1 826.378 |
| Hydroxyl (OH) | 1 834.745 | 1 832.911 | 1 836.580 |  | 1 832.911 | 1 843.187 |
| Hydroxyl (OH) | 1 837.741 | 1 835.903 | 1 839.578 |  |  |  |
| Hydroxyl (OH) | 1 837.812 | 1 835.974 | 1 839.649 |  |  |  |
| Carbon monoxide (CO) | 1 841.346 | 1 839.504 | 1 843.187 |  |  |  |
| Water vapour (H218O) | 1 846.872 | 1 845.025 | 1 848.719 |  | 1 845.025 | 1 850.154 |
| Heavy water (HDO) | 1 848.306 | 1 846.458 | 1 850.154 |  |  |  |
| Heavy water (HDO) | 1 853.873 | 1 852.019 | 1 855.727 |  | 1 852.019 | 1 855.727 |
| Hydrogen cyanide (HCN) | 1 858.077 | 1 856.219 | 1 859.935 |  | 1 856.219 | 1 859.935 |
| Carbon monoxide (C18O) | 1 863.039 | 1 861.176 | 1 864.902 |  | 1 861.176 | 1 864.902 |
| Water vapour (H2O) | 1 867.749 | 1 865.881 | 1 869.616 |  | 1 865.881 | 1 874.481 |
| Water vapour (H2O) | 1 867.819 | 1 865.951 | 1 869.687 |  |  |  |
| Formylium (HCO+) | 1 869.893 | 1 868.023 | 1 871.763 |  |  |  |
| Carbon monoxide (13CO) | 1 870.141 | 1 868.271 | 1 872.011 |  |  |  |
| Heavy water (HDO) | 1 872.609 | 1 870.736 | 1 874.481 |  |  |  |
| Heavy water (HDO) | 1 877.487 | 1 875.609 | 1 879.364 |  | 1 875.609 | 1 883.172 |
| Water vapour (H2O) | 1 880.753 | 1 878.872 | 1 882.634 |  |  |  |
| Heavy water (HDO) | 1 881.291 | 1 879.410 | 1 883.172 |  |  |  |
| Heavy water (HDO) | 1 890.757 | 1 888.866 | 1 892.647 |  | 1 888.866 | 1 896.218 |
| Water vapour (H2O) | 1 893.687 | 1 891.793 | 1 895.580 |  |  |  |
| Water vapour (H218O) | 1 894.324 | 1 892.429 | 1 896.218 |  |  |  |
| Water vapour (H218O) | 1 899.604 | 1 897.705 | 1 901.504 |  | 1 897.705 | 1 902.438 |
| Carbon ion (C II) | 1 900.537 | 1 898.637 | 1 902.438 |  |  |  |
| Heavy water (HDO) | 1 909.602 | 1 907.693 | 1 911.512 |  | 1 907.693 | 1 911.512 |
| Water vapour (H2O) | 1 918.475 | 1 916.557 | 1 920.394 |  | 1 916.557 | 1 922.353 |
| Water vapour (H2O) | 1 918.485 | 1 916.567 | 1 920.404 |  |  |  |
| Water vapour (H2O) | 1 919.360 | 1 917.440 | 1 921.279 |  |  |  |
| Water vapour (H2O) | 1 919.360 | 1 917.440 | 1 921.279 |  |  |  |
| Heavy water (HDO) | 1 920.433 | 1 918.513 | 1 922.353 |  |  |  |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 1 929.255 | 1 927.326 | 1 931.184 |  | 1 927.326 | 1 932.259 |
| Heavy water (HDO) | 1 930.329 | 1 928.398 | 1 932.259 |  |  |  |
| Heavy water (HDO) | 1 941.797 | 1 939.855 | 1 943.739 |  | 1 939.855 | 1 943.739 |
| Hydrogen cyanide (HCN) | 1 946.235 | 1 944.288 | 1 948.181 |  | 1 944.288 | 1 948.181 |
| Heavy water (HDO) | 1 950.155 | 1 948.204 | 1 952.105 |  | 1 948.204 | 1 952.159 |
| Heavy water (HDO) | 1 950.209 | 1 948.259 | 1 952.159 |  |  |  |
| Azanylidene (NH) | 1 955.027 | 1 953.072 | 1 956.982 |  | 1 953.072 | 1 960.579 |
| Carbon monoxide (CO) | 1 956.018 | 1 954.062 | 1 957.974 |  |  |  |
| Azanylidene (NH) | 1 958.198 | 1 956.240 | 1 960.156 |  |  |  |
| Formylium (HCO+) | 1 958.621 | 1 956.662 | 1 960.579 |  |  |  |
| Heavy water (HDO) | 1 965.554 | 1 963.588 | 1 967.519 |  | 1 963.588 | 1 967.519 |
| Carbon monoxide (C18O) | 1 972.211 | 1 970.239 | 1 974.183 |  | 1 970.239 | 1 987.905 |
| Water vapour (H218O) | 1 974.636 | 1 972.661 | 1 976.611 | G |  |  |
| Azanylidene (NH) | 1 978.464 | 1 976.485 | 1 980.442 | G |  |  |
| Carbon monoxide (13CO) | 1 979.727 | 1 977.747 | 1 981.707 | G |  |  |
| Heavy water (HDO) | 1 982.064 | 1 980.082 | 1 984.046 | G |  |  |
| Water vapour (H218O) | 1 985.919 | 1 983.933 | 1 987.905 | G |  |  |
| Heavy water (HDO) | 1 994.285 | 1 992.291 | 1 996.280 | G | 1 992.291 | 1 996.280 |
| Heavy water (HDO) | 2 003.495 | 2 001.491 | 2 005.498 |  | 2 001.491 | 2 008.785 |
| Heavy water (HDO) | 2 005.180 | 2 003.175 | 2 007.186 |  |  |  |
| Methylidyne (CH) | 2 006.779 | 2 004.772 | 2 008.785 |  |  |  |
| Methylidyne (CH) | 2 010.799 | 2 008.788 | 2 012.810 |  | 2 008.788 | 2 016.880 |
| Heavy water (HDO) | 2 014.370 | 2 012.355 | 2 016.384 |  |  |  |
| Water vapour (H2O) | 2 014.865 | 2 012.850 | 2 016.880 |  |  |  |
| Heavy water (HDO) | 2 019.134 | 2 017.115 | 2 021.154 |  | 2 017.115 | 2 021.154 |
| Heavy water (HDO) | 2 031.743 | 2 029.711 | 2 033.774 |  | 2 029.711 | 2 033.774 |
| Water vapour (H2O) | 2 040.477 | 2 038.436 | 2 042.517 |  | 2 038.436 | 2 042.517 |
| Oxygen (O I) | 2 060.070 | 2 058.010 | 2 062.130 |  | 2 058.010 | 2 062.130 |
| Heavy water (HDO) | 2 064.690 | 2 062.625 | 2 066.755 |  | 2 062.625 | 2 066.755 |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Carbon monoxide (CO) | 2 070.616 | 2 068.545 | 2 072.687 |  | 2 068.545 | 2 076.519 |
| Water vapour (H2O) | 2 074.432 | 2 072.358 | 2 076.507 |  |  |  |
| Water vapour (H2O) | 2 074.444 | 2 072.370 | 2 076.519 |  |  |  |
| Carbon monoxide (C18O) | 2 081.311 | 2 079.229 | 2 083.392 |  | 2 079.229 | 2 085.118 |
| Heavy water (HDO) | 2 082.718 | 2 080.635 | 2 084.801 |  |  |  |
| Heavy water (HDO) | 2 083.035 | 2 080.952 | 2 085.118 |  |  |  |
| Heavy water (HDO) | 2 087.226 | 2 085.139 | 2 089.313 |  | 2 085.139 | 2 091.330 |
| Carbon monoxide (13CO) | 2 089.241 | 2 087.151 | 2 091.330 |  |  |  |
| Water vapour (H218O) | 2 099.963 | 2 097.863 | 2 102.063 |  | 2 097.863 | 2 102.063 |
| Heavy water (HDO) | 2 110.993 | 2 108.882 | 2 113.104 |  | 2 108.882 | 2 113.104 |
| Heavy water (HDO) | 2 121.216 | 2 119.095 | 2 123.338 |  | 2 119.095 | 2 123.338 |
| Water vapour (H218O) | 2 143.752 | 2 141.608 | 2 145.895 |  | 2 141.608 | 2 154.357 |
| Water vapour (H218O) | 2 147.733 | 2 145.585 | 2 149.880 |  |  |  |
| Heavy water (HDO) | 2 149.224 | 2 147.075 | 2 151.373 |  |  |  |
| Heavy water (HDO) | 2 152.205 | 2 150.052 | 2 154.357 |  |  |  |
| Water vapour (H2O) | 2 164.132 | 2 161.968 | 2 166.296 |  | 2 161.968 | 2 166.296 |
| Carbon monoxide (CO) | 2 185.135 | 2 182.950 | 2 187.320 |  | 2 182.950 | 2 187.320 |
| Water vapour (H2O) | 2 196.346 | 2 194.149 | 2 198.542 |  | 2 194.149 | 2 201.998 |
| Heavy water (HDO) | 2 199.798 | 2 197.599 | 2 201.998 |  |  |  |
| Heavy water (HDO) | 2 204.707 | 2 202.503 | 2 206.912 |  | 2 202.503 | 2 206.912 |
| Heavy water (HDO) | 2 213.707 | 2 211.493 | 2 215.921 |  | 2 211.493 | 2 215.921 |
| Water vapour (H2O) | 2 221.751 | 2 219.529 | 2 223.972 |  | 2 219.529 | 2 223.972 |
| Water vapour (H218O) | 2 227.872 | 2 225.644 | 2 230.100 |  | 2 225.644 | 2 230.100 |
| Water vapour (H218O) | 2 242.198 | 2 239.956 | 2 244.440 |  | 2 239.956 | 2 244.440 |
| Water vapour (H2O) | 2 264.150 | 2 261.886 | 2 266.414 |  | 2 261.886 | 2 266.414 |
| Heavy water (HDO) | 2 269.686 | 2 267.416 | 2 271.956 |  | 2 267.416 | 2 271.956 |
| Heavy water (HDO) | 2 278.017 | 2 275.739 | 2 280.295 |  | 2 275.739 | 2 280.295 |
| Heavy water (HDO) | 2 283.942 | 2 281.658 | 2 286.226 |  | 2 281.658 | 2 288.499 |
| Heavy water (HDO) | 2 286.213 | 2 283.926 | 2 288.499 |  |  |  |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 2 291.309 | 2 289.018 | 2 293.601 |  | 2 289.018 | 2 293.601 |
| Heavy water (HDO) | 2 296.248 | 2 293.952 | 2 298.544 |  | 2 293.952 | 2 302.002 |
| Heavy water (HDO) | 2 297.652 | 2 295.355 | 2 299.950 |  |  |  |
| Carbon monoxide (CO) | 2 299.569 | 2 297.269 | 2 301.868 |  |  |  |
| Heavy water (HDO) | 2 299.702 | 2 297.402 | 2 302.002 |  |  |  |
| Carbon monoxide (13CO) | 2 308.031 | 2 305.723 | 2 310.339 |  | 2 305.723 | 2 310.339 |
| Water vapour (H218O) | 2 318.554 | 2 316.236 | 2 320.873 |  | 2 316.236 | 2 320.873 |
| Heavy water (HDO) | 2 327.429 | 2 325.101 | 2 329.756 |  | 2 325.101 | 2 329.756 |
| Heavy water (HDO) | 2 335.788 | 2 333.452 | 2 338.123 |  | 2 333.452 | 2 338.556 |
| Heavy water (HDO) | 2 336.220 | 2 333.883 | 2 338.556 |  |  |  |
| Heavy water (HDO) | 2 343.735 | 2 341.392 | 2 346.079 |  | 2 341.392 | 2 368.266 |
| Water vapour (H2O) | 2 344.290 | 2 341.945 | 2 346.634 |  |  |  |
| Methylene (CH2) | 2 344.726 | 2 342.381 | 2 347.071 |  |  |  |
| Methylene (CH2) | 2 348.622 | 2 346.274 | 2 350.971 |  |  |  |
| Heavy water (HDO) | 2 351.731 | 2 349.380 | 2 354.083 |  |  |  |
| Heavy water (HDO) | 2 355.192 | 2 352.837 | 2 357.548 |  |  |  |
| Ammonia (NH3) | 2 357.210 | 2 354.853 | 2 359.568 |  |  |  |
| Ammonia (NH3) | 2 357.727 | 2 355.369 | 2 360.084 |  |  |  |
| Ammonia (NH3) | 2 358.563 | 2 356.205 | 2 360.922 |  |  |  |
| Heavy water (HDO) | 2 360.340 | 2 357.980 | 2 362.700 |  |  |  |
| Trihydrogen (H2D+) | 2 363.306 | 2 360.943 | 2 365.669 |  |  |  |
| Amino (NH2) | 2 364.268 | 2 361.903 | 2 366.632 |  |  |  |
| Water vapour (H2O) | 2 365.900 | 2 363.534 | 2 368.266 |  |  |  |
| Heavy water (HDO) | 2 382.166 | 2 379.784 | 2 384.548 |  | 2 379.784 | 2 384.548 |
| Water vapour (H218O) | 2 388.325 | 2 385.936 | 2 390.713 |  | 2 385.936 | 2 393.964 |
| Water vapour (H2O) | 2 391.573 | 2 389.181 | 2 393.964 |  |  |  |

| TABLE 1 (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 2 399.218 | 2 396.819 | 2 401.618 |  | 2 396.819 | 2 407.526 |
| Ammonia (NH3) | 2 400.018 | 2 397.618 | 2 402.418 |  |  |  |
| Ammonia (NH3) | 2 400.578 | 2 398.178 | 2 402.979 |  |  |  |
| Ammonia (NH3) | 2 402.265 | 2 399.863 | 2 404.667 |  |  |  |
| Ammonia (NH3) | 2 405.121 | 2 402.716 | 2 407.526 |  |  |  |
| Heavy water (HDO) | 2 411.827 | 2 409.415 | 2 414.239 |  | 2 409.415 | 2 420.888 |
| Carbon monoxide (CO) | 2 413.924 | 2 411.510 | 2 416.338 |  |  |  |
| Water vapour (H218O) | 2 418.469 | 2 416.051 | 2 420.888 |  |  |  |
| Nitrogen ion (N II) | 2 459.379 | 2 456.920 | 2 461.839 |  | 2 456.920 | 2 468.338 |
| Water vapour (H2O) | 2 462.933 | 2 460.470 | 2 465.396 |  |  |  |
| Heavy water (HDO) | 2 463.054 | 2 460.591 | 2 465.517 |  |  |  |
| Hydrogen fluoride (HF) | 2463.427 | 2 460.964 | 2 465.891 |  |  |  |
| Heavy water (HDO) | 2 465.872 | 2 463.406 | 2 468.338 |  |  |  |
| Heavy water (HDO) | 2 477.453 | 2 474.976 | 2 479.931 |  | 2 474.976 | 2 486.788 |
| Heavy water (HDO) | 2 480.807 | 2 478.326 | 2 483.287 |  |  |  |
| Heavy water (HDO) | 2 484.303 | 2 481.819 | 2 486.788 |  |  |  |
| Hydroxyl (18OH) | 2 494.674 | 2 492.179 | 2 497.169 |  | 2 492.179 | 2 516.847 |
| Hydroxyl (18OH) | 2 498.970 | 2 496.471 | 2 501.469 |  |  |  |
| Hydroxyl (17OH) | 2 501.856 | 2 499.355 | 2 504.358 |  |  |  |
| Heavy water (HDO) | 2 502.167 | 2 499.665 | 2 504.669 |  |  |  |
| Hydroxyl (17OH) | 2 506.186 | 2 503.680 | 2 508.692 |  |  |  |
| Hydroxyl (OH) | 2 509.965 | 2 507.455 | 2 512.475 |  |  |  |
| Hydroxyl (OH) | 2 514.333 | 2 511.819 | 2 516.847 |  |  |  |
| Water vapour (H218O) | 2 526.741 | 2 524.214 | 2 529.268 |  | 2 524.214 | 2 531.838 |
| Carbon monoxide (CO) | 2 528.166 | 2 525.638 | 2 530.694 |  |  |  |
| Heavy water (HDO) | 2 529.308 | 2 526.779 | 2 531.838 |  |  |  |

| TABLE 1 (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 2 541.065 | 2 538.524 | 2 543.606 |  | 2 538.524 | 2 547.650 |
| Amino (NH2) | 2 545.105 | 2 542.560 | 2 547.650 |  |  |  |
| Amino (NH2) | 2 553.966 | 2 551.412 | 2 556.520 |  | 2 551.412 | 2 563.471 |
| Heavy water (HDO) | 2 558.905 | 2 556.346 | 2 561.464 |  |  |  |
| Amino (NH2) | 2 560.910 | 2 558.349 | 2 563.471 |  |  |  |
| Heavy water (HDO) | 2 577.667 | 2 575.089 | 2 580.244 |  | 2 575.089 | 2 582.211 |
| Heavy water (HDO) | 2 579.631 | 2 577.052 | 2 582.211 |  |  |  |
| Water vapour (H218O) | 2 591.047 | 2 588.456 | 2 593.638 |  | 2 588.456 | 2 595.117 |
| Heavy water (HDO) | 2 592.524 | 2 589.932 | 2 595.117 |  |  |  |
| Heavy water (HDO) | 2 598.288 | 2 595.690 | 2 600.886 |  | 2 595.690 | 2 606.031 |
| Hydroxyl (OH) | 2 603.427 | 2 600.824 | 2 606.031 |  |  |  |
| Heavy water (HDO) | 2 616.165 | 2 613.549 | 2 618.781 |  | 2 613.549 | 2 618.781 |
| Water vapour (H218O) | 2 622.939 | 2 620.316 | 2 625.562 |  | 2 620.316 | 2 625.562 |
| Water vapour (H2O) | 2 630.960 | 2 628.329 | 2 633.590 |  | 2 628.329 | 2 633.682 |
| Water vapour (H2O) | 2 631.051 | 2 628.420 | 2 633.682 |  |  |  |
| Water vapour (H2O) | 2 640.474 | 2 637.833 | 2 643.114 |  | 2 637.833 | 2 644.963 |
| Carbon monoxide (CO) | 2 642.321 | 2 639.679 | 2 644.963 |  |  |  |
| Water vapour (H2O) | 2 657.666 | 2 655.008 | 2 660.323 |  | 2 655.008 | 2 660.323 |
| Water vapour (H2O) | 2 664.561 | 2 661.897 | 2 667.226 |  | 2 661.897 | 2 677.662 |
| Water vapour (H218O) | 2 666.732 | 2 664.065 | 2 669.398 |  |  |  |
| Heavy water (HDO) | 2 669.415 | 2 666.745 | 2 672.084 |  |  |  |
| Heavy water (HDO) | 2 674.283 | 2 671.609 | 2 676.957 |  |  |  |
| Deuterated hydrogen (HD) | 2 674.987 | 2 672.312 | 2 677.662 |  |  |  |
| Water vapour (H2O) | 2 685.639 | 2 682.953 | 2 688.325 |  | 2 682.953 | 2 688.325 |
| Heavy water (HDO) | 2 735.277 | 2 732.541 | 2 738.012 |  | 2 732.541 | 2 738.012 |
| Water vapour (H218O) | 2 741.675 | 2 738.933 | 2 744.416 |  | 2 738.933 | 2 744.416 |
| Heavy water (HDO) | 2 748.312 | 2 745.564 | 2 751.060 |  | 2 745.564 | 2 751.060 |
| Carbon monoxide (CO) | 2 756.383 | 2 753.626 | 2 759.139 |  | 2 753.626 | 2 759.139 |
| TABLE 1 (continued) |  |  |  |  |  |  |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Water vapour (H2O) | 2 773.977 | 2 771.203 | 2 776.751 |  | 2 771.203 | 2 776.751 |
| Methylene (CH2) | 2 783.064 | 2 780.281 | 2 785.847 |  | 2 780.281 | 2 785.847 |
| Heavy water (HDO) | 2 794.002 | 2 791.208 | 2 796.796 |  | 2 791.208 | 2 796.829 |
| Heavy water (HDO) | 2 794.035 | 2 791.241 | 2 796.829 |  |  |  |
| Water vapour (H218O) | 2 805.384 | 2 802.579 | 2 808.190 |  | 2 802.579 | 2 816.325 |
| Heavy water (HDO) | 2 807.498 | 2 804.691 | 2 810.306 |  |  |  |
| Heavy water (HDO) | 2 809.406 | 2 806.597 | 2 812.216 |  |  |  |
| Heavy water (HDO) | 2 810.880 | 2 808.069 | 2 813.691 |  |  |  |
| Heavy water (HDO) | 2 813.511 | 2 810.698 | 2 816.325 |  |  |  |
| Heavy water (HDO) | 2 834.411 | 2 831.577 | 2 837.245 |  | 2 831.577 | 2 837.245 |
| Water vapour (H218O) | 2 845.980 | 2 843.134 | 2 848.826 |  | 2 843.134 | 2 848.826 |
| Heavy water (HDO) | 2 855.631 | 2 852.776 | 2 858.487 |  | 2 852.776 | 2 861.891 |
| Heavy water (HDO) | 2 859.032 | 2 856.173 | 2 861.891 |  |  |  |
| Carbon monoxide (CO) | 2 870.338 | 2 867.468 | 2 873.208 |  | 2 867.468 | 2 875.782 |
| Heavy water (HDO) | 2 872.909 | 2 870.036 | 2 875.782 |  |  |  |
| Water vapour (H2O) | 2 880.025 | 2 877.145 | 2 882.905 |  | 2 877.145 | 2 890.914 |
| Water vapour (H2O) | 2 884.279 | 2 881.395 | 2 887.163 |  |  |  |
| Water vapour (H2O) | 2 884.312 | 2 881.427 | 2 887.196 |  |  |  |
| Water vapour (H2O) | 2 884.941 | 2 882.056 | 2 887.826 |  |  |  |
| Water vapour (H2O) | 2 884.950 | 2 882.065 | 2 887.835 |  |  |  |
| Water vapour (H218O) | 2 888.026 | 2 885.138 | 2 890.914 |  |  |  |
| Heavy water (HDO) | 2 900.172 | 2 897.272 | 2 903.072 |  | 2 897.272 | 2 906.294 |
| Heavy water (HDO) | 2 903.391 | 2 900.487 | 2 906.294 |  |  |  |
| Heavy water (HDO) | 2 916.109 | 2 913.193 | 2 919.025 |  | 2 913.193 | 2 923.819 |
| Heavy water (HDO) | 2 920.898 | 2 917.977 | 2 923.819 |  |  |  |
| Water vapour (H218O) | 2 938.998 | 2 936.059 | 2 941.937 |  | 2 936.059 | 2 944.831 |
| Water vapour (H218O) | 2 939.000 | 2 936.061 | 2 941.939 |  |  |  |
| Heavy water (HDO) | 2 941.889 | 2 938.947 | 2 944.831 |  |  |  |

| TABLE 1 (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Species | Rest frequency (GHz) | Suggested minimum frequency (GHz) | Suggested maximum frequency (GHz) | Remarks | Consolidated minimum frequency (GHz) | Consolidated maximum frequency (GHz) |
| Heavy water (HDO) | 2 948.042 | 2 945.094 | 2 950.990 |  | 2 945.094 | 2 955.592 |
| Ammonia (NH3) | 2 948.411 | 2 945.462 | 2 951.359 |  |  |  |
| Ammonia (NH3) | 2 948.669 | 2 945.721 | 2 951.618 |  |  |  |
| Ammonia (NH3) | 2 949.480 | 2 946.531 | 2 952.430 |  |  |  |
| Ammonia (NH3) | 2 950.815 | 2 947.864 | 2 953.765 |  |  |  |
| Ammonia (NH3) | 2 952.640 | 2 949.687 | 2 955.592 |  |  |  |
| Water vapour (H2O) | 2 962.111 | 2 959.149 | 2 965.073 |  | 2 959.149 | 2 975.081 |
| Heavy water (HDO) | 2 966.081 | 2 963.115 | 2 969.047 |  |  |  |
| Water vapour (H2O) | 2 968.749 | 2 965.780 | 2 971.717 |  |  |  |
| Water vapour (H218O) | 2 969.868 | 2 966.899 | 2 972.838 |  |  |  |
| Water vapour (H2O) | 2 970.800 | 2 967.829 | 2 973.771 |  |  |  |
| Water vapour (H2O) | 2 970.801 | 2 967.830 | 2 973.772 |  |  |  |
| Oxonium (H3O+) | 2 972.109 | 2 969.137 | 2 975.081 |  |  |  |
| Oxonium (H3O+) | 2 980.735 | 2 977.754 | 2 983.715 |  | 2 977.754 | 3 002.430 |
| Carbon monoxide (CO) | 2 984.168 | 2 981.183 | 2 987.152 |  |  |  |
| Heavy water (HDO) | 2 984.559 | 2 981.575 | 2 987.544 |  |  |  |
| Ammonia (NH3) | 2 989.643 | 2 986.653 | 2 992.632 |  |  |  |
| Water vapour (H218O) | 2 990.139 | 2 987.149 | 2 993.129 |  |  |  |
| Ammonia (NH3) | 2 991.555 | 2 988.564 | 2 994.547 |  |  |  |
| Ammonia (NH3) | 2 994.786 | 2 991.792 | 2 997.781 |  |  |  |
| Heavy water (HDO) | 2 997.115 | 2 994.118 | 3 000.112 | H |  |  |
| Ammonia (NH3) | 2 999.430 | 2 996.431 | 3 002.430 | H | 2 977.754 | 3 002.430 |
| NOTE 1 – G in the “Remarks” indicates that the line is observable from the ground under very good atmospheric condition. H in the “Remarks” indicates that maximum suggested frequencies are exceeding 3 000 GHz. Suggested minimum and maximum frequencies correspond to Doppler-shifted frequencies for +300 km/s and –300 km/s, respectively. References: IAU Resolution A.2 (1991, Buenos Aires; revised 2009, Rio de Janeiro) JPL Molecular Spectroscopy Database (http://spec.jpl.nasa.gov/) The Cologne Database for Molecular Spectroscopy (http://www.astro.uni-koeln.de/cdms/). |  |  |  |  |  |  |

TABLE 2

Bands in Fig. 1 in which the atmospheric transmittance
meets or exceeds the specified amount (GHz)

| 1% | 5% | 10% | 20% |
| --- | --- | --- | --- |
| 998.4-1 060.4 | 1 001.7-1 058.6 | 1 004.7-1 028.4 1 029.3-1 054.9 | 1 011.8-1 013.8 1 014.3-1 019.3 1 019.7-1 026.2 1 029.9-1 041.7 |
| 1 061.9-1 069.6 | – | – | – |
| 1 245.9-1 277.5 | 1 253.1-1 276.6 | 1 259.7-1 275.6 | – |
| 1 279.0-1 295.9 | 1 279.9-1 294.9 | 1 281.3-1 294.1 | 1 285.6-1 286.8 1 287.9-1 289.4 |
| 1 296.9-1 318.0 | 1 297.9-1 316.4 | 1 298.8-1 314.7 | 1 301.8-1 305.4 |
| 1 326.0-1 388.5 | 1 327.7-1 380.9 | 1 329.4-1 374.9 | 1 335.5-1 343.9 1 345.9-1 355.7 1 357.7-1 359.0 |
| 1 434.4-1 438.2 | – | – | – |
| 1 442.7-1 465.4 | 1 445.7-1 464.6 | 1 451.2-1 463.4 | – |
| 1 468.2-1 524.4 | 1 469.0-1 524.0 | 1 469.9-1 523.3 | 1 478.6-1 481.0 1 482.8-1 489.3 1 495.3-1 505.0 1 505.5-1 506.0 1 508.3-1 512.1 |
| 1 525.8-1 540.0 | 1 526.3-1 538.6 | 1 526.9-1 536.6 | – |
| 1 543.9-1 573.9 | 1 545.6-1 566.5 1 567.6-1 568.4 | 1 550.3-1 553.8 | – |
| 1 575.0-1 579.9 | – | – | – |
| 1 827.2-1 833.1 | – | – | – |
| 1 946.7-2 009.2 | 1 977.5-1 983.6 1 987.9-1 999.6 | – | – |
| 2 102.7-2 109.7 | – | – | – |

Based on consideration of the bands at which atmospheric transmittance at a good astronomical site exceeds 5%, enabling useful observations, the following bands are suggested minimum bands for protection of radio astronomy observations conducted from the surface of the Earth:

TABLE 3

Suggested minimum bands for
ground-based radio astronomy

1 000-1 060 GHz

1 250-1 320 GHz

1 325-1 385 GHz

1 445-1 540 GHz

1 545-1 570 GHz

1 975-2 000 GHz

Due to excessive atmospheric attenuation even at high and dry ground-based astronomical observing sites, the use of bands above 2 000 GHz for radio astronomy is limited to airborne- and space-based applications.

Polehampton, E. T. et al. [2007] The ISO LWS high-resolution spectral survey towards Sagittarius B2*. Monthly Notices of the Royal Astronomical Society (MNRAS), Vol. 377, p. 1122.

Lerate, M. R. et al. [2006] A far-infrared molecular and atomic line survey of the Orion KL region. Monthly Notices of the Royal Astronomical Society (MNRAS), Vol. 370, p. 597.

http://spec.jpl.nasa.gov/.

http://www.astro.uni-koeln.de/cdms/.

See http://www.cfa.harvard.edu/~spaine/am/ (queried June 2008).

Specifically, the calculations employed the Sairecabur profile of Example 4 of: Paine, S. [2004]  The am Atmospheric Model. Submillimeter Array Technical Memo #152 (Revision 3). Available at 
http://sma-www.harvard.edu/private/memos/tech_no.html (queried June 2008).