RECOMMENDATION  479-3*

PROTECTION  OF  FREQUENCIES  FOR  RADIOASTRONOMICAL
MEASUREMENTS  IN  THE  SHIELDED  ZONE  OF  THE  MOON

(Question 7/2)

(1974-1978-1982-1990)

Rec. 479-3

The CCIR,

CONSIDERING

(a)	that Annex I** contains a discussion of the shielded zone of the Moon, and preliminary guidelines on the use of the frequency spectrum in this zone;

(b)	that radioastronomical discoveries resulting from observations from spacecraft above the atmosphere of the Earth will reveal unexpected new astronomical phenomena;

(c)	that, in addition to the establishment of line-of-sight communication links for scientific and other purposes between the Earth and spacecraft, it may be necessary to establish links between stations on the far side of the Moon and other stations on or visible from the Earth;

(d)	that Article 29, Nos. 2632-2635 of the Radio Regulations recognizes the necessity of maintaining the shielded zone of the Moon as an area of great potential for observations by the radioastronomy service and by passive space research and consequently as free as possible from transmissions;

(e)	that earth satellites with high apogees, deep-space probes and transmitters located on the Moon may each illuminate the shielded zone,

UNANIMOUSLY RECOMMENDS

1.	that in planning the use of the radio spectrum, both nationally and internationally, account be taken of the need to provide for radioastronomy observations in the shielded zone of the Moon;

2.	that, in taking account of such a need, special attention should be given to those frequency bands in which observations are difficult or impossible from the surface of the Earth;

3.	that the frequency spectrum should be used in the shielded zone of the Moon in keeping with the preliminary guidelines contained in Annex I;

4.	that in the frequency bands which would be considered for joint use by active and passive space stations in the shielded zone of the Moon, radioastronomy observations should be protected from harmful interference. To this end appropriate discussions between concerned administrations may be conducted.

ANNEX  I

THE  PROTECTION  OF  RADIOASTRONOMY  OBSERVATIONS
IN  THE  SHIELDED  ZONE  OF  THE  MOON***

The electromagnetic spectrum is now so heavily used on Earth that much of its potential value for passive scientific research has already been seriously affected. Because of the general increase in radiocommunications, especially involving earth satellites, spacecraft, and deep-space probes, it is important that frequency allocations by the International Telecommunication Union be coordinated to minimize interference with radioastronomy. In particular, since the far side of the Moon is the remaining accessible place where radio observations of the Universe will be possible without interference over the whole radio spectrum; it will be necessary to allocate frequencies for active use by deep-space probes, lunar satellites, scientific instrument packages and research stations on the lunar surface in such a way, that interference with such passive observations is avoided.

Part of the Moon’s surface is always protected from interfering signals generated on and near the Earth because the Moon always presents nearly the same side towards the Earth. It has a period of rotation about its axis equal to its period of revolution around the Earth, but because its orbit is slightly elliptical and inclined, observers on Earth can see somewhat more than half the surface of the Moon. If, in addition, the Moon is viewed from an earth satellite in an orbit of 100 000 km radius, another small fraction is seen. The remaining invisible portion of the Moon’s surface is that which lies more than 23.2° beyond the mean limb of the Moon as seen from the centre of the Earth. The shielded zone of the Moon consists of the shielded area of the Moon’s surface together with an adjacent volume which is shielded from interference originating within a distance of 100 000 km from the centre of the Earth (Article 29, No. 2632.1 of the Radio Regulations).

It is important that low-frequency radioastronomical studies be shielded from interfering signals from the Earth and satellites (such as very low frequency topside ionosonde experiments), since such observations are difficult to perform on Earth below 40 MHz, and particularly below 10 MHz, due to ionospheric opacity and inhomogeneities. These low-frequency studies are capable of providing important data on solar activity, cosmic rays and magnetic fields in our galaxy, and on the low frequency spectra of quasars and pulsars.

At frequencies above 20 GHz, radioastronomy observations made from the Earth’s surface are hampered by atmospheric opacity due to strong transitions of H2O and O2. Although observations are successfully carried out in the atmospheric windows, some important observations of galactic and intergalactic objects and of complex and interesting interstellar molecules can only be made in the absence of an atmosphere, and the absence of interference of these transition frequencies is of great importance in the shielded zone of the Moon.

The shielded zone of the Moon is expected to be free from terrestrial interference over the whole frequency spectrum. It is a unique site for scientific observations. As it is expected that radioastronomical and other scientific experiments will soon be carried out in this zone, it is essential to regulate the activities of the radio services whose facilities may illuminate it. Account must be taken of the requirements of Earth satellites, deep space probes and transmitters located in the shielded zone of the Moon, on the understanding, however, that it is desirable to maintain the shielded zone of the Moon as a zone free from radio interference and hence of great value for passive observations.

The use of the frequency spectrum by services with facilities which are located in the shielded zone of the Moon or which illuminate it could be based on the following preliminary set of guidelines, which will need to be reviewed as additional information is received.

The entire radio frequency spectrum in the shielded zone of the Moon is designated as available for passive users (the Radioastronomy Service and other passive users as defined in the Radio Regulations), with the following exceptions:

–	Frequency bands currently available and allocated in the future to the Space Research Service, and those frequency bands in the Space Operation Service, the Earth Exploration-Satellite Service and the Radiodetermination-Satellite Service, that are required to support space research;

–	Frequency bands currently available or allocated in the future for radiocommunication and for space research transmissions within the lunar shielded zone.

The proposed guidelines do not impose any restriction on existing or future terrestrial radio services or on existing or future space radio services, the transmitters of which are switched on at a distance of less than 100 000 km from the centre of the Earth.

Under the proposed guidelines, existing or future space radio services for which transmitters are switched on at a distance of more than 100 000 km from the Earth and which operate in accordance with the Radio Regulations should coordinate their activities with the Radio Astronomy Service. It is essential that provisions governing compatibility between the Radio Astronomy Service and other services, based on the technical features of the services, be specified by a decision adopted by an ITU Administrative Conference.

_________________

*	This Recommendation is brought to the attention of Study Groups 1 and 5.

**	Annex I contains text from Report 539 which is hereby cancelled.

***	Propagation aspects of this subject are discussed in Report 336, of Study Group 5.