RECOMMENDATION  ITU-R  SA.1024

NECESSARY  BANDWIDTHS  AND  PREFERRED  FREQUENCY  BANDS  FOR
DATA  TRANSMISSION  FROM  EARTH  EXPLORATION  SATELLITES
(NOT  INCLUDING  METEOROLOGICAL  SATELLITES)

(Question ITU-R 139/7)

(1994)

Rec. ITU-R SA.1024

The ITU Radiocommunication Assembly,

considering

a)	that sensors on Earth exploration satellites, currently operating or under development, produce data rates ranging from 2 kbit/s up to 120 Mbit/s;

b)	that, in the future, it is expected that two or more sensors on board the same satellite will produce total data rates up to 600 Mbit/s;

c)	that, although on-board data compression techniques could allow a reduction of symbol rates, it may be necessary to use error correcting coding techniques to achieve the performance objectives thus resulting in increased bandwidths;

d)	that several frequency bands are allocated to the Earth exploration-satellite service (space-to-Earth) and to the inter-satellite service for use by the Earth exploration-satellite service;

e)	that the band 2 200-2 290 MHz is extensively utilized by space and terrestrial services;

f)	that transmission through the Earth’s atmosphere at frequencies above around 10 GHz is affected by rain,

recommends

1.	that Earth exploration-satellite space-to-Earth and space-to-space links requiring low data rates use assignments in the allocated bands below 2 300 MHz;

2.	that Earth exploration-satellite space-to-Earth links requiring medium and high data rates use assignments in the 8 025-8 400 MHz band;

3.	that Earth exploration-satellite space-to-space links requiring high data rates and all-weather data transmission capability use assignments in the 25.25-27.5 GHz band;

4.	that Earth exploration-satellite space-to-Earth links requiring high data rates but with a possible reduction of transmission performance in case of rain use assignments in frequency bands above 25 GHz that are allocated to the earth exploration-satellite service.

_________________