RECOMMENDATION  ITU-R  SA.1163

INTERFERENCE  CRITERIA  FOR  SERVICE  LINKS  IN  DATA
COLLECTION  SYSTEMS  IN  THE  EARTH EXPLORATION- AND
METEOROLOGICAL-SATELLITE  SERVICES

(Question ITU-R 142/7)

(1995)

Rec. ITU-R SA.1163

The ITU Radiocommunication Assembly,

considering

a)	that the hypothetical reference system specified in Recommendation ITU-R SA.1020 defines links for data collection, and data collection platform interrogation;

b)	that interference criteria are needed to ensure that systems can be designed to achieve adequate performance in the presence of interference;

c)	that the interference criteria may be determined using the methodology described in Recommendation ITU-R SA.1022 and the performance objectives listed in Recommendation ITU-R SA.1162;

d)	that interference criteria assists in the development of criteria for sharing bands among systems, including those operating in other services;

e)	that systems in the Earth exploration-satellite (including meteorological-satellite) service must accept an interference threshold at least as high as the threshold of permissible interference;

f)	that Annex 1 presents the parameters of representative systems that provide the basis for permissible levels of interference for pertinent transmissions in the Earth exploration-satellite service,

recommends

1	that the interference levels specified in Table 1 be used as the permissible total levels of interfering signal power at the antenna output of stations operating for service links in the Earth exploration- and meteorological-satellite service.

TABLE  1

Interference criteria for service links of stations in the Earth
exploration-and meteorological-satellite service

| Frequency band (MHz) | Function and type of earth station | Station subject to interference | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than 20% of the time | Interfering signal power (dBW) in the reference bandwidth to be exceeded for no more than p% of the time |
| --- | --- | --- | --- | --- |
| 401-403 | Non-GSO data collection, low-gain antenna | Space station | –178.6 dBW per 1 600 Hz during transmission at elevation angles ³ 5° | –174.7 dBW per 1 600 Hz during transmission at elevation angles ³ 5° p = 0.1 |
| Earth-to-space | GSO data collection, low-gain antenna | Space station | –168.2 dBW per 100 Hz for transmission at elevation angles ³ 3° | –162.4 dBW per 100 Hz for transmission at elevation angles ³ 0° p = 0.1 |
| 460-470 space-to-Earth | GSO data collection platform interrogation | Earth station | –180.5 dBW per 100 Hz for reception at elevation angles ³ 3° | –178.2 dBW per 100 Hz for reception at elevation angles ³ 0° p = 0.1 |
| NOTE 1 – The total interfering signal power level that may be exceeded for no more than x% of the time, where x is less than 20% but greater than the specified short-term time percentage (p% of the time), may be determined by interpolation between the specified values using a logarithmic scale (base 10) for percentage of time and a linear scale for interfering signal power density (dB). NOTE 2 – Using the guidelines in Recommendation ITU-R SA.1022, the permissible levels of interference may be scaled for application to stations with antenna gain or bandwidth values that differ from the specified values. NOTE 3 – Although the interference criteria are based on the systems described in Annex 1, the interference criteria apply to all systems that operate in the subject frequency bands and which provide the specified service functions. NOTE 4 – The interference criteria are specified with respect to the percentage of time of reception by the earth station. Thus, receiver performance statistics associated with reception from or by one particular satellite (i.e., cumulative distribution of BER) are the same as the statistics for reception from several similar satellites. The total time of reception includes time periods associated with signal acquisition (i.e., before and during local ascension of the satellite), receiver synchronization to the data, and synchronized reception of data. Consequently, for interrogation by satellites in low-Earth orbit, the time required for initial signal acquisition and synchronization may constitute up to several tens of seconds out of total satellite visibility periods averaging on the order of 9 min. However, the analyses of short-term performance that are presented in Annex 1 (i.e., performance exceeded for all but a small percentage of time p, p £ 1%) assume that the satellite is located at the minimum elevation angle associated with the applicable performance objective. This yields the BER performance exceeded for all but p% of the time because Eb /N0 and BER are monotonically related to elevation angle. NOTE 5 – The elevation angle exceeded for all but 20% of the time during reception is well approximated by the angle exceeded for all but 20% of the time that the satellite is visible above the minimum elevation angle specified in the performance objective. This approximation is made in the performance analyses presented in Annex 1 because the underlying cumulative time error cannot exceed 1% (i.e., p% of the time) and the associated total error in satellite antenna gain, free space loss, excess path loss, and earth station parameter values are negligible. The resulting elevation angle that is exceeded for all but 20% of the time of reception yields the BER performance exceeded for all but 20% of the time because Eb /N0 and BER are monotonically related to elevation angle. NOTE 6 – Data collection platform interrogation from non-geostationary satellites will be available in the near future. |  |  |  |  |

ANNEX  1

Basis for interference criteria

# 1	Introduction

This Annex 1 presents the parameters used as inputs to the methodology described in Recommendation ITU-R SA.1022 to determine the interference criteria. Tables 2 to 4 summarize these parameters.

TABLE  2

Uplink performance analysis used as a basis for interference criteria
of space stations on board of low orbiting satellites

| Performance factor | ARGOS DCS | ARGOS DCS |
| --- | --- | --- |
| - Link function | Data | Data |
| - Modulation type | PSK | PSK |
| - Frequency range (MHz) | 401-403 | 401-403 |
| - Time (%) | 0.1 | 20 |
| - 17. Received signal power (dBW) | - –167 | - –161 |
| - Modulated power/total power (modulation index 1.1 rad) (dB) | - –1.0 | - –1.0 |
| - 18. Data rate (bit/s) | - 400 | - 400 |
| - 19. Reference bandwidth (Hz) | - 1 600 | - 1 600 |
| - 20. Data rate (dB bit/s) | - 26.0 | - 26.0 |
| - Demodulator implementation loss (dB) | - 2.0 | - 2.0 |
| - 21. Received energy/bit, Eb (dB(W/Hz)) | - –196 | - –190 |
| - 22. Receiver system noise temperature (K) | - 600 | - 600 |
| - 23. Receiver noise spectral density (dB(W/Hz)) | - –200.8 | - –200.8 |
| - 24. Adjacent channel interference power (dB(W/Hz)) | – | – |
| - 25. Inter-system Interference (dB(W/Hz)) | – | – |
| - 26. Total system I+N power density (dB(W/Hz)) | - –200.8 | - –200.8 |
| - 27. Eb /N0 (dB) | - 4.8 | - 10.8 |
| - 28. Link bit error ratio | - 1 ´ 10–5 | - 1 ´ 10–5 |
| - 29. Satellite data storage/handling error ratio | - 0 | - 0 |
| - 30. Total bit error ratio | - 1 ´ 10–5 | - 1 ´ 10–5 |
| - 31. Required Eb /N0 (dB) | - 9.8 | - 9.8 |
| - 32. Margin (dB) | - –5.0 | - 1.0 |
| - 33. Long-term or short-term margin (dB) | - 1.0 | - –5.0 |
| - 34. Permissible degradation, I/N (dB) | - –5.9 | - –9.8 |
| - 35. Interference power (dBW) | - –174.7 | - –178.6 |
| - NOTE 1 – Values on line 17 are derived from Fig. 1. |  |  |

TABLE  3

Uplink performance analysis used as a basis for interference criteria of
space stations on board satellites in geostationary orbit

| Performance factor |  |  |
| --- | --- | --- |
| - Link function | DCP | DCP |
| - Modulation type | BPSK | BPSK |
| - Frequency range (MHz) | 401-403 | 401-403 |
| - Time (%) | 0.1 | 20 |
| - 1. DCP transmitter output power (dBW) | - 7 | - 7 |
| - 2. Filter/cable line losses (dB) | - 3 | - 3 |
| - 3. Impedance mismatch losses (dB) | - 0 | - 0 |
| - 4. Antenna pointing error (dB) | - 0 | - 0 |
| - 5. DCP antenna gain (dBi) | - 11 | - 11 |
| - 6. DCP e.i.x.p. (dBW) | - 15 | - 15 |
| - 7. Antenna elevation angle (degrees) | - 3 | - 3 |
| - 8. Satellite altitude (km) | - 35 880 | - 35 880 |
| - 9. Free space loss (dB) | - 177 | - 177 |
| - 10. Excess path loss including rain attenuation (dB) | - 1 | - 0 |
| - 11. Satellite antenna gain (dBi) | - 9.9 | - 9.9 |
| - 12. Antenna pointing error (dB) | - 0 | - 0 |
| - 13. Receiver line loss (dB) | - 1.9 | - 1.9 |
| - 14. Polarization mismatch loss (dB) | - 0.2 | - 0.2 |
| - 15. Residual carrier loss (dB) | - 0 | - 0 |
| - 16. Demodulator implementation loss (dB) | - 0 | - 0 |
| - 17. Received signal power (dBW) | - –155.1 | - –154.1 |
| - 18. Data rate (bit/s) | - 100 | - 100 |
| - 19. Reference bandwidth (Hz) | - 100 | - 100 |
| - 20. Data rate (dB bit/s) | - 20 | - 20 |
| - 21. Received energy/bit, Eb (dB(W/Hz)) | - –175.1 | - –174.1 |
| - 22. Receiver system noise temperature (K) | - 395.4 | - 395.4 |
| - 23. Receiver noise spectral density (dB(W/Hz)) | - –202.6 | - –202.6 |
| - 24. Adjacent channel interference power (dB(W/Hz)) | - – | – |
| - 25. Inter-system interference (dB(W/Hz)) | - – | – |
| - 26. Total system I+N power density (dB(W/Hz)) | - –202.6 | - –202.6 |
| - 27. Eb /N0 (dB) | - 27.6 | - 28.6 |
| - 28. Link bit error ratio | - 1 ´ 10– 4 | - 1 ´ 10– 4 |
| - 29. Satellite data storage/handling error ratio | - – | – |
| - 30. Total bit error ratio | - 1 ´ 10– 4 | - 1 ´ 10– 4 |
| - 31. Required Eb /N0 (dB) | - 8.4 | - 8.4 |
| - 32. Margin (dB) | - 19.2 | - 20.2 |
| - 33. Long-term or short-term margin (dB) | - 20.2 | - 19.2 |
| - 34. Permissible degradation, I/N (dB) | - 20.2 | - 14.4 |
| - 35. Interference power (dBW) | - –162.4 | - –168.2 |

TABLE  4

Downlink performance analysis used as a basis for interference criteria of 
earth stations operating with satellites in geostationary orbit

| Performance Factor |  |  |
| --- | --- | --- |
| Link function | DCPI | DCPI |
| Modulation type | BPSK | BPSK |
| Frequency range (MHz) | 460-470 | 460-470 |
| Time (%) | 0.1 | 20 |
| - 1. DCP transmitter output power (dBW) | - 6.2 | - 6.2 |
| - 2. Filter/cable Line Losses (dB) | - 1.5 | - 1.5 |
| - 3. Impedance mismatch losses (dB) | - 0 | - 0 |
| - 4. Antenna pointing error (dB) | - 0 | - 0 |
| - 5. DCP antenna gain (dBi) | - 10.3 | - 10.3 |
| - 6. DCP e.i.r.p. (dBW) | - 15.0 | - 15.0 |
| - 7. Antenna elevation angle (degrees) | - 3 | - 3 |
| - 8. Satellite altitude (km) | - 35 880 | - 35 880 |
| - 9. Free space loss (dB) | - 178.3 | - 178.3 |
| - 10. Excess path loss including rain attenuation (dB) | - 1 | - 0 |
| - 11. Satellite antenna gain (dBi) | - 3 | - 3 |
| - 12. Antenna pointing error (dB) | - 0 | - 0 |
| - 13. Receiver line loss (dB) | - 1 | - 1 |
| - 14. Polarization mismatch loss (dB) | - 0.5 | - 0.5 |
| - 15. Demodulator implementation loss (dB) | - 2 | - 2 |
| - 16. Data modulation loss (dB) | - 1.2 | - 1.2 |
| - 17. Received signal power (dBW) | - –165.9 | - –164.9 |
| - 18. Data rate (bit/s) | - 100 | - 100 |
| - 19. Reference bandwidth (Hz) | - 100 | - 100 |
| - 20. Data rate (dB bit/s) | - 20 | - 20 |
| - 21. Received energy/bit, Eb (dB(W/Hz)) | - –185.9 | - –184.9 |
| - 22. Receiver system noise temperature (K) | - 1338.3 | - 1338.3 |
| - 23. Receiver noise spectral density (dB(W/Hz)) | - –197.3 | - –197.3 |
| - 24. Adjacent channel interference power (dB(W/Hz)) | - – | - – |
| - 25. Inter-system interference (dB(W/Hz)) | - – | - – |
| - 26. Total system I + N power density (dB(W/Hz)) | - –197.3 | - –197.3 |
| - 27. Eb /N0 (dB) | - 11.4 | - 12.4 |
| - 28. Link bit error ratio | - 1 ´ 10–5 | - 1 ´ 10–5 |
| - 29. Satellite data storage/handling error ratio | - – | - – |
| - 30. Total bit error ratio | - 1 ´ 10–5 | - 1 ´ 10–5 |
| - 31. Required Eb /N0 (dB) | - 9.8 | - 9.8 |
| - 32. Margin (dB) | - 1.6 | - 2.6 |
| - 33. Long-term or short-term margin (dB) | - 2.6 | - 1.6 |
| - 34. Permissible degradation, I/N (dB) | - –0.9 | - –3.2 |
| - 35. Interference power (dBW) | - –178.2 | - –180.5 |

# 2	Meteorological-satellite service in the 401-403 MHz band

The ARGOS data collection system (DCS) uplink (see Fig. 1) transmits split-phase, Manchester-encoded, phase-shift keyed (PSK) signals through satellites in low-Earth orbit and operates at a 400 bit/s data transmission rate. The data collection platform (DCP) typically uses a low-gain (5 dBi maximum at 20° elevation angle) antenna and can be a mobile or fixed platform.

# 3	Meteorological-satellite service in the 460-470 MHz band

In the 460-470 MHz band, the geostationary satellites transmit data collection platform interrogations (DCPI) using BPSK modulation.

FIGURE  11163...[D01]

_________________