RECOMMENDATION  ITU-R  SA.1629*

Sharing between command links in the space research and space operation services with the fixed, mobile and mobile-satellite services in the 
frequency band 257-262 MHz

(2003)

The ITU Radiocommunication Assembly,

considering

a)	that in the range between 0.1 and 1 GHz, the space research service (SRS) and space operation service have a combined primary allocation of 3.9 MHz and a combined secondary allocation of 10.35 MHz in the space-to-Earth direction, but only a combined 2.4 MHz in the Earth-to-space direction (telecommand links), the latter allocation being subject to agreement under No. 9.21 of the Radio Regulations;

b)	that Recommendation ITU-R SA.363 contains recommendations on the necessary bandwidth and protection criteria for space operation systems in the frequency range from 0.1 GHz to 30 GHz;

c)	that Recommendations ITU-R SA.364 and ITU-R SA.609 contain recommendations on the necessary bandwidth and protection criteria for manned and unmanned near-Earth satellites in the SRS;

d)	that Recommendation ITU-R SA.514 contains interference criteria for command links in the Earth-to-space direction in bands below 1 GHz;

e)	that Recommendation ITU-R SA.1017 provides a method for calculating link performance in the SRS;

f)	that criteria are given in Recommendation ITU-R F.758 for sharing between the terrestrial fixed service and other services which may be used in bands below 1 GHz;

g)	that the method of determining the coordination area around an earth station operating with non-GSO satellites in frequency bands shared with terrestrial services is set out in Recommendations ITU-R SM.1448 and ITU-R M.1185,

noting

a)	that a small number of earth stations (less than ten) operating in the space research and space operation services (Earth-to-space) may be expected to be deployed in the frequency band 257-262 MHz by a very limited number of administrations (one or two),

noting further

a)	that necessary sharing, compatibility and protection criteria need to be established in the frequency band 257-262 MHz,

recommends

1	that analysis of compatibility with other services in the frequency band 257-262 MHz be based on the characteristics of space research and space operations (Earth-to-space) systems given in Annex 1;

2	that in those countries operating SRS and space operation service systems in the band 257-262 MHz, the coordination distance for earth stations of these services be 400 km.

Annex 1

Link performance analysis and technical equipment characteristics
for command links of space research and space operation systems

This Annex sets out the parameters used under the method in Recommendation ITU-R SA.1017 for analysing the performance characteristics of command links in the SRS and space operation service in the band 257-262 MHz. A generalized link performance analysis is given in Table 1. The technical characteristics of command systems that may be used for analysing compatibility with other radio systems in the band 257-262 MHz are given in Table 2.

TABLE  1

| No. | Characteristic | Value |  |
| --- | --- | --- | --- |
| 1 | Frequency (MHz) | 257-262 |  |
| 2 | Satellite circular orbit height (km) | 400.0 | 2 000.0 |
| 3 | Maximum link distance (km) (for 5° elevation angle) | 1 800.0 | 5 000.0 |
| 4 | Earth station e.i.r.p. (dBW) | 12.0 | 20.0 |
| 5 | Earth station antenna gain (dB) | 12.0 |  |
| 6 | Free-space propagation loss (dB) | 145.85 | 154.73 |
| 7 | Atmospheric attenuation (dB) | 0.2 |  |
| 8 | Space station antenna gain (dB) | 0 |  |
| 9 | Polarization loss (dB) | 0.3 |  |
| 10 | Modulation/demodulation loss (dB) | 1.0 |  |
| 11 | Implementation loss (dB) | 2.0 |  |
| 12 | Necessary bandwidth (kHz) | 20.0 |  |
| 13 | Space station receiver noise temperature (K) | 1 000 |  |
| 14 | Link function/modulation | Data/PSK |  |

TABLE  1 (end)

| No. | Characteristic | Value |  |
| --- | --- | --- | --- |
| 15 | Data transmission rate (dB/Hz) | 40.0 |  |
| 16 | Applicable energy per bit Eb (dB(W/Hz)) | –177 | –178.23 |
| 17 | Spectral noise power density N0 (dB(W/Hz)) | –198.6 |  |
| 18 | Ratio Eb/N0 (dB) | 21.58 | 20.37 |
| 19 | Threshold ratio Eb/N0, (dB) (for an error probability 10–6) | 10.5 |  |
| 20 | Link margin (dB) | 11.08 | 9.87 |

TABLE  2

Technical characteristics of command systems in the SRS and 
space operation service which may be used for compatibility analysis

| No. | Characteristic | Minimum value | Maximum value |
| --- | --- | --- | --- |
| 1 | Height of orbit (km) | 400.0 | 2 000 |
| 2 | Power at input to earth station antenna (dBW) | 0 | 10.0 |
| 3 | Gain of earth station transmitting antenna (dB) | 12.0 | 20.0 |
| 4 | Necessary emission bandwidth (kHz) | 2.4 | 26.0 |
| 5 | Width of main lobe of transmitting antenna pattern (degrees) | 10.0 | 20.0 |
| 6 | Level of side lobes of transmitting antenna (dB) (from main lobe) | –20 (far side lobes) | –15 (first side lobe) |
| 7 | Space station antenna gain (dB) | 0 | 0 |
| 8 | Noise temperature of space station receiver (K) | 1 000 | 1 000 |

*	 	This Recommendation should be brought to the attention of Radiocommunication Study Groups 8 and 9.