RECOMMENDATION  674

POWER  FLUX-DENSITY  VALUES  TO  FACILITATE  THE  APPLICATION
OF  ARTICLE  14  FOR  FSS  IN  RELATION  TO  THE  FIXED-SATELLITE  SERVICE
IN  THE  11.7-12.2  GHz  BAND  IN  REGION  2

(Study Programme 32F/4)

(1990)

Rec. 674

The CCIR,

CONSIDERING

(a)	that systems in the fixed-satellite service and the fixed service share the band 11.7-12.2 GHz in Region 2;

(b)	that the use of this band for the fixed-satellite service is subject to prior agreement with administrations with services that comply with the Table of Frequency Allocations of the Radio Regulations;

(c)	that the identification of affected administrations using the fixed service in Region 2 will be facilitated by the use of appropriate power flux-density values;

(d)	that the power flux-density (pfd) is one of the technical characteristics of space stations in the fixed-satellite service that have already been used for the determination of criteria which facilitate sharing with terrestrial services;

(e)	that it was never intended to introduce any power flux-density limits in this band due to the nature of the fixed-satellite services that are to be provided in this band in Region 2,

UNANIMOUSLY  RECOMMENDS

1.	that in the frequency band 11.7-12.2 GHz shared between systems in the fixed-satellite service and line-of-sight radio-relay systems in Region 2, the values of power flux-density on the territory of the affected administrations using the fixed service which would be used to identify these administrations to facilitate the application of Article 14 of the Radio Regulations, should be, in any 4 kHz band:

–150 + 0,5 (q – 5) dB(W/m2) in 4 kHz for 0° £ q £ 05°

–150 + 0.5 (q – 5) dB(W/m2) in 4 kHz for 5° < q < 25°

–140 + 0,5 (q – 5) dB(W/m2) in 4 kHz for 5 <  q ³ 25°

where q is the angle of arrival, in degrees, on the surface of the Earth of the radio-frequency wave;

2.	that the above values relate to the power flux-density and angles of arrival which would be obtained under free-space propagation conditions;

3.	that the technical criteria in this Recommendation do not apply to Regions 1 and 3;

4.	that the following Note should be regarded as part of this Recommendation.

Note – Power flux-density limits where appropriate are contained in the Radio Regulations. The values in RECOMMENDS 1 are intended to facilitate the application of Article 14 and are not intended as pfd limits.

–––––––––––