SECTION  1A:	ENGINEERING  PRINCIPLES  AND  TECHNIQUES,  INCLUDING  COMPUTER-AIDED
ANALYSIS  FOR  EFFECTIVE  SPECTRUM  MANAGEMENT

RECOMMENDATION  ITU-R  SM.1045*

FREQUENCY TOLERANCE OF TRANSMITTERS

(Question ITU-R 54/1)

(1994)

Rec. ITU-R SM.1045

The ITU Radiocommunication Assembly,

considering

a)	that Appendix 7 to the Radio Regulations (RR) specifies permissible frequency tolerances applicable to certain categories of stations in the frequency range from 9 kHz to 40 GHz;

b)	that in many cases tighter frequency tolerances may contribute to a better utilization of the radio-frequency spectrum;

c)	that due to technological progress transmitters with more stringent frequency stability than required by RR Appendix 7 can be manufactured at reasonable costs;

d)	that it may be desirable to revise RR Appendix 7.

e)	that long-term design objectives for frequency tolerances of transmitters should be developed based on an improvement of radio-frequency spectrum utilization and the operational, technical and economical requirements of the various radio services,

recommends

1.	that the frequency tolerances given in column 1 of Table 1 should be applied on a provisional basis for the installation of new stations;

2.	that more stringent values than those given in column 1 of Table 1 should be used when operational and technical reasons require;

3.	that the values given in column 2 of Table 1 for some frequency bands and categories of stations should be taken into consideration as the long-term design objective of transmitters based on advances in technology;

4.	that further studies should be carried out by Radiocommunication Study Groups and administrations to examine tolerances achievable now and to determine long-term objective values of frequency tolerance for frequency bands and categories of stations for which values cannot be set at present in column 2 of Table 1.

µTABLE  1

1.	Transmitter frequency tolerance is defined in RR Article 1 and is expressed in parts in 106, unless otherwise indicated ±10-6.

2.	The power shown for the various categories of stations is the peak envelope power (pep) for single-sideband transmitters and the mean power for all other transmitters, unless otherwise indicated. The term “power of a radio transmitter” is defined in RR Article 1.

| Frequency bands |  | Frequency tolerance |  |
| --- | --- | --- | --- |
| (lower limits exclusive, upper limits inclusive) | Categories of stations | Achievable now (Column 1) | Long-term design objective (Column 2) |
| 9-535 kHz | Fixed (9-50 kHz) | 100 |  |
|  | Fixed (50-535 kHz) | 150 | 20 Hz |
|  | Land/coast | 100(1) |  |
|  | Land/coast – digital selective calling (DSC) | 110 Hz |  |
|  | Land/aeronautical | 100 | (>200 W, 50) |
|  | Mobile/ship, emergency, survival | 120 Hz(2) |  |
|  | Mobile/ship – digital selective calling (DSC) | 110 Hz |  |
|  | Mobile/aircraft | 100 |  |
|  | Land mobile | 100 | 20 Hz |
|  | Radiodetermination | 100 |  |
|  | Broadcasting | 110 Hz |  |
| 535-1 606.5 kHz (1 605 Region 2) | Broadcasting | 110 Hz |  |
| 1 606.5 kHz (1 605 | Fixed | 115 |  |
| Region 2) - 4 000 kHz | Fixed – SSB radiotelephony | 120 Hz |  |
|  | Fixed – FSK radiotelegraphy | 110 Hz |  |
|  | Land/coast, aeronautical, base | 150 (£200 W, 100)(1) | 50 |
|  | Land/coast, base – SSB radiotelephony | 120 Hz |  |
|  | Land/coast – digital selective calling (DSC) | 110 Hz |  |
|  | Land/aeronautical – SSB | 110 Hz |  |
|  | Mobile/ship | 120 Hz (A1A, 50)(2) |  |
|  | Mobile/ship – digital selective calling (DSC) | 110 Hz |  |
|  | Mobile/survival | 120 Hz |  |
|  | Mobile/aircraft, EPIRBs | 100 | 50 |
|  | Mobile/aircraft – SSB | 120 Hz |  |
|  | Mobile/land | 150 |  |
|  | Mobile/land – SSB radiotelephony, FSK radiotelegraphy | 140 Hz |  |
|  | Radiodetermination | 110 (£200 W, 20) |  |
|  | Broadcasting | 110 Hz |  |

TABLE  1 (continued)

| Frequency bands |  | Frequency tolerance |  |
| --- | --- | --- | --- |
| (lower limits exclusive, upper limits inclusive) | Categories of stations | Achievable now (Column 1) | Long-term design objective (Column 2) |
| 4-29.7 MHz | Fixed | 110 (£500 W, 20) |  |
|  | Fixed – SSB | 120 Hz |  |
|  | Fixed – digital telegraphy | 110 Hz |  |
|  | Land/coast | 120 Hz(1) |  |
|  | Land/coast – A1A | 110 |  |
|  | Land/coast – digital selective calling (DSC) | 110 Hz |  |
|  | Land/aeronautical | 150 (£500 W, 100) |  |
|  | Land/aeronautical – SSB | 110 Hz |  |
|  | Land/base | 120 |  |
|  | Land/base – SSB radiotelephony | 120 Hz (£500 W, 50 Hz) |  |
|  | Mobile/ship | 150 Hz(2), (3) |  |
|  | Mobile/ship – A1A | 110 |  |
|  | Mobile/ship – digital selective calling (DSC) | 110 Hz |  |
|  | Mobile/survival | 150 Hz |  |
|  | Mobile/aircraft | 100 |  |
|  | Mobile/aircraft – SSB | 120 Hz |  |
|  | Mobile/land mobile | 140(4) |  |
|  | Broadcasting | 110 Hz(5) |  |
|  | Earth | 120 |  |
|  | Space | 120 |  |
| 29.7-108 MHz | Fixed | 120 (£50 W, 30) |  |
|  | Land | 120 |  |
|  | Mobile | 120 (portables £5 W, 40) | 12 |
|  | Radiodetermination | 150 |  |
|  | Broadcasting | 112 kHz (£50 W, 3 kHz) |  |
|  | Broadcasting – TV (sound and vision) | 500 Hz (£50 W pep, 1 kHz) |  |
|  | Earth | 120 |  |
|  | Space | 120 |  |
| 108-470 MHz | Fixed | 115 |  |
|  | Fixed multi-hop radio-relay with direct frequency conversion | 115 |  |
|  | Land/coast | 115 (£3 W, 10) |  |
|  | Land/aeronautical | 120 |  |
|  | Land/base | 115 |  |
|  | Mobile/ship | 110 |  |
|  | Mobile/ship on-board outside 156-174 MHz | 115 |  |

TABLE  1 (continued)

| Frequency bands |  | Frequency tolerance |  |
| --- | --- | --- | --- |
| (lower limits exclusive, upper limits inclusive) | Categories of stations | Achievable now (Column 1) | Long-term design objective (Column 2) |
| 108-470 MHz (cont.) | Mobile/survival | 11 50 (156-174 MHz, 10) |  |
|  | Mobile/aircraft | 11 30 (50 kHz channels, 50) | 10 |
|  | Mobile/land mobile | 111 5 (portables £5 W, 15) | 15 |
|  | Radiodetermination | 11 50 (108-117.975 MHz, 20)(6) |  |
|  | Broadcasting | 11 52 kHz |  |
|  | Broadcasting – TV (sound and vision) | 1 500 Hz (£100 W pep, 2 kHz; £1 W pep, 5 kHz) |  |
|  | Earth | 11 20 |  |
|  | Space | 11 20 |  |
| 470-960 MHz | Fixed | 11 15 | 15 |
|  | Land | 111 5 |  |
|  | Mobile | 111 5 (£3 W, 15) | 15 |
|  | Mobile/aircraft | 11 20 |  |
|  | Radiodetermination | 1 500(6) |  |
|  | Broadcasting – TV (sound and vision) | 1 500 Hz (£100 W pep, 2 kHz; £1 W pep, 10 kHz) | (£100 W pep, 1 kHz) |
|  | Earth | 11 20 | 10 |
|  | Space | 11 20 |  |
| 960-1 215 MHz | Aeronautical radionavigation/land, ship | 11 20(7) |  |
|  | Aeronautical radionavigation/aircraft | 11 50(7) |  |
| 1 215-2 450 MHz | Fixed | 11 50 | 15 |
|  | Land | 11 20 |  |
|  | Mobile | 11 20 |  |
|  | Radiodetermination | 1 500(6) |  |
|  | Broadcasting | 1 100 |  |
|  | Earth | 11 20 | 10 |
|  | Space | 11 20 |  |
| 2 450 MHz-10.5 GHz | Fixed | 11 50 | 30 |
|  | Land | 11 50 |  |
|  | Mobile | 11 50 |  |
|  | Radiodetermination | 1 250(6) |  |
|  | Earth | 11 50 | 10 |
|  | Space | 11 50 | 20 |

TABLE  1 (continued)

| Frequency bands |  | Frequency tolerance |  |
| --- | --- | --- | --- |
| (lower limits exclusive, upper limits inclusive) | Categories of stations | Achievable now (Column 1) | Long-term design objective (Column 2) |
| 10.5-30 GHz | Fixed | 1 100 | 30 |
|  | Land | 1 100 |  |
|  | Mobile | 1 100 |  |
|  | Radiodetermination | 5 000(6) |  |
|  | Broadcasting | 1 100 |  |
|  | Earth | 1 100 | 50 |
|  | Space | 1 100 | 50 |
| Above | Fixed | 1 150 | 50 |
| 30 GHz-275 GHz | Land | 1 150 |  |
|  | Mobile | 1 150 |  |
|  | Radiodetermination | 5 000(6) |  |
|  | Broadcasting | 1 100 |  |
|  | Earth | 1 100 | 75 |
|  | Space | 1 100 | 75 |
| - (1) For coast station transmitters used for direct-printing telegraphy or for data transmission, the tolerance is: – 5 Hz for narrow-band phase-shift keying; – 10 Hz for frequency-shift keying. - (2) For ship station transmitters used for direct-printing telegraphy or for data transmission, the tolerance is: – 5 Hz for narrow-band phase-shift keying; – 10 Hz for frequency-shift keying. - (3) For ship station transmitters in the band 26 175-27 500 kHz, on board small craft, with a carrier power not exceeding 5 W in or near coastal waters and utilizing A3E or F3E and G3E emissions, the frequency tolerance is 40 parts in 106. - (4) The tolerance is 50 Hz for single-sideband radiotelephone transmitters, except for those transmitters operating in the band 26 175-27 500 kHz, and not exceeding a peak envelope power of 15 W, for which the basic tolerance of 40 parts in 106 applies. - (5) For A3E emissions with carrier power of 10 kW or less the tolerance is 15 parts in 106 and 10 parts in 106 in the bands 4-5.95 MHz and 5.95-29.7 MHz respectively. - (6) Where specific frequencies are not assigned to radar stations, the bandwidth occupied by the emissions of such stations shall be maintained wholly within the band allocated to the service and the indicated tolerance does not apply. (7) The tolerance for selective identification features (SIF) of secondary surveillance radar (SSR) interrogators is 200 kHz, and for SIF-SSR transponders 3 MHz. |  |  |  |

§

_________________

* 	This Recommendation should be brought to the attention of Radiocommunication Study Groups 4, 7, 8, 9, 10 and 11.