SECTION  1C:	TECHNIQUES  FOR  SPECTRUM  MONITORING

RECOMMENDATION  ITU-R  SM.1051-1

PRIORITY  OF  IDENTIFYING  AND  ELIMINATING  RADIO  INTERFERENCE
IN  THE  BAND  406-406.1  MHz

(Question ITU-R 63/1)

(1994-1995)

(1994-1995)

Rec. ITU-R SM.1051-1

The ITU Radiocommunication Assembly,

considering

a)	that the band 406-406.1 MHz has been allocated in the Radio Regulations to the mobile-satellite (Earth-space) service for use by emergency position-indicating radio beacons (EPIRBs) in cases of distress or emergency;

b)	that administrations may also authorize use of personal locator beacons, emergency locator transmitters, or equivalent systems, on a national basis using transmitter radiation parameters and characteristics similar to those of EPIRBs;

c)	that as part of the Global Maritime Distress and Safety System, the International Maritime Organization has required carriage of EPIRBs aboard ships operating under the Safety of Life at Sea Convention;

d)	that the purpose of EPIRBs and equivalent systems is to facilitate search and rescue operations by means of satellite locating techniques;

e)	that the use of satellite EPIRBs and related satellite systems is designed to save lives by efficiently bringing rescue services quickly and directly to those in distress;

f)	that satellite receivers have revealed the presence of many interference sources in the band 406-406.1 MHz in regions of the Earth;

g)	that presence of any interference leads to deterioration in the search and rescue satellite system which works on the basis of statistical processing of low-power signals and that such interference may threaten the safety of life or property;

h)	that severe interference in the band 406-406.1 MHz has already been shown to be able to completely mask EPIRB transmissions over ocean areas covering thousands of square kilometres;

j)	that the Radiocommunication Bureau has a programme of coordinating reports of interference in the band 406-406.1 MHz on a worldwide basis and communicates those reports to appropriate administrations, requesting their assistance in eliminating such interference;

k)	that national radio monitoring services may be in a good position to assist in detecting, localizing, and identifying interference sources in this band and thereby significantly contribute to protection of life and property,

recommends

1	that administrations having the capability for monitoring and identifying interference should participate and report regularly to the Radiocommunication Bureau;

2	that additional administrations install and operate local user terminals (LUTs) for detecting interference so as to provide coverage in areas presently unserved;

3	that administrations should immediately locate and eliminate interference in the band 406-406.1 MHz when it is made known;

4	that full use be made of available monitoring and direction-finding capabilities to detect, localize, identify and eliminate radio interference in the band 406-406.1 MHz on a priority basis;

5	that the information regarding the operation of EPIRBs and the associated satellite processing systems given in Annex 1 should be used as an aid to eliminating interference in this band.

ANNEX  1

Operation of the Cospas-Sarsat 406 MHz system

# 1	Principle of 406 MHz EPIRB detection and location

Once activated, a 406 MHz EPIRB transmits a 0.5 s burst every 50 s (see Recommendation ITU-R M.633). A digital message containing identification data is modulated onto the burst. EPIRBs transmit independently of each other, resulting in random timing between bursts from different EPIRBs.

The frequency of each received burst is measured by a receiver-processor aboard the Cospas-Sarat satellites. The frequency is stored, along with the time of reception and the identification data, and this information is retransmitted to Cospas-Sarsat ground stations around the world.

The position of each EPIRB is calculated by the ground stations using the frequencies and times obtained from the satellite, and the satellite position at each of the burst times. This calculation is based on the well-known Doppler effect which relates the received burst frequency to the satellite's relative speed.

The first generation satellite receiver-processors have an input bandwidth of 25 kHz, centred on 406.025 MHz and, from 1996, second generation instruments will have input bandwidths selectable up to 80 kHz, centred at 406.05 MHz.

In addition to having on-board receiver-processors, some polar orbiting satellites are also equipped with repeaters to relay transmissions in the 406-406.1 MHz band directly to ground stations for further processing. The time and frequency measurements are made by the ground station and the location is then determined in a manner similar to that described above. Some geostationary satellites are also equipped with such repeaters, which allow 406 MHz signals to be detected, but not located due to the lack of a significant Doppler shift.

# 2	Processing 406 MHz interference signals

Any signals in the 406-406.1 MHz band which are not transmitted by EPIRBs can interfere with the  detection of real EPIRB signals. Such interfering signals are not necessarily produced by transmitter-operating in the 406-406.1 MHz band, but can result from out-of-band emissions, sidebands, spurious emissions or harmonies falling in the 406-406.1 MHz band from transmitters operating at other frequencies.

In accordance with Resolution No. 205 of the World Administrative Radio Conference for the Mobile Services (WARC MOB-83), administrations were invited to monitor and report on such interference.

This requires installation of a local user terminal LUT having the capability to monitor signals received from the space station. Sources of harmful interference received at the LUT can be used to fix the origin of the interference. It is after the detection of such interference and notification to the appropriate administration by the Radiocommunication Bureau, that terrestrial monitoring facilities can locate the precise station causing the interference.

406 MHz interferers generally transmit continuous signals for a long period of time compared to the half-second EPIRB bursts. Like the EPIRB signals, these near-continuous signals exhibit a Doppler frequency variation that can be used to compute the interferer location. Unlike the processing of EPIRB emissions, no identification code can be extracted from an interfering signal, since its modulation, if any, would not be in the correct format. Interference sources must be identified by spectrum shape-recognition techniques.

The 406 MHz repeater on the Sarsat satellites relays all signals received in the 406-406.1 MHz band to specially equipped ground stations, where EPIRB signals and some interfering signals can be detected and located. This method of interferer location is only possible within about 4 000 km of a Cospas-Sarsat ground station, because it relies upon the mutual visibility of the interferer, the satellite and the ground station.

The first Cospas satellite processor had the capability for measuring and reporting the received frequency of interference signals as well as EPIRB signals. Messages generated by such signals, termed “pseudo-messages” were used to locate a number of interferers in various parts of the world.

The capability for on-board measurement of the frequency of interfering signals was not available in follow-on Cospas and Sarsat processors. However, this capability is reinstated in second generation processors which are being launched, commencing in 1996. In these processors, the “pseudo-message mode” can be enabled periodically by ground command. The “pseudo messages” are then stored on-board the satellites, along with actual EPIRB messages, thereby making it possible to locate interferers on a global basis, rather than only in the vicinity of Cospas-Sarsat ground stations.

# 3	Harmful interference levels

Harmful interference to the Cospas-Sarsat 406 MHz system can occur when interference signals in the 406-406.1 MHz band exceed –190 dB(W(m2 · Hz)) at the satellite antenna (at 850 km), which increases the background noise level by 0.3 dB. This corresponds to an emitter on Earth having an e.i.r.p. of –60 dB(W/Hz) for broadband noise or –40 dBW for a cw signal. Harmful interference from pulsed signals is described in Report ITU-R M.1042.

# 4	Observed locations of interference

Many of the Cospas-Sarsat ground stations monitor and locate interference sources on 406 MHz, and these are reported periodically to the Radiocommunication Bureau. Table 1 lists areas where 406 MHz interference has been detected and located by the Cospas-Sarsat system. Additional interference sources are identified as new reporting ground stations are added.

TABLE  1

Number of observed locations of 406 MHz interference

| Observed location | 1990 | 1991 | 1992 | 1993 | 1994 |
| --- | --- | --- | --- | --- | --- |
| North America | – | – | – | – | – |
| Central/South America | 13 | 11 | 2 | 3 | 3 |
| North Africa | 4 | 2 | 2 | 1 | 5 |
| Europe | 11 | 7 | 7 | 14 | 27 |
| Asia | 5 | 6 | 11 | 9 | 14 |
| Australia | – | – | – | – | – |
| Total | 33 | 26 | 22 | 27 | 49 |

# 5	Locations of receiving ground stations

The following Cospas-Sarsat LUTs are capable of monitoring 406 MHz interference. The area where interference can be monitored by these LUTs is shown in Fig. 1.

| Location LUTs1) |  | Comments(2) |
| --- | --- | --- |
| Australia: | Alice Springs | Periodic monitoring |
| Canada: | Churchill Edmonton Goose Bay | Routine monitoring Routine monitoring Routine monitoring |
| France: | Toulouse | Routine monitoring |
| Hong Kong: | Hong Kong | Routine monitoring |
| India: | Bangalore Lucknow | Periodic monitoring Periodic monitoring |
| Indonesia: | Ambon Jakarta | Periodic monitoring Periodic monitoring |
| Italy: | Bari | Routine monitoring |
| Japan: | Yokohama | Not available for interference monitoring |
| New Zealand: | Wellington | Periodic monitoring |
| Norway: | Tromsø | Routine monitoring |
| Pakistan: | Lahore | Periodic monitoring |
| Russia: | Moscow | Periodic monitoring |
| Singapore: | Singapore | Periodic monitoring |
| Spain: | Maspalomas | Routine monitoring |
| United Kingdom: | Lasham | Routine monitoring |
| United States of America: | Alaska California Guam Hawaii Puerto Rico Texas Maryland (OSE, SSE) | Routine monitoring Routine monitoring Routine monitoring Routine monitoring Periodic monitoring Routine monitoring Routine monitoring |
| (1) Country or geographical area. (2) Periodic monitoring: the LUT can be set by the Cospas-Sarsat Mission Control Centre operator to a special operating mode to check for 406 MHz interference periodically (e.g. once per month) or as needed. Routine monitoring: the LUT automatically monitors for 406 MHz interference each day (e.g. for about 15 min, three or four times a day). |  |  |

FIGURE 1...[D01] = 3 CM

_________________