RECOMMENDATION  ITU-R  SM.1053

METHODS  OF  IMPROVING  HF  DIRECTION-FINDING
ACCURACY  AT  FIXED  STATIONS

(1994)

Rec. ITU-R SM.1053

The ITU Radiocommunication Assembly,

considering

a)	that long-range direction finding is necessary to locate unidentified sources of interference;

b)	that long-range direction finding is necessary to identify many signals;

c)	that a small incremental improvement in bearing accuracy can substantially reduce the fix area;

d)	that a small fix area is necessary to identify the administrations and operators responsible for the signal;

e)	that fixed stations are capable of obtaining class A bearings ( 2 accuracy);

f)	that Recommendation ITU-R SM.854 covers general aspects of HF direction-finding techniques,

recommends

1.	that trained experienced personnel be used to operate the equipment;

2.	that the antenna system be deployed at a location of flat terrain, minimum of interfering structures and remote from industrial activity;

3.	that direction finders be dispersed to allow wide variations in the bearing angles constituting the fix;

4.	that the antenna capture area to wavelength ratio be kept large;

5.	that bearings be classified and more importance be given to highly classified bearings;

6.	that the relative field-strength level of signals be noted and more importance be given to bearings taken on high relative field-strength levels;

7.	that the results of a large number of individual bearings be processed to produce a final bearing;

8.	that automated mathematical procedures be applied to composite bearings and calculate a best fix;

9.	that for sky-wave signals the distance between the direction-finding site and the signal source be considered with less importance given to very close or very distant signals and an optimum distance of approximately 1 000 km for frequencies less than 10 MHz, ranging to values less than 2 000 km in the upper part of the HF spectrum, and depending on propagation conditions;

10.	that check bearings be obtained on known stations and the observed deviations be considered when calculating the fix.