Contents

Description of Proposed Modifications of COURIER-3	2

System Parameters	3

Operational Considerations	4

Derivation of Antenna Pattern for Simulations	4

Analysis	8

Analysis under Rule Of Procedure No. 9.27	8

Analysis of Static PFD	9

Analysis of Dynamic PFD	10

Dynamic Analysis of I/N	13

GSO Systems Assessed in the Downlink	13

Downlink GSO Simulation Results	15

NGSO Systems Assessed in the Downlink	39

Downlink NGSO Simulation Results	41

GSO Systems Assessed in the Uplink	67

NGSO Systems Assessed in the Uplink	67

Uplink NGSO Simulation Results	68

Conclusion	83

# Description of Proposed Modifications of COURIER-3

This document provides an analysis to show there is no more interference caused to systems filed after the COURIER-3 satellite system due to the modification presented herein. The modification adds a single satellite in a new orbital configuration (“Sat 73”) to the existing configuration of the COURIER-3 system (“Main Configuration”). Sat 73 operates at a lower altitude of 575 km and at an inclination of 97.5 degrees. Sat 73 works in conjunction with the Main Configuration and is not a mutually exclusive configuration. While Sat 73 is not mutually exclusive, it is meant to supplement capacity within the Main Configuration rather than replace it. As a result, it is not required to be on all the time. In order to maintain the same date of priority for the modification, it is shown that the addition of Sat 73 does not cause more interference. In addition, the German Administration will not seek additional protection beyond that which was required for the original COURIER-3 system. The scope of this modification is to the groups between the frequency bands 1 610 – 1 626.5 MHz and 2 483.5 – 2 500 MHz. Specifically, these apply to beams T01, T02, T08, T20, and R01, R02, R08 and R20.

## System Parameters

Table 1 below provides an overview of the basic characteristics used in this analysis. It should be noted that the Originally filed configuration and parameters do not change unless Sat 73 is active. This is discussed further under the Operational Considerations heading.

Table 1 – Orbital Characteristics and System Parameters

| Parameter | Originally filed | Proposed for modification |  |
| --- | --- | --- | --- |
| Orbit altitude, km | 800 | 800 | 575 |
| Orbit inclination, degrees | 76 | 76 | 97.5 |
| Number of satellites | 72 in 8 orbital planes | 72 in 8 orbital planes | 1 in 1 orbital plane |
| RAAN (per plane) | 0, 45, 90, 135, 180, 225, 270, 315 | 0, 45, 90, 135, 180, 225, 270, 315 | 50 |
| Starting phase angle (per plane) | 0, 0, 0, 0, 0, 0, 0, 0 | 0, 0, 0, 0, 0, 0, 0, 0 | 0 |
| Phase angle spacing | 40 | 40 | - |
| Earth station antenna maximum power spectral density in uplink beams, dBW / Hz 1610.2 - 1626.3 MHz | -60 | -60 |  |
| Space station maximum EIRP power spectral density, dBW / 1 MHz 2483.7-2499.8 MHz | 14.4 | 14.4 | 11.5 |

## Operational Considerations

In order to ensure the interference environment is not increased, beams/satellites within the Main Configuration shut off when Sat 73 is in view and operational. Whenever the cumulative PFD of the Main Configuration and Sat 73 beams would cause an increase in the originally perceived PFD at the surface of the earth, the Main Configuration’s satellites would be individually shut off to accommodate the new satellite. Alternatively Sat 73 could shut its beam off, however this is not portrayed in this analysis as it would simply result in the original filing parameters and would have the same interference scenario. An alternative method of operation would be to have Sat 73 and the Main Configuration operate on independent frequencies, thus not causing a cumulative PFD increase. For the purpose of this assessment, it is assumed that there is frequency overlap as this creates the worst-case scenario for interference.

It should be noted that the proposed modification allows for optionality in the operation of Sat 73, meaning the interference environment effectively remains at the levels seen within the original Main Configuration, unless Sat 73 is active. In other words, unless Sat 73 is transmitting within the configuration, there is no change in the operational parameters of the Main Configuration in the modification. While Sat 73 is transmitting, all the satellites of the Main Configuration and Sat 73 reduce their power by 2.5 dB to ensure the interference level remains below the levels seen in the original Main Configuration.

Given the difference in inclination of Sat 73 to the satellites in the Main Configuration, it does not operate above a latitude of 76 degrees or below -76 degrees.

## Derivation of Antenna Pattern for Simulations

The original parameters filed for COURIER-3 include 4 beams in the receive direction and 4 beams in the transmit direction for the bands under analysis. These beams are labeled R01, R02, R08 and R20 as well as T01,T02,T08, and T20, and are radially distributed around the satellite subpoint in a hexagonal lattice.

Figure 1. COURIER-3 Satellite transmit antenna pattern

Each beam may span the full frequency range of the bands being assessed and for the purpose of this assessment, they are aggregated to derive the interference environment.

In the original submission, coverage is formed by 37 different fixed beams. For this modification, these 37 beams will be represented by one beam to simplify the presentation of operational parameters.  For the purpose of the analysis undertaken herein, it is assumed that the aggregated beams operate co-frequency and in tandem with the victim systems. In operation, this beam will be formed by several beams in the same way in which it was described in the CR/C publication.

Figure 2. Derived original COURIER-3 PFD from aggregation of 37 beams

The PFD mask shown above (pfd(q) where q is the angle of arrival (elevation angle) at the Earth’s surface), can be used to mathematically determine an EIRP mask (EIRP (γ, H) where γ is the satellite off-axis angle and H is the altitude of the orbit). This conversion happens in two steps. First, γ is converted to an equivalent angle of arrival, q for each beam. Then, the length of the propagation path for the angle of arrival q is determined and used to calculate the spreading loss for the path and the resulting EIRP.

Step 1: Calculation of an angle of arrival in degrees, q, from γ and H:

q=90∘-sin-1Re+HsinγRe

where:

q :	angle of arrival

Re :	earth radius (6 378 km)

H :	altitude of the orbit (km)

γ :	angle below horizontal.

Step 2: Calculation of the EIRP value from the defined pfd(q):

EIRPγ,H=PFDq+10log104πd2

where:

d :	Distance between space station and the considered point on the Earth’s surface (m)

PFD(q) :	dB ( W / ( m2 · MHz ) )

EIRPγ,H :	dB ( W / MHz )

Using these equations, the resulting aggregate EIRP mask was calculated and modeled for use in the simulation software.

Figure 3. Derived Satellite EIRP from Aggregated PFD

It should be noted that the EIRPs shown in Figure 3 above are the EIRP of satellites in the Main Configuration (orange), and Sat 73 (blue) in order to maintain the PFD originally filed in the COURIER-3 system. When the Main Configuration and Sat 73 are both active, the satellites reduce their power by 2.5 dB in order to ensure the interference environment remains below what was seen with the original configuration. The results of this operational scenario are presented under the Dynamic Analysis of I/N heading.

# Analysis

## Analysis under Rule Of Procedure No. 9.27

According to the Rules of Procedures under No. 9.27, taking into account that for the bands in question only frequency overlap is used to trigger coordination, the modified part of the network will need to effect coordination with respect to:

a)	Space networks with a “2D-Date” before D1;

b)	Space networks with a “2D-Date” between D1 and D2, where the nature of the change is such as to increase the interference to or from, as the case may be, the assignments of these networks.

According to Paragraph 2.3.1, where the coordination requirements of the modification involves any network under b) above, the modified assignments will have D2 as their “2D-Date”. Otherwise, they will retain D1 as their “2D‑Date”.

To fulfill these requirements with the intention of maintaining the original date of receipt, the following principles have been applied in the analysis that was carried out:

- With respect to GSO/non-GSO networks; A dynamic I/N analysis is provided to demonstrate that the modification would not increase interference to GSO/non-GSO networks between D1 and D2.

- With respect to Terrestrial and Space networks alike; A dynamic PFD analysis is carried out in order to demonstrate that downlink transmissions would produce a lower power density at the surface of the earth using the modified parameters rather than the original. This provides further assurance that there is no increase in interference to all potentially affected services, including those for which coordination under 9.14 is applicable.

- With respect to interference received from networks and systems between D1 and D2, as stipulated in Section 6 of Radiocommunication Bureau Director’s Report to Radio Regulations Board (Doc. RRB17-2/3 rev.1), this Administration wishes to commit to not requiring any more protection from other non-GSO systems or GSO networks than that required for the original parameters.

The provided analysis follows the guidance given in Section 6 of the Radiocommunication Bureau Director’s Report to the Radio Regulations Board (Doc. RRB17-2/3 rev.1). That is “in the absence of appropriate criteria or calculation methods to verify that there is no increase of interference or protection, the Bureau will thoroughly study the technical justifications provided by the notifying administration to make its finding and publish them to ensure the transparency of the process. Such justifications may be based on static and dynamic interference assessments. For the later one, calculation may be e.g. in the form of a cumulative distribution function of the interference level, expressed as an interference-to-noise (I/N) ratio for varying percentages of time and locations into the subsequently filed non-GSO FSS systems.”

Frequency assignments of COURIER-3 are subject to coordination under Nos. 9.12, 9.12A, 9.14 and 9.21. Since frequency overlap is used both under 9.11A and 9.21, the same list of affected networks is analyzed under 9.11A and 9.21.

## Analysis of Static PFD

As described above, when the Main Configuration is active without Sat 73 - the PFD is depicted by the orange line. As Sat 73 mimics the PFD contour of a satellite in the Main Configuration, and is only operational with Main Configuration satellites in its vicinity turned off, the PFD seen as a function of Sat 73 is the same curve shifted down by 2.5 dB due to the power reduction.

Figure 4. Aggregated PFD for Main Configuration and Sat 73

As shown in the Figure above, the introduction of Sat 73 provides a significant decrease in the downlink interference environment when active in conjunction with the Main Configuration. When Sat 73 is not in operation, the interference environment remains the same as originally filed, as the Main Configuration continues to operate at the original parameters.

In addition to the above, when Sat 73 is active, the uplink EIRP emitted by the associated earth stations are reduced by 1 dBW / MHz from -60 to -61 dBW/MHz.

## Analysis of Dynamic PFD

The purpose of this analysis is to provide a statistical envelope of PFD-levels produced by the modification.

The following assumptions are used:

- For the original configuration, all satellites in the Main Configuration are transmitting constantly.

- For the Modified configuration, Sat 73 is constantly transmitting, and satellites in the Main Configuration are turned off when their beams are in view of Sat 73’s.

- The analysis is performed independently over four latitudes.

Figure 4. Worst case PFD at 80-degree latitude

Figure 5. Worst case PFD at 60-degree latitude

Figure 6. Worst case PFD at 30-degree latitude

Figure 7. Worst case PFD at 0-degree latitude

It is clear from the above figures that the interference environment is significantly reduced when Sat 73 is active in conjunction with the Original Configuration.

## Dynamic Analysis of I/N

The analysis being performed considers two frequency bands.

In the 2483.5 - 2500 MHz downlink band, the frequency assignments associated with COURIER-3 (2483.7 - 2499.8 MHz) have a date of receipt of 15 June 2015. Systems assessed in relation to this frequency range filed their characteristics with the ITU after this date.

In the 1610 - 1626.5 MHz uplink band, the frequency assignments associated with COURIER-3 (1610.2 - 1626.3 MHz) have a date of receipt of 10 Feb 2016. Systems assessed in relation to this frequency range filed their characteristics with the ITU after this date.

Given most earth stations operating in the 2483.5 - 2500 MHz range are navigation or mobile earth stations that have non-directional antenna patterns, both COURIER-3 and the victim earth stations use highest elevation tracking in order to select the satellite they communicate with. Aggregation of interfering power from multiple satellites in the original configuration is not considered for two reasons:

- Given the satellite antenna has been modelled as a co-frequency aggregate of all 37 beams, neighboring satellites in this scenario would invariably not operate co-frequency.

- The assessment using Sat-73 assumes that neighboring satellites in the Original Configuration are turned off, thus no aggressor aggregation occurs. Implementing aggressor aggregation to the analysis of the original configuration would simply increase the level of interference perceived by the victim system and provide more margin for the modification.

For the purpose of this analysis, it is assumed that COURIER-3 and victim earth stations are geographically co-located. In order to determine the most interfering latitude for each system, simulations with an increased timestep were run at 0, 15, 45, 75 and 85 degrees latitude. From these initial simulations, the worst case for each system was rerun using a refined timestep. The results of the refined simulations are presented herein.

### GSO Systems Assessed in the Downlink

The table below captures the GSO network characteristics used in the simulations of systems operating between 2483.7 - 2499.8 MHz and which filed after COURIER-3. Given the typical nature of the associated earth stations, and the consistent orbital characteristics, GSO simulations were carried out with typical victim earth stations found across the relevant systems.

| Satellite Name | Pattern | Reference to APL | Gain (dB) | Noise Temperature (K) |
| --- | --- | --- | --- | --- |
| GSO | A-25*LOG(FI) | APENST806V01 | 10 | 250 |
| GSO | A-25*LOG(FI) | APENST806V01 | 16 | 250 |
| GSO | A-25*LOG(FI) | APENST806V01 | 22 | 250 |
| GSO | A-25*LOG(FI) | APENST806V01 | 25 | 250 |
| GSO | A-25*LOG(FI) | APENST806V01 | 28 | 250 |
| GSO | ABCDphi1 | APENST807V01 | 4 | 250 |
| GSO | ABCDphi1 | APENST807V01 | 10 | 250 |
| GSO | ABCDphi1 | APENST807V01 | 17 | 250 |
| GSO | AP8 | APERR_001V01 | 29 | 250 |
| GSO | AP8 | APERR_001V01 | 30 | 250 |
| GSO | AP8 | APERR_001V01 | 35 | 250 |
| GSO | APEMLA240V01 | APEMLA240V01 | 5 | 250 |
| GSO | APEMLA242V01 | APEMLA242V01 | 14 | 250 |
| GSO | APEMLA244V01 | APEMLA244V01 | 20 | 250 |
| GSO | APEMLA244V01 | APEMLA244V01 | 21 | 250 |
| GSO | APEREC028V01 | APEREC028V01 | 6 | 250 |
| GSO | APEREC028V01 | APEREC028V01 | 9 | 250 |
| GSO | APEREC028V01 | APEREC028V01 | 12 | 250 |
| GSO | APEREC028V01 | APEREC028V01 | 15 | 250 |
| GSO | APEREC028V01 | APEREC028V01 | 16 | 250 |
| GSO | APEUAE229V01 | APEUAE229V01 | 12 | 250 |
| GSO | APEUAE230V01 | APEUAE230V01 | 6 | 250 |
| GSO | APEUAE235V01 | APEUAE235V01 | 16 | 250 |
| GSO | ND-EARTH | APEND_099V01 | -4 | 250 |
| GSO | ND-EARTH | APEND_099V01 | -3 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 0 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 1 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 2 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 3 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 4 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 6 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 7 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 10 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 12 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 15 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 17 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 20 | 250 |
| GSO | ND-EARTH | APEND_099V01 | 5 | 250 |
| GSO | APEREC015V01_G3_91 | APEREC015V01_G3_91 | 32.8 | 250 |
| GSO | APEREC015V01_G8 | APEREC015V01_G8 | 26.3 | 250 |
| GSO | APEREC015V01_G9_6 | APEREC015V01_G9_6 | 24.7 | 250 |
| GSO | APEREC015V01_G12 | APEREC015V01_G12 | 22.7 | 250 |
| GSO | APEREC015V01_G16 | APEREC015V01_G16 | 20.2 | 250 |
| GSO | APERR_001V01_G_29 | APERR_001V01_G_29 | 29 | 250 |
| GSO | APERR_001V01_G_30 | APERR_001V01_G_30 | 30 | 250 |
| GSO | APERR_001V01_G_35 | APERR_001V01_G_35 | 65 | 250 |

### Downlink GSO Simulation Results

Figure 1.1. Downlink I/N for GSO APENST806V01 gain 10 at 0-degree latitude

Figure 1.2. Downlink I/N for GSO APENST806V01 gain 16 at 0-degree latitude

Figure 1.3. Downlink I/N for GSO APENST806V01 gain 22 at 0-degree latitude

Figure 1.4. Downlink I/N for GSO APENST806V01 gain 25 at 0-degree latitude

Figure 1.5. Downlink I/N for GSO APENST806V01 gain 28 at 0-degree latitude

Figure 1.6. Downlink I/N for GSO ABCDphi1 gain 4 at 0-degree latitude

Figure 1.7. Downlink I/N for GSO ABCDphi1 gain 10 at 0-degree latitude

Figure 1.8. Downlink I/N for GSO ABCDphi1 gain 17 at 0-degree latitude

Figure 1.9. Downlink I/N for GSO APERR_001V01 gain 29 at 0-degree latitude

Figure 1.10. Downlink I/N for GSO APERR_001V01 gain 30 at 0-degree latitude

Figure 1.11. Downlink I/N for GSO APERR_001V01 gain 35 at 0-degree latitude

Figure 1.12. Downlink I/N for GSO APEMLA240V01 at 0-degree latitude

Figure 1.13. Downlink I/N for GSO APEMLA242V01 at 0-degree latitude

Figure 1.14. Downlink I/N for GSO APEMLA244V01 gain 20 at 0-degree latitude

Figure 1.15. Downlink I/N for GSO APEMLA244V01 gain 21 at 0-degree latitude

Figure 1.16. Downlink I/N for GSO APEREC028V01 gain 6 at 0-degree latitude

Figure 1.17. Downlink I/N for GSO APEREC028V01 gain 9 at 0-degree latitude

Figure 1.18. Downlink I/N for GSO APEREC028V01 gain 12 at 0-degree latitude

Figure 1.19. Downlink I/N for GSO APEREC028V01 gain 15 at 0-degree latitude

Figure 1.20. Downlink I/N for GSO APEREC028V01 gain 16 at 0-degree latitude

Figure 1.21. Downlink I/N for GSO APEUAE229V01 at 0-degree latitude

Figure 1.22. Downlink I/N for GSO APEUAE230V01 gain 6 at 0-degree latitude

Figure 1.23. Downlink I/N for GSO APEUAE235V01 at 0-degree latitude

Figure 1.24. Downlink I/N for GSO ND gain -4 at 0-degree latitude

Figure 1.25. Downlink I/N for GSO ND gain -3 at 0-degree latitude

Figure 1.26. Downlink I/N for GSO ND gain 0 at 0-degree latitude

Figure 1.27. Downlink I/N for GSO ND gain 1 at 0-degree latitude

Figure 1.28. Downlink I/N for GSO ND gain 2 at 0-degree latitude

Figure 1.29. Downlink I/N for GSO ND gain 3 at 0-degree latitude

Figure 1.30. Downlink I/N for GSO ND gain 4 at 0-degree latitude

Figure 1.31. Downlink I/N for GSO ND gain 5 at 0-degree latitude

Figure 1.32. Downlink I/N for GSO ND gain 6 at 0-degree latitude

Figure 1.33. Downlink I/N for GSO ND gain 7 at 0-degree latitude

Figure 1.34. Downlink I/N for GSO ND gain 10 at 0-degree latitude

Figure 1.35. Downlink I/N for GSO ND gain 12 at 0-degree latitude

Figure 1.36. Downlink I/N for GSO ND gain 15 at 0-degree latitude

Figure 1.37. Downlink I/N for GSO ND gain 17 at 0-degree latitude

Figure 1.38. Downlink I/N for GSO ND gain 20 at 0-degree latitude

Figure 1.39. Downlink I/N for GSO APERC015V01 gain 32.8 at 0-degree latitude

Figure 1.40. Downlink I/N for GSO APERC015V01 gain 26.3 at 0-degree latitude

Figure 1.41. Downlink I/N for GSO APERC015V01 gain 24.7 at 0-degree latitude

Figure 1.42. Downlink I/N for GSO APERC015V01 gain 22.7 at 0-degree latitude

Figure 1.43. Downlink I/N for GSO APERC015V01 gain 20.2 at 0-degree latitude

Figure 1.44. Downlink I/N for GSO APER_001V01 gain 29 at 0-degree latitude

Figure 1.45. Downlink I/N for GSO APER_001V01 gain 30 at 0-degree latitude

Figure 1.45. Downlink I/N for GSO APER_001V01 gain 35 at 0-degree latitude

### NGSO Systems Assessed in the Downlink

The table below captures the NGSO network characteristics used in the simulations of systems operating between 2483.7 - 2499.8 MHz and which were filed after COURIER-3. Systems that use steerable beams were assumed to have non directional antennas.

Table 2. NGSO Systems assessed in the Downlink

| Satellite Name | Notice ID | Earth Station | Gain (dB) | Noise Temperature (K) |
| --- | --- | --- | --- | --- |
| A4NG-C | 118520317 | TM-S2M_TM-S2R | 22.8 | 300 |
| AST-NG-C-2 | 115520171 | TYP_S_A | 0 | 500 |
| AST-NG-C-2 | 115520171 | TYP_S_B | 4 | 500 |
| AST-NG-C-3 | 116520228 | TYP_LS_A-LNU10 | 0 | 500 |
| AST-NG-C-3 | 116520228 | TYP_LS_B-LNU14 | 4 | 500 |
| AST-NG-C-4 | 117520372 | TYP_LS_A-LNU10 | 0 | 500 |
| AST-NG-C-4 | 117520372 | TYP_LS_B-LNU14 | 4 | 500 |
| CLEOSAT_001 | 116520069 | USER | 0 | 400 |
| EB-SAT-LEO-1 | 117520487 | TYP_LUT2_5 | 30 | 200 |
| EB-SAT-LEO-1 | 117520487 | TYP-BAL4 | 3 | 300 |
| EB-SAT-LEO-1B | 117520492 | TYP_LUT2_5 | 30 | 200 |
| EB-SAT-LEO-1B | 117520492 | TYP-BAL4 | 3 | 300 |
| F-SAT-NG-14 | 120520162 | TYP_LS_A | 0 | 500 |
| F-SAT-NG-14 | 120520162 | TYP_LS_B | 4 | 500 |
| GALILEO-NG | 119520308 | TYP-NAV-1 | 4.5 | 100 |
| GW-1 | 119520120 | LT-1 | 24 | 100 |
| GW-2 inc-30 | 120545225 | TYPICAL-NAV-1 | 0 | 440 |
| GW-2 inc-30 | 120545225 | TYPICAL-RDD-2 | 4 | 440 |
| GW-2 inc-30 | 120545225 | TYPICAL-NAV-2 | 4.5 | 440 |
| GW-2 inc-40 | 120545225 | TYPICAL-NAV-1 | 0 | 440 |
| GW-2 inc-40 | 120545225 | TYPICAL-RDD-2 | 4 | 440 |
| GW-2 inc-40 | 120545225 | TYPICAL-NAV-2 | 4.5 | 440 |
| GW-2 inc-50 | 120545225 | TYPICAL-NAV-1 | 0 | 440 |
| GW-2 inc-50 | 120545225 | TYPICAL-RDD-2 | 4 | 440 |
| GW-2 inc-50 | 120545225 | TYPICAL-NAV-2 | 4.5 | 440 |
| GW-2 inc-60 | 120545225 | TYPICAL-NAV-1 | 0 | 440 |
| GW-2 inc-60 | 120545225 | TYPICAL-RDD-2 | 4 | 440 |
| GW-2 inc-60 | 120545225 | TYPICAL-NAV-2 | 4.5 | 440 |
| HISPASAT-LEO-NB | 119520285 | TYP_LUT2_5 | 30 | 200 |
| HISPASAT-LEO-NB | 119520285 | TYP-BAL4 | 3 | 300 |
| INSAT-NAV-GS17 | 117520274 | IRIM | 4 | 1260 |
| INSAT-NAV-GS17 | 117520274 | USER | -3 | 250 |
| MICRONSAT | 118520082 | S-1 | 3 | 298 |
| MICRONSAT | 118520082 | S-1-HT | 3 | 2367 |
| MICRONSAT | 118520082 | S-2 | 0 | 298 |
| MICRONSAT | 118520082 | S-2-HT | 3 | 2367 |
| MICRONSAT | 118520082 | TM-2 | 39.7 | 100 |
| MICRONSAT | 118520082 | TT_C | 38 | 472 |
| NSAT01 | 119520187 | NSSTC | 3 | 400 |
| NSAT01.001 | 119520188 | NSSTC | 3 | 400 |
| SSG-CSL | 116520381 | LRG1 | 31 | 450 |
| SSG-CSL | 116520381 | SM1 | 7.4 | 500 |
| KELYPSIS | 118520098 | UT-S-1 | 2 | 140 |
| KELYPSIS | 118520098 | UT-S-2 | 5 | 140 |
| KELYPSIS | 118520098 | UT-S-3 | 11 | 140 |
| KELYPSIS | 118520098 | UT-S-4 | 17 | 140 |
| KELYPSIS | 118520098 | UT-S-5 | 23 | 140 |
| KELYPSIS | 118520098 | UT-S-6 | 29 | 140 |
| GW-A59 | 120520170 | TYPICAL-NAV-1 | 0 | 440 |
| GW-A59 | 120520170 | TYPICAL-NAV-2 | 4.5 | 440 |
| GW-A59 | 120520170 | TYPICAL-RDD-1 | 0 | 440 |
| GW-A59 | 120520170 | TYPICAL-RDD-2 | 4 | 440 |

Note:

- At the time of writing, GW-A59 is 	still under review by the Bureau.

- GW-2 has multiple defined configurations which have been simulated independently.

- SSG-CSL has a limited operational area of 15 degrees North and South of the equator.

### Downlink NGSO Simulation Results

Figure 2.1. Downlink I/N for A4NG-C TM - S2M_TM-S2R at 45-degree latitude

Figure 2.2. Downlink I/N for AST-NG-C-2 TYP_S_A at 0-degree latitude

Figure 2.3. Downlink I/N for AST-NG-C-2 TYP_S_B at 0-degree latitude

Figure 2.4. Downlink I/N for AST-NG-C-3 LNU10 at 0-degree latitude

Figure 2.5. Downlink I/N for AST-NG-C-3 LNU14 at 0-degree latitude

Figure 2.6  Downlink I/N for AST-NG-C-4 LNU10 at 45-degrees latitude

Figure 2.7. Downlink I/N for AST-NG-C-4 LNU14 at 0-degree latitude

Figure 2.8.  Downlink I/N for CLEOSAT 001 at 0-degree latitude

Figure 2.9.  Downlink I/N for EB-SAT-LEO-1 TYP_LUT2_5 at 15-degrees latitude

Figure 2.10. Downlink I/N for EB-SAT-LEO-1 TYP-BAL4 at 15-degrees latitude

Figure 2.11 Downlink I/N for EB-SAT-LEO-1B TYP_LUT2_5 at 0-degree latitude

Figure 2.12   Downlink I/N for EB-SAT-LEO-1B TYP-BAL4 at 0-degree latitude

Figure 2.13 Downlink I/N for F-SAT-NG-14 TYP_LS_A at 0-degree latitude

Figure 2.14 Downlink I/N for F-SAT-NG-14 TYP_LS_B at 0-degree latitude

Figure 2.15 Downlink I/N for GALILEO-NG TYP-NAV-1 at 0-degree latitude

Figure 2.16 Downlink I/N for GW-1 LT-1 at 0-degree latitude

Figure 2.17. Downlink I/N for GW-2 config 1 (inclination 30) ND gain 0 at 0-degree latitude

Figure 2.18. Downlink I/N for GW-2 config 1 (inclination 30) ND gain 4 at 0-degree latitude

Figure 2.19. Downlink I/N for GW-2 config 1 (inclination 30) ND gain 4.5 at 0-degree latitude

Figure 2.20. Downlink I/N for GW-2 config 2 (inclination 40) ND gain 0 at 0-degree latitude

Figure 2.21. Downlink I/N for GW-2 config 2 (inclination 40) ND gain 4 at 0-degree latitude

Figure 2.22.  Downlink I/N for GW-2 config 2 (inclination 40) ND gain 4.5 at 0-degree latitude

Figure 2.23. Downlink I/N for GW-2 config 3 (inclination 50) ND gain 0 at 0-degree latitude

Figure 2.24. Downlink I/N for GW-2 config 3 (inclination 50) ND gain 4 at 0-degree latitude

Figure 2.25. Downlink I/N for GW-2 config 3 (inclination 50) ND gain 4.5 at 0-degree latitude

Figure 2.26. Downlink I/N for GW-2 config 4 (inclination 60) ND gain 0 at 0-degree latitude

Figure 2.27. Downlink I/N for GW-2 config 4 (inclination 60) ND gain 4 at 0-degree latitude

Figure 2.28. Downlink I/N for GW-2 config 4 (inclination 60) ND gain 4.5 at 0-degree latitude

Figure 2.29. Downlink I/N for HISPASAT-LEO TYP_LUT2_5 at 15-degrees latitude

Figure 2.30. Downlink I/N for HISPASAT-LEO TYP-BAL4 at 15-degrees latitude

Figure 2.31. Downlink I/N for INSAT-NAV-GS17 IRIM at 0-degree latitude

Figure 2.32. Downlink I/N for INSAT-NAV-GS17 USER at 0-degree latitude

Figure 2.33. Downlink I/N for MICRONSAT S-1 at 0-degree latitude

Figure 2.34. Downlink I/N for MICRONSAT S-1-HT at 0-degree latitude

Figure 2.35. Downlink I/N for MICRONSAT S-2 at 0-degree latitude

Figure 2.36. Downlink I/N for MICRONSAT S-2-HT at 0-degree latitude

Figure 2.37. Downlink I/N for MICRONSAT TM-2 at 0-degree latitude

Figure 2.38 Downlink I/N for MICRONSAT TT&C at 0-degree latitude

Figure 2.39. Downlink I/N for NSAT01 NSSTC at 0-degree latitude

Figure 2.40. Downlink I/N for NSAT01.001 NSSTC at 0-degree latitude

Figure 2.41. Downlink I/N for SSG-CSL LRG1 at 15-degrees latitude

Figure 2.42. Downlink I/N for SSG-CSL SM1 at 15-degrees latitude

Figure 2.43. Downlink I/N for KELYPSIS UT-S-1 at 0-degree latitude

Figure 2.44. Downlink I/N for KELYPSIS UT-S-2 at 0-degree latitude

Figure 2.45. Downlink I/N for KELYPSIS UT-S-3 at 0-degree latitude

Figure 2.46. Downlink I/N for KELYPSIS UT-S-4 at 0-degree latitude

Figure 2.47. Downlink I/N for KELYPSIS UT-S-5 at 0-degree latitude

Figure 2.48. Downlink I/N for KELYPSIS UT-S-6 at 0-degree latitude

Figure 2.49. Downlink I/N for GW-A59 TYPICAL-NAV-1 (RNSD1_RDLU2) at 0-degree latitude

Figure 2.50. Downlink I/N for GW-A59 TYPICAL-NAV-2 at 0-degree latitude

Figure 2.51. Downlink I/N for GW-A59 TYPICAL-RDD-1 at 0-degree latitude

Figure 2.52. Downlink I/N for GW-A59 TYPICAL-RDD-2 at 0-degree latitude

### GSO Systems Assessed in the Uplink

For the uplink analysis, the EIRP that was provided in the operational parameters table is sufficient to demonstrate the decreased level of potential interference. COURIER-3 uses earth stations that have a fixed pointing antenna, and given that no operational parameters are changing in terms of the deployment of COURIER-3 earth stations, the change of orbital characteristics would not affect the uplink interference. Nevertheless, this section provides a comparison of I/N on the uplink.

Table 3. GSO parameters used for assessment in the Uplink

| Satellite Name | Steerable | Gain (dB) | Noise Temperature (K) |
| --- | --- | --- | --- |
| GSO | Yes | 35 | 800 |

Note: Simulations results are not provided for GEO in the uplink direction as they are simply vertical lines separated by the difference in uplink power. This is due to the fact that the distance between the GEO satellite and earth station are relatively fixed, and a steered antenna is assumed.

### NGSO Systems Assessed in the Uplink

Table 4. NGSO Systems assessed in the Uplink

| Satellite Name | Notice ID | Beam | Steerable | Gain (dB) | Noise Temperature (K) |
| --- | --- | --- | --- | --- | --- |
| AST-NG-C-3 | 116520228 | LNU7 | Yes | 17 | 290 |
| AST-NG-C-3 | 116520228 | LNU8 | Yes | 18.4 | 290 |
| AST-NG-C-3 | 116520228 | LNU10 | Yes | 20.3 | 290 |
| AST-NG-C-3 | 116520228 | LNU14 | Yes | 23.2 | 290 |
| AST-NG-C-4 | 117520372 | LNU08 | Yes | 18.4 | 290 |
| AST-NG-C-4 | 117520372 | LNU10 | Yes | 20.3 | 290 |
| AST-NG-C-4 | 117520372 | LNU12 | Yes | 21.9 | 290 |
| AST-NG-C-4 | 117520372 | LNU14 | Yes | 23.2 | 290 |
| CLEOSAT | 116520069 | 037 | No | 20 | 600 |
| CLEOSAT | 116520069 | 046 | No | 22 | 600 |
| CLEOSAT | 116520069 | 022 | No | 23 | 600 |
| CLEOSAT | 116520069 | 001 | No | 25 | 600 |
| EB-SAT-LEO-1 | 117520487 | UL | No | 3 | 600 |
| EB-SAT-LEO-1B | 117520492 | UL | No | 3 | 600 |
| F-SAT-NG-14 | 120520162 | L1U | Yes | 21.9 | 290 |
| GW-2 inc-30 | 120520172 | RDLU2 | Yes | 25 | 560 |
| GW-2 inc-40 | 120520172 | RDLU2 | Yes | 25 | 560 |
| GW-2 inc-50 | 120520172 | RDLU2 | Yes | 25 | 560 |
| GW-2 inc-60 | 120520172 | RDLU2 | Yes | 25 | 560 |
| GW-A59 | 120520170 | RDLU2 | Yes | 25 | 560 |
| HIBLEO-2 | 92520145 | 016 | No | 16.4 | 500 |
| HIBLEO-2 | 92520145 | 008 | No | 22.6 | 500 |
| HIBLEO-2 | 92520145 | 013 | No | 22.8 | 500 |
| HIBLEO-2 | 92520145 | 001 | No | 23.9 | 500 |
| HISPASAT-LEO-NB | 119520285 | UL | No | 3 | 600 |
| HOL-MG-A006 | 116520407 | ULLFE | No | 13 | 300 |
| HOL-MG-A006 | 116520407 | ULLFELOW | No | 3.1 | 300 |
| SI-SAT-KURUKURU | 116520443 | UUL1 | No | 4.5 | 450 |
| SI-SAT-KURUKURU | 116520443 | UUL2 | No | 5 | 450 |

### Uplink NGSO Simulation Results

Figure 3.1. Uplink I/N for AST-NG-C-3 LNU7 at 0-degree latitude

Figure 3.2. Uplink I/N for AST-NG-C-3 LNU8 at 0-degree latitude

Figure 3.3. Uplink I/N for AST-NG-C-3 LNU10 at 0-degree latitude

Figure 3.4. Uplink I/N for AST-NG-C-3 LNU14 at 0-degree latitude

Figure 3.5. Uplink I/N for AST-NG-C-4 LNU08 at 15-degrees latitude

Figure 3.6. Uplink I/N for AST-NG-C-4 LNU10 at 45-degrees latitude

Figure 3.7. Uplink I/N for AST-NG-C-4 LNU12 at 15-degrees latitude

Figure 3.8. Uplink I/N for AST-NG-C-4 LNU14 at 0-degree latitude

Figure 3.9. Uplink I/N for CLEOSAT 001 at 0-degree latitude

Figure 3.10. Uplink I/N for CLEOSAT 022 at 0-degree latitude

Figure 3.11. Uplink I/N for CLEOSAT 037 at 0-degree latitude

Figure 3.12. Uplink I/N for CLEOSAT 046 at 0-degree latitude

Figure 3.13. Uplink I/N for EB-SAT-LEO-1 at 15-degrees latitude

Figure 3.14. Uplink I/N for EB-SAT-LEO-1B at 0-degree latitude

Figure 3.15. Uplink I/N for F-SAT-NG-14 at 0-degree latitude

Figure 3.16. Uplink I/N for GW-2 config 1 (inclination 30) at 0-degree latitude

Figure 3.17. Uplink I/N for GW-2 config 2 (inclination 40) at 0-degree latitude

Figure 3.18. Uplink I/N for GW-2 config 3 (inclination 50) at 0-degree latitude

Figure 3.19. Uplink I/N for GW-2 config 4 (inclination 60) at 0-degree latitude

Figure 3.20. Uplink I/N for GW-A59 at 0-degree latitude

Figure 3.21. Uplink I/N for HIBLEO-2 001 at 0-degree latitude

Figure 3.22. Uplink I/N for HIBLEO-2 008 at 0-degree latitude

Figure 3.23. Uplink I/N for HIBLEO-2 013 at 0-degree latitude

Figure 3.24. Uplink I/N for HIBLEO-2 016 at 0-degree latitude

Figure 3.25. Uplink I/N for HISPASAT-LEO-NB at 15-degrees latitude

Figure 3.26. Uplink I/N for HOL-MG-A006 ULLFE at 0-degree latitude

Figure 3.27. Uplink I/N for HOL-MG-A006 ULLFELOW at 0-degree latitude

Figure 3.28. Uplink I/N for SI-SAT-KURUKURU UUL1 at 0-degree latitude

Figure 3.29. Uplink I/N for SI-SAT-KURUKURU UUL2 at 0-degree latitude

# Conclusion

The objective of the analysis carried out was to demonstrate that the addition of Sat 73 and the modification of parameters associated with the COURIER-3 filing do not increase the potential for interference towards terrestrial services, GSO networks or non-GSO systems.

In each scenario presented, the level of interference produced by this modification is lower than the originally filed parameters.

It should also be noted that Sat 73 implements a software defined radio capable of dynamically adjusting operational parameters to ensure that any unforeseen interference not captured in this analysis will be reconciled. Given the above demonstration, the fact that the use of Sat 73 is optional, that the interference environment remains unchanged with the Original Configuration or is reduced when Sat 73 is active, and following the guidance of the Rules of Procedures, under No. 9.27, coordination requirements are not affected.

Power density adjusted to -61 dBW / Hz based on whether Sat 73 is active or not.

EIRP adjusted down by 2.5 dB based on whether Sat 73 is active or not.

EIRP based on spreading loss adjustment to maintain original PFD. While the satellites is capable of transmitting at this EIRP, it will operate 2.5 dB below this value, see Operational Considerations.

The “2D-Date” is the date from which an assignment is taken into account as defined in § 1 e) of Appendix 5.

D1 is the original “2D-Date” of the network undergoing modification.

D2 is the date of receipt of request for modification. Concerning the date of receipt, see the Rule of Procedure on Receivability.

Factors affecting the PFD of Sat 73 due to the difference in altitude are already accounted for. The 2.5 dB decrease is in addition to these factors.

The plot is not meant to represent the aggregation between the Main Configuration and Sat 73, but rather the PFD for an aggregated beam for each independently.