ATTACHMENT 22

NOTES OF ADMINISTRATION OF RUSSIAN FEDERATION

Note 1

Administration of the Russian Federation guarantees that all space-to-Earth transmissions will comply with the PFD limits in Article 21 of the Radio Regulations, by utilizing the method of Annex 1 to the Rule of Procedure concerning No. 21.16 for steerable beams.

Note 2

Administration of the Russian Federation guarantees that in addition to the limits given in Table 21-4, in accordance with No. 21.16.2 the PFD across the 200 MHz in the frequency band 18.6-18.8 GHz produced at the surface of the Earth by emission from the space station PRSS under assumed free-space propagation conditions will not exceed -95 dB(W/m2), except for less than 5% of time, when the limit may be exceeded by up to 3 dB.

The aggregate power of emissions from the space station in the band 200 MHz will be controlled by varying the number and power of the carriers in order to match the pfd limits specified in No.21.16.2 of the Radio Regulations.

Note 3

The antenna coverage of each beam actually consists of multiple steerable beams that can be pointed to any part of the visible Earth. Multiple spot beams may be combined to form larger coverage beams.

Note 4

Each uplink beam has the capability of being strapped to any downlink beam on a transponder-by-transponder basis. Therefore, the Part "Overall link characteristics" is not required.

Note 5

In the frequency bands 29.5-30.0 GHz and 19.7-20.2 GHz Administration of the Russian Federation provides its commitment in respect of item 1.4 of Resolution 156 (WRC-15) for any terrestrial systems operating in the countries listed in No. 5.542 of the Radio Regulations.

Note 6

For the frequency band 14.5-14.75 GHz in Regions 1, 2 and 3 for each beam the Service Area is defined as the countries stipulated in Resolution 163 (WRC-15) and Resolution 164 (WRC-15). Furthermore, the Administration of the Russian Federation provides its commitment that the earth station associated with the filed system will meet the separation distance as specified in No. 5.509E and the power flux-density limits that are specified in No. 5.509D (Final Acts WRC-15, 2-27 November 2015).

Note 7

In the frequency band 42–42.5 GHz Administration of the Russian Federation guarantees that all space-to-Earth transmissions will comply with the power flux-density limits that are specified in No. 5.551I of the Radio Regulations.

Note 8

In the frequency band 13.75−14.0 GHz Administration of the Russian Federation guarantees that all Earth-to-space transmissions of Earth stations will comply with the power flux-density and e.i.r.p limits that are specified in No. 5.502 of the Radio Regulations.

Note 9

For beams U02, U03 Administration of the Russian Federation refers to Nos. 22.31 and 22.34 of the Radio Regulations.

Note 10

Details of the new Operating Agency for inclusion in reference database:

- Federal state unitary enterprise Radio research and development institute (NIIR)

- 16, Kazakova Street, Moscow, Russia, 105064

- Tel.: +7 (499) 261-36-94

- Fax: +7 (499) 261-00-90

- E-Mail: info@niir.ru