BS 02: UHF digital Television broadcasting assignment

Prepare an electronic notice file of frequency 521 MHz assigned to a TV broadcasting station based on the information below, for its recording in the Master Register.

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “Wizard” facility for recording in the Master Register,

Solution

- The notice type to use for the notification of a television broadcasting station in the UHF band is:  T02

- The purpose of the exercise is to record in the Master Register, the relevant fragment is: Article 11

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “ Wizard “  from File Menu

- Specify the Fragment and continue to choose the required action.

- Choose the class of station as “BT” for Television Broadcasting.

Note that the wizard has selected the appropriate Notice type, at the bottom of the screen.

Then press Finish and confirm the summary to open the T02 notice.

- Fill in the required fields as requested.

- The Effective Antenna Height can be calculated using the SRTM3 Terrain Database from Tools menu.

- Enter the Geographical coordinates and the height of Antenna above ground level, and submit. Note that the Effective Antenna Heights and the Altitude of site will be filled automatically.

- Once the required fields are completed, validate the notice. Then save file with no error/warning which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)