BS03 Solution: Modification of an assignment which is recorded in the Master register

Prepare an electronic notice file proposing the assigned Frequency to 497 MHz to a frequency assignment which is recorded in the Master Register having the unique identification code C38/1 for the Administration of Surinam (SUR).

Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “Open a notice from the database” functionality to modify an assignment.

Solution

- TerRaNotices provides a tool “File/Open a notice from the DB”. The frequency assignment to modify can be identified by its unique identification code.

- The frequency assignment notice belongs to the FMTV domain and it is a notice proposing modification to a frequency assignment notice in the Master register, therefore the relevant fragment is Article 11.

- The purpose of the exercise is to modify the radiated power of a recorded assignment; therefore the intent shall be Modify.

- We are submitting changes to a recorded assignment, the identifying elements of a BS assignment is either:

  - Frequency and geographical coordinates, or

  - Unique identification code of the assignment given by the administration, if this item is notified previously and recorded with the Frequency assignment to be modified.

  - For this exercise we use the Unique identification code to identify the Frequency assignment to be modified.

Electronic notice file:

- Open TerRaNotices and select “Open a notice from the DB” option from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- According to the requirement of the exercise, we choose the following items:

Notifying administration: SUR Domain: FMTV   Fragment: Article 11 Recorded assignment

- Then enter: “C38/1” as Unique identifier given by the Administration as requested by the exercise and Add.

- After executing, the required Frequency assignment from the Database will be displayed.

- Change the Notification intention to “Modification”. You will have a dialog box requesting the Target information. We have a choice to enter Assigned Frequency and Geographical coordinates or a Unique identifier. For this exercise, we can enter the given Unique identifier “C38/1 “

Please note by pressing the button “ID” at the bottom left side of the box, the required target identifier fields will be filled automatically.

- Then modify the required item. For this exercise the Assigned Frequency will be modified.

- To finalize, Validate and save the Modification notice in order to notify to ITU through WISFAT.