SOLUTION: FXM 01: Fixed service (point-to-point)

Prepare an electronic notice of frequency 42.2 MHz used for the operation of fixed link, for its recording in the Master Register.

Solution

- The purpose of the exercise is to notify a fixed station for its recording in the Master Register.

- The notice type for fixed station is T11.

- The identifying elements for fixed and Mobile are :

  - Assign Frequency

  - Geographical coordinates

  - Designation of emission (bandwidth code and class of emission)

  - Class of station

  - Operating hours

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list of Nature of Services in Chapter IV, Section 7.

- The class of station is a mandatory field and in this case it is FX; the preface to the BR IFIC provides a complete list of class of stations in Chapter IV, Section 6

- The Frequency does not fall within the bands shared on equal basis with space services, the values for Antenna gain and Radiated power shall be notified in dipole reference.

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use: not earlier than 3 months as it is not within the bands shared with Space Services.

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

Electronic notice file:

- Open TerRaNotices and select new file ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise, select the following from the New notice dialog box;  -Administration CHN;  FXM notices category- T11  type of notice for Transmitting station in the fixed service .

- You will get the T11 notice form shown below; then select Provision RR11.2 and Notification intended for  Addition as requested by the exercise and start to fill the required fields.  Note that the missing fields are listed at the bottom left of the screen. The red color of the boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal.  Fill the remaining missing fields to complete the notice.

- The complete Notice form is shown below with the following screens.

- Validate and save the text file containing the complete T11 notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.