SOLUTION: FXM 02: Fixed service (Point-to-Multipoint) in shared bands

Prepare an electronic notice of frequency 6.960 GHz, which falls within the bands shared on equal basis with the space services, used for the operation of two fixed links for its recording in the Master Register.

Solution

- The purpose of the exercise is to notify two fixed links for its recording in the Master Register.

- The notice type for fixed station is T11. The identifying elements for fixed station are:  Assign Frequency; Geographical coordinates; Designation of emission (bandwidth code and class of emission); Class of station Operating hours

Therefore, if an Administration submits a notification for each link, the identifying elements of those two links are identical, only the antenna characteristics will be different.

The processing system in the BR, allow administrations to change the submitted data at any time, for this, the system needs to identify the concerned notice (with the identifying elements) therefore, the first notice will be replaced by the second notice. 
Administrations need to submit a single notice and the two links are notified within the antenna characteristics (or Operations)

- The assigned frequency falls within the bands shared on equal basis with space services, the following fields are mandatory

  - Altitude of site above sea level

  - Height of Antenna above ground level

  - Elevation angle

- The value of the Maximum antenna gain and radiated power shall be notified in isotropical values.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list of “Nature of services” in Chapter IV, Section 7.

- The class of station is a mandatory field and in this case it is FX; the preface to the BR IFIC provides a complete list of “class of station” in Chapter IV, Section 6

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 years

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- Electronic notice file:

- Open TerRaNotices and Select “Wizard” from File menu. ( TerRaNotices software can be installed from BRIFIC DVD)

The Wizard functionality of TerRaNotices, guides you to select the notice type to use according to the given criteria.

For this exercise we select Viet Nam (VTN) as notifying Administration, and “Update of the Master International Frequency Register (MIFR) under Article 11 of the RR” as requested by the exercise.

- Next we specify the required  action as “ADD” .  Note the candidate notice  types at the bottom for the specified criteria at each stage.

- The Class of station and Station Type shall be selected from the ” Notice’s Parameter” menu  as “FX, Transmitting station”  for the Fixed station. (Please refer to the PREFACE to BRIFIC  which can be found within the BRIFIC DVD) . And enter the Frequency.

- Note that the Notice type selected by the wizard for this requirement is “T11”

- Then confirm the summary to generate the required Notice form.  And fill all the required fields to complete the notice.

- The complete Notice form is shown below with the following screens.

- Validate and save the text file containing the complete T11 notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.